3GPP TSG-SA5 Meeting #164	S5-255226

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0746 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.552 Add time stamp of DL PDU set received by PSA UPF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom Corporation Ltd., Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_Trace_MDT_QoE-OAM |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The PDU set, is introduced to support XR services (AR/VR applications) and interactive media services, comprised of one or more PDUs carrying an application layer payload such as a video frame or video slice. XR services require the transmission delay of the PDU Set to be within the PDU Set Delay Budget (PSDB). Therefore, it is useful to collect the received time of the PDU set at related network nodes for the PDU set delay calculation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add XR service related measurements: Time stamp of DL PDU set received by PSA UPF Fix the prefix of the measurement name in 5.4.14.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot monitor average DL PDU set delay for XR measurement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.14.1, 5.4.14.X(new), A.143 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

5.4.14.1	average DL PDU set delay between NG-RAN and PSA UPF

a)	This measurement provides the average DL PDU set delay between NG-RAN(N3) and PSA UPF(N6).

b)	DER (n=1).

c)	The measurement is obtained by the following method:

For each PDU set (PDU Set Sequence Number PSSNi, see 38.415 [31]), the UPF records the following time stamps and information (see 29.281 [42], 23.501 [4]):

-	T1 is the time stamp when the PSA UPF received the first PDU of a PDU set (PSSNi) at the N6 termination point;

-	T2 is obtained by DL sending time stamp IE in UL PDU SESSION INFORMATION message (see 38.415 [31]) sent from NG-RAN to UPF when the last DL PDU of the PDU set (PSSNi) have been successfully received by NG-RAN at the N3 termination point;

The UPF counts the successfully received number (N) of PDU sets and takes the following calculation:

i=1N(T2i-T1i)N

d)	Each measurement is a real representing the average delay in microseconds.

e)	The measurement name has the form GTP.PDUSetDelayDlUpfNgranMean

f)	UPFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

next change

5.4.14.X	Time stamp of DL PDU set received by PSA UPF

a)	This measurement provides a list of timestamps of DL PDU sets received by PSA UPF over N6 interface.

b)	DER (n=1).

c)	The measurement is a collection of Timestamps. Each timestampe is the first PDU of a PDU set (PSSNi) received by the PSA UPF at the N6 termination point. The number of measurements is equal to the number of PSSNs received by the UPF.

d)	Each measurement is a real representing the local time.

e)	The measurement name has the form GTP.PDUSetTimeDlUpfReceived.PSSN,

Where PSSN indicates the PDU Set Sequence Number (see 38.415 [31]).

f)	EP_N6 (contained by UPFFunction)

g)	Valid for packet switched traffic.

h)	5GS.

next change

A.143	Use case of monitoring of PDU sets management for XR

eXtended Reality (XR) service is considered as one of the key candidate services in 5GA and 6G networks. To support XR services, PDU Set is defined (see TS 26.522 clause 3.1), i.e. one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. frame(s), video slice(s), metadata, etc.).

The PDU Set based QoS handling by the 5G-AN is determined by PDU Set QoS Parameters, such as PDU Set Delay Budget (PSDB), PDU Set Error Rate (PSER), etc., which can be used by the 5G-AN to support the configuration of scheduling and link layer functions.

The DL PSDB applies to the DL PDU Set received by the PSA UPF. To enable support for PSDB, it is required that a maximum inter arrival time between the first received PDU and the last received PDU of a PDU Set complies with SLA. It is therefore necessary to monitor whether the DL PDU set delay exceeds the PSDB is necessary to support the network with frame scheduling and guarantees. The PDU Set end-to-end delay from the UE to the UPF can be obtained by calculating the timestamps of the DL PDU set received by the PSA UPF and the timestamps measured from the UE instruments.

End of changes