3GPP TSG-SA5 Meeting #164	S5-255251

Dallas, USA, 17 - 21 November 2025

Source:	China Unicom

Title:	Rel-20 pCR TR 28.886 Add solution for radio resource optimization based on per SSB usage

Document for:	Approval

Agenda item:	6.20.7

Spec:	3GPP TR 28.886

Version:	0.1.0

Work Item:	FS_eMDAS_Ph4

Comments

Current use cases in TS 28.104 do not have radio resource optimization based on per SSB usage. This use case is proposed to provide intelligently adjust the SSB beam configuration to improve the efficiency of radio resource.

Proposed Changes

* * * First Change * * * *

### 5.3.3	Use case#y: Radio resource optimization based on per SSB usage

#### 5.3.3.1	Description

5G NR adopts massive MIMO technology, where the base station employs directional beams for both UE initial access on common channels and subsequent connection during the RRC_CONNECTED state. Common signals/channels for initial access are transmitted within the Synchronization Signal Block (SSB). SSBs are designed with different patterns, corresponding to the number of SSB beams and their distinct directions.

In Clause 4.3.39 of TS 28.541[x], <<IOC>>CommonBeamformingFunction is defined to represents common beamforming functionality (eg: SSB beams). The CommonBeamformingFunction provides capability to configure the advanced antenna for a sector carrier. The configuration capability is provided by selection of coverageShape, digitalTilt and digitalAzimuth. These attributes represent the wanted coverage area and radiation pattern on a sector carrier related to an antenna transmission point.

For example, the area where a large outdoor stadium holds some competitions is characterized by a high number of users which is challenging to guarantee enough radio resource in this scenario. It is necessary to consider using MDAS to improve radio resource usage. MnS consumer could request MDAS to acquire beam-level management data and recommend coverage shape based on analysis. The beam-level data includes Number of UE related the SSB beam Index (mean), DL PRB Usage per SSB, etc. The recommendation could be used for 3GPP Management System to update the configuration of the target cell's coverage shape on gNodeB.

Figure 5.3.y.1-1 depicts SSB beams covering full cell coverage area. Figure 5.3.y.1-2 depicts SSB beams modifying to meet the UE centralized coverage after updating the coverage shape for target cell of gNodeB using the recommendation by MDAS.

It is anticipated that MDAS can be used to obtain an analysis of the usage of each SSB. This information can then be used to adjust the coverage shape.

| Figure 5.3.y.1-1: Illustration of SSB beams covering full cell coverage area | Figure 5.3.y.1-2: Illustration of SSB beams modifying to meet the UE centralized coverage with MDAS recommendation |
| --- | --- |

#### 5.3.3.2	Potential requirements

REQ-RRO-FUN-01: MDA should have capability for radio resource optimization analysis.

REQ-RRO-FUN-02: MDA capability for radio resource optimization analysis should include the capability of the beam-level radio resource utilisation assessment to indicate if coverage shape configuration of gNodeB is proper based on beam-level management data analytics.

REQ-RRO-FUN-03: MDA capability for radio resource optimization analysis should include the capability to recommend coverage shape configuration for target cell of gNodeB.

#### 5.3.3.3		Potential solutions

Update TS 28.104 [2] to add a new MDA type MDARadioResourceOptimization and MDARadioResourceOptimization.SSBUsageAnalysis. MnS consumer could request MDAS to acquire beam-level management data and recommend coverage shape based on analysis.

The enabling data for MDARadioResourceOptimization.SSBUsageAnalysis would include as follows:

-	Timing Advance distribution for NR Cell per SSB

-	DL data transmission time per SSB

-	Number of UE related the SSB beam Index (mean)

-	DL PRB Usage per SSB

The analytics output for MDARadioResourceOptimization.SSBUsageAnalysis would include the following:

-	The beam-level radio resource utilisation issues indicating if the radio resource is too short or too much

-	The recommendation on updating the coverage shape configuration for target cell of gNodeB

#### 5.3.3.4	Evaluation of solutions

#### TBD

* * * End of Changes * * * *