3GPP TSG-SA5 Meeting #164	S5-255288

Dallas, USA, 17 - 21 November 2025

Source:	L.M. Ericsson Limited

Title:	Rel-20 pCR TR 28.883 Clarification of NDTJob Modification Behaviour

Document for:	Approval

Agenda item:	6.20.3

Spec:	3GPP TR 28.883

Version:	V0.1.0

Work Item:	FS_NDT_Ph2

Comments

There is an operation to modify the MOI representing the NDTJob. However, it’s not clear what the consequences of modification of the NDTJob MOI entails, and whether all NDTJobs could even support modification – there are statements that indicate the NDTJob can be modified “at any time”, which are problematic. 
This pCR is recommending studying this aspect and clarifying the appropriate handling where applicable. 
(i.e. this could be left to implementation specific behaviour.)

Proposed Changes

* * * First Change * * * *

# 4	Concepts and background

Editor’s note: This clause provides a description of concepts and background.

# 5	Use cases

## 5.X	Use Case #<X>: Clarification of NDTJob Modification Behaviour

### 5.X.1	Description

The MnS Consumer requires clarity on the conditions and procedures for modifying an existing NDTJob instance.

The current specification does not define a procedure or lifecycle rule governing the modification of an NDTJob. TS 28.561 Clause 4.4 (“NDT life-cycle management”) states that “the NDT job instance can be configured by the MnS Consumer at any time”, yet no guidance is provided on permissible modification timing, allowable attribute changes, or the expected system behaviour when an NDTJob is already under execution.

This ambiguity raises questions such as:

- When, during the NDTJob lifecycle, modification requests are acceptable.

- Whether modification during execution affects job consistency or results.

- How the MnS Producer should report modification status or failures.

A clearer model describing NDTJob state transitions and management conditions would help ensure consistent interpretation across implementations. Such a model could make it explicit when an NDTJob is in a state that permits modification and when it is not (e.g. initialised, executing, completed, or terminated).

Excerpt from 4.4 NDT Life-cycle Management:

“NDT job instantiation: NDT MnS producer receives the request to create a job. The NDT MnS producer instantiates and executes the simulation/emulation for this job, which is an NDT job instance. The NDT job instance can be configured by the MnS consumer at any time.”

RESTful HTTP-based solution set (see Table 7.1-2: SS to support NDT job management in TS 28.561):

| NDT job management | IS operation | HTTP Method | Resource URI |
| --- | --- | --- | --- |
| Create an NDT job | createMOI operation | POST | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part} |
| Delete an NDT job | deleteMOI operation | DELETE | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{NDTJob}={id} |
| Modify an NDT job | modifyMOIAttributes operation | PUT PATCH | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{NDTJob}={id} |
| Query an NDT job | getMOIAttributes operation | GET | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{NDTJob}={id} |

### 5.X.2	Potential requirements

### 5.X.3	Potential solutions

### 5.X.4	Evaluation of potential solutions

# 6	Conclusions and Recommendations

## 6.X	Use case #<X>: <use case title>

Editor's note:	This clause provides conclusions and recommendations for the corresponding use case.

* * * End of Changes * * * *