3GPP TSG-SA5 Meeting #164	S5-255299

Dallas, USA, 17 - 21 November 2025

Title:	Reply LS on Renewable energy support in R20

Response to:	LS S2-2509827 on Renewable energy support in R20 from SA2

Release:	Rel-20

Work Item:	FS_Energy_Ph4_OAM

Source:	SA5

To:	SA2

Cc:

Contact person:	Srilakshmi S

srilakshmi.s@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

SA5 thanks SA2 for queries sent in the LS.

Q: SA2 kindly asks SA5 to confirm whether the SA5 specifications support Renewable Energy per node level (i.e., gNB and UPF) to be provided by OAM to the EIF. If supported, kindly provide the corresponding references and clarify which information related to Renewable Energy can be provided.

A: SA5 likes to inform SA2 that 3GPP management system, from Rel-19, supports association of energy-related information with the Network Elements (see clause 5.1.6.2, clause 5.2.5.1 and clause 8 of TS 28.310 [1]). This information is configured by the operator with the information obtained from sources external to 3GPP, and includes:

- A list of energy supplies that are used to power the Network Element.

- Energy supply information includes the following:

  - energy supply mode, i.e., grid electricity or backup energy or locally generated

  - the composition of energy sources used to produce energy (i.e., “energy supply mix” as defined in TS 22.261 [3])

- Each energy source can be associated with information related to:

  - energy source type, for example, the values can be "solar", "wind", “coal”, “nuclear”, "biogas", "biofuel", "aerothermal", "geothermal", "hydropower", "biomass", "landfillgas", “unknown”, etc.

  - Carbon Emission Factor (CEF) (see clause 3.1 of TS 28.310 [1]) of the energy source,

  - information if the energy source is renewable or non-renewable energy source, and

  - percentage of energy source type used to produce energy for the energy supply

Additionally, the following energy-related KPIs are defined at node level (i.e., gNB and UPF):

- NF Energy Consumption (EC) defined in clause 6.7.3.1 of TS 28.554 [2]

- NG-RAN EC defined in clause 6.7.3.4.1 of TS 28.554 [2]

- gNB EC defined in clause 6.7.3.4.2 of TS 28.554 [2]

- gNB Estimated Carbon Emission (ECE) defined in clause 6.7.7.1 of TS 28.554 [2]

- NG-RAN Estimated Carbon Emission defined in clause 6.7.7.2 of TS 28.554 [2]

The energy-related information and the KPIs supported in Rel-19 relates to renewable energy, e.g., the "energy supply mode" (e.g., when "locally generated" of source type "solar") and the Estimated Carbon Emission KPIs where the CEF takes the renewable or non-renewable information of the energy sources used to produce electricity into consideration. This information can be used within the 5GC to take decisions based on the renewable energy powering the network elements.

Further, as part of the Rel-20 study (FS_Energy_Ph4_OAM, see [5]), use cases related renewable energy and carbon emissions are currently being studied (see TR 28.885 [3]). The Rel-20 study is expected to complete in Jun 2026 (TSG#112) and the normative phase in Dec 2026 (SA#114).

References:

[1]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[2]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[3]	3GPP TR 28.885: "Study on energy efficiency and energy saving aspects of 5G Advanced networks".

[4]	3GPP TS 22.261: "Service requirements for the 5G system".

[5]	SP-251025: "Revised SID on Study on energy efficiency and energy saving aspects of 5G Advanced".

# 2	Actions

To SA2

ACTION: 	SA5 kindly asks SA2 to consider the above information into account.

# 3	Dates of next TSG SA WG5 meetings

SA5#164		17 November - 21 November 2025		Dallas, USA

SA5#165		9 February - 13 February 2026		Goa, India