3GPP TSG-SA5 Meeting #164	S5-255306

Dallas, USA, 17 - 21 November 2025

Source:	Nokia

Title:	Pseudo-CR on Rel-20 TR 28.885 Add new use case for Enhancements to gNB Energy Consumption

Document for:	Approval

Agenda item:	6.20.5

Spec:	3GPP TR 28.885

Version:	0.1.0

Work Item:	FS_Energy_Ph4_OAM

Comments

To introduce a new use case to study for " Enhancements to gNB Energy Consumption", related to WT-4.

This pCR introduces use case and potential requirements related to WT4 of the SI FS_Energy_Ph4_OAM.

"

WT-4. Study enhancements to Energy Consumption and Energy Efficiency measurements and KPIs for Network Functions and Network Slice.

"

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[4]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[5]	ETSI GS OEU 020 (v1.1.1): "Operational energy Efficiency for Users (OEU); Carbon equivalent Intensity measurement; Operational infrastructures; Global KPIs; Global KPIs for ICT Sites".

[6]	ETSI ES 202 706-1 V1.7.1 (2022-08): "Environmental Engineering (EE); Metrics and measurement method for energy efficiency of wireless access network equipment; Part 1: Power consumption - static measurement method".

[7] 	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[8]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[9]	3GPP TR 23.700-67: "Study on Energy Efficiency and Energy Saving; Phase 2".

[10]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[11]	3GPP TS 32.130: " Telecommunication management; Network sharing; Concepts and requirements".

[T]	3GPP TS 38.401: "NG-RAN; Architecture description".

[S]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

* * * Next Change * * * *

### 5.4.A	Use case #<A>: Enhancements to gNB Energy Consumption

#### 5.4.A.1	Description

gNB Energy Consumption KPI is used by operators to monitor the energy consumed by a gNB, and used by 5GC network function EIF for internal processing, hence the correctness of the KPI is important.

gNB EC KPI is defined in clause 6.7.3.4 of TS 28.554 [4], and specified that it is obtained by summing up the Energy Consumption of all the Network Functions (NF) that constitute the gNB. The NFs that constitute the gNB are not defined. Hence this KPI is not implementable.

The NG-RAN data Energy Efficiency (EE) KPI defined in clause 6.7.3.4 of TS 28.554 [4] defines EC of NG-RAN as the Energy Consumption (in kWh) that is obtained by measuring the PEE.Energy of the considered network elements over the same period of time.

The 3GPP defined Network Functions and entities are specified in clause 4.2.2 of TS 23.501 [8], where "(Radio) Access Network ((R)AN)" is listed as one of the Network Functions in the 5G System architecture.

gNB is defined in TS 38.300 [S] as node providing NR user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC. A gNB can consist of a gNB-CU and one or more gNB-DU(s) (see clause 6.1.1 of TS 38.401 [T]). gNB Central Unit (gNB-CU), gNB Distributed Unit (gNB-DU), gNB-CU-Control Plane (gNB-CU-CP), gNB-CU-User Plane (gNB-CU-UP) are defined as logical nodes hosting different protocols (see definitions in clause 3.1 of TS 38.401 [T]). The RAN specification TS 38.300 [S] and TS 38.401 [T] do not define any Network Functions (NF) that constitute the gNB, and do not define the gNBs to be implemented in the virtualized environment.

This use case is to identify potential solutions to correct the above inconsistences in the gNB EC KPI.

#### 5.4.A.2	Potential requirements

PREQ-Energy_Consumption_gNB-1: The 3GPP management system should be able to measure and report the energy consumption of a gNB.

#### 5.4.A.3	Potential solutions

#### 5.4.A.3.a	Potential solution #<a>: gNB EC KPI using PNF Energy consumption measurement

##### 5.4.A.3.a.1	Introduction

This potential solution describes the enhancements to the gNB EC KPI is defined in clause 6.7.3.4 of TS 28.554 [4].

##### 5.4.A.3.a.2	Description

gNB EC KPI is proposed to be based on PNF Energy consumption measurement (PEE.Energy) defined in clause 5.1.1.19.3 of TS 28.552 [10].

This potential solution proposes the gNB EC KPI to use be updated and defined as follows:

a) ECgNB

b) This KPI describes the Energy Consumption (EC) of the gNB. It is obtained using the measurement PNF Energy consumption defined in TS 28.552 [10].

b-1)	Integer, J

b-2)	CUM

c) ECgNB = PEE.Energy

d) ManagedElement

e) This KPI is applicable when a ManagedElement is used to represent the combined functionality of 3GPP defined gNB.

##### 5.4.A.3.i	Potential solution #<i>: <Potential Solution i Title>

###### 5.4.A.3.i.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

###### 5.4.A.3.i.2	Description

Editor's Note:	This clause further details the potential solution, including all of its aspects and any assumptions made.

#### 5.4.A.4	Evaluation of potential solutions

Editor's Note:	This clause provides the evaluation of potential solutions listed in 5.4.A.3.

* * * End of Changes * * * *