3GPP TSG-SA5 Meeting #164	S5-255326

Dallas, USA, 17 - 21 November 2025

Source:	Ericsson

Title:	Pseudo-CR on Types of failure covered by the study

Document for:	Approval

Agenda item:	7.5.2

Spec:	3GPP TR 32.872

Version:	0.1.0

Work Item:	FS_RoamRE_CH

Comments

Addition of description on types of failure covered by the study.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.255: "5G data connectivity domain charging stage 2".

[3]	3GPP TS 32.290: "5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[x]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[y]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

* * * Next Change * * * *

## 4.x	Error and failure handling

A failure can be detected either before any request or response is sent, no response to request, or no request received for an ongoing session. There are several ways these failures can be detected either by heartbeat type of signals, no response or request within a time limit etc.

There are six levels of failure scenarios possible:

-	NF Service Instance

-	NF Service Set

-	NF Instance

-	NF Set

-	NF Group

-	PLMN

For all except the PLMN (which is only applicable for the HPLMN) errors can happen in both HPLMN and VPLMN in the LBO scenario. There is also a legacy scenario where there are a primary CHF instance and optionally a secondary CHF instance. The failure handling when there is an error on NF Service Instance, NF Service Set, NF Instance and NF Set are all described in clause 5.21 of TS 23.502 [x], and TS 29.500 [y]. The failure scenario for NF Group and PLMN is not describe neither is the case when failure to reach both primary and secondary CHF, i.e. when there is no more redundancy.

* * * End of Changes * * * *