3GPP TSG-SA5 Meeting #164	S5-255356

# Dallas, USA, 17 - 21 Oct 2025

Source:	Samsung

Title:	Rel-20 pCR 28.887 UE Data Collection for UE-side model

Document for:	Agreement

Agenda item:	6.20.8

Spec:	3GPP TR 28.887

Version:	0.2.0

Work Item:	FS_MADCOL_Ph3

# 1	Decision/action requested.

The group is asked to endorse the proposal.

# 2	References

None

# 3	Rationale

This contribution provides a solution for UE Data Collection.

* * * First Change * * * *

## 4.2 	UE Data Collection

### 4.2.A	Use Case#1: Management support to training for UE-side model

#### The related use case and requirements are defined in clasue 5.1.1.1 of TR 28.882

#### 4.1.A.4      Potential Solution

The solution proposes to use MDT mechanism, as defined in 3GPP TS 32.422, to collect UE-side model training data.

The solution further proposes to enhance the existing ManagementDataInfo IOC (Instance Object Class) with attributes specifying metadata for the available trace metrics (as management data) collected, by the management system, using MDT mechanism. The proposed metadata includes:

- Single or a group of UE: This will specify the identification of UE or group of UE for each collected data. This can be defined as IMSI or N4 session identifier.

- Geographical location: This will specify the location for each collected data. The collected data belong to a particular location i.e data from all the UE in a particular location. Categorization based on location can be used to identify the input data to train an ML model for handover optimization for a particular VIP location. This can be defined as a geographical coordinate i.e a set of latitude and longitude, a civic address, TAI list etc.

- Time: This will specify the time at which the data was collected. The collected data may belong to a particular time window i.e data from all the UE in a particular time window.

- UE selection criteria: This will specify multiple UE related meta data for each collected data.

  - UE Energy Consumption: The collected data may belong to a UE that is consuming huge amount of energy i.e the energy consumption of the UE is going beyond a particular threshold.

  - Reduced Capability UE: The collected data may belong to only RedCap (Reduced Capability) UE.

  - Packet Delay threshold: The collected data may belong to a UE where packet delay is beyond a particular threshold. This attribute will provide that threshold. UE packet delay provides the average (arithmetic mean) time it takes for packet transmission over the air-interface in the downlink direction.

  - Packet Loss: The collected data may belong to a UE where packet loss is beyond a particular threshold. This attribute will provide that threshold. UE packet loss provides the DL Packet (i.e., RLC SDU) Loss rate on Uu interface.

  - Throughput: The collected data may belong to a UE where throughput falls beyond a particular threshold. This attribute will provide that threshold. UE throughout provides the average UE throughput in downlink.

In order to model the above metadata information in the existing NRM defined as part of 3GPP TS 28.622 the following options are provided:

- Option 1: Update the existing attribute supportedManagementData with supportedManagementDataInfo, which further includes supportedManagementData and relatedMetadata including the metadata proposed above. This will imply that the all the available management data, represented by supportedManagementData attribute, pertains to the given metadata (provided by the attribute relatedMetadata).

- Option 2: Add the attribute relatedMetadata directly into MgmtDataInfo IOC. This will imply that all the available management data, as part of MgmtDataInfo instance, pertains to the given metadata (provided by the attribute relatedMetadata). For different set of meta data, respective MgmtDataInfo instances will be created.

- Option 3: A new IOC (RelatedMetadata IOC) is created, the instance of which shall contain the metadata for the available management data. This IOC will be name contained in MgmtDataInfo IOC.

The solution further proposes a UESideAIMLDataCollectionJob IOC that can be used by the UE Server to request required UE-side model training data from the management system. This new IOC contains the following attributes.

- AdministrativeState: It indicates the administrative state of the UESideAIMLDataCollectionJob. It describes the permission to use or prohibition against using the UESideAIMLDataCollectionJob, imposed through the OAM services.

- OperationalState: It indicates the operational state of the UESideAIMLDataCollectionJob instance. It describes whether the resource is installed and partially or fully operable (Enabled) or the resource is not installed or not operable (Disabled).

- RequiredDataInfo: This will define the requirement for the requested data. This may be specified based on the following

  - Single or a group of UE: This indicates that the provided data should be from a single or a group of UE. This can be defined as IMSI or N4 session identifier.

  - Geographical location: This indicates that the provided data should be pertaining to a geographical location. This can be defined as a geographical coordinate i.e a set of latitude and longitude, a civic address, TAI list etc.

  - Time: This indicates that the provided data should be pertaining to a time window. This is defined as a time window in DDMMYYYY:HHMM:SS format.

  - UE selection criteria: The indicates that the provided data should pertain to a subset of UEs. This will define multiple way to select that subset of UE.

    - UE Energy Consumption: The collected data may belong to a UE that is consuming huge amount of energy i.e the energy consumption of the UE is going beyond a particular threshold.

    - Reduced Capability UE: The collected data may belong to only RedCap (Reduced Capability) UE.

    - Packet Delay threshold: The collected data may belong to a UE where packet delay is beyond a particular threshold. This attribute will provide that threshold. UE packet delay provides the average (arithmetic mean) time it takes for packet transmission over the air-interface in the downlink direction.

    - Packet Loss: The collected data may belong to a UE where packet loss is beyond a particular threshold. This attribute will provide that threshold. UE packet loss provides the DL Packet (i.e., RLC SDU) Loss rate on Uu interface.

    - Throughput: The collected data may belong to a UE where throughput falls beyond a particular threshold. This attribute will provide that threshold. UE throughout provides the average UE throughput in downlink.

The following figure shows the over all procedure for UE data collection

Figure 1: Procedure for UE-Side model training management

- The Management System, collects the data for UE-side model training using MDT/Trace mechanism defined in 3GPP TS 32.422.

- The Management System instantiate the MgmtDataInfo IOC to contain the UE-side model training data.

- When the UE-Side Training Entity requires the data, it send getMOIAttribute request to read the instance of MgmtDataInfo.

- The Managements system send the response using getMOIAttribute response with the available UE-side model training data.

- UE-Side Training Entity decides what type of data is required for the training. For example, to train an ML model for handover optimization for a particular VIP location, the training data (including UE related data) should belong to the same location. Other training data type may include UE identification, time, energy consumption of the UE etc. A ML model can be trained to inference for a particular UE(s) at a particular time. The type of energy consumption may also be useful to train an ML model that is meant to optimize UE energy consumption.

- UE-Side Training Entity send a createMOI request to instantiate UESideAIMLDataCollectionJob. This will represent the request for data collection from Management System to UE Server. This request will contain the UE Server requirement for the requested data based on the metadata describe above.

- The Management System instantiate the UESideAIMLDataCollectionJob and send a response.

- The UE Sever will receive the requested data as per the File Reporting or Streaming mechanism defined in 3GPP TS 28.532.

* * * Next Change * * * *

* * * End of Change * * * *