3GPP TSG-SA5 Meeting #164	S5-255383

Dallas, US 17 - 21 Nov 2025

Source:	Rakuten Mobile

Title:	pCR TR 28.884 Management Data Streaming based on Message Bus Potential Solution 3

Document for:	Approval

Agenda item:	6.20.4

Spec:	3GPP TR 28.884

Version:	0.1.0

Work Item:	FS_SBMA_Ph4

Comments

To address WT-1 of Study on Service Based Management Architecture enhancement phase 4, this contribution proposes to add potential solution for management data streaming based on message bus.

Proposed Changes

* * * First Change * * * *

## Use case #2: Management data streaming based on Message Bus

### 5.2.3	Potential solutions

5.2.3.X	Solution #3: Multipoint-to-Multipoint management data streaming based on message bus communication reference points

This potential solution proposes introduction of message bus communication reference points to enable multipoint-to-multipoint streaming data reporting between the MnS producers and MnS consumers.

Figure 5.2.3.X-1: Potential solution for multipoint-to-multipoint management data streaming based on message bus communication reference point

As shown in Figure 5.2.3.X-1, in this solution, the MnS producers interact with an external message broker implementation (outside 3GPP management system) via the message bus communication reference point A to publish data while the MnS consumers interact with the external system to receive data via the message bus communication reference point B. Different APIs may be used at the two reference points for data publishing and data receiving respectively. 3GPP management system acts as the consumer of the APIs provided by the external system over the reference point. This approach implies the MnS producers do not interact with MnS consumers directly for streaming data reporting. They interact with an external system implementing the message brokers via the reference points for data publishing and receiving. This approach results in major architecture change to 3GPP SBMA and unstandardized behaviours of the MnS producers and consumers.

* * * End of Changes * * * *