3GPP TSG-SA5 Meeting #164	S5-255392

Dallas, USA, 17 - 21 November 2025

Source:	Nokia

Title:	Pseudo-CR on Add use case and requirements on access control on notifications

Document for:	Approval

Agenda item:	6.20.9

Spec:	3GPP TR 28.888

Version:	0.1.0

Work Item:	FS_EnExpo

Comments

<Proposals, reason for change, abstract, comments if necessary (optional)>

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[3]	3GPP TS 28.579: "Management services exposure to external consumers through CAPIF".

[4]	3GPP TS 28.319: "Access control for management services".

[Y1]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration   Reference Point (IRP); Information Service (IS)".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

## 5.X	Use case #<X>: Access control on notifications

### 5.X.1	Description

Currently, the 3GPP management system supports notification subscription via the NtfSubscriptionControl IOC (see clause 4.3.22 of TS 28.622[Y1]), enabling MnS consumers to subscribe to notifications from MnS producers.

The NtfSubscriptionControl IOC (as defined in clause 4.3.22 of TS 28.622[Y1]) includes the following attributes:

Table 5.X.1-1: NtfSubscriptionControl properties

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| notificationRecipientAddress | M | T | T | F | T |
| notificationTypes | O | T | T | F | T |
| scope | O | T | T | F | T |
| notificationFilter | O | T | T | F | T |
| lastSequenceNo | O | T | F | F | F |
| operationalState | O | T | F | F | T |
| availabilityStatus | O | T | F | F | T |

Further, clause 7.3 of TS 28.319[4] defines the AccessRule class associated to a given MnS consumer identified by their Identity and Role classes. The AccessRule class includes the following attributes as defined in clause 7.3.3 of TS 28.319[4]):

Table 5.X.1-2: AccessRule properties

| Attribute Name | S |
| --- | --- |
| ruleName | M |
| dataNodeSelector | M |
| operations | M |
| actions | O |
| componentCData | O |

From Table 5.X.1-2, there is no defined mechanism for the MnS producer to determine whether a given MnS consumer is authorized to receive notifications for a specific scope (i.e., a defined set of Managed Object instances or data nodes). Additionally, the behaviour of the MnS producer when an MnS consumer subscribes to a scope that exceeds its authorized access is not specified. The lack of standardized behaviour regarding access control on notifications may result in inconsistent implementations and potential exposure of unauthorized data to MnS consumers.

* * * Next Change * * * *

### 5.X.2	Potential requirements

PREQ-FS_EnExpo-Ntf-01: The 3GPP Management System should provide the capability to configure, per MnS consumer, the set of notifications that the MnS consumer is authorized to receive for a given scope.

PREQ-FS_EnExpo-Ntf-02: The 3GPP Management System should provide the capability ensuring that an MnS consumer only receives notifications that are explicitly authorized for the MnS consumer within the configured scope.

* * * End of Changes * * * *