3GPP TSG-SA5 Meeting #164	S5-255403

Dallas, USA, 17 - 21 November 2025

# Source:	Nokia

# Title:	Open Issues in the Continuous MDT (C-MDT) Solution

# Document for:	 Endorsement

Agenda Item:	6.19.14

# 1	Decision/action requested

The group is requested to discuss and endorse the proposals below.

# 2	References

[1]	S5-253907 Rel-19 CR TS 28.622 Continuous MDT

[2]	S5-253908 Rel-19 CR TS 28.623 Continuous MDT

[3]	S5-254070 Rel-19 CR TS 32.423 Add additional TRSR on Trace Record Header for continuous MDT

[4] 	S5-254099 Rel-19 CR TS 32.421 Continuous MDT

[5]	S5-254111 Rel-19 CR TS 32.422 Continuous MDT

[6]	S5-253296/R3-253958 Reply LS to S5-252114 on Continuous MDT

[7]	S5-251326/R3-250891 LS on Continuous MDT

[8]		S5-255404 LS on Analysis on the Security impacts when the same TR/TRSR is resent multiple times over the air interface, Nokia, SA5#164, Dallas, USA, November 2025

[9]	S5-255409 Rel-19 CR TS 32.422 Corrections for C-MDT, Nokia, SA5#164, Dallas, USA, November 2025

[10]	S5-255410 Rel-19 CR TS 32.422 Corrections for TRSR Prefix Configuration, Nokia, SA5#164, Dallas, USA, November 2025

[11]	S5-255411 Rel-19 CR TS 28.622 Corrections for TRSR Prefix Configuration, Nokia, SA5#164, Dallas, USA, November 2025

[12]	S5-255412 Rel-20 CR TS 28.622 Corrections for TRSR Prefix Configuration, Nokia, SA5#164, Dallas, USA, November 2025

[13] 	S5-255413 Rel-19 CR TS 28.623 Corrections for TRSR Prefix Configuration, Nokia, SA5#164, Dallas, USA, November 2025

# 3	Rationale

SA5 introduced Continuous MDT (C-MDT) solution to various SA5 specifications (see CRs in S5-253907[1], S5-253908[2], S5-254070[3], S5-254099[4] and S5-254111[5]) after an LS on requirements from RAN3 in S5-253296/R3-253958[6] and S5-251326/R3-250891[7]. In what follows next, we provide a list of issues in the current continuous MDT solution, and we provide proposals to address those.

Issue #1

In S5-253296/R3-253958[6] RAN3 provided to SA5 the following principles for continuous MDT:

- Information from OAM to the NG-RAN nodes participating to Continuous MDT, allowing for the identification of a continuous MDT process, are provided e.g. in the form of specific Trace Reference(s).

- Solutions for Continuous MDT shall not have an impact on the 5GC in Rel19 from a RAN3 perspective.

- RAN3 agrees to use TR(s) and TRSR(s) to resolve the MDT measurement correlation problem.

So, according to the RAN3 LS to SA5, a continuous MDT process is identified, for instance, by specific Trace Reference. According to the RAN3 agreements, TR(s) and TRSR(s) are used for the MDT measurement correlation problem, but TRSR is not used for the identification of continuous MDT.

However, in TS 32.422 clause 4.1.1.9.4, it states:

The management system shall either send two Trace Session activation requests (one for Immediate MDT, another for Logged MDT) with same TR to the gNB, or send one Trace Session activation request with Job Type (IMMEDIATE_MDT_AND_ LOGGED_MDT) to the gNB. In additional of the trace control and configuration parameters specified in subclause 4.1.1.9.2 step 1, the Trace Session activation request(s) shall contain a TRSR Prefix Configuration parameter, which reserves a TRSR range(s) for the gNB during TRSR assignment. This TRSR Prefix Configuration parameter is used to identify this is a C-MDT activation request.

As can be seen from the above agreements, the TRSR prefix is used for the correlation purpose, but not in order to identify a C-MDT activation request to the RAN.

In addition, the usage of specific TR range will enable future enhancements of the C-MDT future e.g. introduction of appropriate handling when a UE configured with C-MDT reconnects to an NG-RAN node that is not involved in the C-MDT job.

Proposal 1: 	Rephrase the sentence “This TRSR Prefix Configuration parameter is used to identify that this is a C-MDT activation request.” in clause 4.1.1.9.4 from TS 32.422 to “This TRSR Prefix Configuration parameter is by the NG-RAN node for TRSR assignment”.

Issue #2

SA5 defined Continuous management-based MDT as follows:

Continuous management-based MDT: a management-based MDT functionality that enables continuous collection of MDT data, including both Immediate MDT and Logged MDT, within a specified area.

Area Scope parameter in MDT Configuration is optional. When it is not included, the MDT Configuration is applied throughout the PLMN (TS 32.422, clause 5.10.2). One aspect that affects the area over which MDT may be configured is the TRSR Prefix Configuration parameter which reserves a TRSR range for a gNB during the TRSR assignment. Continuous MDT needs to be configured across the gNBs of the PLMN if we want to make sure that data collection continues in an uninterrupted way regardless of UEs’ trajectories (idle mode, connected mode). Depending on the area/region of the configuration of the operator network a PLMN may span thousands of gNBs (e.g., as many as 20K gNBs in crowded regions such as Japan). The current range of TRSRs is therefore not sufficient to cover large PLMN sizes. Therefore, we think that Area Scope shall always be included in case of C-MDT.

Proposal 2: 	Update clause 5.10.2 in TS 32.422 as follows: “The Area Scope parameter shall be present in case of C-MDT.”.

Issue #3

Another issue relates to the configuration of continuous MDT to a UE. TS 32.422 clause 4.1.1.9.4 states the following:

“The gNB shall configure the selected UE with an Immediate MDT session and a Logged MDT session using the configurations from the Trace Session activation request(s). A unique TRSR is assigned to the UE for Immediate MDT and Logged MDT. The TRSR is allocated based on the received TRSR Prefix Configuration parameter.

In our view, there is an issue related to the above highlighted sentence.

According to TS 32.422, clause 4.1.1.9.2 in legacy management-based MDT a gNB selects suitable UEs for MDT data collection. “The selection is based on the area received from the management system and the area where the UE is located, user consent information which is part of the MDT PLMN List IE […]. If the user is not in the specified area or if the management-based MDT PLMN List IE is not present in the UE context the UE is not selected by the gNB for MDT data collection.” Also, in case of logged MDT, UE capability is taken into account before selecting a UE for logged MDT.

A question that arises is what shall be the gNB behavior in the presence of both signalling based configuration and C-MDT configuration at the gNB:

If a UE is already configured with signalling-based MDT, then, following the understanding that signalling based MDT has priority over management-based MDT, the UE shall not be selected for C-MDT.

Proposal 3: If a UE is already configured with signalling-based MDT, it shall not be selected for C-MDT.

As another possibility, upon reception of the signalling-based MDT configuration, the UE is already configured with C-MDT and it is, therefore, configured with both an Immediate and Logged C-MDT.

TS 37.320, clause 5.4.0, states that “The management-based MDT configuration should not overwrite signalling-based MDT configuration”. However, in this scenario the issue relates to how to handle an already configured C-MDT session in case of subsequent activation of signalling-based MDT. It is important that the specification describes this new scenario, indicating that signalling-based MDT configuration has priority compared to continuous MDT configuration. Therefore, in case a gNB receives a signalling-based MDT configuration it shall either pause or stop the corresponding continuous MDT for the UE and prioritize the signalling-based MDT configuration. Since, continuous MDT requires a UE to provide measurements in a continuous way, pausing of a continuous MDT configuration would correspond also to interrupting the continuity and hence continuous MDT would need to be stopped.

Proposal 4: 	When a gNB receives signalling-based MDT for an already configured C-MDT UE, C-MDT (immediate and logged MDT) at the UE shall be stopped.

However, if UE is configured with Logged MDT (corresponding to continuous MDT) and similarly to legacy behavior, the gNB can overwrite the configured C-MDT Logged MDT session with a received signalling-based Logged MDT session. However, if a signalling-based immediate MDT session is received, the NG-RAN node is not able to stop the UE from performing Logged MDT. This will lead to the UE continuing to log MDT measurements for C-MDT when it is in RRC_IDLE, without corresponding immediate MDT measurement collection. So, there may be some C-MDT measurements received by TCE that will lack a corresponding immediate MDT measurement. Since this is an issue that cannot be resolved (and also relates to legacy logged MDT behavior), we propose to capture this with a note in the TS 32.422.

Proposal 5: Introduce a note in clause 4.1.1.9.4 of TS 32.422 as follows:

NOTE: If the gNB receives signalling-based immediate MDT request for a continuous MDT UE, it shall replace the immediate MDT configuration for C-MDT. However, there is no mechanism to stop logged MDT in the UE.

Issue #4

Another issue relates to how long Continuous MDT shall continue. TS 32.422, assumes that a gNB shall reconfigure both immediate MDT and logged MDT to the UE after a connected mode Handover:

Clause 4.1.1.9.4 of TS 32.422 states the following:

“Upon transitioning into RRC_CONNECTED, the UE may report Logged MDT measurements, including TR and TRSR, to the gNB, as specified in subclause 4.1.1.9.2 step 7 and 8. If the reported TR is same as the Trace Reference received in the Trace Session Activation request(s), the gNB shall identify the UE as part of the same C-MDT job. If the area-based selection and user consent conditions are satisfied, the gNB shall reconfigure the UE with an Immediate MDT and a Logged MDT.”

This seems unnecessary and poses the risk that C-MDT continues without any control of the duration of the data collection. One approach to stop C-MDT is that OAM stops the Trace for C-MDT in the area of involved gNBs, but this solution is not flexible and assumes that while a C-MDT session is configured in the RAN (which can be for a very long period of time) all the selected UEs continue reporting continuously immediate and logged MDT measurements. This solution is also hard to manage from the operator’s perspective, since it needs to manage the C-MDT configuration and de-configuration manually, without the possibility to control for how long different UEs are reporting measurements (UEs that were selected earlier will report longer than UEs that were selected later).

A simple solution to that is to only reconfigure the UE only with immediate C-MDT upon transition from RRC_IDLE or RRC_INACTIVE to RRC_CONNECTED. That is the UE is configured once with both logged and immediate C-MDT but it is only measurements for immediate MDT that are subsequently reconfigured to the UE.

Our understanding is that reconfiguration of logged MDT measurement in the UE was specified in order to comply with SA3’s requirement that UE IDs transmitted over the air need to be regularly updated for security and privacy reasons. And for the case of C-MDT, because the TR/TRSR is used to correlate measurements from a given UE, it is possible to track the UE using this information. However, this situation already exists with legacy logged MDT, and any new requirement, e.g. update of the TR/TRSR for security reason would require SA3 involvement. Aspects that would need to be checked relate to the repeated transmission of TR/TRSR from the NG-RAN node to the UE (DL direction), combined with the new situation for TRSR selection for C-MDT where each NG-RAN node can select TRSR only from a limited range (block of TRSRs) instead of from the full 16-bit range allowed for legacy MDT.

The impact on security related to reconfiguration of TR/TRSR is therefore uncertain, which is to us an additional reason that TR/TRSR reconfiguration should not be specified.

Proposal 6: 	SA5 to choose between two solutions to the problem of endless continuity of C-MDT that poses a risk of security to a C-MDT UE by sending repeatedly in the UL and DL the TR/TRSR identifier:

- Reconfigure only the C-MDT configuration corresponding to immediate MDT upon transition from RRC_IDLE to RRC_CONNECTED.

- Introduce a mechanism to stop C-MDT beyond deconfiguration of the Trace Job in the area of involved gNBs and send an LS to SA3 to request analysis on the security impacts when the same TR/TRSR is resent multiple times over the air interface.

Issue #5

When specifying the C-MDT feature in TS 32.422, SA5 made the choice of describing the feature in a new clause 4.1.1.9.4 in order to avoid impact in the description of legacy management-based MDT for which the activation mechanism is described in clause 4.1.1.9.2. The new clause 4.1.1.9.4 is described to provide additions on top of the legacy clause, but we observe that there is also a modification needed of step 7 in clause 4.1.1.9.2.

TS 32.422 clause 4.1.1.9.4 states the following:

##### 4.1.1.9.4	NG-RAN activation mechanisms for C-MDT data collections for non-split RAN architecture

For non-split RAN architecture, the C-MDT data collection procedure shall be same as specified in subclause 4.1.1.9.2 with following additions.

Furthermore, clause 4.1.1.9.2 step 7 says the following:

For Immediate MDT when the gNB receives the MDT report from the UE in the RRC message the gNB shall capture it and put the UE’s serving cell CGI together with the MDT report from the UE to the trace record. A UE configured to perform Logged MDT measurements in IDLE or INACTIVE indicates the availability of MDT measurements, by means of a one-bit indicator, in RRC messages definitions in 3GPP TS 38.331 [43], e.g., RRCSetupComplete message and RRCReconfigurationComplete message. The gNB can decide to retrieve the logged measurements based on this indication by sending the UEInformationRequest message to the UE. The UE can answer with the collected MDT logs in UEInformationResponse message.

However, this legacy behavior needs to change for NG-RAN nodes that are involved in C-MDT. In order to ensure continuity, the specification should mandate such NG-RAN node to retrieve the logged measurements every time a UE indicates availability of MDT measurements. Once logged measurements, and corresponding TR/TRSR, are retrieved from the UE, the NG-RAN node can determine whether the logged MDT session corresponds to C-MDT.

Proposal 7: 	Modify clause 4.1.1.9.4 of TS 32.422 to also include modifications of the legacy NG-RAN activation mechanism for management-based MDT.

Proposal 8: 	Modify step 7 of the legacy logged MDT description (clause 4.1.1.9.2 in TS 32.422) for C-MDT such that a gNB that has received a C-MDT configuration from OAM is mandated to retrieve the logged measurements every time a UE indicates availability of MDT measurements in RRC messages such as in RRCSetupComplete message or RRCReconfigurationComplete message.

Issue #6

Clause 5.10.43 TRSR Prefix Configuration is intended to provide a concise way to represent a block of TRSRs according to the semantics captured in this clause. However, it currently defines a “base TRSR prefix” (2 byte octet string) and “the size of the TRSR prefix” (number of bits).

Depending on the operator network a PLMN may span as many as 20K gNBs e.g., in crowded regions such as Japan. In the current MDT framework, where TRSR can be reused by different gNBs, each gNB can configure with MDT up to 216-1 UEs. In C-MDT, where a prefix is provided to each gNB and where TRSR reuse is not allowed, the number of gNBs as well as the number of C-MDT UEs decreases. If 15 bits are used for the prefix, then PLMNs of the size of the Japan scenario can be covered, but this would only leave 1 bit for the TRSR assignments towards a UE. This means that a single UE can be configured with continuous MDT. If we assume smaller PLMN sizes (in the order of 10K gNBs), then 14 bits are sufficient meaning that Continuous MDT can only be configured to 3 UEs within a gNB. This is a serious limitation of the feature applicability.  However, luckily, we observe that the usage of a prefix is not required since for the C-MDT trace correlation it is not necessary to identify a gNB and should therefore not be defined.

We therefore propose that this parameter is given the name TRSR Range Configuration.

Proposal 9: 	Rename clause 5.10.43 from “TRSR Prefix Configuration” to “TRSR Range Configuration”.

One issue is that the TRSR is currently defined as a 2 byte octet string, and not as a range of values from 0 to 65535. However, reserving one or more bits of the TRSR per NG-RAN node would drastically limit the use of C-MDT, e.g. if 16 gNBs are involved each of them could configure 2 C-MDT sessions each. Therefore, as a first step, for the purpose of C-MDT, the TRSR needs to be considered as a range of values from 0 to 65535.

Proposal 10: 	For the purpose of C-MDT, the TRSR is considered as a range of values from 0 to 65535.

Furthermore, for a proper definition, SA5 would need to determine whether the block of TRSRs corresponds to a range of contiguous TRSR values, or whether some other block definition is required. For simplicity, we believe a range of contiguous values can be used.

Proposal 11: 	The block of TRSRs used by NR-RAN at TRSR assignment for a given C-MDT job is a range of contiguous values.

This block of TRSRs could then be represented according to one of the following two options:

- Option 1: The block of TRSRs is defined by the first and last TRSR value of the block.

- Option 2: The block of TRSRs is defined by the first TRSR value of the block, and number of TRSR values within the block (size of the block).

If option 1 is used, the definition could be updated as follows:

This parameter is mandatory if C-MDT is supported in NG-RAN. It is used by the NG-RAN at TRSR assignment for a given C-MDT job. The block of TRSRs used by NG-RAN at TRSR assignment for a given C-MDT job is a range of contiguous values. It is defined by the first TRSR value of the block and the last TRSR value of the block. The first TRSR value of the block is an integer. The last TRSR value of the block is an integer.

Proposal 12: Update the definition of the TRSR Range Configuration as per TP provided above.

Proposal 13: 	Update TS 28.622 (stage 2) and TS 28.623 (stage 3) according to Proposal 9 to 11.

Issue #7

Several other corrections are needed in TS 32.422. In particular, in clause 4.1.1.9.5 “NG-RAN activation mechanisms for C-MDT data collections for split RAN architecture” there is a reference to subclause “4.1.1.9.x step 2”. This subclause needs to be updated to 4.1.1.9.2.

Proposal 14: 	Update 4.1.1.9.x step 2 in clause 4.1.1.9.5 “NG-RAN activation mechanisms for C-MDT data collections for split RAN architecture” to 4.1.1.9.2 step 2.

The same clause mentions the following:

“In addition to the procedures specified in subclause 4.1.1.9.3, the gNB-CU-CP shall configure the selected UE with an Immediate MDT session and a Logged MDT session as specified in subclause 4.1.1.9.4 step 3.”

As discussed above in Issue #4, only the immediate MDT session shall be configured to the selected UE.

Proposal 15: 	Update clause 4.1.1.9.5 to indicate that only the immediate MDT session shall be configured to the selected UE as specified in subclause 4.1.1.9.4 step 3.

Finally, on a less critical aspect, we noticed that the term “data collections” has been introduced in several parts of TS 32.422. We think that in its singular form it corresponds to better terminology.

Proposal 16: 	Update all references to “data collections” in TS 32.422 to the singular form “data collection”.

We have submitted corresponding CRs S5-25xxxx [8], S5-25xxxx [9], S5-25xxxx [10] reflecting the proposals discussed above.

# 4	Detailed proposals

Proposal 1: 	Rephrase the sentence “This TRSR Prefix Configuration parameter is used to identify that this is a C-MDT activation request.” in clause 4.1.1.9.4 from TS 32.422 to “This TRSR Prefix Configuration parameter is by the NG-RAN node for TRSR assignment”.

Proposal 2: 	Update clause 5.10.2 in TS 32.422 as follows: “The Area Scope parameter shall be present in case of C-MDT.”.

Proposal 3: If a UE is already configured with signalling-based MDT, it shall not be selected for C-MDT.

Proposal 4: 	When a gNB receives signalling-based MDT for an already configured C-MDT UE, C-MDT (immediate and logged MDT) at the UE shall be stopped.

Proposal 5: Introduce a note in clause 4.1.1.9.4 of TS 32.422 as follows:

NOTE: If the gNB receives signalling-based immediate MDT request for a continuous MDT UE, it shall replace the immediate MDT configuration for C-MDT. However, there is no mechanism to stop logged MDT in the UE.

Proposal 6: 	SA5 to choose between two solutions to the problem of endless continuity of C-MDT that poses a risk of security to a C-MDT UE by sending repeatedly in the UL and DL the TR/TRSR identifier:

- Reconfigure only the C-MDT configuration corresponding to immediate MDT upon transition from RRC_IDLE to RRC_CONNECTED.

- Introduce a mechanism to stop C-MDT beyond deconfiguration of the Trace Job in the area of involved gNBs and send an LS to SA3 to request analysis on the security impacts when the same TR/TRSR is resent multiple times over the air interface.

Proposal 7: 	Modify clause 4.1.1.9.4 of TS 32.422 to also include modifications of the legacy NG-RAN activation mechanism for management-based MDT.

Proposal 8: 	Modify step 7 of the legacy logged MDT description (clause 4.1.1.9.2 in TS 32.422) for C-MDT such that a gNB that has received a C-MDT configuration from OAM is mandated to retrieve the logged measurements every time a UE indicates availability of MDT measurements in RRC messages such as in RRCSetupComplete message or RRCReconfigurationComplete message.

Proposal 9: 	Rename clause 5.10.43 from “TRSR Prefix Configuration” to “TRSR Range Configuration”.

Proposal 10: 	For the purpose of C-MDT, the TRSR is considered as a range of values from 0 to 65535.

Proposal 11: 	The block of TRSRs used by NR-RAN at TRSR assignment for a given C-MDT job is a range of contiguous values.

Proposal 12: Update the definition of the TRSR Range Configuration as per TP provided above.

Proposal 13: 	Update TS 28.622 (stage 2) and TS 28.623 (stage 3) according to Proposal 9 to 11.

Proposal 14: 	Update 4.1.1.9.x step 2 in clause 4.1.1.9.5 “NG-RAN activation mechanisms for C-MDT data collections for split RAN architecture” to 4.1.1.9.2 step 2.

Proposal 15: 	Update clause 4.1.1.9.5 to indicate that only the immediate MDT session shall be configured to the selected UE as specified in subclause 4.1.1.9.4 step 3.

Proposal 16: 	Update all references to “data collections” in TS 32.422 to the singular form “data collection”.