| 3GPP TR 28.883 V0.2.0 (2025-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Study on management aspects of Network Digital Twins phase 2 (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

Annex <X> (informative):  Change history	7

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

New and updated use cases, as well as potential requirements, are emerging for managing 5G networks and services utilizing Network Digital Twin (NDT). This document captures the corresponding potential enhancements to the NDT as introduced in TS 28.561.

1	Scope

The present document describes use cases, potential requirements, and potential solutions aimed at enhancing the Network Digital Twin (NDT) defined in TS 28.561, with aspects on interaction and collaboration between NDT and network functions/automation functions, multiple NDT collaborations, data collection requirements to support NDTs and any other new use cases. It also presents conclusions and recommendations regarding the next steps in the 3GPP standardization process.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.312: " Management and orchestration; Intent driven management services for mobile networks".

[3]	3GPP TS 28.561: " Management and orchestration; Management aspects of Network Digital Twins".

[4]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[5]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2"

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Concepts and background

Editor’s note: This clause provides a description of concepts and background.

5	Use cases

5.1	Use Case #1: NDT support intent pre-evaluation

5.1.1	 Description

TS 28.312 [1] introduces the intent pre-evaluation which includes Intent Feasibility check and Intent Exploration. Intent feasibility check enables the MnS consumer to check if the proposed intent can be supported by the MnS producer. Intent exploration enables the MnS consumer and the MnS producer to find the intent for fulfilment that is best aligned with MnS producer's capabilities. NDTF can be used to support for intent pre-evaluation.

The relation between NDTF and Intent Handling Function follows the relation between NDTFs and network automation functions defined in TS 28.561 clause 4.3. An adaption figure based on Figure 4.3-1 in TS 28.561 is shown below.

Figure 5.X.1-1: Relation between NDTF and Intent Handling Function

In the scenario of radio network/service intent pre-evaluation, the MnS consumer may request to obtain the best values for a given target or context, e.g., the number of terminal devices. NDTF can support intent handling function to evaluate the number of terminal devices (e.g., UE) given certain simulation conditions, such as intent objects scope (e.g., area scope, cell lists, civic address, etc), intent object type (e.g., radio service, radio network, etc). IHF requests NDTF to simulate the network performance under various conditions, NDTF simulates the network performance and reports to IHF with simulation results.

This use case proposes to enhance the description related to network automation in TS 28.561 to capture the relationship between NDT and IHF and specify that NDT can support network automation capability including intent pre-evaluation.

5.1.2	Potential requirements

No new requirements

5.1.3 	Potential solutions

This solution proposes following updates in TS 28.561[3]:

1. In clause 4.3, add Intent Handling function as one additional example of network automation functions.

2. In clause 5.2.2.1, add the text below:

“NDT can support for Intent Handling Function enabling intent handling capability, e.g., feasibility check and exploration capability. For example, in the scenario of radio network/service intent feasibility check and exploration, the MnS consumer may request to obtain the best values for a given target or context, e.g., the number of terminal devices. NDTF can support intent handling function to evaluate the number of terminal devices (e.g., UE) given certain simulation conditions, such as intent objects scope (e.g., area scope, cell lists, civic address, etc), intent object type (e.g., radio service, radio network, etc). IHF requests NDTF to simulate the network performance under various conditions, NDTF simulates the network performance and reports to IHF with simulation results.”

5.1.4 	Evaluation of potential solutions

5.2	Use Case #2: Improvement of data generation

5.2.1	Description

TS 28.561 clause 5.4.2.2 [3] introduces the data generation capability provided by NDT to enable ML training. For different ML models, the preferences of its training data are different, which can vary in the dimension of data source object, data type, data quantity. The data source object is used to specify the simulated object from which the synthetic ML training data is collected. It is the subset of the NDT synchronization objects. The data quantity is used to specify how many data needs to be reported by NDT for ML training.

The existing solution of NDT NRMs can support the data generation by specifying data source objects and data type to be collected. The data source objects can be specified by the managedEntitiesScope defined in ScopeDefinition <<choice>> which represents the synchronization scope. The data type can be specified by the simulationData defined in SimulationDataDescriptor <<dataType>>. However, the existing solution cannot satisfy the data generation scenario where the requested data source objects are the subset of the synchronized network objects with specific data quantity.

5.2.2	Potential requirements

REQ-NDTDG-01: The 3GPP management system should support a capability to allow an authorized MnS consumer to express data generation preferences on data source object, data type, and data quantity.

5.2.3	Potential solutions

This solution aims to enhance NDT NRMs as shown below:

1. Enhancement for NDTJob IOC

- Add new attribute “nDTDataGenObject” which represents the data source object from which the synthetic data is collected. This attribute allows the MnS consumer to express data generation preferences on data source objects.

- Add new attribute “dataQuantity” which represents the requested synthetic quantity of data used for ML training. This attribute allows the MnS consumer to express data generation preferences on data quantity. This attribute can either be an integer which represent the collection times for the synthetic data, or be a combination of time window and frequency which implicitly represents the data quantity.

5.2.4	Evaluation of potential solutions

5.3	Use case #3: Collaborate with ML training Producer to generate data

5.3.1	Description

In 3GPP TS 28.561 [3], the existing use case and requirements for using NDT to generate ML training data is described in clause 5.4.2.2. However, using NDT alone to generate data may be insufficient to support the following scenarios:

-	Due to the coverage limitations of the physical network simulated by NDT, some extreme scenario data (such as sudden traffic peaks or unforeseen equipment failures) may not be generated.

-	NDT-based data generation depends on complex procedures such as simulating network topology and device interactions, making it time-consuming when generating large volumes of data.

Therefore, for scenarios requiring extreme data generation or large-scale data generation, it is considered to introduce AI-based data generation models, generated by the ML training Producer, into NDT to enable rapid, batch, and comprehensive data generation.

As shown in the Figure 5.3.1-1, the MnS Consumer can request the MnS producer to create an NDT instance for generating data with an indication of simulation object, data type, and data requirements, etc. Data requirements may specify large-scale data, extreme data requirements. The MnS producer creates an NDT instance based on the request and sends a response to the MnS consumer. The MnS producer can act as an ML training consumer to send a request to the ML training producer for generating a data generation model. Subsequently, the MnS producer executes simulation based on the NDT instance to obtain simulation data (e.g.,the generated UE throughput data), which is then sent to the ML training producer. This simulation data is used as training data to update and train the data generation model. The MnS producer can act as an ML inference function to receive the updated model from the ML training producer, execute it to obtain the final generated data, and send this data to the MnS consumer.

Figure 5.3.1-1 Collaborate with ML training Producer to generate data

Through this method, after the ML training producer completes ML model training and updates based on the initial NDT simulation data, it only needs to perform AI inference for subsequent data generation, which can reduce certain resource consumption.

5.3.2	Potential requirements

REQ-NDTDG-AI-1: The 3GPP management system should support a capability that enables an authorized MnS consumer to obtain the enabler information of the NDT data generation.

5.3.3 	Potential solutions

5.3.4 	Evaluation of potential solutions

5.4	Use Case #4: Enhancement for multiple NDT collaborations

5.4.1	Description

In 3GPP TS 28.561 [3], the existing use case and requirements for collaboration between NDTs are described in clause 5.5.2.1 and the requirement in clause 5.5.3. The 3GPP management system should support a capability enabling an authorized MnS consumer to configure the relationship between NDTs during simulation/emulation. However, there are no specific solutions to support this scenario, and there are no further details regarding the collaboration of multiple NDTs.

Therefore, the scenario needs the further investigate, a single NDT Function might not be able to fulfil a task by itself and may depend on or need to use the service or outputs of another NDT Function during the simulation/emulation activity. This require the 3GPP management to support the capabilities and report the relationships between NDTs regarding the collaboration of multiple NDTs.

5.4.2	Potential requirements

REQ-NDT-Colla-1: The 3GPP management system should support a capability that enables an authorized MnS consumer to request a report on the relationships between NDTs regarding the collaboration of multiple NDTs.

5.4.3	Potential solutions

5.4.4	Evaluation of potential solutions

5.5	Use Case #5: Enhancement on NDT reporting method

5.5.1	Description

In TS 28.561 [3], the reporting method of NDT output is notification based reporting. The NDT MnS consumer receives an NDT report from NDT producer by invoking generic provisioning management service operations and notifications in TS 28.532 [4]. However, different NDT job may have different requirement on reporting method. For example, in the scenario of signalling storm, it is more appropriate to use streaming data reporting service to timely report the simulation/emulation information on potential network impact.

It is proposed to enable the NDT MnS consumer to select the reporting method of simulation/emulation output based on different requirement. In addition to notification based reporting, streaming data reporting service also need to be supported for NDT output reporting.

5.5.2	Potential requirements

REQ-NDTRM-01: The 3GPP management system should support a capability to allow an authorized MnS consumer to request NDT report to be provided by streaming data reporting service.

REQ-NDTRM-02: The 3GPP management system should support a capability for NDT MnS producer to support delivery of NDT report by streaming data reporting service.

5.5.3	Potential solutions

It is proposed to make the following changes to TS 28.561 [3] clause 6.2.1.3.2:

- update the NDTJob IOC attribute to add a new optional attribute reportingMethod, which can be used to indicate the required reporting method of NDT report, the allowed values of reportingMethod can include streaming and notification. The new attribute can only be used when the requested NDTFunction supports the capability that allow a MnS consumer to indicate the reporting method.

5.5.4	Evaluation of potential solutions

5.6	Use Case #6: Capability Discovery of NDT in NDT Collaboration

5.6.1	Description

NDT collaboration among different NDTFunctions located in different domain is critical to enable end to end use cases, In Rel-19 the supported domain of NDT is represented by NDTFunctionScope attribute. Then, the NDTFunction that triggers the collaboration can involve NDT(s) that supports modelling of different domain.

When simulating end-to-end user experience data generation, the consumer may want to specify diverse traffic model and UE distribution for different scenarios. For example, major event such as sporting events, the UE distribution and user’s communication behaviour is quite different, in which the uplink traffic will increase significantly. Therefore, if consumers want to trigger NDT collaboration for E2E user experience data generation, they should know which candidate NDT supports the simulation of different traffic. Therefore, NDTFunction needs further enhancement to expose more detailed capability information (e.g., capability of supporting UE distribution model and traffic model). Then, the consumers can accurately identify and select appropriate collaboration participants as NDT component, ensure cross-domain traffic consistency, and effectively validate end-to-end scenario continuity.

5.6.2	Potential requirements

| REQ- NDTAUT-01 | The 3GPP management system should support a capability enabling authorized MnS Consumer to discover detailed capability of candidate NDT components for NDT collaboration. |
| --- | --- |

5.6.3	Potential solutions

Add a new the GenericModellingCapabilites <<dataType>> with the following new attributes to support modelling capability discovery of NDT in NDT collaboration:

- EnviromentModelling, it represents the NDTFunction supports to model the network topology and network infrastructure.

- UETrafficModelling, an ENUM, which indicates the capability of traffic modelling supported by the  NDTFunction and may include the following values:

  - FixedUEModel, it represents that the NDTFunction supports to model the location of the UE. It means that the NDT can support to model the traffic of a single UE. The information of specific UE is unknown for the NDFFunction.

  - MovingUEModel, it represents that the NDTFunction supports to model the trajectory of the moving UE. It only means that the NDT can support to model the traffic of a moving UE. The information of specific UE is unknown for the NDTFunction.

  - DistributedUEsModel, it represents that the NDTFunction supports to model the distribution of UEs.

- ServiceModelling, an ENUM, which represents the service that the NDTFunction supports, e.g., URLLC,

5.6.4	Evaluation of potential solutions

TBD

5.7	Use Case #7 Defining the Lifecycle and Runtime Behaviour of NDT Jobs

5.7.1	Description

The NDTJob lifecycle is not clear, there are some items open to interpretation and some items missing which are described below.

Issue#1: The MnS Consumer should have a method to know which configuration of NDTJob has produced a given report. In the current specification NDTReport refer to the NDT job (i.e NDTJobRef). In the existing specification, it is possible to modify the NDTJob which may result in the different NDTReport. This will result in loosing the link between produced NDT Report and the original NDT Job. The MnS Consumer would benefit from understanding the implications of reconfiguring the NDTJob. At the moment, the NDTReports refer to the DN of the NDTJob – if a NDTJob is reconfigured, this means the same DN is applied to each report, even if the simulation has changed. Likewise, there is no clear procedure described in Clause 6.4, despite the “Modify” use-case being possible for the NDTJob Instances.

Excerpt: Table 6.2.1.3.8.2-1 describes the attributes associated with the NDTReport <<IOC>>

Excerpt: From 6.3 Attribute definitions, the explanation of the ndtJobRef of the NDTReport <<IOC>> refers to

5.7.2	Potential requirements

5.7.3	Potential solutions

5.7.4	Evaluation of potential solutions

5.8	Use Case #8: Using external data for NDT modelling

5.8.1	Description

As defined in TS 28.561[3], Network Digital Twin (NDT) is used as a replica of a mobile network, in order to learn how an actual mobile network would behave in certain scenarios, without causing any changes to the actual mobile network. To provide meaningful results, an NDT needs to model the behaviour of the mobile network, so that the result of the operations on the virtual replica is a good approximation to the result of similar operations on the actual network. The accuracy of the approximation relies on how much the NDT modelling mimics the live network, which is further impacted by the data collected for NDT modelling.

Especially in the case of RAN domain NDT, the RAN communication performance may be additionally impacted by the environment, such as environmental buildings, weather, etc. NDT can make use of such external data for NDT modelling to provide a more accurate approximation of the physical network. The existing solution of NDT NRMs can specify the NDT job synchronization scope which is either the DN(s) of managed entities or the geographical area information. How the external data can by specified by NDT MnS consumer is to be addressed.

5.8.2	Potential requirements

REQ-NDTDG-01: The 3GPP management system should support a capability to allow an authorized MnS consumer to specify external data information used for NDT modelling.

5.8.3	Potential solutions

TS 28.622 defines the ExternalDataType IOC which specifies a type of external management data and the associated meta data. This solution aims to enhance NDTJob IOC and NDTFunction IOC by reusing ExternalDataType IOC to carry the NDT MnS consumer’s external data preference for an NDT job and show the supported external data of NDT Function used for NDT modelling. Details are as shown below:

1. Enhancement for NDTJob IOC

- Add new attribute “externalDataTypeRefList” representing the NDT Job is associated with one or more other ExternalDataType MOIs that are used for NDT modelling.

- Add new attribute “supportedExternalDataList” for NDTFunction IOC which represents the external data supported by the NDTFunction for NDT modelling.

5.8.4	Evaluation of potential solutions

5.9	Use Case #9: NDT for Non-Terrestrial Network (NTN) Performance and Optimization Evaluation

5.9.1	Description

According to 3GPP 38.300 [5] a Non-Terrestrial Network (NTN) is defined as an NG-RAN consisting of gNBs, which provide non-terrestrial NR access to UEs by means of an NTN payload embarked on an airborne or space-borne NTN vehicle and an NTN Gateway. NTN Gateway is an earth station located at the surface of the earth, providing connectivity to the NTN payload using the feeder link which is a wireless transport link between the NTN Gateway and the NTN payload. NTN payload is a network node, embarked on board a satellite or high-altitude platform station, providing connectivity functions, between the service link and the feeder link.

Compared with terrestrial networks, NTNs exhibit unique characteristics such as high mobility of nodes, dynamic topologies, long propagation delays, and variable link conditions. These aspects make planning, optimization, and management challenging for mobile network operators.

This use case proposes using Network Digital Twin (NDT) technology to simulate, analyze, and optimize the behavior of NTNs. For example, the NDT can create a virtual representation of the satellite constellation, NTN Gateways, and user terminals to predict network conditions and support decision-making for service continuity and QoS/QoE management.

The NDT framework for NTN can be used for example to:

- Predict and mitigate handover failures during satellite transitions for NTN entities' constellations.

- Optimize beam management and power allocation based on user distribution and traffic demand.

5.9.2	Potential requirements

5.9.3	Potential solutions

5.9.4	Evaluation of potential solutions

5.10	Use Case #10: Clarification of NDTJob Modification Behaviour

5.10.1	Description

The MnS Consumer requires clarity on the conditions and procedures for modifying an existing NDTJob instance. At the moment, the specification leaves this open to interpretation, even though there are clear limitations associated with different implementation techniques for NDTJobs. For instance, if an NDTJob is implemented as a script that performs calculations based on input from the MnS Consumer and the replicated network, modifying such a script during execution would effectively mean rerunning the script with new parameters as resumption of certain scripts would be impossible. On the other hand, if the implementation is utilising a technology which can enable modification during runtime, then modification during the initial stages—or at specific points within the runtime—may still be possible.

Currently, the specification does not allow for clear modelling of the different potential stages executed during an NDTJob. As a result, modification of an NDTJob will be inconsistent across implementations, making MnS Consumer integration bespoke to specific NDTs.

The current specification does not define a procedure or consequences of the modification of an NDTJob. TS 28.561 [3] Clause 4.4 (“NDT life-cycle management”) states that “the NDT job instance can be configured by the MnS Consumer at any time”, yet no guidance is provided on permissible modification timing, allowable attribute changes, or the expected system behaviour when an NDTJob is already under execution.

This ambiguity raises questions such as:

- When, during the NDTJob lifecycle, modification requests are acceptable.

- Whether modification during execution affects job consistency or results.

- How the MnS Producer should report modification status or failures.

A clearer model describing NDTJob state transitions and management conditions would help ensure consistent interpretation across implementations. Such a model could make it explicit when an NDTJob is in a state that permits modification and when it is not.

5.10.2	Potential requirements

5.10.3	Potential solutions

5.10.4	Evaluation of potential solutions

5.11	Use Case #11: Create and Execute NDT Job

5.11.1	Description

The MnS Consumer requires clarity on whether the creation and execution of an NDTJob are distinct phases or a single combined procedure.

By having creation of an NDTJob separate to the execution of the NDTJob, it means certain MnS Consumers can define the NDTJob, and other MnS Consumers could execute the NDT jobs. This is beneficial to the MnS Consumers as users with different roles can create the NDTJob such as subject matter experts, then a different set of users can execute the simulation, which creates a clear separation of responsibilities.

In the current specification, the relationship between these phases is inconsistent across clauses:

- Clause 4.4 (NDT Life-cycle Management) describes “NDT job instantiation” as including both creation and execution within a single step, suggesting automatic execution upon creation.

- Clause 6.4 (Procedures for NDT operations) depicts separate interactions between the MnS Consumer and MnS Producer for creation and execution, implying that execution follows a distinct request.

- Clause 7.1 (RESTful HTTP-based solution set) defines only the createMOI operation for NDTJob creation, with no corresponding operation for initiating execution.

This discrepancy makes it unclear how an MnS Consumer can prepare an NDTJob configuration and decide when to initiate its execution.

5.11.2	Potential requirements

5.11.3	Potential solutions

5.11.4	Evaluation of potential solutions

5.12	Use Case #12: Clarification of Suspension and Resumption Capabilities for NDTJobs

5.12.1	Description

The MnS Consumer requires clarity on whether an NDTJob can be suspended and subsequently resumed, when such capabilities are supported by the NDT MnS Producer.

The MnS Consumer should have the capability to suspend NDTJobs such as an optimization activity in order to free up resources to prioritize different NDTJobs such as those surrounding faults, otherwise the NDT MnS Producer may not have the available capacity to execute multiple NDTJobs in parallel.

The current specification references the suspension and resumption of NDTJobs in multiple clauses but provides no corresponding procedures, state descriptions, or information model attributes. This creates ambiguity regarding:

- Whether suspension and resumption are supported capabilities or illustrative examples;

- How the MnS Consumer is expected to request suspension or resumption;

- What the expected lifecycle state transitions are following suspension or resumption;

- How results or reports are handled when an NDTJob is suspended.

The capability to suspend and resume NDTJobs should remain optional, dependent on MnS Producer implementation. However, explicit clarification in the specification would help ensure consistent behaviour and interface expectations across implementations.

5.12.2	Potential requirements

5.12.3	Potential solutions

5.12.4	Evaluation of potential solutions

6	Conclusions and Recommendations

6.X	Use case #<X>: <use case title>

Editor's note:	This clause provides conclusions and recommendations for the corresponding use case.

Annex <X> (informative): 
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA5#162 |  |  |  |  | Initial skeleton | V0.0.0 |
| 2025-10 | SA5#163 | 1. S5-254290 2. S5-254670 3. S5-254842 4. S5-254672 5. S5-254843 6. S5-254396 7. S5-254674 8. S5-254733 |  |  |  | 1. Rel-20 pCR TR 28.883 Add structure proposal 2. Rel-20 pCR TR 28.883 Use case on NDT support intent pre-evaluation 3. Rel-20 pCR TR 28.883 Improvement of data generation 4. pCR TR 28.883 Add a use case of NDT data generation 5. pCR TR 28.883 Add use case and requirements on enhancement for multiple NDT collaborations 6. Rel-20 pCR TR 28.883 Enhancement on NDT reporting method 7. Pseudo-CR on TR 28.883 Add New Use Case on Capability Discovery of NDT in NDT Collaboration 8. Rel-20 pCR TR 28.883 Defining the Lifecycle and Runtime Behaviour of NDT Jobs | V0.1.0 |
| 2025-11 | SA5#164 | 1. S5-255521 2. S5-255522 3. S5-255523 4. S5-255524 5. S5-255525 6. S5-255526 7. S5-255527 8. S5-255528 9. S5-255529 10. S5-255531 11. S5-255532 |  |  |  | 1. Rel-20 pCR TR 28.883 Add introduction and scope 2. Rel-20 pCR TR 28.883 Solution for NDT supporting intent feasibility check and exploration 3. Pseudo-CR TR 28.883 Add Solution for Capability Discovery of NDT in NDT Collaboration 4. Rel-20 pCR TR 28.883 Solution for improvement of data generation 5. pCR TR 28.883 Add requirements for NDT data generation 6. Rel-20 pCR TR 28.883 Using external data for NDT modelling 7. Pseudo-CR on TR28.883 Use case about NDT for NTN 8. pCR on TR 28.883 Add solution of NDT reporting method 9. Rel-20 pCR TR 28.883 Clarification of NDTJob Modification Behaviour 10. Rel-20 pCR TR 28.883 Create and Execute NDT Job 11. Rel-20 pCR TR 28.883 Clarification of Suspension and Resumption Capabilities for NDTJobs | V0.2.0 |