3GPP TSG-SA5 Meeting #165	S5-260XXX

Goa, India 09 – 15 February  2026

Source:	Nokia

Title:	Pseudo-CR on 6G principles on Network functions and management functions

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR32.801

Version:	0.0.0

Work Item:	Study on 6G Management and Orchestration (FS_6G_OAM)

Comments

Similar to 5G, some 6G management capabilities are needed to be supported by RAN and core network entities. This pCR is to add an architectural principle emphasizing that RAN and Core Network entities can embed management service producers that are  implemented to produce management services

Proposed Changes

* * * First Change * * * *

# 5	6G Management Architecture Principles

## 5.K	RAN and Core Network entities can act as management functions

Similar to 5G, some 6G management capabilities are needed to be supported on RAN and core network entities. For example, as illustrated in Figure 5.K-1, a core network NF can play the role of a producer of management service(s) for

- Provisioning which enables an MnS consumer to configure the NF,

- Data Collection enabling an MnS consumer to instantiate data collection on the NF, i.e. for the NF to collect different kinds of data e.g. PM data, inventory data, alarm information etc.

- ML Training MnS enabling an MnS consumer to instantiate and manage ML training Jobs.

Principle K.1: In the 6G management framework, RAN and Core Network entities can embed management functions that can be implemented to produce management services specified in 3GPP SA5 specifications.

Figure 5.K-1: RAN nodes and Core Network Functions may embed management functions - any management service can be implemented in the same network element as any Network Function. Some network functions may only consume MnSs(e.g. NF3), other may only produce MnS (e.g. NF2) while others may both consume and producer MnSs (e.g. NF1). Some network functions (e.g. NF y) may also rely on another function for access to management capabilities

* * * End of Changes * * * *