3GPP TSG-SA5 Meeting #165	S5-260107

# India, India, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 1665 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.541 Add openAPI definition for SectorEquipmentFunction and AntennaFunction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AdNRM_Ph4-OAM |  |  |  |  |  | Date: |  |  | 2026-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | SectorEquipmentFunction IOC and AntennaFunction IOC is defined in TS 28.662 and referenced by corresponidng IOCs in TS 28.541. The YANG models for SectorEquipmentFunction IOC and AntennaFunction IOC are defined in TS 28.541, however, corresponding openAPI definitions are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add openAPI definition for SectorEquipmentFunction IOC and AntennaFunction IOC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The openAPI definiton for SectorEquipmentFunction IOC and AntennaFunction IOC is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Following YAML files normatively defined in the forge are updated: - TS28541_GenericRanNrm.yaml |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/2026 at commit e013e56ff065cf9e7ffb387c449eeaff1ea25ee9 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/2026 at commit e013e56ff065cf9e7ffb387c449eeaff1ea25ee9

*** START OF CHANGE 1 ***

*** OpenAPI/TS28541_GenericRanNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Generic RAN NRM

version: 20.2.0

description: >-

OAS 3.0.1 specification of the NR NRM

© 2026, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.541; 5G NRM, Generic RAN NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.541/

paths: {}

components:

schemas:

#-------- Definition of types for name containment-----------------------------------------------------

ManagedElement-ncO-GenericRanNrm:

type: object

properties:

AntennaFunction:

$ref: '#/components/schemas/AntennaFunction-Multiple'

SectorEquipmentFunction:

$ref: '#/components/schemas/SectorEquipmentFunction-Multiple'

#-------- Definition of concrete IOCs --------------------------------------------

AntennaFunction-Single:

type: object

properties:

beamTilt:

type: integer

format: int32

elevation:

type: number

retTiltValue:

type: integer

format: int32

bearing:

type: integer

format: int32

retGroupName:

type: string

maxAzimuthValue:

type: integer

format: int32

minimum: 0

maximum: 360

minAzimuthValue:

type: integer

format: int32

minimum: 0

maximum: 360

horizBeamwidth:

type: integer

format: int32

minimum: 0

maximum: 360

vertBeamwidth:

type: integer

format: int32

minimum: 0

maximum: 360

latitude:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Latitude'

Longitude:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Longitude'

referenceFrom:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

SectorEquipmentFunction-Single:

type: object

properties:

fqBandList:

description: >-

The list of frequency bands/ranges supported by the hardware associated

with the SectorEquipmentFunction.

type: array

uniqueItems: true

items:

type: string

confOutputPower:

type: integer

format: int32

referenceFrom:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

referenceTo:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

AntennaFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/AntennaFunction-Single'

SectorEquipmentFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/SectorEquipmentFunction-Single'

#-------- Definitions in TS 28.541 for TS 28.532 ---------------------------------

resources-genericRanNrm:

oneOf:

- $ref: '#/components/schemas/AntennaFunction-Single'

- $ref: '#/components/schemas/SectorEquipmentFunction-Single'

<CODE ENDS>

*** END OF CHANGE 1 ***

==============End of change==============