3GPP TSG-SA5 Meeting #165	S5-260108

# India, India, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0609 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.623 Add openAPI definition for SectorEquipmentFunction and AntennaFunction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AdNRM_Ph4-OAM |  |  |  |  |  | Date: |  |  | 2026-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | SectorEquipmentFunction IOC and AntennaFunction IOC is defined in TS 28.662 and referenced by corresponidng IOCs in TS 28.541. The YANG models for SectorEquipmentFunction IOC and AntennaFunction IOC are defined in TS 28.541, however, corresponding openAPI definitions are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add openAPI definition for SectorEquipmentFunction IOC and AntennaFunction IOC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The openAPI definiton for SectorEquipmentFunction IOC and AntennaFunction IOC is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Following YAML files normatively defined in the forge are updated: - TS28623_FeatureNrm.yaml - OpenAPI/TS28623_GenericNrm.yaml |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.541 CR 1665 |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/2026 at commit e013e56ff065cf9e7ffb387c449eeaff1ea25ee9 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

*** START OF CHANGE 2 ***

*** OpenAPI/TS28623_FeatureNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: NRM Feautres

version: 19.6.0

description: >-

OAS 3.0.1 definition of the Features of NRM

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.623; Generic NRM, NRM Features

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.623/

paths: {}

components:

schemas:

#----- Definitions in TS 28.623 for TS 28.532 --------------------------#

resources-feature:

anyOf:

- $ref: 'TS28104_MdaNrm.yaml#/components/schemas/resources-mdaNrm'

- $ref: 'TS28105_AiMlNrm.yaml#/components/schemas/resources-AiMlNrm'

- $ref: 'TS28111_FaultNrm.yaml#/components/schemas/resources-faultNrm'

- $ref: 'TS28312_IntentNrm.yaml#/components/schemas/resources-intentNrm'

- $ref: 'TS28317_RanScNrm.yaml#/components/schemas/resources-RanScNrm'

- $ref: 'TS28318_DsoNrm.yaml#/components/schemas/resources-DSORecovery'

- $ref: 'TS28319_MsacNrm.yaml#/components/schemas/resources-msacNrm'

- $ref: 'TS28536_CoslaNrm.yaml#/components/schemas/resources-coslaNrm'

- $ref: 'TS28567_CclNrm.yaml#/components/schemas/resources-CCLNrm'

- $ref: 'TS28538_EdgeNrm.yaml#/components/schemas/resources-edgeNrm'

- $ref: 'TS28541_GenericRanNrm.yaml#/components/schemas/resources-genericRanNrm'

- $ref: 'TS28541_NrNrm.yaml#/components/schemas/resources-nrNrm'

- $ref: 'TS28541_5GcNrm.yaml#/components/schemas/resources-5gcNrm'

- $ref: 'TS28541_SliceNrm.yaml#/components/schemas/resources-sliceNrm'

- $ref: 'TS28561_NdtNrm.yaml#/components/schemas/resources-ndtNrm'

- $ref: 'TS28623_FileManagementNrm.yaml#/components/schemas/resources-fileMgmtNrm'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/resources-genericNrm'

- $ref: 'TS28623_ManagementDataCollectionNrm.yaml#/components/schemas/resources-mgmtDataCollectionNrm'

- $ref: 'TS28623_MnSRegistryNrm.yaml#/components/schemas/resources-mnSRegistryNrm'

- $ref: 'TS28623_PmControlNrm.yaml#/components/schemas/resources-pmControlNrm'

- $ref: 'TS28623_QoEMeasurementCollectionNrm.yaml#/components/schemas/resources-qoEMeasuremetCollectionNrm'

- $ref: 'TS28623_SubscriptionControlNrm.yaml#/components/schemas/resources-subscriptionControlNrm'

- $ref: 'TS28623_ThresholdMonitorNrm.yaml#/components/schemas/resources-thresholdMonitorNrm'

- $ref: 'TS28623_TraceControlNrm.yaml#/components/schemas/resources-traceControlNrm'

- $ref: 'TS28310_EnergyInformationNrm.yaml#/components/schemas/resources-energyInformationNrm'

- $ref: 'TS28623_ExternalDataMgmtNrm.yaml#/components/schemas/resources-externalDataMgmtNrm'

#----- Definitions in TS 28.623 for TS 28.532 --------------------------#

<CODE ENDS>

*** END OF CHANGE 2 ***

*** START OF CHANGE 3 ***

*** OpenAPI/TS28623_GenericNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Generic NRM

version: 19.6.0

description: >-

OAS 3.0.1 definition of the Generic NRM

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.623; Generic NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.623/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

VnfParameters:

description: ->

This <<dataType>> represents the parameter set of the VNF instance(s) corresponding to an NE.

type: object

properties:

vnfInstanceId:

type: string

vnfdId:

type: string

flavourId:

type: string

autoScalable:

type: boolean

default: FALSE

PeeParameters:

description: ->

This <<dataType>> represents the parameter list for the control and monitoring of power, energy and environmental parameters.

type: object

properties:

siteIdentification:

type: string

siteLatitude:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Latitude'

siteLongitude:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Longitude'

siteAltitude:

type: number

format: float

siteDescription:

type: string

equipmentType:

type: string

environmentType:

type: string

powerInterface:

type: string

TransportProtocol:

anyOf:

- type: string

enum:

- TCP

- type: string

SupportedPerfMetricGroup:

type: object

properties:

performanceMetrics:

type: array

uniqueItems: true

items:

type: string

readOnly: true

minItems: 1

granularityPeriods:

type: array

uniqueItems: true

items:

type: integer

minimum: 1

readOnly: true

reportingMethods:

type: array

uniqueItems: true

items:

type: string

enum:

- FILE_BASED_LOC_SET_BY_PRODUCER

- FILE_BASED_LOC_SET_BY_CONSUMER

- STREAM_BASED

readOnly: true

reportingPeriods:

type: array

uniqueItems: true

items:

type: integer

minimum: 1

readOnly: true

ReportingCtrl:

oneOf:

- type: object

properties:

fileReportingPeriod:

type: integer

- type: object

properties:

fileReportingPeriod:

type: integer

notificationRecipientAddress:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

- type: object

properties:

fileReportingPeriod:

type: integer

fileLocation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

- type: object

properties:

streamTarget:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

ProcessMonitor:

description: >-

This data type is the "ProcessMonitor" data type without specialisations.

type: object

properties:

id:

type: string

readOnly: true

status:

type: string

enum:

- NOT_STARTED

- RUNNING

- FINSHED

- FAILED

- PARTIALLY_FAILED

- CANCELLING

- CANCELLED

readOnly: true

progressPercentage:

type: integer

minimum: 0

maximum: 100

readOnly: true

progressStateInfo:

type: string

readOnly: true

resultStateInfo:

type: string

readOnly: true

startTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

endTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

timer:

type: integer

AreaScope:

allOf:

- type: array

items:

$ref: 'TS28541_NrNrm.yaml#/components/schemas/PlmnInfoList'

maxItems: 16384

- oneOf:

- type: array

items:

$ref: '#/components/schemas/EutraCellId'

minItems: 1

maxItems: 32

- type: object

properties:

nrCellId:

type: array

items:

$ref: '#/components/schemas/NrCellId'

minItems: 1

maxItems: 32

nPNIdentityList:

type: array

items:

$ref: '#/components/schemas/NpnId-Type'

- type: object

properties:

tacList:

type: array

items:

$ref: '#/components/schemas/Tac'

minItems: 1

maxItems: 8

cAGIdList:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/CagId'

maxItems: 256

- type: object

properties:

taList:

type: array

items:

$ref: '#/components/schemas/Tai'

minItems: 1

maxItems: 8

nPNIdentityList:

type: array

items:

$ref: '#/components/schemas/NpnId-Type'

- type: object

properties:

nTNGeoAreaList:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

minItems: 1

maxItems: 8

- type: array

items:

$ref: '#/components/schemas/NpnId-Type'

minItems: 1

Tai:

type: object

properties:

mcc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mcc'

mnc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mnc'

tac:

$ref: '#/components/schemas/Tac'

Tac:

type: string

pattern: '(^[A-Fa-f0-9]{4}$)|(^[A-Fa-f0-9]{6}$)'

EutraCellId:

type: string

pattern: '^[A-Fa-f0-9]{7}$'

NrCellId:

type: string

pattern: '^[A-Fa-f0-9]{9}$'

IpAddr:

oneOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/Ipv4Addr'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/Ipv6Addr'

SchedulingTime:

oneOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

- type: object

properties:

timeIntervals:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeInterval'

- type: object

properties:

timeIntervals:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeInterval'

daysOfWeek:

type: array

uniqueItems: true

items:

type: string

enum:

- MONDAY

- TUESDAY

- WEDNESDAY

- THURSDAY

- FRIDAY

- SATURDAY

- SUNDAY

minItems: 1

maxItems: 7

- type: object

properties:

timeIntervals:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeInterval'

daysOfMonth:

type: array

uniqueItems: true

items:

type: integer

minimum: 0

maximum: 31

NpnId-Type:

type: object

properties:

plmnId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

cAGIdList:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/CagId'

maxItems: 256

nIDList:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Nid'

maxItems: 16

oneOf:

- required: [plmnId, cAGIdList]

- required: [plmnId, nIDList]

#-------- Definition of abstract IOC Top -----------------------------------------

Top-Attr:

#  This definition will be deprecated, when all occurances of Top-Attr

#  are replaced by Top.

type: object

properties:

id:

type: string

nullable: true

objectClass:

type: string

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

VsDataContainer:

$ref: '#/components/schemas/VsDataContainer-Multiple'

required:

- id

Top:

type: object

properties:

id:

type: string

nullable: true

objectClass:

type: string

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

VsDataContainer:

$ref: '#/components/schemas/VsDataContainer-Multiple'

required:

- id

#-------- Definition of IOCs with new name-containments defined in other TS ------

SubNetwork-Attr:

type: object

properties:

dnPrefix:

type: string

userLabel:

type: string

userDefinedNetworkType:

type: string

setOfMcc:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mcc'

readOnly: true

minItems: 1

priorityLabel:

type: integer

readOnly: true

supportedPerfMetricGroups:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/SupportedPerfMetricGroup'

supportedTraceMetrics:

type: array

uniqueItems: true

items:

type: string

ManagedElement-Attr:

type: object

properties:

dnPrefix:

type: string

managedElementTypeList:

type: array

uniqueItems: true

items:

type: string

userLabel:

type: string

locationName:

type: string

managedBy:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

vendorName:

type: string

readOnly: true

userDefinedState:

type: string

swVersion:

type: string

readOnly: true

priorityLabel:

type: integer

supportedPerfMetricGroups:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/SupportedPerfMetricGroup'

supportedTraceMetrics:

type: array

uniqueItems: true

items:

type: string

readOnly: true

SubNetwork-ncO:

type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

ManagedElement:

$ref: '#/components/schemas/ManagedElement-Multiple'

ManagementNode:

$ref: '#/components/schemas/ManagementNode-Multiple'

MnsAgent:

$ref: '#/components/schemas/MnsAgent-Multiple'

MeContext:

$ref: '#/components/schemas/MeContext-Multiple'

AlarmList:

$ref: 'TS28111_FaultNrm.yaml#/components/schemas/AlarmList-Single'

Scheduler:

$ref: '#/components/schemas/Scheduler-Multiple'

ConditionMonitor:

$ref: '#/components/schemas/ConditionMonitor-Multiple'

SupportedNotifications:

$ref: '#/components/schemas/SupportedNotifications-Single'

ManagedElement-ncO:

type: object

properties:

MnsAgent:

$ref: '#/components/schemas/MnsAgent-Multiple'

AlarmList:

$ref: 'TS28111_FaultNrm.yaml#/components/schemas/AlarmList-Single'

Scheduler:

$ref: '#/components/schemas/Scheduler-Multiple'

ConditionMonitor:

$ref: '#/components/schemas/ConditionMonitor-Multiple'

SupportedNotifications:

$ref: '#/components/schemas/SupportedNotifications-Single'

#-------- Definition of abstract IOCs --------------------------------------------

ManagedFunction-Attr:

type: object

properties:

userLabel:

type: string

vnfParametersList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/VnfParameters'

peeParametersList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/PeeParameters'

priorityLabel:

type: integer

supportedPerfMetricGroups:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/SupportedPerfMetricGroup'

supportedTraceMetrics:

type: array

uniqueItems: true

items:

type: string

EP_RP-Attr:

type: object

properties:

userLabel:

type: string

farEndEntity:

type: string

readOnly: true

supportedPerfMetricGroups:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/SupportedPerfMetricGroup'

ManagedFunction-ncO:

type: object

properties:

PerfMetricJob:

$ref: 'TS28623_PmControlNrm.yaml#/components/schemas/PerfMetricJob-Multiple'

ThresholdMonitor:

$ref: 'TS28623_ThresholdMonitorNrm.yaml#/components/schemas/ThresholdMonitor-Multiple'

TraceJob:

$ref: 'TS28623_TraceControlNrm.yaml#/components/schemas/TraceJob-Multiple'

#-------- Definition of concrete IOCs --------------------------------------------

VsDataContainer-Single:

type: object

properties:

id:

type: string

attributes:

type: object

properties:

vsDataType:

type: string

readOnly: true

vsDataFormatVersion:

type: string

readOnly: true

vsData:

nullable: true

VsDataContainer:

$ref: '#/components/schemas/VsDataContainer-Multiple'

ManagementNode-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

userLabel:

type: string

managedElements:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

vendorName:

type: string

readOnly: true

userDefinedState:

type: string

locationName:

type: string

readOnly: true

swVersion:

type: string

readOnly: true

MnsAgent:

$ref: '#/components/schemas/MnsAgent-Multiple'

MnsAgent-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

systemDN:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

MeContext-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

dnPrefix:

type: string

readOnly: true

Scheduler-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

schedulingTimes:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/SchedulingTime'

minItems: 1

schedulerStatus:

type: boolean

readOnly: true

ConditionMonitor-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

condition:

type: string

conditionStatus:

type: boolean

readOnly: true

SupportedNotifications-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

notificationTypes:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationType'

notificationProtocols:

type: array

uniqueItems: true

items:

type: string

enum:

- HTTP

- HTTP_VES_ENCAPS

minItems: 1

readOnly: true

SubNetwork-Single:

allOf:

- $ref: '#/components/schemas/Top'

- $ref: '#/components/schemas/SubNetwork-Attr'

- $ref: '#/components/schemas/SubNetwork-ncO'

- $ref: '#/components/schemas/SubNetwork-Multiple'

- $ref: '#/components/schemas/ManagedElement-Multiple'

- $ref: 'TS28623_FileManagementNrm.yaml#/components/schemas/SubNetwork-ncO-FileManagementNrm'

- $ref: 'TS28623_PmControlNrm.yaml#/components/schemas/SubNetwork-ncO-PmControlNrm'

- $ref: 'TS28623_SubscriptionControlNrm.yaml#/components/schemas/SubNetwork-ncO-SubscriptionControlNrm'

- $ref: 'TS28623_ThresholdMonitorNrm.yaml#/components/schemas/SubNetwork-ncO-ThresholdMonitorNrm'

- $ref: 'TS28623_TraceControlNrm.yaml#/components/schemas/SubNetwork-ncO-TraceControlNrm'

- $ref: 'TS28623_QoEMeasurementCollectionNrm.yaml#/components/schemas/SubNetwork-ncO-QoEMeasurementCollectionNrm'

- $ref: 'TS28623_ManagementDataCollectionNrm.yaml#/components/schemas/SubNetwork-ncO-ManagementDataCollectionNrm'

- $ref: 'TS28623_MnSRegistryNrm.yaml#/components/schemas/SubNetwork-ncO-MnSRegistryNrm'

- $ref: 'TS28104_MdaNrm.yaml#/components/schemas/SubNetwork-ncO-MdaNrm'

- $ref: 'TS28105_AiMlNrm.yaml#/components/schemas/SubNetwork-ncO-AiMlNrm'

- $ref: 'TS28312_IntentNrm.yaml#/components/schemas/SubNetwork-ncO-IntentNrm'

- $ref: 'TS28317_RanScNrm.yaml#/components/schemas/SubNetwork-ncO-RanScNrm'

- $ref: 'TS28536_CoslaNrm.yaml#/components/schemas/SubNetwork-ncO-CoslaNrm'

- $ref: 'TS28538_EdgeNrm.yaml#/components/schemas/SubNetwork-ncO-EdgeNrm'

- $ref: 'TS28541_SliceNrm.yaml#/components/schemas/SubNetwork-ncO-SliceNrm'

- $ref: 'TS28541_NrNrm.yaml#/components/schemas/SubNetwork-ncO-NrNrm'

- $ref: 'TS28541_5GcNrm.yaml#/components/schemas/SubNetwork-ncO-5GcNrm'

- $ref: 'TS28318_DsoNrm.yaml#/components/schemas/SubNetwork-ncO-DsoNrm'

- $ref: 'TS28310_EnergyInformationNrm.yaml#/components/schemas/SubNetwork-ncO-EnergyInformationNrm'

ManagedElement-Single:

allOf:

- $ref: '#/components/schemas/Top'

- $ref: '#/components/schemas/ManagedElement-Attr'

- $ref: '#/components/schemas/ManagedElement-ncO'

- $ref: 'TS28623_FileManagementNrm.yaml#/components/schemas/ManagedElement-ncO-FileManagementNrm'

- $ref: 'TS28623_PmControlNrm.yaml#/components/schemas/ManagedElement-PmControlNrm'

- $ref: 'TS28623_SubscriptionControlNrm.yaml#/components/schemas/ManagedElement-ncO-SubscriptionControlNrm'

- $ref: 'TS28623_ThresholdMonitorNrm.yaml#/components/schemas/ManagedElement-ncO-ThresholdMonitorNrm'

- $ref: 'TS28623_TraceControlNrm.yaml#/components/schemas/SubNetwork-ncO-TraceControlNrm'

- $ref: 'TS28623_QoEMeasurementCollectionNrm.yaml#/components/schemas/ManagedElement-ncO-QoEMeasurementCollectionNrm'

- $ref: 'TS28104_MdaNrm.yaml#/components/schemas/ManagedElement-ncO-MdaNrm'

- $ref: 'TS28105_AiMlNrm.yaml#/components/schemas/ManagedElement-ncO-AiMlNrm'

- $ref: 'TS28536_CoslaNrm.yaml#/components/schemas/ManagedElement-ncO-CoslaNrm'

- $ref: 'TS28541_NrNrm.yaml#/components/schemas/ManagedElement-ncO-NrNrm'

- $ref: 'TS28541_5GcNrm.yaml#/components/schemas/ManagedElement-ncO-5GcNrm'

- $ref: 'TS28541_GenericRanNrm.yaml#/components/schemas/ManagedElement-ncO-GenericRanNrm'

NrmRoot:

oneOf:

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

- type: object

properties:

ManagedElement:

$ref: '#/components/schemas/ManagedElement-Multiple'

#-------- Definition of YAML arrays for name-contained IOCs ----------------------

VsDataContainer-Multiple:

type: array

items:

$ref: '#/components/schemas/VsDataContainer-Single'

ManagementNode-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagementNode-Single'

MnsAgent-Multiple:

type: array

items:

$ref: '#/components/schemas/MnsAgent-Single'

MeContext-Multiple:

type: array

items:

$ref: '#/components/schemas/MeContext-Single'

Scheduler-Multiple:

type: array

items:

$ref: '#/components/schemas/Scheduler-Single'

ConditionMonitor-Multiple:

type: array

items:

$ref: '#/components/schemas/ConditionMonitor-Single'

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

ManagedElement-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagedElement-Single'

#-------- Definitions in TS 28.623 for TS 28.532 ---------------------------------

resources-genericNrm:

oneOf:

- $ref: '#/components/schemas/NrmRoot'

- $ref: '#/components/schemas/VsDataContainer-Single'

- $ref: '#/components/schemas/ManagementNode-Single'

- $ref: '#/components/schemas/MnsAgent-Single'

- $ref: '#/components/schemas/MeContext-Single'

- $ref: '#/components/schemas/Scheduler-Single'

- $ref: '#/components/schemas/ConditionMonitor-Single'

- $ref: '#/components/schemas/SupportedNotifications-Single'

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/ManagedElement-Single'

<CODE ENDS>

*** END OF CHANGE 3 ***

==============End of change==============