3GPP TSG-SA5 Meeting #165	S5-260146

# India, India, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.422 | CR | 0550 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 32.422 Enhance Geo area scope for NTN MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2026-01-29 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | According to TS 38.413 (agreed R3-255996), the MDT Configuration-NR attribute including two kind of PLMN list, one is “Signalling Based MDT PLMN List”, and another one is “MDT PLMN List” which is included in NTN Geographical Area. Where the “Signalling Based MDT PLMN List” may be different than the content of the PLMN list for geographical area of NTN MDT. And as discribed in TS 37.320, it states “If the list of PLMNs is included, the UE will only log measurements for the cells belonging to the listed PLMNs.”. However, in SA5, only one PLMN list is defined in NR MDT configuration, therefore, we believe that reusing the existing PLMN list IE may cause issues, and it is recommended to keep it consistent with the one defined by the RAN. Therefore, it is propose the following enhancements. - Enhance the description in section 4.3.38.1 to indicate that the NTN geographical area identified by geographical area and PLMN list. - Modify defintion of attribute nTNGeoAreaList to make it as a datatype. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Enhance the description in section 4.3.38.1 to indicate that the NTN geographical area identified by geographical area and PLMN list. - Modify defintion of attribute nTNGeoAreaList to make it as a datatype, which is including the attribute` nTNGeographicalArea and mDTplmnList. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The implementation of geographical area of NTN MDT will be inaccurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.10.2	Area Scope

The Area Scope optional parameter defines the area in terms of Cells or Tracking Area/Routing Area/Location Area/Geographical Area where the MDT data collection shall take place. The area scope specified in an MDT session shall support the PLMNs of the MDT PLMN list (defined in clause 5.10.24). If the parameter is not present the MDT data collection shall be done throughout the PLMNs of the MDT PLMN list. In case of NR NPN scenarios, the area scope parameter may also contain a combination of NPN ID and any of the existing parameters such as cells or tracking area or tracking area identity. For further details see also TS 37.320 [30] and 38.413 [49].

The Area Scope parameter in UMTS is either:

-	list of Cells, identified by CGI. Maximum 32 CGI can be defined.

-	List of Routing Area, identified by RAI. Maximum of 8 RAIs can be defined.

-	List of Location Area, identified by LAI. Maximum of 8 LAIs can de defined.

The Area Scope parameter in LTE and NR contains one of the followings:

-	list of Cells, identified by E-UTRAN-CGI or NG-RAN CGI. Maximum 32 CGI can be defined.

-	List of Tracking Area, identified by TAC. Maximum of 8 TAC can be defined.

-	List of Tracking Area Identity, identified by TAC with associated plmn-Identity per TAC-List containing the PLMN identity for each TAC. Maximum of 8 TAI can be defined. For further details see also TS 36.331[32].

-	List of NTN Geographical Area in NR, identified by geographical area and PLMN list. Maximum 8 geographical areascan be defined. Each geographical area is either a Circle identified by reference location and distance radius or Polygon identified by a set of geo-coordinates. If the list of PLMNs is included, the UE will only log measurements for the cells belonging to the listed PLMNs. For further details see Geographical Area for NTN MDT in clause 9.3.3.68 in TS 38.413[49] and in clause 5.1.1.1.1 in TS 37.320 [30].

-	List of NPN IDs in NR. It is either a list of PNI-NPNs identified by CAG ID with associated plmn-Identity (Maximum 256 PNI-NPNs can be defined) or a list of SNPN by Network ID with associated plmn-Identity (Maximum 16 SNPNs can be defined).

The Area Scope parameter in NR can also contain:

-	List of Network slices in NR. It is a list of Network Slices identified by PLMN-Id and S-NSSAI. Maximum of 16 PLMN-Id with 1024 S-NSSAI can be defined. For further details see Network Slice Area Scope of MDT in clause 9.3.3 in TS 38.413[49].

End of changes