3GPP TSG SA5 Meeting #165	S5-260205

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	pCR TR 28.884 Evaluation of potential solutions for inventory management

Document for:	Approval

Agenda item:	6.20.4

Spec:	3GPP TR 28.884

Version:	0.2.0

Work Item:	FS_SBMA_Ph4

Comments

This contribution proposes to add evaluation of potential solutions for inventory management.

Proposed Changes

* * * First Change * * * *

### 5.5.4	Evaluation of potential solutions

This clause provides an evaluation of the potential solution described in clause 5.5.3, which proposes reusing the inventory NRMs defined in TS 28.632 [15] to support hardware related inventory management in the SBMA framework. The solution aims to address REQ-SBMA-IM-1 and REQ-SBMA-IM-2 in clause 5.5.2 by enabling MnS based inventory management for 5G systems.

Although the scope of TS 28.631 [14] and TS 28.632 [15] is limited to the communication between IRPAgent and IRPManager, the information model content defined in TS 28.632 [15] remains applicable for representing hardware inventory in a technology agnostic manner. The inventory NRMs defined in TS 28.632 [15], including InventoryUnit, InventoryUnitNE, and InventoryUnitHw, provide a generic and extensible representation of hardware components. These models are largely independent of the underlying RAT or network generation, and therefore can be reused as a basis for 5G hardware inventory representation.

The evaluation concludes that the proposed solution is technically feasible, aligns with SBMA architectural principles, and minimizes specification effort by reusing existing models. It effectively addresses REQ-SBMA-IM-1 and REQ-SBMA-IM-2 by enabling:

-	A standardized representation of 5G hardware inventory;

-	MnS based exposure of inventory information;

-	Cross-generation consistency between 4G and 5G inventory models.

The solution is therefore considered a candidate for supporting hardware inventory management within the SBMA framework.

* * * End of Changes * * * *