3GPP TSG SA5 Meeting #165	S5-260231

# Goa, India, 9-13 February 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0618 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.255 Correct some descriptions on QoS flow Based Charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SBI_CH, TEI18 |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The NOTE2 in the Table 5.2.1.6.1 ‘The columns CHF allowed to change category, and CHF allowed enable and disable are only applicable for the PDU session establishment, for other cases they are not applicable’ is conflict with the description in clause 5.2.1.7 that: The "Roaming Charging Profile" resulting from the exchange between the VPLMN and HPLMN at PDU session establishment may be changed by the H-CHF under the following conditions: - At each V-SMF change in Home routed scenario, the "Roaming Charging Profile" may be renegotiated between the VPLMN and HPLMN. - During the PDU session lifetime if V-SMF includes the Roaming charging profile in the Charging Data Request. The updated Roaming Charging profile is valid until the next change. It described in clause 5.2.1.7 that the Roaming Charging Profile can be changed during the PDU session lifetime, not only during the PDU session establishment. The table name does not match the description before the table, the description mentions that Table 5.2.1.6.1 summarizes the set of default trigger conditions, but the table name is default chargeable events. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove inconsistent note in Table 5.2.1.6.1. - Update the summarize description before Table 5.2.1.6.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent descriptions in the charging specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

#### 5.2.1.6	QoS flow Based Charging (QBC)

QoS flow Based Charging allows the SMF to collect charging information related to data volumes per PDU session, categorized per QoS Flow for roaming charging scenarios. Quota management is not applicable for QBC.The user can be identified by SUPI.

The SMF categorizes the volume within PDU session by QoS Flow identified by QoS Flow Identifier (QFI).

The amount of data counted for the QoS Flow shall be the user plane payload at the UPF.

Table 5.2.1.6.1 summarizes the set of default chargeable events and their category which shall be supported by the SMF in QBC. For "immediate report" category, the table also provides the corresponding Charging Data Request [Initial, Update, Termination] message sent from SMF towards the CHF.

Table 5.2.1.6.1: Default Chargeable events in SMF for QBC

| Chargeable event | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Start of PDU session | PDU session | Immediate | Not Applicable | Not Applicable | Charging Data Request [Initial] |
| Start of a QoS Flow | QoS Flow | Deferred | Not Applicable | Not Applicable | Charging Data Request [Update] |
| Change of Charging conditions |  |  |  |  |  |
| QoS change | QoS Flow | Deferred | Yes | Yes |  |
| GFBR guaranteed status change | QoS Flow | Deferred | Yes | Yes |  |
| User Location change | PDU session | Deferred | Yes | Yes |  |
| Serving Node change | PDU session | Deferred | Yes | Yes |  |
| Change of 3GPP PS Data off Status | PDU session | Deferred | Yes | Yes |  |
| Tariff time change | PDU session | Deferred | No | No |  |
| UE time zone change | PDU session | Immediate | Yes | Yes |  |
| PLMN change | PDU session | Immediate | Yes | Yes |  |
| RAT type change | PDU session | Immediate | Yes | Yes |  |
| Session-AMBR change | PDU session | Immediate | Yes | Yes |  |
| Addition of UPF | PDU session | Immediate | Yes | Yes |  |
| Removal of UPF | PDU session | Immediate | Yes | Yes |  |
| Handover cancel | PDU session | Immediate | Yes | Yes |  |
| Handover start | PDU session | Immediate | Yes | Yes |  |
| Handover complete | PDU session | Immediate | Yes | Yes |  |
| Redundant transmission change | QoS Flow | Immediate | Yes | Yes |  |
| Satellite backhaul category change | PDU session | Deferred | Yes | Yes |  |
| Satellite Backhaul QoS change | QoS Flow | Deferred | Yes | Yes |  |
| GEO satellite ID change | PDU session | Deferred | Yes | Yes |  |
| S-NSSAI replacement | PDU session | Immediate | Yes | Yes |  |
| Limit per PDU session |  |  |  |  |  |
| Expiry of data time limit per PDU session | PDU session | Immediate | No | Yes |  |
| Expiry of data volume limit per PDU session | PDU session | Immediate | No | Yes |  |
| Expiry of data event limit per PDU session | PDU session | Immediate | No | Yes |  |
| Expiry of limit of number of charging condition changes | PDU session | Immediate | No | Yes |  |
| Limit per QoS Flow |  |  |  |  |  |
| Expiry of data time limit per QoS Flow | QoS Flow | Deferred | Yes | Yes |  |
| Expiry of data volume limit per QoS Flow | QoS Flow | Deferred | Yes | Yes |  |
| Others |  |  |  |  |  |
| End of QoS Flow | QoS Flow | Deferred | Yes | Yes |  |
| Management intervention | PDU session | Immediate | No | No | Charging Data Request [Update] Charging Data Request [Termination] |
| V-SMF change | PDU session | Immediate | Not Applicable | Not Applicable | Charging Data Request [Initial] Charging Data Request [Termination] |
| End of PDU session | PDU session | Immediate | No | No | Charging Data Request [Termination] |
| Abort request is received from the CHF | PDU session | Immediate | No | No |  |
| NOTE 1: If GFBR guaranteed status change is enabled, SMF needs to ensure the request for the notification from the access network (i.e. 3GPP RAN) when the GFBR can no longer (or can again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow. |  |  |  |  |  |

The default "Limit" trigger conditions, are trigger thresholds configured in the Charging Characteristics applied to the PDU session for QBC. It shall be possible for the CHF to override these default triggers when providing Charging Data Response [Initial], either to disable the triggers, or to enable triggers new thresholds value.

The "Limit" trigger conditions applied to the QoS Flow level of QBC is common for all QFIs, and applies the limit for each QFI in the PDU session.

For QBC the following details of chargeable events and corresponding actions in the SMF are defined in Table 5.2.1.6.2:

Table 5.2.1.6.2: Chargeable events and their related actions in SMF for QBC

| Chargeable event | Conditions | SMF action |
| --- | --- | --- |
| Start of PDU session |  | Charging Data Request [Initial]. |
| Start of a QoS Flow | Start of the QoS Flow associated with the default QoS rule | Start new counts with time stamps. |
|  | Start of a QoS Flow | Start new counts with time stamps. |
| V-SMF change | If the session is moved to the V-SMF | Charging Data Request [Initial]. Start new counts with time stamps |
|  | If the session is moved from the V-SMF | Charging Data Request [Termination] Close the counts with time stamps |
| End of a QoS Flow | If the corresponding trigger is enabled | Close the counts with time stamps for the QoS flows |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | Charging Data Request [Update] |
| End of PDU session |  | Charging Data Request [Termination] Close the counts with time stamps |
| Change of charging condition in the SMF | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps for all active QoS flows. |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | Charging Data Request [Update] |
| Handover start | If the corresponding trigger is enabled | Close the counts with time stamps and start new counts with time stamps. |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | Charging Data Request [Update]. |
| Handover cancel | If the corresponding trigger is enabled | Close the counts with time stamps and start new counts with time stamps for active QoS flows. |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | Charging Data Request [Update]. |
| Handover complete | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps for active QoS flows. |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | Charging Data Request [Update] |
| Redundant transmission change | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps. |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | Charging Data Request [Update]. |
| Satellite backhaul category change | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | Charging Data Request [Update]. |
| GEO satellite ID change | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | Charging Data Request [Update]. |
| Satellite backhaul QoS change | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | Charging Data Request [Update]. |
| Addition of UPF | If the corresponding trigger is enabled | Start new counts with time stamps for the added UPF. |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | Charging Data Request [Update]. |
| Removal of UPF | If the corresponding trigger is enabled | Close the counts with time stamps for the removed UPF |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | Charging Data Request [Update]. |
| S-NSSAI replacement | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | Charging Data Request [Update]. |
| Expiry of data time limit per QoS Flow | If the corresponding trigger is enabled | Close the counts with time stamps. |
|  | If the category is set to "immediate reporting" | Charging Data Request [Update] |
|  | If the QoS Flow is still active | Start new counts with time stamps |
| Expiry of data volume limit per QoS Flow | If the corresponding trigger is enabled | Close the counts with time stamps |
|  | If the category is set to "immediate reporting" | Charging Data Request [Update] |
|  | If the QoS Flow is still active | Start new counts with time stamps |
| Expiry of data time limit per PDU session | If the corresponding trigger is enabled | Charging Data Request [Update] Close the counts with time stamps for all QoS flows. |
|  | If the PDU session is still active | Start new counts with time stamps for all active QoS flows |
| Expiry of data volume limit per PDU session | If the corresponding trigger is enabled | Close the counts with time stamps for all QoS flows. |
|  | If the PDU session is still active | Start new counts with time stamps for all active QoS flows |
| Expiry of data event limit per PDU session | If the corresponding trigger is enabled | Charging Data Request [Update] Close the counts with time stamps for all QoS flows. |
| Expiry of a limit of number of charging condition changes per PDU session | If the corresponding trigger is enabled | Charging Data Request [Update] Close the counts with time stamps for all QoS flows. |
|  | If the PDU session is still active | Start new counts with time stamps for all active QoS flows |
| Management intervention | If the management intervention causes an update e.g., a reauthorization request. | Charging Data Request [Update] Close the counts with time stamps for all QoS Flows |
|  | If the PDU session is still active | Start new counts with time stamps |
|  | If the management intervention causes a termination e.g., an abort charging request. | Charging Data Request [Termination] Close the counts with time stamps |
| Abort request is received from the CHF | If the corresponding trigger is enabled | Charging Data Request [Termination] Close the counts with time stamps |

The CDR generation mechanism processed by the CHF upon receiving Charging Data Request [Initial, Update, Termination] issued by the SMF for these chargeable events in QBC, is specified in clause 5.2.3.

==============End of change==============