3GPP TSG SA5 Meeting #165	S5-260233

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	Rel-20 pCR 32.872 update solution #1.2

Document for:	Approval

Agenda item:	7.5.2

Spec:	3GPP TR 32.872

Version:	TR V0.2.0

Work Item:	FS_RoamRE_CH

Comments

This pCR proposes to update handling method of abnormal messages received by V-CHF for the solution#1.2 in Topic#1.

Proposed Changes

* * * First Change * * * *

#### 5.1.4.2	Solution #1.2: Handling of abnormal messages received by V-CHF

A possible solution for key issue #1 covering requirements REQ-3GPPCH-LBIC-1, enhanced failure handling for scenarios where a failure is detected between the CTF and the V-CHF.

When a Charging Data Request [Update, or Termination] received by the V-CHF, which cannot be associated to any existing charging session (i.e., resource in V-CHF), should be handled as specified in clause 5.5.1.2 of 3GPP TS 32.290[3]. As the associated charging session between the V-CHF and H-CHF has been released, the V-CHF sends a Charging Data Request [Initial] to the H-CHF with the unit usage received from CTF.

* * * End of Changes * * * *