3GPP TSG SA5 Meeting #165	S5-260237

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	Rel-20 pCR 32.872 Use triggers from V-CTF in V-CHF charging request

Document for:	Approval

Agenda item:	7.5.2

Spec:	3GPP TR 32.872

Version:	TR V0.2.0

Work Item:	FS_RoamRE_CH

Comments

This pCR proposes to introduce a new solution on Local Breakout Inter CHFs scenario key issue3.

Proposed Changes

* * * First Change * * * *

#### 5.1.4.c	Solution #1.c: Use triggers from V-CTF in V-CHF charging request

The solution for key issue #1 covering requirements REQ-3GPPCH-LBIC-3, identify the trigger conditions in V-CHF for the Local Breakout N40+N107 charging scenario.

When a Charging Data Request is sent from V-CHF to H-CHF for inter CHFs communication, the value of the Triggers and Unit Count Inactivity Timer parameters in this request are derived from the Charging Data Request previously received from the V-SMF.

When a Charging Data Response is sent from V-CHF to CTF for inter CHFs communication, the value of the Triggers and Unit Count Inactivity Timer parameters included in this response are derived from the Charging Data Response previously received from the H-CHF.

The Charging Data Request message contents are shown below as an example:

NOTE:	Only relevant information elements are shown in table 5.1.4.c.1.

Table 5.1.4.c.1: Charging Data Request message contents in V-CHF

| Information Element | Converged Charging Category | Offline Only Charging Category | Description |
| --- | --- | --- | --- |
| Session Identifier | OC | OC | / |
| Triggers | OC | OC | This field identifies the event(s) triggering the request and is common to all Multiple Unit Usage occurrences. For inter-CHF communication, this field identifies the triggers from original NF consumer. |
| Multiple Unit Usage | OC | OC | / |
| Rating Group | M | M | / |
| Requested Unit | OC | - | / |
| Used Unit Container | OC | OC | / |
| Triggers | OC | Oc | This field holds reason for charging information reporting or closing for the used unit container. For inter-CHF communication, this field holds reason for charging information reporting or closing for the used unit container from original NF consumer. |
| Trigger Timestamp | OC | OC | This field holds the timestamp of the trigger. For inter-CHF communication, this field holds the timestamp of the trigger from original NF consumer. |

* * * End of Changes * * * *