3GPP TSG SA5 Meeting #165	S5-260296

Goa, India, 9-13 February 2026

Source:	Nokia

Title:	Pseudo-CR on Rel-20 TR 28.885 Update UC Management mechanisms to support service adjustments to adapt to energy-related characteristics and energy rationing

Document for:	Approval

Agenda item:	6.20.5

Spec:	3GPP TR 28.885

Version:	0.3.0

Work Item:	FS_Energy_Ph4_OAM

Comments

To update the evaluation of the potential solutions, conclusions and recommendations for the following use case defined in clause 5.1.1 of TR 28.885.

- Use case #2: Management mechanisms to support service adjustments to adapt to energy-related characteristics and energy rationing

There were no potential solutions studied for this use case, and hence no work proposed for normative phase.

Proposed Changes

* * * First Change * * * *

### 5.1.2	Use case #2: Management mechanisms to support service adjustments to adapt to energy-related characteristics and energy rationing

#### 5.1.2.1	Description

Under energy rationing constraints (i.e., when energy availability is limited or restricted), there is a potential risk for the entire network to shut-down causing service disruption. During such unavoidable hard constraints, the operators would like the network to provide services to their consumers to extend the time duration for which the services are available, and this could be by degrading the network service performance. For example, the operators might want to prioritize the voice and emergency services (e.g. eMBB) over some data services (e.g., URLLC or Gaming service or Video service). The 3GPP management system can utilize the information related to energy rationing and other energy-related characteristics (such as energy capacity and energy availability), and provide mechanisms to the operator to prevent their network from going out of service and extend the duration for which the services are available.

Energy rationing and energy-related characteristics are defined in TS 22.261 [2].

The requirement specified in clause 6.15a.2.2 of TS 22.261 [2] includes providing means to enable the operator to degrade service performance to meet energy rationing constraints.

Another requirement specified in clause 6.15a.2.2 of TS 22.261 [2] include providing mechanisms to adjust communication service considering change of energy supply mix of the network as one of the factors. For example, adjustments based on the carbon emissions of different network functions.

This use case is to study

-	management mechanisms to enable the operator to provide means to adapt the services provided by the network (e.g., by modifying the QoS requirements) to meet energy rationing control.

-	management mechanisms to enable the network to adjust communication service (e.g., activate or deactivate communication services) adapting to energy-related characteristics.

#### 5.1.2.2	Potential requirements

PREQ-Energy_Related_Service_Adjustment-1: The 3GPP management system should enable the operator to adapt the network performance of existing communication services to meet energy rationing control utilizing the available energy rationing information.

PREQ-Energy_Related_Service_Adjustment-2: The 3GPP management system should enable the network to adjust communication services adapting to energy-related characteristics.

#### 5.1.2.3	Potential solutions

None

#### 5.1.2.4	Evaluation of potential solutions

There were no potential solutions studied for this use case.

* * * Next Change * * * *

### 6.1.A	Use case #2: Management mechanisms to support service adjustments to adapt to energy-related characteristics and energy rationing

The use case and potential requirement for Management mechanisms to support service adjustments to adapt to energy-related characteristics and energy rationing are described in clause 5.1.2. There is no potential solution for this use case, hence no impacts identified for the normative phase.

* * * End of Changes * * * *