3GPP TSG SA5 Meeting #165	S5-260301

Goa, India, 9-13 February 2026

Source:	Samsung

Title:	pCR on NDT State Transition

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.883

Version:	V1.0.0

Work Item:	FS_NDT_Ph2

Comments

This pCR provides the potential solution for Use case 10, 11 and 12.

Proposed Changes

* * * First Change * * * *

## 5.10	Use Case #10: Clarification of NDTJob Modification Behaviour

### 5.10.1	Description

The MnS Consumer requires clarity on the conditions and procedures for modifying an existing NDTJob instance. At the moment, the specification leaves this open to interpretation, even though there are clear limitations associated with different implementation techniques for NDTJobs. For instance, if an NDTJob is implemented as a script that performs calculations based on input from the MnS Consumer and the replicated network, modifying such a script during execution would effectively mean rerunning the script with new parameters as resumption of certain scripts would be impossible. On the other hand, if the implementation is utilising a technology which can enable modification during runtime, then modification during the initial stages—or at specific points within the runtime—may still be possible.

Currently, the specification does not allow for clear modelling of the different potential stages executed during an NDTJob. As a result, modification of an NDTJob will be inconsistent across implementations, making MnS Consumer integration bespoke to specific NDTs.

The current specification does not define a procedure or consequences of the modification of an NDTJob. TS 28.561 [3] Clause 4.4 (“NDT life-cycle management”) states that “the NDT job instance can be configured by the MnS Consumer at any time”, yet no guidance is provided on permissible modification timing, allowable attribute changes, or the expected system behaviour when an NDTJob is already under execution.

This ambiguity raises questions such as:

- When, during the NDTJob lifecycle, modification requests are acceptable.

- Whether modification during execution affects job consistency or results.

- How the MnS Producer should report modification status or failures.

A clearer model describing NDTJob state transitions and management conditions would help ensure consistent interpretation across implementations. Such a model could make it explicit when an NDTJob is in a state that permits modification and when it is not.

### 5.10.2	Potential requirements

REQ-NDTST-01: The 3GPP management system should support a capability to enable an authorized MnS consumer to query the current state of NDTJob.

REQ-NDTST-01: The 3GPP management system should support a capability to allow an authorized MnS consumer to to update the NDTJob without effecting the current job consistency and/or results.

### 5.10.3	Potential solutions

The solution requires defining the clear NDT state transition diagram indicating various stages, and the transition between them, of NDT job during its execution. The following stages in NDT state transition are defined:

- Acknowledged: The NDTJob is accepted and checked for its validity. The consumer sends “createMOI (NDTJob)” request to the producer. Producer checks for the validity and create the NDTJob MOI i.e instantiate the NDTJob IOC.

- In-Execution: The NDTJob execution is started. The consumer send “modifyMOIAttributes” request for nDTAdminState attribute setting its value to “ACTIVATED”. This requires introducing an attribute called “nDTAdminState” in NDTJob IOC as follows:

| Attribute Name | Description | Attribute Properties |
| --- | --- | --- |
| nDTAdminState | It describes the NDTJob administrative state, which enables: MnS Producer to indicate the acceptance of an NDTJob; MnS consumer to suspend an NDTJob; MnS consumer to cancel the suspension for a suspended NDTJob; A suspended NDTJob means this NDTJob is not considered for execution. allowedValues: “ACCEPTED”, "ACTIVATED", "DEACTIVATED" OR. allowedValues: “ACKNOWLEDGED”, "SUSPENDED", "RESUMED" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "ACCEPTED" OR “ACKNOWLEDGED” isNullable: False |

- Suspended: The NDTjob is temporarily suspended. The consumer send “modifyMOIAttributes” request for nDTAdminState attribute setting its value to “DEACTIVATED”.

- Updated: The properties of the NDTJob is updated. The consumer send “modifyMOIAttributes” request for various attributes of NDTJob MOI. The transition to “Update” state from “In-Execution” and “Reported” state is not allowed. A request to update the NDTJob attributes, while the NDTJob is in “In-Execution” or in “Reported” state, shall be rejected by the producer.

- Reported: The NDTJob has resulted in NDTReport(s). After every simulation the producer creates the report i.e instantiate the NDTReport IOC. In other words, create NDTReport MOI.

- Completed: The NDTJob has resulted in all the expected NDTReport(s). All simulation has ended in the NDTJob.

- Terminated: The NDTJob is deleted. The MnS Consumer send deleteMOI (NDTJob) request to the producer.

Depending on the above stages, the following state transition diagram is proposed:

Figure 1: NDT State Transition

| Transition number | The state transition events | State |
| --- | --- | --- |
| 1 | The MnS producer creates the NDTJob instances based on the received NDTJob creation request. | ACKNOWLEDGED |
| 2 | The MnS producer receives a request to execute the NDTJob | IN-EXECUTION |
| 2a | The MnS producer generates the report for the simulation(s) | REPORTED |
| 3 | The MnS producer completes the execution of NDTJob and generates the NDT Report | COMPLETED |
| 4 | The MnS producer deletes NDTJob based on the received NDTJob deletion request. | TERMINATED |
| 5 | MnS producer configures attribute nDTJobAdminState with the value "SUSPENDED" | SUSPENDED |
| 5a | MnS producer configures attribute nDTJobAdminState with the value "SUSPENDED" | SUSPENDED |
| 6 | MnS producer configures attribute intentAdminState with the value "RESUMED" | IN-EXECUTION |
| 7 | The MnS producer deletes NDTJob based on the received NDTJob deletion request. | TERMINATED |
| 8 | The MnS producer modifies the NDTJob instances based on the received NDTJob modification request. Note: the NDTJob cannot be updated when NDTJob instance is in IN-EXECUTION or REPORTED state. | UPDATED |
| 9 | The MnS producer receives a request to execute the NDTJob | IN-EXECUTION |
| 10 | The MnS producer modifies the NDTJob instances based on the received NDTJob modification request. | UPDATED |
| 11 | The MnS producer deletes NDTJob based on the received NDTJob deletion request. | TERMINATED |
| 12 | The MnS producer modifies the NDTJob instances based on the received NDTJob modification request. | UPDATED |

### 5.10.4	Evaluation of potential solutions

The solution proposed a clear state transition diagram for an NDTJob. This clarifies when/if a NDTJob can be updated and hence fulfilling the requirements. The solution proposed is feasible and should be considered during the normative phase.

* * * Next Change * * * *

## 5.11	Use Case #11: Create and Execute NDT Job

### 5.11.1	Description

The MnS Consumer requires clarity on whether the creation and execution of an NDTJob are distinct phases or a single combined procedure.

By having creation of an NDTJob separate to the execution of the NDTJob, it means certain MnS Consumers can define the NDTJob, and other MnS Consumers could execute the NDT jobs. This is beneficial to the MnS Consumers as users with different roles can create the NDTJob such as subject matter experts, then a different set of users can execute the simulation, which creates a clear separation of responsibilities.

In the current specification, the relationship between these phases is inconsistent across clauses:

- Clause 4.4 (NDT Life-cycle Management) describes “NDT job instantiation” as including both creation and execution within a single step, suggesting automatic execution upon creation.

- Clause 6.4 (Procedures for NDT operations) depicts separate interactions between the MnS Consumer and MnS Producer for creation and execution, implying that execution follows a distinct request.

- Clause 7.1 (RESTful HTTP-based solution set) defines only the createMOI operation for NDTJob creation, with no corresponding operation for initiating execution.

This discrepancy makes it unclear how an MnS Consumer can prepare an NDTJob configuration and decide when to initiate its execution.

### 5.11.2	Potential requirements

REQ-NDTSR-01: The 3GPP management system should support a capability to allow an authorized MnS consumer to request for the execution of an accepted NDTJob.

### 5.11.3	Potential solutions

Please see solution in clause 5.10.3, that defines NDTJob transition stages. In addition, the solution here requires:

The functionality of starting the execution of an accepted/acknowledge NDTJob is provided. The following attribute is proposed to be added in NDTJob IOC.

| Attribute Name | Description | Attribute Properties |
| --- | --- | --- |
| toBeExecuted | It describes the intent of the MnS Consumer to start the NDT execution. The MnS Conumer can update the value of this attribute to request for the NDT execution. | type: Boolen multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |

### 5.11.4	Evaluation of potential solutions

The solution proposed fulfils all the requirements. The solution proposed is feasible and should be considered during the normative phase.

* * * Next Change * * * *

## 5.12	Use Case #12: Clarification of Suspension and Resumption Capabilities for NDTJobs

### 5.12.1	Description

The MnS Consumer requires clarity on whether an NDTJob can be suspended and subsequently resumed, when such capabilities are supported by the NDT MnS Producer.

The MnS Consumer should have the capability to suspend NDTJobs such as an optimization activity in order to free up resources to prioritize different NDTJobs such as those surrounding faults, otherwise the NDT MnS Producer may not have the available capacity to execute multiple NDTJobs in parallel.

The current specification references the suspension and resumption of NDTJobs in multiple clauses but provides no corresponding procedures, state descriptions, or information model attributes. This creates ambiguity regarding:

- Whether suspension and resumption are supported capabilities or illustrative examples;

- How the MnS Consumer is expected to request suspension or resumption;

- What the expected lifecycle state transitions are following suspension or resumption;

- How results or reports are handled when an NDTJob is suspended.

The capability to suspend and resume NDTJobs should remain optional, dependent on MnS Producer implementation. However, explicit clarification in the specification would help ensure consistent behaviour and interface expectations across implementations.

### 5.12.2	Potential requirements

REQ-NDTSR-01: The 3GPP management system should support a capability to allow an authorized MnS consumer to suspend and resume the existing NDTJob.

### 5.12.3	Potential solutions

Please see solution in clause 5.10.3, that defines when an NDTJob can be suspended and resumed. In addition, the solution here requires:

The following attribute for NDTJob IOC, indicating the current lifecycle state of the NDTJob. The value of this attribute is updated by the MnS Producer to reflect the corresponding state of the NDTJob. The consumer is not authorized to update this attribute.

| Attribute Name | Description | Attribute Properties |
| --- | --- | --- |
| nDTLifecycleState | It describes the NDTJob lifecycle state. allowedValues: “ACKNOWLEDGE”, "IN-EXECUTION", "UPDATED", “SUSPENDED”, “REPORTED”, “COMPLETED”, “TERMINATED” | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

### 5.12.4	Evaluation of potential solutions

The solution proposed fulfils all the requirements. The solution proposed is feasible and should be considered during the normative phase.

* * * End of Changes * * * *