3GPP TSG SA5 Meeting #165	S5-260330

Goa, India, 9-13 February 2026

Source:	Nokia

Title:	Pseudo-CR on Add potential solution and evaluation on access control on notifications

Document for:	Approval

Agenda item:	6.20.9

Spec:	3GPP TR 28.888

Version:	0.2.0

Work Item:	FS_EnExpo

Comments

<Proposals, reason for change, abstract, comments if necessary (optional)>

Proposed Changes

* * * First Change * * * *

### 5.2.3	Potential solutions

Editor's note:	This clause provides one or more solutions. Further (sub-)clause(s) may be added to capture details.

#### 5.2.3.X Access control on notifications

##### 5.2.3.X.1 Configuration of access control on notifications

In this proposed solution, a new NotificationAccessRule is proposed as another access rule type to be added to the class diagram in clause 7.2 of of TS 28.319[4].  The NotificationAccessRule provides access control on a specific notification type or all notification types. As illustrated in Figure 5.2.3.X.1-1, each NotificationAccessRule is associated to zero, one or more Role(s).

In the NotificationAccessRule, the notificationType attribute represents the notification type event for which the access rule applies, whereas the ruleName, operations, and actions attribute definitions are as defined in clause 7.4.1 of TS 28.319[4]. Accordingly, for an MnS consumer to be able to receive a given notification type event from the MnS producer, the Role associated to the MnS consumer should be configured with the corresponding NotificationAccessRule and the “read” operation of the NotificationAccessRule should be permitted (i.e., by setting the value of the action attribute to “ALLOW”). Also, for an MnS consumer to receive notifications from one or more managed object instance (represented by the “dataNodeSelector” attribute of the AccessRule), the Role associated to the MnS consumer needs to be configured to have “read” operation permitted on those managed object instances.

Figure 5.2.3.X.1-1: Classes for role-based access control including the notification access rule (See Figure 7.2-1 of TS 28.319[4])

The following definition for the notificationType attribute for the NotificationAccessRule is provided:

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| notificationType | It describes the notification type that the NotificationAccessRule applies to. allowedValues: - notifyMOICreation - notifyMOIDeletion - notifyMOIAttributeValueChanges - notifyMOIChanges - notifyEvent - notifyNewAlarm - notifyAckStateChanged - notifyComments - notifyCorrelatedNotificationChanged - notifyChangedAlarmGeneral - notifyClearedAlarm - notifyAlarmListRebuilt - notifyPotentialFaultyAlarmList - notifyFileReady - notifyFilePreparationError - notifyThresholdCrossing - notifyPotentialFaultyDataNodeTree -notifyDataNodeTreeSyncRecommended - * NOTE: a value of ‘*’ for this attribute means that the NotificationAccessRule applies for all notification types. | type: String multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |

##### 5.2.3.X.2 Enforcement of the access control on notifications

In the following clauses, it is explained how the proposed solution is used by the MnS producer when it comes to enforcement on notification subscriptions and reporting considering different scenarios.

###### 5.X.3.X.2.1 Only the notification recepeint address is provided in the notification subscription request

When an MnS consumer subscribes to receive notifications using the NtfSubscriptionControl IOC(see clause 4.3.22 of TS 28.622[6])  by providing only the notificationRecipientAddress attribute, the MnS producer evaluates the subscription request by checking if the Role(s)associated to the MnS consumer has any NotificationAccessRule(s) defined. If there are existing NotificationAccessRule(s) defined, the MnS producer responds with a “201 created” status code and the notification subscription request succeeds. However, if no NotificationAccessRule(s) are defined for the Role(s) associated to the MnS consumer, then the notification subcription request is denied.

Subsequently, when a notification event occurs, the MnS producer checks the following access rules sequentially for a given Role:

- If the managed object instance that produced the notification is part of the scope represented by the “dataNodeSelector” attribute of the AcessRule and the “read” operation is permitted for this managed object instance.

- If the notification event that has occured is part of the configured “notificationType” attribute of the NotifiicationAccessRule(s).

If statements (i) and (ii) are true, then the notification event will be sent out to all MnS consumer(s) (associated with given Role(s)) that subscribed to receive the given notification types, however if either statement is false, no notification will be sent out.

###### 5.X.3.X.2.2 The notification recepeint address and scope or notification types are provided in the notification subscription request

When an MnS consumer subscribes to receive notifications using the NtfSubscriptionControl IOC (see clause 4.3.22 of TS 28.622) by providing the notificationRecipientAddress and notificationTypes or scope attributes, similar to clause 5.X.3.X.2.1, the MnS producer evaluates the subscription request by checking if the Role(s)associated to the MnS consumer has any NotificationAccessRule(s) defined. If no NotificationAccessRule(s) are defined for the Role(s) associated to the MnS consumer, then the notification subcription request is denied. However, if NotificationAccessRule(s) are defined, the MnS producer further checks the following:

- If the scope in the notification subscription request is a subset or corresponds to the “allowed scope” represented by the “dataNodeSelector” attribute of the AcessRule class for the Role(s)associated to a given MnS consumer.

- For the notificationType(s) in the notification subscription request by the MnS consumer, the MnS producer checks if there is a NotificationAccessRule(s) defined for each notificationType(s) for the Role(s) associated to the MnS consumer.

If statements (i) and (ii) are true, then the MnS producer evaluates the subscription request and responds with a “201 created” status code. And once a notification event occurs, the MnS producer behaves as described in clause 5.X.3.X.2.1.

However, if statement (i) or (ii) is false which means:

- either the MnS consumer is requesting to receive notifications on a scope different or wider than the “allowed scope” represented by the “dataNodeSelector” attribute of the AcessRule class or

- the MnS consumer is requesting to receive a notification type(s) than the configured “notification types” represented by the “NotificationType” attribute of the  NotificationAccessRule(s) associated with the Role(s) of the MnS consumer.

Then, in this case, two different behaviours of the MnS producer are proposed, i.e., the strict scope and partial scope enforcement models:

- Strict scope enforcement model: here the MnS producer responds with a “403 FORBIDDEN” error status code with the appropriate reason indicating that the notification subscription request wasn’t successful. In this case, the error should be “Unauthorized Scope” with the following reason: "The requested subscription scope exceeds the MnS consumer’s authorized access. Partial subscriptions are not supported." Accordingly, in the strict scope enforcement model, the MnS consumer is not allowed to subscribe to receive notification types beyond the configured notificationType(s) authorized for the Role(s) associated to the MnS consumer.

- Partial scope enforcement model: here the MnS producer still responds with a “201 CREATED” status code indicating that the notification subscription request has been successfully created. Optionally, the MnS producer might indicate to the MnS consumer that the subscription is accepted but the notifications will be delivered only for the authorized subset of the requested scope or notification types.

NOTE 1: The proposed solution assumes that if the MnS consumer is authorized to receive a given notification type event, then the MnS consumer is authorized to receive all the data contained within the notification.  Access control definitions on the contents within the notifications is out-of-scope of the present document.

NOTE 2:  The proposed solution assumes that the MnS producer’s responsibility is limited to ensuring that the MnS consumer is authorized to subscribe and receive given notification type(s). So, it’s upto the MnS consumer to ensure that the entity that receives the notifications (identified by the notificationRecipientAddress attribute of the NtfSubscriptionControl IOC) is authorized to receive such notification type(s).

* * * Next Change * * * *

### 5.2.4	Evaluation of potential solutions

Editor's note:	This clause provides evaluation of potential solutions.

The proposed solution extends the existing access control model defined in TS 28.319[4] by adding a new access control rule for notifications called NotificationAccessRule. Further, the proposed solution specifies how the defined NotificationAccessRule(s) can be used by the MnS producer to enforce notification subscriptions and reporting. The proposed solution satisifies the potential requirements in clause 5.2.2.

* * * End of Changes * * * *