3GPP TSG SA5 Meeting #165	S5-260360

Goa, India, 9-13 February 2026

Source:	Vodafone

Title:	Pseudo-CR on potential solutions for use case #1

Document for:	Approval

Agenda item:	6.20.15

Spec:	3GPP TS 28.892

Version:	v0.1.0

Work Item:	FS_UMMR_OAM

Comments

Proposed Changes

* * * First Change * * * *

### 5.1.4	Potential solutions

#### 5.1.4.1	Solution #1: Introduction of a new standalone specification for SBMA-based management support of 4G nodes

##### 5.1.4.1.1		Description

This solution considers the introduction of a new dedicated 3GPP specification defining SBMA-based configuration management support for 4G nodes. The specification would include the necessary SBMA-related definitions, such as YANG and OpenAPI based representations and the exposure of corresponding Management Services, while referencing the existing IRP-based NRMs defined in legacy 4G specifications.

Under this approach, existing IRP specifications and current 5G management specifications would remain unchanged, and the new specification would provide SBMA-based access to 4G configuration management information.

This solution addresses the prerequisites defined in REQ-UMMR_CoMgt-01, REQ-UMMR_CoMgt-02 and REQ-UMMR_CoMgt-03.

##### 5.1.4.1.2	Advantages

- Clear separation between legacy IRP-based management and SBMA-based management.

- No modifications required to existing IRP or 5G management specifications.

- Preserves backward compatibility with existing IRP-based deployments.

- Enables SBMA-based co-management of 4G and 5G nodes.

##### 5.1.4.1.3	Disadvantages

- Risk of partial duplication of concepts already defined in 5G SBMA specifications.

- Requires careful alignment with existing 5G SBMA models to avoid inconsistencies.

- Introduces an additional specification that must be maintained independently.

#### 5.1.4.2	Solution #2: Extension of existing 5G management specifications with SBMA-based support for 4G nodes

##### 5.1.4.2.1		Description

This solution considers extending existing 5G management specifications, such as 3GPP TS 28.540 [3] and 3GPP TS 28.541 [4], to include SBMA-based configuration management definitions for 4G nodes.

By introducing the missing SBMA representations for 4G NRMs within the existing 5G specification framework, this approach enables SBMA-based exposure of configuration management information for both 4G and 5G nodes. However, it results in increased specification complexity and introduces management definitions for multiple RAT generations within the same specifications.

This solution addresses the prerequisites defined in REQ-UMMR_CoMgt-01, REQ-UMMR_CoMgt-02 and REQ-UMMR_CoMgt-03.

##### 5.1.4.2.2	Advantages

- Provides a single specification framework for SBMA-based management across multiple RAT generations.

- Enables direct reuse of existing SBMA mechanisms without additional specifications.

- Simplifies reference handling for implementers using a single specification set.

##### 5.1.4.2.3	Disadvantages

- Increases the complexity and size of already extensive 5G management specifications.

- Makes the specifications more difficult to read, navigate and maintain.

- Mixes management definitions for different RAT generations, potentially reducing clarity.

- Increases the risk of unintended side effects when evolving or extending 5G management features.

#### 5.1.4.3	Solution #3: Extension of existing IRP specifications with SBMA-based representations

##### 5.1.4.3.1		Description

This solution considers extending existing IRP-based management specifications to include SBMA-based representations of 4G NRMs. YANG and OpenAPI based definitions would be introduced alongside the existing IRP Information Services and Solution Sets within the same specifications, enabling SBMA-based access to 4G configuration management information.

This solution addresses the prerequisites defined in REQ-UMMR_CoMgt-01, REQ-UMMR_CoMgt-02 and REQ-UMMR_CoMgt-03.

##### 5.1.4.3.2	Advantages

- Preserves close alignment with existing 4G IRP specifications.

- Maintains backward compatibility with current IRP-based management deployments.

- Avoids the need for new standalone specifications.

##### 5.1.4.3.3	Disadvantages

- Introduces multiple management paradigms (IRP-based and SBMA-based) within the same specifications.

- Further increases the complexity of IRP specifications, which already include management definitions spanning multiple generations (e.g. 3G and 4G).

- Blurs architectural boundaries between legacy and service-based management frameworks.

- Requires changes across multiple legacy specifications, increasing editorial effort and coordination overhead.

- Distributes SBMA-related content across several specifications, complicating implementation and tracking.

#### 5.1.4.4	Solution #4: Introduction of a delta specification aligned with 3GPP TS 28.541

##### 5.1.4.4.1		Description

This solution considers the introduction of a new 3GPP specification that provides SBMA-based configuration management support for 4G nodes as a delta to the existing 5G SBMA framework defined in 3GPP TS 28.540 [3] and 3GPP TS 28.541 [4].

The new specification would not duplicate existing SBMA definitions but would instead reference 3GPP TS 28.541 [4] and define only the additional elements required to support 4G NRMs, such as the missing YANG and OpenAPI based representations and related Management Services. Existing specifications could include references to this new specification.

In this approach, the new specification could assume that the relevant management requirements are already captured in 3GPP TS 28.540 [3] and would therefore focus on the SBMA-based realization of configuration management for 4G nodes. The specification would leverage the existing IRP Information Services as the authoritative source of 4G management semantics, enabling a systematic derivation of YANG and OpenAPI based representations aligned with the SBMA framework. This allows reuse of established information models while ensuring consistency with existing SBMA definitions.

This solution addresses the prerequisites defined in REQ-UMMR_CoMgt-01, REQ-UMMR_CoMgt-02 and REQ-UMMR_CoMgt-03.

##### 5.1.4.4.2	Advantages

- Maintains a clean separation between 4G and 5G management specifications.

- Avoids duplication of SBMA concepts already defined in 3GPP TS 28.541 [4].

- Provides a clear and modular extension path aligned with the existing SBMA framework.

- Limits the impact on existing IRP and 5G management specifications.

- Improves readability, maintainability and long-term evolution of the specification set.

##### 5.1.4.4.3	Disadvantages

- Introduces an additional 3GPP specification.

- Requires clear definition of dependencies and normative references between specifications.

- Implementation guidelines must ensure consistent interpretation across specifications.

* * * End of Changes * * * *