3GPP TSG SA5 Meeting #165	S5-260448

Goa, India, 9-13 February 2026

Source:	Ericsson

Title:	Pseudo-CR on Solution topic 2 issue 1 using SBA

Document for:	Approval

Agenda item:	7.5.2

Spec:	3GPP TR 32.872

Version:	0.2.0

Work Item:	FS_RoamRE_CH

Comments

Addition of using service-based architecture for H-SMF (CTF) detected H-CHF failure solution.

Proposed Changes

* * * First Change * * * *

#### 5.1.4.x	Solution #2.x: Using service-based architecture for H-SMF (CTF) detected H-CHF failure

A possible solution for key issue #1 covering requirements REQ-3GPPCH-LBHR-01, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF.

When the H-SMF (CTF) detects a failure, it may use the procedure outlined in TS 29.500 [x] clause 6.5.3.3, to restore a connection to the H-CHF. If the restoring of the connection fails it is operator specific if the PDU session is allowed to continue or not.

* * * End of Changes * * * *