3GPP TSG-SA5 Meeting #165	S5-260458

Goa, India, 9 - 13 February 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.315 | CR | 0003 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | - x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.315 Add procedure for WAB-node connecting to management system |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AdNRM_Ph4-OAM |  |  |  |  |  | Date: |  |  | 2026-01-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A mobile NR node can implement different network features, including IAB and WAB. When IAB, it becomes an IAB-node. When WAB, it becomes a WAB-node. The management of a mobile NR node includes the node connectivity to management system (specified in TS 28.314/28.315) and node configuration (specified in TS 28.531). In TS 28.315, the mobile NR node connecting to management system only covers IAB feature; WAB feature is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure for WAB-node connecting to management system, so both IAB and WAB features are covered. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure of node-OAM connectivity for WAB feature is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6, A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 5.6	Mobile NR node connects to management system

Mobile NR node connects to management system at power up and during mobility following PnC steps as described in clause 6.1.2 of TS 28.314 [2]. The management system that mobile NR node connects to may change due to node mobility behaviours. The procedures for mobile NR node connecting to management system based on its locations are illustrated in Figure 5.6.1, this includes step 2 of obtaining IP configuration via notification mechanism instead of initial IP autoconfiguration procedure.

In step 1, mobile NR node has established a connection with its management system via PnC

In step 2, it reuses the existing notifications and operations as defined in TS 28.532 [4] to obtain IP configuration for OAM connectivity:

The provisioning MnS producer of mobile NR node should provide node’s location information via any of the following notifications:

- notifyMOICreation

- notifyMOIAttributeValueChanges

- notififyMOIChanges

The provisioning MnS consumer (management system) should provide IP configuration for OAM connectivity via any of the following operations upon the reception of notifications:

- createMOI

- modifyMOIAttributes

In step 3, mobile NR node connects to new management system by following subsequent PnC steps including connection to CA/RA, SeGW and SCS.

Figure 5.6.1: Procedure flows for mobile NR node connecting to management system

Next Change

# A.6	Mobile NR node connects to management system

The following PlantUML source code is used to describe the procedure flows for a mobile NR node connectingto management system, as depicted by Figure 5.6.1:

@startuml

actor “Mobile NR node” as MRN

participant "management system 1" as OAM_1

participant "management system 2" as OAM_2

Ref over MRN, OAM_1: 1. Precondition: Mobile NR node has established connection with its management system (1) via PnC

Ref over MRN, OAM_1: 2. Obtain IP configuration for new management system via notifications based on the mobile NR node's location

Ref over MRN, OAM_2: 3. Connect to new management system (2) by following subsequent PnC procedures including connection to CA/RA, SeGW and SCS

hide footbox

skinparam DefaultFontSize 11

skinparam sequenceActorBackgroundColor #FFFFFF

skinparam sequenceParticipantBackgroundColor #FFFFFF

skinparam noteBackgroundColor #FFFFFF

autonumber "#'.'"

skinparam monochrome true

skinparam shadowing false

@enduml

End of Changes