3GPP TSG-SA5 Meeting #165	S5-260461

# Goa, India, 09 – 13 February 2026

Source:	Ericsson, NEC

Title:	Pseudo-CR TR 28.882 Rapporteur cleanup

Document for:	Approval

Agenda item:	6.20.2

Spec:	3GPP TR 28.882

Version:	0.2.0

Work Item:	FS_AIML_MGT_Ph3

Comments

This cleanup proposes making editorial changes and moving “Enhancement on LCM of Federated Learning” from clause 5.1 (ML model training) to clause 6.1 (Sustainability for ML training). The reason is that this feature operates under the tenets of the sustainable AI/ML framework defined in clause 6.0.

Proposed Changes

* * * First change * * * *

Contents

3	Definitions of terms, symbols and abbreviations	3

3.1	Terms	3

3.2	Symbols	3

3.3	Abbreviations	3

5	Management capabilities for AI/ML lifecycle	4

5.1	ML model training	4

5.1.1	Use cases	4

5.1.1.1	Management support to training for UE-side model	4

5.1.1.1.1	Management support to AI/ML-based beam management	4

5.1.1.2	Management support to OAM-centric training for NG-RAN NW-side model	5

5.1.1.3	Management of Vertical Federated Learning	6

5.1.1.4	 Management support to data collection for two-sided model training	7

5.1.1.5	Enhanced RL training with performance targets	9

5.2	ML model testing	9

5.3	AI/ML inference emulation	9

5.4	ML model deployment	9

5.4.1	ML model transfer delivery to UE	9

5.4.1.1	Description	9

5.4.1.2	Potential requirements	10

5.4.1.3	Possible solutions	10

5.4.1.4	Possible solutions evaluation	10

5.5	AI/ML inference	10

6	AI/ML sustainability	11

6.0	Sustainable aspects of ML model training and inference	11

6.0.1	Description	11

6.0.2	Potential requirements	11

6.0.3	Possible solutions	11

6.0.3.1	Update Energy Information NRM fragment	12

6.0.3.2	Update MLTrainingFunction	12

6.0.4	Possible solutions evaluation	12

6.1	Sustainability for ML training	12

6.1.1	Enhancement of LCM of Federated Learning	12

6.1.1.1	Description	12

6.1.1.2	Potential requirements	13

6.1.1.3	Possible solutions	13

6.1.1.4	Possible solutions evaluation	13

6.2	Sustainability for AI/ML inference	13

7	Registration and discovery for AI/ML	13

8	Relation with other management capabilities	13

9	Conclusion and recommendations	13

* * * Next Change * * * *

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

OTT	Over the top

CSI	Channel State Information

VFL	Vertical Federated Learning

* * * Next change * * * *

# 5	Management capabilities for AI/ML lifecycle

## 5.1	ML model training

### 5.1.1	Use cases

#### 5.1.1.1	Management support to training for UE-side model

#### 5.1.1.1.1	Management support to AI/ML-based beam management

##### 5.1.1.1.1.1	Description

To support AI/ML-based beam management defined in TS 38.300 [2], for beam prediction management, UE can send the data of beam prediction management to UE-side training entity (e.g. a server inside MNO or an OTT server) via gNB and 3GPP management system for UE-side model training (see NOTE 1). 3GPP management system needs to collect data from gNB and report it to UE-side training entity to management support of training for UE-side model.

Figure 5.1.1.1.1.1-1: Management of UE-side data collection and reporting for UE-side model training

Being a MnS consumer, the UE-side training entity requests 3GPP management system to collect the UE-side data of beam prediction management, then 3GPP management system configures the gNB(s) to collect data from specified UEs, and gNB shall send configuration information to the UE(s). The request from UE-side training entity shall include the type of data (e.g. based on UE/UE Group, location, time) to be produced and the gNB(s) where the data shall be collected.

After the UE-side training data such as beam prediction data are produced, UE(s) shall report the UE-side data to gNB(s), and 3GPP management system shall collect data from the gNB(s). 3GPP management system reports the collected data to UE-side training entity depending on access information specified by operator (see NOTE 2).

NOTE 1:	The UE-side data are subject to further discussion, pending ongoing correspondence and confirmation by RAN2 and RAN1.

NOTE 2:	Access control information is for further discussion.

##### 5.1.1.1.1.2	Potential requirements

REQ-ML_UESIDE-01: The 3GPP management system should have a capability allowing the authorized UE-side training entity to request the UE-side training data for the UE-side model training.

REQ-ML_UESIDE-02: The 3GPP management system should have a capability to request and get the UE-side training data from gNB(s) for the UE-side model training.

REQ-ML_UESIDE-03: The 3GPP management system should have a capability to report the UE-side data to authorized UE-side training entity for UE-side model training.

##### 5.1.1.1.1.3	Possible solutions

TBD

##### 5.1.1.1.1.4	Possible solutions evaluation

TBD

#### 5.1.1.2	Management support to OAM-centric training for NG-RAN NW-side model

##### 5.1.1.2.1	Management support to AI/ML-based beam management

##### 5.1.1.2.1.1	Description

To support AI/ML-based beam management defined in TS 38.300 [2], UE can provide data of beam management to 3GPP management system via gNB (see NOTE 1). 3GPP management system needs to collect data from gNB for OAM-centric training for NG-RAN NW-side model.

Figure 5.1.1.2.1.1-1: Management of NW-side data collection and reporting for NW-side model training

The 3GPP management system requests gNB(s) to collect the NW-side training data of beam management and provides the configuration to gNB(s), then gNB(s) shall send configuration information to the UE(s) for NW-side training data collection. After the NW-side training data are produced, UE(s) shall report the NW-side training data to gNB(s), and 3GPP management system shall collect data from the gNB(s). 3GPP management system collect data should depend on access control information specified by operator (see NOTE 2).

NOTE 1:	The OAM centric NW-side training data are subject to further discussion, pending ongoing correspondence and confirmation by RAN2.

NOTE 2:	Access control information is for further discussion.

Editor’s notes:	The need for user consent is pending ongoing correspondence and confirmation by SA3 and RAN2.

##### 5.1.1.2.1.2	Potential requirements

REQ-ML_NWSIDE-01: The 3GPP management system should have a capability to configure the NW-side training data collection for OAM-centric NW-side model training.

REQ-ML_NWSIDE-02: The 3GPP management system should have a capability to obtain the NW-side training data for OAM-centric NW-side model training.

##### 5.1.1.2.1.3	Possible solutions

TBD

##### 5.1.1.2.1.4	Possible solutions evaluation

TBD

#### 5.1.1.3	Management of Vertical Federated Learning

##### 5.1.1.3.1	Description

In Rel-19, Vertical Federated Learning (VFL) is introduced in core network for NWDAF(s) and AF(s). There may be one NWDAF or one AF acting as a VFL server, and one or multiple NWDAF(s) and/or one or multiple AF(s) acting as VFL Client(s). Vertical Federated Learning is available among NWDAFs or between NWDAF(s) and AF(s) within a single PLMN or between an AF and NWDAF(s) in a single PLMN. When AF is acting as VFL Server, NWDAF(s) is VFL Client(s).

In Rel-19, federated learning is introduced in the 3GPP management system for multiple ML training functions. The MnS Consumer can specify the FL Requirements in ML Training Request for FL process, where the  FLClientSelectionCriteria including the consumer’s requirements on sample (e.g., minimumAvailableDataSamples) for client selection. However, current mechanism doesn’t allow the consumer to specify requirements on feature for client selection in VFL process. As HFL and VFL are two distinct paradigms within the broader FL framework , in a VFL process, participants possess different feature space for potentially overlapping samples, which is ideal and beneficial to be introduced in management system. Since the MLTFunctions within the operator domain may be located in cross-domain management system, RAN domain management system and Core domain management system, available datasets for these MLTFunctions may share common samples but offer complementary features. The sample alignment is needed to ensure that different VFL participators share the same sample space before initiating VFL process.

The model training approach for HFL and VFL is also different. Training method allows each VFL Client owning its own local model but not needing to share the same model architectures, since different feature space in VFL Clients may lead to different structures for local models. But the intermediate information of the interaction is implementation specific. The VFL training architecture is fully applicable to the characteristics of management system and can fully exploit the potential of data in management system. This use case proposes to support management of VFL leveraging sample alignment among the entities participating in VFL process.

NOTE 1:  	The technical details of how the sample alignment is performed, including specific algorithms or mechanisms for matching or encrypting sample identifiers, are implementation-specific  and outside the scope of 3GPP SA5.

NOTE 2: The management coordination in VFL process is only applicable within the operator domain.

##### 5.1.1.3.2	Potential requirements

REQ-VFL_MGMT-01: The ML training MnS producer should have a capability allowing an authorized consumer to get the VFL role (VFL server or VFL client) of an ML Training Function in VFL process.

REQ-VFL_MGMT-02: The ML training MnS producer should have a capability allowing an authorized consumer to specify requirements on sample alignment for client selection in VFL.

##### 5.1.1.3.3	Possible solutions

To support VFL in 3GPP management system, the following enhancements are proposed:

- Enhancements Aspects #1, extending the MLTrainingFunction IOC with the following aspects:

1) Extend learningTechnologyName by changing the allowed value “FL” to “VFL” and “HFL” to differentiate different FL training types supported by the ML Training Function.

2) Extend FLParticipationInfo, by adding a new attribute FLType to indicate the applied FL training types, i.e., HFL or VFL.

##### 5.1.1.3.4	Possible solutions evaluation

#### 5.1.1.4		Management support to data collection for two-sided model training

##### 5.1.1.4.1	Management support to CSI compression

##### 5.1.1.4.1.1	Description

To support Channel State Information (CSI) compression defined in TR 38.843 [3], the operator can deliver relevant data for two-sided model training (see NOTE 1) to a UE-side model training entity (e.g. a server deployed by an MNO or by an OTT service provider). The UE-side model training entity uses the received data to perform UE-part model training for CSI compression.

For this use case, the following approach is considered: gNB -> OAM -> UE-side training entity (a server inside MNO or an OTT server), where the gNB is the data-collection entity for relevant data for two-sided model training.

The operator uses the 3GPP management system to control and supervise how relevant data for CSI compression is delivered to the UE-side training entity (see NOTE 2).

Figure 5.1.1.4.1.1-1 Illustration of data collection for two-sided CSI model training

The UE-side training entity sends a subscription request (see NOTE 3) to the 3GPP management system, expressing its interest to receive the data for CSI compression. The UE-side training entity can also include some conditions related to locations (e.g., geographic area) or time (e.g., time windows) for when it expects this data. The 3GPP management system registers this request, which includes the identifier of the UE-side training entity. If the subscribed data is not available at the 3GPP management system, the 3GPP management system then decides from which gNB(s) this data needs to be collected and proceeds with their configuration. Each selected gNB (see NOTE 4) is configured with information that specifies what data this gNB needs to produce for CSI compression, and how this data when available needs to be reported to the 3GPP management system (see NOTE 5). Based on this configuration and the network status, the gNB reports the requested data to the 3GPP management system.

Upon collecting the data from the different gNBs, the management system delivers it to the UE-side training entity based on access control information specified by the operator (see NOTE 2).

NOTE 1:	The relevant data for two-sided model (see arrows in Figure 5.1.1.4.1.1-1) is subject to further discussion, pending ongoing correspondence and confirmation by RAN2.

NOTE 2:	Access control information is for further discussion.

NOTE 3:	The term ‘subscription request’ does not specifically imply adoption of a subscribe/notify mechanism in the solution. It is used to capture the requirement that the UE-side training entity can declare its interest in data delivery.

NOTE 4:	The selected gNB(s) support the AI/ML feature for CSI compression use case.

NOTE 5:	The UE-side training entity identity is managed by the operator within the 3GPP management system, whether it needs to be included in the configuration for traceability purposes is for further discussion.

##### 5.1.1.4.1.2	Potential requirements

REQ-ML_TWOSIDE-01: The 3GPP management system should have a capability allowing a UE-side training entity to subscribe for receiving relevant data for CSI compression.

REQ-ML_TWOSIDE-02: The 3GPP management system should have a capability to configure one or more gNBs to produce and report relevant data for CSI compression.

REQ-ML_TWOSIDE-03: The 3GPP management system should have a capability to deliver relevant data to a subscribed UE-side training entity.

-

#### 5.1.1.5	Enhanced RL training with performance targets

##### 5.1.1.5.1	Description

During RL training, a policy is learnt to maximize a reward aggregated over time. The reward function is defined by the producer from a set of targets set by the consumer. The targets include thresholds such as “Call drop rate(CDR) < 1%”, “Call setup success rate (CSSR) > 90%”, but also optimization instructions such as “minimize CDR”, “maximize CSSR”. Together, these are referred to as “RL performance targets”.

On the other hand, the policy is also required to ensure that the network performance does not degrade along certain other KPIs. This can be specified as a set of thresholds referred to as RL performance constraints, in which the consumer may specify its preference on the policy values that should not be violated during training, for the producer to take into consideration while performing RL training. The constraints may be necessary because otherwise, training the RL model aggressively to optimize the target KPIs may result in degradation of other KPIs.

Specifying only the performance constraints does not suffice, because in the absence of performance targets, the producer may learn only to avoid constraint violation.

In 3GPP TS 28.105 [4], only RL performance constraints are specified for online RL training through the attribute RLRequirement.rLPerformanceRequirements of the IOC MLTrainingRequest. RL performance targets are not specified.

##### 5.1.1.5.2	Potential requirements

REQ-ENH_RL_TRAINING-01: The ML training MnS producer should have a capability to allow an authorized MnS consumer to specify the RL performance targets in an ML model training request.

## 5.2	ML model testing

Editor’s note: Similar clause structure as in subclause 5.1 will be adopted.

## 5.3	AI/ML inference emulation

Editor’s note: Similar clause structure as in subclause 5.1 will be adopted.

## 5.4	ML model deployment

Editor’s note: Similar clause structure as in subclause 5.1 will be adopted.

### 5.4.1	ML model transfer delivery to UE

Editor’s note:	The content in this clause remains for further discussion and depends on further correspondence and confirmation from the relevant RAN WGs.

#### 5.4.1.1	Description

RAN1 and RAN2 started in Rel-18 a joint study on AI/ML for NR air interface (see FS_NR_AIML_air [5]) whose outcomes were reported in TR 38.843 [3], approved in RAN#102 (Dec. 2023). One of the features in scope of this study was ML model transfer/delivery to UE. The clause 7.2.1.4 of TR 38.843 [3] documented several potential solutions for this feature. One of these solutions is Solution 4b, which involves OAM. For Solution 4b, TR 38.843 [3] quotes the following:

“Note: For Solution 4b, RAN2 discussed the following two solutions but did not study or analyse their feasibility:
- OAM can transfer/deliver AI/ML models to UE via OAM→RAN→UE, where Control Plane (CP) signalling is used for RAN→UE.
- OAM can transfer/deliver AI/ML models to UE via OAM→UE, e.g., via IP tunnel.”

Based on this quote, SA5 decided to include this feature in the scope of FS_AIML_MGT_Ph2 [6], a Rel-19 study whose outcomes were reported in TR 28.858 [7]. However, after investigating the two use cases defined for solution 4b, SA5 did not draw any definite conclusions. Clause 5.3.2.5 of TR 28.858 [7] quoted the following:

“[…] further enhancements to ML model loading capabilities defined by SA5 may still be needed in the option of OAM transferring/delivering AI/ML models to UE via OAM→RAN→UE, where Control Plane (CP) signalling is used for RAN→UE and should be investigated in the normative work. Furthermore, the option of OAM transferring/delivering AI/ML models to UE via OAM →UE, e.g. via IP tunnel is FFS and needs to be investigated in SA5 once the requirements are clear from RAN working groups”.

Despite clause 6 of TR 28.858 [7] recommended SA5 to specify in Rel-19 normative phase the AI/ML management capabilities for the ML model delivery/transfer feature as defined by RAN1/2, this feature was never revisited. The reason was the lack of progress in RAN1/2 for this feature during Rel-19. On one side, RAN2 concluded their work in Rel-18 with the documentation of potential solutions in clause 7.2.1.4 of TR 38.843 [3], and did not follow up the work in Rel-19. On the other side, RAN1 was tasked to analyse on the need and feasibility of these potential solutions. After concluding this analysis, RAN1 could not reach a consensus in Rel-19 on the need of a standardized solution for the ML model delivery/transfer feature. This is captured in the RAN1#120 meeting Chair notes [8], section 9.1.4.2.

#### 5.4.1.2	Potential requirements

None.

#### 5.4.1.3	Possible solutions

SA5 should look at conclusion documented in RAN1#120 meeting Chair notes [8] section 9.1.4.2, and should not discuss solution 4b for now, given that RAN has not provided any guidance to SA5 on the need to standardize a solution for this feature.

#### 5.4.1.4	Possible solutions evaluation

With no further action required from RAN, the analysis of the two options defined for Solution 4b would remain as follows:

The option “OAM can transfer/deliver AI/ML models to UE via OAM→UE, e.g., via IP tunnel” in Solution 4b is not feasible, given that the 3GPP management system does not have a direct interface with the UE.

The option “OAM can transfer/deliver AI/ML models to UE via OAM→RAN→UE, where Control Plane (CP) signalling is used for RAN→UE”  in Solution 4b is not feasible, given that RAN2 has not introduced any CP signalling to support this in Rel-19, and that is not in scope of Rel-20 in RAN2.

## 5.5	AI/ML inference

Editor’s note: Similar clause structure as in subclause 5.1 will be adopted.

# 6	AI/ML sustainability

## 6.0	Sustainable aspects of ML model training and inference

### 6.0.1	Description

ML model training and AI/ML inference can require substantial computing power and therefore significant energy.

For sustainability goals in AI/ML management, it is valuable for the 3GPP management system to reflect the type of energy supplied to the managed entities that are used for ML model training and AI/ML inference (for example, renewable and/or carbon related information). The operator handles this energy related information (e.g., see EnergySupplyInfo in TS 28.310 [9]) in the 3GPP management system, based on knowledge of the infrastructure it manages. In particular:

- the operator obtains energy related information typically based on data from external energy suppliers; the operator sets, maintains and updates the association of energy related information to managed entities that support ML model training and inference, by configuration of the Energy Information defined in TS 28.310 [9].

- the operator defines access control rules/policies to determine what managed entities (e.g. NFs, AI/ML functions) can gain access to this information.

In the context of ML model training and AI/ML inference, the MnS producer can read the operator-configured energy related information, subject to operator-defined access control. By having access to this information, the MnS producer can support receiving training/inference requests containing energy related information. These requests are triggered by the operator, who performs the role of MnS consumer.

In the context of ML model training, the MnS producer can also expose the operator-configured energy related information to other training functions, for the case these training functions need to know this information for selection criteria. This happens for example in Federated Learning (where FL client, acting as MnS producer, exposes energy related information to other training functions acting as FL servers, for them to manage FL lifecycle, e.g. choose FL clients based on energy criteria) and Distributed Training.

The MnS producer reads operator-configured information and applies it when selecting functions for model training or inference. The accuracy of this information depend on operator configuration and external agreements, and are outside the scope of MnS producer verification.

### 6.0.2	Potential requirements

REQ-ML_SUST-01. The 3GPP management system should enable the operator to configure and update the energy related information for the managed entities representing an ML model training function.

REQ-ML_SUST-02. The 3GPP management system should expose the energy information of the managed entities representing an ML model training function to authorised MnS consumers.

REQ-ML_SUST-03. The 3GPP management system should enable the operator to configure and update the energy information for the managed entities representing AI/ML inference.

REQ-ML_SUST-04. ML Training Function should enable an MnS consumer to request for ML model training with consideration to energy related information.

REQ-ML_SUST-05. ML Training Function should be able to adapt its training to the energy related information updates, according to the conditions provided by MnS consumer which requests the ML model training.

### 6.0.3	Possible solutions

It is proposed one solution consisting of two parts: one impacting TS 28.310 [9] (see clause 6.0.3.1), and other impacting TS 28.105 [4] (see clause 6.0.3.2).

#### 6.0.3.1	Update Energy Information NRM fragment

It is proposed to update the Energy Information NRM fragment represented in 8.2.1 of TS 28.310 [9], by adding in ManagedEntity the following:

- The MLTrainingFunction IOC (see clause 7.3a.1.2.1 in TS 28.105 [4])

- The list of IOCs represented by AIMLSupportedFunction (see clause 7.3a.4.1.1 in TS 28.105 [4])

These additions require importing IOCs from 3GPP TS 28.105 [4].

#### 6.0.3.2	Update MLTrainingFunction

It is proposed to add a new attribute to MLTrainingFunction IOC: “energyInfoGroupRef”. This attribute allows specifying the DN of the energyInfoGroup MOI representing the operator-configured energy information.

### 6.0.4	Possible solutions evaluation

The solution described in clause 6.0.3 is feasible.

The part of the solution described in clause 6.0.3.1 allows satisfying REQ-ML_SUST-01 and REQ-ML_SUST-03.

The part of the solution described in clause 6.0.3.2 allows satisfying REQ-ML-SUST-02.

## 6.1	Sustainability for ML training

Editor’s note: Similar clause structure as in subclause 5.1 will be adopted.

### 6.1.1	Enhancement of LCM of Federated Learning

#### 6.1.1.1	Description

Federated learning (FL) is a distributed machine learning approach that allows multiple FL clients to collaboratively train an ML model on local datasets contained in each FL Client without explicitly exchanging data samples.

When receiving an FL training request, the ML training MnS Producer acting as FL server needs to evaluate the FL specific training requirements such as FL client selection criteria, based on which a FL server has to select appropriate FL clients. FL client selection is a crucial component of FL that directly impacts the performance, efficiency, and fairness of the learning process. However, selecting the optimal set of clients to participate in each round of FL poses several challenges. While some of the requirements are studied in 3GPP to select a FL client but the existing work is not exhaustive enough, especially when it comes to selecting FL clients based on criteria related to renewable energy usage, carbon emission etc. So, it is desirable to consider such aspects also while selecting FL clients so that a sustainable FL based approach can be maintained. Such selection of FL clients by a Producer in FL server can result in overall decreased carbon emission, and reduced non-renewable energy consumption thus promoting overall sustainable development of AI/ML in 5G systems. While FL offers advantages in data privacy, FL introduces significant energy consumption and a substantial carbon footprint from client-side computations. Without energy-aware client selection, FL can be unsustainable; with it, sustainability can be improved. Hence, there is a need to develop more environmentally friendly and sustainable federated learning mechanisms by addressing the energy consumption inherently in FL.

One of the approaches to achieve environment friendly FL is by promoting and monitoring usage of renewable energy in a FL process. This can be achieved by making design choices that lower non-renewable energy consumption, use more of renewable energy sources and reduce carbon footprint of network. Hence it is important to consider factors like FL client’s renewable energy source, renewable energy consumption and carbon emission of FL client. The approach of energy-aware FL client selection ensuring nodes acting as FL clients have considerable renewable energy usage, suitable renewable energy source and considerably lower carbon emission can practically make a FL system as a sustainable AI practice and contribute to environment which is the need of the hour in any operator’s network.

#### 6.1.1.2	Potential requirements

REQ-FL_MGMT-01: A ML training function supporting FL should enable a MnS consumer to request for training in consideration of energy information based selection criteria of FL clients.

REQ-FL_MGMT-02: The 3GPP management system should have a capability allowing an MLTrainingFunction acting as the FL Server to select FL clients based on consumer’s requirements on renewable energy source availability.

REQ-FL_MGMT-03: The 3GPP management system should have a capability allowing an MLTrainingFunction acting as the FL Server to select FL clients based on consumer’s requirements on renewable energy usage percentage.

REQ-FL_MGMT-04: The 3GPP management system should have a capability allowing an MLTrainingFunction acting as the FL Server to select FL clients based on consumer’s requirements on carbon emission of FL client.

#### 6.1.1.3	Possible solutions

It is proposed to add these criteria of renewable energy availability, renewable energy percentage and carbon emission information as optional attributes in FLClientSelectionCriteria <<dataType>> defined in clause 7.4.22.2 of TS 28.105 [4] to be able to use them as a criteria for selecting the FL clients by the FL server for collaboratively training a ML model in federated learning.

#### 6.1.1.4	Possible solutions evaluation

The solution described in clause 6.1.1.3 is feasible as it just enhances existing NRM i.e. FLClientSelectionCriteria <<dataType>> with information of renewable energy availability, renewable energy percentage and carbon emission and uses them as criteria for FL client selection in a federated learning.

## 6.2	Sustainability for AI/ML inference

Editor’s note: Similar clause structure as in subclause 5.1 will be adopted.

# 7	Registration and discovery for AI/ML

Editor’s note: Similar clause structure as in subclause 5.1 will be adopted.

# 8	Relation with other management capabilities

Editor’s note: Similar clause structure as in subclause 5.1 will be adopted.

# 9	Conclusion and recommendations

Annex A (informative): 
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA5#162 |  |  |  |  | Initial skeleton | 0.0.0 |
| 2025-10 | SA5#163 | S5-254664 S5-254666 S5-254667 S5-254744 | pCR pCR pCR pCR |  |  | pCR TR 28.882 initial ToC Pseudo-CR on TR 28.882 add Management support to UE-Side model training use case Pseudo-CR on TR 28.882 add Management support to NW-Side model training use case Pseudo-CR on TR 28.882 Add New Use Case on Management of Vertical Federated Learning | 0.1.0 |
| 2025-11 | SA5#164 | S5-255505 S5-255506 S5-255515 S5-255518 S5-255519 S5-255687 | pCR pCR pCR pCR pCR pCR |  |  | Pseudo-CR TR 28.882 Use case on ML model transfer delivery to UE Pseudo-CR on TR 28.882 add Management support use case and requirement for two-sided model training Pseudo-CR TR 28.882 Add Solution for Management of Vertical Federated Learning pCR on Rel-20 TR 28.882 Adding Enhancements on LCM of Federated Learning Pseudo-CR TR 28.882 Sustainability aspects of ML model training and inference Rel-20 Pseudo-CR TR 28.882 Use case on enhancing Reinforcement Learning with performance targets | 0.2.0 |