TSG SA5 Meeting #165	S5-26047

Goa, India,  9 – 13 February 2026

Source:	Mavenir

Title:	Discussion on PDU Session ID Handling for V-SMF Charging in EPS–5GS Interworking

Document for:	Discussion

Agenda item:	7.3 Charging Maintenance and Rel-20 Charging small enhancements

Spec:	3GPP TS 32.291 Telecommunication management; Charging management; 5G system, charging service; Stage 3

Version:	18.12.0/19.5.0

Work Item: 	5GS_Ph1-SBI_C

Decision/action requested

The group is asked to discuss the recommendations in clause 3.

1		Introduction

As per TS 32.255 section 5.2.2.11.7 EPS to 5GS handover for roaming in Home routed scenario.

In the EPS–5GS interworking scenario with/without N26 support for Home-Routed PDU sessions, upon receiving the Create SM Context Request from the AMF, the V-SMF initiates charging interaction with the V-CHF and sends a Charging Data Request Initial. According to current specifications, this message mandates inclusion of the PDU Session ID within the PDUSessionInformation IE.

However, in EPS–5GS interworking with or without N26 for Home-Routed sessions, the V-SMF does not receive the PDU Session ID in the Create SM Context Request from the AMF. The PDU Session ID becomes available only after interaction with the H-SMF and is provided in the PDUSessionCreate Response from the H-SMF.

Consequently, at the stage where the V-SMF is required to send the Charging Data Create Request Initial to the V-CHF, the PDU Session ID is not yet available.

The following call flow explains the existing behavior:

2		Proposal

Step B.1 During the handover, when V-SMF do not have PDU session id,  the already defined provisional identifier/correlation key (e.g., tuple of {SUPI/GPSI, DNN, S-NSSAI, V-SMF instance ID, access type}) which are provided in charging data request (Initial) at Create time, shall be used.

Steo D.1 V-SMF has to send Charging Data Update Request  to the V-CHF. Now the PDU Session ID is available and the mandatory subsequent update is used to populate the PDU Session ID

3		Conclusions & recommendations

Conclusions:

- During the handover from EPS to 5G, the V-SMF do not have access to PDU session ID. Hence V-SMF cannot send it in Charging Data Request Initial.

- At this point, tuple of {SUPI/GPSI, DNN, S-NSSAI, V-SMF instance ID, access type}) shall be used to correlate the session at the V-SMF

- The PDU session ID is available at V-SMF only after interaction with H-SMF. Hence the  subsequent update Request, shall be used to populate the PDU Session ID

Recommendations:

- pduSessionID shall be marked as conditional attribute in the PDUSessionInformation object sent in Charging Data Request.  Currently it is marked as mandatory in specification 32.291 section 6.1.6.2.2.8. Making mandatory do not help as V-SMF will never have it during the Charging Data Request Initial as explained in this paper.