| 3GPP TR 28.891 V0.3.0 (2026-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Service and System Aspects; Study on Charging Aspects of CAPIF phase 3 (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Concepts and background	9

4.1	General Description	9

4.2	Background	9

5	  CAPIF Charging Scenarios and Topics	12

5.1	Topic #1 CAPIF Converged Charging support for service API/AEF	12

5.1.1	General description and assumptions	12

5.1.2	Use Cases	12

5.1.2.1	Use Case #1.1: API Invokers Service Charging	12

5.1.2.2	Use Case #1.2: Converged Charging for API Invoker Onboarding and Discovery in Non-Trusted Domain	12

5.1.3	Potential charging requirements	12

5.1.4	Key Issues	13

5.1.5	Possible Solutions	13

5.1.5.1	Solution #1.1: CTF in CCF for CAPIF-1 and CAPIF-1e	13

5.1.5.2	Solution #1.2: Chargeable services for CAPIF-1 and CAPIF-1e	14

5.1.5.3	Solution #1.3: CTF in CCF for CAPIF-4 and CAPIF-4e	14

5.1.5.4	Solution #1.4: Chargeable services for CAPIF-4 and CAPIF-4e	15

5.1.6	Evaluation	16

5.1.7	Conclusion	16

5.2	Topic #2 CAPIF Converged Charging of multiple API Providers	16

5.2.1	General description and assumptions	16

5.2.2	Use Cases	16

5.2.2.1	Use Case #2.1: API Invoker accessing Service APIs across Interconnected CAPIF Provider Domains	16

5.2.2.2	Use Case #2.2 API publishing in CAPIF core function by API Provider	16

5.2.2.3	Use Case #2.3: Converged Charging for API Updates and Management Across Interconnected CAPIF Provider Domains	17

5.2.3	Potential charging requirements	17

5.2.4	Key issues#2: Charging events and charging information required	17

5.2.5	Possible Solutions	18

5.2.6	Evaluation	18

5.2.7	Conclusion	18

5.3	Topic #3 CAPIF Converged Charging of API Invoker Authorization and Authentication Impact	18

5.3.1	General description and assumptions	18

5.3.2	Use Cases	18

5.3.2.1	Use Case #3.1: Converged Charging for API Invoker Authentication and Authorization Events Impact	18

5.3.2.2	Use Case #3.2 Authorization in CAPIF core function	18

5.3.2.3	Use Case #3.3: Authorization and authentication in application exposure function	19

5.3.3	Potential charging requirements	19

5.3.4	Key issues#3: Charging events and charging information required	19

5.3.5	Possible Solutions	20

5.3.6	Evaluation	20

5.3.7	Conclusion	20

6	  Evaluation	20

7	  Conclusion	20

Annex A (informative): PlantUML	21

A.1.1	 Figure 5.1.5.1-1	21

A.1.2	 Figure 5.1.5.3-1	21

Annex B (informative): Change history	22

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document studies on how Coverged Charging support CAPIF enhancements and new Charging scenarios brought by the CAPIF framework, as defined in 3GPP TS 23.222 [2].

The following items are studied:

- possible charging scenarios and requirements related to service API/AEF Instantiation and Multiple Provider Interoperability.

- potential charging solutions and impacts for API Invoker authorization and authentication.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[3]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs	"

[4]	3GPP TS 32.290: " 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

3GPP		3rd Generation Partnership Project

5GS	5g System

AEF	API Exposing Function

API	Application Programming Interface

CAPIF	Common API Framework

CCF	CAPIF Core Function

CDR	Charging Data Record

CTF	Charging Trigger Function

RNAA	Resource owner-aware Northbound API Access

# 4	Concepts and background

## 4.1	General Description

The CAPIF core function provides the centralized CAPIF APIs that enable onboarding, discovery, security, and monitoring of exposed APIs. API invokers connect to the CAPIF core via CAPIF-1 and CAPIF-2 interfaces for onboarding and discovery of APIs, while CAPIF-1e extend these capabilities by enabling the authorization and authentication, service API discovery by an API Invoker outside the PLMN Trust domain, CAPIF-2e enable the API Invoker to communicate to the service APIS which belong to different trust domains.

The inter-operability of different CAPIF Providers is supported by the CAPIF-6e reference point, which enables integration with CCF of a trusted domain (i.e. 3rd Party, PLMN). This ensures that API invokers from one trust domain can securely discover and consume APIs exposed in another trust domain, preserving interoperability and trust management.

Figure 4.1-1: CAPIF interconnection with multiple CAPIF provider domains

## 4.2	Background

The Common API Framework defined, in 3GPP TS 23.222 [2], offers a uniform approach for securely exposing and utilizing network APIs. It incorporates shared functional components and reference points to facilitate API invoker verification, permission control, oversight and administration. This setup allows developers and service users to interact with APIs via standardized methods, regardless of the network operators or API suppliers involved.

Under the existing standards (3GPP TS 23.222 [2]), CAPIF is outlined operation between different CAPIF Core Function (via CAPIF-6) within the same trust domain (ilustrated in Figure 4.2-1).

Figure 4.2-1 - CAPIF interconnection within a CAPIF provider domain (Figure 6.2.2-2 of 3GPP TS 23.222 [2])

In this setup, the API Exposing Function serves as key element for making service APIs available through CAPIF core. An API Provider can deploy several AEF instances.

With the growth of network-exposed APIs expands, various CAPIF providers are anticipated to operate alongside one another and collaborate. API invokers might require access to APIs from providers in separate administrative areas, necessitating systems for inter-domain trust establishment, verification, permission granting, linkage, and coordination among CAPIF core functions from different providers, which are dependant on the MNO business relations.

To enable this, new reference point CAPIF-6e has been added to support cross-provider verification, permission, and linkage between CAPIF core functions. These connections permit API invokers in one trust area to safely access APIs hosted in another, independent of the specific AEF instance, thereby promoting seamless operation, expandability, and reliable trust oversight (ilustrated in Figure 4.2-2).

Figure 4.2-2 - CAPIF interconnection with multiple CAPIF provider domain (Figure 6.2.2-1 of 3GPP TS 23.222 [2])

This framework and its interconnection aspects are going to be further studied in this document to evaluate potential enhancements, and its Converged Charging support.

# 5			CAPIF Charging Scenarios and Topics

## 5.1	Topic #1 CAPIF Converged Charging support for service API/AEF

### 5.1.1	General description and assumptions

The reference points used for the API invoker within and outside the PLMN trust domain and to discover service APIs are the CAPIF-1 and CAPIF-1e reference points (TS 23.222 [2] clauses 6.4.2 and 6.4.3), which exists between the API invoker and the CAPIF core function.

### 5.1.2	Use Cases

#### 5.1.2.1	Use Case #1.1: API Invokers Service Charging

An operator provides CAPIF Core Functions and CAPIF-1 and CAPIF-1e reference points towards API Invokers and wants to be able to charge the API Invokers for services it provides.

#### 5.1.2.2	Use Case #1.2: Converged Charging for API Invoker Onboarding and Discovery in Non-Trusted Domain

The API Invokers outside the PLMN trust domain reach CAPIF Core function through CAPIF-1e reference point. The API Invoker performs onboarding to authenticate and authorize access, followed by service API discovery queries. Charging is applied based on the success of the onboarding attempt and discovery operations.

This use case addresses differentiated charging for external invokers, potentially with higher tariffs due to trusted and non-trusted management overhead.

Charging Party: CAPIF Provider

Charged Party: API Invoker

### 5.1.3	Potential charging requirements

REQ-CH_CAPIF_RP-01: Charging for services provided via the CAPIF-1 and CAPIF-1e reference points shall be supported.

REQ-CH_CAPIF_RP-03: Charging for onboarding and discovery events via CAPIF-1e.

REQ-CH_CAPIF_RP-04: The system shall enable higher or differentiated tariffs for non-trusted API Invokers in onboarding/discovery, reflecting trusted and non-trusted connectivity overhead.

Editor’s Note: REQ-CH_CAPIF_RP-02 and REQ-CH_CAPIF_RP-03 should be merged

### 5.1.4	Key Issues

Key issue #1.1: Identify the chargeable events for reference point CAPIF-1 and CAPIF-1e.

Key issue #1.2: Identify the placement of the charging trigger function for reference point CAPIF-1 and CAPIF-1e.

Key issue #1.3: Identify the chargeable events and information for onboarding/discovery in non-trusted domains via CAPIF-1e.

Key issue #1.4: Determine CTF placement and impacts for handling differentiated tariffs in trusted and non-trusted onboarding/discovery scenarios.

Editor’s Note: check potential merge

### 5.1.5	Possible Solutions

#### 5.1.5.1	Solution #1.1: CTF in CCF for CAPIF-1 and CAPIF-1e

A possible solution for key issue #1.2 covering requirement REQ-CH_CAPIF_RP-01.

The CAPIF-1 and CAPIF-1e reference points (TS 23.222 [2] clauses 6.4.2 and 6.4.3), which exists between the API invoker and the CAPIF core function, is used for the API invoker within and outside the PLMN trust domain and to discover service APIs, to authenticate and to get authorization.

For CAPIF-1and CAPIF-1e there are only two entities involved and that is the API Invoker and the CCF, and since the API Invoker may be outside the operator’s trusted domain the place to trigger charging is the CCF.

Figure 5.1.5.1-1: Invocation of service APIs PEC with CCF acting as CTF

Based on the flow in TS 23.222 [2] clause 8.20.

1.	CAPIF-2: The API Invoker invokes an API towards the API Exposing Function in the API provider domain, for more details see TS 29.222 [3].

2.	CAPIF-2: After the execution of the service provided by the API invoked the API Exposing Function in the API provider domain responds with the result, for more details see TS 29.222 [3].

3.	Charging Data Request [Event]: The API Exposing Function in the API provider domain sends Charging Data Request [Event] to the CHF for reporting the charging information including API invoker id and result (e.g. failure or success) of the API invocation, for more details see TS 32.290 [4].

4.	Create CDR: the CHF stores received information and creates a CDR related to the service, for more details see TS 32.290 [4].

5.	Charging Data Request [Event]: The CHF acknowledges by sending Charging Data Response [Event] to the API Exposing Function in the API provider domain, for more details see TS 32.290 [4].

#### 5.1.5.2	Solution #1.2: Chargeable services for CAPIF-1 and CAPIF-1e

A possible solution for key issue #1.1 covering requirement REQ-CH_CAPIF_RP-01.

The CAPIF-1 and CAPIF-1e reference points (TS 23.222 [2] clauses 6.4.2 and 6.4.3), which exists between the API invoker and the CAPIF core function, is used for the API invoker within and outside the PLMN trust domain and to discover service APIs, to authenticate and to get authorization.

The CAPIF-1 and CAPIF-1e reference points currently supports the following services:

-	Onboarding the new API invokers and offboarding API invokers.

-	Authenticating the API invoker based on the identity and credentials of the API invoker.

-	Mutual authentication between the API invoker and the CAPIF core function.

-	Providing authorization for the API invoker prior to accessing the service API.

-	Providing authorization for the API invoker based on RNAA.

-	Discovering the service APIs information.

Of the supported services the following is suggested to be main chargeable service:

-	Onboarding the new API invokers and offboarding API invokers.

Optionally also the following services can be used to include charging in the authorization:

-	Discovering the service APIs information

-	Providing authorization for the API invoker prior to accessing the service API.

#### 5.1.5.3	Solution #1.3: CTF in CCF for CAPIF-4 and CAPIF-4e

A possible solution for key issue #1.2 covering requirement REQ-CH_CAPIF_RP-01.

The CAPIF-4 and CAPIF-4e reference points (TS 23.222 [2] clauses 6.4.7 and 6.4.10), which exists between the API publishing function within the PLMN trust domain or 3rd party trust domain and the CAPIF core function within the PLMN trust domain, is used for publishing the service API information.

For CAPIF-4and CAPIF-4e there are only two entities involved and that is the API Provider and the CCF, and since the API Provider may be outside the operator’s trusted domain the only place to trigger charging is the CCF.

Figure 5.1.5.3-1: Publishing the service API PEC with CCF acting as CTF

Based on the flow in TS 23.222 [2] clause 8.3.

1.	CAPIF-4: The API Publishing Function in the API provider domain sends a service API publish request to the CAPIF Core Function, for more details see TS 29.222 [3].

2.	CAPIF-4: After the CAPIF core function have performed the publishing process the CAPIF Core Function provides a service API publish response to the API Publishing Function in the API provider domain, for more details see TS 29.222 [3].

3.	Charging Data Request [Event]: The CAPIF Core Function sends Charging Data Request [Event] to the CHF for reporting the charging information including API provider id and outcome of the publishing, for more details see TS 32.290 [4].

4.	Create CDR: the CHF stores received information and creates a CDR related to the service, for more details see TS 32.290 [4].

5.	Charging Data Request [Event]: The CHF acknowledges by sending Charging Data Response [Event] to the CAPIF Core Function, for more details see TS 32.290 [4].

#### 5.1.5.4	Solution #1.4: Chargeable services for CAPIF-4 and CAPIF-4e

A possible solution for key issue #1.2 covering requirement REQ-CH_CAPIF_RP-01.

The CAPIF-4 and CAPIF-4e reference points (TS 23.222 [1] clauses 6.4.7 and 6.4.10), which exists between the API publishing function within the PLMN trust domain or 3rd party trust domain and the CAPIF core function within the PLMN trust domain, is used for publishing the service API information.

The CAPIF-4 and CAPIF-4e reference points currently supports the following service:

-	Publishing the service APIs information by the API publishing function.

Of the supported service the following is suggested to be main chargeable service:-	Publishing the service APIs information.

### 5.1.6	Evaluation

### 5.1.7	Conclusion

## 5.2	Topic #2 CAPIF Converged Charging of multiple API Providers

### 5.2.1	General description and assumptions

Clause 6.2.2 of 3GPP TS 23.222 [2] describes on how CAPIF Interconnection is supported.

The reference points used for publishing the service API information are the CAPIF-4 and CAPIF-4e reference points (TS 23.222 [2] clauses 6.4.7 and 6.4.10), which exists between the API publishing function within the PLMN trust domain or 3rd party trust domain and the CAPIF core function within the PLMN trust domain.

The reference points used for management of service API, API invoker and API provider domain function information are the CAPIF-5 and CAPIF-5e reference points (TS 23.222 [2] clauses 6.4.8 and 6.4.11), which exists between the API management function within the PLMN trust domain or 3rd party trust domain and the CAPIF core function within the PLMN trust domain.

Therefore, it would be necessary to evaluate the possibility to support Converged charging in such a deployment model.

### 5.2.2	Use Cases

#### 5.2.2.1	Use Case #2.1: API Invoker accessing Service APIs across Interconnected CAPIF Provider Domains

An API Invoker onboarded in CAPIF Provider A discovers and invokes service APIs published by CAPIF Provider B via inter-domain reference points (CAPIF-6e) as defined in TS 23.222 [2] Clause 6.2.2.

The same API Invoker subscription may be charged for:

- Service API invocations in Provider B

There is the possibility to share the revenues between CAPIF Provider A and CAPIF Provider B.

Charging Party: CAPIF Provider A

Charged Party: API Invoker

#### 5.2.2.2	Use Case #2.2 API publishing in CAPIF core function by API Provider

An operator provides CAPIF Core Functions and CAPIF-4 and CAPIF-4e reference points towards API Providers and wants to be able to charge the API Providers for services it provides.

Charging party: CAPIF Provider

Charged party: API Provider

#### 5.2.2.3	Use Case #2.3: Converged Charging for API Updates and Management Across Interconnected CAPIF Provider Domains

An API Provider B (in a non-trusted domain) publishes service APIs via CAPIF-4e to CAPIF Provider A's for discovery. Subsequently, the API Provider performs updates, unpublishes, or retrieves API information across domains. Charging is triggered for these management operations, with potential fees for cross-domain synchronization and storage usage in Provider A's domain.

Please refer to 3GPP TS 23.222 [2], clause 4.12 for a reference on the CAPIF Providers interconnection architecture.

Charging Party: API Provider B

Charged Party: API Invoker

Editor’s Note: This needs further clarification

### 5.2.3	Potential charging requirements

The following are potential high-level charging requirements, derived from the requirements in TS 23.222 [2].

REQ-CH_CAPIF_IC-01: The CAPIF should support converged charging for APIs service management either per CAPIF provider A or B.

REQ-CH_CAPIF_IC-02: Charging for services provided via the CAPIF-4 and CAPIF-4e reference points shall be supported.

REQ-CH_CAPIF_IC-03: Charging for API updates/unpublishes/retrievals via CAPIF-4e in different trusted domains.

REQ-CH_CAPIF_IC-04: Converged charging should support revenue sharing models for API invocations in different trusted domains.

REQ-CH_CAPIF_IC-05: Charging for services provided via the CAPIF-6 and CAPIF-6e reference points should be supported.

Editor’s Note: REQ-CH_CAPIF_IC-02 and REQ-CH_CAPIF_IC-03 should be merged

### 5.2.4	Key issues#2: Charging events and charging information required

Key issue #2.1: Identification of the charging information and chargeable event for API Invoker accessing Service APIs of a CAPIF Provider in an inter-connection CAPIF scenario;

Key issue #2.2: Identify the chargeable events for reference point CAPIF-4 and CAPIF-4e.

Key issue #2.3: Identify the placement of the charging trigger function for reference point CAPIF-4 and CAPIF-4e.

Key issue #2.4: Identify the chargeable events for API updates, unpublishes, and retrievals in multi-provider scenarios in different trusted domains.

Key issue #2.5: Evaluate mechanisms which should allow revenue sharing between CAPIF providers.

Key issue #2.6: Identify the chargeable events for reference point CAPIF-6 and CAPIF-6e.

Key issue #2.7: Identify the placement of the charging trigger function for reference point CAPIF-6 and CAPIF-6e.

### 5.2.5	Possible Solutions

### 5.2.6	Evaluation

### 5.2.7	Conclusion

## 5.3	Topic #3 CAPIF Converged Charging of API Invoker Authorization and Authentication Impact

### 5.3.1	General description and assumptions

Clause 6.3.7 describes on CAPIF Authentication is supported and Clause 8.25.2.17 of 3GPP TS 23.222 [2] describes on how CAPIF Interconnections Authentication and Authorization is supported.

The reference points used for the API invoker to authentication and authorization are the CAPIF-2 and CAPIF-2e reference points (TS 23.222 [2] clauses 6.4.4 and 6.4.5), which exists between the API invoker and the API exposing function belonging to the same or different trust domains.

The reference points used for the API exposing function to authentication and authorization of an API invoker are the CAPIF-3 and CAPIF-3e reference points (TS 23.222 [2] clauses 6.4.6 and 6.4.9), which exists between the API exposing function within the PLMN trust domain or 3rd party trust domain and the CAPIF core function within the PLMN trust domain.

Therefore, it would be necessary to evaluate the possibility to support Converged charging, on such scenarios and its impact.

### 5.3.2	Use Cases

#### 5.3.2.1	Use Case #3.1: Converged Charging for API Invoker Authentication and Authorization Events Impact

An API Invoker is onboarded in a single CAPIF Core Function (CCF) and performs authentication (e.g., OAuth 2.0 token request) and authorization (scope validation, policy enforcement) prior to service API invocation via CAPIF-1,  CAPIF-2, CAPIF-3 reference points.

The following events may be considered:

- Authorization decisions (scope granted/revoked, API Invoker Overlimit usage)

Charging Party: CAPIF Provider

Charged Party: API Invoker

#### 5.3.2.2	Use Case #3.2 Authorization in CAPIF core function

An operator provides CAPIF Core Functions and CAPIF-3 and CAPIF-3e reference points towards API Provider and wants to be able to charge the API Provider for services it provides.

Charging party: CAPIF Provider

Charged party: API Provider

#### 5.3.2.3	Use Case #3.3: Authorization and authentication in application exposure function

An operator provides Application Exposure Function and CAPIF-2 and CAPIF-2e reference points towards API Invokers and wants to be able to charge the API Invokers for services it provides.

Charging party: CAPIF Provider

Charged party: API Provider

### 5.3.3	Potential charging requirements

The following are potential high-level charging requirements, derived from the requirements in TS 23.222 [2].

REQ-CH_CAPIF_AA-01: The Charging Function may trigger charging for authorization events: scope granted/revoked, service API overlimit usage.

REQ-CH_CAPIF_AA-02: The generated CDRs would be possible to correlate.

REQ-CH_CAPIF_AA-03: Charging for services provided via the CAPIF-3 and CAPIF-3e reference points shall be supported.

REQ-CH_CAPIF_AA-04: Charging for services provided via the CAPIF-2 and CAPIF-2e reference points shall be supported.

### 5.3.4	Key issues#3: Charging events and charging information required

Key issue #3.1: Identification of the charging information and chargeable events for APIs service Operation and Management Authorization;

Key issue #3.2: Identify the chargeable events for reference point CAPIF-3 and CAPIF-3e.

Key issue #3.3: Identify the placement of the charging trigger function for reference point CAPIF-3 and CAPIF-3e.

Key issue #3.4: Identify the chargeable events for reference point CAPIF-2 and CAPIF-2e.

Key issue #3.5: Identify the placement of the charging trigger function for reference point CAPIF-2 and CAPIF-2e.

### 5.3.5	Possible Solutions

### 5.3.6	Evaluation

### 5.3.7	Conclusion

# 6			Evaluation

# 7			Conclusion

Annex A (informative):
PlantUML

# A.1	Flows

## A.1.1	 Figure 5.1.5.1-1

@startuml

<style>

element {

BackGroundColor: white;

}

</style>

hide footbox

participant APP as "API\nInvoker"

participant CCF as "CCF\n(CTF)"

participant CHF as "CHF"

APP -> CCF : 1. CAPIF-1 [Onboarding of API Invoker]

APP <- CCF : 2. CAPIF-1 [Onboarding response]

group Post-delivery procedure

CCF -> CHF : 3. Charging Data request [Event]

rnote over CHF: 4. Create CDR

CCF <- CHF : 5. Charging Data response [Event]

end

@enduml

## A.1.2	 Figure 5.1.5.3-1

@startuml

<style>

element {

BackGroundColor: white;

}

</style>

hide footbox

participant NEF as "API\nProvider"

participant CCF as "CCF\n(CTF)"

participant CHF as "CHF"

NEF -> CCF : 1. CAPIF-4 [Service API publish request]

NEF <- CCF : 2. CAPIF-4 [Service API publish response]

group Post-delivery procedure

CCF -> CHF : 3. Charging Data request [Event]

rnote over CHF: 4. Create CDR

CCF <- CHF : 5. Charging Data response [Event]

end

@enduml

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-8 | SA5#162 | S5-253360 |  |  |  | Initial Skeleton | 0.0.0 |
| 2025-10 | SA5#163 | S5-254806 S5-254807 S5-254808 S5-254809 S5-254810 |  |  |  | Document Structure Update Update of the Scope Key issue regarding reference points and chargeable events General description Background | 0.1.0 |
| 2025-11 | SA5#164 | S5-255104 S5-255422 S5-255423 S5-255424 S5-255425 S5-255462 S5-255426 S5-255427 S5-255428 S5-255429 S5-255431 S5-255432 S5-255430 S5-255433 |  |  |  | Scope Correction Background Update General description for topic 1 General description for topic 3 CAPIF AA Charging Requirements CAPIF AA Impact Use Case CAPIF AA Key Issues Use case for topic 3 CCF Use case for topic 3 AEF General description for topic 2 CAPIF Interconnection Charging Requirements CAPIF Interconnection Key Issues CAPIF Interconnection Use Case Use case for topic 2 | 0.2.0 |
| 2026-02 | SA5#165 | S5-260440 S5-260529 S5-260530 S5-260531 S5-260532 S5-260533 S5-260534 S5-260535 S5-260536 S5-260537 S5-260538 S5-260539 |  |  |  | Use case 2 requirements and issues CAPIF-1 correction Solution topic 1 CTF placement Solution topic 1 chargeable services Solution topic 2 CTF placement Solution topic 2 chargeable services Use Case for CAPIF Converged Charging of Multiple API Providers Key Issues for CAPIF Converged Charging of Multiple API Providers Charging Requirements CAPIF Converged Charging of Multiple API Providers Use Case for Converged Charging for API Invoker Onboarding and Discovery in External Trust Domains Charging Requirements for the Converged Charging for API Invoker Onboarding Key Issues for the Converged Charging for API Invoker Onboarding | 0.3.0 |