| 3GPP TR 32.801-02 V0.3.0 (2026-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Charging Aspects of 6G System (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	High-level description	8

4.1	Background	8

4.1.1	5G Converged Charging System	8

4.1.2	Potential SA1 requirements	9

4.1.3	Potential New Services with Charging Concern	10

4.1.3.1	UE assisted services	10

4.2	Business Model for 6G Charging	10

4.2.1	Considerations	10

4.2.2	Business roles	10

4.2.3	Business models	10

5	Topics	11

5.1	Topic 1: Charging Architecture	11

5.1.1	General description	11

5.1.2	Use cases	11

5.1.2.1	Use Case #1.1: Charging for edge applications	11

5.1.2.2	Use Case #1.2: 6G charging architecture	12

5.1.2.3	Use Case #1.3: Refund	12

5.1.3	Potential charging requirements	12

5.1.4	Key issues	12

5.1.4.1	Key Issue #1.1: Charging for edge applications	12

5.1.4.2	Key Issue #1.2: Charging architecture	12

5.1.5	Solutions	12

5.1.5.1	 Solution #1.1: Distributed Charging Architecture with Local CHS for Edge Applications	12

5.1.5.1.1	General description	12

5.1.5.1.2	Procedure description	13

5.1.6	Evaluation	13

5.1.7	Conclusion	13

5.2	Topic 2: Charging Mechanism	13

5.2.1	General Description	13

5.2.2	Use cases	14

5.2.2.1	Use Case #2.1: Unified Charging Data	14

5.2.2.2	Use Case #2.2: Organization and structuring of OpenAPI	14

5.2.2.3	Use Case #2.3: Organization and structuring of ASN.1	14

5.2.2.4	Use Case #2.4: Charging for Multi-dimensional Resources	14

5.2.2.5	Use Case #2.5: Optimization Charging information collection and reporting	15

5.2.2.6	Use Case #2.6: Minimizing the number of charging request	15

5.2.2.7	Use Case #2.7: Optimization of charging sessions	15

5.2.2.8	Use Case #2.8: Charging for service with burst traffic	15

5.2.3	Potential charging requirements	15

5.2.4	Key issues	16

5.2.4.1	Key Issue #2.1: Charging Data Interoperability	16

5.2.4.2	Key Issue #2.2: Optimize the handling of the OpenAPI resource attributes	16

5.2.4.3	Key Issue #2.3: Optimize the handling of the ASN.1 attributes	16

5.2.4.4	Key Issue #2.4: Charging Mechanism for Multi-dimensional Resources	16

5.2.4.5	Key Issue #2.5: The charging information collection and reporting mechanism	16

5.1.5.6	Key Issue #2.6: Optimize the number of charging triggering	16

5.2.5.7	Key Issue #2.7: Optimize the number of charging sessions	16

5.2.4.8	Key Issue #2.8: Charging for service with burst traffic	16

5.2.5	 Solutions	16

5.2.5.1	Solution #2.1: Enhanced CHF for Unified Charging Data	16

5.2.5.2	Solution #2.2: Have separate YAML-files for the domains, subsystem, and services level attributes	16

5.2.5.3	Solution #2.3: Have separate ASN.1-files for the domains, subsystem, and services level attributes	17

5.2.6	 Evaluation	17

5.2.7	 Conclusion	17

6	Conclusions and Recommendations	17

Annex <X> : Change history	18

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

1	Scope

The present document studies the charging aspects of 6G system. This includes new charging business models and potential charging metrics, 6G charging architecture and charging mechanism, and the charging aspects of 6G services and frameworks.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.240: "Charging management; Charging architecture and principles".

[3]	3GPP TR 22.870: "Study on 6G Use Cases and Service Requirements".

[4]	3GPP TR 23.801-01: "Study on Architecture for 6G System; Stage 2".

[5]	3GPP TS 32.254: "Charging management; Exposure function Northbound Application Program Interfaces (APIs) charging".

[6]	3GPP TS 32.255: "Charging management; 5G Data connectivity domain charging; stage 2".

[7]	3GPP TS 32.256: "Charging management; 5G connection and mobility domain charging; stage 2".

[8]	3GPP TS 32.291: "Charging management; 5G system; Charging service, stage 3".

[9]	3GPP TS 32.298: " Charging management; Charging Data Record (CDR) parameter description".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ABMF	Account Balance Management Function

CDR	Charging Data Record

CGF	Charging Gateway Function

CHF	Charging Function

CTF	Charging Trigger Function

ECUR	Event Charging with Unit Reservation

GenAI	Generative Artificial Intelligence

IEC	Immediate Event Charging

PEC	Post Event Charging

RF	Rating Function

SCUR	Session Charging with Unit Reservation

4	High-level description

4.1	Background

4.1.1	5G Converged Charging System

The converged charging online and offline charging system is introduced for 5G system, as described in the TS 32.240 [2] clause 4.3.3.2. The following figure 4.1.1-1 specified the converged charging architecture.

Figure 4.1.1-1: Logical ubiquitous converged charging architecture

The Converged charging system includes the CHF (CHarging Function), ABMF (Account Balance Management Function), CGF (Charging Gateway Function) and RF (Rating Function).

The 5G converged charging is to provide a common process for a single, unified charging system, support the utilized online and offline charging information, as well as charging information record generation via CHF.

-	Converged online and offline charging, with and without Quota management.

-	Service Basde Interfaces Nchf via unified resource for all NF consumers.

-	CHF CDR generation: CHF CDR(s) are generated by CHF for both online and offline charging, for all of the 5G services.

-	Support "session" and "event" based charging, i.e. SCUR, ECUR, IEC and PEC.

-	Charging information collection and reporting: the usage of online charging and offline charging are collected and reported in one charging message.

4.1.2	Potential SA1 requirements

A set of charging requirements are identified in SA1 TR 22.870 [3] and copied below:

-	[PR 5.5.9.6-1] Subject to operator policy and user consent, the 6G system shall support means to provide users with differentiation of QoS and charging based on users’ digital identity information issued by a third party and users’ subscription information.

-	[PR 5.9.2.2-5] The 6G system shall be able to provide charging and accounting mechanisms for 6G System Data

-	[PR-5.9.8.2-3] The 6G system shall provide appropriate charging support for differentiated services per media component (e.g. to meet SLA requirements).

-	[PR 6.2.6-2] The 6G network shall support charging for the usage of network resources by third parties.

-	[PR 6.11.6-2] The 6G network shall support charging information collection for the intelligent communication assistant service.

-	[PR 6.14.6-1] The 6G system shall be able to collect charging information for the computing resource(s) coordinated by the 6G network e.g. per request, per UE.

-	[PR 6.21.6-5] The 6G network shall support charging information collection per UE and/or for the usage of network resources related to multiple 3GPP services (e.g. communication service, sensing service, AI service) to satisfy the intents received from the UE.

-	[PR 6.22.6-2] The 6G network (e.g. in conjunction to IMS) shall support charging information collection for the intelligent calling service.

-	[PR.6.25.6-3]	Subject to operator’s policy, the 6G network shall support the collection of charging information for the AI/ML model training, AI model inference and management which are within the service hosting environment for authorized third parties.

-	[PR 6.26.6-4] The 6G network shall be able to collect charging information based on unique traffic characteristics pertinent to e.g. specific GenAI applications, or enhanced/guaranteed user experience.

-	[PR 6.29.6-2] The 6G network shall be able to collect charging information for AI service per user.

-	[PR 6.36.6-3] The 6G network shall be able to collect charging information for the usage of AI/ML models that are stored or generated within the Service Hosting Environment.

-	[PR 6.44.6-3] Based on operator policy and user consent, the 6G network shall support mechanism to collect charging information for customized service based on received intent(s) from user.

-	[PR 6.55.6-2] Based on regulatory requirements, operators’ policy and agreement with 3rd party, 6G network shall support charging for services provided to 3rd party AI agents (e.g. combination of communication, computing service, sensing service, etc.).

-	[PR 7.17.6-4] The 6G system shall be able to support charging for the 6G wireless sensing service (e.g. considering sensing KPIs, duration).

-	[PR 9.10.6-3] The 6G system shall support collection of charging information associated with initiating and terminating an intelligent immersive calling service.

Editor's Note: The requirements for 6G charging are still to be completed and further updates can be made in this clause.

4.1.3	Potential New Services with Charging Concern

Table 4.1.3-1: Potential New Services with Charging Concern

| Potential New Services | Description |
| --- | --- |
| UE assisted services | services involving UE contributions, e.g., certain types of sensing services |

4.1.3.1	UE assisted services

The new services enabled by 6G networks will be different from the legacy communication services. In legacy networks, the UE acts as a service consumer. However, in 6G networks, the UE will assist MNO to provide new services, contributing capabilities such as sensing and the 6G service consumers will expand beyond the UE to include any authorized entities – application functions, 3rd parties, etc.

Consequently, the enhancing business models in which UEs not only consume services but also assist MNO to provide 6G services with better QoE should be considered.

4.2	Business Model for 6G Charging

4.2.1	Considerations

The 6G network will introduce new and enhanced services that may lead to new charging business models. To enable 6G, it is essential to support new capabilities and services, engage new ecosystem stakeholders, and evaluate the associated business models. Therefore, the following are considered in the 6G business model:

-	6G Services: the consumed services in 6G system, e.g. AI service

-	Target Customers: Charged party, e.g., individual customers (i.e., subscriber, consumer), industry/verticals customers (i.e., business subscriber)

-	Charging Model: Extend charging metrics beyond the volume and time (e.g., KPI (e.g., target QoS) fulfilment, AI token, AI task for AI services); new charging scenarios support the customized charging and billing package plan.

-	Stakeholders: MVNO, industry/vertical partners, service/content providers, subscriber.

4.2.2	Business roles

This clause analyzes the business roles, including charging party (who to charge) and charged party (who to be charged).

Table 4.2.2-1: Description and Examples of Business roles in 6G

| Business Role | Description | Examples |
| --- | --- | --- |
| Charged party | The entity that is charged for the consumed 6G service or resource. | Individual customers, industry/vertical customers, MVNO |
| Charging party | The entity that is responsible for providing 6G service or resource and charge the charged party. | MNO, MVNO, service/content providers, subscriber |

4.2.3	Business models

This clause aims to explore innovative business models required to monetize the new capabilities and services introduced by the 6G system. The potential models may move beyond the traditional volume/time/event-based charging of 5G to incorporate value, quality and specific resource utilization.

Table 4.2.3-1: Description and Examples of Business Models from View of Commercial Relationship in 6G

| Business Model | Description | Examples |
| --- | --- | --- |
| B2C (Business-to-Consumer) | The model where a business entity provides and charges or pays a customer for a 6G service or resource. | MNO, MVNO, service/content providers charge or pay an individual customer for a 6G service or resource. |
| B2B (Business-to-Business) | The model where a business entity provides and charges or pays another business entity for wholesale 6G service or resource. | MNO charges or pays an industry/vertical customer or MVNO for the consumed wholesale 6G service or resource. Service/content providers charge an MNO for consumed service/ content. |
| B2X2X (Business-to-X-to-X) | The model involving multi-party charging where a business entity partners with an intermediary business entity to deliver a 6G service or resource to the ultimate consumer. | A service/content provider charges an MNO for usage of 6G service/content, then the MNO charges an individual customer for the 6G service or resource. |

NOTE 1:	The business entity (e.g., MNO) can provide services or resources directly or with assistance from UEs.

5	Topics

5.1	Topic 1: Charging Architecture

5.1.1	General description

This topic focuses on the 6G charging architecture which defines the charging network functions and interfaces. The 6G charging architecture will need to handle increased traffic and data with diverse value generated by new services, and to be in line with the overall 6G system architecture studied in TR 23.801-01 [4]. The charging architecture covers the following aspects:

-	6G charging architecture supporting across diverse networks, services and resources

-	6G charging interface

-	CDRs transmission

-	Interworking

Table 5.1.1-1: Mapping of Use cases, Potential charging requirements, Key issues and Solutions for Topic 1

| Use cases | Potential charging requirements | Key issues | Solutions |
| --- | --- | --- | --- |
| #1.1 | ARC-01 | #1.1 | #1.1 |
| #1.2 | ARC-02 | #1.2 |  |
| #1.3 | ARC-03 | #1.3 |  |

Editor’s Note: Addition of service/feature into the table is ffs.

5.1.2	Use cases

5.1.2.1	Use Case #1.1: Charging for edge applications

In the 6G era, the MNOs can provide the customized industry/vertical private network services to enterprises/companies (e.g., smart factories, mines, ports). For example, in the low-latency charging scenario, edge applications demand extremely fast response times and low-latency for charging control from 6G charging system.

The potential charging requirements for this UC is: REQ-3GPP6GCH-ARC-01.

5.1.2.2	Use Case #1.2: 6G charging architecture

Considering the evolution of 6G system (e.g. increased traffic and data with diverse value generated by new services), the 6G charging architecture should support the charging scenarios for 6G charging systems.

The potential charging requirements for this UC is: REQ-3GPP6GCH-ARC-02.

5.1.2.3	Use Case #1.3: Refund

In the 6G era, the MNOs may provide 6G services with assistance from UEs. So when charging for this kind of services, the 6G charging system should consider the UE’s contribution. For example, the UE's involvement can be evaluated based on the volume of data it contributes. Therefore, timely refund for UEs is required.

The potential charging requirements for this UC is: REQ-3GPP6GCH-ARC-03.

5.1.3	Potential charging requirements

The following are potential charging requirements:

-	REQ-3GPP6GCH-ARC-01: The 6G charging system may support charging for edge applications.

-	REQ-3GPP6GCH-ARC-02: The 6G charging system shall support evolved charging architecture.

-	REQ-3GPP6GCH-ARC-03: The 6G charging system may support refund.

5.1.4	Key issues

5.1.4.1	Key Issue #1.1: Charging for edge applications

This key issue addresses the REQ-3GPP6GCH-ARC-01, to investigate charging for edge applications, including

-	The interaction and coordination for low-latency charging scenario

5.1.4.2	Key Issue #1.2: Charging architecture

This key issue addresses the REQ-3GPP6GCH-ARC-02, to investigate charging architecture including

-	Whether and how to define the charging functionalities in the 6G charging system.

-	The reference points provided by 6G charging architecture.

#### 5.1.4.3	Key Issue #1.3: Refund

This key issue addresses the REQ-3GPP6GCH-ARC-03, to investigate the refund, including

- when to get the UE’s contribution information

-	the detailed definition of UE’s contribution information

-	where to get and process the UE’s contribution information

5.1.5	Solutions

5.1.5.1		Solution #1.1: Distributed Charging Architecture with Local CHS for Edge Applications

5.1.5.1.1	General description

This solution addresses Key Issue #1.1 by introducing a distributed charging architecture. The traditional centralized charging system is divided into a Central CHarging System (C-CHS) and a Local CHarging System (L-CHS). C-CHS is located in the central core network while L-CHS is deployed at the network edge.

5.1.5.1.2	Procedure description

The figure 5.1.5.1.2-1 describes the high-level procedure for distributed charging architecture with local CHS for edge applications.

Figure 5.1.5.1.2-1: Message flow for Distributed Charging Architecture with Local CHS for Edge Applications

1. When a 6G UE uses an edge service, the CTF interacts with the nearest CHS (L-CHS).

2. The L-CHS performs local quota management, rating and account balance.

3. The L-CHS collects charging information from CTF and reporting to the C-CHS.

4. The C-CHS generates CDRs.

5.1.6	Evaluation

5.1.7	Conclusion

5.2	Topic 2: Charging Mechanism

5.2.1	General Description

This topic focuses on the necessary enhancements to the charging mechanisms which are used for charging services. This is specifically driven by the need to support requirements of the charging mechanism, and to enable the introduction of 6G services. The charging mechanism covers the following aspects:

-	Charging services

-	Reliability

-	Charging data management

Table 5.2.1-1: Mapping of Use cases, Potential charging requirements, Key issues and Solutions for Topic 2

| Use cases | Potential charging requirements | Key issues | Solutions |
| --- | --- | --- | --- |
| #2.1 | DAT-01 | #2.1 | #2.1 |
| #2.2 | DOC-01 | #2.2 | #2.2 |
| #2.3 | DOC-02 | #2.3 | #2.3 |
| #2.4 | MDR-01 | #2.4 |  |
| #2.5 | EFF-01 | #2.5 |  |
| #2.6 | EFF-02 | #2.6 |  |
| #2.7 | EFF-03 | #2.7 |  |
| #2.8 | BST-01 | #2.8 |  |

Editor's note:	A new column may be added to address the relationship with the services/features.

5.2.2	Use cases

5.2.2.1	Use Case #2.1: Unified Charging Data

A MNO has a service that relies on the combined functionality and resources of both 6G NFs and existing 5G NFs within the core network. The MNO wants the charging mechanism to ensure that all service charging information generated across the coexisting 5G and 6G domains is uniformly collected, correlated and processed.

The potential charging requirements for this UC is REQ-3GPP6GCH-DAT-01.

5.2.2.2	Use Case #2.2: Organization and structuring of OpenAPI

Precondition is that OpenAPI will be used for 6G.

The stage 2 definitions of the Information Elements for the domains, subsystem, and services are described in the 5G service charging specifications (e.g., TS 32.254 [5], TS 32.255 [6], and TS 32.256 [7]) and stage 3 definitions of the OpenAPI resource attributes in the TS 32.291 [8], together with Forge.

The handler of the specifications needs to keep the definitions consistent.

The potential charging requirements for this use case is REQ-3GPP6GCH-DOC-01.

5.2.2.3	Use Case #2.3: Organization and structuring of ASN.1

Precondition is that ASN.1 will be used for 6G.

The stage 2 definitions of the Information Elements for the domains, subsystem, and services are described in the 5G service charging specifications (e.g., TS 32.254 [5], TS 32.255 [6], and TS 32.256 [7]) and stage 3 definitions of the ASN.1 attributes in the TS 32.298 [9], together with Forge.

The handler of the specifications needs to keep the definitions consistent.

The potential charging requirements for this use case is REQ-3GPP6GCH-DOC-02.

5.2.2.4	Use Case #2.4: Charging for Multi-dimensional Resources

A 6G service may require a bundle of composite capabilities that includes not only network resources but also computing, AI, data or other capabilities, i.e. a multi-dimensional resources. When a UE triggers a 6G service instance, one or multiple 6G NFs may generate charging information. The MNO requires the charging system to handle the charging information for multi-dimensional resources.

The potential charging requirements for this UC is REQ-3GPP6GCH-MDR-01.

Editor’s Note: needs further clarification.

5.2.2.5	Use Case #2.5: Optimization Charging information collection and reporting

In the 6G era, the number of on simultaneous ongoing sessions and other chargable events will be huge, and thereby the charging information carried in the charging requests. The following charging information collection and reporting issues will become increasingly prominent:

-	What charging information that is useful in a specific charging scenario depends on a lot of other factors as well as the configuration and the chargeable event, for example, device type.

-	The interface size will continuously increase, with a large number of charging information containers which may impact the performance of the ConvergedCharging API.

The potential charging requirements for this UC is: REQ-3GPP6GCH-EFF-01.

5.2.2.6	Use Case #2.6: Minimizing the number of charging request

In the 6G era the number of on simultaneous ongoing data sessions and other events will be huge, and thereby the number of charging requests. Today the number of requests from the network function to the charging is controlled by the number events that require immediate charging triggering. This means that being able to have less immediate charging triggers will also minimize the number of requests, while being able to optimize the deferred triggers will optimize the message size. Therefore, an MNO would like to have the possibility to optimise the number of immediate triggers, to improve the charging signaling efficiency and save capacity, in the end saving energy.

5.2.2.7	Use Case #2.7: Optimization of charging sessions

In the 6G era the number of on simultaneous ongoing data sessions will be huge. Today there is a one-to-one relationship between a data session and a charging session meaning, it may even be more since one data session may trigger exposure charging etc. Therefore, an MNO would like to have the possibility to optimise the number of charging sessions, to improve performance and save capacity, in the end saving energy and avoid overload situations.

5.2.2.8	Use Case #2.8: Charging for service with burst traffic

Emerging services, such as those enabled by GenAI applications, are typically characterized by burst traffic with dynamic QoS requirements. These services introduce traffic patterns and service expectations distinct from traditional applications. It requires 6G system to provide efficient charging mechanisms for burst type traffic to optimize resource efficiency.

5.2.3	Potential charging requirements

The following are potential charging requirements:

-	REQ-3GPP6GCH-DAT-01: The 6G charging system shall support unified charging data that are interoperable for 5G and 6G.

-	REQ-3GPP6GCH-DOC-01: The OpenAPI specification shall support keeping the stage 2 definitions of domains, subsystem, and services information element consistent with the stage 3 definition of OpenAPI resource attributes.

-	REQ-3GPP6GCH-DOC-02: The specifications shall support keeping the stage 2 definitions of domains, subsystem, and services information element consistent with the stage 3 definition of ASN.1 attributes.

-	REQ-3GPP6GCH-MDR-01: The 6G charging system shall support mechanisms to handle charging information for multi-dimensional resources of the same service.

-	REQ-3GPP6GCH-EFF-01: The 6G charging system shall support the optimization of charging information at collection and reporting.

-	REQ-3GPP6GCH-EFF-02: The 6G charging architecture may support optimization of charging triggering.

-	REQ-3GPP6GCH-EFF-03: The 6G charging system may support optimization of charging sessions.

-	REQ-3GPP6GCH-BST-01: The 6G charging system may support charging for service with burst type traffic.

5.2.4	Key issues

5.2.4.1	Key Issue #2.1: Charging Data Interoperability

This key issue is for investigating on how to support unified charging data to address REQ-3GPP6GCH-DAT-01.

5.2.4.2	Key Issue #2.2: Optimize the handling of the OpenAPI resource attributes

This key issue addresses REQ-3GPP6GCH-DOC-01.

Optimize the handling of the information elements and OpenAPI resource attributes for domains, subsystem, and service in specifications.

5.2.4.3	Key Issue #2.3: Optimize the handling of the ASN.1 attributes

This key issue addresses REQ-3GPP6GCH-DOC-02.

Optimize the handling of the information elements and ASN.1 attributes for domains, subsystem, and service in specifications.

5.2.4.4	Key Issue #2.4: Charging Mechanism for Multi-dimensional Resources

This key issue addresses REQ-3GPP6GCH-MDR-01 and is to investigate how to support charging for multi-dimensional resources.

5.2.4.5	Key Issue #2.5: The charging information collection and reporting mechanism

This key issue addresses REQ-3GPP6GCH-EFF-01.

-	Investigate and identify possible optimizations for charging information at collection and reporting for 6G charging system.

5.1.5.6	Key Issue #2.6: Optimize the number of charging triggering

Optimize the number of charging triggering required to support the 6G charging.

5.2.5.7	Key Issue #2.7: Optimize the number of charging sessions

Optimize the number of charging sessions required to support the 6G charging.

5.2.4.8	Key Issue #2.8: Charging for service with burst traffic

This key issue addresses the REQ-3GPP6GCH-BST-01, to investigate charging mechanism for service with burst type traffic.

5.2.5		Solutions

5.2.5.1	Solution #2.1: Enhanced CHF for Unified Charging Data

This solution addresses Key Issue #2.1 and proposes enhancing the existing CHF to natively support 6G charging service interfaces while maintaining backward compatibility with 5G charging messages. This enhanced CHF will process and correlate cross-generation charging data uniformly.

5.2.5.2	Solution #2.2: Have separate YAML-files for the domains, subsystem, and services level attributes

Having one common OpenAPI part for the Nchf which then references all the domains, subsystem, and services level YAML-files. Using the current TS 32.291 [8] as an example, it would mean that the current TS32.291_NchfConvergedCharging.yaml it would mean to change from $ref: '#/components/schemas/SMSChargingInformation' to $ref: 'TS32.291_Nchf_SMSChargingData.yaml#/components/schemas/SMSChargingInformation', for all domains, subsystem, and services level attributes. In this example the TS32.291_Nchf_SMSChargingData.yaml would be its own file with its own version handling.

5.2.5.3	Solution #2.3: Have separate ASN.1-files for the domains, subsystem, and services level attributes

Having one common ASN.1 part for the 6G CDR which then references all the domains, subsystem, and services level ASN.1-files. Using the current TS 32.298 [9] as an example, it would mean that the current TS32.298_CHFChargingDataTypes.asn would import SMSChargingInformation from SMSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) sMSChargingDataTypes (xy) asn1Module (0) version1 (0)} instead of defining the SMSChargingInformation in the CHFChargingDataTypes, for all domains, subsystem, and services level attributes. In this example the TS32.298_Nchf_SMSChargingDataTypes.asn would be its own file with its own version handling.

5.2.6		Evaluation

5.2.7		Conclusion

6	Conclusions and Recommendations

Annex <X> :
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-10 | SA5#163 | S5‑254357 |  |  |  | Initial skeleton | 0.0.0 |
| 2025-10 | SA5#163 | S5‑254818 S5‑254816 S5‑254817 S5‑254819 S5‑254820 S5‑254821 |  |  |  | Update of the skeleton Update of the Scope Add background for the 6G Charging Introduce a topic of new charging business models and charging metrics for 6G Study structure 6G charging architecture Add the new topic for the charging mechanism | 0.1.0 |
| 2025-11 | SA5#164 | S5-255443 S5-255444 S5-255445 S5-255447 S5-255448 S5-255450 S5-255451 S5-255452 S5-255453 |  |  |  | Add SA1 requirements to background for the 6G Charging Update of Business Model for 6G Charging Add business model for UE assisted services Add General description for Topic 1 Charging Architecture Add General description for Topic 2 Charging Mechanism Add Use Case for Distributed Charging Architecture Add content on interworking of 6G charging system Structuring of OpenAPI Structuring of ASN.1 | 0.2.0 |
| 2026-02 | SA5#165 | S5-260559 S5-260560 S5-260561 S5-260562 S5-260563 S5-260564 S5-260565 S5-260566 S5-260567 S5-260568 S5-260570 S5-260571 |  |  |  | Clarification on the Background Update Business Models in 6G Charging Add Use case for 6G Charging Architecture Add use case and charging requirements for 6G Converged Charging Architecture Add Use Case, Requirement, Key Issue of Aggregated Charging Mechanism for Multi-dimensional Resources Add Use Case about the charging information collection and reporting Use case charging request Use case charging session Add use case of charging for GenAI service Solution for KI#1.1 Distributed Charging Architecture with Local CHF for Edge Applications Solution structuring of OpenAPI Solution structuring of ASN.1 | 0.3.0 |