| 3GPP TR 28.884 V0.3.0 (2026-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Service Based Management Architecture enhancement phase 4; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Concepts and background	8

4.1	Introduction	8

4.2	Management data streaming based on Message Bus technologies	8

4.3	Message Bus principles integrated in SBMA	9

5	Use cases and potential solutions	10

5.1	Use case #1: Integration of SBMA with 5GC and 5G Access Network architecture	10

5.1.1	Description	10

5.1.2	Potential requirements	10

5.1.3	Potential solutions	10

5.1.4	Evaluation of potential solutions	10

5.2	Use case #2: Management data streaming based on Message Bus	10

5.2.1	Description	10

5.2.2	Potential requirements	11

5.2.3	Potential solutions	11

5.2.4	Evaluation of potential solutions	12

5.3	Use case #3: Historical alarms	12

5.3.1	Description	12

5.3.2	Potential requirements	13

5.3.3	Potential solutions	13

5.4	Use case#4 on software management for 5G	14

5.4.1	Description	14

5.4.2	Potential requirements	16

5.4.3	Potential solutions	16

5.4.4	Evaluation of potential solutions	16

5.5	Use case#5 on inventory management for 5G	17

5.5.1	Description	17

5.5.2	Potential requirements	17

5.5.3	Potential solutions	17

5.5.4	Evaluation of potential solutions	17

5.6	Use case #6: Merge operation	17

5.6.1	Description	17

5.6.2	Potential requirements	19

5.6.3	Potential solutions	19

5.6.4	Evaluation of potential solutions	20

5.7	Use case#7: MnS selection in distributed deployment scenarios for SBMA	20

5.7.1	Description	20

5.7.2	Potential requirements	21

5.X	Use case #<X>: <use case title>	21

5.X.1	Description	21

5.X.2	Potential requirements	21

5.X.3	Potential solutions	21

5.X.4	Evaluation of potential solutions	21

6	Conclusions and recommendations	21

6.X	Use case #<X>: <use case title>	21

Annex A: Change history	23

# Foreword

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

New and updated use cases, as well as potential requirements, are emerging for managing 5G networks and services within the Service Based Management Architecture (SBMA). This document captures the corresponding potential enhancements to the SBMA.

# 1	Scope

The present document describes use cases, potential requirements, and potential solutions aimed at enhancing the Service Based Management Architecture (SBMA) in the context of 5G-Advanced (5G-A). It also presents conclusions and recommendations regarding the next steps in the standardization process.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[3]	3GPP TS 28.532: " Management and orchestration; Generic management services".

[4]	3GPP TS 28.537: " Management and orchestration; Management capabilities".

[5]	3GPP TS 28.552: " Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: " Management and orchestration; 5G end to end Key Performance Indicators (KPIs)".

[7]	3GPP TS 32.423: " Telecommunication management; Subscriber and equipment trace: Trace data definition and management".

[8]	https://datatracker.ietf.org/doc/html/rfc6455

[9]	https://websocket.org/guides/websocket-protocol/

[10]	3GPP TS 28.111: "Management and orchestration; Fault management (FM)".

[11]	3GPP TS 32.531: "Telecommunication management; Software management (SwM); Concepts and Integration Reference Point (IRP) Requirements".

[12]	3GPP TS 32.532: "Telecommunication management; Software management (SwM); Integration Reference Point (IRP); Information Service (IS)".

[13]	3GPP TS 32.533: "Telecommunication management; Software management (SwM); Integration Reference Point (IRP); Common Object Request Broker Architecture (CORBA) Solution Set (SS)".

[14]	3GPP TS 28.631: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements".

[15]	3GPP TS 28.632: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[16]	3GPP TS 28.633: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[17]	SP-250863: Study on SBMA enhancement phase 4

[18]	3GPP TS 32.158: " Design rules for REpresentational State Transfer (REST) Solution Sets (SS)"

[19]	RFC 6241 Network Configuration Protocol (NETCONF)

[20]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[21]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[22]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[23]		3GPP TS 28.620: "Telecommunication management; Fixed Mobile Convergence (FMC) Federated Network Information Model (FNIM) Umbrella Information Model (UIM)"

[24]	3GPP TS 28.623: “Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions

[25]	3GPP TS 28.111: “Management and orchestration; Generic management services”

[26]	3GPP TS 32.161: “Management and orchestration; JSON expressions (Jex)”

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Concepts and background

## 4.1	Introduction

SBMA adopts a modular, service-oriented approach that enables flexible, scalable, and interoperable management solutions.

The architecture for the present study builds upon the SBMA framework in TS 28.533 [2]. All potential enhancements and solutions are expected to comply with the principles outlined in TS 28.533 [2], including service abstraction, interface standardization, and functional decoupling. In addition, this study considers emerging requirements in 5G-Advanced networks, such as evolving the architecture reference model for management and orchestration, management data streaming based on message bus, enhancement of fault management, software management, inventory management, merge operation, and MnS selection in distributed deployment scenarios.

The present study aims to preserve architectural continuity with the SBMA baseline model while expanding its applicability to address these emerging requirements in 5G Advanced networks.

## 4.2	Management data streaming based on Message Bus and WebSocket technologies

3GPP TS 28.532 [3] defines Stage 2 and Stage 3 of the streaming data reporting service. Currently, the 3GPP management system supports WebSocket-based data streaming for PM, tracing, and analytics, establishing a point-to-point transport connection between the streaming data reporting MnS consumer and MnS producer. The producer initiates the connection with the consumer and exchanges metadata (informing the consumer about its identity and the nature of the data to be reported, i.e., streamInfo). After this phase, the producer reports streaming data to the consumer via the established connection.

Using point-to-point WebSocket communication protocol, which operates over a single TCP connection between a client and a server, a separate connection would be established between each consumer and the producer, causing the producer to generate and transmit multiple copies of the same data. The WebSocket protocol is standardized by the IETF in RFC 6455 [8]. It defines WebSocket as a protocol that enables ongoing, full-duplex, bidirectional communication between web servers and web clients over an underlying TCP connection, see [9]. Though a server can maintain multiple WebSocket connections simultaneously and broadcast messages to all connected clients. However, this is not a protocol-level feature, it is an application-level point-to-multipoint behaviour.

NOTE:	The management data includes performance measurements or KPIs defined in TS 28.552 [5] and TS 28.554 [6], trace reporting data defined in TS 32.423 [7] and analytic reporting data defined in TS 28.532 [3].

## 4.3	Message bus principles integrated in SBMA

Message bus principles

A message bus is a middleware architecture that enables different systems or services to communicate through a shared messaging infrastructure in a decoupled way. Key principles include the following:

- Loose Coupling: Services interact without knowing each other’s internal details. Enhances modularity and allows independent evolution of components.

- Asynchronous Messaging: Messages are sent without waiting for immediate responses. Improves system responsiveness and fault tolerance.

- Publish/Subscribe Model: Producers publish messages to a topic; consumers subscribe to topics of interest. Enables one-to-many and many-to-many communication.

- Scalability and Resilience: Supports high-throughput, fault-tolerant communication with buffering, retries, and failover etc. Maintains service continuity under load or failure.

Table 4.3-1 shows how the message bus principles are supported in the SBMA.

Table 4.3-1: Message bus principles supported in the SBMA

| Message bus principle | SBMA implementation |
| --- | --- |
| Loose Coupling | MnS producers and consumers interact via service exposure. Scaling of one side, e.g. MnS producer, does not affect the other side, e.g. MnS consumer. Adding a greater number of MnS consumers, does not affect the MnS producer. Error handling and failure recover does not require coupled proprietary implementation between MnS producer and MnS consumer. |
| Asynchronous Messaging | SBMA supports subscription/notification and streaming interfaces for non-blocking data flow (i.e., MnS producer sends the message data and continues its work without delay, while MnS consumer processes the message independently). |
| Publish/Subscribe | Publish: MnS producers register their management capabilities or management data information to MnS discovery service producer which could be used by MnS consumers. Subscribe: MnS consumers can subscribe to events or data streams exposed by MnS producers. |
| Scalability and Resilience | SBMA supports multipoint-to-multipoint streaming, improving efficiency and fault tolerance. |

# 5	Use cases and potential solutions

## 5.1	Use case #1: Integration of SBMA with 5GC and 5G Access Network architecture

### 5.1.1	Description

In a 5G-Advanced network deployment, a mobile operator adopts an SBMA that leverages a modular reference model as defined in annex A.11 of TS 28.533 [2]. This model abstracts Management Functions (MnFs) as logical entities, independent of their physical implementation, and enables them to interact exclusively through Management Services (MnSs).

Each MnF registers its MnSs with a service registry, enabling dynamic discovery and selection of services across domains and vendors.

For example:

- A fault management function in the RAN management domain autonomously detects faults using AI/ML models and triggers a healing workflow.

- This MnF can then orchestrate actions across both RAN and Core domains, invoking MnSs such as performance analytics, configuration updates, and resource scaling.

- The orchestration logic is implementation-agnostic, which means it does not rely on where or how the functions are deployed, only on the standardized MnS interfaces.

- The SBMA framework ensures that each MnF clearly declares its role as a producer or consumer of MnSs, supporting modularity and reusability.

Issue 1:

In Release-19, a modular reference model has been defined in annex A.11 of TS 28.533 [2]. It needs to continue evolving the architecture reference model for management and orchestration to support new requirements such as AI/ML integration, autonomy, and cross-domain orchestration.

For MnF NDTF, the content of the column "Reference management capability of MnS defined in Annex F" is "FFS". It needs to be updated according to the conclusion of NDT normative work.

Issue 2:

In TS 28.533 [2]: Annex A.11 Table A.11-b defines each MnF in the architecture reference model for management and orchestration along with the corresponding reference specifications that define their functionality. The content of the column "Reference management capability of MnS defined in Annex F" needs to be aligned with the content of corresponding column "Management Capability" of Annex F Table F-1: Overview of management capabilities and corresponding solution sets.

### 5.1.2	Potential requirements

REQ-SBMA-ARM-1:	The 3GPP management architecture should provide a consolidated view of management functions and their relationships.

### 5.1.3	Potential solutions

As a solution for issue 1, for MnF NDTF in Table A.11-b of TS 28.533 [2], it is proposed to update its content of column "Reference management capability of MnS defined in Annex F" from "FFS" to the following:

- NDT Lifecycle Management.

As a solution for issue 2:

- For table A.11-b of TS 28.533 [2], it is proposed to add new MnFs and their reference specifications information in normative work stage. No dedicated new MnFs are identified.

- To fix the gap of issue 2, it is proposed to update the content of the column "Reference management capability of MnS defined in Annex F" of Table A.11-b in TS 28.533 [2] to align with the content of corresponding column "Management Capability" of Annex F Table F-1 in TS 28.533 [2]. This improves clarity and traceability, and improving the usability of TS 28.533 [2].

### 5.1.4	Evaluation of potential solutions

## 5.2	Use case #2: Management data streaming based on message bus

### 5.2.1	Description

The 5G network employs a distributed, service-oriented architecture, leading to scenarios where multiple consumers may request the same management data for multiple uses. In a typical network performance data reporting scenario, an MnS producer collects and transmits performance data to multiple MnS consumers within the network management system (e.g., real-time analytics, network optimization engines, visualization dashboards, and audit services).

Using point-to-point WebSocket communication protocol, which operates over a single TCP connection between a client and a server, a separate connection would be established between each consumer and the producer. For the case of multiple consumers requesting the same performance data, it causes the producer to retrieve and duplicate the relevant performance data from its cache for each connection individually.

By adopting a message bus architecture, the producer establishes connection(s) with the message bus and publishes the performance data only once. Consumers retrieve the data whenever they want (i.e., the consumer either pulls the data from the message bus or subscribes and receives messages pushed by the bus) - hence the principle of loose coupling. No additional connections are initiated by the producer, and no duplicate data reporting occurs on its side for the case that multiple consumers request the same performance data. As a comparison example between WebSocket and message bus for the case that multiple consumers request the same performance data as an assumption:

- WebSocket option: duplicate data reporting occurs N times on the producer's side for sending the data to N consumers who request the same performance data.

- Message bus option: depending on the collection job, no duplicate data reporting occurs on the producer's side for sending the data to N consumers (regardless of the number of consumers) who request the same performance data because the only one copy of data buffered on the message bus broker (i.e., a middleware that facilitates communication between different participants by acting as an intermediary for messages) can be polled by all the consumers.

While the message bus architecture introduces certain operational considerations, such as the need for proper message category segmentation and consumer coordination management to avoid throughput bottlenecks. These aspects are addressed through established internal mechanisms and configuration practices. Such implementation details are outside the scope of this document. The message bus interface is not used on ManagedElement/NF level but only on management function level.

### 5.2.2	Potential requirements

REQ-SBMA-MB-01: The 3GPP management system should support loose coupling between the production of performance metrics and the consumption of performance metrics.

REQ-SBMA-MB-02: The 3GPP management system should ensure that message bus interface is applied only at the subnetwork level and not at the ManagedElement or Network Function level.

### 5.2.3	Potential solutions

5.2.3.1	Solution #1: Multipoint-to-Multipoint management data streaming based on message broking MnF

This potential solution proposes introduction of a new MnF implementing the message broker(s) that mediates data with the help of new MnS services, namely, message bus data reporting MnS and message bus data retrieval MnS (for data subscribers) to enable multipoint-to-multipoint streaming data reporting. This solution is meant for use at the management level and not at the managedElement/NF level. This solution also does not impact existing MnS services and can coexist beside such services as an additional option.

Figure 5.2.3.1-1: Potential solution for multipoint-to-multipoint management data streaming based on message bus

As shown in Figure 5.2.3.1-1, in this solution, message bus data reporting MnS producers send management data to Message broking MnF using the message bus data reporting service based on message bus publishing protocol. Message bus data retrieval MnS consumers receive the management data using the message bus data retrieval service based on message bus retrieval protocol. This approach allows the same copy of management data produced by the message bus data reporting MnS producer(s) to be shared across multiple message bus data retrieval MnS consumers thus enabling multipoint-to-multipoint communication and a publishing-and-subscribing based communication pattern.

Editor’s note: The following aspects are FFS

- What types of management data (e.g. PM) to be supported.

- What specific message bus-based communication protocol to be supported (e.g. Kafka) and why?

3.    How to do data access control.

4.    How the information related to the message bus (e.g., end-point URIs) is conveyed or configured to the MnS producers and MnS consumers needs to be clarified.

5.    Whether new MnSs is needed is to be studied

### 5.2.4	Evaluation of potential solutions

## 5.3	Use case #3: Historical alarms

### 5.3.1	Description

The "AlarmList" defined in TS 28.111 [10] contains "AlarmRecords" that represent currently active alarms. Alarms, that are cleared and acknowledged (inactive alarms) are removed from the alarm list and cannot be retrieved any more by MnS consumers.

However, old inactive alarms (historical alarms) are a valuable source of information in many use cases:

-	AI/ML training: Historical alarms provide context information for AI/ML training.

-	Security auditing: Historical security alarms provide information for security monitoring and auditing.

- Fault Repair: By analyzing historical alarms associated with previous faults, operators can identify recurring issues, understand root causes more effectively, and accelerate troubleshooting. This reduces mean time to repair (MTTR) and improves network and service reliability.

- Predictive Maintenance: Historical alarm can be used to forecast potential future failures. For example, repeated alarms from a specific network element may indicate impending hardware degradation. Predictive analytics based on alarm history enables proactive maintenance, minimizing service disruption.

- Enhanced analytics: Multiple MDA capabilities utilize alarm information as enabling data (see TS 28.104 [21], clause 8). Currently the enabling data is defined as "Alarm information and notifications as per TS 28.111 [10]", but the MDA capabilities would be more valuable if historical alarm data is also considered.

### 5.3.2	Potential requirements

REQ-SBMA-ALM-1: The 3GPP Management system should have the capability allowing to retrieve alarms that were active in the past (historical alarms).

REQ-SBMA-ALM-2: The 3GPP Management system should have the capability allowing to retrieve alarms that were active at a certain point in time in the past.

REQ-SBMA-ALM-3: The 3GPP Management system should have the capability allowing to retrieve alarms that were active during a certain time window in the past.

REQ-SBMA-ALM-4: The 3GPP Management system should have the capability allowing to retrieve the dates and times when the perceived severity of an alarm changed

REQ-SBMA-ALM-5: The 3GPP management system should have the capability allowing to retrieve the periods when the retrieved historical alarms may not fully represent the real situation in the network at that point in time.

REQ-SBMA-ALM-6: The 3GPP management system shall have the capability allowing retrieval of historical alarms independent of whether the notifications about the alarm changes were sent out or not.

Note: The size of the log should not be set by the standard, leaving it to be a vendor decision.

### 5.3.3	Potential solutions

#### 5.3.3.1	Overview

The functionality includes:

- storing alarms and alarm changes even after they become inactive.

- the producer shall provide an interface for retrieving historical alarms.

- it shall be possible to filter the retrieval request to get a subset of all historical alarm records

- access control shall be provided for retrieving historical alarms

#### 5.3.3.2	Content of alarm history

The historical alarm list shall contain one or more records for all raised alarms. The records shall include some metadata like the alarmId, timestamp and the body of the alarm change notification following the format of the attribute notificationContent, see TS 28.622[a].

The definition of the alarmId shall be updated in TS 28.111. Currently alarmId is unique only within a single AlarmList MOI. A specific alarmId value may be reused immediately after the previous alarm with this alarmId is removed from the AlarmList. Alarm history records may be provided corresponding to multiple alarmlists and for weeks or even many months. Handling multiple separate alarms with the same alarmId is difficult, for easier handling the alarmId shall be unique within a longer period of time and at least within a Subnetwork.

The first record shall be created when the alarm is raised.

A subsequent record shall be added when a relevant change occurs. A change is relevant for an alarm if one of the following properties change:

- perceivedSeverity. This includes setting the perceivedSeverity to cleared.

- ackState

Implementers may select other properties, the change of which results in creation of additional records.

Initially, alarms will be available both via the AlarmList MOI and the historical alarm list. Once an alarm becomes inactive it will only be available in the historical alarms list.

It is assumed that alarm history will be able to keep alarms available for a substantial period e.g. 2-30 days. However, the size of the historical alarm list is implementation specific. When the historical alarm list is full the producer may discard alarm history records. In this case the oldest records shall be discarded.

The consumer should be able to discover what time period the alarm history covers without uploading and parsing notifications. The alarmId and the timestamp of the oldest historical alarm should be readable by the consumer.

If the fault management system detects that it may not fully represent the alarm situation it shall create in the alarm history a record containing the notifyPotentialFaultyAlarmList and notifyAlarmListRebuilt notifications (even if there is no subscription for these notifications).

- Note: This clause does not imply anything about storage or implementation of the alarm history. It only specifies what information shall be available for retrieval.

#### 5.3.3.3	Retrieving alarm history

##### 5.3.3.3.1	Dedicated AlarmHistoryUploadJob

A new IOC could be defined: AlarmHistoryUploadJob under AlamList. This IOC shall be used to order the transfer of historical alarms from the producer to an outside location.

Attributes

- alarmScope a list of DNs as root of subtrees that should include the alarm's source

- TimeWindow (see TS 28.622[a] clause 5.4.1)

- ReportingCtrl, but without the streaming possibility (see TS 28.622[a] clause 4.3.33)

##### 5.3.3.3.2	Using ManagementDataCollection

The ManagementDataCollection could be used to arrange the transfer of historical alarms.

Define alarm history as a new data category.

Instead of targetNodeFilter a simple list of DNs shall be used like the NodeFilter.objectInstances. Maybe areaOfInterest can also be used.

reportingCtrl should be restricted as streaming is not useful for alarm history.

dataScope is not useful for alarm history, it should not be used

condition, processMonitor, consolidateOutput, jobid is not needed for a simple file upload job.

##### 5.3.3.3.3	Modeling alarm history in the NRM

NRM could be extended with a new IOC to model the historical alarms. Two variants can be considered:

An IOC like AlarmList can be defined where a multivalued historicalAlarmRecord attribute has a separate value for each alarm change. The attribute can use the current data type AlarmRecord in TS 28.111 clause 7.3.1.

IOC HistoricalAlarmRecord (Not preferred.): A separate MOI for each historic record. The attributes would be the same as those defined for the data type AlarmRecord in TS 28.111 clause 7.3.1.

##### 5.3.3.3.4	New dedicated operation

A new retrieval operation could be introduced that allows filtering based on time and alarm source.

#### 5.3.3.4	Filtering retrieval

The consumer should be able to retrieve a subset of all historical alarms. It shall be possible to specify the following retrieval criteria (filter):

- Alarms active at a point of time – the time must not be in the future

- Alarms active in a timewindow – the timewindow must not be in the future

- Alarms where the source of the alarm (objectInstance parameter) is in a subtree of the NRM. Filtering similarly as the AlarmList.unreliableAlarmScope attribute

More advanced filtering e.g. by using more parameters and/or JEX or Xpath would also be possible but is not proposed at this time.

#### 5.3.3.5	Access control

Accessing alarm history provides access to all information that any alarm can carry, including potentially sensitive, secret or personal/private information.

Providing one general access control right, that provides full access to the complete alarm history is relatively simple and allows definition of an FM superuser. Providing detailed access control rules based on the source and the content of the individual alarms is seen as overly complicated and thus is not proposed.

Retrieval solutions based on NRM data nodes are already covered by the current access control solution. For retrieval with a dedicated operation access control shall be implemented.

### 5.3.4	Evaluation of potential solutions

It is proposed to provide historical alarm records in the JSON format of the notification body that would report the alarm change using the datatype NotificationEntry (see TS28.622[a]). This will make parsing historical records simple as the same code can be used that is utilized for active alarms.

It is proposed to define a dedicated AlarmHistoryUploadJob according to clause 5.3.3.3.1 The other solutions are not proposed because:

- Many of the attributes of ManagementDataCollection are not relevant for the historical alarm list, thus it is a bad fit. It is also currently only used on the Subnetwork level. Clause 5.3.3.3.2 is not proposed.

- Historical alarms are retrieved relatively infrequently so exposing this data in the NRM seems unneeded. Exposing big amounts of unneeded data in the NRM is also resource intensive. Clause 5.3.3.3.3 is not proposed.

- SBMA generally tries to avoid dedicated operations. Clause 5.3.3.3.4 is not proposed.

It is proposed to implement only the relatively simple filtering solution described in clause 5.3.3.4. If more sophisticated filtering is needed, it can be added later.

It is proposed to implement only a general access control providing full access.

## 5.4	Use case #4 Software management for 5G

### 5.4.1	Description

3GPP TS 32.531 [11], TS 32.532 [12], and TS 32.533 [13] define the concepts, requirements, Information Service, and CORBA solution set for the software management of NEs for 4G, encompassing both automated and non-automated software management approaches.

Software management for 5G enhances 5G network operational efficiency. The benefits of software management retain for 5G network management:

-	One benefit of software management is interoperability: standardized procedures make it possible for operators to manage software on heterogeneous network elements in a multi-vendor environment and reduces operational complexity.

-	Another benefit is network service continuity. By supporting mechanisms such as staged upgrades, version control, and fallback strategies, software management capability minimizes downtime and protects user experience.

Currently, TS 28.533 [2] does not include support for software management. As 5G networks evolve in scale and complexity, it is essential to introduce software management capabilities into SBMA. These capabilities will enable operators to maintain software of NEs and NFs.

Editor's note:	To enable software management within SBMA, the associated potential requirements and potential solutions are FFS, building upon the legacy of software management of NEs for LTE while adapting to the principles of SBMA.

Based on 4G definitions in [11], when describing SW management in this document we use the following terminology:

Download – move the SW from a remote server to the producer.

Verify – verify the downloaded SW's integrity and that it is fit for its purpose.

Activation: Activation of software makes it ready to be used and the software starts providing service.

Installation: Installation of software puts it into a form suitable for activation or use. (Activation may include steps like unpacking, unzipping, checks, but does not include starting to provide service.)

Fallback: The process of activating a previous SW version. (Fallback may involve moving to a previous configuration content of NRM).

Cancellation: Is the process where a consumer stops an ongoing operation (e.g. download, activation). If the process is stopped by the producer, it is considered a failure of the process not a cancellation.

Deactivate: Remove a SW from providing service.

Delete: delete SW from the producer

Use cases for PNF SW management

The following are examples of possible/typical use-cases. (They are presented here as informative examples)

- UC1: Download software on MnS producer: In preparation for a software upgrade, relevant software artefacts are downloaded by suitable means to the MnS producer. After download, other preparatory steps (a.k.a. software installation) may be performed (such as unzipping, integrity validation) to make sure the software artefacts are suitable and ready for subsequent activation. This download/installation activity is typically not traffic-impacting and may happen sometime before the software is activated. The activity might be triggered by an MnS consumer, or might be triggered by some other event (perhaps the installation of new hardware on the PNF, requiring a software upgrade). The download/installation is typically long-running, and its progress can be externally observed. At the end of this use case, the software is ready to be activated. In exceptional error situations (such as unsuitable software being installed; checksum errors), activation of this software will not be possible.

- UC2: Activate software: Previously downloaded/installed software is activated. The activation of the software brings the software into service, effectively replacing the previously running software. As part of the software activation, the PNF’s configuration may be automatically updated. Software activation may or may not be traffic-impacting, depending on the particulars of the PNF implementation. The activation activity is usually triggered by an MnS consumer; but may be triggered by an event or a scheduler. The software activation is typically long-running, and its progress can be externally observed. At the end of this use case, the activated software has been brought into service. In exceptional error situations, the software activation procedure is aborted, the previous software brought (back) into service, and any automated updates to the PNFs configuration undone.

- UC3: Cancellation: The software installation and activation are typically long-running operations. In some scenarios (e.g., ongoing emergency, slow/faulty O&M network, a mistake having been made), the ongoing operation is cancelled. A cancellation is typically triggered by an MnS consumer. After the cancellation, the MnS producer will be in the same state as it was before the operation started. It is possible to re-start the same operation.

- UC4: Fallback to previous software: After the PNF’s software has been successfully upgraded, after observing the system's behaviour for some time, a decision may be made that the upgraded software is unsatisfactory (perhaps it is functionally or performance-wise inferior to the previously installed software). In this case, a software fallback may be triggered. The software fallback will re-activate previously activated software and effectively repeat the steps of the software activation (see UC2) for the previously activated software.

- UC5: Remove inactive software (house cleaning): Software residing on the producer may become unneeded. Unneeded and in-active software is removed from the producer. This operation is typically triggered by the MnS consumer. At the end of this use case the in-active software has been removed from the PNF and therefore is not available anymore for fallback or activation. One typical  case when software becomes unneeded is when a PNF’s software is successfully activated, and found to be operating satisfactorily (perhaps after having been observed for a while), other software still present on the PNF, but not in service, may be removed in order to free resources on the PNF.

Implementation considerations for further study

The producer should check the SW intended to be activated. Whether these checks are executed immediately after download, in a separate step, or at the beginning of activation is implementation dependent.

The producer may take additional preparatory steps before activation (e.g. unpacking the SW). Whether these are executed immediately after download, in a separate installation step or at the beginning of activation is implementation dependent.

Open issues for further study:

-Whether fallback shall be initiated by the consumer or by the producer or potentially both

- Whether activation shall be triggered explicitly by the consumer or automatically in the producer (e.g. by a scheduler) or both

Two modes shall be considered:

- a single step activation that includes download, any checks and activation. Activation may be immediate or  delayed, triggered by a scheduler or some other condition.

- a multi-step activation, where after download, activation is executed as a separate step within the producer. The separate activation step shall be initiated by a consumer operation.

### 5.4.2	Potential requirements

#### 5.4.2.1	Potential general requirements

REQ-SBMA-SWM-1: The 3GPP management system should support software management.

#### 5.4.2.2	Potential requirements for PNF

REQ-SBMA-PNFSWM-1: The 3GPP management system shall support the capability to download SW to the MnS producer.

REQ-SBMA-PNFSWM-2 The 3GPP management system shall support the capability to activate the new SW, that is to start running it by bringing it into service.

REQ-SBMA-PNFSWM-3: The 3GPP management system shall support the capability to monitor the progress and the result of software download, activation and fallback and any other vendor specific steps e.g. installation.

REQ-SBMA-PNFSWM-4: The 3GPP management system should support the capability to allow the consumer to cancel all SW management operations e.g. download.

REQ-SBMA-PNFSWM-5: The 3GPP management system shall support the capability to allow the consumer to retrieve information about the SW on the producer, including its state.

Editor's note: The relation between this requirement and the inventory information is FFS.

REQ-SBMA-PNFSWM-6: The 3GPP management system shall support the capability to remove inactive SW from the producer.

REQ-SBMA-PNFSWM-7: The 3GPP management system shall support the capability to fallback to a previous SW version. Fallback may include returning to the previous configuration.

Editor's note:	Unless the consumer or the producer is explicitly mentioned, the requirements do not imply whether the consumer or the producer initiates an activity. Unless explicitly specified the requirements do not imply whether an activity is executed as a separate step or potentially together with other activities. These issues are FFS.

REQ-SBMA-PNFSWM-8: The 3GPP management system shall support the capability to enable policy based autonomous SW management including (not limited to) SW initiation, download, activate, monitor, rollback.

REQ-SBMA-PNFSWM-9: The 3GPP management system shall support the capability for a node to trigger software management procedure for itself.

#### 5.4.2.3	Potential requirements for VNF

### 5.4.3	Potential solutions

It is the goal to define a flexible solution for PNF SW management.

It shall be possible to handle upgrades that consist of a single overall SW package but also upgrades that use a set of separate SW packages in a single activation. It shall be possible to handle systems where the full functionality of the system has to be upgraded at the same time (using one or more SW packages) but also systems that allow upgrading different parts of the system separately at different times.

It is assumed that the consumer knows a priori which SW packages can be installed/activated on which ManagedElements, and whether there are dependencies between SW packages.

A solution is proposed that follows SBMA principles. IOCs are defined in the NRM to facilitate SWM. The solution takes ideas and some solution elements from TS 32.532.

New IOCs SwPnfPackage, SwPnfJob, SwPnfProcess are introduced to satisfy the requirements given in clause 5.4.2. Relationship and inheritance diagrams are captured in 5.4.3.x -1 and 5.4.3.x -2 respectively. The <<ProxyClass>>ManagedEntity representing ManagedELement is only applicable when the MnS producer is embedded in the NE.

Figure 5.4.3.x.1 -1: Relationship UML diagram for software management

Figure 5.4.3.x.1 -2: Inheritance UML diagram for software management

The SwPnfPackage IOC includes attributes inherited from Top IOC and the following attributes in Table 5.4.3.x-1.

The attribute name is a human readable name of a software package.

The attribute version is a version identifier of the software package.

The attribute size represents the size of software. It is an integer. The unit is KB.

The attribute location represents the location of software. It is a string of the software file path on a server.

The attribute integrityCheck is a hash value which is used for software package integrity verification. It helps to ensure the downloaded software package is not corrupted during the transmission or storage.

The attribute vendorName is a name of inventory unit vendor (or vendors may provide manufacturer name)

The attribute userLabel is a user friendly free string for the operator to use. It has no effect on the procedures or states of the IOCs.

The attribute activationTime indicates the time when the SwPnfPackage was activated.

The attribute installationTime indicates the time when the SwPnfPackage was installed.

The attribute creationTime indicates the time when the SwPnfPackage was created by the implementing vendor.

The attribute downloadTime indicates the time when the SwPnfPackage was downloaded to the producer. E.g. if the IOC is under SubNetwork this can be the download to the element manager, if the IOC is under ManagedElement  this can be the download to the network element.

The attribute state represents the state of a software package. The ENUM value could be remote, downloaded, activated, etc. It is conditionally mandatory when this package is associated with one single network element. A state transit figure is given below.

Table 5.4.3.x.1-1 SwPnfPackage attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| name | M | T | T | T | T |
| version | M | T | T | T | T |
| userLabel | O | T | T | N | T |
| type | O | T | T | T | T |
| size | O | T | T | T | T |
| location | M | T | T | T | T |
| integrityCheck | O | T | T | T | T |
| vendorName | O | T | T | T | T |
| creationTime | M | T | F | T | T |
| downloadTime | M | T | F | F | T |
| state | CM | F | T | T | T |
| installationTime | CM | T | F | T | T |
| activationTime | CM | T | F | T | T |

The SwPnfJob IOC represents MnS consumer's requirements for PNF software management. It includes attributes inherited from Top IOC and the following attributes in Table 5.4.3.x-2.

The attribute nEInformation specifies the NEs or NE types for which this SwPnfJob instance is valid. If theIOC is under a ManagedElement the attribute nEInformation  has no effect and should not be used.

The attribute swJobId is a unique identifier of a software management job.The attribute name is a human readable name of a software management job.

The attribute type represents the actions for which this software management job asks. Its type is string. Examples of allowed value could be UPGRADE, FALLBACK, etc.

Note: Whether this attribute can be removed and its functionality included in steps should be decided in the normative TS phase.

The attribute startTime represents the start time of a software management job.

The attribute stopTime represents the end time of a software management job. If the SW management process is still running at the stop time the process is cancelled.

vendorParameters is list of key value pairs that can hold additional vendor specificvendor parameters. There is no guarantee that the producer will handle these.

The attribute steps represents the steps to execute (only steps in SWMcapabilities are accepted). The following SWM steps are specified:

- Download

- Verify

- Install

- Activate –  may imply a full or partial restart

- Deactivate – step is only applicable if partial upgrade is supported, as it is not allowed to remove all SW from the activated state

- Delete

Vendors are allowed not to expose some step explicitly, e.g. the activate step may implicitly include installation or it might be possible that the downloaded SW package is ready for installation thus there is no need for installation. The activate step is mandatory to expose.
Vendors may specify additional step.

Table 5.4.3.x.1-2 SwPnfJob attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nEInformation | M | T | T | T | T |
| swJobId | M | T | T | T | T |
| swJobName | M | T | T | T | T |
| type | M | T | T | T | T |
| startTime | O | T | T | T | T |
| stopTime | O | T | T | T | T |
| steps | TBD | T | T | T | T |
| vendorParameters | O | T | T | N | T |
| Attribute related roles |  |  |  |  |  |
| packageListRef | M | T | T | T | T |

The SwPnfProcess IOC represents the software management process for a RAN NE, which allows the MnS consumer to configure and be informed about the current situation of the software management process. It includes attributes inherited from Top IOC and the following attributes in Table 5.4.3.x-3. While a SwPnfJob MOI can handle one or more network elements, a separate SwPnfProcess MOI is created for each network element. wPnfProcess MOIs are created by the producer.

The SwProcessMonitor attribute represents the status of software management process for a single RAN NE and includes information the MnS consumer can use to monitor the progress and result of the software management process. The data type of this attribute is ProcessMonitor in TS 28.622 [x]. The following specializations are provided for this data type for the software management process:

-	The value of attribute status are "NOT_STARTED", "RUNNING", "CANCELLING","FINISHED", "SUSPENDED","FAILED" and "CANCELLED". The values "PARTIALLY_FAILED" is not used.

-	The attribute progessPercentage indicates progress of the process as percentage.

-	When the attribute status is equal to "RUNNING" the attribute progressStateInfo attribute shall indicate states like: "PRECHECK_BEFORE_DOWNLOAD", "SW_DOWNLOAD", "PRECHECK_BEFORE_ACTIVATION", "SW_ACTIVATION". Vendor specific information may be provided though.

Editor’s note: The exact status values shall be defined during execution.

-	For the case that the attribute status is equal to "FAILED" the attribute resultStateInfo shall indicate one of the failure reason, e.g. "UNKNOWN", " NE_HEALTH_CHECK_FAILED", "SW_DOWNLOAD_FAILED", " SW_ACTIVATION_FAILED", "DISCONNECTION_BETWEEN_NE_AND_OAM", "RESPONSE_TIMEOUT_BETWEEN_NE_AND_OAM","FTPS_CONNECTTION_FAILED_BETWEEN_NE_AND_OAM". Vendor specific information may be provided though.

The attribute stopPointSetIndication represents a True or False indication on whether this software management process instance will stop when the progressStateInfo attribute changes its value. If the value of stopPointSetIndication is True, when the progressStateInfo attribute changes its value, e.g., from "SW_DOWNLOAD" to "PRECHECK_BEFORE_ACTIVATION", this software management process will automatically stop and the value of stopPointStatus becomes "STOPPED". In this case only when the MnS consumer sets the value of stopPointStatus becomes "CONTINUED", this software management process will continue. If the value of stopPointSetIndication is False, this software management process will not automatically stop.

The attribute stopPointStatus represents whether the software management process is stopped or continued. It is applicable and mandatory when the value of stopPointSetIndication is True.

MnS consumer can terminate a software management process which is currently ongoing (the value of the attribute status is "RUNING" and the attribute progressStateInfo is not "SW_ACTIVATION") by configuring attribute cancelScProcess to "True". Then the attribute status will change from "RUNGING" to "CANCELLED" when MnS producer have terminated the software management process as MnS consumer requested.

Table 5.4.3.x.1-3 SwPnfProcess attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nEId | M | T | F | T | T |
| swProgressMonitor | M | T | F | F | T |
| detailedResult | M | T | F | F | T |
| stopPointSetIndication | O | T | T | F | T |
| stopPointStatus | CM | T | T | F | T |
| cancelSwProgress | O | T | T | F | T |

The SwMCapabilities IOC shall indicate which of the 3GPP specified steps (see clause 5.4.3.x.1) the producer exposes (and thus can be invoked explicitly) and whether any additional vendor specific steps are supported.

The IOC provides a read-only ordered set of steps. Each step can be invoked by setting the appropriate data in the SwPnfJob.  The SwMCapabilities IOC is created by the producer and is strictly read-only.

Attributes

Exposed steps

Name (extensible enumeration)

additionalProperties: nameValuePair – this allows both for future standard and for vendor extensions

#### 5.4.3.x.2	Procedures for PNF software management

Figure 5.4.3.x.2 -1: Procedures for software management

1. MnS consumer sends a request to create a SwPnfPackage instance (see createMOI operation defined in TS 28.532 [3]) to MnS producer with software package information.

2. Based on the received request, the MnS producer creates the concrete SwPnfPackage instance (i.e. instance of SwPnfPackage).

3. MnS Producer sends a response (see createMOI operation defined in TS 28.532 [3]) to the MnS Consumer with attribute "objectInstance" of the created SwPnfPackage instance.

4. MnS consumer sends a request to create a SwPnfJob instance (see createMOI operation defined in TS 28.532 [3]) to MnS producer with software package information.

5. Based on the received request, the MnS producer creates the concrete SwPnfJob instance (i.e. instance of SwPnfJob) for NE(s) or NE type(s) specified by attribute "neInformation".

6. MnS Producer sends a response (see createMOI operation defined in TS 28.532 [3]) to the MnS Consumer with attribute "objectInstance" of the created SwPnfJob instance.

7. For each NE (specified in the created SwPnfJob) starting its software management process, MnS producer creates an SwPnfProcess instance for the started software management process and configure the created SwPnfProcess instance with DN of the associated SwPnfJob.

8. MnS producer notifies (see notifyMOICreation notification defined in TS 28.532 [3]) the MnS consumer about the creation of a new SwPnfProcess instance, including DN of SwPnfProcess instance.

9. In the case that MnS consumer manually controls the software management process, the MnS consumer requests to configure attribute stopPointSetIndication of the SwPnfProcess instance with value "TRUE", which means when the attribute progressStateInfo changes, the process will stop and wait for MnS consumer’s command to continue.

10. When the value of attribute progressStateInfo changes, MnS producer stops handling the software management process.

11. MnS producer notifies the MnS consumer about the value change of the SwPnfProcess instance.

12. MnS consumer requests to configure attribute stopPointStatus of the SwPnfProcess instance with value "CONTUNUED" to continue the process.

The following step 13 describes the procedures for MnS consumer to monitor software management progress and result. These steps can happen any time after the SwPnfProcess instance is created until the SwPnfProcess instance is deleted

13a)	The MnS producer sends notification (see notifyMOIAttributeValueChanges defined in TS 28.532 [3]) to MnS consumer to notify the progress and result for software management process (see attributes in SwPnfProcess IOC).

13b)	The MnS consumer sends query request to MnS producer to query the attribute values of SwPnfProcess instance (see getMOIAttributes operation defined in TS 28.532 [3]) to obtain the progress and result (including DN of the SwPnfProcess instance and other attributes of SwPnfProcess instance) for software management process.

Following Steps 14 and 15 are the steps for MnS consumer request to terminate an ongoing software management process.

14. The MnS consumer sends a request (see modifyMOIAttributes operation defined in TS 28.532 [3]) to MnS producer to configure cancelScProcess attribute of the SwPnfProcess instance with value "TRUE" to terminate an ongoing software management process.

15. The MnS producer sends a response (see modifyMOIAttributes operation defined in TS 28.532 [3]) for terminating an ongoing software management process to MnS consumer.

16. MnS consumer deletes the SwPnfProcess instance when software management process is completed or terminated.

17. MnS producer notifies the MnS consumer about the deletion of a SwPnfProcess instance.

The following steps describes the procedures for MnS consumer request to delete a SwPnfJob instance to ultimate deactivation of requirements for software management management for a set of RAN NEs.

18. MnS consumer sends a request to delete a SwPnfJob instance (see deleteMOI operation defined in TS 28.532 [3]) to MnS producer with the DN of the SwPnfJob instance.

19. Based on the received request, the MnS producer delete the concrete SwPnfJob instance

20. MnS Producer sends a response (see deleteMOI operation defined in TS 28.532 [3]) to the MnS Consumer.

### 5.4.4	Evaluation of potential solutions

## 5.5	Use case #5 Inventory management for 5G

### 5.5.1	Description

3GPP TS 28.631 [14], TS 28.632 [15], and TS 28.633 [16] define the requirements, Information Service definitions, and XML solution set for inventory management. These specifications cover inventory information of NEs, including hardware, software, licences, Tower Mounted Amplifiers (TMAs) and antennas.

The benefits of inventory management retain for 5G network management:

Inventory management enables operators to track equipment status, plan capacity expansions, schedule upgrades, and manage decommissioning more effectively, which improves consistency across large, geographically distributed networks

Inventory management shows the links/topology among network elements, network functions and physical network resources, which helps operators for faster fault localization and root-cause analysis when problems occur. The insight into these dependencies also help operators minimize risks during maintenance or upgrades.

Currently, TS 28.533 [2] does not include support for Inventory Management (IM). As 5G networks evolve in scale and complexity, it is essential to introduce IM capabilities into SBMA. These capabilities will enable operators to maintain accurate and dynamic records of static network resources, supporting planning, assurance, and operational efficiency.

Editor's note 1:	To enable inventory management in SBMA, the associated potential requirements, and potential solutions are FFS, building upon the legacy of inventory management for LTE while adapting to the principles of SBMA.

Editor's note 2:	For topology or the relationship among different inventory objects, the difference between IM and CM for 5G is FFS.

### 5.5.2	Potential requirements

REQ-SBMA-IM-1: The 3GPP management system should support inventory management.

REQ-SBMA-IM-2: The 3GPP management system should support inventory management for hardware.

### 5.5.3	Potential solutions

Though the scope of TS 28.631 [14]/TS 28.632 [15] is limited to the communication between IRPAgent and IRPManager, the content of the inventory NRMs defined in TS 28.632 [15] can be reused to support inventory management for hardware. This solution addresses REQ-SBMA-IM-1 and REQ-SBMA-IM-2 in clause 5.5.2 and proposes to:

-	Define the Inventory NRMs, which are static for read only, for 5G which are communicated between MnS consumer and MnS producer;

-	Reuse the content of 4G Inventory NRMs (i.e., InventoryUnit, InventoryUnitNE and InventoryUnitHw defined in TS 28.632 [15]) as a basis to model 5G inventory NRMs.

### 5.5.4	Evaluation of potential solutions

This clause provides an evaluation of the potential solution described in clause 5.5.3, which proposes reusing the inventory NRMs defined in TS 28.632 [15] to support hardware related inventory management in the SBMA framework. The solution aims to address REQ-SBMA-IM-1 and REQ-SBMA-IM-2 in clause 5.5.2 by enabling MnS based inventory management for 5G systems.

Although the scope of TS 28.631 [14] and TS 28.632 [15] is limited to the communication between IRPAgent and IRPManager, the information model content defined in TS 28.632 [15] remains applicable for representing hardware inventory in a technology agnostic manner. The inventory NRMs defined in TS 28.632 [15], including InventoryUnit, InventoryUnitNE, and InventoryUnitHw, provide a generic and extensible representation of hardware components. These models are largely independent of the underlying RAT or network generation, and therefore can be reused as a basis for 5G hardware inventory representation.

The evaluation concludes that the proposed solution is technically feasible, aligns with SBMA architectural principles, and minimizes specification effort by reusing existing models. It effectively addresses REQ-SBMA-IM-1 and REQ-SBMA-IM-2 by enabling:

-	A standardized representation of 5G hardware inventory;

-	MnS based exposure of inventory information;

-	Cross-generation consistency between 4G and 5G inventory models.

The solution is therefore considered a candidate for supporting hardware inventory management within the SBMA framework.

## 5.6	Use case #6: Merge operation

### 5.6.1	Description

The changeMOIs operation and the notifyMOIChanges notification (as described in TS 28.532 [12]) both support multiple operation types, defined by the "modifyOperator" in changeMOIs and as "op" in notifyMOIChanges. These operations are optimized for handling individual data nodes e.g. replacing a full object with all its attributes or updating a single attribute (or attribute field).

A common use case is updating a multiple attributes of multiple MOIs while leaving other attributes of the same MOIs unchanged. This is not properly handled and documented.

#### 5.6.1.1		Problem when updating multiple MOIs

The changeMOIs operation should be used, as using individual createMOI, deleteMOI and modifyMOIAttributes operations would require using multiple operations requiring more processing and network resources. See TS 28.532 [12] clause 11.1.1.4a.

Figure 5.6.1.1-1 shows the following example where an immediate heartbeat is ordered in one ManagedElement and multiple attributes of a PerfMetricJob are updated in another ManagedElement.

Figure 5.6.1.1-1: Example configuration, modifying two separate objects

As multiple MOIs are to be modified by changeMOIs the

- baseObjectInstance shall identify a common ancestor MOI for all MOIs to-be-modified or to-be-reported, this could potentially be the root ManagedElement or the SubNetwork. As the ManagedElements may belong to different SubNetworks the baseObjectInstance can only be set to a common ancestor Subnetwork:

"baseObjectInstance": "SubNetwork=Buda-1"

The rest of the path to each argument must be defined in the "path" element using long multi-segment strings. Each "path" element needs to specify the path from the baseObjectInstance to the individual attribute or attribute field. E.g. for updating 5 attributes the following 5 "paths" have to be created and parsed:

"path": "/SubNetwork=Lagymanyos/ManagedElement=BudaME1/NtfSubscriptionControl=sub1/HeartbeatControl=hb1/triggerHeartbeatNtf"

"path": "/SubNetwork=Obuda/ManagedElement=BudaME17/PerfMetricJob=pmJob1/administrativeState"

"path": "/SubNetwork=Obuda/ManagedElement=BudaME17/PerfMetricJob=pmJob1/granularityPeriod"

"path": "/SubNetwork=Obuda/ManagedElement=BudaME17/PerfMetricJob=pmJob1/objectInstances"

"path": "/SubNetwork=Obuda/ManagedElement=BudaME17/PerfMetricJob=pmJob1/scheduler"

- "node Value" shall contain the new value of each attribute(element) e.g. true, UNLOCKED ,30, SubNetwork=SubNetwork=Buda-1,Scheduler=Sched-1

This means that for each updated attribute(element) the consumer has to produce a long "path" element which the consumer has to read/parse.

A computationally cheaper solution would be to use the "path" element to address each modified MOI, and let the "nodeValue" element contain all attribute changes; to have an operation that allows modifying multiple attributes of an MOI while leaving other attributes of the same MOI unchanged. That would allow the consumer to address each MOI only once. In the example above that would mean creating/parsing only 2 "path" elements instead of 5.

In Netconf, JSON Merge patch or the 3GPP JSON Merge patch this is possible using the "merge" operation. While this is described both for the Netconf_YANG and for the OpenAPI solution sets, it is not described on stage-2.

Note: While the above example is about a SubNetwork level producer, the same problem is valid for ManagedElement level producers as even inside a single ManagedElement there are IOCs that are embedded in the ManagedElement 5 levels deep.

#### 5.6.1.2		Problem use case when notifying the update of multiple MOIs

When reporting the changes made in the example of the previous clause the notification notifyMOIChanges should be used. The alternative, using some of the notifications notifyMOICreation, NotifyMOIDeletion, notifyMOIAttributeValueChanges, would mean sending multiple notifications instead of one, which requires more processing and network resources.

When using notifyMOIChanges just as described above we will need to set

"objectInstance": "SubNetwork=Buda-1"

5 separate moiChanges values will be needed, where the "path" element will specify the path to each attribute separately. (The replace operation towards the PerfMetricJob=pmJob1 MOI cannot be used as that would delete all not listed attributes.)

It would be good if we could collect all the attribute changes for a single MOI into one moiChanges value, thereby requiring only a single "path" element for each MOI to be created/parsed even if multiple attributes are updated.

An operation similar to "merge" would help. However, for notifyMOIChanges there is no such operation defined on stage-2 or stage-3.

With a proposed "merge" operation only 2 paths would need to be handled:

"path": "/SubNetwork=Lagymanyos/ManagedElement=BudaME1/NtfSubscriptionControl=sub1/HeartbeatControl=hb1"

"path": "/SubNetwork=Obuda/ManagedElement=BudaME17/PerfMetricJob=pmJob1"

### 5.6.2	Potential requirements

REQ-SBMA-MERGE-1: Add a simple solution for indicating the update of multiple attributes of multiple MOIs with a single notification while minimizing the need for parsing long "path" parameters.

### 5.6.3	Potential solutions

Two separate solutions are proposed for the problems described in clause 5.6.1.1 and 5.6.1.2.

a) Describe the merge operation for the changeMOIs operation on stage-2 in TS 28.532 [12].

Note: Exact details of merge shall not be described on stage-2 as they are solution set specific and already described in RFC 6241 [19] and in 3GPP TS 32.158 [18].

b) Add a new "merge" value to the "moiChanges.op" parameter of the notifyMOIChanges notification. This will allow

- the (base)ObjectInstance to identify a common ancestor MOI for all MOIs to-be-modified or to-be-reported.

- the use of  a single "moiChanges" value for each MOI thus avoiding the need to create/read a separate "path" element for each attribute. Within the single moiChanges value

- the "path" shall to the individual MOI

- the "value" element shall contain all modified attribute values (created, deleted, updated)

Editor’s note: How to report a deleted attribute within merge for YANG based schemas is FFS. Maybe null as a value can be used, although that’s not part of RFC 7951..

As attribute (elements) can have a complex structure too, the merge operation shall be applicable for any kind of target: MOI, attribute, attribute field, or attribute element.

The "value" element shall not contain any child MOIs, thereby ensuring that changes to separate MOIs are reported as separate "moiChange" items.

### 5.6.4	Evaluation of potential solutions

Only one potential solution has been identified for each of the two issues. The solution is feasible.

## 5.7	Use case #7: MnS selection in distributed deployment scenarios for SBMA

### 5.7.1	Description

3GPP management system introduces Service-Based Management Architecture in 5G which has improved system flexibility to adapt to different types of deployment scenarios. In SBMA, MnS producer and MnS consumer are two fundamental entities as defined in TS 28.533 [2]. According to definitions in and TS 28.537 [4] and TS 32.101 [20], figure 5.7.1-1 shows the discovery mechanism of MnS. MnS producer registers service to MnS registry. MnS consumer retrieves service from MnS registry. Then MnS consumer invokes service from MnS producer.

Figure 5.7.1-1. Discovery mechanism of MnS

In distributed deployment scenarios, MnS consumers may get multiple MnSs returned when retrieving MnSs from the MnS registry. MnS consumer has to decide which MnS to invoke. Figure 5.7.1-2 depicts the discovery mechanism in the distributed scenario. The current challenge is how an MnS consumer can select the most appropriate MnS from a set of discovered MnS(s) from the MnS registry.

Figure 5.7.1-2. Discovery mechanism in the distributed deployment scenario

### 5.7.2	Potential requirements

REQ-SBMA-DD-1: The 3GPP management system should have capability to assist the MnS consumer to select the appropriate MnS(s) from multiple discovered MnS(s).

### 5.7.3 	Potential solutions

As specified in clause 4.3.42 of TS 28.622[6], the MnSInfo IOC provides information that enables an MnS consumer to discover a management service.

For the purpose of MnS selection, the mnsCapability attribute of the MnSInfo IOC can be utilized by the MnS consumer when performing an “MnS discovery request” towards the MnS Registry. This attribute allows the MnS consumer to retrieve only those MnS producer instances that support the requested MnS capability.

### 5.7.3	Evaluation of potential solutions

The proposed solution adds some clarification text to the MnSInfo IOC, explaining how the mnsCapability attribute can be used by the MnS consumer for MnS selection. This clarification ensures that the MnS consumer can identify MnS producer instances offering specific MnS capabilities and satisfies the use case requirements in clause 5.7.2.

## 5.8	Use case #8: Clarification on the usage of MnSAgent

### 5.8.1	Description

The MnsAgent IOC is defined in TS 3GPP 28.622 [22], clause 4.3.2a as representing the Management-service (MnS) producers, including the supporting hardware and software, available for a certain management scope. The MnSAgent can be “name-contained” under an IOC as follows:

- ManagementNode;

- SubNetwork, if the SubNetwork does not contain a ManagementNode;

- ManagedElement, if it is the root element.

As per the information model, the MnsAgent IOC inherits from the Top_IOC  defined in TS 28.620 [23], and introduces a mandatory attribute systemDN which is readable and notifyable. MnSAgent is referenced in the Notification Header Parameter in TS 28.532 [3] which says “If an MnSAgent MOI is present, systemDN shall be the DN of an MnSAgent. If no MnSAgent is present the DN of the root MOI (e.g. ManagedElement or Subnetwork) shall be used. “

MnSAgent does not imply any particular orchestration, automation, or closed-loop behaviour by itself. It simply marks the availability of management services in the NRM model by grouping them under a scope.

Given that the root MOI DN can be used in all scenarios, and MnS scope information is already available in the MnSInfo IOC defined in TS 3GPP, 28.622 [22], clause 4.3.42, the MnsAgent IOC appears to offer no additional value in the current model.

### 5.8.2	Potential requirements

None

### 5.8.3	Potential solutions

Beyond its representation in the NRM, MnSAgent serves no additional functional purpose within the management framework. MnSAgent is not a MnF in the sense of a logical or functional module, it is more like an optional “container / descriptor” for one or more MnS producers. Although the systemDN attribute is mandatory in the MnsAgent managed object, the conditions for using MnSAgent are not clearly specified; if no MnSAgent is instantiated, the DN of the root MOI (e.g. ManagedElement or Subnetwork) shall be used.

As the conditions for using the MnSAgent are not clearly specified, this solution proposes:

- Deprecate MnSAgent in TS 28.622 [22] and TS 28.623 [24] in Rel-20

- Delete MnsAgent in all specification in Rel-21 and ongoing TR which reference MnsAgent

  - 3GPP TS 28.622 [22] – Generic NRM IRP; Information Service

  - 3GPP TS 28.623 [24] – Generic NRM IRP; Solution Set definitions

  - 3GPP TS 28.532 [3] – Management and orchestration; Generic management services

  - 3GPP TS 28.111 [25] – Fault Management (FM) MnS

  - 3GPP TS 32.161 [26] – Management and orchestration; JSON expressions (Jex)

### 5.8.4	Evaluation of potential solutions

The proposed solution satisfies the objectives stated in clause 5.8.3 and is proposed for the normative work. The proposed solution doesn't impact current functionality, as root MOI DNs already serve all scoping needs, and no existing deployments rely on MnSAgent as a functional entity.

## 5.9	Use case #9: Define the management and orchestration model separate from the network resource models allowing multiple subnetwork support

### 5.9.1 Description

An operator’s preference to partition their network (i.e. into multiple subnetworks) should not impact their ability to manage their network.

A management system should be capable of simultaneously managing multiple resource model instances (i.e. subnetworks). There are however limitations in the current modelling of the NRM which prevents this

For example, an instance of PerfMetricJob is contained within a particular SubNetwork/ME:

This creates an artificial split of management objects into multiple instances (e.g. PM jobs, notification subscriptions) to provide management across subnetworks as each management object instance (e.g. PerfMetricJob MOI) can only be configured to manage entities within the same SubNetwork/ME.

For example, two different PerfMetricJob instances are required to allow collection across two subnetworks:

http://operatorA.com/3gppManagement/pm/ProvMnS/v18/SubNetwork=1/.../PerfMetricJob=1
	- PerfMetricJob MOIs with scope of SubNetwork=1

http://operatorA.com/3gppManagement/pm/ProvMnS/v19/SubNetwork=2/.../PerfMetricJob=1
- PerfMetricJob MOIs with scope of SubNetwork=2

To better support management system implementations capable of consistently managing multiple resource model instances, the objects required by the management system related to management should be able to be fully decoupled from the NRM models they are managing.

### 5.9.2 Potential requirements

REQ-MNS-MgmtModel-1: A management model for the management of the 3GPP (managed) resources shall be defined and exposed by 3GPP management system.

REQ-MNS-MgmtModel-2: The 3GPP management model shall be defined separately from the resource models.

REQ-MNS-MgmtModel-3: The 3GPP management model shall be capable of managing multiple resource model instances.

REQ-MNS-MgmtModel-4: The 3GPP management model shall be versioned independent of the resource model versions.

### 5.9.3 Potential solutions

#### 5.9.3.1 Potential solution #1: Add Management and Orchestration Entity Model

A new ‘Management and Orchestration Entity’ (MOE) model is defined.

This new IOC MOE would be a root element implemented in a 3GPP Mgmt System with specific management scope of specified root Subnetwork and ManagedElement (accessible from the 3GPP Mgmt System) instances being managed by a set of 3GPP management and orchestration services. Multiple instances of MOE can be deployed for different management scopes.

Objects currently defined purely for management purposes would be updated to allow name-containment under MOE, including:

- MnsRegistry

- MnsAgent

- NtfSubscriptionControl

- IntentHandlingFunction

- MDAFunction

Objects defined to support management of network resources would allow optional name-containment under MOE, in addition to their current NRM containments:

- PerfMetricJob

- TraceJob

- ManagementDataCollection

- ThresholdMonitor

- AlarmList

The main change would be the name-containment of these IOCs.  Some of these objects already contain attributes such as objectInstances and rootObjectInstances which can continue to be used to define their scope.

NOTE:  the specific list of IOCs and names is subject to change during the study phase

Following is potential attributes definition for IOC MOE,

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| vendorName | M | T | F | F | T |
| userDefinedState | M | T | T | F | T |
| locationName | M | T | F | F | T |
| swVersion | M | T | F | F | T |

#### 5.9.3.1 Potential solution #2: Redefine ManagementNode NRM

Similar to Solution 1 but instead of defining a new IOC, the existing ManagementNode IOC would be redefined as the management system model implemented in a 3GPP Mgmt System.

ManagementNode IOC would become a root element with management scope of specified root Subnetwork and ManagedElement instances (accessible from the 3GPP Mgmt System) being managed by a common set of 3GPP management services.

All IOCs comprising the management objects currently residing within the common NRM would be updated to allow name-containment within ManagementNode:

Objects currently defined purely for management purposes would be updated to allow name-containment to be name-contained under ManagementNode including:

- MnsRegistry

- MnsAgent

- NtfSubscriptionControl

Objects defined to support management of network resources would allow optional name-containment under ManagementNode, in addition to their current containments.

- PerfMetricJob

- TraceJob

- ManagementDataCollection

- ThresholdMonitor

The main change would be the name-containment of these IOCs.  Some of these objects already contain attributes such as objectInstances and rootObjectInstances which can continue to be used to define their scope.

### 5.9.4 Evaluation of potential solutions

TBD

## 5.10	Use case #10: Define multiple release support

### 5.10.1 Description

An operator’s preference to support multiple versions (e.g. R20, R19) in parallel should not impact their ability to manage their network.

A management system should be capable of simultaneously managing multiple resource model instances of mixed versions in a consistent manner.  There are however limitations in the current modelling of the NRM which prevents this.

Due to a lack of explicit definition in the current specifications, one conclusion that can be drawn is that the inlining of the management related objects in the version specific subtrees of the objects they manage couples the management objects to the same version as the subtree which contains them.  Another interpretation is that MOIs of different versions can be mixed in the same data node tree.  This should be clarified in the specifications.

For example, an instance of PerfMetricJob is contained and versioned same as the SubNetwork/ME which contains it:

The current schema definitions are also separated into version specific definitions, and each MnS version only refers to the one defined for the same release as itself.  For example, a v18 ProvMnS has v18 definitions of the management objects:

Despite providing common management functionality these objects are needlessly of different versions and the management system must use version specific definitions tied to the version of the resources being managed.  This creates an artificial split of management objects into multiple instances (e.g. PM jobs, notification subscriptions) to provide management across subnetworks or releases.

Meaning that even in cases where a generic management object definition (e.g. PerfMetricJob IOC) could be used to manage multiple resource releases there are still version specific definitions provided and resource namespace specific instances.  This prevents the management services (e.g. ProvmMnS) from providing common managementt.

At present, two different ProvMnS would need to be exposed, to allow management of v18 and v19 management objects:

http://operatorA.com/3gppManagement/pm/ProvMnS/v18/SubNetwork=1/.../PerfMetricJob=1
	- PerfMetricJob MOIs using a v18 specific version

http://operatorA.com/3gppManagement/pm/ProvMnS/v19/SubNetwork=2/.../PerfMetricJob=1
- PerfMetricJob MOIs using a v19 specific version

This also means that each version-specific management object instance (e.g. PerfMetricJob MOI) can only be configured to manage entities of the same release

To better support management system implementations capable of consistently managing multiple resource model versions, the management system object versioning should be decoupled from the NRM models they are managing.

### 5.10.2 Potential requirements

REQ-MNS-MgmtModelVersion-1: The 3GPP management system shall be capable of managing multiple versions of resource models.

### 5.10.3 Potential solutions

TBD

### 5.10.4 Evaluation of potential solutions

TBD

## 5.11	Use case #11: PNF Equipment modelling

### 5.11.1	Description

There is a need to model managed PNF HW used to host telecommunication functionality including, but not limited to, computation equipment units and radio units.

### 5.11.2	Potential requirements

REQ-Dev-Mod-1: The 3GPP management system should provide an SBMA based management capability for managed, non-virtualized PNF equipment.

### 5.11.3	Potential solutions

Extend the NRM models with IOCs representing managed, non-virtualized PNF equipment. The model should contain the managed equipment themselves and the ports and links inter-connecting them.

Accordingly, the models should allow for computation units hosting core and/or RAN functionality, AI functionality, sensing functionality, edge applications dependent on co-localization with telecommunication functionality, radios, antennas, other auxiliary equipment like a cooling or power units. The models should also allow for interconnecting the units into equipment topology trees. Relations between equipment units should be specified using associations rather than containment structures.

### 5.11.4	Evaluation of potential solutions

Only one potential solution has been identified. The solution is feasible.

# 6	Conclusions and recommendations

## 6.1	Use case #1: Integration of SBMA with 5GC and 5G Access Network architecture

## 6.2	TBD

TBD

## 6.3	Use case #3: Historical alarms

The requirements for historical alarms are listed in clause 5.3.2.

All requirements are recommended for normative work based on the potential solution described in clauses 5.3.3 according to the conclusions documented in clause 5.3.4.

## 6.4	TBD

TBD

## 6.5	TBD

TBD

## 6.6	Use case #6: Merge operation

The requirement for the merge operation is specified in clause 5.6.2. A potential solution is described in clause 5.6.3.

The requirement and the solution are recommended for normative work.

## 6.7	Use case#7: MnS selection in distributed deployment scenarios for SBMA

In clause 5.7, the study has analysed the use case, corresponding requirement and potential solution to enable MnS selection by MnS consumers in distributed deployment scenarios for SBMA. The proposed solution in clause 5.7.X adds clarification text to the MnSInfo IOC definition, explaining how the mnsCapability attribute can be used by an MnS consumer for MnS selection.

At the normative phase, it is recommended to add this clarification text to clause 4.3.42.1 of TS 28.622[6].

## 6.8	Use case #8: Clarification on the usage of MnSAgent

TBD

## 6.9	TBD

TBD

## 6.10	TBD

TBD

## 6.11	Use case #11: PNF Equipment modelling

The requirement for the equipment modelling is specified in clause 5.11.2. A potential solution is described in clause 5.11.3.

The requirement and the solution are recommended for normative work.

# Annex A (informative): PlantUML source code

## A.1	Relationship UML diagram for PNF software management

@startuml

'title " Relationship UML diagram for PNF software management "

hide circle

hide methods

hide members

skinparam class {

AttributeIconSize 0

BackgroundColor white

BorderColor black

ArrowColor black

}

skinparam   Shadowing false

skinparam  Monochrome true

skinparam  ClassBackgroundColor White

skinparam  NoteBackgroundColor White

class "<<ProxyClass>> \n  ManagedEntity " as ManagedEntity{}

class "<<InformationObjectClass>>\n SwPnfJob " as SwPnfJob {}

class "<<InformationObjectClass>>\n SwPnfProcess" as SwPnfProcess{}

class "<<InformationObjectClass>>\n SwPnfPackage" as SwPnfPackage{}

class "<<InformationObjectClass>>\n SwMCapabilities " as SwMCapabilities {}

ManagedEntity "1" *-- "*" SwPnfPackage : <<names>>

ManagedEntity "1" *-- "*" SwPnfJob : <<names>>

SwPnfJob "1" *-d- "*" SwPnfProcess : <<names>>

ManagedEntity "1" *-- "1" SwMCapabilities : <<names>>

SwPnfJob "*" -left-> "*"SwPnfPackage

note left of ManagedEntity

Represents the folllowing IOCs:

SubNetwork, ManagedElement

end note

@enduml

## A.2	Inheritance UML diagram for PNF software management

@startuml

'title Inheritance UML diagram for PNF software management

hide circle

hide methods

hide members

skinparam class {

AttributeIconSize 0

BackgroundColor white

BorderColor black

ArrowColor black

}

skinparam   Shadowing false

skinparam  Monochrome true

skinparam  ClassBackgroundColor White

skinparam  NoteBackgroundColor White

class "<<InformationObjectClass>>\n Top" as Top{}

class "<<InformationObjectClass>>\n SwPnfPackage " as SwPnfPackage {}

class "<<InformationObjectClass>>\n SwPnfJob" as SwPnfJob{}

class "<<InformationObjectClass>>\n SwPnfProcess" as SwPnfProcess{}

class "<<InformationObjectClass>>\n SwMCapabilities" as SwMCapabilities{}

Top <|--SwPnfPackage

Top <|--SwPnfJob

Top <|--SwPnfProcess

Top <|--SwMCapabilities

@enduml

## A.3	SwPnfPackage state diagram

@startuml

title SwPnfPackage state diagram

[*] -d-> Remote : CreateMOI

Remote -d----> [*] : DeleteMOI

Remote -r---> Downloaded : Download

Downloaded --right--> Verified : Verify

Verified -d-> Installed : Install

Installed -d-> Activated : Activate

Activated --> Installed : The activation of another SwPnfPackage \n  results in the deactivation of this SwPnfPackage,\n as a side effect

Activated --> Installed : Deactivate

Activated --> Installed : Fallback

Downloaded -l----> Remote : Delete

Verified --> Remote : Delete

Installed --> Remote : Delete

@enduml

## A.4	Procedures UML diagram for PNF software management

@startuml

title " Procedures for PNF software management "

participant "SWM MnS Consumer" as SC

participant "SWM MnS Producer" as SP

group Create a SwPnfPackage instance

SC -> SP: 1. Request to create a SwPnfPackage instance \n (list of attributes of SwPnfPackage IOC)

SP -> SP: 2. Create and configure SwPnfPackage MOI

SP -> SC: 3. Response for creating an SwPnfPackage instance

end

group Create a SwPnfJob instance

SC -> SP: 4. Request to create a SwPnfJob instance \n (list of attributes of SwPnfJob IOC)

SP -> SP: 5. Create and configure SwPnfJob MOI

SP -> SC: 6. Response for creating an SwPnfJob instance

end

loop  [Corresponding RAN NE start its software management process]

SP -> SP: 7. Create an SwPnfProcess instance for the software management process \nand configure the created SwPnfProcess instance with \n DN of associated SwPnfJob instance

SP -> SC: 8. Notify the creation of SwPnfProcess instance

opt Configure the Stop Point for SwPnfProcess

SC -> SP: 9. Request to configure attribute stopPointSetIndication\n of the SwPnfProcess instance with value "TRUE"

loop

SP -> SP: 10. Stop hanlding the SwPnfProcess \nwhen the value of attribute progressStateInfo is changed

SP -> SC: 11. Notify the value change of SwPnfProcess instance

SC -> SP: 12. Request to configure attribute stopPointStatus\n of the SwPnfProcess instance to continue the process

end

end

alt  [obtain the progress and result by subscribe-notification method]

loop

SP -> SC: 13a. Notify the progress and result information

end

else [[obtain the progress and result by query method]]

SC -> SP: 13b-1. Request to query the SwPnfProcess information (DN of SwPnfProcess)

SP -> SC: 13b-2. Response with the SwPnfProcess information

end

opt Terminate a SwPnfProcess

SC -> SP: 14. Request to configure "cancelScProcess" attribute \n of the SwPnfProcess with value "TRUE"

SP -> SC: 15. Response with SwPnfProcess configuration result

end

|||

SC -> SP: 16. Request to delete a SwPnfProcess instance \n software management process must be completed or terminated

SP -> SP: 16a. Delete SwPnfProcess MOI

SP->SC: 17. Response for deleting an SwPnfProcess instance

end

group Delete a SwPnfJob instance

SC -> SP: 18. Request to delete a SwPnfJob instance \n (DN of SwPnfJob MOI)

SP -> SP: 19. Delete SwPnfJob MOI

SP -> SC: 20. Response for deleting an SwPnfJob instance

end

skinparam sequenceActorBackgroundColor #FFFFFF

skinparam sequenceParticipantBackgroundColor #FFFFFF

skinparam noteBackgroundColor #FFFFFF

autonumber "#'.'"

skinparam monochrome true

skinparam shadowing false

hide footbox

@enduml

# Annex B: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA5#162 |  |  |  |  | Initial skeleton | V0.0.0 |
| 2025-10 | SA5#163 | S5-254454 |  |  |  | pCR TR 28.884 Add structure for SBMA study report | V0.1.0 |
| 2025-10 | SA5#163 | S5-254676 |  |  |  | pCR TR 28.884 Add introduction for SBMA study report | V0.1.0 |
| 2025-10 | SA5#163 | S5-254677 |  |  |  | pCR TR 28.884 Add scope for SBMA study report | V0.1.0 |
| 2025-10 | SA5#163 | S5-254678 |  |  |  | pCR TR 28.884 Add concepts and background | V0.1.0 |
| 2025-10 | SA5#163 | S5-254681 |  |  |  | pCR TR 28.884 Use case on integration of SBMA with 5GC and 5G Access Network architecture | V0.1.0 |
| 2025-10 | SA5#163 | S5-254682 |  |  |  | pCR TR 28.884 Management data streaming based on message bus | V0.1.0 |
| 2025-10 | SA5#163 | S5-254732 |  |  |  | New KI on Historical alarms | V0.1.0 |
| 2025-10 | SA5#163 | S5-254844 |  |  |  | pCR TR 28.884 Use case on software management | V0.1.0 |
| 2025-10 | SA5#163 | S5-254845 |  |  |  | pCR TR 28.884 Use case on inventory management | V0.1.0 |
| 2025-11 | SA5#164 | S5-255534 |  |  |  | pCR TR 28.884 Update use case on management data streaming based on message bus | v0.2.0 |
| 2025-11 | SA5#164 | S5-255535 |  |  |  | pCR TR 28.884 Message Bus principles integrated in SBMA | v0.2.0 |
| 2025-11 | SA5#164 | S5-255537 |  |  |  | pCR TR 28.884 Management data streaming based on message bus potential solution 2 | v0.2.0 |
| 2025-11 | SA5#164 | S5-255541 |  |  |  | pCR TR 28.884 Add new use case on wide-area distributed deployment scenarios for SBMA | v0.2.0 |
| 2025-11 | SA5#164 | S5-255542 |  |  |  | pCR TR 28.884 SW Management additions | v0.2.0 |
| 2025-11 | SA5#164 | S5-255544 |  |  |  | pCR TR 28.884 Solution for inventory management | v0.2.0 |
| 2025-11 | SA5#164 | S5-255545 |  |  |  | pCR TR 28.884 Historical alarm list requirements | v0.2.0 |
| 2025-11 | SA5#164 | S5-255546 |  |  |  | Rel-20 pCR 28.884 Add NRM based solution for historical alarms | v0.2.0 |
| 2025-11 | SA5#164 | S5-255580 |  |  |  | pCR TR 28.884 Merge operation | v0.2.0 |
| 2025-11 | SA5#164 | S5-255581 |  |  |  | Rel-20 pCR 28.884 Add Management data based solution for historical alarms | v0.2.0 |
| 2026-02 | SA5#165 | S5-260197 |  |  |  | pCR TR 28.884 Add solution for integration of SBMA with 5GC and 5G Access Network architecture | v0.3.0 |
| 2026-02 | SA5#165 | S5-260198 |  |  |  | pCR TR 28.884 Evaluation of potential solutions for inventory management | 0.3.0 |
| 2026-02 | SA5#165 | S5-260200 |  |  |  | Rel-20 pCR TR 28.884 Add conclusion and recommendation for MnS selection in distributed deployment scenarios for SBMA | 0.3.0 |
| 2026-02 | SA5#165 | S5-260205 |  |  |  | Pseudo CR on clarification on the usage of MnSAgent | 0.3.0 |
| 2026-02 | SA5#165 | S5-260339 |  |  |  | pCR TR 28.884 Merge operation conclusion | 0.3.0 |
| 2026-02 | SA5#165 | S5-260383 |  |  |  | pCR TR 28.884 Update of management data streaming based on message bus | 0.3.0 |
| 2026-02 | SA5#165 | S5-260464 |  |  |  | pCR TR 28.884 Add potential requirements for management data streaming based on message bus | 0.3.0 |
| 2026-02 | SA5#165 | S5-260715 |  |  |  | Rel-20 pCR TR 28.884 Add potential solution and evaluation for MnS selection in distributed deployment scenarios for SBMA | 0.3.0 |
| 2026-02 | SA5#165 | S5-260716 |  |  |  | pCR on TR 28.884 Add UC for management model | 0.3.0 |
| 2026-02 | SA5#165 | S5-260717 |  |  |  | pCR TR 28.884 Add solution for software management | 0.3.0 |
| 2026-02 | SA5#165 | S5-260718 |  |  |  | pCR TR 28.884 Historical alarm list solution | 0.3.0 |
| 2026-02 | SA5#165 | S5-260719 |  |  |  | pCR TR 28.884 Equipment model | 0.3.0 |
| 2026-02 | SA5#165 | S5-260720 |  |  |  | Rel-20 pCR 28.884 Autonomous software management | 0.3.0 |
| 2026-02 | SA5#165 | S5-260721 |  |  |  | pCR TR 28.884 Add solution for integration of SBMA with 5GC and 5G Access Network architecture | 0.3.0 |
| 2026-02 | SA5#165 | S5-260835 |  |  |  | pCR TR 28.884 Evaluation of potential solutions for inventory management | 0.3.0 |