| 3GPP TR 28.885 V0.4.0 (2026-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Study on energy efficiency and energy saving aspects of 5G advanced networks (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Concepts and Background	8

5	Use Cases	8

5.1	Enhancements to support energy efficiency as a service criteria	8

5.2	Enhancements to support the information required by Energy Information Function (EIF)	14

5.3	Improve energy saving, energy efficiency and reducing carbon footprint of 5G network	18

5.4	Enhancements to Energy Consumption and Energy Efficiency measurements and KPIs	18

6	Conclusions and recommendations	26

6.1	Enhancements to support energy efficiency as a service criteria	26

6.2	Enhancements to support the information required by Energy Information Function (EIF)	26

6.3	Improve energy saving, energy efficiency and reducing carbon footprint of 5G network	27

6.4	Enhancements to Energy Consumption and Energy Efficiency measurements and KPIs	27

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document is to study energy efficiency and energy saving aspects of 5G Advanced networks.

# 1	Scope

The present document  identifies potential use cases and requirements for enhancements to 3GPP management system to support requirements related to energy efficiency as a service criteria (defined in TS 22.261 [2]), to support information required by Energy Information Function (EIF) (see TR 23.700-67 [9]) and enhancements to Energy Consumption and Energy Efficiency measurements and KPIs. It documents and evaluates potential solutions and provides recommendations for the normative work.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[4]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[5]	ETSI GS OEU 020 (v1.1.1): "Operational energy Efficiency for Users (OEU); Carbon equivalent Intensity measurement; Operational infrastructures; Global KPIs; Global KPIs for ICT Sites".

[6]	ETSI ES 202 706-1 V1.7.1 (2022-08): "Environmental Engineering (EE); Metrics and measurement method for energy efficiency of wireless access network equipment; Part 1: Power consumption - static measurement method".

[7] 	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[8]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[9]	3GPP TR 23.700-67: "Study on Energy Efficiency and Energy Saving; Phase 2".

[10]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[11]	3GPP TS 32.130: " Telecommunication management; Network sharing; Concepts and requirements".

[12]	ETSI ES 202 336-1 V1.2.1: "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 1: Generic Interface"

[13]	ETSI ES 202 336-2 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 2: DC power system control and monitoring information model"

[14]	ETSI ES 202 336-3 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 3: AC UPS power system control and monitoring information model"

[15]	ETSI ES 202 336-4 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 4: AC distribution power system control and monitoring information model"

[16]	ETSI ES 202 336-5 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 5: AC diesel back-up generator system control and monitoring information model"

[17]	ETSI ES 202 336-6 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 6: Air Conditioning System control and monitoring information model"

[18]	ETSI ES 202 336-7 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 7: Other utilities system control and monitoring information model"

[19]	ETSI ES 202 336-8 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 8: Remote Power Feeding System control and monitoring information model"

[20]	ETSI ES 202 336-9 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 9: Alternative Power Systems"

[21]	ETSI ES 202 336-10 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 10: AC inverter power system control and monitoring information model"

[22]	ETSI ES 202 336-11 V1.1.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 11: Battery system with integrated control and monitoring information model"

[23]	ETSI ES 202 336-12 V1.2.1 "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks) Part 12: ICT equipment power, energy and environmental parameters monitoring information model"

[24]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[25]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[26]	3GPP TS 38.401: "NG-RAN; Architecture description".

[27]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

carbon emission: quantity of equivalent carbon dioxide emitted (e.g. kg of CO2 equivalent).

NOTE 1:	This definition is taken from 3GPP TS 22.261 [2].

energy availability: the remaining amount of energy (e.g. in kWh) locally available for consumption. For devices, network elements and functions, energy availability may be limited and/or intermittent, in particular when relying on batteries and/or renewable energy sources (e.g. off-grid base stations, satellites etc) or during power grid heavy load or disruptions.

NOTE 2:	This definition is taken from 3GPP TS 22.261 [2].

energy capacity: the maximum amount of energy (e.g. in kWh) that can be locally available for consumption (either locally produced and/or stored) by a device or a network element or function.

NOTE 3:	This definition is taken from 3GPP TS 22.261 [2].

Energy Consumption (EC): integral of power consumption over time.

NOTE 4: This definition is taken from TS 28.310 [3]

Energy Efficiency (EE): ratio between performance and energy consumption.

NOTE 5: The performance may be measured based on e.g. data volume, latency, number of active users, etc.

NOTE 6:	This definition is taken from 3GPP TS 28.310 [3].

energy rationing: A situation in which the availability of energy either across the network or at a particular network element or function is limited or reduced.

NOTE 7:	This definition is taken from 3GPP TS 22.261 [2].

energy-related characteristics: information which characterize the energy to power the operator’s network in terms of energy consumption, energy supply mix, carbon footprint, energy capacity and availability conditions.

NOTE 8:	Which energy-related characteristics are relevant depends on the scenario.

Network Slice: A logical network that provides specific network capabilities and network characteristics.

NOTE 90:	 This definition is taken from 3GPP TS 23.501 [8].

renewable energy: energy from renewable sources as energy from renewable non-fossil sources, namely wind, solar, aerothermal, geothermal, hydrothermal and ocean energy, hydropower, biomass, landfill gas, sewage treatment plant gas and biogases

NOTE 10:	This definition is taken from TS 22.261 [2].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

EC	Energy Consumption

EE	Energy Efficiency

EIF	Energy Information Function

S-NSSAI	Single Network Slice Selection Assistance Information

# 4	Concepts and Background

Climate change and the rise in total energy usage due to increased traffic and dense networks have motivated operators for having mechanisms in place that can lead to enhanced energy saving and energy efficiency in a network. Energy efficiency is thus, a strategic priority for telecom operators around the world.

In the previous releases, 3GPP management system has specified use cases, requirements and solutions, based on management mechanisms, related to energy efficiency and energy saving (see TS 28.310 [3]). Energy Consumption (EC) and Energy Efficiency (EE) related measurements (see TS 28.552 [10]) and KPIs (see TS 28.554 [4]) are also specified for the managed entities.

SA1 TS 22.261 [2] includes enhancements related to collection and exposure (to authorized consumers) of network energy-related characteristics, handling of energy rationing, network internal optimization actions utilizing the network energy-related characteristics. SA2 study on Rel-20 (see TR 23.700-67 [9]), includes potential dependencies on 3GPP management system.

# 5	Use Cases

## 5.1	Enhancements to support energy efficiency as a service criteria

### 5.1.1	Use case #1: Enhancements to support the energy-related characteristics for Network Elements and Network Functions

#### 5.1.1.1	Description

Energy-related characteristics, energy availability, energy capacity, energy supply mix, carbon emission and carbon intensity are defined in TS 22.261 [2]. Several new use cases and requirements have been introduced in clause 6.15a of TS 22.261 [2] that utilize this energy-related characteristics such as adjustments to communication service, information exposure, performance requirements related to the exposure of network energy-related characteristics, network internal optimization as part of Rel-20 enhancements.

Energy-related characteristics are not reported by the network to the 3GPP management system and are required to be obtained by the operator from external sources and associated into the 3GPP management system for the 3GPP network to utilize this information. 3GPP management system currently support parts of energy-related characteristics. This use case identifies the energy-related characteristics that are currently supported by 3GPP management system.

Energy supply information can be associated with the Managed Elements (representing the Network Elements) or SubNetwork using EnergyInfoGroup IOC (see clause 8.3.3. of TS 28.310 [3]). Energy-related information for each energy source (see EnergySourceInfo defined in clause 8.3.2 of TS 28.310 [3]) of the energy supply (see EnergySupplyInfo defined in clause 8.3.1 of TS 28.310 [3]) includes information of different energy source, with source type, carbon emission factor, renewable and non-renewable energy information.

Energy consumption related KPIs are defined in clause 6.7.3 of TS 28.554 [4] and gNB Estimated Carbon Emission and NG-RAN Estimated Carbon Emission KPIs are defined in clause 6.7.7. of TS 28.554 [4]. Currently, the gNB Estimated Carbon Emission and NG-RAN Estimated Carbon Emission KPIs are limited to the scenario when they are powered using a single energy supply only. This use case is to study the gaps and enhancements to the energy-related characteristics associated with Managed Elements to satisfy the requirements in TS 22.261 [2]. This includes the following aspects for Network Elements and/or Network Functions:

1)	Carbon emission: Enhancements (if necessary) for the carbon emission information Network Element and Network Function that can be made available and/or estimated in 3GPP management system.

2)	Energy capacity: How energy capacity information of each energy supply of the Network Element can be made available and/or estimated in 3GPP management system.

3)	Energy availability: How energy availability information of each energy supply of the Network Element can be made available and/or estimated in 3GPP management system.

#### 5.1.1.2	Potential requirements

PREQ-Energy_Related_Characteristics-1: The 3GPP management system should be able to estimate and report the carbon emission information Network Element and Network Function.

PREQ-Energy_Related_Characteristics-2: The 3GPP management system should enable an authorized consumer to retrieve energy capacity information of each energy supply of the Network Element.

PREQ-Energy_Related_Characteristics-3: The 3GPP management system should enable an authorized consumer to retrieve energy availability information of each energy supply of the Network Element.

#### 5.1.1.3	Potential solutions

##### 5.1.1.3.1	Potential solution #1: Energy capacity and energy availability information as external management data

###### 5.1.1.3.1.1	Introduction

The NG-RAN Node, gNB or UPF defined in RAN and SA2 3GPP specification does not specify measurements of such energy capacity and energy availability related information. Energy capacity and energy availability information is external management data, as this data is not available or reported inherently by the 3GPP network elements. This information corresponds to the stored energy in the backup batteries. The management of the backup batteries for the Network Elements are specified in ETSI ES 202 336-1 [12] that defines Generic interface for monitoring and control of power, cooling and building environment systems, while the monitoring and control for specific power systems are defined in ETSI ES 202 336-2 [13] to ETSI ES 202 336-11 [22].

Managing external management data is specified in clause 6.4 of TS 28.537 [25] and solutions in TS 28.622 [24].

Monitoring and control of power, cooling and building environment systems is specified in ETSI ES 202 336-1 [12] to ETSI ES 202 336-12 [23].

This potential solution utilizes the framework for managing external management data and illustrates how this energy capacity and energy availability related external information can be made available within the 3GPP management system and be associated with the 3GPP network elements. Such information can be utilized within the 3GPP system to access this external information for further use within the 3GPP network.

###### 5.1.1.3.1.2	Description

Generic interface for monitoring and control of power, cooling and building environment systems is specified in ETSI ES 202 336-1 [12], while the monitoring and control for specific power systems are defined in ETSI ES 202 336-2 [13] to ETSI ES 202 336-11 [22], and measurement and monitoring of power, energy and environmental parameters for ICT equipment in telecommunications or datacenter or customer premises is defined in ETSI ES 202 336-12 [23].

The monitoring and control information in ETSI ES 202 336-1 [12] to ETSI ES 202 336-12 [23] includes energy capacity and energy availability information, as described below:

- Data as per ETSI ES 202 336-1 [12] to ETSI ES 202 336-12 [23]:

  - Data related to backup battery, e.g., Estimated remaining battery autonomy (time) during discharge, Estimated remaining battery capacity (due to ageing) at full charge, Battery age since date of installation, Battery discharge alarm duration, for any power supply system with battery back-up.

- Config as per ETSI ES 202 336-1 [12] to ETSI ES 202 336-12 [23]:

  - Data related to backup battery, e.g., Battery capacity.

The 3GPP network can utilize this external management data related to energy capacity and energy availability information, when associated with the applicable scope.

ExternalDataType IOC (defined in TS 28.622 [24]) specifies a type of external management data and the associated meta data, and the attribute externalDataScope (see TS 28.622 [24]) describes the concrete scope which the external management data is applicable.

The MOI of ExternalDataType can be used to represent the external management data from ETSI ES 202 336-1 [12] to ETSI ES 202 336-12 [23], and the attribute externalDataScope can include the list of DNs for which this information is applicable. This is illustrated in the following examples.

An illustration of using ExternalDataType MOI with example configurations of attributes for an Energy info group scope is described below:

- externalDataType: "Energy system"

- mediaLocation: < URI of the media, i.e., data from monitoring and control of power, cooling and building environment systems, which includes the transfer protocol >

- externalDataTypeSchema: < URI of the schema to parse monitoring and control of power, cooling and building environment systems of external management data.>

- externalDataScope: represented by ExternalDataScope data type with following attributes:

  - geoAreas: ""

  - objectInstancesIncluded: <List of EnergyInfoGroup DN(s)>

  - objectInstancesExcluded: ""

An illustration of using ExternalDataType MOI with example configurations of attributes for an Energy supply info (e.g., MOI of EnergySupplyInfo representing energy supply from a backup energy) scope is described below:

- externalDataType: "Energy system"

- mediaLocation: < URI of the media, i.e., data from monitoring and control of power, cooling and building environment systems, which includes the transfer protocol >

- externalDataTypeSchema: < URI of the schema to parse monitoring and control of power, cooling and building environment systems of external management data.>

- externalDataScope: represented by ExternalDataScope data type with following attributes:

  - geoAreas: ""

  - objectInstancesIncluded: <List of EnergySupplyInfo DN(s)>

  - objectInstancesExcluded: ""

##### 5.1.1.3.2	Potential solution #2: Enhancement to estimation of carbon emission information

###### 5.1.1.3.2.1	Introduction

This potential solution is to enhance the KPIs related to estimated carbon emission for scenarios where the network elements are powered using multiple energy supplies, and energy consumption (EC) can be measured at per energy supply granularity. This potential solution is for the requirement PREQ-Energy_Related_Characteristics-1.

The estimated carbon emission of a gNB, would be based on the energy consumption measured from each energy supply, the composition percentage of the energy sources of each energy supply, and the CEF of each energy source. For example, consider a gNB that can be powered from three energy supplies as follows:

1)	Grid provider A (GRID_ELECTRICITY), where energy is produced using energy sources coal (50%), solar (30%) and wind (20%).

2)	Grid provider B (GRID_ELECTRICITY), where energy is produced using energy sources nuclear (70%), solar (30%).

3)	Locally generated energy from solar panels (LOCALLY_GENERATED_ENERGY), where energy is produced using energy source solar (100%).

In this example, if the CEF configured by the operator is 0.8 kg CO2eq/kWh for coal, 0.01 kg CO2eq/kWh for nuclear, 0 kg CO2eq/kWh for solar and wind energy sources, and if, during the collection period, the EC for the gNB was 5kWh from energy supply #1, 15kWh from energy supply #2 and 7kWh from energy supply #3, then the estimated carbon emission of this gNB during the collection period would be:

ECEgnb = [(0.8 * (5 * 50%)) + (0.00 * (5 * 30%)) + (0.00 * (5 * 20%))] +

[(0.01 * (15 * 70%)) + (0.00 * (15 * 30%))] +

[(0.00 * (7 * 100%))]

= 2.105 kg CO2eq

###### 5.1.1.3.2.2	Description

gNB Estimated Carbon Emission and NG-RAN Estimated Carbon Emission KPIs are defined in clause 6.7.7. of TS 28.554 [4]. Currently, the gNB Estimated Carbon Emission and NG-RAN Estimated Carbon Emission KPIs are limited to the scenario when they are powered using a single energy supply only.

The following enhancements are proposed for the KPI defined in clause 6.7.7. of TS 28.554 [4], to enhance the KPI related to estimated carbon emission (ECEgNB) for scenarios where the gNB is powered using multiple energy supplies.

a)	ECEgNB.

b)	This KPI provides the Estimated Carbon Emission of a gNB (ECEgNB) over a time period. The Estimated Carbon Emission of a gNB (ECEgNB) is the sum of estimated carbon emission of a gNB from all energy supplies which powers the gNB. The KPI object, i.e., ManagedElement is associated with EnergySupplyInfo through IOC EnergyInfoGroup as described in TS 28.310 [3].

b-1)	Integer, kg CO2eq

b-2)	CUM

c)	This calculation formula is obtained as:

ECEgNB=energy supply ECEgNB_energySupply

where:

-	ECEgNB_energySupply is the Estimated Carbon Emissions of a gNB from one energy supply which is obtained by the sum of estimated carbon emission of a gNB from all energy sources of the energy supply which powers the gNB. Estimated carbon emission of a gNB from one energy source is the Energy Consumption of the gNB (ECgNB) proportional to the composition percentage of the energy source in the energy supply, as configured in the attribute energyCompositionPercentage, multiplied by the Carbon Emission Factor (CEF) of the energy source, as configured in energySourceCef (see TS 28.310 [3]).

ECEgNB_energySupply=energy sources CEFenergySource*(ECgNB_energySource*EnergySourceCompositionPercentage)

-	ECgNB_energySource is the Energy Consumption of the gNB using power from a particular energy supply. Its unit is kWh.

-	CEFenergySoure indicates Carbon Emission Factor of the energy source of the energy supply as configured in attribute energySourceCef (see TS 28.310 [3]). Its unit is kg CO2eq/kWh.

-	EnergySourceCompositionPercentage indicates the percentage of energy source type used by the energy supply, as configured in attribute energyCompositionPercentage (see TS 28.310 [3]).

NOTE:	The accuracy of this KPI is dependent on the accuracy of the CEF information configured by the operator.

#### 5.1.1.4	Evaluation of potential solutions

##### 5.1.1.4.1	Evaluation of potential #1: Energy capacity and energy availability information as external management data

The potential solution #1 Energy capacity and energy availability information as external management data (described in clause 5.1.1.3.1), illustrates how 3GPP network can utilize this external management data related to energy capacity and energy availability information, when associated with the applicable scope, utilizing the ExternalDataType IOC (defined in TS 28.622 [24]).

The potential solution #1 fulfils the requirement PREQ-Energy_Related_Characteristics-2 and PREQ-Energy_Related_Characteristics-3. This is a feasible candidate, to describe the solution in an informative annex in TS 28.310 [3], as input to the normative phase.

##### 5.1.1.4.2	Evaluation of potential #2: Enhancement to estimation of carbon emission information

The potential solution #2: Enhancement to estimation of carbon emission information (described in clause 5.1.1.3.1), proposes enhancements to the ECEgNB KPI (defined in clause 6.7.7. of TS 28.554 [4]) to support estimated carbon emission for scenarios where the network elements are powered using multiple energy supplies. This potential solution utilizes Energy Consumption of the gNB using power from a particular energy supply, and the EnergySourceCompositionPercentage and energySourceCef information configured by the operator (see TS 28.310 [3]).

The potential solution #1 fulfils the requirement PREQ-Energy_Related_Characteristics-1. This is a feasible candidate as input to the normative phase.

### 5.1.2	Use case #2: Management mechanisms to support service adjustments to adapt to energy-related characteristics and energy rationing

#### 5.1.2.1	Description

Under energy rationing constraints (i.e., when energy availability is limited or restricted), there is a potential risk for the entire network to shut-down causing service disruption. During such unavoidable hard constraints, the operators would like the network to provide services to their consumers to extend the time duration for which the services are available, and this could be by degrading the network service performance. For example, the operators might want to prioritize the voice and emergency services (e.g. eMBB) over some data services (e.g., URLLC or Gaming service or Video service). The 3GPP management system can utilize the information related to energy rationing and other energy-related characteristics (such as energy capacity and energy availability), and provide mechanisms to the operator to prevent their network from going out of service and extend the duration for which the services are available.

Energy rationing and energy-related characteristics are defined in TS 22.261 [2].

The requirement specified in clause 6.15a.2.2 of TS 22.261 [2] includes providing means to enable the operator to degrade service performance to meet energy rationing constraints.

Another requirement specified in clause 6.15a.2.2 of TS 22.261 [2] include providing mechanisms to adjust communication service considering change of energy supply mix of the network as one of the factors. For example, adjustments based on the carbon emissions of different network functions.

This use case is to study

-	management mechanisms to enable the operator to provide means to adapt the services provided by the network (e.g., by modifying the QoS requirements) to meet energy rationing control.

-	management mechanisms to enable the network to adjust communication service (e.g., activate or deactivate communication services) adapting to energy-related characteristics.

#### 5.1.2.2	Potential requirements

PREQ-Energy_Related_Service_Adjustment-1: The 3GPP management system should enable the operator to adapt the network performance of existing communication services to meet energy rationing control utilizing the available energy rationing information.

PREQ-Energy_Related_Service_Adjustment-2: The 3GPP management system should enable the network to adjust communication services adapting to energy-related characteristics.

#### 5.1.2.3	Potential solutions

None

#### 5.1.2.4	Evaluation of potential solutions

There were no potential solutions studied for this use case.

### 5.1.3	Use case #3: Energy Rationing Information Management

#### 5.1.3.1	Description

Clause 3.1 in TS 22.261 [2] defines energy rationing as a situation in which the availability of energy either across the network or at a particular network element or function is limited or reduced. This use case is for enabling management of energy rationing and energy saving in a network. The configuration of energy rationing information in a network can be used to apply controlled energy usage and energy-saving in an operator’s network. This information could be crucial for maintaining network performance and achieving energy saving objectives. This information can help a consumer to know which services can be modified if a node which has energy rationing applied on it is used.

The energy rationing-related information in the network can assist in prioritizing a node for selection over others in a given scenario. The cause due to which the energy rationing is applied in an operator’s network can be related to regulatory requirements, economic reasons, electricity brownouts, or energy blackouts (full outage of energy supplier). Economic reason refers to raising the price of power (energy units) by an energy supplier during certain periods or locations to force operators to use less energy. In such cases, the MNO might need to reduce energy usage to keep its services economical. Electricity brownout refers to partial power reduction, energy blackout refers to a total energy outage at the energy supplier’s end (e.g., due to faults), requiring the MNO to rely on local power backup sources (battery banks, generators, etc.) and potentially reduce energy consumption to continue providing services until power from the energy supplier resumes.

#### 5.1.3.2	Potential requirements

REQ-Energy_Rationing-CON-1: The 3GPP management system should enable its authorised consumers to provide energy rationing related information in an operator’s network.

#### 5.1.3.3	Potential solutions

##### 5.1.3.3.1	Potential solution #1: Energy Rationing Information Configuration

###### 5.1.3.3.1.1	Introduction

The proposed solution is to provide mechanism to configure information related to Energy Rationing as there exist none. Such information includes, but not limited to, energy rationing causes, maximum energy consumption limits, energy rationing limits of network nodes for particular locations, specific time duration and applicable services. A network operator can use it to apply controlled energy usage and energy-saving in their network. This could involve ensuring that only network elements or functions with the lowest current energy consumption relative to their maximum energy consumption limit are selected, thereby maintaining optimal performance even as traffic grows (for example, it can be ensured that such nodes are selected which are providing services that are eligible for their QoS (Quality of Service) characteristics modifications to save network energy in case energy rationing is in-force). This approach can also assist in prioritizing a node for selection over others based on its energy rationing reason.

###### 5.1.3.3.1.2	Description

The proposed solution specifies energy rationing-related information in the network. Such information includes energy rationing causes, maximum energy consumption limits, or energy rationing limits of network nodes for particular locations, for specific time durations and applicable services when energy rationing is in-force.

In particular it is proposed to configure following information in the network:

A pre-configured maximum energy consumption limit or energy rationing limit allowed for a network node when energy rationing is applied. It represents a threshold beyond which the energy consumption of the node should not exceed when energy rationing is in force. The difference between this limit and the current energy consumption of a node can be utilized as a crucial metric for selecting that node for a given use case scenario. The unit can be in Joules or kWhr.

Information about the location where energy rationing is applied within an operator’s network. This information can be used while selecting a node for a certain use case, like whether a node with energy rationing applied can be chosen or not for a given use case scenario. If yes, a node from that location can be chosen or if not, then it can be ensured that node from that location is not chosen to meet required performance. The location can refer to latitude/longitude, CAG cell, any geographical location/cartesian coordinates/area polygon or a group of impacted network elements in operator network     Information about the time period during which energy rationing is applied in an operator’s network. This information can be used while selecting a node for a certain use case, depending on whether a node with energy rationing applied during a specific time period should be chosen or not for a given use case scenario.

Information indicating the reason to apply energy rationing in an operator’s network. Reasons for energy rationing include, but are not limited to regulatory requirements, economic reason, electicity brownouts, or energy blackouts (full outage of energy supplier). Economic reason refers to raising the price of power (energy units) by an energy supplier during certain periods or locations to force operators to use less energy. In such cases, the MNO shall need to reduce energy usage to keep its service economical. Electricity brownout refers to partial power reduction. Energy blackout refers to a total energy outage at the energy supplier’s end (e.g., due to faults), requiring the MNO to rely on local power backup sources (battery banks, generators, etc.) and potentially reduce energy consumption to continue providing services until power from the energy supplier resumes. A producer can prioritize the selection of a node for slice creation, load balancing, handover, etc., based on the energy rationing cause. For instance, if two nodes are under energy rationing for a given location and time period, the producer can prefer to use the node whose energy rationing cause is driven by economic reasons rather than regulatory reasons, as economic rationing can be relaxed over time, whereas regulatory mandates cannot be relaxed until governing authorities decide so.

Information indicating the type of services, for instance, that can be eligible for  QoS characteristics modifications allowing a consumer to select node for certain service which does not belong to energy rationing location or time period i.e. which does not has energy rationing in-force on it.

Information indicating the remaining amount of energy (e.g. in kWh) locally available for consumption.

#### 5.1.3.4	Evaluation of potential solutions

The mutually agreed solution, potential solution #1 Energy Rationing Information Configuration (described in clause 5.1.3.3.1), illustrates energy rationing information that is to be configured in an operator’s network.

The potential solution #1 fulfils the requirement REQ-Energy_Rationing-CON-1. Thus, it is a feasible candidate to be included in TS 28.310 [3], as input to the normative phase.

## 5.2	Enhancements to support the information required by Energy Information Function (EIF)

Editor’s note: The use cases in this clause are subject to be updated based on the conclusion of SA2 study (FS_EnergySys_Ph2) in TR 23.700-67.

### 5.2.1	Use case #1: Energy consumption and Energy Efficiency estimation and reporting at per network slice granularity

#### 5.2.1.1	Description

Energy Consumption at network slice (as defined in TS 23.501 [8]) granularity is an essential information that 5GC NFs uses for further optimization. Energy consumption of a NetworkSlice is defined in TS 28.554 [4]. But NetworkSlice IOC is not equivalent to the network slice as defined in TS 23.501 [8]. Energy consumption KPI at network slice (as defined in TS 23.501 [8]) granularity is missing in TS 28.554 [4]. This use case is to study the inconsistencies and enhance or introduce KPIs to enable the 5GC NF requirements.

Further, the requirements for energy consumption monitoring at per network slice granularity is specified in clause 6.15a.4.2 of TS 22.261 [2]. Several solutions identified for the key issues in TR 23.700-67 [9] include 5GC NFs to collect slice level energy consumption and energy efficiency from 3GPP management system.

The network slice identifiers in the 3GPP network are defined in clause TS 23.501 [8] as: "An S-NSSAI identifies a Network Slice.".  Further the standardized and non-standardized values of S-NSSAI are defined as " An S-NSSAI can have standard values (i.e. such S-NSSAI is only comprised of an SST with a standardised SST value, see clause 5.15.2.2 of TS 23.501 [8] and no SD) or non-standard values (i.e. such S-NSSAI is comprised of either both an SST and an SD or only an SST without a standardised SST value and no SD). An S-NSSAI with a non-standard value identifies a single Network Slice within the PLMN with which it is associated.".

The problem with the existing Network Slice related EC and EE KPIs are described below:

-	Network Slice Energy Consumption (EC) KPI is specified in clause 6.7.3.3 of TS 28.554 [4]. This is reported at KPI object NetworkSlice IOC (defined in clause 6.2.1 of TS 28.541 [7]), while “network slice” in TS 23.501 [8] are identified using S-NSSAI and PLMN ID as described above.

-	Network slice Energy Efficiency (EE) KPIs are specified in clause 6.7.2 of TS 28.554 [4]. This is reported at KPI object NetworkSlice IOC (defined in clause 6.2.1 of TS 28.541 [7]), while “network slice” in TS 23.501 [8] are identified using S-NSSAI and PLMN ID as described above.

-	The NetworkSlice IOC is defined in clause 6.3.1 of TS 28.541 [7], with attribute serviceProfileList that holds a list of ServiceProfile (defined in clause 6.3.3 of TS 28.541 [7]) supported by the NetworkSlice MOI. ServiceProfile is a data type with attribute pLMNInfoList (a list of data type PLMNInfo defined in clause 4.3.41 of TS 28.541 [7]) that defines which PLMN and S-NSSAI combinations that are assigned for the service to satisfy service requirements represented by the ServiceProfile in case of network slicing feature is supported. A NetworkSlice MOI can be configured, in attribute serviceProfileList, with a list of ServiceProfile value, where the list can hold one or more than one element. Each element in the list attribute serviceProfileList can be configured with a list PLMN and S-NSSAI combinations in attribute pLMNInfoList. Hence a NetworkSlice MOI can be configured with a list of PLMN and S-NSSAI combinations.

This use case is to study the enhancements to support for energy consumption and energy efficiency monitoring, estimation and reporting at per network slice (as defined in TS 23.501 [8]) granularity.

#### 5.2.1.2	Potential requirements

PREQ-Energy_Reporting_Network_Slice-1: The 3GPP management system should be able to estimate and report the energy consumption at per network slice granularity.

PREQ-Energy_Reporting_Network_Slice-2: The 3GPP management system should be able to estimate and report the energy efficiency at per network slice granularity.

#### 5.2.1.3	Potential solutions

##### 5.2.1.3.1	Potential solution #1: Enhancements to estimation of network slice EC KPIs

###### 5.2.1.3.1.1	Introduction

This potential solution defines enhancements estimation of network slice energy consumption KPI.

###### 5.2.1.3.1.2	Description

The existing network slice EC KPI specified in clause 6.7.3.3 of TS 28.554 [4] can be enhanced to report support reporting for each SNSSAI. The KPI description in clause 6.7.3.3 of TS 28.554 [4] already describes how to estimate the energy consumption of gNB and 5GC NFs that are part of the NetworkSlice.

This KPI can be enhanced to report the following:

-	Estimated ECns_SNSSAI: The KPI defined in clause 6.7.3.3 of TS 28.554 [4] is applicable for the estimated EC of an S-NSSAI.

-	Estimated ECns KPI: The combined estimated EC for the NetworkSlice can be obtained by adding the Estimated ECns_SNSSAI of each SNSSAI that is configured in the NetworkSlice MOI.

The following is the updated KPI:

a)	ECns_SNSSAI and ECns.

b)	This KPI describes the Estimated Energy Consumption (ECns_SNSSAI) of a network slice (S-NSSAI) and Estimated Energy Consumption (ECns) of the all the network slices (S-NSSAI) that are configured in the NetworkSlice MOI.

Estimated Energy Consumption (ECns_SNSSAI) of a network slice (S-NSSAI) is obtained by summing up the Estimated Energy Consumption of all the Network Functions (ECNF) that compose the network slice.

Estimated Energy Consumption (ECns) of the all the SNSSAIs that are configured in the NetworkSlice MOI is obtained by summing up the Estimated Energy Consumption (ECns_SNSSAI) of each network slices (S-NSSAI) that are configured in the NetworkSlice MOI.

b-1)	Integer, J.

b-2)	CUM.

c)

ECns_SNSSAI=NFECNF

ECns=SNSSAIECns_SNSSAI

The definition of Estimated Energy Consumption (ECns_SNSSAI) of a network slice (S-NSSAI) is based on the following principles:

For all gNBs in the network slice, clause 5.1.1.19.3 (PNF Energy consumption) of TS 28.552 [6] applies. This measurement is obtained according to the method defined in ETSI ES 202 336-12 [10] – clauses 4.4.3.1, 4.4.3.4, Annex A

In case a 5GC NF is composed of Virtualized Network Functions (VNF) and/or Physical Network Functions (PNF), clause 6.7.3.1 of this document defines the NF Energy Consumption (EC).

In case a gNB or 5GC NF is configured with multiple network slices (S-NSSAI), the participation of the gNB or 5GC NF to the energy consumption of the network slice (S-NSSAI) has to be estimated, as it can't be measured:

-	In case of a gNB configured with multiple network slices (S-NSSAI), the energy consumption attributable to each network slice is estimated as a proportion of the total gNB energy consumption, where the proportion is calculated as the data volume of the network slice (S-NSSAI) relatively to the total data volume carried by the gNB.

-	In case of a AMF is configured with multiple network slices (S-NSSAI), the energy consumption attributable to each network slice (S-NSSAI) is estimated as a proportion of the total estimated AMF energy consumption, where the proportion is calculated as the mean number of registered subscribers of the network slice (S-NSSAI) relatively to the overall mean number of registered subscribers of the AMF during the same time period (see TS 28.552 [6] clause 5.2.1.1 for the definition of the mean number of registered subscribers).

-	In case of a SMF is configured with multiple network slices (S-NSSAI), the energy consumption attributable to each network slice (S-NSSAI) is estimated as a proportion of the total estimated SMF energy consumption, where the proportion is calculated as the mean number of PDU sessions of the network slice (S-NSSAI) relatively to the overall mean number of PDU sessions of the SMF during the same time period (see TS 28.552 [6] clause 5.3.1.1 for the definition of the mean number of PDU sessions).

-	In case of a UPF is configured with multiple slices (S-NSSAI), the energy consumption attributable to each network slice (S-NSSAI) is estimated as a proportion of the total estimated UPF energy consumption, where the proportion is calculated as the data volume of the network slice (S-NSSAI) relatively to the overall data volume of the UPF during the same time period.

-	In case of a UPF with N3 interface(s), the data volume of the UPF is obtained by summing up, for all N3 interface(s), the number of octets of incoming GTP data packets on the N3 interface, from (R)AN to UPF (see TS 28.552 [6] clause 5.4.1.3) and the number of octets of outgoing GTP data packets on the N3 interface, from UPF to (R)AN (see TS 28.552 [6] clause 5.4.1.4).

-	In case of a PSA UPF with no N3 interface(s), the data volume of the UPF is obtained by summing up, for all N9 interface(s), the number of octets of incoming GTP data packets on the N9 interface for PSA UPF (see TS 28.552 [6] clause 5.4.4.2.3) and the number of octets of outgoing GTP data packets on the N9 interface for PSA UPF (see TS 28.552 [6] clause 5.4.4.2.4).

-	The case of other 5GC NFs configured with multiple network slices (S-NSSAI) is not addressed in the present document.

The Estimated Energy Consumption (ECns) is the combined estimated EC for the NetworkSlice and is obtained by adding the Estimated Energy Consumption (ECns_SNSSAI) of each network slices (S-NSSAI) that are configured in the NetworkSlice MOI.

d)	NetworkSlice

##### 5.2.1.3.i	Potential solution #<i>: <Potential Solution i Title>

###### 5.2.1.3.i.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

###### 5.2.1.3.i.2	Description

Editor's Note:	This clause further details the potential solution, including all of its aspects and any assumptions made.

#### 5.2.1.4	Evaluation of potential solutions

The potential solution #1 Enhancements to estimation of network slice EC KPIs (described in clause 5.2.1.3.1), provides enhancements to the existing network slice EC KPI (defined in clause 6.7.3.3 of TS 28.554 [4]) to support the Estimated Energy Consumption (ECns_SNSSAI) of a network slice (S-NSSAI) and Estimated Energy Consumption (ECns) of the all the network slices (S-NSSAI) that are configured in the NetworkSlice MOI.

The potential solution #1 fulfils the requirement PREQ-Energy_Reporting_Network_Slice-1. This is a feasible candidate as input to the normative phase.

There are no potential solutions studied for PREQ-Energy_Reporting_Network_Slice-2.

Editor's Note:	This clause provides the evaluation of potential solutions listed in 5.2.1.3.

### 5.2.2	Use case #2: Enhancements to 5GC NF Profile to support energy saving and energy efficiency

#### 5.2.2.1	Description

Several solutions identified for the key issues in TR 23.700-67 [9] include enhancements to 5GC NF Profile (see TS 23.501 [8]) to support energy saving and energy efficiency.

5GC NF Profile related attributes are modelled in ManagedNFProfile data type (see clause 5.3.54 of TS 28.541 [7]).

This use case is to identify the enhancements to ManagedNFProfile data type to support the energy saving and energy efficiency in the 5GC.

#### 5.2.2.2	Potential requirements

PREQ-NFProfile_Energy_Conf-1: The 3GPP management system should be able to support energy saving and energy related configurations in the managed 5GC NF Profile.

#### 5.2.2.3	Potential solutions

##### 5.2.2.3.1	Potential solution #1: Enhancements to  ManagedNFProfile data type

###### 5.2.2.3.1.1	Introduction

The conclusions for KI#3 of SA WG2 study are documented in clause 8.3 of TR 23.700-67 [9]. This includes the potential enhancements for NF Profile to include parameters "Energy Priority Information", "Schedule of Energy State" and "Energy State" (see Table 8.3-1 of TR 23.700-67 [9]).

###### 5.2.2.3.1.2	Description

5GC NF Profile related attributes are modelled in ManagedNFProfile data type (see clause 5.3.54 of TS 28.541 [7]). This data type needs enhancements to include additional attributes to support the parameters "Energy Priority Information", "Schedule of Energy State" and "Energy State" studied by SA WG2 (see Table 8.3-1 of TR 23.700-67 [9]).

NOTE:	The attributes to be introduced in ManagedNFProfile data type (defined in clause 5.3.54 of TS 28.541 [7]) needs to be aligned with the SA WG2 and CT WG4 during the normative phase

#### 5.2.2.4	Evaluation of potential solutions

The potential solution #1 Enhancements to  ManagedNFProfile data type (described in clause 5.2.2.3.1), identifies enhancements to introduce new attributes to ManagedNFProfile data type (defined in clause 5.3.54 of TS 28.541 [7]). The potential solution 1 fulfils the requirement PREQ-NFProfile_Energy_Conf-1. This is a feasible candidate as input to the normative phase.

NOTE:	The attributes to be introduced in ManagedNFProfile data type (defined in clause 5.3.54 of TS 28.541 [7]) needs to be aligned with the SA WG2 and CT WG4 during the normative phase.

## 5.3	Improve energy saving, energy efficiency and reducing carbon footprint of 5G network

There are no use-cases studied to improve energy saving, energy efficiency and reducing carbon footprint of 5G network.

## 5.4	Enhancements to Energy Consumption and Energy Efficiency measurements and KPIs

### 5.4.1	Use case #1: Renewable energy consumption

#### 5.4.1.1	Description

The operators are interested in using renewable energy sources to reduce emissions and boost network efficiency. Therefore, it’s important to track and report the share of energy consumption from renewable sources in their networks.

Due to the highly variable and unpredictable nature of renewable energy sources, the supply of renewable energy varies substantially by time and location. There is a need for the 3GPP management system to support to report of renewable energy consumption on different granularities (e.g., NF, NE, Sub-network), and the total energy consumption considering energy consumption from energy supplies with renewable and non-renewable sources in order to estimate of renewable energy consumption, and to report the renewable energy consumption for different granularities.

Therefore, the 3GPP management system need to support to provide the renewable energy consumption based on energy related information obtained from various sources, or to provide the renewable energy consumption for different granularities, and support to report this information to authorized consumers, e.g., EIF.

#### 5.4.1.2	Potential requirements

REQ-RECM-CON-1: The 3GPP management system shall be able to allow its authorized consumers to request reporting about renewable energy consumption at different granularities.

#### 5.4.1.3	Potential solutions

None.

#### 5.4.1.4	Evaluation of potential solutions

There were no potential solutions studied for this use case.

### 5.4.2	Use case #2: Support EC measurement of NE at per Energy Supply granularity

#### 5.4.2.1	Description

Network Elements can be powered using multiple energy supplies (grid, backup energy, locally generated energy). The Network Element consumes energy from different energy supplies during different periods of time based on various conditions in the operator network. The 3GPP management system needs to measure and report energy consumption separately for each energy supply to calculate accurate carbon emissions. The operator configures the system to associate each energy supply. The operator and the internal optimization need the 3GPP management system to collect EC data per energy supply and report them.

The existing energy consumption measurements are based on PEE measurements and assume a single energy supply for the NE (represented by ManagedElement) and do not provide energy consumption related information at the energy supply granularity. This use case identifies the existing measurements and studies the enhancements that need to be supported.

Energy supply information can be associated with the Managed Elements (representing the Network Elements) using EnergyInfoGroup IOC (see clause 8.3.3. of TS 28.310 [3]). Energy-related information for each energy source (see EnergySourceInfo defined in clause 8.3.2 of TS 28.310 [3]) of the energy supply (see EnergySupplyInfo defined in clause 8.3.1 of TS 28.310 [3]) includes information of different energy source, with source type, carbon emission factor, renewable and non-renewable energy information.

Power, Energy and Environmental (PEE) measurements are defined in clause 5.1.1.19 of TS 28.552 [10], and PNF Energy consumption for ManagedElement is defined in clause 5.1.1.19.3 of TS 28.552 [10].

Energy consumption related KPIs are defined in clause 6.7.3 of TS 28.554 [4]. gNB Estimated Carbon Emission and NG-RAN Estimated Carbon Emission KPIs are defined in clause 6.7.7 of TS 28.554 [4]. A Network Element (represented by Managed Element) can be powered by one or more energy supplies (see TS 28.310 [3]). The Carbon Emission related KPIs defined in TS 28.554 [4] are currently restricted to the scenarios when a Network Element is powered by a single energy supply. This limitation is due to the lack of EC measurements at a per energy supply granularity for a Network Element.

Energy consumption of a Network Element is essential for monitoring and reporting of carbon emissions, carbon intensity, and network energy optimization use cases considering energy supply mix, and energy-related characteristics (defined in TS 22.261 [2]).

This use case is to study the enhancements to enable measurements and reporting of EC of a Network Element at a per energy supply granularity.

#### 5.4.2.2	Potential requirements

PREQ-Energy_Consumption-1: The 3GPP management system should be able to measure and report the energy consumption of a Network Element at a per energy supply granularity.

#### 5.4.2.3	Potential solutions

##### 5.4.2.3.1	Potential solution #1: Enhancements to EnergyInfoGroup and Energy consumption measurement

###### 5.4.2.3.1.1	Introduction

A Network Element can be powered using multiple energy supplies.

ETSI ES 202 336-12 [23] clause 4.3.2 specifies the following, which indicates that the measurements can be provided at individual power inputs on the same ICT equipment. An example is illustrated in Figure 3 of ETSI ES 202 336-12 [23].

"

When there are several power interface A or A3 inputs on the same ICT equipment, the sum of all power and energy measurements shall be provided in the monitored data in addition to individual values.

"

PNF Energy consumption for the ManagedElement IOC is defined in clause 5.1.1.19.3 of TS 28.552 [10]. This measurement is obtained according to the method defined in ETSI ES 202 336-12 [23] – clauses 4.4.3.1, 4.4.3.4, Annex A.

ManagedElement IOC (defined in TS 28.622 [24])) does not have any attributes related to the power, energy and environmental (PEE) parameters. The attribute peeParametersList in ManagedFunction IOC (defined in TS 28.622 [24]) represents a list of PeeParameters relates to power, energy and environmental (PEE) parameters representing the equipment information defined in clause 4.4.1 of ETSI ES 202 336-12 [23].

The measurement and monitoring defined in clause 4.4 ETSI ES 202 336-12 [23] includes "Internal measurements type 1 (Built-in in ICT equipment)" (see clause 4.4.1 of ETSI ES 202 336-12 [23]) and "External measurements type 2 (external sensors) for ICT equipment" (clause 4.4.1 of ETSI ES 202 336-12 [23]). This is illustrated in an example in Figure 5.4.2.3.1.1-1. These measurements and monitoring are supported for ICT Equipment and Network Elements, and do not relate to ManagedFunctions (which represent a telecommunication function either realized by software running on dedicated hardware or realized by software running on virtualized infrastructure).

The Figure 5.4.2.3.1.1-1, reproduced from clause 4.4.1 of ETSI ES 202 336-12 [23], depicts an example of monitoring of PEE in a complex site where the Telecon/ICT Equipment can have multiple energy supplies (depicted by "AC or DC power source 1" and "AC or DC power source 1") and PEE monitoring using different types of PEE sensors.

Figure 5.4.2.3.1.1-1: "Figure 3: Example of cohabitation of ICT equipment internal and external power/energy/environment measurement acquisition in a site considering 3 cases of implementations (power frame measurement, input junction box measurements, built-in measurements)”, reproduced from clause 4.4.1 of ETSI ES 202 336-12 [23]

This potential solution identifies the issues and enhancements to support measure and report the energy consumption of a Network Element at a per energy supply granularity utilizing the measurements defined in ETSI ES 202 336-12 [23].

Editor's Note:	This clause describes briefly the potential solution at a high-level.

###### 5.4.2.3.1.2	Description

As described in clause 5.4.2.3.1.1, the energy consumption cannot be measured at ManagedFunction granularity.

EnergyInfoGroup IOC (defined in TS 28.310 [3]) is used to associate multiple energy supplies that are used to power the Managed Elements (representing the Network Elements). The power, energy and environmental (PEE) parameters should be associated with EnergyInfoGroup IOC to enable to associate the monitored data as defined in ETSI ES 202 336-12 [23].

The association of PeeParameters with ManagedFunction IOC is incorrect as the power, energy and environmental (PEE) parameters as described in ETSI ES 202 336-12 [23] do not relate to managed functions.

As described in clause 5.4.2.3.1.1 and ETSI ES 202 336-12 [23], the energy measurements are provided in monitored data at individual power interfaces when multiple PEE sensors are used to measure the energy consumption for different sources, that is utilized by this potential solution.

PNF Energy consumption measurement (defined in clause 5.1.1.19.3 of TS 28.552 [10]) can be enhanced to include filters per energy supply, such that the measurement can be used to measure

1)	Total PNF Energy consumption

2)	PNF Energy consumption from each energy supply.

The PNF Energy consumption measurement can be enhanced as below:

a)	This measurement provides the energy consumed. The measurement is optionally split into subcounter per associated energy supplies.

b)	SI.

c)	This measurement is obtained according to the method defined in ETSI ES 202 336-12 [L] – clauses 4.4.3.1, 4.4.3.4, Annex A.

NOTE:	The measurement is obtained at energy supply granularity when supported using PEE monitoring for each power input lines is as described in clause 4.3.2 of ETSI ES 202 336-12 [L].

d)	A real value in kilowatt-hours (kWh).

e)	The measurement name has the form PEE.Energy, or optionally PEE.Energy.EnergySupply where EnergySupply refers to the energy supply used to power the Network Element.

f)	ManagedElement

g)	Valid for packet switching.

h)	5GS.

These proposed enhancements enable the 3GPP management system to support the measurements and report the energy consumption of a Network Element at a per energy supply granularity.

Editor's Note:	This clause further details the potential solution, including all of its aspects and any assumptions made.

#### 5.4.2.4	Evaluation of potential solutions

The potential solution #1 Enhancements to EnergyInfoGroup and Energy consumption measurement (described in clause 5.4.2.3.1), describes the enhancements to ManagedFunction IOC (defined in TS 28.622 [24]), EnergyInfoGroup IOC (defined in TS 28.310 [3]) and PNF Energy consumption measurement (defined in clause 5.1.1.19.3 of TS 28.552 [10]).

The potential solution #1 fulfils the requirement PREQ-Energy_Consumption-1. This is a feasible candidate as input to the normative phase.

### 5.4.3	Use case #3: Support estimation of EC and EE KPIs at per PLMN-ID granularity in Network sharing scenario

#### 5.4.3.1	Description

When a NG-RAN is shared between multiple operators in the network sharing (see TS 32.130 [11]), the participating operators are interested in the operator-specific energy consumption (EC) and energy efficiency (EE) of the gNB.

The gNB can support and serve multiple operators (i.e., PLMN-IDs) in Network sharing scenarios. The existing EE and EC KPIs do not consider network sharing scenarios, and do not provide KPIs at operator-specific (i.e., PLMN-ID) granularity.

Following are the currently supported measurements and KPIs related to energy consumption and energy efficiency:

- Power, Energy and Environmental (PEE) measurements are defined in clause 5.1.1.19 of TS 28.552 [10], and PNF Energy consumption for ManagedElement is defined in clause 5.1.1.19.3 of TS 28.552 [10].

- Energy consumption and energy efficiency related KPIs are defined in clause 6.7 of TS 28.554 [4].

This use case is to study the enhancements to enable monitoring and/or estimation of energy consumption and energy efficiency KPIs and reporting of a gNB at operator-specific (i.e., at a per PLMN-ID) granularity in network sharing scenarios.

#### 5.4.3.2	Potential requirements

PREQ-Energy_Consumption_Network_Sharing-1: The 3GPP management system should be able to report the energy consumption and energy efficiency of a gNB at operator-specific, i.e., at a per PLMN-ID granularity.

#### 5.4.3.3	Potential solutions

##### 5.4.3.3.1	Potential solution #1: Estimation of gNB EC per PLMN-ID granularity

###### 5.4.3.3.1.1	Introduction

This potential solution describes estimation of energy consumption of a gNB at a per PLMN-ID granularity.

Editor's Note:	This clause describes briefly the potential solution at a high-level.

###### 5.4.3.3.1.2	Description

The gNB EC KPI defined in clause 6.7.3.4.2 of TS 28.554 [4] can be enhanced to optionally support reporting at PLMN-ID granularity as follows:

a)	EC_gNB_PLMNID

b)	This KPI describes the energy consumption of an gNB at operator-specific, i.e., at a per PLMN-ID granularity. It is obtained using the of the PNF Energy Consumption measurement (defined in 5.1.1.19.3 of TS 28.552 [10]).

b-1)	Real, kilowatt-hours (kWh).

b-2)	RATIO

c)	EC_gNB_PLMNID =   PEE.Energy * W

Where,

-	PEE.Energy is PNF Energy consumption as defined in clause 5.1.1.19.3 of TS 28.552 [10].

-	W is the measurement for the weighted average, it is one of the following:

-	proportion of combined DL and UL data volume of the all the NR Cells of the gNB delivered for the particular PLMN, over the same observation period, as described below;

-	for non-split gNBs.

WPLMN=cellsDRB.PdcpSduVolumeUl_PLMN+DRB.PdcpSduVolumeDl_PLMNcells(DRB.PdcpSduVolumeUL+DRB.PdcpSduVolumeDL)

Where,

-	Measurements DRB.PdcpSduVolumeDL and DRB.PdcpSduVolumeDL_PLMN are DL Cell PDCP SDU Data Volume (defined in clause 5.1.2.1.1.1 of TS 28.552 [10])

-	Measurements DRB.PdcpSduVolumeUL and DRB.PdcpSduVolumeUL_PLMN are UL Cell PDCP SDU Data Volume (defined in clause 5.1.2.1.1.2 of TS 28.552 [10]).

-	for split gNBs.

WPLMN=cellsDRB.F1uPdcpSduVolumeUl_PLMN+DRB.XnuPdcpSduVolumeUl_PLMN+DRB.X2uPdcpSduVolumeUl_PLMN+DRB.F1uPdcpSduVolumeDl_PLMN+DRB.XnuPdcpSduVolumeDl_PLMN+DRB.X2uPdcpSduVolumeDl_PLMNcells(DRB.F1uPdcpSduVolumeUl+DRB.XnuPdcpSduVolumeUl+DRB.X2uPdcpSduVolumeUl+DRB.F1uPdcpSduVolumeDl+DRB.XnuPdcpSduVolumeDl+DRB.X2uPdcpSduVolumeDl)

Where,

-	Measurements DRB.F1uPdcpSduVolumeDl (EP_F1U (F1-U interface)) and DRB.F1uPdcpSduVolumeDl_PLMN (PLMN filter) are DL PDCP SDU Data Volume per interface (F1-U interface) defined in clause 5.1.3.6.2.3 of TS 28.552 [10].

-	Measurements DRB.F1uPdcpSduVolumeUl (EP_F1U (F1-U interface)) and DRB.F1uPdcpSduVolumeUl_PLMN (PLMN filter) are UL PDCP SDU Data Volume per interface (F1-U interface) defined in clause 5.1.3.6.2.4 of TS 28.552 [10].

-	Measurements DRB.XnuPdcpSduVolumeDl (EP_XnU (Xn-U interface)) and DRB. XnuPdcpSduVolumeDl_PLMN (PLMN filter) are DL PDCP SDU Data Volume per interface (EP_XnU (Xn-U interface)) defined in clause 5.1.3.6.2.3 of TS 28.552 [10].

-	Measurements DRB.XnuPdcpSduVolumeUl (EP_XnU (Xn-U interface)) and DRB. XnuPdcpSduVolumeUl_PLMN (PLMN filter) are UL PDCP SDU Data Volume per interface (EP_XnU (Xn-U interface)) defined in clause 5.1.3.6.2.4 of TS 28.552 [10].

-	Measurements DRB.X2uPdcpSduVolumeDl (EP_X2U (X2-U interface)) and DRB. X2uPdcpSduVolumeDl_PLMN (PLMN filter) are DL PDCP SDU Data Volume per interface (EP_X2U (X2-U interface)) defined in clause 5.1.3.6.2.3 of TS 28.552 [10].

-	Measurements DRB. X2uPdcpSduVolumeUl (EP_X2U (X2-U interface)) and DRB. X2uPdcpSduVolumeUl_PLMN (PLMN filter) are UL PDCP SDU Data Volume per interface (EP_X2U (X2-U interface)) defined in clause 5.1.3.6.2.4 of TS 28.552 [10].

-	a weight defined by the consumer of KPI

NOTE:	An example of the weight factor (W) defined by the consumer of the KPI can be based on the agreement between the operators on the proportion of the NG-RAN resources shared between the PLMNs or based on the energy consumption of the gNB without traffic. These examples are not subject to standardization.

d)	ManagedElement

e)	This KPI is applicable when a ManagedElement is used to represent the combined functionality of 3GPP defined gNB and designed to run on dedicated hardware, and the NG-RAN is shared in network sharing scenarios as described in 32.130 [11]

Editor's Note:	This clause further details the potential solution, including all of its aspects and any assumptions made.

#### 5.4.3.4	Evaluation of potential solutions

The potential solution #1 Estimation of gNB EC per PLMN-ID granularity (described in clause 5.4.3.3.1), provides enhancements to the gNB EC KPI (defined in clause 6.7.3.4.2 of TS 28.554 [4]) to optionally support reporting at PLMN-ID granularity. This potential solution utilizes measurements with PLMN filters (defined in TS 28.552 [10]).

The potential solution #1 fulfils the requirement PREQ-Energy_Consumption_Network_Sharing-1. This is a feasible candidate as input to the normative phase.

Editor's Note:	This clause provides the evaluation of potential solutions listed in 5.4.3.3.

### 5.4.4	Use case #4: Support estimation of EE KPI of HDLLC slice

#### 5.4.4.1	Description

Within the 3GPP 5GS, seven standard SSTs have been defined in TS 23.501 [8] to support key scenarios such as eMBB, URLLC, and MIoT, among others. The standardization of SST values facilitates global interoperability for network slicing, enabling PLMNs to support roaming more efficiently for the most commonly deployed slice types.

In particular, SST with value 6 - High Data-rate & Low-Latency Communication (HDLLC)—has been introduced to handle Extended Reality Media (XRM) referred to Table 5.15.2.2-1 in TS 23.501 [8]. This service type is characterized by stringent requirements for high data throughput and low latency.

This use case is to study the estimation of EE KPI of HDLLC slice.

#### 5.4.4.2	Potential requirements

PREQ-Energy_Efficiency_HDLLC-1: The 3GPP management system should be able to estimate the energy efficiency for HDLLC.

#### 5.4.4.3	Potential solutions

None.

#### 5.4.4.4	Evaluation of potential solutions

There were no potential solutions studied for this use case.

### 5.4.5	Use case #5: Enhancements to gNB Energy Consumption

#### 5.4.5.1	Description

gNB Energy Consumption KPI is used by operators to monitor the energy consumed by a gNB, and used by 5GC network function EIF for internal processing, hence the correctness of the KPI is important.

gNB EC KPI is defined in clause 6.7.3.4 of TS 28.554 [4], and specified that it is obtained by summing up the Energy Consumption of all the Network Functions (NF) that constitute the gNB. The NFs that constitute the gNB are not defined. Hence this KPI is not implementable.

The NG-RAN data Energy Efficiency (EE) KPI defined in clause 6.7.3.4 of TS 28.554 [4] defines EC of NG-RAN as the Energy Consumption (in kWh) that is obtained by measuring the PEE.Energy of the considered network elements over the same period of time.

The 3GPP defined Network Functions and entities are specified in clause 4.2.2 of TS 23.501 [8], where "(Radio) Access Network ((R)AN)" is listed as one of the Network Functions in the 5G System architecture.

gNB is defined in TS 38.300 [27] as node providing NR user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC. A gNB can consist of a gNB-CU and one or more gNB-DU(s) (see clause 6.1.1 of TS 38.401 [26]). gNB Central Unit (gNB-CU), gNB Distributed Unit (gNB-DU), gNB-CU-Control Plane (gNB-CU-CP), gNB-CU-User Plane (gNB-CU-UP) are defined as logical nodes hosting different protocols (see definitions in clause 3.1 of TS 38.401 [26]). The RAN specification TS 38.300 [27] and TS 38.401 [26] do not define any Network Functions (NF) that constitute the gNB.

This use case is to identify potential solutions to correct the above inconsistences in the gNB EC KPI.

#### 5.4.5.2	Potential requirements

PREQ-Energy_Consumption_gNB-1: The 3GPP management system should be able to measure and report the energy consumption of a gNB designed to run on dedicated hardware.

#### 5.4.5.3	Potential solutions

#### 5.4.5.3.1	Potential solution #1: gNB EC KPI using PNF Energy consumption measurement

##### 5.4.5.3.1.1	Introduction

This potential solution describes the enhancements to the gNB EC KPI is defined in clause 6.7.3.4 of TS 28.554 [4].

##### 5.4.5.3.1.2	Description

gNB EC KPI is proposed to be based on PNF Energy consumption measurement (PEE.Energy) defined in clause 5.1.1.19.3 of TS 28.552 [10] when the gNB is designed to run on dedicated hardware.

This potential solution proposes the gNB EC KPI to be updated and defined as follows:

a) ECgNB

b) This KPI describes the Energy Consumption (EC) of the gNB. It is obtained using the measurement PNF Energy consumption defined in clause 5.1.1.19.3 of TS 28.552 [10].

b-1)	Integer, J

b-2)	CUM

c) ECgNB = PEE.Energy

d) ManagedElement

e) This KPI is applicable when a ManagedElement is used to represent the combined functionality of 3GPP defined gNB, and the gNB is designed to run on dedicated hardware.

NOTE:	The existing KPI in clause 6.7.3.4 of TS 28.554 [4] can be enhanced or a new KPI can be introduced should be considered.

#### 5.4.5.4	Evaluation of potential solutions

The potential solution #1 gNB EC KPI using PNF Energy consumption measurement (described in clause 5.4.5.3.1), provides the enhancements to the gNB EC KPI is defined in clause 6.7.3.4 of TS 28.554 [4]. This potential solution utilizes measurement PNF Energy consumption defined in clause 5.1.1.19.3 of TS 28.552 [10].

The potential solution #1 fulfils the requirement PREQ-Energy_Consumption_gNB-1. This is a feasible candidate as input to the normative phase.

# 6	Conclusions and recommendations

## 6.1	Enhancements to support energy efficiency as a service criteria

### 6.1.1	Use case #1: Enhancements to support the energy-related characteristics for Network Elements and Network Functions

The use case, requirements (PREQ-Energy_Related_Characteristics-2 and PREQ-Energy_Related_Characteristics-3) and potential solutions for the Enhancements to support the energy-related characteristics for Network Elements and Network Functions are described in clause 5.1.1. The evaluation and feasibility of the potential solution is described in clause 5.1.1.4.1. It is proposed to describe the solution in an informative annex in TS 28.310 [3], as input to the normative phase.

The use case, requirements (PREQ-Energy_Related_Characteristics-1) and potential solutions for the Enhancement to estimation of carbon emission information are described in clause 5.1.1. The evaluation and feasibility of the potential solution is described in clause 5.1.1.4.2. It is proposed to enhance the ECEgNB KPI defined in clause 6.7.7. of TS 28.554 [4] in the normative phase.

### 6.1.2	Use case #2: Management mechanisms to support service adjustments to adapt to energy-related characteristics and energy rationing

The use case and potential requirement for Management mechanisms to support service adjustments to adapt to energy-related characteristics and energy rationing are described in clause 5.1.2. There is no potential solution for this use case, hence no impacts identified for the normative phase.

### 6.1.3	Use case #3: Energy Rationing Information Management

The use case, requirement (REQ-Energy_Rationing-CON-1) and potential solution for the Energy Rationing Information Management are described in clause 5.1.3. The evaluation and feasibility of the potential solution is described in clause 5.1.3.4. It is proposed to define the solution in TS 28.310 [3], as input to the normative phase.

## 6.2	Enhancements to support the information required by Energy Information Function (EIF)

### 6.2.1	Use case #1: Energy consumption and Energy Efficiency estimation and reporting at per network slice granularity

The use case, requirements (PREQ-Energy_Consumption_Network_Sharing-1) and potential solutions for Energy consumption and Energy Efficiency estimation and reporting at per network slice granularity are described in clause 5.2.1. The evaluation and feasibility of the potential solution is described in clause 5.2.1.4. For the normative phase, it is proposed to enhance the existing network slice EC KPI (defined in clause 6.7.3.3 of TS 28.554 [4]) to support the Estimated Energy Consumption (ECns_SNSSAI) of a network slice (S-NSSAI) and Estimated Energy Consumption (ECns) of the all the network slices (S-NSSAI) that are configured in the NetworkSlice MOI.

### 6.2.2	Use case #2: Enhancements to 5GC NF Profile to support energy saving and energy efficiency

The use case, requirements (PREQ-NFProfile_Energy_Conf-1) and potential solutions enhancements to introduce new attributes to ManagedNFProfile data type (defined in clause 5.3.54 of TS 28.541 [7]) are described in clause 5.2.2. The evaluation and feasibility of the potential solution is described in clause 5.2.2.4. It is proposed to enhance ManagedNFProfile data type (defined in clause 5.3.54 of TS 28.541 [7]) in the normative phase.

NOTE:	The attributes to be introduced in ManagedNFProfile data type (defined in clause 5.3.54 of TS 28.541 [7]) needs to be aligned with the SA WG2 and CT WG4 during the normative phase.

## 6.3	Improve energy saving, energy efficiency and reducing carbon footprint of 5G network

There are no use-cases studied to improve energy saving, energy efficiency and reducing carbon footprint of 5G network, hence no impacts identified for the normative phase.

## 6.4	Enhancements to Energy Consumption and Energy Efficiency measurements and KPIs

### 6.4.1	Use case #1: Renewable energy consumption

The use case and potential requirement for Renewable energy consumption are described in clause 5.4.1. There is no potential solution for this use case, hence no impacts identified for the normative phase.

### 6.4.2	Use case #2: Support EC measurement of NE at per Energy Supply granularity

The use case, requirements (PREQ-Energy_Consumption-1) and potential solutions to support EC measurement of NE at per Energy Supply granularity are described in clause 5.4.2. The evaluation and feasibility of the potential solution is described in clause 5.4.2.4. It is proposed to enhance ManagedFunction IOC (defined in TS 28.622 [24]), EnergyInfoGroup IOC (defined in TS 28.310 [3]) and PNF Energy consumption measurement (defined in clause 5.1.1.19.3 of TS 28.552 [10]) in the normative phase.

### 6.4.3	Use case #3: Support estimation of EC and EE KPIs at per PLMN-ID granularity in Network sharing scenario

The use case, requirements (PREQ-Energy_Consumption_Network_Sharing-1) and potential solutions to support estimation of EC and EE KPIs at per PLMN-ID granularity in Network sharing scenario are described in clause 5.4.3. The evaluation and feasibility of the potential solution is described in clause 5.4.3.4. It is proposed to enhance the gNB EC KPI (defined in clause 6.7.3.4 of TS 28.554 [4]) to optionally support reporting at PLMN-ID granularity in the normative phase.

### 6.4.4	Use case #4: Support estimation of EE KPI of HDLLC slice

The use case and potential requirement for Support estimation of EE KPI of HDLLC slice are described in clause 5.4.4. There is no potential solution for this use case, hence no impacts identified for the normative phase.

### 6.4.5	Use case #5: Enhancements to gNB Energy Consumption

The use case, requirements (PREQ-Energy_Consumption_gNB-1) and potential solutions for enhancements to gNB Energy Consumption gNB EC KPI using PNF Energy consumption measurement are described in clause 5.4.5. The evaluation and feasibility of the potential solution is described in clause 5.4.5.4. It is proposed to enhance the gNB EC KPI (defined in clause 6.7.3.4 of TS 28.554 [4]) in the normative phase.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | - | N/A | - | - | - | TR skeleton | 0.0.0 |
| 2025-10 | SA5#163 | S5-254683 | pCR | - | - | pCR on Rel-20 TR 28.885 Add TR structure | 0.1.0 |
| 2025-10 | SA5#163 | S5-254698 | pCR | - | - | pCR on Rel-20 TR 28.885 Add new use case support of energy-related characteristics | 0.1.0 |
| 2025-10 | SA5#163 | S5-254699 | pCR | - | - | pCR on Rel-20 TR 28.885 Add new use case related to service adjustments to adapt to energy-related characteristics | 0.1.0 |
| 2025-10 | SA5#163 | S5-254700 | pCR | - | - | Rel-20 pCR TR 28.885 Add New Use Case on Energy Rationing management | 0.1.0 |
| 2025-10 | SA5#163 | S5-254736 | pCR | - | - | pCR TR 28.885 Add use case and requirements for renewable energy consumption | 0.1.0 |
| 2025-10 | SA5#163 | S5-254738 | pCR | - | - | pCR on Rel-20 TR 28.885 Add new use case for enhancements to reporting EC and EE at per network slice granularity | 0.1.0 |
| 2025-10 | SA5#163 | S5-254739 | pCR | - | - | pCR on Rel-20 TR 28.885 Add new use case to study Enhancements to NF Profile to support energy saving and energy efficiency | 0.1.0 |
| 2025-10 | SA5#163 | S5-254740 | pCR | - | - | pCR on Rel-20 TR 28.885 Add new use case to study measuring of EC of NE at per Energy Supply granularity | 0.1.0 |
| 2025-10 | SA5#163 | S5-254741 | pCR | - | - | pCR on Rel-20 TR 28.885 Add new use case to study EC and EE per PLMNID in Network sharing scenario | 0.1.0 |
| 2025-11 | SA5#164 | S5-255553 | pCR | - | - | pCR on Rel-20 TR 28.885 Add potential solution for Enhancements to network slice EC KPIs | 0.2.0 |
| 2025-11 | SA5#164 | S5-255670 | pCR | - | - | pCR on Rel-20 TR 28.885 Add new use case for supporting estimation of EE KPI of HDLLC slice | 0.2.0 |
| 2025-11 | SA5#164 | - | - | - | - | v0.2.0 published in S5-255474 was incomplete and required v0.3.0 in S5-255708 | 0.3.0 |
| 2026-02 | SA5#165 | S5-260296 | pCR | - | - | pCR on Rel-20 TR 28.885 Update UC Management mechanisms to support service adjustments to adapt to energy-related characteristics and energy rationing | 0.4.0 |
| 2026-02 | SA5#165 | S5-260674 | pCR | - | - | pCR on Rel-20 TR 28.885 Add potential solution for energy capacity and energy availability information | 0.4.0 |
| 2026-02 | SA5#165 | S5-260676 | pCR | - | - | pCR on Rel-20 TR 28.885 Add Potential Solution for Energy Rationing Information Management | 0.4.0 |
| 2026-02 | SA5#165 | S5-260688 | pCR | - | - | pCR on Rel-20 TR 28.885 Update potential solution for Enhancements to network slice EC KPIs | 0.4.0 |
| 2026-02 | SA5#165 | S5-260689 | pCR | - | - | pCR on Rel-20 TR 28.885 Add potential solution for Enhancements to 5GC NF Profile to support ES and EE | 0.4.0 |
| 2026-02 | SA5#165 | S5-260691 | pCR | - | - | pCR on Rel-20 TR 28.885 Add potential solution for EC measurement of NE at per Energy Supply granularity | 0.4.0 |
| 2026-02 | SA5#165 | S5-260693 | pCR | - | - | pCR on Rel-20 TR 28.885 Add potential solution for Estimation of NG-RAN EC per PLMN-ID granularity | 0.4.0 |
| 2026-02 | SA5#165 | S5-260694 | pCR | - | - | pCR on Rel-20 TR 28.885 Add new use case for Enhancements to gNB Energy Consumption | 0.4.0 |
| 2026-02 | SA5#165 | S5-260695 | pCR | - | - | pCR on Rel-20 TR 28.885 Add Introduction, Scope, Concepts and Background | 0.4.0 |
| 2026-02 | SA5#165 | S5-260675 | pCR | - | - | pCR on Rel-20 TR 28.885 Add potential solution for Estimation of carbon emission information | 0.4.0 |