| 3GPP TR 28.892 V0.2.0 (2026-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on unified management interface for multi-RAT support (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Concepts and background	8

4.1	IRP and SBMA overview	8

4.2	NRM in 4G and 5G	9

4.2.1	IRP framework definitions	9

4.2.2	SBMA framework definitions	10

4.2.3	Observations	11

5	Use cases	11

5.1	Use case #1: Management support for co-management of 4G nodes and 5G nodes	11

5.1.1	Description	11

5.1.2	Example	11

5.1.3	Potential requirements	12

5.1.4	Potential solutions	12

5.1.4.1	Solution #1: Introduction of a new standalone specification for SBMA-based management support of 4G nodes	12

5.1.4.1.1	 Description	12

5.1.4.1.2	Advantages	13

5.1.4.1.3	Disadvantages	13

5.1.4.2	Solution #2: Extension of existing 5G management specifications with SBMA-based support for 4G nodes	13

5.1.4.2.1	 Description	13

5.1.4.2.2	Advantages	13

5.1.4.2.3	Disadvantages	13

5.1.4.3	Solution #3: Extension of existing IRP specifications with SBMA-based representations	13

5.1.4.3.1	 Description	13

5.1.4.3.2	Advantages	13

5.1.4.3.3	Disadvantages	14

5.1.4.4	Solution #4: Introduction of a delta specification aligned with 3GPP TS 28.541	14

5.1.4.4.1	 Description	14

5.1.4.4.2	Advantages	14

5.1.4.4.3	Disadvantages	14

5.1.5	Evaluation of potential solutions	14

5.2	Use Case #2: Scope of SBMA-based configuration management for 4G nodes	15

5.2.1	Description	15

5.2.2	Examples	15

5.2.3	Potential requirements	15

5.2.4	Potential solutions	16

5.2.4.1 Solution #1: Limited SBMA-based CM scope	16

5.2.4.2 Solution #2: Extended SBMA-based CM scope	16

5.2.5	Evaluation of potential solutions	16

6	Conclusions and recommendations	16

Annex X (informative): Change history	16

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The management of mobile networks has historically evolved in generation-specific solutions within 3GPP specifications. As a result, 4G and 5G rely on distinct management frameworks and data modelling approaches. This lack of alignment has introduced challenges in achieving unified management and automation across Radio Access Technologies (RATs).

This study analyses the existing management models defined for 4G and 5G systems, identifies inconsistencies between IRP and SBMA frameworks, and explores potential solutions to harmonize them under a unified interface. The outcome aims to support consistent multi-RAT operation and provide a consolidated management framework for both 4G and 5G.

# 1	Scope

The present document studies management aspects, with emphasis on the NRM definitions, required to provide a unified multi-RAT management interface (netconf/yang) for 4G and 5G.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.530: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3"

[3]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM); Stage 1"

[4]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3"

[5]	3GPP TS 28.621: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Requirements	"

[6]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)	"

[7]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions	"

[8]	3GPP TS 28.657: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements"

[9]	3GPP TS 28.658: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)"

[10]	3GPP TS 28.659: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions"

[11]	3GPP TS 28.661: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements"

[12]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)"

[13]	3GPP TS 28.663: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions"

[14]	3GPP TS 32.612: “Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP): Information Service (IS)”

[15]	3GPP TS 32.615: “Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP): eXtensible Markup Language (XML) file format definition”

[16]	3GPP TS 28.537: “Management and orchestration; Management capabilities”

[17]	3GPP TS 28.572: “Management and orchestration; Management of planned configurations”

[18]	3GPP TS 32.509: “Telecommunication management; Data formats for multi-vendor plug and play eNode B connection to the network”

[19]	3GPP TS 28.314: “Management and orchestration; Plug and Connect; Concepts and requirements”

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CM				Configuration Management

DM				Domain Manager

IOC				Information Object Classes

IS				Information Services

MnS				Management Services

NF				Network Function

SBMA				Service-Based Management Architecture

SS				Solution Sets

# 4	Concepts and background

## 4.1	IRP and SBMA overview

The 3GPP management architecture has evolved across generations, reflecting a transition from interface-oriented to model-driven and service-based paradigms. Two main frameworks are currently defined in 3GPP specifications: IRP and SBMA.

The IRP framework, introduced and widely applied in legacy systems such as 4G/E-UTRAN, defines management through standardized NRMs specified using XML-based schemas. IRP also standardizes the Itf-N interface, while the direct interface between the DM/EM and the network elements remains proprietary. This model provides a mature and reliable foundation for multiple network domains. It includes standardised interface operations, such as basic and bulk CM, and can support NETCONF for Component A.

With the introduction of 5G, the SBMA model was specified to enable a more flexible, service-oriented, and model-driven approach to network management. SBMA is characterized by the definition of MnS, which expose standardized management capabilities through service-based interfaces and NRMs specified using YANG and OpenAPI. The SBMA interfaces towards the NFs are standardized, in contrast to the proprietary element interfaces in IRP. SBMA management operations follow a CRUD paradigm, in contrast to the basic and bulk CM operations in IRP. This approach enhances automation, interoperability, and alignment with cloud-native network principles.

While 5G management fully adopts the SBMA model with YANG and OpenAPI based NRMs, 4G management remains entirely IRP-based, lacking a corresponding YANG and OpenAPI representation. The two frameworks also differ in management capabilities and protocols because IRP relies on Itf-N, XML schemas, and may use NETCONF, whereas SBMA uses service-based interfaces with CRUD operations, supporting automation and programmability. This creates a gap that prevents the consistent use of a unified management framework across generations.

## 4.2	NRM in 4G and 5G

The management specifications of 3GPP have evolved over successive generations, transitioning from the IRP–based framework used in 4G systems towards the SBMA adopted for 5G. This evolution reflects the shift from a traditional, interface-oriented management paradigm to a service-oriented and model-driven approach aligned with automation and virtualization principles.

### 4.2.1	IRP framework definitions

The 4G management model is defined using the IRP framework, which specifies the exchange of management information between an IRPManager and an IRPAgent through standardized ISs and corresponding SS. Each managed entity is represented by an NRM, consisting of IOCs, their attributes, relationships, and behaviour.

To provide a clearer view of how the IRP framework defines NRM across 4G domains, the relevant IRP specifications are listed in the following tables.

Table 4.2.1-1 summarizes the Generic RAN NRM IRP specifications, which define the common requirements, information model, and solution set applicable across all RAN technologies.

Table 4.2.1-1: Generic RAN NRM IRP Specifications

| Specification | Scope | Description |
| --- | --- | --- |
| 3GPP TS 28.661 [11] | Generic RAN NRM IRP – Requirements | Defines general requirements for RAN management under the IRP framework. |
| 3GPP TS 28.662 [12] | Generic RAN NRM IRP – Information Service (IS) | Specifies semantics and behaviour of generic RAN information objects in a protocol-neutral way. |
| 3GPP TS 28.663 [13] | Generic RAN NRM IRP – Solution Set (SS) | Provides encoding and protocol-specific solution sets for the generic RAN NRM. |

Table 4.2.1-2 presents the E-UTRAN-specific NRM IRP specifications, which build upon the Generic RAN model by introducing management requirements and information objects unique to E-UTRAN.

Table 4.2.1-2: E-UTRAN NRM IRP Specifications

| Specification | Scope | Description |
| --- | --- | --- |
| 3GPP TS 28.657 [8] | E-UTRAN NRM IRP – Requirements | Defines requirements specific to the management of E-UTRAN nodes (eNB). |
| 3GPP TS 28.658 [9] | E-UTRAN NRM IRP – Information Service (IS) | Specifies E-UTRAN-specific IOCs and their relations, reusing elements from the Generic RAN NRM. |
| 3GPP TS 28.659 [10] | E-UTRAN NRM IRP – Solution Set (SS) | Provides the protocol-specific representation for the E-UTRAN NRM. |

Beyond the RAN-specific NRMs, 3GPP also defines a Generic NRM applicable across both RAN and Core. The table 4.2.1-3 summarizes the Generic NRM specifications that provide common requirements, information models, and YANG and OpenAPI based solution sets reusable across both IRP and SBMA management architectures.

Table 4.2.1-3. Generic NRM IRP Specifications

| Specification | Scope | Description |
| --- | --- | --- |
| 3GPP TS 28.621 [5] | Generic NRM IRP – Requirements | Defines general requirements for network management applicable to multiple domains (RAN and Core). |
| 3GPP TS 28.622 [6] | Generic NRM IRP – Information Service (IS) | Specifies semantics of information object classes for both IRP and SBMA deployment scenarios, enabling reuse of common classes across technologies. |
| 3GPP TS 28.623 [7] | Generic NRM IRP – Solution Set (SS) | Provides solution sets and encoding formats applicable to the Generic NRM, supporting YANG solution set representation. |

The 4G IRP-based framework provides a complete and well-established management model. However, it is not aligned with the SBMA paradigm and does not include YANG and OpenAPI based definitions. The model syntax and semantics are designed around XML schemas and protocol mappings defined in earlier 3GPP management frameworks.

### 4.2.2	SBMA framework definitions

For 5G, 3GPP adopted SBMA, which enable MnS Producer to provide as MnS. This model introduces a consistent use of YANG-based data definitions to represent NRMs and their associated behaviour, improving flexibility, programmability, and interoperability.

The following tables summarize the main SBMA specifications defining 5G network management.

The set of specifications from tables 4.2.1-1 and 4.2.1-3 and are also used in 5G, providing a common foundation across 4G and 5G management frameworks.

Table 4.2.2-1 presents the specifications defining the 5G NRM under the SBMA framework, including stage-1 management requirements and the corresponding information models and YANG solution sets for 5G networks.

Table 4.2.2-1: Management and Orchestration of 5G Networks specifications

| Specification | Scope | Description |
| --- | --- | --- |
| 3GPP TS 28.540 [3] | Management and Orchestration of 5G Networks – Stage 1 (Requirements) | Defines high-level functional requirements for 5G network management, including NR, NG-RAN, 5GC and network slicing. |
| 3GPP TS 28.541 [4] | Management and Orchestration of 5G Networks – Stage 2 and Stage 3 (Information Model and Solution Set) | Specifies the 5G NRM, including information models and YANG-based solution sets, fulfilling the requirements from 3GPP TS 28.540. |

### 4.2.3	Observations

The comparison between 4G and 5G specification sets reveals a clear continuity in structure but a major divergence in modelling technology and representation. The 4G NRMs define the necessary information objects and relationships for E-UTRAN management but do so under the IRP paradigm and with XML-based Solution Sets. Conversely, 5G introduces a service-based representation, replacing XML with YANG modules.

While the underlying management concepts remain compatible, the absence of a YANG-based NRM for 4G nodes prevents consistent implementation of unified management functions across technologies. Introducing YANG and OpenAPI based NRMs for 4G elements, derived from the existing IRP specifications, would therefore ensure continuity, facilitate multi-RAT management, and align future network evolution under a common management architectural framework.

# 5	Use cases

## 5.1	Use case #1: Management support for co-management of 4G nodes and 5G nodes

### 5.1.1	Description

This section focuses on how legacy 4G NRMs can be represented within the SBMA. SBMA enables the exposure of management capabilities as standardized MnS, allowing NFs from different generations to interoperate under common architectural principles.

3GPP TS 28.530 [2] defines the overall structure of SBMA and its relationship to legacy management frameworks. In particular, clause 6.6 describes the integration of IRP-based and SBMA-based domains and identifies several options for achieving interoperability between them. Among these, Option B outlines a migration approach that allows legacy domains, such as those managing E-UTRAN or EPC, to remain functionally unchanged while being represented within the SBMA environment.

Under this approach, the existing NRM of the legacy domain is treated as an MnS component type B, corresponding to the managed domain model in SBMA, allowing its information and management functions to be accessed through service-based interfaces. To achieve this integration, the NRM must be expressed using the same data modelling principles applied in SBMA, which rely on model-driven, YANG and OpenAPI based definitions instead of the XML representations used in the IRP framework.

While 5G NRMs defined in 3GPP TS 28.540 [3] and 3GPP TS 28.541 [4] already comply with these principles and include full YANG and OpenAPI based representations, the 4G NRMs are limited to XML-based Solution Sets under the IRP framework.

This inconsistency prevents 4G network elements from being managed within the SBMA model, thus creating a structural gap in the realization of multi-generation management consistency.

### 5.1.2	Example

To illustrate the practical implications of the gaps between IRP and SBMA models, a useful starting point is the examination of 3GPP TS 28.541 [4]. Since this specification focuses on 5G management, the content related to 4G is imported from previous releases. In particular, clause 4.1.1 of 3GPP TS 28.541 [4] lists the imported information entities and their local labels for the NR NRM.

EUtranCellFDD and EUtranCellTDD:

Clause 4.1.1 in 3GPP TS 28.541 [4] indicates that the IOCs EUtranCellFDD and EUtranCellTDD are further described in 3GPP TS 28.658 [9]. This is a positive indication, as 3GPP TS 28.658 [9] is one of the E-UTRAN specifications identified in section 5.1.1 when defining the 4G management model.

However, in 3GPP TS 28.658 [9], EUtranCellFDD and EUtranCellTDD are only described textually, in clauses 4.3.5 and 4.3.7, respectively, without any associated data model or mapping to a YANG structure. Within their descriptions, the same three parameters appear: earfcn, sfAssignment and specialSfPatterns, defined in radio interface specifications, which are outside the scope of management data modelling.

Returning to 3GPP TS 28.541 [4], a search across all YANG and YAML files included in the official archive (28541-k00.zip) shows no match for either EUtranCellFDD or EUtranCellTDD.

This confirms that, although these 4G IOCs are referenced in the 5G NRM, they are not defined in any YANG model.

AdjacentCell:

A similar situation occurs with the AdjacentCell entity. Clause 4.1.1 of 3GPP TS 28.541 [4] also points to 3GPP TS 28.658 [9] for its definition. Indeed, in 3GPP TS 28.658 [9] clause 4.3.9, AdjacentCell appears as an attribute of EUtranRelation. Consequently, it does appear in the YANG module “3gpp-nr-nrm-eutrancellrelation.yang” within the “28541-k00.zip” archive, where the following code is defined:

leaf adjacentCell {

description "Reference to an EUtranCellFDD/TDD or

ExternalEUtranCellFDD/TDD instance.";

mandatory true;

type types3gpp:DistinguishedName;

}

This snippet clearly references EUtranCellFDD and EUtranCellTDD, but only as external objects. Their definitions are missing in YANG form. The model therefore depends on entities that are not formally defined within the SBMA-compliant representation.

### 5.1.3	Potential requirements

REQ-UMMR_CoMgt-01: The 3GPP management system should be able to expose configuration management information of 4G nodes using SBMA-based interfaces.

REQ-UMMR_CoMgt-02: The 3GPP management system should enable an authorized consumer to retrieve and modify configuration data of both 4G and 5G nodes using consistent SBMA principles.

REQ-UMMR_CoMgt-03: The 3GPP management system should support coexistence of IRP-based and SBMA-based management for 4G nodes, preserving backward compatibility with existing IRP deployments.

### 5.1.4	Potential solutions

#### 5.1.4.1	Solution #1: Introduction of a new standalone specification for SBMA-based management support of 4G nodes

##### 5.1.4.1.1		Description

This solution considers the introduction of a new dedicated 3GPP specification defining SBMA-based configuration management support for 4G nodes. The specification would include the necessary SBMA-related definitions, such as YANG and OpenAPI based representations and the exposure of corresponding Management Services, while referencing the existing IRP-based NRMs defined in legacy 4G specifications.

Under this approach, existing IRP specifications and current 5G management specifications would remain unchanged, and the new specification would provide SBMA-based access to 4G configuration management information.

This solution addresses the prerequisites defined in REQ-UMMR_CoMgt-01, REQ-UMMR_CoMgt-02 and REQ-UMMR_CoMgt-03.

##### 5.1.4.1.2	Advantages

- Clear separation between legacy IRP-based management and SBMA-based management.

- No modifications required to existing IRP or 5G management specifications.

- Preserves backward compatibility with existing IRP-based deployments.

- Enables SBMA-based co-management of 4G and 5G nodes.

##### 5.1.4.1.3	Disadvantages

- Risk of partial duplication of concepts already defined in 5G SBMA specifications.

- Requires careful alignment with existing 5G SBMA models to avoid inconsistencies.

- Introduces an additional specification that must be maintained independently.

#### 5.1.4.2	Solution #2: Extension of existing 5G management specifications with SBMA-based support for 4G nodes

##### 5.1.4.2.1		Description

This solution considers extending existing 5G management specifications, such as 3GPP TS 28.540 [3] and 3GPP TS 28.541 [4], to include SBMA-based configuration management definitions for 4G nodes.

By introducing the missing SBMA representations for 4G NRMs within the existing 5G specification framework, this approach enables SBMA-based exposure of configuration management information for both 4G and 5G nodes. However, it results in increased specification complexity and introduces management definitions for multiple RAT generations within the same specifications.

This solution addresses the prerequisites defined in REQ-UMMR_CoMgt-01, REQ-UMMR_CoMgt-02 and REQ-UMMR_CoMgt-03.

##### 5.1.4.2.2	Advantages

- Provides a single specification framework for SBMA-based management across multiple RAT generations.

- Enables direct reuse of existing SBMA mechanisms without additional specifications.

- Simplifies reference handling for implementers using a single specification set.

##### 5.1.4.2.3	Disadvantages

- Increases the complexity and size of already extensive 5G management specifications.

- Makes the specifications more difficult to read, navigate and maintain.

- Mixes management definitions for different RAT generations, potentially reducing clarity.

- Increases the risk of unintended side effects when evolving or extending 5G management features.

#### 5.1.4.3	Solution #3: Extension of existing IRP specifications with SBMA-based representations

##### 5.1.4.3.1		Description

This solution considers extending existing IRP-based management specifications to include SBMA-based representations of 4G NRMs. YANG and OpenAPI based definitions would be introduced alongside the existing IRP Information Services and Solution Sets within the same specifications, enabling SBMA-based access to 4G configuration management information.

This solution addresses the prerequisites defined in REQ-UMMR_CoMgt-01, REQ-UMMR_CoMgt-02 and REQ-UMMR_CoMgt-03.

##### 5.1.4.3.2	Advantages

- Preserves close alignment with existing 4G IRP specifications.

- Maintains backward compatibility with current IRP-based management deployments.

- Avoids the need for new standalone specifications.

##### 5.1.4.3.3	Disadvantages

- Introduces multiple management paradigms (IRP-based and SBMA-based) within the same specifications.

- Further increases the complexity of IRP specifications, which already include management definitions spanning multiple generations (e.g. 3G and 4G).

- Blurs architectural boundaries between legacy and service-based management frameworks.

- Requires changes across multiple legacy specifications, increasing editorial effort and coordination overhead.

- Distributes SBMA-related content across several specifications, complicating implementation and tracking.

#### 5.1.4.4	Solution #4: Introduction of a delta specification aligned with 3GPP TS 28.541

##### 5.1.4.4.1		Description

This solution considers the introduction of a new 3GPP specification that provides SBMA-based configuration management support for 4G nodes as a delta to the existing 5G SBMA framework defined in 3GPP TS 28.540 [3] and 3GPP TS 28.541 [4].

The new specification would not duplicate existing SBMA definitions but would instead reference 3GPP TS 28.541 [4] and define only the additional elements required to support 4G NRMs, such as the missing YANG and OpenAPI based representations and related Management Services. Existing specifications could include references to this new specification.

In this approach, the new specification could assume that the relevant management requirements are already captured in 3GPP TS 28.540 [3] and would therefore focus on the SBMA-based realization of configuration management for 4G nodes. The specification would leverage the existing IRP Information Services as the authoritative source of 4G management semantics, enabling a systematic derivation of YANG and OpenAPI based representations aligned with the SBMA framework. This allows reuse of established information models while ensuring consistency with existing SBMA definitions.

This solution addresses the prerequisites defined in REQ-UMMR_CoMgt-01, REQ-UMMR_CoMgt-02 and REQ-UMMR_CoMgt-03.

##### 5.1.4.4.2	Advantages

- Maintains a clean separation between 4G and 5G management specifications.

- Avoids duplication of SBMA concepts already defined in 3GPP TS 28.541 [4].

- Provides a clear and modular extension path aligned with the existing SBMA framework.

- Limits the impact on existing IRP and 5G management specifications.

- Improves readability, maintainability and long-term evolution of the specification set.

##### 5.1.4.4.3	Disadvantages

- Introduces an additional 3GPP specification.

- Requires clear definition of dependencies and normative references between specifications.

- Implementation guidelines must ensure consistent interpretation across specifications.

### 5.1.5	Evaluation of potential solutions

TBD

## 5.2	Use Case #2: Scope of SBMA-based configuration management for 4G nodes

### 5.2.1	Description

This use case focuses on clarifying the scope of CM capabilities to be supported for 4G nodes when managed through SBMA-based interfaces under UMMR. While UMMR aims at enabling unified management across generations, CM in 4G and 5G is specified using different abstractions and functional decompositions.

In legacy 4G systems, CM is specified through multiple IRPs, each covering different aspects of CM functionality. In contrast, SBMA exposes CM capabilities through one or more MnS, following a model-driven and service-based paradigm. As a result, providing functionally equivalent CM support for 4G nodes within SBMA may require the use of multiple MnS, potentially involving different SBMA components (e.g. Component A, B or C), as defined in 3GPP TS 28.530 [2].

This use case aims to identify which CM capabilities are expected to be supported for a 4G node under UMMR, which SBMA-based MnS are required to support those capabilities, and which management functions are considered out of scope of UMMR. In particular, it addresses whether SBMA-based management of a 4G node is expected to cover only basic CM operations, to also include bulk-oriented CM capabilities, or to provide full functional equivalence with existing 4G management systems.

### 5.2.2	Examples

In 4G, CM capabilities are specified through different IRPs, including:

- Basic CM, as specified in 3GPP TS 32.612 [14];

- Bulk CM, as specified in 3GPP TS 32.615 [15].

In the SBMA framework, equivalent CM capabilities are exposed through different MnS:

- Basic CM capabilities can be supported using the Generic Provisioning MnS, as specified in 3GPP TS 28.537 [16];

- Bulk-oriented CM capabilities may require the use of Plan Management services, as specified in 3GPP TS 28.572 [17].

This illustrates that CM support for a 4G node within SBMA cannot be assumed to be provided by a single MnS, and that multiple MnS may be required depending on the set of CM use cases to be supported.

As a result, managing a 4G node via SBMA-based interfaces does not automatically imply that all CM capabilities currently provided by existing 4G management systems are available. Instead, the set of supported CM use cases depends on which SBMA-based Management Services are considered in scope for UMMR. This use case therefore serves to set clear expectations on the CM functionality exposed for 4G nodes and to avoid implicit assumptions regarding full replacement of legacy 4G management systems.

In addition, certain 4G management functions, such as node commissioning and initial connectivity procedures, are specified outside the scope of CM:

- Plug and Play procedures for 4G nodes are specified in 3GPP TS 32.509 [18];

- Equivalent procedures for 5G nodes are specified as Plug and Connect in 3GPP TS 28.314 [19].

These functions are not natively covered by SBMA-based CM services and may continue to rely on legacy 4G management mechanisms.

### 5.2.3	Potential requirements

REQ-UMMR_CM-01: The 3GPP management system should clearly define the scope of CM capabilities to be supported for 4G nodes when exposed through SBMA-based interfaces.

REQ-UMMR_CM-02: The 3GPP management system should identify the SBMA-based MnS required to provide functionally equivalent CM capabilities for 4G nodes, taking into account both basic and bulk CM use cases.

REQ-UMMR_CM-03: The scope of UMMR-based management for 4G nodes should explicitly distinguish CM functions from other lifecycle-related functions, such as node commissioning.

REQ-UMMR_CM-04: The introduction of SBMA-based CM support for 4G nodes should not mandate that a 4G node implements both IRP-based and SBMA-based interfaces for the same management functions.

### 5.2.4	Potential solutions

#### 5.2.4.1 Solution #1: Limited SBMA-based CM scope

This solution considers supporting a limited subset of CM capabilities for 4G nodes through SBMA-based interfaces, focusing on basic CM use cases. Under this approach, Generic Provisioning MnS would be used to expose configuration data, while bulk CM and other advanced capabilities would remain supported through existing IRP-based mechanisms.

#### 5.2.4.2 Solution #2: Extended SBMA-based CM scope

This solution considers supporting both basic and bulk CM capabilities for 4G nodes through SBMA-based interfaces. In addition to Generic Provisioning MnS, Plan Management services would be used to provide bulk-oriented CM functionality. This approach requires the coordinated use of multiple SBMA-based MnS to achieve functional equivalence with existing 4G IRP-based CM.

### 5.2.5	Evaluation of potential solutions

Solution #1 limits the scope of SBMA-based CM for 4G nodes and minimizes the number of MnS required. This approach reduces complexity and avoids overextending the scope of UMMR but may result in partial coverage of existing 4G CM use cases.

Solution #2 provides broader CM coverage by supporting both basic and bulk CM capabilities through SBMA-based interfaces. While this approach improves functional equivalence with existing 4G management solutions, it increases architectural complexity and requires careful coordination between multiple SBMA-based MnS.

Editor’s note:	Based on this evaluation, further study is required to determine the appropriate scope of SBMA-based CM for 4G nodes under UMMR and to assess the architectural and implementation implications of supporting extended CM capabilities.

# 6	Conclusions and recommendations

Editor's Note: This clause captures the conclusions and the recommendations of the study.

Annex X (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-10 | SA5#163 | S5-254377 |  |  |  | Initial skeleton | 0.0.1 |
|  |  | S5-254730 |  |  |  | Scope |  |
| 2025-11 | SA5#164 | S5-255606 |  |  |  | Introduction | 0.1.0 |
|  |  | S5-255607 |  |  |  | Section in concepts and background |  |
|  |  | S5-255608 |  |  |  | Section in concepts and background and description of use case 1 |  |
| 2026-02 | SA5#165 | S5-260359 |  |  |  | Potential requirements for use case #1 | 0.2.0 |
|  |  | S5-260360 |  |  |  | Potential solutions for use case #1 |  |
|  |  | S5-260474 |  |  |  | Description, requirements and potential solutions for use case #2 |  |