3GPP TSG-SA5 Meeting #165	S5-260628

Goa, India, 09 - 13 February 2026

Source:	Samsung

Title:	pCR on TR 32.801-01 Add use case on agent-driven multi-RAT management

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 32.801-01

Version:	0.0.0

Work Item:	FS_6G_OAM

Comments

This contribution proposes a use case on agent-driven multi-RAT management. By enabling agents to autonomously discover and consume Management Services (MnS) across different generations (e.g., LTE IRP and NR/6G MnS), we can achieve intelligent, integrated E2E automation. This approach ensures forward compatibility and eliminates the need for manual logic updates when underlying management functions evolve.

Proposed Changes

* * * First Change * * * *

6	6G Management Scenarios

6.1 Identified 6G management scenarios

6.1.X Agent-driven Autonomous Management

6.1.X.Y Management Scenario #Y: Agent-driven multi-RAT management

6.1.X.Y.1 Description

In existing network management architectures, achieving end-to-end (E2E) automation across multi-domain, multi-generation (multi-RAT), and multi-vendor environments has been challenging due to fragmented management interfaces and static integration logic. Traditional systems often require manual updates or specific integration efforts whenever a management service (e.g., IRP in LTE or MnS in NR) is updated or a new network function is introduced.

With the evolution towards 6G, the management architecture is expected to incorporate AI-powered agents to play a central role. This scenario focuses on how an agent can autonomously manage and optimize RAN resources in a hybrid environment where LTE and NR (and future 6G) stations coexist.

Instead of being hard-coded to specific versions of management services, the agent can dynamically browse, discover, and access available MnS. For instance, in an energy-saving use case, the agent can simultaneously access performance data and provisioning services from both legacy IRP-based systems (LTE) and service-based MnS (NR). By aggregating information from diverse sources, the agent can make intelligent decisions; such as, selective deactivation of overlapping cells of different RATs to minimize power consumption while ensuring the coverage.

Key advantages of this approach include:

•	Intelligent Integrated Control: Enabling AI to perform holistic optimization across heterogeneous network elements.

•	Forward Compatibility: The management solution, operated by an agent, remains valid even when underlying management services are updated or evolved, as the agent relies on capability discovery rather than fixed interface definitions.

•	Enhanced Automation: Reducing human intervention by allowing agents to autonomously determine the best management services to fulfill a specific intent.

•	Data-Driven Decision Making: Leveraging a broader set of data across multiple RATs to achieve better performance compared to traditional management systems.

6.1.X.Y.2 Potential Requirements

REQ-01: The 6G management architecture shall provide a mechanism for agents to discover available management services and their associated capabilities within a multi-domain/multi-RAT environment.

REQ-02: The 6G management architecture shall support the ability for an agent to access and consume multiple management services simultaneously to perform integrated resource optimization.

REQ-03: The 6G management architecture shall provide semantic descriptions of management services to enable agents to understand the functionality and input/output parameters of the services without manual configuration.

* * * End of Changes * * * *