3GPP TSG SA5 Meeting #165	S5-260731

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	Pseudo-CR on Clarification on supportedDataScope in MgmtDataInfo IOC

Document for:	Approval

Agenda item:	6.20.8

Spec:	3GPP TR 28.887

Version:	0.3.0

Work Item:	FS_MADCOL_Ph3

Comments

This contribution proposes to clarify the attribute supportedDataScope in MgmtDataInfo IOC.

Proposed Changes

* * * First Change * * * *

4.4 	Clarification of Mechanisms to Discover, Request and Retrieve Management Data

4.4.X	Use Case#4-X: Clarification on supportedDataScope in MgmtDataInfo IOC

4.4.X.1      Description

In TS 28.622 [2],  the MgmtDataInfo IOC is defined as follows:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedManagementData | M | T | T/F | F | T |
| supportedGranularityPeriods | CM | T | T/F | F | T |
| supportedReportingPeriods | CM | T | T/F | F | T |
| historicalDataPeriod | CM | T | T/F | F | T |
| supportedReportingMethod | M | T | T/F | F | T |
| supportedDataScope | O | T | T/F | F | T |
| Atttibutes related to roles |  |  |  |  |  |
| supportedDataRequestMnSRef | M | T | T/F | F | T |
| supportedDataReportingMnSRef | M | T | T/F | F | T |
| Note: For all attributes of MgmtDataInfo, the property “isWritable=T” for the Separate MnSRegistry deployment scenario described in TS 28.537 [65], and “isWritable=F” for the Embedded MnSRegistry deployment scenario. |  |  |  |  |  |

The attribute “supportedDataScope” is defined to represent a list of supported sub counter capabilities for the associated management data.

4.4.X.2      Problem Statement

1. The attribute name supportedDataScope is not aligned with the definition “a list of supported sub counter capabilities for the associated management data”. The data scope mainly used to present the following information instead of a list of supported sub counter capabilities for the associated management data:

-	The name or category of supported management data. It is already represented by the attribute “supportedManagementData”.

-	The managed objects (e.g. cell, gNB) which support the capability to produce the supported management data. This information is missing in the MgmtDataInfo IOC. MnS consumer needs to discover what managed object instances where the supported management data can be associated to.

4.4.X.3      Potential Requirements

REQ-MDM-DIS-1: The 3GPP management system shall enable an authorized data consumer to discover what managed object instances where the supported management data can be associated.

4.4.X.4      Potential Solution

Enhancement Aspect 1:  Change existing attribute “supportedDataScope” to “supportedSubCounters” to represent a list of supported sub counter capabilities for the associated management data.

Enhancement Aspect 2: Add new attribute “supportedDataMgmtScope” for the MgmtDataInfo IOC to represent the managed object instances where the supported management data can be associated. The attribute “supportedDataMgmtScope” will reuse the NodeFilter <<dataType>> defined in TS 28.622 as follows:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| areaOfInterest | O | T | T | T | N/A |
| networkDomain | O | T | T | T | N/A |
| cpUpType | O | T | T | T | N/A |
| sst | O | T | T | T | N/A |
| objectInstances | O | T | T | T | N/A |

* * * End of Changes * * * *