3GPP TSG SA5 Meeting #165	S5-260783

# Goa, India, 9-13 February 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0770 | rev | 1 | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.552 Measurements related to LTM cell-switch with almost invalid TA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_Trace_MDT_QoE-OAM |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | New measurement definitions added related to LTM cell switch for intra and inter DU scenarios with declared almost invalid TA |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following measurements are added: - Number of successful inter DU LTM cell switches with declared almost invalid TA per cell pair - Number of successful inter DU LTM cell switches per cell pair - Number of successful intra DU LTM cell switch cell switches with declared almost invalid TA per cell pair Number of successful intra DU LTM cell switch per cell pair |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will not be any possibility to make use of new measurement definitions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.6.y, 5.1.1.6.y.a, 5.1.1.6.y.b, 5.1.1.6.y.c, 5.1.1.6.y.d |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

5.1.1.6.y Measurements related to LTM cell-switch with almost invalid TA (intra and inter DU scenarios)

5.1.1.6.y.a      Number of successful inter DU LTM cell switches with declared almost invalid TA per cell pair

a)	This measurement provides the number of successful inter DU LTM cell switches with declared almost invalid TA from source to target cell

b)	CC

c)	On time interval between the successful inter DU LTM cell switch which is triggered by successful reception of the RRC Reconfiguration complete or first UL message by target DU for previously sent Cell switch command (TS 38.331) from source DU to UE which is signalled by CU to target DU via CU-DU CELL SWITCH NOTIFICATION message with included acquired TA of the target cell in the TA Information List (TS 38.473) and first TA command (MAC CE) (TS 38.321) containing the TA value different from the acquired TA of the target cell  below a configured threshold (vendor specific). The measurement is reported in the target DU per cell pair, i.e. source cell and target cell.

d)	A single integer value.

e)	MM.InterDULTMalmostInvalidTA.

f)	NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for MRO LTM.

==============Next change==============

5.1.1.6.y Measurements related to LTM cell-switch with almost invalid TA (intra and inter DU scenarios)

5.1.1.6.y.b   Number of successful inter DU LTM cell switches per cell pair

a)	This measurement provides the number of successful inter DU LTM cell switches from the source to the target cell.

b)	CC

c)	The successful inter DU LTM cell switch is triggered by successful reception of the RRC Reconfiguration complete or first UL message by target DU for previously sent Cell switch command (TS 38.331) from source DU to UE which is signalled by CU to target DU via CU-DU CELL SWITCH NOTIFICATION message with included acquired TA of the target cell in the TA Information List (TS 38.473) and there is no TA command sent by the target cell within the configured threshold.  The measurement is reported in the target DU per cell pair, i.e. the source and the target cell.

d)	A single integer value.

e)	MM.InterDULTMSucc.

f)	NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for MRO LTM.

==============Next change==============

5.1.1.6.y Measurements related to LTM cell-switch with almost invalid TA (intra and inter DU scenarios)

5.1.1.6.y.c   Number of successful intra DU LTM cell switch cell switches with declared almost invalid TA per cell pair

a)	This measurement provides the number of successful intra DU LTM cell switch with declared almost invalid TA from the source to the target cell.

b)	CC

c)	On time interval between the successful intra DU LTM cell switch is triggered by successful reception of the RRC Reconfiguration complete or first UL message by DU for previously sent Cell switch command containing acquired TA value of the target cell (TS 38.331) from the DU to UE and first TA command (MAC CE) (TS 38.321) containing the TA value different from the acquired TA of the target cell below a configured threshold (vendor specific). The measurement is reported in the DU per cell pair, i.e. source cell and target cell.

d)	A single integer value.

e)	MM.IntraDULTMSuccalmostInvalidTA.

f)	NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for MRO LTM.

==============Next change==============

5.1.1.6.y Measurements related to LTM cell-switch with almost invalid TA (intra and inter DU scenarios)

5.1.1.6.y.d   Number of successful intra DU LTM cell switch per cell pair

a)	This measurement provides the number of successful intra DU LTM cell switch from the source to the target cell.

b)	CC

c)	The successful intra DU LTM cell switch is triggered by successful reception of the RRC Reconfiguration complete or first UL message by DU for previously sent Cell switch command containing acquired TA value of the target cell (TS 38.331) from DU to UE and there is no TA command sent by the target gNB-DU within the configured threshold.  The measurement is reported in the DU per cell pair, i.e. source cell and target cell.

d)	A single integer value.

e)	MM.IntraDULTMSucc.

f)	NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for MRO LTM.

==============End of change==============