3GPP TSG-SA5 Meeting #165	S5-260836

Goa, India, 09 - 13 February 2026

Source:	China Mobile, ZTE Corporation, Samsung, China Unicom

Title:	pCR on TR 32.801-01 Consolidated-Category 2: Autonomous agent within 3GPP management system

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 32.801-01

Version:	0.0.0

Work Item:	FS_6G_OAM

Comments

This contribution is a consolidation of the following documents and has been prepared to assist with efficiency of the discussions:

1. S5-260154 Autonomous Agent Reshapes 6G Network Operation and Management, ZTE

2. S5-260171 Autonomous agents in the 6G management system, China Mobile

3. S5-260628 Agent-driven multi-RAT management, Samsung

4. S5-260318 Agent-based predictive maintenance Scenarios, China Unicom

Proposed Changes

* * * First Change * * * *

6	6G Management Scenarios

6.1 Identified 6G management scenarios

6.1.X Autonomous Agent

6.1.X.Y Category 2: Autonomous agent within 3GPP management system

6.1.X.Y.1 Description

1. General description

6G networks introduce new capabilities that significantly broaden the scope of network including services such as ISAC, which introduce more diverse managed entities and more complex operating environments. These scenarios greatly increase the heterogeneity and operational complexity of 6G networks and place significantly higher demands on the management system.

It should be explored how to support collaborative invocation of management capabilities to optimize and support the above scenarios through management Agents.

An Autonomous Agents is a continuously operating intelligent entity capable of perception, reasoning & planning, action execution, and iterative learning. It can dynamically adapt to changes, generalize to unfamiliar situations, and continuously refine its behavior based on feedback.

* * * End of Changes * * * *