3GPP TSG-SA5 (Telecom Management)	S5-070131

- Meeting SA5#51, 22 - 26 Jan 2007, Seville, SPAIN

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0058 |  rev | - |  Current version: | 7.1.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add new cause values for closing a service data container - Align with 32.251 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Telefónica (blanca.segurovelasco@telefonica.es) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | PCC-CH |  |  |  |  |  |  | Date:  |  | 26/01/07 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | Event counters have been defined as triggers for the addition of service data flow containers. However the ServiceConditionChange parameter has not been updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | The ServiceConditionChange parameter is corrected to include two new values needed for event based charging. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | In certain scenarios related to event based charging, the ServiceConditionChange parameter will not correctly describe the cause for closing a service data container. Misalignment with 32.251. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

#### 5.2.2.2	PS domain CDRs

This subclause contains the abstract syntax definitions that are specific to the GPRS CDR types defined in TS 32.251 [11].

GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

CallDuration, CalledNumber, RecordType, CallingNumber, CallReferenceNumber, CellId, DefaultSMS-Handling, Diagnostics, Ext-GeographicalInformation, IMSI, IMEI, IPAddress, ISDN-AddressString, LCSCause, LCSClientExternalID, LCSClientIdentity, LCSClientInternalID, LCSClientType, LCS-Priority, LCSQoSInfo, LevelOfCAMELService, LocalSequenceNumber, LocationAreaAndCell, LocationAreaCode, LocationType, ManagementExtensions, MessageReference, MSISDN, NotificationToMSUser, PositioningData, RecordingEntity, ServiceKey, SMSResult, SmsTpDestinationNumber, TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

DefaultGPRS-Handling, RAIdentity

FROM MAP-MS-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version6 (6)}

--

-- from TS 29.002 [60]

LocationMethod

FROM SS-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Access (2) modules (3) ss-DataTypes (2) version7 (7)}

--

-- from TS 24.080 [61]

MBMS2G3GIndicator, FileRepairSupported, MBMSServiceType, MBMSUserServiceType, RequiredMBMSBearerCapabilities, MBMSSessionIdentity, TMGI, MBMSInformation

FROM MBMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) mbmsChargingDataTypes (8) asn1Module (0) version1 (0)}

Editor’s note:  consider moving the above 2 items also into the generic module in order to avoid again copying from external sources.

;

------------------------------------------------------------------------------

--

--  GPRS RECORDS

--

------------------------------------------------------------------------------

GPRSRecord	::= CHOICE

--

-- Record values 18..27 are GPRS specific

-- Record value 70 is FBC specific

-- Record values 76..77 are GPRS and MBMS specific

{

sgsnPDPRecord			[18] SGSNPDPRecord,

ggsnPDPRecord			[19] GGSNPDPRecord,

sgsnMMRecord			[20] SGSNMMRecord,

sgsnSMORecord			[21] SGSNSMORecord,

sgsnSMTRecord			[22] SGSNSMTRecord,

sgsnLCTRecord			[26] SGSNLCTRecord,

sgsnLCORecord			[27] SGSNLCORecord,

sgsnLCNRecord			[28] SGSNLCNRecord,

--

egsnPDPRecord			[70] EGSNPDPRecord,

--

sgsnMBMSRecord			[76] SGSNMBMSRecord,

ggsnMBMSRecord			[77] GGSNMBMSRecord

}

GGSNPDPRecord 	::= SET

{

recordType					[0] RecordType,

networkInitiation			[1] NetworkInitiatedPDPContext OPTIONAL,

servedIMSI					[3] IMSI,

ggsnAddress					[4] GSNAddress,

chargingID					[5] ChargingID,

sgsnAddress					[6] SEQUENCE OF GSNAddress,

accessPointNameNI			[7] AccessPointNameNI OPTIONAL,

pdpType						[8] PDPType OPTIONAL,

servedPDPAddress			[9] PDPAddress OPTIONAL,

dynamicAddressFlag			[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes		[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[13] TimeStamp,

duration					[14] CallDuration,

causeForRecClosing			[15] CauseForRecClosing,

diagnostics					[16] Diagnostics OPTIONAL,

recordSequenceNumber		[17] INTEGER OPTIONAL,

nodeID						[18] NodeID OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

localSequenceNumber			[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[21] APNSelectionMode OPTIONAL,

servedMSISDN				[22] MSISDN OPTIONAL,

chargingCharacteristics		[23] ChargingCharacteristics,

chChSelectionMode			[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext			[25] NULL OPTIONAL,

externalChargingID			[26] OCTET STRING OPTIONAL,

sgsnPLMNIdentifier			[27] PLMN-Id OPTIONAL,

servedIMEISV				[29] IMEI OPTIONAL,

rATType						[30] RATType OPTIONAL,

mSTimeZone 					[31] MSTimeZone OPTIONAL,

userLocationInformation		[32] OCTET STRING OPTIONAL,

cAMELChargingInformation	[33] OCTET STRING OPTIONAL

}

EGSNPDPRecord 	::= SET

{

recordType						[0] RecordType,

networkInitiation				[1] NetworkInitiatedPDPContext OPTIONAL,

servedIMSI						[3] IMSI,

ggsnAddress						[4] GSNAddress,

chargingID						[5] ChargingID,

sgsnAddress						[6] SEQUENCE OF GSNAddress,

accessPointNameNI				[7] AccessPointNameNI OPTIONAL,

pdpType							[8] PDPType OPTIONAL,

servedPDPAddress				[9] PDPAddress OPTIONAL,

dynamicAddressFlag				[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes			[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime				[13] TimeStamp,

duration						[14] CallDuration,

causeForRecClosing				[15] CauseForRecClosing,

diagnostics						[16] Diagnostics OPTIONAL,

recordSequenceNumber			[17] INTEGER OPTIONAL,

nodeID							[18] NodeID OPTIONAL,

recordExtensions				[19] ManagementExtensions OPTIONAL,

localSequenceNumber				[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode				[21] APNSelectionMode OPTIONAL,

servedMSISDN					[22] MSISDN OPTIONAL,

chargingCharacteristics			[23] ChargingCharacteristics,

chChSelectionMode				[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext				[25] NULL OPTIONAL,

externalChargingID				[26] OCTET STRING OPTIONAL,

sgsnPLMNIdentifier				[27] PLMN-Id OPTIONAL,

pSFurnishChargingInformation	[28] PSFurnishChargingInformation OPTIONAL,

servedIMEISV					[29] IMEI OPTIONAL,

rATType							[30] RATType OPTIONAL,

mSTimeZone 						[31] MSTimeZone OPTIONAL,

userLocationInformation			[32] OCTET STRING OPTIONAL,

cAMELChargingInformation		[33] OCTET STRING OPTIONAL,

listOfServiceData				[34] SEQUENCE OF ChangeOfServiceCondition OPTIONAL

}

SGSNMMRecord	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

servedIMEI					[2] IMEI OPTIONAL,

sgsnAddress					[3] GSNAddress OPTIONAL,

msNetworkCapability			[4] MSNetworkCapability OPTIONAL,

routingArea					[5] RoutingAreaCode OPTIONAL,

locationAreaCode			[6] LocationAreaCode OPTIONAL,

cellIdentifier				[7] CellId OPTIONAL,

changeLocation				[8] SEQUENCE OF ChangeLocation OPTIONAL,

recordOpeningTime			[9] TimeStamp,

duration					[10] CallDuration OPTIONAL,

sgsnChange					[11] SGSNChange OPTIONAL,

causeForRecClosing			[12] CauseForRecClosing,

diagnostics					[13] Diagnostics OPTIONAL,

recordSequenceNumber		[14] INTEGER OPTIONAL,

nodeID						[15] NodeID OPTIONAL,

recordExtensions			[16] ManagementExtensions OPTIONAL,

localSequenceNumber			[17] LocalSequenceNumber OPTIONAL,

servedMSISDN				[18] MSISDN OPTIONAL,

chargingCharacteristics		[19] ChargingCharacteristics,

cAMELInformationMM 			[20] CAMELInformationMM OPTIONAL,

rATType						[21] RATType OPTIONAL,

chChSelectionMode			[22] ChChSelectionMode OPTIONAL

}

SGSNPDPRecord 	::= SET

{

recordType					[0] RecordType,

networkInitiation			[1] NetworkInitiatedPDPContext OPTIONAL,

servedIMSI					[3] IMSI,

servedIMEI					[4] IMEI OPTIONAL,

sgsnAddress					[5] GSNAddress OPTIONAL,

msNetworkCapability			[6] MSNetworkCapability OPTIONAL,

routingArea					[7] RoutingAreaCode OPTIONAL,

locationAreaCode			[8] LocationAreaCode OPTIONAL,

cellIdentifier				[9] CellId OPTIONAL,

chargingID					[10] ChargingID,

ggsnAddressUsed				[11] GSNAddress,

accessPointNameNI			[12] AccessPointNameNI OPTIONAL,

pdpType						[13] PDPType OPTIONAL,

servedPDPAddress			[14] PDPAddress OPTIONAL,

listOfTrafficVolumes		[15] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[16] TimeStamp,

duration					[17] CallDuration,

sgsnChange					[18] SGSNChange OPTIONAL,

causeForRecClosing			[19] CauseForRecClosing,

diagnostics					[20] Diagnostics OPTIONAL,

recordSequenceNumber		[21] INTEGER OPTIONAL,

nodeID						[22] NodeID OPTIONAL,

recordExtensions			[23] ManagementExtensions OPTIONAL,

localSequenceNumber			[24] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[25] APNSelectionMode OPTIONAL,

accessPointNameOI			[26] AccessPointNameOI OPTIONAL,

servedMSISDN				[27] MSISDN OPTIONAL,

chargingCharacteristics		[28] ChargingCharacteristics,

rATType						[29] RATType OPTIONAL,

cAMELInformationPDP  		[30] CAMELInformationPDP OPTIONAL,

rNCUnsentDownlinkVolume	   [31] DataVolumeGPRS OPTIONAL,

chChSelectionMode			[32] ChChSelectionMode OPTIONAL,

dynamicAddressFlag			[33] DynamicAddressFlag OPTIONAL

}

SGSNSMORecord	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

servedIMEI					[2] IMEI OPTIONAL,

servedMSISDN				[3] MSISDN OPTIONAL,

msNetworkCapability			[4] MSNetworkCapability OPTIONAL,

serviceCentre				[5] AddressString OPTIONAL,

recordingEntity				[6] RecordingEntity OPTIONAL,

locationArea				[7] LocationAreaCode OPTIONAL,

routingArea					[8] RoutingAreaCode OPTIONAL,

cellIdentifier				[9] CellId OPTIONAL,

messageReference			[10] MessageReference,

eventTimeStamp				[11] TimeStamp,

smsResult					[12] SMSResult OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

nodeID						[14] NodeID OPTIONAL,

localSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		[16] ChargingCharacteristics,

rATType						[17] RATType OPTIONAL,

destinationNumber			[18] SmsTpDestinationNumber OPTIONAL,

cAMELInformationSMS			[19] CAMELInformationSMS OPTIONAL,

chChSelectionMode			[20] ChChSelectionMode OPTIONAL

}

SGSNSMTRecord	::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedIMEI				[2] IMEI OPTIONAL,

servedMSISDN			[3] MSISDN OPTIONAL,

msNetworkCapability		[4] MSNetworkCapability OPTIONAL,

serviceCentre			[5] AddressString OPTIONAL,

recordingEntity			[6] RecordingEntity OPTIONAL,

locationArea			[7] LocationAreaCode OPTIONAL,

routingArea				[8] RoutingAreaCode OPTIONAL,

cellIdentifier			[9] CellId OPTIONAL,

eventTimeStamp			[10] TimeStamp,

smsResult				[11] SMSResult OPTIONAL,

recordExtensions		[12] ManagementExtensions OPTIONAL,

nodeID					[13] NodeID OPTIONAL,

localSequenceNumber		[14] LocalSequenceNumber OPTIONAL,

chargingCharacteristics	[15] ChargingCharacteristics,

rATType					[16] RATType OPTIONAL,

chChSelectionMode		[17] ChChSelectionMode OPTIONAL,

cAMELInformationSMS		[18] CAMELInformationSMS OPTIONAL

}

SGSNMTLCSRecord			::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType,

lcsClientIdentity		[3] LCSClientIdentity,

servedIMSI				[4] IMSI,

servedMSISDN			[5] MSISDN OPTIONAL,

sgsnAddress				[6] GSNAddress OPTIONAL,

locationType			[7] LocationType,

lcsQos					[8] LCSQoSInfo OPTIONAL,

lcsPriority				[9] LCS-Priority OPTIONAL,

mlcNumber				[10] ISDN-AddressString,

eventTimeStamp			[11] TimeStamp,

measurementDuration		[12] CallDuration OPTIONAL,

notificationToMSUser	[13] NotificationToMSUser OPTIONAL,

privacyOverride			[14] NULL OPTIONAL,

location				[15] LocationAreaAndCell OPTIONAL,

routingArea				[16] RoutingAreaCode OPTIONAL,

locationEstimate		[17] Ext-GeographicalInformation OPTIONAL,

positioningData			[18] PositioningData OPTIONAL,

lcsCause				[19] LCSCause OPTIONAL,

diagnostics				[20] Diagnostics OPTIONAL,

nodeID					[21] NodeID OPTIONAL,

localSequenceNumber		[22] LocalSequenceNumber OPTIONAL,

chargingCharacteristics	[23] ChargingCharacteristics,

chChSelectionMode		[24] ChChSelectionMode OPTIONAL,

rATType					[25] RATType OPTIONAL,

recordExtensions		[26] ManagementExtensions OPTIONAL,

causeForRecClosing		[27] CauseForRecClosing

}

SGSNMOLCSRecord			::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType OPTIONAL,

lcsClientIdentity		[3] LCSClientIdentity OPTIONAL,

servedIMSI				[4] IMSI,

servedMSISDN			[5] MSISDN OPTIONAL,

sgsnAddress				[6] GSNAddress OPTIONAL,

locationMethod			[7] LocationMethod,

lcsQos					[8] LCSQoSInfo OPTIONAL,

lcsPriority				[9] LCS-Priority OPTIONAL,

mlcNumber				[10] ISDN-AddressString OPTIONAL,

eventTimeStamp			[11] TimeStamp,

measurementDuration		[12] CallDuration OPTIONAL,

location				[13] LocationAreaAndCell OPTIONAL,

routingArea				[14] RoutingAreaCode OPTIONAL,

locationEstimate		[15] Ext-GeographicalInformation OPTIONAL,

positioningData			[16] PositioningData OPTIONAL,

lcsCause				[17] LCSCause OPTIONAL,

diagnostics				[18] Diagnostics OPTIONAL,

nodeID					[19] NodeID OPTIONAL,

localSequenceNumber	    [20] LocalSequenceNumber OPTIONAL,

chargingCharacteristics	[21] ChargingCharacteristics,

chChSelectionMode		[22] ChChSelectionMode OPTIONAL,

rATType					[23] RATType OPTIONAL,

recordExtensions		[24] ManagementExtensions OPTIONAL,

causeForRecClosing		[25] CauseForRecClosing

}

SGSNNILCSRecord			::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType OPTIONAL,

lcsClientIdentity		[3] LCSClientIdentity OPTIONAL,

servedIMSI				[4] IMSI OPTIONAL,

servedMSISDN			[5] MSISDN OPTIONAL,

sgsnAddress				[6] GSNAddress OPTIONAL,

servedIMEI				[7] IMEI OPTIONAL,

lcsQos					[8] LCSQoSInfo OPTIONAL,

lcsPriority				[9] LCS-Priority OPTIONAL,

mlcNumber				[10] ISDN-AddressString OPTIONAL,

eventTimeStamp			[11] TimeStamp,

measurementDuration		[12] CallDuration OPTIONAL,

location				[13] LocationAreaAndCell OPTIONAL,

routingArea			   [14] RoutingAreaCode OPTIONAL,

locationEstimate		[15] Ext-GeographicalInformation OPTIONAL,

positioningData			[16] PositioningData OPTIONAL,

lcsCause				[17] LCSCause OPTIONAL,

diagnostics				[18] Diagnostics OPTIONAL,

nodeID					[19] NodeID OPTIONAL,

localSequenceNumber	   [20] LocalSequenceNumber OPTIONAL,

chargingCharacteristics	[21] ChargingCharacteristics,

chChSelectionMode		[22] ChChSelectionMode OPTIONAL,

rATType					[23] RATType OPTIONAL,

recordExtensions		[24] ManagementExtensions OPTIONAL,

causeForRecClosing		[25] CauseForRecClosing

}

SGSNMBMSRecord 	::= SET

{

recordType					[0] RecordType,

ggsnAddress					[1] GSNAddress,

chargingID					[2] ChargingID,

listofRAs					[3] SEQUENCE OF RAIdentity OPTIONAL,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

servedPDPAddress			[5] PDPAddress OPTIONAL,

listOfTrafficVolumes		[6] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[7] TimeStamp,

duration					[8] CallDuration,

causeForRecClosing			[9] CauseForRecClosing,

diagnostics					[10] Diagnostics OPTIONAL,

recordSequenceNumber		[11] INTEGER OPTIONAL,

nodeID						[12] NodeID OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

localSequenceNumber			[14] LocalSequenceNumber OPTIONAL,

sgsnPLMNIdentifier			[15] PLMN-Id OPTIONAL,

numberofReceivingUE			[16] Integer OPTIONAL,

mbmsInformation				[17] MBMSInformation OPTIONAL

}

GGSNMBMSRecord 	::= SET

{

recordType					[0] RecordType,

ggsnAddress					[1] GSNAddress,

chargingID					[2] ChargingID,

listofDownstreamNodes		[3] SEQUENCE OF GSNAddress,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

servedPDPAddress			[5] PDPAddress OPTIONAL,

listOfTrafficVolumes		[6] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[7] TimeStamp,

duration					[8] CallDuration,

causeForRecClosing			[9] CauseForRecClosing,

diagnostics					[10] Diagnostics OPTIONAL,

recordSequenceNumber		[11] INTEGER OPTIONAL,

nodeID						[12] NodeID OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

localSequenceNumber			[14] LocalSequenceNumber OPTIONAL,

mbmsInformation				[15] MBMSInformation OPTIONAL

}

------------------------------------------------------------------------------

--

--  GPRS DATA TYPES

--

------------------------------------------------------------------------------

AccessPointNameNI	::= IA5String (SIZE(1..63))

--

-- Network Identifier part of APN in  dot representation.

-- For example, if the complete APN is 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs'

-- NI is 'apn1a.apn1b.apn1c' and is presented in this form in the CDR..

--

AccessPointNameOI	::= IA5String (SIZE(1..37))

--

-- Operator Identifier part of APN in dot representation.

-- In the 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs' example, the OI portion is 'mnc022.mcc111.gprs'

-- and is presented in this form in the CDR.

--

APNSelectionMode::= ENUMERATED

{

--

-- See Information Elements TS 29.060 [75]

--

mSorNetworkProvidedSubscriptionVerified				(0),

mSProvidedSubscriptionNotVerified					(1),

networkProvidedSubscriptionNotVerified				(2)

}

CAMELAccessPointNameNI	::= AccessPointNameNI

CAMELAccessPointNameOI	::= AccessPointNameOI

CAMELInformationMM		::= SET

{

sCFAddress						[1] SCFAddress OPTIONAL,

serviceKey						[2]	ServiceKey OPTIONAL,

defaultTransactionHandling	   [3]	DefaultGPRS-Handling OPTIONAL,

numberOfDPEncountered  		   [4] NumberOfDPEncountered OPTIONAL,

levelOfCAMELService			   [5] LevelOfCAMELService OPTIONAL,

freeFormatData					[6] FreeFormatData OPTIONAL,

fFDAppendIndicator	     		[7]	FFDAppendIndicator OPTIONAL

}

CAMELInformationPDP	::= SET

{

sCFAddress						[1] SCFAddress OPTIONAL,

serviceKey						[2]	ServiceKey OPTIONAL,

defaultTransactionHandling		[3]	DefaultGPRS-Handling OPTIONAL,

cAMELAccessPointNameNI			[4] CAMELAccessPointNameNI OPTIONAL,

cAMELAccessPointNameOI			[5] CAMELAccessPointNameOI OPTIONAL,

numberOfDPEncountered			[6] NumberOfDPEncountered OPTIONAL,

levelOfCAMELService				[7] LevelOfCAMELService OPTIONAL,

freeFormatData					[8]	FreeFormatData OPTIONAL,

fFDAppendIndicator				[9]	FFDAppendIndicator OPTIONAL

}

CAMELInformationSMS		::= SET

{

sCFAddress								[1] SCFAddress OPTIONAL,

serviceKey								[2]	ServiceKey OPTIONAL,

defaultSMSHandling						[3]	DefaultSMS-Handling OPTIONAL,

cAMELCallingPartyNumber					[4] CallingNumber OPTIONAL,

cAMELDestinationSubscriberNumber		[5] SmsTpDestinationNumber OPTIONAL,

cAMELSMSCAddress						[6] AddressString OPTIONAL,

freeFormatData							[7]	FreeFormatData OPTIONAL,

smsReferenceNumber						[8] CallReferenceNumber	OPTIONAL

}

CauseForRecClosing	::= INTEGER

{

--

-- In GGSN the value sGSNChange should be used for partial record

-- generation due to SGSN Address List Overflow

--

-- LCS related causes belong to the MAP error causes acc. TS 29.002 [60]

--

-- cause codes 0 to 15 are defined 'CauseForTerm' (cause for termination)

--

normalRelease					(0),

abnormalRelease			    	(4),

cAMELInitCallRelease			(5),

volumeLimit				    	(16),

timeLimit						(17),

sGSNChange						(18),

maxChangeCond					(19),

managementIntervention	    	(20),

intraSGSNIntersystemChange		(21),

rATChange						(22),

mSTimeZoneChange				(23),

unauthorizedRequestingNetwork	(52),

unauthorizedLCSClient			(53),

positionMethodFailure			(54),

unknownOrUnreachableLCSClient	(58),

listofDownstreamNodeChange		(59)

}

ChangeCondition	::= ENUMERATED

{

--

-- Failure Handling values used in eGCDR only

--

qoSChange				       			(0),

tariffTime				       			(1),

recordClosure			       			(2),

failureHandlingContinueOngoing		   	(3),

failureHandlingRetryandTerminateOngoing (4),

failureHandlingTerminateOngoing	   		(5),

cGI-SAICHange							(6),

rAIChange								(7)

}

ChangeOfCharCondition	::= SEQUENCE

{

--

-- Used in PDP context record only

-- failureHandlingContinue field used in eGCDR only

--

qosRequested				[1] QoSInformation OPTIONAL,

qosNegotiated				[2] QoSInformation OPTIONAL,

dataVolumeGPRSUplink		[3] DataVolumeGPRS,

dataVolumeGPRSDownlink		[4] DataVolumeGPRS,

changeCondition				[5] ChangeCondition,

changeTime					[6] TimeStamp,

failureHandlingContinue		[7] FailureHandlingContinue OPTIONAL,

userLocationInformation		[8] OCTET STRING OPTIONAL

}

ChangeOfMBMSCondition	::= SEQUENCE

{

--

-- Used in MBMS record

--

qosRequested				[1] QoSInformation OPTIONAL,

qosNegotiated				[2] QoSInformation OPTIONAL,

dataVolumeMBMSUplink		[3] DataVolumeMBMS OPTIONAL,

dataVolumeMBMSDownlink		[4] DataVolumeMBMS,

changeCondition				[5] ChangeCondition,

changeTime					[6] TimeStamp,

failureHandlingContinue		[7] FailureHandlingContinue OPTIONAL

}

ChangeOfServiceCondition	::= SEQUENCE

{

--

-- Used for Flow based Charging service data container

--

ratingGroup 					[1] RatingGroupId,

chargingRuleBaseName			[2] ChargingRuleBaseName OPTIONAL,

resultCode						[3] ResultCode OPTIONAL,

localSequenceNumber				[4] LocalSequenceNumber OPTIONAL,

timeOfFirstUsage				[5] TimeStamp OPTIONAL,

timeOfLastUsage					[6] TimeStamp OPTIONAL,

timeUsage 						[7] CallDuration OPTIONAL,

serviceConditionChange			[8] ServiceConditionChange,

qoSInformationNeg				[9] QoSInformation OPTIONAL,

sgsn-Address 					[10] GSNAddress OPTIONAL,

sGSNPLMNIdentifier 				[11] SGSNPLMNIdentifier OPTIONAL,

datavolumeFBCUplink				[12] DataVolumeGPRS OPTIONAL,

datavolumeFBCDownlink			[13] DataVolumeGPRS OPTIONAL,

timeOfReport					[14] TimeStamp,

rATType							[15] RATType OPTIONAL,

failureHandlingContinue			[16] FailureHandlingContinue OPTIONAL,

serviceIdentifier				[17] ServiceIdentifier OPTIONAL,

pSFurnishChargingInformation	[18] PSFurnishChargingInformation OPTIONAL,

aFRecordInformation				[19] SEQUENCE OF OCTET STRING OPTIONAL,

userLocationInformation			[20] OCTER STRING OPTIONAL,

eventBasedChargingInformation   [21] EventBasedChargingInformation OPTIONAL

}

ChangeLocation	::= SEQUENCE

{

--

-- used in SGSNMMRecord only

--

locationAreaCode		[0] LocationAreaCode,

routingAreaCode			[1] RoutingAreaCode,

cellId					[2] CellId OPTIONAL,

changeTime				[3] TimeStamp

}

ChargingCharacteristics	::= OCTET STRING (SIZE(2))

--

--	Bit 0-3: Profile Index

--	Bit 4-15: For Behavior

--

ChargingID	::= INTEGER (0..4294967295)

--

-- Generated in GGSN, part of PDP context, see TS 23.060 [74]

-- 0..4294967295 is equivalent to 0..2**32-1

--

ChargingRuleBaseName ::= IA5String (SIZE(1..16))

--

-- identifier for the group of charging rules

-- see Charging-Rule-Base-Name AVP as desined in TS 29.210 [85]

--

ChChSelectionMode		::= ENUMERATED

{

sGSNSupplied				(0),	-- For GGSN only

subscriptionSpecific		(1),	-- For SGSN only

aPNSpecific					(2),	-- For SGSN only

homeDefault					(3),	-- For SGSN and GGSN

roamingDefault				(4),	-- For SGSN and GGSN

visitingDefault				(5)		-- For SGSN and GGSN

}

DataVolumeGPRS	::= INTEGER

--

-- The volume of data transferred in octets.

--

DynamicAddressFlag	::= BOOLEAN

ETSIAddress	::= AddressString

--

-- First octet for nature of address, and numbering plan indicator (3 for X.121)

-- Other octets TBCD

-- See TS 29.002 [60]

-- See TS 29.002 [60]

--

EventBasedChargingInformation  ::= SEQUENCE

{

numberOfEvents		[1] INTEGER,

eventTimeStamps  	[2] SEQUENCE OF TimeStamp OPTIONAL

}

FailureHandlingContinue  ::= BOOLEAN

--

-- This parameter is included when the failure handling procedure has been executed and new

-- containers are opened. This parameter shall be included in the first and subsequent

-- containers opened after the failure handling execution.

--

FFDAppendIndicator	::= BOOLEAN

FreeFormatData	::=	OCTET STRING (SIZE(1..160))

--

-- Free formated data as sent in the FurnishChargingInformationGPRS

-- see TS 29.078 [66]

--

GSNAddress	::= IPAddress

MSNetworkCapability	::= OCTET STRING (SIZE(1..8))

-- see TS 24.008 [64]

NetworkInitiatedPDPContext	::= BOOLEAN

--

-- Set to true if PDP context was initiated from network side

--

NodeID	::= IA5String (SIZE(1..20))

NumberOfDPEncountered  ::= INTEGER

PDPAddress	::= CHOICE

{

iPAddress				[0] IPAddress,

eTSIAddress			   [1] ETSIAddress

}

PDPType		::= OCTET STRING (SIZE(2))

--

-- OCTET 1: PDP Type Organization

-- OCTET 2: PDP Type Number

-- See TS 29.060 [75]

--

PLMN-Id		::= OCTET STRING (SIZE (3))

--

--	This is a 1:1 copy from the Routing Area Identity (RAI) IE specified in TS 29.060 [75]

--  as follows:

--	OCTET 1 of PLMN-Id = OCTET 2 of RAI

--	OCTET 2 of PLMN-Id = OCTET 3 of RAI

--	OCTET 3 of PLMN-Id = OCTET 4 of RAI

PSFurnishChargingInformation  ::= SEQUENCE

{

pSFreeFormatData		[1] FreeFormatData,

pSFFDAppendIndicator  	[2] FFDAppendIndicator OPTIONAL

}

QoSInformation	::= OCTET STRING (SIZE (4..15))

--

-- This  octet string

-- is a 1:1 copy of the contents (i.e. starting with octet 4) of the "Quality of

-- service Profile" information element specified in TS 29.060 [75].

RatingGroup ::= INTEGER

--

-- IP service flow identity (DCCA), range of 4 byte (0...4294967259)

-- see Rating-Group AVP as used in TS 32.299 [40]

--

RATType	::= INTEGER (0..255)

--

-- Ihis integer is 1:1 copy of the RAT type value as defined in TS 29.060 [75].

--

ResultCode ::= INTEGER
--

-- charging protocol return value, range of 4 byte (0...4294967259)

-- see Result-Code AVP as used in 3GPP 29.210 [85]

--

RoutingAreaCode	::= OCTET STRING (SIZE(1))

--

-- See TS 24.008 [64]

--

ServiceConditionChange	::= BIT STRING 
{

qoSChange 										(0),

sGSNChange 										(1),

sGSNPLMNIDChange 								(2),

tariffTimeSwitch 								(3),

pDPContextRelease 								(4),

rATChange 										(5),

serviceIdledOut 								(6),

qCTExpiry 										(7),

configurationChange 							(8),

serviceStop 									(9),

timeThresholdReached 							(10),

volumeThresholdReached 							(11),

serviceSpecificUnitThresholdReached				(12),

timeExhausted 									(13),

volumeExhausted 								(14),

timeout 										(15),

returnRequested 								(16),

reauthorisationRequest 							(17),

continueOngoingSession 							(18),

retryAndTerminateOngoingSession					(19),

terminateOngoingSession 						(20),

cGI-SAIChange									(21),

rAIChange										(22),

serviceSpecificUnitExhausted					(23)

}

-- Bits 0-5 and 21-22 are cause values for Gn update/release and TTS

-- Bits 6-9 are cause values for service stop

-- Bits 10-14 and bit 23 are cause values for service reauthorization request

-- Bits 15-17 are cause values for quota return

-- Bits 18-20: are cause values for Failure Handling Procedure

-- Bits 24-32: are unused and will always be zero

-- some of the values are non-exclusive

-- serviceIdledOut – bit 6 is equivalent to service release by QHT

SCFAddress	::= AddressString

--

-- See TS 29.002 [60]

--

ServiceIdentifier	::= INTEGER (0..4294967295)

--

-- The service identifier is used to identify the service or the service component

-- the service data flow relates to. See Service-Identifier AVP as defined

-- in TS 29.210 [85]

--

SGSNChange	::= BOOLEAN

--

-- present if first record after inter SGSN routing area update

-- in new SGSN

--

END

End of modifications