3GPP TSG-SA5 (Telecom Management)	S5-070173

- Meeting SA5#51, 22 - 26 Jan 2007, Seville, SPAIN

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.732 | CR | 0001 |  rev | - |  Current version: | 7.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add HssFunction to IMS NRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Nokia, Ericsson (Gyula Bodog - Nokia) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM7-NIM |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | HssFunction is missing from the IMS NRM |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Adding HssFunction and its link objects Link_As_Hss, Link_Hss_Scscf and Link_Cscf_Hss to the IMS Network Resource Model. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | If HSS is not modelled there will be no possibility for managing the HSS in the 3GPP network. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 6.2, 6.3, 6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  | x |  | O&M Specifications |  |  | 32.733, 32.735, 32.632, 32.633, 32.635 |  |  |  |
| Other comments:  |  | Related R7 CR to 32.733 is in S5-070174, CR to 32.735 is in S5-070175, CR to 32.632 is in S5-070176, CR to 32.633 is in S5-070177 and CR to 32.635 is in S5-070178 |  |  |  |  |  |  |  |  |

1st Modified Section

- 6.2	Class diagram

- 6.2.1	Attributes and relationships

This clause depicts the set of IOCs that encapsulate information relevant for this service. This clause provides the overview of all information object classes in UML. Subsequent clauses provide more detailed specification of various aspects of these information object classes.

The figures below show the containment/naming hierarchy and the associations of the information object classes defined in the present document.

NOTE:	The listed cardinality numbers represent transient as well as steady-state numbers, and reflect all managed object creation and deletion scenarios in all figures.

Figure 6.2.1.1: IMS NRM Containment/Naming relationships

Note:	If this NRM has interfaces (Link IOCs) modelled to IOCs in other NRM(s), the Link IOC definitions may be defined in the other NRM(s) and need to be considered in implementations. For example, see Link_Bgcf_Mgcf and Link_Cscf_Mgcf in CN NRM TS 32.632  [11].

Figure 6.2.1.2: IMS NRM Link Associations 1

Figure 6.2.1.3: IMS NRM Link Associations 2

Figure 6.2.1.4: IMS NRM Link Associations 3

Figure 6.2.1.5: IMS NRM Link Associations 4

Next Modified Section

### 6.2.2	Inheritance

This clause depicts the inheritance relationships that exist between  IOCs.

Figure 6.2.2.1: IMS NRM Inheritance Hierarchy 1

Figure 6.2.2.2: IMS NRM Inheritance Hierarchy 2

Figure 6.2.2.3: IMS NRM Inheritance Hierarchy 3

Next Modified Section

- 6.3.1	AsFunction

- 6.3.1.1	Definition

This IOC represents AS functionality. For more information about the AS, see 3GPP TS 23.002 [8].

- 6.3.1.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| asFunctionId | + | M | M | - |

- 6.3.2	BgcfFunction

- 6.3.2.1	Definition

This IOC represents BGCF functionality. For more information about the BGCF, see 3GPP TS 23.002 [8].

- 6.3.2.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| bgcfFunctionId | + | M | M | - |

- 6.3.3	CscfFunction

- 6.3.3.1	Definition

This IOC represents CSCF functionality. For more information about the CSCF, see 3GPP TS 23.002 [8].  CscfFunction shall not be instantiated to be name-contained by a ManagedElement instance that contains any of the role based CSCFs, namely PcscfFunction, IcscfFunction or ScscfFunction.

- 6.3.3.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| cscfFunctionId | + | M | M | - |

- 6.3.4	IcscfFunction

- 6.3.4.1	Definition

This IOC represents I-CSCF functionality. For more information about the I-CSCF, see 3GPP TS 23.002 [8].

- 6.3.4.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| icscfFunctionId | + | M | M | - |

- 6.3.5	ImsMgwFunction

- 6.3.5.1	Definition

This IOC represents IMS-MGW functionality. For more information about IMS-MGW, see 3GPP TS 23.002 [8].

- 6.3.5.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| imsMgwFunctionId | + | M | M | - |

- 6.3.6	MgcfFunction

- 6.3.6.1	Definition

This IOC represents MGCF functionality. For more information about the MGCF, see 3GPP TS 23.002 [8].

- 6.3.6.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| mgcfFunctionId | + | M | M | - |

- 6.3.7	MrfcFunction

- 6.3.7.1	Definition

This IOC represents MRFC functionality. For more information about the MRFC, see 3GPP TS 23.002 [8].

- 6.3.7.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| mrfcFunctionId | + | M | M | - |

- 6.3.8	MrfpFunction

- 6.3.8.1	Definition

This IOC represents MRFP functionality. For more information about the MRFP, see 3GPP TS 23.002 [8].

- 6.3.8.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| mrfpFunctionId | + | M | M | - |

- 6.3.9	PcscfFunction

- 6.3.9.1	Definition

This IOC represents P-CSCF functionality. For more information about the P-CSCF, see 3GPP TS 23.002 [8].

- 6.3.9.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| pcscfFunctionId | + | M | M | - |

- 6.3.10	ScscfFunction

- 6.3.10.1	Definition

This IOC represents S-CSCF functionality. For more information about the S-CSCF, see 3GPP TS 23.002 [8].

- 6.3.10.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| scscfFunctionId | + | M | M | - |

- 6.3.11	SlfFunction

- 6.3.11.1	Definition

This IOC represents SLF functionality. For more information about the SLF, see 3GPP TS 23.002 [8].

- 6.3.11.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| slfFunctionId | + | M | M | - |

- 6.3.12	Link_As_Cscf

- 6.3.12.1	Definition

This IOC models the Isc reference point as defined in TS 23.002 [8].

- 6.3.13	Link_As_Scscf

- 6.3.13.1	Definition

This IOC models the Isc reference point as defined in TS 23.002 [8].

- 6.3.14	Link_As_Slf

- 6.3.14.1	Definition

This models the Dh reference point as defined in TS 23.002 [8].

- 6.3.15	Link_Bgcf_Bgcf

- 6.3.15.1	Definition

This models the Mk reference point as defined in TS 23.002 [8].

- 6.3.16	Link_Bgcf_Cscf

- 6.3.16.1	Definition

This models the Mi reference point as defined in TS 23.002 [8].

- 6.3.17	Link_Bgcf_Mgcf

- 6.3.17.1	Definition

This models the Mj reference point as defined in TS 23.002 [8].

- 6.3.18	Link_Bgcf_Scscf

- 6.3.18.1	Definition

This models the Mi reference point as defined in TS 23.002 [8].

- 6.3.19	Link_Cscf_Cscf

- 6.3.19.1	Definition

This models the Mw reference point as defined in TS 23.002 [8].

- 6.3.20	Link_Cscf_Icscf

- 6.3.20.1	Definition

This models the Mw reference point as defined in TS 23.002 [8].

- 6.3.21	Link_Cscf_Mgcf

- 6.3.21.1	Definition

This IOC models the Mg reference point as defined in TS 23.002 [8].

- 6.3.22	Link_Cscf_Mrfc

- 6.3.22.1	Definition

This IOC models the Mr reference point as defined in TS 23.002 [8].

- 6.3.23	Link_Cscf_Pcscf

- 6.3.23.1	Definition

This models the Mw reference point as defined in TS 23.002 [8].

- 6.3.24	Link_Cscf_Scscf

- 6.3.24.1	Definition

This models the Mw reference point as defined in TS 23.002 [8].

- 6.3.25	Link_Cscf_Slf

- 6.3.25.1	Definition

This IOC models the Dx reference point as defined in TS 23.002 [8].

- 6.3.26	Link_Icscf_Slf

- 6.3.26.1	Definition

This models the Dx reference point as defined in TS 23.002 [8].

- 6.3.27	Link_ImsMgw_Mgcf

- 6.3.27.1	Definition

This models the Mn reference point as defined in TS 23.002 [8].

- 6.3.28	Link_Mgcf_Scscf

- 6.3.28.1	Definitions

This models the Mg reference point as defined in TS 23.002 [8].

- 6.3.29	Link_Mrfc_Mrfp

- 6.3.29.1	Definition

This IOC models the Mp reference point as defined in TS 23.002 [8].

- 6.3.30	Link_Mrfc_Scscf

- 6.3.30.1	Definition

This IOC models the Mr reference point as defined in TS 23.002 [8].

- 6.3.31	Link_Scscf_Scscf

- 6.3.31.1	Definition

This models the Dh reference point as defined in TS 23.002 [8].

- 6.3.32	Link_Scscf_Slf

- 6.3.32.1	Definition

This IOC models the Dx reference point as defined in TS 23.002 [8].

### 6.3.xx	HssFunction

#### 6.3.xx.1	Definition

This IOC represents HSS functionality. For more information about the HSS, see 3GPP TS 23.002 [8].

#### 6.3.xx.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| hssFunctionId | + | M | M | - |

### 6.3.yy	Link_As_Hss

#### 6.3.yy.1	Definition

This IOC models the Sh reference point as defined in TS 23.002 [8].

### 6.3.zz	Link_Hss_Scscf

#### 6.3.zz.1	Definition

This IOC models the Cx reference point as defined in TS 23.002 [8].

### 6.3.vv	Link_Cscf_Hss

#### 6.3.vv.1	Definition

This IOC models the Cx reference point as defined in TS 23.002 [8].

Next Modified Section

## 6.5	Information attribute definitions

### 6.5.1	Definition and legal values

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| asFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| bgcfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| cscfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| icscfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| imsMgwFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| mgcfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| mrfcFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| mrfpFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| pcscfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| scscfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| slfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| hssFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |

End of modifications

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | R | Subject/Comment | Cat | Old | New |
| Sep 2006 | SA_33 | SP-060563 | -- | -- | Submitted to TSG SA #33 for Information | -- | -- | 1.0.0 |
| Dec 2006 | SA_34 | SP-060750 | -- | -- | Submitted to TSG SA #34 for Approval | -- | 2.0.0 | 7.0.0 |