3GPP TSG-SA5 (Telecom Management)	S5-070177

- Meeting SA5#51, 22 - 26 Jan 2007, Seville, SPAIN

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.633 | CR | 0019 |  rev | - |  Current version: | 7.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Removing IMS related links from the CN NRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Nokia, Ericsson (Gyula.bodog@nokia.com) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM7-NIM |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | The link between HSS and the IMS NEs are defined in the IMS NRM. No need to define links between HLR and IMS NEs |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Removing from the modelling the following links: Link_Hlr_Scscf, Link_Cscf_Hlr, Link_As_Auc and Link_As_Hlr according to the IS specification |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Not existing and not correct links are defined in the specification, which can lead to wrong implementation. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | Section 1, 5.2.41, 5.2.42, 5.2.48, 5.2.62, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  | x |  | O&M Specifications |  |  | 32.632, 32.635, 32.732, 32.733, 32.735 |  |  |  |
| Other comments:  |  | Related R7 CR to 32.732 is in S5-070173 CR to 32.733 is in S5-070174 CR to 32.735 is in S5-070175 CR to 32.632 is in S5-070176 and CR to 32.635 is in S5-070178 |  |  |  |  |  |  |  |  |

1st Modified Section

# 1	Scope

The purpose of the present document is to define the mapping of the IRP information model (see TS 32.632 [3]) to the protocol specific details necessary for implementation of this IRP in a CORBA/IDL environment.

This Solution Set specification is related to 3GPP TS 32.632 V7.1.X.

Next Modified Section

## 5.2	Information Object Class (IOC) mapping

### 5.2.1	IOC MscServerFunction

Mapping from NRM IOC MscServerFunction attributes to SS equivalent MOC MscServerFunction attributes

| Attributes of IOC MscFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| mscServerFunctionId | mscServerFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |
| mccList | mccList | GenericNetworkResourcesIRPSystem::AttributeTypes:: LongSet | Read-Write, M |
| mncList | mncList | GenericNetworkResourcesIRPSystem::AttributeTypes:: LongSet | Read-Write, M |
| lacList | lacList | GenericNetworkResourcesIRPSystem::AttributeTypes:: LongSet | Read-Write, M |
| sacList | sacList | GenericNetworkResourcesIRPSystem::AttributeTypes:: LongSet | Read-Write, M |
| gcaList | gcaList | GenericNetworkResourcesIRPSystem::AttributeTypes:: LongSet | Read-Write, M |
| mscId | mscId | long | Read-Write, M |
| Associated With/mscServerFunction-GsmCell | mscServerFunction-GsmCell | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReferenceSet | Read-Only, M |
| Associated With/mscServerFunction-ExternalGsmCell | mscServerFunction-ExternalGsmCell | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReferenceSet | Read-Only, M |
| Associated With/mscServerFunction-CsMgwFunction | mscServerFunction-CsMgwFunction | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReferenceSet | Read-Only, M |

### 5.2.2	IOC HlrFunction

Mapping from NRM IOC HlrFunction attributes to SS equivalent MOC HlrFunction attributes

| Attributes of IOC HlrFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| hlrFunctionId | hlrFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.3	IOC VlrFunction

Mapping from NRM IOC VlrFunction attributes to SS equivalent MOC VlrFunction attributes

| Attributes of IOC VlrFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| vlrFunctionId | vlrFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.4	IOC AucFunction

Mapping from NRM IOC AucFunction attributes to SS equivalent MOC AucFunction attributes

| Attributes of IOC AucFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| aucFunctionId | aucFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.5	IOC EirFunction

Mapping from NRM IOC EirFunction attributes to SS equivalent MOC EirFunction attributes

| Attributes of IOC EirFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| eirFunctionId | eirFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.6	IOC SmsIwmscFunction

Mapping from NRM IOC SmsIwmscFunction attributes to SS equivalent MOC SmsIwmscFunction attributes

| Attributes of IOC SmsIwmscFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| smsIwmscFunctionId | smsIwmscFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.7	IOC SmsGmscFunction

Mapping from NRM IOC SmsGmscFunction attributes to SS equivalent MOC SmsGmscFunction attributes

| Attributes of IOC SmsGmscFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| smsGmscFunctionId | smsGmscFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.8	IOC SgsnFunction

Mapping from NRM IOC SgsnFunction attributes to SS equivalent MOC SgsnFunction attributes

| Attributes of IOC SgsnFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| sgsnFunctionId | sgsnFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read-Write, M |
| mccList | mccList | GenericNetworkResourcesIRPSystem::AttributeTypes:: LongSet | Read-Write, M |
| mncList | mncList | GenericNetworkResourcesIRPSystem::AttributeTypes:: LongSet | Read-Write, M |
| lacList | lacList | GenericNetworkResourcesIRPSystem::AttributeTypes:: LongSet | Read-Write, M |
| racList | racList | GenericNetworkResourcesIRPSystem::AttributeTypes:: LongSet | Read-Write, M |
| sacList | sacList | GenericNetworkResourcesIRPSystem::AttributeTypes:: LongSet | Read-Write, M |
| sgsnId | sgsnId | long | Read-Write, M |
| Associated With/sgsnFunction-GsmCell | sgsnFunction-GsmCell | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReferenceSet | Read-Only, M |
| Associated With/sgsnFunction-ExternalGsmCell | sgsnFunction-ExternalGsmCell | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReferenceSet | Read-Only, M |
| proceduralStatus | There is no corresponding SS attribute. |  |  |

### 5.2.9	IOC GgsnFunction

Mapping from NRM IOC GgsnFunction attributes to SS equivalent MOC GgsnFunction attributes

| Attributes of IOC GgsnFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| ggsnFunctionId | ggsnFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |
| proceduralStatus | There is no corresponding SS attribute. |  |  |

### 5.2.10	IOC BgFunction

Mapping from NRM IOC BgFunction attributes to SS equivalent MOC BgFunction attributes

| Attributes of IOC BgFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| bgFunctionId | bgFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.11	IOC GmscFunction

Mapping from NRM IOC GmscFunction attributes to SS equivalent MOC GmscFunction attributes

| Attributes of IOC GmscFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| gmscFunctionId | gmscFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.12	IOC SmlcFunction

Mapping from NRM IOC SmlcFunction attributes to SS equivalent MOC SmlcFunction attributes

| Attributes of IOC SmlcFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| smlcFunctionId | smlcFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.13	IOC GmlcFunction

Mapping from NRM IOC GmlcFunction attributes to SS equivalent MOC GmlcFunction attributes

| Attributes of IOC GmlcFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| gmlcFunctionId | gmlcFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.14	IOC ScfFunction

Mapping from NRM IOC ScfFunction attributes to SS equivalent MOC ScfFunction attributes

| Attributes of IOC ScfFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| scfFunctionId | scfFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.15	IOC SrfFunction

Mapping from NRM IOC SrfFunction attributes to SS equivalent MOC SrfFunction attributes

| Attributes of IOC SrfFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| srfFunctionId | srfFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.16	IOC CbcFunction

Mapping from NRM IOC CbcFunction attributes to SS equivalent MOC CbcFunction attributes

| Attributes of IOC CbcFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| cbcFunctionId | cbcFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.17	IOC CgfFunction

Mapping from NRM IOC CgfFunction attributes to SS equivalent MOC CgfFunction attributes

| Attributes of IOC CgfFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| cgfFunctionId | cgfFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.18	Void

### 5.2.19	IOC GmscServerFunction

Mapping from NRM IOC GmscServerFunction attributes to SS equivalent MOC GmscServerFunction attributes

| Attributes of IOC GmscServerFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| gmscServerFunctionId | gmscServerFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.20	IOC IwfFunction

Mapping from NRM IOC IwfFunction attributes to SS equivalent MOC IwfFunction attributes

| Attributes of IOC IwfFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| iwfFunctionId | iwfFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.21	IOC MnpSrfFunction

Mapping from NRM IOC MnpSrfFunction attributes to SS equivalent MOC IwfFunction attributes

| Attributes of IOC MnpSrfFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| mnpSrfFunctionId | mnpSrfFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.22	IOC NpdbFunction

Mapping from NRM IOC NpdbFunction attributes to SS equivalent MOC NpdbFunction attributes

| Attributes of IOC NpdbFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| npdbFunctionId | npdbFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.23	IOC SgwFunction

Mapping from NRM IOC SgwFunction attributes to SS equivalent MOC SgwFunction attributes

| Attributes of IOC SgwFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| sgwFunctionId | sgwFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.24	IOC SsfFunction

Mapping from NRM IOC SsfFunction attributes to SS equivalent MOC SsfFunction attributes

| Attributes of IOC SsfFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| ssfFunctionId | ssfFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.25	IOC BsFunction

Mapping from NRM IOC BsFunction attributes to SS equivalent MOC BsFunction attributes

| Attributes of IOC BsFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| bsFunctionId | bsFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |

### 5.2.26	IOC IucsLink

Mapping from NRM IOC IucsLink attributes to SS equivalent MOC IucsLink attributes

| Attributes of IOC IucsLink in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| iucsLinkId | iucsLinkId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |
| Connected To/connectedRnc | connectedRnc | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReference | Read-Only, M |
| Connected To/connectedBss | connectedBss | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReference | Read-Only, M |

### 5.2.27	IOC IupsLink

Mapping from NRM IOC IupsLink attributes to SS equivalent MOC IupsLink attributes

| Attributes of IOC IucsLink in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| iupsLinkId | iupsLinkId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |
| Connected To/connectedRnc | connectedRnc | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReference | Read-Only, O |
| Connected To/connectedBss | connectedBss | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReference | Read-Only, O |

### 5.2.28	IOC IubcLink

Mapping from NRM IOC IubcLink attributes to SS equivalent MOC IubcLink attributes

| Attributes of IOC IucsLink in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| iubcLinkId | iubcLinkId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |
| Connected To/connectedRnc | connectedRnc | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReference | Read-Only, M |

### 5.2.29	IOC ALink

Mapping from NRM IOC ALink attributes to SS equivalent MOC ALink attributes

| Attributes of IOC IucsLink in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| aLinkId | aLinkId | string | Read-Only, M |
| userLabel | userLabel | string | Read- Write, M |
| Connected To/connectedBss | connectedBss | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReference | Read-Only, M |

### 5.2.30	IOC GbLink

Mapping from NRM IOC GbLink attributes to SS equivalent MOC GbLink attributes

| Attributes of IOC IucsLink in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| gbLinkId | gbLinkId | string | Read-Only, M |
| userLabel | userLabel | string | Read-Write, M |
| Connected To/connectedBss | connectedBss | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReference | Read-Only, M |

### 5.2.31	IOC CsMgwFunction

Mapping from NRM IOC CsMgwFunction attributes to SS equivalent MOC CsMgwFunction attributes

| Attributes of IOC MgwFunction in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| csMgwFunctionId | csMgwFunctionId | string | Read-Only, M |
| userLabel | userLabel | string | Read-Write, M |
| Associated With/csMgwFunction-MscServerFunction | csMgwFunction-MscServerFunction | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReference | Read-Only, M |
| Connected To/csMgwFunction-IucsLink | csMgwFunction-IucsLink | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReferenceSet | Read-Only, M |
| Connected To /csMgwFunction-ALink | csMgwFunction-ALink | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReferenceSet | Read-Only, M |

### 5.2.32	Void

### 5.2.33	Void

### 5.2.34	Void

### 5.2.35	Void

### 5.2.36	Void

### 5.2.37	Void

### 5.2.38	Void

### 5.2.39	Void

### 5.2.40	Void

### 5.2.41	Void

### 5.2.42	Void

### 5.2.43	Void

### 5.2.44	Void

### 5.2.45	Void

### 5.2.46	Void

### 5.2.47	Void

### 5.2.48	Void

### 5.2.49	Void

### 5.2.50	Void

### 5.2.51	Void

### 5.2.52	Void

### 5.2.53	Void

### 5.2.54	Void

### 5.2.55	Void

### 5.2.56	Void

### 5.2.57	Void

### 5.2.58	Void

### 5.2.59	Void

### 5.2.60	Void

### 5.2.61	Void

### 5.2.62	Void

5.2.63	Void

### 5.2.64	Void

### 5.2.65	Void

Next Modified Section

Annex A (normative):
CORBA IDL, NRM Definitions

# A.1	IDL specification (file name "CoreNetworkResourcesNRMDefs.idl")

// File: CoreNetworkResourcesNRMDefs.idl

#ifndef _CORENETWORKRESOURCESNRMDEFS_IDL_

#define _CORENETWORKRESOURCESNRMDEFS_IDL_

#include "GenericNetworkResourcesNRMDefs.idl"

#pragma prefix "3gppsa5.org"

/**

* This module defines constants for each MO class name and

* the attribute names for each defined MO class.

*/

module CoreNetworkResourcesNRMDefs

{

/**

*  Definitions for MO class MscServerFunction

*/

interface MscServerFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "MscServerFunction";

// Attribute Names

//

const string mscServerFunctionId = "mscServerFunctionId";

const string mccList = "mccList";

const string mncList = "mncList";

const string lacList = "lacList";

const string sacList = "sacList";

const string gcaList = "gcaList";

const string mscId = "mscId";

const string mscServerFunctionGsmCell = "mscServerFunctionGsmCell";

const string mscServerFunctionExternalGsmCell = "mscServerFunctionExternalGsmCell";

const string mscServerFunctionCsMgwFunction = "mscServerFunctionCsMgwFunction";

};

/**

*  Definitions for MO class HlrFunction

*/

interface HlrFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "HlrFunction";

// Attribute Names

//

const string hlrFunctionId = "hlrFunctionId";

};

/**

*  Definitions for MO class VlrFunction

*/

interface VlrFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "VlrFunction";

// Attribute Names

//

const string vlrFunctionId = "vlrFunctionId";

};

/**

*  Definitions for MO class AucFunction

*/

interface AucFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "AucFunction";

// Attribute Names

//

const string aucFunctionId = "aucFunctionId";

};

/**

*  Definitions for MO class EirFunction

*/

interface EirFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "EirFunction";

// Attribute Names

//

const string eirFunctionId = "eirFunctionId";

};

/**

*  Definitions for MO class SmsIwmscFunction

*/

interface SmsIwmscFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "SmsIwmscFunction";

// Attribute Names

//

const string smsIwmscFunctionId = "smsIwmscFunctionId";

};

/**

*  Definitions for MO class SmsGmscFunction

*/

interface SmsGmscFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "SmsGmscFunction";

// Attribute Names

//

const string smsGmscFunctionId = "smsGmscFunctionId";

};

/**

*  Definitions for MO class SgsnFunction

*/

interface SgsnFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "SgsnFunction";

// Attribute Names

//

const string sgsnFunctionId = "sgsnFunctionId";

const string mccList = "mccList";

const string mncList = "mncList";

const string lacList = "lacList";

const string racList = "racList";

const string sacList = "sacList";

const string sgsnId = "sgsnId";

const string sgsnFunctionGsmCell = "sgsnFunctionGsmCell";

const string sgsnFunctionExternalGsmCell = "sgsnFunctionExternalGsmCell";

};

/**

*  Definitions for MO class GgsnFunction

*/

interface GgsnFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "GgsnFunction";

// Attribute Names

//

const string ggsnFunctionId = "ggsnFunctionId";

};

/**

*  Definitions for MO class BgFunction

*/

interface BgFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "BgFunction";

// Attribute Names

//

const string bgFunctionId = "bgFunctionId";

};

/**

*  Definitions for MO class GmscFunction

*/

interface GmscFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "GmscFunction";

// Attribute Names

//

const string gmscFunctionId = "gmscFunctionId";

};

/**

*  Definitions for MO class SmlcFunction

*/

interface SmlcFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "SmlcFunction";

// Attribute Names

//

const string smlcFunctionId = "smlcFunctionId";

};

/**

*  Definitions for MO class GmlcFunction

*/

interface GmlcFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "GmlcFunction";

// Attribute Names

//

const string gmlcFunctionId = "gmlcFunctionId";

};

/**

*  Definitions for MO class ScfFunction

*/

interface ScfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "ScfFunction";

// Attribute Names

//

const string scfFunctionId = "scfFunctionId";

};

/**

*  Definitions for MO class SrfFunction

*/

interface SrfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "SrfFunction";

// Attribute Names

//

const string srfFunctionId = "srfFunctionId";

};

/**

*  Definitions for MO class CbcFunction

*/

interface CbcFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "CbcFunction";

// Attribute Names

//

const string cbcFunctionId = "cbcFunctionId";

};

/**

*  Definitions for MO class CgfFunction

*/

interface CgfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "CgfFunction";

// Attribute Names

//

const string cgfFunctionId = "cgfFunctionId";

};

/**

*  Definitions for MO class GmscServerFunction

*/

interface GmscServerFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "GmscServerFunction";

// Attribute Names

//

const string gmscServerFunctionId = "gmscServerFunctionId";

};

/**

*  Definitions for MO class IwfFunction

*/

interface IwfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "IwfFunction";

// Attribute Names

//

const string iwfFunctionId = "iwfFunctionId";

};

/**

*  Definitions for MO class MnpSrfFunction

*/

interface MnpSrfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "MnpSrfFunction";

// Attribute Names

//

const string mnpSrfFunctionId = "mnpSrfFunctionId";

};

/**

*  Definitions for MO class NpdbFunction

*/

interface NpdbFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "NpdbFunction";

// Attribute Names

//

const string npdbFunctionId = "npdbFunctionId";

};

/**

*  Definitions for MO class SgwFunction

*/

interface SgwFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "SgwFunction";

// Attribute Names

//

const string sgwFunctionId = "sgwFunctionId";

};

/**

*  Definitions for MO class SsfFunction

*/

interface SsfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "SsfFunction";

// Attribute Names

//

const string ssfFunctionId = "ssfFunctionId";

};

/**

*  Definitions for MO class BsFunction

*/

interface BsFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "BsFunction";

// Attribute Names

//

const string bsFunctionId = "bsFunctionId";

};

/**

*  Definitions for MO class IucsLink

*/

interface IucsLink : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "IucsLink";

// Attribute Names

//

const string iucsLinkId = "iucsLinkId";

const string connectedRnc = "connectedRnc";

const string connectedBss = "connectedBss";

};

/**

*  Definitions for MO class IupsLink

*/

interface IupsLink : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "IupsLink";

// Attribute Names

//

const string iupsLinkId = "iupsLinkId";

const string connectedRnc = "connectedRnc";

const string connectedBss = "connectedBss";

};

/**

*  Definitions for MO class IubcLink

*/

interface IubcLink : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "IubcLink";

// Attribute Names

//

const string iubcLinkId = "iubcLinkId";

const string connectedRnc = "connectedRnc";

};

/**

*  Definitions for MO class ALink

*/

interface ALink : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "ALink";

// Attribute Names

//

const string aLinkId = "aLinkId";

const string connectedBss = "connectedBss";

};

/**

*  Definitions for MO class GbLink

*/

interface GbLink : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "GbLink";

// Attribute Names

//

const string gbLinkId = "gbLinkId";

const string connectedBss = "connectedBss";

};

/**

*  Definitions for MO class CsMgwFunction

*/

interface CsMgwFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "CsMgwFunction";

// Attribute Names

//

const string csMgwFunctionId = "csMgwFunctionId";

const string csMgwFunctionMscServerFunction = "csMgwFunctionMscServerFunction";

const string csMgwFunctionIucsLink = "csMgwFunctionIucsLink";

const string csMgwFunctionALink = "csMgwFunctionALink";

};

};

#endif // _CORENETWORKRESOURCESNRMDEFS_IDL_

End of modifications

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Jun 2001 | SA_12 | SP-010283 | -- | -- | Approved at TSG SA #12 and placed under Change Control | -- | 2.0.0 | 4.0.0 |
| Jun 2002 | SA_16 | SP-020302 | 0001 | -- | Align with Rel-4 Network Architecture (23.002) by changing Roaming Signalling Gateway (R-SGW) to Signalling Gateway (SGW) | F | 4.0.0 | 4.1.0 |
| Sep 2002 | SA_17 | SP-020489 | 0002 | -- | Upgrade to Rel-5 the CORBA SS for Core Network NRM (add Managed Object Classes (MOCs) | C | 4.1.0 | 5.0.0 |
| Dec 2002 | SA_18 | SP-020747 | 0003 | -- | Removal of faulty attribute uraList (alignment with Rel-5 32.632 Network Resource Model) | F | 5.0.0 | 5.1.0 |
| Sep 2004 | SA_25 | SP-040567 | 0004 | -- | Correction in Rules for NRM extensions - Align with 32.622 (Generic NRM IS) | F | 5.1.0 | 5.2.0 |
| Sep 2004 | SA_25 | SP-040582 | 0005 | -- | Correction of modelling of Media GateWay (MGW) | F | 5.1.0 | 5.2.0 |
| Sep 2004 | SA_25 | SP-040581 | 0006 | -- | Add Inheritance in CORBA IDL | B | 5.2.0 | 6.0.0 |
| Dec 2004 | SA_26 | SP-040809 | 0007 | -- | Add new IMS Entities | B | 6.0.0 | 6.1.0 |
| Dec 2004 | SA_26 | SP-040809 | 0008 | -- | Add restart notification to GSN objects using “proceduralStatus” attribute - Align with IS in 32.632 | B | 6.0.0 | 6.1.0 |
| Mar 2005 | SA_27 | SP-050047 | 0011 | -- | Align with 32.632, regarding the IS template and UML repertoire | F | 6.1.0 | 6.2.0 |
| Mar 2005 | SA_27 | SP-050047 | 0012 | -- | Correct List of Long Attributes | A | 6.1.0 | 6.2.0 |
| Mar 2005 | SA_27 | SP-050047 | 0013 | -- | Add IMS links to CN NRM CORBA SS | B | 6.1.0 | 6.2.0 |
| Mar 2006 | SA_31 | SP-060103 | 0014 | -- | Name correction of BgcfFunction - Align with 32.632 Information Service (IS) | F | 6.2.0 | 6.3.0 |
| Sep 2006 | SA_33 | SP-060611 | 0015 | -- | Add missing CscfFunction to CN NRM - Align with 32.632 | F | 6.3.0 | 6.4.0 |
| Dec 2006 | SA_34 | SP-060712 | 0016 | -- | Correct the Link subclasses to model Mg and Mj reference points - Align with 32.632 | F | 6.4.0 | 6.5.0 |
| Dec 2006 | SA_34 | SP-060731 | 0017 | -- | Move IMS part to new IMS NRM (32.733) | C | 6.5.0 | 7.0.0 |