3GPP TSG-SA5 (Telecom Management)	S5-070223

- Meeting SA5#51, 22 - 26 Jan 2007, Seville, SPAIN

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.633 | CR | 0018 |  rev | - |  Current version: | 6.5.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Inclusion of Link_Scscf_Scscf object in the SS - Align with the IS in 32.632 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Alcatel-Lucent (ajaysingh@alcatel-lucent.com) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM-NIM |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-6 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | The SS is not aligned with the IS. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Defining an IOC for Link_Scscf_Scscf Adding the link object in the IDL. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | The SS would not be aligned with the IS and it would not be possible for an IRPManager to reference a Link_Scscf_Scscf instance |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | New clause 5.2.66, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

### IOC Link_Scscf_Scscf

Mapping from NRM IOC Link_Scscf_Scscf attributes to SS equivalent MOC Link_Scscf_Scscf

| NRM Attributes of Link_Scscf_Scscf in TS 32.632 [3] | SS Attributes | SS Type | Qualifier |
| --- | --- | --- | --- |
| linkId inherited from Link | linkId | string | Read-Only, M |
| objectClass inherited from Link | CLASS | string | Read-Only, M |
| objectInstance inherited from Link | No direct mapping. | string | Read-Only, M |
| userLabel inherited from Link | userLabel | string | Read-Only, M |
| aEnd inherited from Link | aEnd | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReference | Read-Only, M |
| zEnd inherited from Link | zEnd | GenericNetworkResourcesIRPSystem::AttributeTypes::MOReference | Read-Only, M |
| linkType inherited from Link | linkType | LinkTypeType | Read-Only, O |
| protocolName inherited from Link | protocolName | string | Read-Only, O |
| protocolVersion inherited from Link | protocolVersion | string | Read-Only, O |

Next Modified Section

Annex A (normative):
CORBA IDL, NRM Definitions

# A.1	IDL specification (file name "CoreNetworkResourcesNRMDefs.idl")

// File: CoreNetworkResourcesNRMDefs.idl

#ifndef _CORENETWORKRESOURCESNRMDEFS_IDL_

#define _CORENETWORKRESOURCESNRMDEFS_IDL_

#include "GenericNetworkResourcesNRMDefs.idl"

#pragma prefix "3gppsa5.org"

/**

* This module defines constants for each MO class name and

* the attribute names for each defined MO class.

*/

module CoreNetworkResourcesNRMDefs

{

/**

*  Definitions for MO class MscServerFunction

*/

interface MscServerFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "MscServerFunction";

// Attribute Names

//

const string mscServerFunctionId = "mscServerFunctionId";

const string mccList = "mccList";

const string mncList = "mncList";

const string lacList = "lacList";

const string sacList = "sacList";

const string gcaList = "gcaList";

const string mscId = "mscId";

const string mscServerFunctionGsmCell = "mscServerFunctionGsmCell";

const string mscServerFunctionExternalGsmCell = "mscServerFunctionExternalGsmCell";

const string mscServerFunctionCsMgwFunction = "mscServerFunctionCsMgwFunction";

};

/**

*  Definitions for MO class HlrFunction

*/

interface HlrFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "HlrFunction";

// Attribute Names

//

const string hlrFunctionId = "hlrFunctionId";

};

/**

*  Definitions for MO class VlrFunction

*/

interface VlrFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "VlrFunction";

// Attribute Names

//

const string vlrFunctionId = "vlrFunctionId";

};

/**

*  Definitions for MO class AucFunction

*/

interface AucFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "AucFunction";

// Attribute Names

//

const string aucFunctionId = "aucFunctionId";

};

/**

*  Definitions for MO class EirFunction

*/

interface EirFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "EirFunction";

// Attribute Names

//

const string eirFunctionId = "eirFunctionId";

};

/**

*  Definitions for MO class SmsIwmscFunction

*/

interface SmsIwmscFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "SmsIwmscFunction";

// Attribute Names

//

const string smsIwmscFunctionId = "smsIwmscFunctionId";

};

/**

*  Definitions for MO class SmsGmscFunction

*/

interface SmsGmscFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "SmsGmscFunction";

// Attribute Names

//

const string smsGmscFunctionId = "smsGmscFunctionId";

};

/**

*  Definitions for MO class SgsnFunction

*/

interface SgsnFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "SgsnFunction";

// Attribute Names

//

const string sgsnFunctionId = "sgsnFunctionId";

const string mccList = "mccList";

const string mncList = "mncList";

const string lacList = "lacList";

const string racList = "racList";

const string sacList = "sacList";

const string sgsnId = "sgsnId";

const string sgsnFunctionGsmCell = "sgsnFunctionGsmCell";

const string sgsnFunctionExternalGsmCell = "sgsnFunctionExternalGsmCell";

};

/**

*  Definitions for MO class GgsnFunction

*/

interface GgsnFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "GgsnFunction";

// Attribute Names

//

const string ggsnFunctionId = "ggsnFunctionId";

};

/**

*  Definitions for MO class BgFunction

*/

interface BgFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "BgFunction";

// Attribute Names

//

const string bgFunctionId = "bgFunctionId";

};

/**

*  Definitions for MO class GmscFunction

*/

interface GmscFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "GmscFunction";

// Attribute Names

//

const string gmscFunctionId = "gmscFunctionId";

};

/**

*  Definitions for MO class SmlcFunction

*/

interface SmlcFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "SmlcFunction";

// Attribute Names

//

const string smlcFunctionId = "smlcFunctionId";

};

/**

*  Definitions for MO class GmlcFunction

*/

interface GmlcFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "GmlcFunction";

// Attribute Names

//

const string gmlcFunctionId = "gmlcFunctionId";

};

/**

*  Definitions for MO class ScfFunction

*/

interface ScfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "ScfFunction";

// Attribute Names

//

const string scfFunctionId = "scfFunctionId";

};

/**

*  Definitions for MO class SrfFunction

*/

interface SrfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "SrfFunction";

// Attribute Names

//

const string srfFunctionId = "srfFunctionId";

};

/**

*  Definitions for MO class CbcFunction

*/

interface CbcFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "CbcFunction";

// Attribute Names

//

const string cbcFunctionId = "cbcFunctionId";

};

/**

*  Definitions for MO class CgfFunction

*/

interface CgfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "CgfFunction";

// Attribute Names

//

const string cgfFunctionId = "cgfFunctionId";

};

/**

*  Definitions for MO class ImsMgwFunction

*/

interface ImsMgwFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "ImsMgwFunction";

// Attribute Names

//

const string imsMgwFunctionId = "imsMgwFunctionId";

};

/**

*  Definitions for MO class GmscServerFunction

*/

interface GmscServerFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "GmscServerFunction";

// Attribute Names

//

const string gmscServerFunctionId = "gmscServerFunctionId";

};

/**

*  Definitions for MO class IwfFunction

*/

interface IwfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "IwfFunction";

// Attribute Names

//

const string iwfFunctionId = "iwfFunctionId";

};

/**

*  Definitions for MO class MnpSrfFunction

*/

interface MnpSrfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "MnpSrfFunction";

// Attribute Names

//

const string mnpSrfFunctionId = "mnpSrfFunctionId";

};

/**

*  Definitions for MO class NpdbFunction

*/

interface NpdbFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "NpdbFunction";

// Attribute Names

//

const string npdbFunctionId = "npdbFunctionId";

};

/**

*  Definitions for MO class SgwFunction

*/

interface SgwFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "SgwFunction";

// Attribute Names

//

const string sgwFunctionId = "sgwFunctionId";

};

/**

*  Definitions for MO class SsfFunction

*/

interface SsfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "SsfFunction";

// Attribute Names

//

const string ssfFunctionId = "ssfFunctionId";

};

/**

*  Definitions for MO class ScscfFunction

*/

interface ScscfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "ScscfFunction";

// Attribute Names

//

const string scscfFunctionId = "scscfFunctionId";

};

/**

*  Definitions for MO class PcscfFunction

*/

interface PcscfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "PcscfFunction";

// Attribute Names

//

const string pcscfFunctionId = "pcscfFunctionId";

};

/**

*  Definitions for MO class IcscfFunction

*/

interface IcscfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "IcscfFunction";

// Attribute Names

//

const string icscfFunctionId = "icscfFunctionId";

};

/**

*  Definitions for MO class SlfFunction

*/

interface SlfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "SlfFunction";

// Attribute Names

//

const string slfFunctionId = "slfFunctionId";

};

/**

*  Definitions for MO class BgcfFunction

*/

interface BgcfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "BgcfFunction";

// Attribute Names

//

const string bgcfFunctionId = "bgcfFunctionId";

};

/**

*  Definitions for MO class MrfcFunction

*/

interface MrfcFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "MrfcFunction";

// Attribute Names

//

const string mrfcFunctionId = "mrfcFunctionId";

};

/**

*  Definitions for MO class MrfpFunction

*/

interface MrfpFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "MrfpFunction";

// Attribute Names

//

const string mrfpFunctionId = "mrfpFunctionId";

};

/**

*  Definitions for MO class AsFunction

*/

interface AsFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "AsFunction";

// Attribute Names

//

const string asFunctionId = "asFunctionId";

};

/**

*  Definitions for MO class MgcFunction

*/

interface MgcFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "MgcFunction";

// Attribute Names

//

const string mgcFunctionId = "mgcFunctionId";

};

/**

*  Definitions for MO class BsFunction

*/

interface BsFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "BsFunction";

// Attribute Names

//

const string bsFunctionId = "bsFunctionId";

};

/**

*  Definitions for MO class IucsLink

*/

interface IucsLink : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "IucsLink";

// Attribute Names

//

const string iucsLinkId = "iucsLinkId";

const string connectedRnc = "connectedRnc";

const string connectedBss = "connectedBss";

};

/**

*  Definitions for MO class IupsLink

*/

interface IupsLink : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "IupsLink";

// Attribute Names

//

const string iupsLinkId = "iupsLinkId";

const string connectedRnc = "connectedRnc";

const string connectedBss = "connectedBss";

};

/**

*  Definitions for MO class IubcLink

*/

interface IubcLink : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "IubcLink";

// Attribute Names

//

const string iubcLinkId = "iubcLinkId";

const string connectedRnc = "connectedRnc";

};

/**

*  Definitions for MO class ALink

*/

interface ALink : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "ALink";

// Attribute Names

//

const string aLinkId = "aLinkId";

const string connectedBss = "connectedBss";

};

/**

*  Definitions for MO class GbLink

*/

interface GbLink : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "GbLink";

// Attribute Names

//

const string gbLinkId = "gbLinkId";

const string connectedBss = "connectedBss";

};

/**

*  Definitions for MO class CsMgwFunction

*/

interface CsMgwFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "CsMgwFunction";

// Attribute Names

//

const string csMgwFunctionId = "csMgwFunctionId";

const string csMgwFunctionMscServerFunction = "csMgwFunctionMscServerFunction";

const string csMgwFunctionIucsLink = "csMgwFunctionIucsLink";

const string csMgwFunctionALink = "csMgwFunctionALink";

};

/**

*  Definitions for MO class Link_As_Auc

*/

interface Link_As_Auc : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_As_Auc";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_As_Hlr

*/

interface Link_As_Hlr : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_As_Hlr";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_As_Scscf

*/

interface Link_As_Scscf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_As_Scscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_As_Slf

*/

interface Link_As_Slf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_As_Slf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Bgcf_Bgcf

*/

interface Link_Bgcf_Bgcf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Bgcf_Bgcf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Bgcf_Mgcf

*/

interface Link_Bgcf_Mgcf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Bgcf_Mgcf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Bgcf_Scscf

*/

interface Link_Bgcf_Scscf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Bgcf_Scscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Hlr_Scscf

*/

interface Link_Hlr_Scscf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Hlr_Scscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Icscf_Slf

*/

interface Link_Icscf_Slf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Icscf_Slf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_ImsMgw_Mgcf

*/

interface Link_ImsMgw_Mgcf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_ImsMgw_Mgcf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Mgcf_Scscf

*/

interface Link_Mgcf_Scscf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Mgcf_Scscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Mrfc_Mrfp

*/

interface Link_Mrfc_Mrfp : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Mrfc_Mrfp";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Mrfc_Scscf

*/

interface Link_Mrfc_Scscf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Mrfc_Scscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Scscf_Slf

*/

interface Link_Scscf_Slf : GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Scscf_Slf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class CscfFunction

*/

interface CscfFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "CscfFunction";

// Attribute Names

//

const string cscfFunctionId = "cscfFunctionId";

};

/**

*  Definitions for MO class Link_Cscf_Cscf

*/

interface Link_Cscf_Cscf: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Cscf_Cscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_As_Cscf

*/

interface Link_As_Cscf: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_As_Cscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Cscf_Mgcf

*/

interface Link_Cscf_Mgcf: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Cscf_Mgcf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Cscf_Mrfc

*/

interface Link_Cscf_Mrfc: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Cscf_Mrfc";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Cscf_Slf

*/

interface Link_Cscf_Slf: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Cscf_Slf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Bgcf_Cscf

*/

interface Link_Bgcf_Cscf: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Bgcf_Cscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Cscf_Hlr

*/

interface Link_Cscf_Hlr: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Cscf_Hlr";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Cscf_Scscf

*/

interface Link_Cscf_Scscf: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Cscf_Scscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Cscf_Icscf

*/

interface Link_Cscf_Icscf: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Cscf_Icscf";

// All Attributes inherited from Link

};

/**

*  Definitions for MO class Link_Cscf_Pcscf

*/

interface Link_Cscf_Pcscf: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Cscf_Pcscf";

// All Attributes inherited from Link

};

/**

* Definitions for MO class Link_Scscf_Scscf

*/

interface Link_Scscf_Scscf: GenericNetworkResourcesNRMDefs::Link

{

const string CLASS = "Link_Scscf_Scscf";

// All Attributes inherited from Link

};

};

#endif // _CORENETWORKRESOURCESNRMDEFS_IDL_

End of modifications