3GPP TSG-SA5 (Telecom Management)	S5-070245

- Meeting SA5#51, 22 - 26 Jan 2007, Seville, SPAIN

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.405 | CR | 0025 |  rev | - |  Current version: | 7.3.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add measurements related to MAC-hs data rate |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | China Mobile (liangshuangchun@sjy.chinamobile.com, liyewen@chinamobile.com) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM7- PM |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | It is important for operator to learn the date rate of HSDPA in one utrancell for HSDPA enhanced data rates and spectrum efficiency. We suggest mean data rate of each utrancell is used to reflect HSDPA performance and it is given as follows: Mean data rate of each UtranCell for HSDPA = Number of octets of acknowledged MAC-hs PDU/ (Number of TTI with user data packets in MAC-hs buffer * TTI (TransmissionTimeInterval=2ms)). |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Add measurement about HSDPA. NbrAckdMacHsOcts, HSDPA. NbrTTINonEmptyBuffer |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | New clauses 4.x, 4.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| Affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

## 4.x	Number of octets of acknowledged MAC-hs PDUs

- This measurement provides the number of octets of downlink acknowledged MAC-hs PDUs in the serving HS-DSCH cell.

- CC.

- On transmission by the NodeB of an acknowledged PDU on the MAC-hs layer. The number of octets of MAC-hs layer in one serving HS-DSCH cell is calculated by sum of size of each MAC-hs PDU header including MAC-hs PDU header.

- A single integer value.

- HSDPA.NbrAckdMacHsOcts

- UtranCell.

- Valid for packet switched traffic.

- UMTS.

## 4.y	Number of TTIs with user data packets in buffer

- This measurement provides the number of TTIs with user data packets in MAC-hs buffer.

- CC.

- On receipt by the NodeB of a HSDPA frame with user data packets in MAC-hs buffer.

- A single integer value.

- HSDPA.NbrTTINonEmptyBuffer

- UtranCell.

- Valid for packet switched traffic.

- UMTS.

End of modifications

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Mar 2006 | SA_31 | SP-060108 | -- | -- | Split of 32.403-710 into four new TSs 32.404, 32.405, 32.406, 32.408. Submitted to TSG SA #31 for Approval. | -- | 1.0.0 | 7.0.0 |
| Jun 2006 | SA_32 | SP-060260 | 0001 | -- | Add TDD Counters to Measure Mean Code Resources Used | B | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0002 | -- | Add ATM layer performance measurements | B | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0003 | -- | Add Power Counters for radio channel monitoring | B | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0004 | -- | Add measurements about Radio Access Bearer (RAB) related to Iu release request | B | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0005 | -- | Add paging counters for UTRAN | B | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0006 | -- | Correction for wrong reference to TS 25.331 | F | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0007 | -- | Correction for TDD Counters to Measure Max Code Resources Used | F | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0008 | -- | Add measurements about SHO radio link performance | B | 7.0.0 | 7.1.0 |
| Sep 2006 | SA_33 | SP-060663 | 0009 | 1 | Add RRC Connection Usage Measurements to fully gauge the usage of the UTRAN | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060663 | 0010 | 1 | Correction to TDD measurements for Received Total Wideband Power and Transmitted Carrier Power | F | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0011 | -- | Add measurements about RAB with different data rates of conversational service - Align with RAN2's 25.993 | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0012 | -- | Add measurements about the non-HS carrier power - Align with RAN3's 25.433 | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0013 | -- | Add Uplink ISCP measurements in PM for TDD | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0014 | -- | Add measurements about the non-HS carrier power for TDD | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0015 | -- | Constrain the applicability of performance measurements related to Iur interface | C | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0016 | -- | Add measurements about call duration in utran cell for CS domain | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0017 | -- | Add measurements related to channel switches for HSDPA | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0018 | -- | Add measurements about the HSDPA mobility management | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0019 | -- | Add HSDPA setup related measurements | B | 7.1.0 | 7.2.0 |
| Dec 2006 | SA_34 | SP-060726 | 0020 | -- | Correction of the HHO measurements | C | 7.2.0 | 7.3.0 |
| Dec 2006 | SA_34 | SP-060726 | 0021 | -- | Add Measurements related to MAC-hs | B | 7.2.0 | 7.3.0 |
| Dec 2006 | SA_34 | SP-060726 | 0022 | -- | Correct the measurements related to RAB release request by CN - Align with 25.413 | F | 7.2.0 | 7.3.0 |