3GPP TSG-SA5 (Telecom Management) 	Tdoc S5-070249

Meeting SA5#51, 22 - 26 Jan 2007, Seville, ES

| CR-Form-v9.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0066 |  rev | - |  Current version: | 7.1.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Correction on SDP Media Info - Align with 24.229 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Siemens Networks, Nokia (gerald.goermer@siemens.com, benni.alexander@nokia.com) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | A |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | According to TS 24.229 the S-CSCF shall remove the access-network-charging-info parameter from the P-Charging-Vector header prior to forwarding the message to MGCF and BGCF. Therefore the Access Correlation ID as the part of the access-network-charging-info is not available at MGCF and BGCF. The Acces Network address is already removed but the Access Correlation ID as the part of the SDP Media components is present in the description. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | The Access Correlation ID is not used in the SDP media component parameter for MGCF and BGCF ASN.1 definition. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Error in IMS Billing based on MGCF and BGCF CDRs. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  | x |  | Other core specifications  |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  | 32.260 |  |  |  |
| Other comments:  |  | Rel-7 Mirror CR of S5-070248, Corresponding Rel-7 CR 32.260 in S5-070247 |  |  |  |  |  |  |  |  |

1st Modified Section

### 5.2.3	Subsystem level CDR definitions

This subclause contains the syntax definitions of the CDRs on the subsystem level. At present, only the IM subsystem is defined in 3GPP, thus this subclause comprises the CDR types specified for the IMS in TS 32.260 [20].

#### 5.2.3.1	IMS CDRs

…

SDP-Media-Component ::=  SEQUENCE

{

sDP-Media-Name        	[0] GraphicString OPTIONAL,

sDP-Media-Descriptions	[1] SDP-Media-Description OPTIONAL,

accessCorrelationID			AccessCorrelationID OPTIONAL, 		-- not used in MGCF and BGCF

-- [2] is used by gPRS-Charging-Id

-- [4] is used by accessNetworkChargingIdentifier

authorized-QoS        	[3] GraphicString OPTIONAL

}

…

End of modifications

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev |  | Subject/Comment | Cat | Old | New |
| Dec 2006 | SA_34 | SP-060719 | 0046 |  | -- | Corrections of the ICID description - Align with 32.260 | F | 7.0.0 | 7.1.0 |
| Dec 2006 | SA_34 | SP-060719 | 0047 |  | -- | Add an identifier for the service initiating offline Diameter Accounting | B | 7.0.0 | 7.1.0 |
| Dec 2006 | SA_34 | SP-060719 | 0048 |  | -- | Add an identifier for the served user in offline Diameter Accounting | B | 7.0.0 | 7.1.0 |
| Dec 2006 | SA_34 | SP-060908 | 0049 |  | -- | Correction on MMS CDR record type definition | A | 7.0.0 | 7.1.0 |
| Dec 2006 | SA_34 | SP-060908 | 0050 |  | -- | Correction on MBMS Information ASN.1 syntax - Align with 23.246 | A | 7.0.0 | 7.1.0 |
| Dec 2006 | SA_34 | SP-060908 | 0051 |  | -- | Correction on Private User Id - Align with 32.260 | A | 7.0.0 | 7.1.0 |
| Dec 2006 | SA_34 | SP-060719 | 0052 |  | -- | Correction on Service Specific Data | F | 7.0.0 | 7.1.0 |
| Dec 2006 | SA_34 | SP-060714 | 0053 |  | -- | Introduce support of event based charging for PCC | B | 7.0.0 | 7.1.0 |
| Dec 2006 | SA_34 | SP-060908 | 0054 |  | 1 | Correction on IMS CDRs - Align with 32.260 | A | 7.0.0 | 7.1.0 |
| Dec 2006 | SA_34 | SP-060713 | 0055 |  | -- | Add Access Network Charging Identifier | C | 7.0.0 | 7.1.0 |