3GPP TSG-SA5 (Telecom Management)	 S5-081086

Meeting SA5#60, 7 - 11 July 2008, Sophia-Antipolis, FRANCE

| CR-Form-v9.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.299 | CR | 0213 |  rev | - |  Current version: | 8.3.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add CER/CEA and DWR/DWA to Offline Charging messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | 7.03 Charging Maintenance and Rel-8 small Enhancements |  |  |  |  |  |  | Date:  |  | 2008-06-27 |
| Category:  | D |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) |  |
| Reason for change:  |  | Chapter 6.2.1.1 of TS 32.299 contains a table which describes Diameter messages for offline charging. Chapter 6.4.1.1. contains an equivalent table for online charging. Only the table for online charging contains Diameter messages for connection control (Capabilities-Exchange-Request/Answer, Device-Watchdog-Request/Answer, Disconnect-Peer-Request/Answer) besides the functional requests for online charging, but the table for offline charging does not. As stated underneath the table in chapter 6.4.1.1 CER/CEA and DWR/DWA are mandatory Diameter capabilities for capabilities exchange and transport failure detection. So they need to be supported as well by offline charging, and not only by Online Charging. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | CER/CEA and DWR/DWA are added in 6.2.1.1. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | The current formulations within TS 32299 -830 may be misleading. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 6.2.1.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

### 6.2.1	Summary of Offline Charging Message Formats

#### 6.2.1.1	General

The corresponding Diameter accounting application messages for the Charging Data Transfer operation is Accounting Request (ACR) and Accounting Answer (ACA) as specified in the Diameter Base Protocol Accounting (DBPA) application [401].

The following table describes the use of these messages for offline charging.

Table 6.2.1.1: Offline Charging Messages Reference Table

| Command-Name | Source | Destination | Abbreviation |
| --- | --- | --- | --- |
| Accounting-Request | CTF | CDF | ACR |
| Accounting-Answer | CDF | CTF | ACA |
| Capabilities-Exchange-Request | CTF | CDF | CER |
| Capabilities Exchange Answer | CDF | CTF | CEA |
| Device-Watchdog-Request | CTF | CDF | DWR |
| Device-Watchdog-Answer | CDF | CTF | DWA |

CER/CEA and DWR/DWA are mandatory Diameter capabilities for capabilities exchange and transport failure detection.

End of modifications