3GPP TSG-SA5 (Telecom Management)                       S5-082411

Meeting SA5#62, November 17-21, 2008, Miami, USA

- Source:	Ericsson

- Title:	E-UTRAN Modelling

- Document for:	Approval

Agenda Item:	6.7

1. Introduction

This paper proposes additional text, IOCs, attributes (compared to that specified in draft E-UTRAN NRM IRP IS, TS 32.762 V0.2.0) attributes related to E-UTRAN NRM.

Reason for change: draft TS 32.762 is missing information

Summary of change(s): See section 2 below.

2. pCR to TS 32.762 V0.2.0

1st Modified Section

## 6.1	Information entities imported and local labels

| Label reference | Local label |
| --- | --- |
| 32.672 section 5.3.1.1 StateManagementEntity.administrativeState | administrativeState |
| 32.672 section 5.3.1.1 StateManagementEntity.availabilityStatus | availabilityStatus |
| 32.672 section 5.3.1.1 StateManagementEntity.operationalState | operationalState |

2nd Modified Section

### 6.2.2	Inheritance

Figure X: E-UTRAN NRM Inheritance Hierarchy

3Rd Modified Section

### 6.3.1	ENBFunction

#### 6.3.1.1	Definition

This IOC represents eNB functionality. For more information about the eNB, see 3GPP TS ?? [??].

#### 6.3.1.2	Attributes

Table 6.X.X.X: Attributes of ENBFunction

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| x2BlackList | M | M | M |
| x2WhiteList | M | M | M |

#### 6.3.1.3	Attribute Constraints

#### 6.3.1.4	Notification

Notifications of ENBFunction

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyAttributeValueChange | O |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |
| notifyComments | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyAlarmListRebuilt | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyPotentialFaultyAlarmList | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |

4th Modified Section

### 6.3.2 	EUtranGenericCell

- 6.3.2.1	Definition

This IOC represents the common properties of E-UTRAN generic cell. For more information about cells, see 3GPP TS 23.401 [9].

- 6.3.2.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| userLabel | M | M | M |
| physicalCellIdentity | M | M | M |
| cellIdentity | M | M | M |
| cellType | M | M | - |
| numberOfTransmitAntennas | M | M | M |
| numberOfReceiveAntennas | M | M | M |
| plmnIdList | M | M | M |
| tac | M | M | M |

| Attribute Name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| operationalState | O | M |  |
| administrativeState | O | M | M |
| availabilityStatus | O | M | - |
| NOTE: No state or status propagation shall be implied. |  |  |  |

Editor’s notes:

1.For different cellType, it may need to model accordingly if there is different attributes found.

2.Details to be discussed for physicalCellIdentity.

3.Yellow highlight means FFS.

- 6.3.2.3	Attribute Constraints

- 6.3.2.4	Notification

Notifications of EUtranGenericCell

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyAttributeValueChange | O |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |
| notifyStateChange | O |  |
| notifyComments | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyAlarmListRebuilt | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyPotentialFaultyAlarmList | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |

5th Modified Section

### 6.3.3	ExternalGenericEUtranCell

#### 6.3.3.1	Definition

This abstract IOC represents the properties of a radio cell controlled by another IRPAgent. This IOC contains necessary attributes for inter-system and intra-system handover. It also contains a subset of the attributes of related IOCs controlled by another IRPAgent. The way to maintain consistency between the attribute values of these IOCs is outside the scope of the present document.

#### 6.3.3.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| userLabel | M | M | M |
| physicalCellIdentity | M | M | M |
| cellIdentity | M | M | M |

Editor’s note: Yellowed text is not considered agreed.

#### 6.3.3.3	Attribute Constraints

#### 6.3.3.4	Notification

Notifications of ExternalUtranGenericCell

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAttributeValueChange | O |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |

### 6.3.4	EUtranCellFDD

#### 6.3.4.1	Definition

This IOC represents the properties of EUTRAN cell FDD.

#### 6.3.4.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| earfcnDl | M | M | M |
| earfcnUl | M | M | M |

### 6.3.5	ExternalEUtranCellFDD

#### 6.3.5.1	Definition

This IOC represents the common properties of external EUTRAN cell FDD

#### 6.3.5.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| earfcnDl | M | M | M |
| earfcnUl | M | M | M |

- .4	Notifications

6th Modified Section

### 6.5.1	Definition and legal values

Table 6.5.1.1 defines the attributes that are present in several Information Object Classes (IOCs) of the present document.

Table 6.5.1.1: XXX

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| administrativeState | See Import Table. |  |
| availabilityStatus | See Import Table. |  |
| cellIdentity | Unambiguously identify a cell within a PLMN. | Refer to TS36.331[9] |
| cellType | Cell type for management | Enumerated {femto, pico, macro}. |
| earfcnul | Specifies the channel number for the central UL frequency. The mapping from channel number to physical frequency is described in 3GPP specification TS 36.101 [??] subclause 5.4.3. | See EARFCN in TS 36.101 subclause 5.4.4. |
| earfcndl | Specifies the channel number for the central DL frequency. The mapping from channel number to physical frequency is described in 3GPP specification TS 36.101 [??] subclause 5.4.3. | See EARFCN in TS 36.101 subclause 5.4.4. |
| isRemoveAllowed (see note 1) | This indicates if the subject EUtranRelation can be removed (deleted) or not. If ‘yes’, the subject EUtranRelation instance can be removed (deleted). If ‘no’, the subject EUtranRelation instance shall not be removed (deleted) by any entity but an IRPManager. | yes, no |
| isHOAllowed (see note 1) | This indicates if HO is allowed or prohibited. If ‘yes’, handover is allowed from source cell to target cell. The source cell is identified by the name-containing EUtranGenericCell of the EUtranRelation that has the isHOAllowed. The target cell is referenced by the EUtranRelation that has this isHOAllowed. If ‘no’, handover shall not be allowed. | yes, no |
| physicalCellIdentity | Physical identity of the cell | One of the 504 unique cell identities as described in TS36.300[10] |
| operationalState | See Import Table. |  |
| plmnIdList | List of unique identities for PLMN. Note: A cell can broadcast up to 6 PLMN-id's. This is to support the case that one cell can be used by up to 6 operators’ core networks. See TS 36.331 [9] section 6.2.2: SystemInformationBlockType1/cellAccessRelatedInformation/plmn-IdentityList is a SEQUENCE (SIZE (1..6)) |  |
| tac | Common Tracking Area Code for the PLMNs. The identity used to identify tracking areas. | - It is the Tracking Area Code (TAC). - A cell can only broadcast one TAC. See TS 36.300 [10], section 10.1.7 (PLMNID and TAC relation). |

End of Modified Sections