3GPP TSG-SA5 (Telecom Management)	S5-090567

Meeting SA5#62 bis, 14-16 January 2009, Sophia Antipolis, France

- Source:	HUAWEI

- Title:	Add 3G Home NodeB OAM&P Specification Level Requirements

- Document for:	Discussion and Approval

Agenda Item:	6.1

# 1	Decision/action requested

The group is asked to discuss the proposed specification level requirements for 3G Home NodeB OAM&P (for type 1 interface), and introduce the agreed part into the draft TS 32.XX1.

# 2	References

[1]	3GPP TR 32.821: "Study of Self-Organising Networks (SON) related OAM for Home NodeB (Release 8)"

[2] 3GPP S5-082488: “New WT-level WID on 3G Home NodeB OAM&P (Interface Type 1 Management”)

# 3	Rationale

In 3GPP SA5#62, the meeting discussed [2] and agreed on new work item on 3G Home NodeB OAM&P(Interface Type 1 Management). This contribution is to add specification level requirements to the TS32.xx1, TR32.821[1] is referenced as a part of input.

# 4	Detailed proposal

# Text Proposal for TS 32.XX1

1st Modified Section

- 6.5	Requirements

- 6.5.1	Configuration Management

REQ-HNBOAMP-CM-FUN-001 HMS shall be able to reboot the Home NodeB.

REQ-HNBOAMP-CM-FUN-002 HMS shall have remote access to the Home NodeB to start/stop the radio transmission.

REQ-HNBOAMP-CM-FUN-002bis HMS shall have remote access to the Home NodeB to start/stop the radio transmission on the frequencies specified by HMS.

REQ-HNBOAMP-CM-FUN-003a HMS shall maintain the  configuration data of the Home NodeB.

REQ-HNBOAMP-CM-FUN-003b When the Home NodeB is initially powered up and connected to the HMS, HMS shall send the initially needed configuration data to the Home NodeB.

- 6.5.2	Performance Management

REQ-HNBOAMP-PM-FUN-002

HMS shall have the ability to configure policies for Home NodeB performance data file upload.

- 6.5.3	Fault Management

REQ-HNBOAMP-FM-FUN-001 Home NodeB shall have the ability to send alarm related information to HMS according to operator configured policy.

REQ-HNBOAMP-FM-FUN-003 Home NodeB shall be able to upload files containing alarm related information to HMS as requested by HMS.

REQ-HNBOAMP-FM-FUN-004

The HNB shall upload files containing alarm related information using one of the following mechanisms:

a.HTTPS PUT and GET with/without Username/Password

b.SFTP with/without Username/Password

c.HTTP PUT with/without Username/Password

d.FTP with/without Username/Password

e. FTPS with/without Username/Password

End of 1st modifications