3GPP TS 32.405 V9.1.0 (2009-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Performance Management (PM);

Performance measurements

Universal Terrestrial Radio Access Network (UTRAN)

(Release 9)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

GSM, UMTS, management, performance

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2009, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

1	Scope	11

2	References	11

3	Measurement family and abbreviations	12

3.1	Measurement family	12

3.2	Abbreviations	12

4	Measurements related to the RNC	14

4.1	RAB management	14

4.1.1	Overview	14

4.1.1.1	Measurements are based on the success and failure of procedures	14

4.1.1.2	Combination of Traffic Class and Core Network domains	14

4.1.1.3	Considered Radio Access Bearer management procedures	14

4.1.1.4	Measurements relate to RAB establishment, modification and release	15

4.1.2	RAB establishment for CS domain	16

4.1.2.1	Attempted RAB establishments for CS domain	16

4.1.2.2	Successful RAB establishments without queuing for CS domain	16

4.1.2.3	Failed RAB establishments without queuing for CS domain	17

4.1.2.4	Successful RAB establishments with queuing for CS domain	17

4.1.2.5	Failed RAB establishments with queuing for CS domain	18

4.1.3	RAB establishment for PS domain	19

4.1.3.1	Attempted RAB establishments for PS domain	19

4.1.3.2	Successful RAB establishments without queuing for PS domain	19

4.1.3.3	Failed RAB establishments without queuing for PS domain	20

4.1.3.4	Successful RAB establishments with queuing for PS domain	20

4.1.3.5	Failed RAB establishments with queuing for PS domain	20

4.1.4	RAB modification for CS domain	22

4.1.4.1	Attempted RAB modifications for CS domain	22

4.1.4.2	Successful RAB modifications without queuing for CS domain	22

4.1.4.3	Failed RAB modifications without queuing for CS domain	22

4.1.4.4	Successful RAB modifications with queuing for CS domain	23

4.1.4.5	Failed RAB modifications with queuing for CS domain	23

4.1.5	RAB modification for PS domain	25

4.1.5.1	Attempted RAB modifications for PS domain	25

4.1.5.2	Successful RAB modifications without queuing for PS domain	25

4.1.5.3	Failed RAB modifications without queuing for PS domain	25

4.1.5.4	Successful RAB modifications with queuing for PS domain	26

4.1.5.5	Failed RAB modifications with queuing for PS domain	26

4.1.6	RAB release request by CN for CS domain	28

4.1.6.1	Attempted RAB releases for CS domain	28

4.1.6.2	Successful RAB releases for CS domain	28

4.1.6.3	Failed RAB releases for CS domain	28

4.1.7	RAB release request by CN for PS domain	30

4.1.7.1	Attempted RAB releases for PS domain	30

4.1.7.2	Successful RAB releases for PS domain	30

4.1.7.3	Failed RAB releases for PS domain	30

4.1.7.4	Void	31

4.1.7.5	Void	31

4.1.8	RAB setup time	32

4.1.8.1	RAB CS connection set-up time (Mean)	32

4.1.8.2	RAB CS connection set-up time (Maximum)	32

4.1.8.3	RAB PS connection set-up time (Mean)	32

4.1.8.4	RAB PS connection set-up time (Maximum)	33

4.1.9	RAB release request by UTRAN	34

4.1.9.1	RAB release requests for CS domain	34

4.1.9.2	RAB release requests for PS domain	34

4.1.9.3	The number of RAB related to the Iu release request for CS domain	34

4.1.9.4	The number of RAB related to the Iu release request for PS domain	35

4.2	Void	35

4.3	Signalling connection establishment	36

4.3.1	Attempted signalling connection establishments for CS domain	36

4.3.2	Attempted signalling connection establishments for PS domain	36

4.4	RRC connection establishment	37

4.4.1	RRC connection establishments	37

4.4.1.1	Attempted RRC connection establishments	37

4.4.1.2	Failed RRC connection establishments	37

4.4.1.3	Successful RRC connection establishments	37

4.4.2	RRC connection establishment setup time	39

4.4.2.1	RRC connection set-up time (Mean)	39

4.4.2.2	RRC connection set-up time (Max)	39

4.4.3	RRC connection usage	41

4.4.3.1	 General	41

4.4.3.2	RRC connection usage (Mean)	41

4.4.3.3	RRC connection usage (Maximum)	41

4.5	RRC connection re-establishment	42

4.5.1	Attempted RRC re-establishments	42

4.5.2	Failed RRC re-establishments	42

4.5.3	Successful RRC re-establishments	43

4.6	RRC connection release	44

4.6.1	Attempted RRC connection releases on DCCH	44

4.6.2	Attempted RRC connection releases on CCCH	44

4.7	RLC connection	45

4.7.1	Number of RLC blocks sent (per Mode)	45

4.7.2	Number of RLC blocks Received (per Mode)	45

4.7.3	Discarded RLC blocks by RNC	45

4.7.4	Number of Retransmitted RLC blocks in Acknowledge Mode	46

4.8	Soft handover	47

4.8.1	Radio link additions to active link set (UE side)	47

4.8.1.1	Attempted radio link additions to active link set (UE side)	47

4.8.1.2	Successful radio link additions to active link set (UE side)	47

4.8.1.3	Failed radio link additions to active link set (UE side)	47

4.8.2	Radio link deletions from active link set (UE side)	49

4.8.2.1	Attempted radio link deletions from active link set (UE side)	49

4.8.2.2	Successful radio link deletions from active link set (UE side)	49

4.8.3	Measurements related to the soft handover radio link	50

4.8.3.1	Mean number of the radio link established by the best UTRAN cell	50

4.8.3.2	Mean number of the radio link established by other UTRAN cell	50

4.9	Radio link management procedures	51

4.9.1	Overview	51

4.9.1.1	Considered radio link management procedures	51

4.9.1.2	Relation between Iub measurements and Iur measurements	54

4.9.2	Radio link setups on Iub	55

4.9.2.1	Attempted radio link setups on Iub	55

4.9.2.2	Successful radio link setups on Iub	55

4.9.2.3	Failed radio link setups on Iub	55

4.9.3	Radio link setups on Iur	57

4.9.3.1	Attempted radio link setups on Iur	57

4.9.3.2	Successful radio link setups on Iur	57

4.9.3.3	Failed radio link setups on Iur	57

4.9.4	Radio link additions on Iub	59

4.9.4.1	Attempted radio link additions on Iub	59

4.9.4.2	Successful radio link additions on Iub	59

4.9.4.3	Failed radio link additions on Iub	59

4.9.5	Radio link additions on Iur	61

4.9.5.1	Attempted radio link additions on Iur	61

4.9.5.2	Successful radio link additions on Iur	61

4.9.5.3	Failed radio link additions on Iur	61

4.9.6	Radio link deletions on Iub	63

4.9.6.1	Attempted radio link deletions on Iub	63

4.9.6.2	Successful radio link deletions on Iub	63

4.9.7	Radio link deletions on Iur	64

4.9.7.1	Attempted radio link deletions on Iur	64

4.9.7.2	Successful radio link deletions on Iur	64

4.10	Hard handover	65

4.10.1	Void	65

4.10.2	Outgoing intra-NodeB hard handovers	65

4.10.2.1	Attempted outgoing intra-NodeB hard handovers	65

4.10.2.2	Successful outgoing intra-NodeB hard handovers	65

4.10.2.3	Failed outgoing intra-NodeB hard handovers	65

4.10.3	Outgoing inter-NodeB, intra-RNC hard handovers	67

4.10.3.1	Attempted outgoing inter-NodeB, intra-RNC hard handovers	67

4.10.3.2	Successful outgoing inter-NodeB, intra-RNC hard handovers	67

4.10.3.3	Failed outgoing inter-NodeB, intra-RNC hard handovers	67

4.10.4	Outgoing inter-RNC hard handovers via Iur	69

4.10.4.1	Attempted outgoing inter-RNC hard handovers via Iur	69

4.10.4.2	Successful outgoing inter-RNC hard handovers via Iur	69

4.10.4.3	Failed outgoing inter-RNC hard handovers via Iur	69

4.10.5	Relocation preparation for outgoing inter-RNC hard handovers switching in the CN	71

4.10.5.1	Attempted relocation preparation for outgoing inter-RNC hard handovers switching in the CN	71

4.10.5.2	Successful relocation preparation for outgoing inter-RNC hard handovers switching in the CN	71

4.10.5.3	Failed relocation preparation for outgoing inter-RNC hard handovers switching in the CN	71

4.10.6	Outgoing inter-RNC hard handovers switching in the CN	73

4.10.6.1	Attempted outgoing inter-RNC hard handovers switching in the CN	73

4.10.6.2	Successful outgoing inter-RNC hard handovers switching in the CN	73

4.10.6.3	Failed outgoing inter-RNC hard handovers switching in the CN	73

4.10.7	Outgoing hard handovers per neighbour cell relation	75

4.10.7.1	Attempted outgoing hard handovers per neighbour cell relation	75

4.10.7.2	Successful outgoing hard handovers per neighbour cell relation	75

4.10.7.3	Failed outgoing hard handovers per neighbour cell relation	75

4.11	Relocation	77

4.11.1	Relocations for CS domain	77

4.11.1.1	Relocation preparations with UE involved for CS domain	77

4.11.1.1.1	Attempted relocation preparations with UE involved for CS domain	77

4.11.1.1.2	Successful relocation preparations with UE involved for CS domain	77

4.11.1.1.3	Failed relocation preparations with UE involved for CS domain	77

4.11.1.2	Relocation preparations with UE not involved for CS domain	78

4.11.1.2.1	Attempted relocation preparations with UE not involved for CS domain	78

4.11.1.2.2	Successful relocation preparations with UE not involved for CS domain	78

4.11.1.2.3	Failed relocation preparations with UE not involved for CS domain	79

4.11.1.3	Relocation resource allocations with UE involved for CS domain	79

4.11.1.3.1	Attempted relocations resource allocations with UE involved for CS domain	79

4.11.1.3.2	Successful relocation resource allocations with UE involved for CS domain	79

4.11.1.3.3	Failed relocation resource allocations with UE involved for CS domain	80

4.11.1.4	Relocation resource allocations with UE not involved for CS domain	80

4.11.1.4.1	Attempted relocations resource allocations with UE not involved for CS domain	80

4.11.1.4.2	Successful relocation resource allocations with UE not involved for CS domain	81

4.11.1.4.3	Failed relocation resource allocations with UE not involved for CS domain	81

4.11.1.5	Relocations for CS domain	81

4.11.1.5.1	Successful relocations for CS domain	81

4.11.2	Relocations for PS domain	83

4.11.2.1	Relocation preparations with UE involved for PS domain	83

4.11.2.1.1	Attempted relocation preparations with UE involved for PS domain	83

4.11.2.1.2	Successful relocation preparations with UE involved for PS domain	83

4.11.2.1.3	Failed relocation preparations with UE involved for PS domain	83

4.11.2.2	Relocation preparations with UE not involved for PS domain	84

4.11.2.2.1	Attempted relocation preparations with UE not involved for PS domain	84

4.11.2.2.2	Successful relocation preparations with UE not involved for PS domain	84

4.11.2.2.3	Failed relocation preparations with UE not involved for PS domain	84

4.11.2.3	Relocation resource allocations with UE involved for PS domain	85

4.11.2.3.1	Attempted relocations resource allocations with UE involved for PS domain	85

4.11.2.3.2	Successful relocation resource allocations with UE involved for PS domain	85

4.11.2.3.3	Failed relocation resource allocations with UE involved for PS domain	86

4.11.2.4	Relocation resource allocations with UE not involved for PS domain	86

4.11.2.4.1	Attempted relocations resource allocations with UE not involved for PS domain	86

4.11.2.4.2	Successful relocation resource allocations with UE not involved for PS domain	86

4.11.2.4.3	Failed relocation resource allocations with UE not involved for PS domain	87

4.11.2.5	Relocations for PS domain	87

4.11.2.5.1	Successful relocations for PS domain	87

4.12	Circuit switched inter-RAT handover	88

4.12.1	Relocation preparation for outgoing circuit switched inter-RAT handovers	88

4.12.1.1	Attempted relocation preparation for outgoing circuit switched inter-RAT handovers	88

4.12.1.2	Successful relocation preparation for outgoing circuit switched inter-RAT handovers	88

4.12.1.3	Failed relocation preparation for outgoing circuit switched inter-RAT handovers	88

4.12.2	Outgoing circuit switched inter-RAT handovers	90

4.12.2.1	Attempted outgoing circuit switched inter-RAT handovers	90

4.12.2.2	Successful outgoing circuit switched inter-RAT handovers	90

4.12.2.3	Failed outgoing circuit switched inter-RAT handovers	90

4.12.3	Incoming circuit switched inter-RAT handovers	91

4.12.3.1	Attempted incoming circuit switched inter-RAT handovers	91

4.12.3.2	Successful incoming circuit switched inter-RAT handovers	91

4.12.3.3	Failed incoming circuit switched inter-RAT handovers	91

4.13	Packet switched inter-RAT handover	92

4.13.1	Outgoing packet switched inter-RAT handovers, UTRAN controlled	92

4.13.1.1	Attempted outgoing packet switched inter-RAT handovers, UTRAN controlled	92

4.13.1.2	Successful outgoing packet switched inter-RAT handovers, UTRAN controlled	92

4.13.1.3	Failed outgoing packet switched inter-RAT handovers UTRAN controlled	92

4.13.2	Outgoing packet switched inter-RAT handovers, UE controlled	93

4.13.2.1	Successful outgoing packet switched inter-RAT handovers, UE controlled	93

4.14	Iu connection release	94

4.14.1	Overview	94

4.14.1.1	Considered Iu connection release procedures	94

4.14.2	Iu connection release request by UTRAN	95

4.14.2.1	Attempted Iu connection release request by UTRAN for CS domain	95

4.14.2.2	Attempted Iu connection release request by UTRAN for PS domain	95

4.14.3	Iu connection release by CN	96

4.14.3.1	Attempted Iu connection release by CN for CS domain	96

4.14.3.2	Attempted Iu connection release by CN for PS domain	96

4.14.3.3	Successful Iu connection release by CN for CS domain	96

4.14.3.4	Successful Iu connection release by CN for PS domain	97

4.15	Intra-cell DCA	98

4.15.1	Attempted intra-cell DCA	98

4.15.2	Successful intra-cell DCA	98

4.15.3	Failed intra-cell DCA	99

4.16	TDD Code Resources	100

4.16.1	UTRAN Cell Max Downlink Code Resources Used	100

4.16.2	UTRAN Cell Max Uplink Code Resources Used	100

4.16.3	Mean Downlink Code Resources Used of an UTRAN Cell	101

4.16.4	Mean Uplink Code Resources Used of an UTRAN Cell	101

4.17	Measurements related to TDD UTRAN cell Radio Frequency Carrier	102

4.17.1	Mean Transmitted Carrier Power of an UTRAN Cell	102

4.17.2	Maximum Transmitted Carrier Power of an UTRAN Cell	102

4.17.3	Mean Received Total Wideband Power of an UTRAN Cell	102

4.17.4	The Maximum Received Total Wideband Power of an UTRAN Cell	103

4.17.5	Mean DPCH Transmitted Code Power of an UTRAN Cell	103

4.17.6	Maximum DPCH Transmitted Code Power of an UTRAN Cell	104

4.17.7	Mean DPCH Received Signal Code Power of an UTRAN Cell	104

4.17.8	Maximum DPCH Received Signal Code Power of an UTRAN Cell	104

4.17.9	Mean Interference Signal Code Power of an UTRAN Cell	105

4.17.10	Maximum Interference Signal Code Power of an UTRAN Cell	106

4.17.11	Mean transmitted carrier power of all codes not used for HS-PDSCH , HS-SCCH, E-AGCH or E-HICH transmission	106

4.17.12	Maximum transmitted carrier power of all codes not used for HS-PDSCH , HS-SCCH, E-AGCH or E-HICH transmission	106

4.18	Measurements related to the UTRAN cell FDD carrier (RF Performance metrics)	108

4.18.1	Mean Transmitted Carrier Power of an UTRAN Cell	108

4.18.2	Maximum Transmitted Carrier Power of an UTRAN Cell	108

4.18.3	Mean Received Total Wideband Power of an UTRAN Cell	109

4.18.4	Maximum Received Total Wideband Power of an UTRAN Cell	109

4.18.5	Mean Radio Link Transmitted Code Power of an UTRAN Cell	109

4.18.6	Maximum Radio Link Transmitted Code Power of an UTRAN Cell	110

4.18.7	Mean transmitted carrier power of all codes not used for HS-PDSCH,  HS-SCCH, E-AGCH, E-RGCH or E-HICH transmission of an UTRAN Cell	110

4.18.8	Maximum transmitted carrier power of all codes not used for HS-PDSCH,  HS-SCCH, E-AGCH, E-RGCH or E-HICH transmission of an UTRAN Cell	111

4.19	FDD Code Resources	112

4.19.1	Code Resources Used of an FDD mode UTRAN Cell	112

4.20	Paging	113

4.20.1	Overview	113

4.20.1.1	Considered Paging procedures	113

4.20.2	Paging request from CN	114

4.20.3	Paging Type 1	114

4.20.3.1	Attempted Paging Type 1	114

4.20.4	Paging Type 2	115

4.20.4.1	Attempted Paging Type 2	115

4.21	ATM layer measurement	116

4.21.1	Cell collection	116

4.21.1.1	Ingress cells (whole stream)	116

4.21.1.2	Egress cells (whole stream)	116

4.22	HSDPA setup	117

4.22.1	Mac-d setup for HSDPA	117

4.22.1.1	Attempted Mac-d setups for HSDPA	117

4.22.1.2	Successful Mac-d setups for HSDPA	117

4.22.1.3	Failed Mac-d setups for HSDPA	117

4.22.2	RB setup for HSDPA	119

4.22.2.1	Attempted RB setups for HSDPA	119

4.22.2.2	Successful RB setups for HSDPA	119

4.22.2.3	Failed RB setups for HSDPA	119

4.23	Call duration in UTRAN cell	121

4.23.1	Call duration in UTRAN cell for CS domain	121

4.24	Measurements related to channel switches between FACH/DCH and HS-DSCH	122

4.24.1	Measurements related to channel switches from FACH/DCH to HS-DSCH intra UTRAN cell	122

4.24.1.1	Attempted channel switches from FACH to HS-DSCH	122

4.24.1.2	Successful channel switches from FACH to HS-DSCH	122

4.24.1.3	Attempted channel switches from DCH to HS-DSCH	122

4.24.1.4	Successful channel switches from DCH to HS-DSCH	123

4.24.2	Measurements related to channel switches from HS-DSCH to FACH/DCH intra UTRAN cell	124

4.24.2.1	Attempted channel switches from HS-DSCH to FACH	124

4.24.2.2	Successful channel switches from HS-DSCH to FACH	124

4.24.2.3	Attempted channel switches from HS-DSCH to DCH	124

4.24.2.4	Successful channel switches from HS-DSCH to DCH	125

4.25	HSDPA mobility related measurements	126

4.25.1	Serving HS-DSCH cell change	126

4.25.1.1	Attempted serving HS-DSCH cell changes	126

4.25.1.2	Successful serving HS-DSCH cell changes	126

4.26	Measurements related to MAC-hs	127

4.26.1	Measurements related to MAC-hs feedback decoding	127

4.26.1.1	Number of acknowleged transmitted MAC-hs PDUs	127

4.26.1.2	Number of transmitted MAC-hs PDUs	127

4.27	Mean number of HSDPA users in a serving HS-DSCH cell	128

4.28	Number of octets of acknowledged MAC-hs PDUs	128

4.29	Number of TTIs with user data packets in buffer	128

4.30	HSDPA Release measurements	129

4.30.1	Overview of HSDPA Release	129

4.30.1.1	General	129

4.30.1.2	Considered RRC procedures	129

4.30.1.3	Considered RANAP protocol procedures	132

4.30.2	Normal HSDPA Release	133

4.30.2.1	RNC Initiated Release due to user inactivity	133

4.30.2.2	Switch to DCH/FACH	133

4.30.2.3	HS-DSCH serving cell change	133

4.30.2.4	CN Initiated release	133

4.30.3	Abnormal RB releases for HS-DSCH	134

4.31	RRC Connection mobility	135

4.31.1	Cell update	135

4.31.1.1	Attempted cell update	135

4.32	Measurements related to received uplink transport block	136

4.32.1	Measurements related to received uplink transport block for CS domain	136

4.32.1.1	Number of received uplink transport blocks of DCH for CS domain	136

4.32.1.2	Number of received uplink faulty transport blocks of DCH for CS domain	136

4.32.2	Measurements related to received uplink transport block for PS domain	138

4.32.2.1	Number of received uplink transport blocks of DCH for PS domain	138

4.32.2.2	Number of received uplink faulty transport blocks of DCH for PS domain	138

4.33	Measurements related to equipment resources	140

4.33.1	RNC Processor usage	140

4.33.1.1	Mean processor usage	140

4.33.1.2	Peak processor usage	140

4.34	HSUPA related measurements	141

4.34.1	HSUPA setup	144

4.34.1.1	Mac-d setup for HSUPA	144

4.34.1.1.1	Attempted Mac-d setups for HSUPA	144

4.34.1.1.2	Successful Mac-d setups for HSUPA	144

4.34.1.1.3	Failed Mac-d setups for HSUPA	145

4.34.1.2	RB setup for HSUPA	147

4.34.1.2.1	Attempted RB setups for HSUPA	147

4.34.1.2.2	Successful RB setups for HSUPA	147

4.34.1.2.3	Failed RB setups for HSUPA	148

4.34.2	Active set related to HSUPA	149

4.34.2.1	Radio link additions to the E-DCH active set (UE side)	149

4.34.2.1.1	Attempted radio link additions to the E-DCH active set (UE side)	149

4.34.2.1.2	Successful radio link additions to the E-DCH active set (UE side)	149

4.34.2.1.3	Failed radio link additions to the E-DCH active set (UE side)	150

4.34.2.2	Radio link deletions to the E-DCH active set (UE side)	150

4.34.2.2.1	Attempted radio link deletions to the E-DCH active set (UE side)	150

4.34.2.2.2	Successful radio link deletions to the E-DCH active set (UE side)	151

4.34.3	Serving E-DCH cell change	152

4.34.3.1	Outgoing serving E-DCH cell change	152

4.34.3.1.1	Attempted outgoing serving E-DCH cell change	152

4.34.3.1.2	Successful outgoing serving E-DCH cell change	152

4.34.3.2	Outgoing serving E-DCH cell change per neighbour cell relation	152

4.34.3.2.1	Attempted outgoing serving E-DCH cell change per neighbour cell relation	152

4.34.3.2.2	Successful outgoing serving E-DCH cell change per neighbour cell relation	153

4.34.4	Mean number of HSUPA users of a serving E-DCH cell	154

4.34.5	Measurements related to channel switches between RACH/DCH and E-DCH	155

4.34.5.1	Measurements related to channel switches from RACH/DCH to E-DCH	155

4.34.5.1.1	Attempted channel switches from RACH to E-DCH	155

4.34.5.1.2	Successful channel switches from RACH to E-DCH	155

4.34.5.1.3	Attempted channel switches from DCH to E-DCH	155

4.34.5.1.4	Successful channel switches from DCH to E-DCH	156

4.34.5.2	Measurements related to channel switches from E-DCH to RACH/DCH	156

4.34.5.2.1	Attempted channel switches from E-DCH to RACH	156

4.34.5.2.2	Successful channel switches from E-DCH to RACH	157

4.34.5.2.3	Attempted channel switches from E-DCH to DCH	157

4.34.5.2.4	Successful channel switches from E-DCH to DCH	157

4.34.6	Measurements related to MAC-e feedback decoding	159

4.34.6.1	Number of received MAC-e PDUs	159

4.34.6.2	Number of received acknowledged MAC-e PDUs	159

4.34.7	Number of octets of acknowledged MAC-e PDUs	160

4.34.8	HSUPA Release measurements	161

4.34.8.1	Normal HSUPA Release	161

4.34.8.1.1	RNC initiated release due to user inactivity	161

4.34.8.1.2	RNC initiated release due to switch to DCH/RACH	161

4.34.8.1.3	RNC initiated release due to serving E-DCH cell change	161

4.34.8.1.4	CN initiated release	161

4.34.8.2	Abnormal RB releases for E-DCH	161

4.35	 Radio resource utilization related measurements	162

4.35.1 UTRAN Cell Unavailable Time	162

4.36	MBMS related measurements	162

4.36.1	Number of octets of MBMS data on MAC layer	162

4.36.2	Session related measurements	163

4.36.2.1	Attempted MBMS session start	163

4.36.2.2	Successful MBMS session start	163

4.36.2.3	Attempted MBMS session stop	163

4.36.2.4	Successful MBMS session stop	164

4.37	Dual-Cell HSDPA related measurements	164

4.37.1	Dual-Cell HSDPA Scheduling duration in UTRAN cell	164

4.37.2	Individual cell HSDPA Scheduling duration in UTRAN cell	164

4.37.3	Attempted Dual-Cell HSDPA establishments	165

4.37.4	Successful Dual-Cell HSDPA establishments	165

4.37.5	Failed Dual-Cell HSDPA establishments	166

Annex A (informative):	Change history	167

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; as identified below:

32.401	Performance Management (PM); Concept and requirements

52.402	Performance Management (PM); Performance measurements – GSM

32.404	Performance Management (PM); Performance measurements - Definitions and template

32.405	Performance Management (PM); Performance measurements Universal Terrestrial Radio Access Network (UTRAN)

32.406	Performance Management (PM); Performance measurements Core Network (CN) Packet Switched (PS) domain

32.407	Performance Management (PM); Performance measurements Core Network (CN) Circuit Switched (CS) domain

32.408	Performance Management (PM); Performance measurements Teleservice

32.409	Performance Management (PM); Performance measurements IP Multimedia Subsystem (IMS)

The present document is part of a set of specifications, which describe the requirements and information model necessary for the standardised Operation, Administration and Maintenance (OA&M) of a multi-vendor 3G-system.

During the lifetime of a 3G network, its logical and physical configuration will undergo changes of varying degrees and frequencies in order to optimise the utilisation of the network resources. These changes will be executed through network configuration management activities and/or network engineering, see TS 32.600 [3].

Many of the activities involved in the daily operation and future network planning of a 3G network require data on which to base decisions. This data refers to the load carried by the network and the grade of service offered. In order to produce this data performance measurements are executed in the NEs, which comprise the network. The data can then be transferred to an external system, e.g. an Operations System (OS) in TMN terminology, for further evaluation. The purpose of the present document is to describe the mechanisms involved in the collection of the data and the definition of the data itself.

Annex B of TS 32.404 helps in the definition of new performance measurements that can be submitted to 3GPP for potential adoption and inclusion in the present document. Annex B of TS 32.404 discusses a top-down performance measurement definition methodology that focuses on how the end-user of performance measurements can use the measurements.

# 1	Scope

The present document describes the measurements for UMTS and combined UMTS/GSM.
TS 32.401 [1] describes Performance Management concepts and requirements.
The present document is valid for all measurement types provided by an implementation of a UMTS network and combined UMTS/GSM network. Only measurement types that are specific to UMTS or combined UMTS/GSM networks are defined within the present documents.

Vendor specific measurement types used in UMTS and combined UMTS/GSM networks are not covered. Instead, these could be applied according to manufacturer's documentation.

Measurements related to "external" technologies (such as ATM or IP) as described by "external" standards bodies (e.g. ITU-T or IETF) shall only be referenced within this specification, wherever there is a need identified for the existence of such a reference.

The definition of the standard measurements is intended to result in comparability of measurement data produced in a multi-vendor network, for those measurement types that can be standardised across all vendors' implementations.

The structure of the present document is as follows:

-	Header 1: Network Element (e.g. RNC related measurements);

-	Header 2: Measurement function (e.g. soft handover measurements);

-	Header 3: Measurements.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication and/or edition number or version number) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[2]	3GPP TS 23.107: "Quality of Service (QoS) concept and architecture".

[3]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[4]	3GPP TS 25.331: "Radio Resource Control (RRC) protocol specification".

[5]	3GPP TS 25.413: "UTRAN Iu Interface RANAP signalling".

[6]	3GPP TS 25.423: "UTRAN Iur Interface RNSAP signalling".

[7]	3GPP TS 25.433: "UTRAN Iub Interface NBAP signalling".

[8]	3GPP TS 25.133: "Requirements for support of radio resource management (FDD)".

[9]	3GPP TS 25.123: "Requirements for support of radio resource management (TDD)".

[10]	3GPP TS 25.322: "Radio Link Control (RLC) protocol specification".

[11]	af-nm-0185.000, "M4 Interface ATM Network View CORBA MIB".

[12]	3GPP TS 32.432: "Telecommunication management; Performance measurement: File format definition".

[13]	3GPP TS 25.993: "Typical examples of Radio Access Bearers (RABs) and Radio Bearers (RBs) supported by Universal Terrestrial Radio Access (UTRA)".

[14]	3GPP TS 25.215: "Physical layer – Measurements (FDD)".

[15]	3GPP TS 32.432: "Telecommunication management; Performance measurement: File format definition".

[16]	3GPP TS 25.225: "Physical layer – Measurements (TDD)".

[17]	3GPP TS 25.427: "UTRAN Iub/Iur interface user plane protocol for DCH data streams".

[18]	3GPP TS 32.410: "Key Performance Indicators (KPI) for UMTS and GSM ".

[19]	3GPP TS 25.308: "High Speed Downlink Packet Access (HSDPA): Overall Description; Stage 2".

# 3	Measurement family and abbreviations

## 3.1	Measurement family

The measurement names defined in the present document are all beginning with a prefix containing the measurement family name (e.g. RAB.AttEstabCS.Conv). This family name identifies all measurements which relate to a given functionality and it may be used for measurement administration (see TS 32.401 [1]).

The list of families currently used in the present document is as follows:

-	ATML (measurements related to ATM Layer)

-	CARR (measurements related to UTRAN cell Radio Frequency carrier)

-	CR (measurements related to Code Resources)

-	DCA (measurements related to Dynamic Channel Allocation)

-	EQPT (measurements related to Equipment)

-	FP (measurements related to Frame Protocol)

-	HHO (measurements related to Hard Handover)

-	HSDPA (measurements related to High Speed Downlink Packet Access)

-	HSUPA (measurements related to High Speed Uplink Packet Access)

-	IRATHO (measurements related to inter-Radio Access Technology Handover)

-	IU (measurements related to Iu connection)

-	MBMS (measurements related to Multimedia Broadcast Multicast Service)

-	RAB (measurements related to Radio Access Bearer management)

-	RELOC (measurements related to SRNS Relocation)

-	RLC (measurements related to Radio Link Control)

-	RLM (measurements related to Radio Link Management)

-	RRC (measurements related to Radio Resource Control)

-	SHO (measurements related to Soft Handover)

-	SIG (measurements related to Signalling)

-	TCR (measurements related to TDD Code Resources)

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

ASN.1	Abstract Syntax Notation 1

BER	Basic Encoding Rules

CN	Core Network

DTD	Document Type Definition

EGQM	Enhanced Goal, Question, Metric

EM	(Network) Element Manager

GQM		Goal, Question, Metric

HSDPA	High Speed Downlink Packet Access

HSUPA	High Speed Uplink Packet Access

Itf	Interface

MSC	Mobile services Switching Centre

NE	Network Element

NM	Network Manager

OA&M	Operation, Administration and Maintenance

OS	Operations System (EM, NM)

OSI	Open Systems Interconnection

PM	Performance Management

QoS	Quality of Service

RNC	Radio Network Controller

UMTS	Universal Mobile Telecommunications System

UTRAN	Universal Terrestrial Radio Access Network

You can find below a list of abbreviations used within the measurement types for field E of the measurement template.

Ackd	Acknowledged

Assn	Assign(ment,ed)

Att	Attempt(s,ed)

Bgrd	Background

Call	Call

Chg	Change

Conn	Connection

Combi	Combined

CS	Circuit switched

Ctrl	Controlled

Conv	Conversational

Del	Deletion

Drop	Drop(ped)

Estab	Establish (ed,ment)

Fail	Fail(ed, ure)

FDD	Frequency Division Duplex

FP	Frame Protocol

HHO	Hard Handover

HO	Handover

Inc	Incoming

Intact	Interactive

Inter	Inter

Intra	Intra

Invol	Involve(d)

ISCP	Interference Signal Code Power

Max	Maximum

Nat	National

Netw	Network

NodeB	NodeB

Oct	Octet(s)

Oth	Other

Out	Outgoing

Pkt	Packet(s)

Prep	Preparation

Proc	Procedure

PS	Packet switched

RAB	Radio Access Bearer

RAT	Radio Access Technology

RB	Radio Bearer

ReEstab	Re-establish (ed,ment)

Rel	Released

Reloc	Relocation

Req	Request(s,ed)

Res	Resource

RL	Radio Link

RNC	RNC

RRC	Radio Resource Control

RTWP	Received Total Wideband Power

Setup	Setup

SGSN	SGSN

SHO	Soft Handover

Sig	Signalling

Strm	Streaming

Sub	Subscriber

Succ	Success(es,ful)

TCP	Transmitted Carrier Power

UE	User Equipment

UTRAN	UTRAN

# 4	Measurements related to the RNC

## 4.1	RAB management

### 4.1.1	Overview

#### 4.1.1.1	Measurements are based on the success and failure of procedures

The proposed measurements are not merely based on the counting of a given type of message since a same message may be repeated by an implementation dependent process. The aim here is to provide implementation independent specification.

Proposed measurements are based on the success/failure of procedures identified in the reference documents. The end of a procedure implies a stable state of the communication between the two involved parties. This stable state is normally the object of a common understanding from the two parties. As a consequence, proposed measurements are attached either to the successful or the unsuccessful issue of a procedure.

#### 4.1.1.2	Combination of Traffic Class and Core Network domains

A Radio Access Bearer (RAB) is characterized by several QOS parameters among them is the Traffic Class. Currently there are not any 3GPP specifications including TS 23.107 2 in which may be found restrictions related to the possible combinations between Traffic Class and Core Network domain.

Consequently, as a conservative position, this specification should leave open every possible combination between Traffic Class and Core Network domain as specification TS 23.107 2 does.

#### 4.1.1.3	Considered Radio Access Bearer management procedures

Performance Measurement definitions in this subclause are based on TS 25.413 [5].

The following paragraphs are of interest for this purpose:

-	RAB Assignment;

-	RAB Release Request;

-	RAB ASSIGNMENT REQUEST;

-	RAB ASSIGNMENT RESPONSE;

-	RAB RELEASE REQUEST.

These paragraphs show in particular the following diagrams.

- Figure: RAB Assignment procedure. Successful operation

- Figure: RAB Release Request procedure

#### 4.1.1.4	Measurements relate to RAB establishment, modification and release

RAB management procedure includes RAB Assignment procedure and RAB Release Request procedure. The purpose of RAB Assignment procedure is to establish new RABs and/or to enable modifications and/or releases of already established RABs for a given UE. If RABs are failed to be established or modified, the involved services may fail. RAB release request can be initiated by CN or RNC when the services terminate normally or abnormally.

During daily maintenance of network, measurements regarding RAB establishment, modification and release are useful for operators to evaluate RAB management procedures, to analyze failure reasons of RAB establishment and RAB modification, and to analyze the causes of RAB release, especially in case RAB release abnormally.

### 4.1.2	RAB establishment for CS domain

The five measurement types defined in the clause 4.1.2 for CS domain are subject to the "4 out of 5 approach".

#### 4.1.2.1	Attempted RAB establishments for CS domain

- This measurement provides the number of requested RAB in establishment attempts for CS domain. The measurement is split into subcounters per traffic class.

- CC

- On receipt by the RNC of a RANAP RAB ASSIGNMENT REQUEST message for CS domain, each requested RAB in establishment attempts is added to the relevant measurement according to the traffic class requested. See TS 25.413 [5] and TS 23.107 [2]. For conversational service, the relevant measurement according to the data rates requested, see TS 25.993 [13] as follows:
uplink<U>:
			1: AMR rate (12.2 10.2 7.95 7.4 6.7 5.9 5.15 4.75) kbps
 		2: 12.2 kbps
 		3: 28.8 kbps
 		4: 32 kbps
 		5: 64 kbps
		6: AMR -WB-rate (12.65, 8.8, 6.65) kbps
downlink<D>: 
			1: AMR rate (12.2 10.2 7.95 7.4 6.7 5.9 5.15 4.75) kbps
 		2: 12.2 kbps
 		3: 28.8 kbps
 		4: 32 kbps
 		5: 64 kbps
		6: AMR -WB-rate (12.65, 8.8, 6.65) kbps
As indicated above, <U> and <D> are integer values that map to the conversational service specified uplink and downlink data rates respectively.

NOTE :	The addition is performed with the condition that the RAB has not been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Each measurement is an integer value.

- RAB.AttEstabCS.Conv.<U><D>
RAB.AttEstabCS.Strm
RAB.AttEstabCS.Intact
RAB.AttEstabCS.Bgrd

- RncFunction

- Valid for circuit switched traffic

- UMTS

#### 4.1.2.2	Successful RAB establishments without queuing for CS domain

- This measurement provides the number of successfully established RABs for CS domain in which a queuing process has not been involved. The measurement is split into subcounters per traffic class.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for CS domain, each successfully established RAB is added to the relevant measurement according to the traffic class requested in the RAB ASSIGNMENT REQUEST message. See TS 25.413 [5] and TS 23.107 [2]. For conversational service, the relevant measurement according to the data rates requested, see TS 25.993 [13] as follows:
uplink<U>:
			1: AMR rate (12.2 10.2 7.95 7.4 6.7 5.9 5.15 4.75) kbps
 		2: 12.2 kbps
 		3: 28.8 kbps
 		4: 32 kbps
 		5: 64 kbps
		6: AMR -WB-rate (12.65, 8.8, 6.65) kbps
downlink<D>: 
			1: AMR rate (12.2 10.2 7.95 7.4 6.7 5.9 5.15 4.75) kbps
 		2: 12.2 kbps
 		3: 28.8 kbps
 		4: 32 kbps
 		5: 64 kbps
		6: AMR -WB-rate (12.65, 8.8, 6.65) kbps
As indicated above, <U> and <D> are integer values that map to the conversational service specified uplink and downlink data rates respectively.

NOTE:	The addition is performed with the condition that the RAB has not been mentioned as queued in a previous RANAP RAB ASSIGNMENT RESPONSE and the RAB has not been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Each measurement is an integer value.

- RAB.SuccEstabCSNoQueuing.Conv.<U><D>
RAB.SuccEstabCSNoQueuing.Strm
RAB.SuccEstabCSNoQueuing. Intact
RAB.SuccEstabCSNoQueuing.Bgrd.

- RncFunction

- Valid for circuit switched traffic

- UMTS

#### 4.1.2.3	Failed RAB establishments without queuing for CS domain

- This measurement provides the number of RABs failed to establish for CS domain in which a queuing process has not been involved. The measurement is split into subcounters per failure cause.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for CS domain, each RAB failed to establish is added to the relevant measurement according to the failure cause. Possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of RAB Establishment Failures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

NOTE:	The addition is performed with the condition that the RAB has not been mentioned as queued in a previous RANAP RAB ASSIGNMENT RESPONSE and the RAB has not been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RAB.FailEstabCSNoQueuing.Cause
where Cause identifies the failure cause.

- RncFunction

- Valid for circuit switched traffic

- UMTS

#### 4.1.2.4	Successful RAB establishments with queuing for CS domain

- This measurement provides the number of successfully established RABs for CS domain in which a queuing process has been involved. The measurement is split into subcounters per traffic class.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for CS domain, each successfully established RAB is added to the relevant measurement according to the traffic class. See TS 25.413 [5] and TS 23.107 [2]. For conversational service, the relevant measurement according to the data rates requested, see TS 25.993 [13] as follows:
uplink<U>:
		1: AMR rate (12.2 10.2 7.95 7.4 6.7 5.9 5.15 4.75) kbps
 		2: 12.2 kbps
 		3: 28.8 kbps
 		4: 32 kbps
 		5: 64 kbps
		6: AMR -WB-rate (12.65, 8.8, 6.65) kbps
downlink<D>: 
		1: AMR rate (12.2 10.2 7.95 7.4 6.7 5.9 5.15 4.75) kbps
 		2: 12.2 kbps
 		3: 28.8 kbps
 		4: 32 kbps
 		5: 64 kbps
		6: AMR -WB-rate (12.65, 8.8, 6.65) kbps
As indicated above, <U> and <D> are integer values that map to the conversational service specified uplink and downlink data rates respectively.

NOTE:	The addition is performed with the condition that the RAB has been mentioned as queued in a previous RANAP RAB ASSIGNMENT RESPONSE and the RAB has not been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Each measurement is an integer value.

- RAB.SuccEstabCSQueuing.Conv.<U><D>
RAB.SuccEstabCSQueuing.Strm
RAB.SuccEstabCSQueuing.Intact
RAB.SuccEstabCSQueuing.Bgrd

- RncFunction

- Valid for circuit switched traffic

- UMTS

#### 4.1.2.5	Failed RAB establishments with queuing for CS domain

- This measurement provides the number of RABs failed to establish for CS domain in which a queuing process has been involved. The measurement is split into subcounters per failure cause.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for CS domain, each RAB failed to establish is added to the relevant measurement according to the cause. Possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of RAB Establishment Failures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

NOTE:	The addition is performed with the condition that the RAB has been mentioned as queued in a previous RANAP RAB ASSIGNMENT RESPONSE and the RAB has not been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RAB.FailEstabCSQueuing.Cause
where Cause identifies the failure cause.

- RncFunction

- Valid for circuit switched traffic

- UMTS

### 4.1.3	RAB establishment for PS domain

The five measurement types defined in the clause 4.1.3 for PS domain are subject to the "4 out of 5 approach".

#### 4.1.3.1	Attempted RAB establishments for PS domain

- This measurement provides the number of requested RABs in establishment attempts for PS domain. The measurement is split into subcounters per traffic class.

- CC

- On receipt by the RNC of a RANAP RAB ASSIGNMENT REQUEST message for PS domain, each requested RAB in establishment attempts is added to the relevant measurement according to the traffic class requested. See TS 25.413 [5] and TS 23.107 [2].

NOTE:	The addition is performed with the condition that the RAB has not been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Four integer values.

- RAB.AttEstabPS.Conv
RAB.AttEstabPS.Strm
RAB.AttEstabPS.Intact
RAB.AttEstabPS.Bgrd

- RncFunction

- Valid for packet switched traffic.

- UMTS

#### 4.1.3.2	Successful RAB establishments without queuing for PS domain

- This measurement provides the number of successfully established RABs for PS domain in which a queuing process has not been involved. The measurement is split into subcounters per traffic class.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for PS domain, each successfully established RAB is added to the relevant measurement according to the traffic class. See TS 25.413 [5] and TS 23.107 [2].

NOTE:	The addition is performed with the condition that the RAB has not been mentioned as queued in a previous RANAP RAB ASSIGNMENT RESPONSE and the RAB has not been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Four integer values.

- RAB.SuccEstabPSNoQueuing.Conv
RAB.SuccEstabPSNoQueuing.Strm
RAB.SuccEstabPSNoQueuing.Intact 
RAB.SuccEstabPSNoQueuing.Bgrd

- RncFunction

- Valid for packet switched traffic.

- UMTS

#### 4.1.3.3	Failed RAB establishments without queuing for PS domain

- This measurement provides the number of RABs failed to establish for PS in which a queuing process has not been involved. The measurement is split into subcounters per failure cause.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for PS domain, each RAB failed to establish is added to the relevant measurement according to the failure cause. Possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of RAB Establishment Failures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

NOTE:	The addition is performed with the condition that the RAB has not been mentioned as queued in a previous RANAP RAB ASSIGNMENT RESPONSE and the RAB has not been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RAB.FailEstabPSNoQueuing.Cause
where Cause identifies the failure cause.

- RncFunction

- Valid for packet switched traffic.

- UMTS

#### 4.1.3.4	Successful RAB establishments with queuing for PS domain

- This measurement provides the number of successfully established RABs for PS domain in which a queuing process has been involved. The measurement is split into subcounters per traffic class.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for PS domain, each successfully established RAB is added to the relevant measurement according to the traffic class. 
See TS 25.413 [5] and TS 23.107 [2].

NOTE:	The addition is performed with the condition that the RAB has been mentioned as queued in a previous RANAP RAB ASSIGNMENT RESPONSE and the RAB has not been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Four integer values.

- RAB.SuccEstabPSQueuing.Conv
RAB.SuccEstabPSQueuing.Strm
RAB.SuccEstabPSQueuing.Intact
RAB.SuccEstabPSQueuing.Bgrd

- RncFunction

- Valid for packet switched traffic.

- UMTS

#### 4.1.3.5	Failed RAB establishments with queuing for PS domain

- This measurement provides the number of RABs failed to establish for PS domain in which a queuing process has been involved. The measurement is split into subcounters per failure cause.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for PS domain, each RAB failed to establish is added to the relevant measurement according to the cause. Possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of RAB Establishment Failures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

NOTE:	The addition is performed with the condition that the RAB has been mentioned as queued in a previous RANAP RAB ASSIGNMENT RESPONSE and the RAB has not been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RAB.FailEstabPSQueuing.Cause
where Cause identifies the failure cause.

- RncFunction

- Valid for packet switched traffic.

- UMTS

### 4.1.4	RAB modification for CS domain

The five measurement types defined in the clause 4.1.4 for CS domain are subject to the "4 out of 5 approach".

#### 4.1.4.1	Attempted RAB modifications for CS domain

- This measurement provides the number of requested RABs in modification attempts for CS domain. The measurement is split into subcounters per traffic class.

- CC

- On receipt by the RNC of a RANAP RAB ASSIGNMENT REQUEST message for CS domain, each requested RAB in modification attempts is added to the relevant measurement according to the traffic class requested. See TS 25.413 and TS 23.107.

NOTE:	The addition is performed with the condition that the RAB has been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Four integer values.

- RAB.AttModCS.Conv 
RAB.AttModCS.Strm
RAB.AttModCS.Intact
RAB.AttModCS.Bgrd

- RncFunction

- Valid for circuit switched traffic

- UMTS

#### 4.1.4.2	Successful RAB modifications without queuing for CS domain

- This measurement provides the number of successfully modified RABs for CS domain in which a queuing process has not been involved. The measurement is split into subcounters per traffic class.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for CS domain, each successfully modified RAB is added to the relevant measurement according to the traffic class requested in the RAB ASSIGNMENT REQUEST message. See TS 25.413 [5] and TS 23.107 [2].

NOTE:	The addition is performed with the condition that the RAB has not been mentioned as queued in a previous RANAP RAB ASSIGNMENT RESPONSE and the RAB has been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Four integer values.

- RAB.SuccModCSNoQueuing.Conv
RAB.SuccModCSNoQueuing.Strm
RAB.SuccModCSNoQueuing.Intact
RAB.SuccModCSNoQueuing.Bgrd

- RncFunction

- Valid for circuit switched traffic

- UMTS

#### 4.1.4.3	Failed RAB modifications without queuing for CS domain

- This measurement provides the number of RABs failed to modify for CS domain in which a queuing process has not been involved. The measurement is split into subcounters per failure cause.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for CS domain, each RAB failed to modify is added to the relevant measurement according to the failure cause. Possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of RAB Modification Failures. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first.

NOTE:	The addition is performed with the condition that the RAB has not been mentioned as queued in a previous RANAP RAB ASSIGNMENT RESPONSE and the RAB has been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RAB.FailModCSNoQueuing.Cause
where Cause identifies the failure cause.

- RncFunction

- Valid for circuit switched traffic

- UMTS

#### 4.1.4.4	Successful RAB modifications with queuing for CS domain

- This measurement provides the number of successfully modified RABs for CS domain in which a queuing process has been involved. The measurement is split into subcounters per traffic class.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for CS domain, each successfully modified RAB is added to the relevant measurement according to the traffic class. 
See TS 25.413 [5] and TS 23.107 [2].

NOTE:	The addition is performed with the condition that the RAB has been mentioned as queued in a previous RANAP RAB ASSIGNMENT RESPONSE and the RAB has been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Four integer values.

- RAB.SuccModCSQueuing.Conv
RAB.SuccModCSQueuing.Strm
RAB.SuccModCSQueuing.Intact
RAB.SuccModCSQueuing.Bgrd

- RncFunction

- Valid for circuit switched traffic

- UMTS

#### 4.1.4.5	Failed RAB modifications with queuing for CS domain

- This measurement provides the number of RABs failed to modify for CS domain in which a queuing process has been involved. The measurement is split into subcounters per failure cause.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for CS domain, each RAB failed to modify is added to the relevant measurement according to the cause. Possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of RAB Modification Failures. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first.

NOTE:	The addition is performed with the condition that the RAB has been mentioned as queued in a previous RANAP RAB ASSIGNMENT RESPONSE and the RAB has been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RAB.FailModCSQueuing.Cause
where Cause identifies the failure cause.

- RncFunction

- Valid for circuit switched traffic

- UMTS

### 4.1.5	RAB modification for PS domain

The five measurement types defined in the clause 4.1.5 for PS domain are subject to the "4 out of 5 approach".

#### 4.1.5.1	Attempted RAB modifications for PS domain

- This measurement provides the number of requested RABs in modification attempts for PS domain. The measurement is split into subcounters per traffic class.

- CC

- On receipt by the RNC of a RANAP RAB ASSIGNMENT REQUEST message for PS domain, each requested RAB in modification attempts is added to the relevant measurement according to the traffic class requested. 
See TS 25.413 [5] and TS 23.107 [2].

NOTE:	The addition is performed with the condition that the RAB has been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Four integer values.

- RAB.AttModPS.Conv
RAB.AttModPS.Strm
RAB.AttModPS.Intact
RAB.AttModPS.Bgrd.

- RncFunction

- Valid for packet switched traffic.

- UMTS

#### 4.1.5.2	Successful RAB modifications without queuing for PS domain

- This measurement provides the number of successfully modified RABs for PS domain in which a queuing process has not been involved. The measurement is split into subcounters per traffic class.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for PS domain, each successfully modified RAB is added to the relevant measurement according to the traffic class. 
See TS 25.413 [5] and TS 23.107 [2].

NOTE:	The addition is performed with the condition that the RAB has not been mentioned as queued in a previous RANAP RAB ASSIGNMENT RESPONSE and the RAB has been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Four integer values.

- RAB.SuccModPSNoQueuing.Conv
RAB.SuccModPSNoQueuing.Strm
RAB.SuccModPSNoQueuing.Intact
RAB.SuccModPSNoQueuing.Bgrd.

- RncFunction

- Valid for packet switched traffic.

- UMTS

#### 4.1.5.3	Failed RAB modifications without queuing for PS domain

- This measurement provides the number of RABs failed to modify for PS in which a queuing process has not been involved. The measurement is split into subcounters per failure cause.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for PS domain, each RAB failed to modify is added to the relevant measurement according to the failure cause. Possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of RAB Modification Failures. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first.

NOTE:	The addition is performed with the condition that the RAB has not been mentioned as queued in a previous RANAP RAB ASSIGNMENT RESPONSE and the RAB has been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RAB.FailModPSNoQueuing.Cause
where Cause identifies the failure cause.

- RncFunction

- Valid for packet switched traffic.

- UMTS

#### 4.1.5.4	Successful RAB modifications with queuing for PS domain

- This measurement provides the number of successfully modified RABs for PS domain in which a queuing process has been involved. The measurement is split into subcounters per traffic class.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for PS domain, each successfully modified RAB is added to the relevant measurement according to the traffic class. 
See TS 25.413 [5] and TS 23.107 [2].

NOTE:	The addition is performed with the condition that the RAB has been mentioned as queued in a previous RANAP RAB ASSIGNMENT RESPONSE and the RAB has been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Four integer values.

- RAB.SuccModPSQueuing.Conv
RAB.SuccModPSQueuing.Strm
RAB.SuccModPSQueuing.Intact
RAB.SuccModPSQueuing.Bgrd.

- RncFunction

- Valid for packet switched traffic.

- UMTS

#### 4.1.5.5	Failed RAB modifications with queuing for PS domain

- This measurement provides the number of RABs failed to modify for PS domain in which a queuing process has been involved. The measurement is split into subcounters per failure cause.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for PS domain, each RAB failed to modify is added to the relevant measurement according to the cause. Possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of RAB Modification Failures. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first.

NOTE:	The addition is performed with the condition that the RAB has been mentioned as queued in a previous RANAP RAB ASSIGNMENT RESPONSE and the RAB has been setup or modified successfully in a previous RANAP RAB ASSIGNMENT RESPONSE or RELOCATION REQUEST ACKNOWLEDGE.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RAB.FailModPSQueuing.Cause
where Cause identifies the failure cause.

- RncFunction

- Valid for packet switched traffic.

- UMTS

### 4.1.6	RAB release request by CN for CS domain

The three measurement types defined in the clause 4.1.6 for CS domain are subject to the "2 out of 3 approach".

#### 4.1.6.1	Attempted RAB releases for CS domain

- This measurement provides the number of requested RABs in release attempts for CS domain. The measurement is split into subcounters per release cause.

- CC

- On receipt by the RNC of a RANAP RAB ASSIGNMENT REQUEST message for CS domain, each requested RAB in release attempts is added to the relevant measurement according to the release cause requested.  Possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of RAB Release Attempts. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first. See TS 25.413 [5] and TS 23.107 [2].

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RAB.AttRelCS.Cause
where Cause identifies the release cause.

- RncFunction

- Valid for circuit switched traffic

- UMTS

#### 4.1.6.2	Successful RAB releases for CS domain

- This measurement provides the number of successfully released RABs for CS domain. The measurement is split into subcounters per release cause.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for CS domain, each successfully released RAB is added to the relevant measurement according to the release cause requested in the RAB ASSIGNMENT REQUEST message. Possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of RAB Release Successes. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first. 
See TS 25.413 [5] and TS 23.107 [2].

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RAB.SuccRelCS.Cause
where Cause identifies the release cause.

- RncFunction

- Valid for circuit switched traffic

- UMTS

#### 4.1.6.3	Failed RAB releases for CS domain

- This measurement provides the number of RABs failed to release for CS domain. The measurement is split into subcounters per failure cause.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for CS domain, each RAB failed to release is added to the relevant measurement according to the failure cause. Possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of RAB Release Failures. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RAB.FailRelCS.Cause
where Cause identifies the failure cause.

- RncFunction

- Valid for circuit switched traffic

- UMTS

- 4.1.6.4	Void4.1.6.5	Void

### 4.1.7	RAB release request by CN for PS domain

The three measurement types defined in the clause 4.1.7 for PS domain are subject to the "2 out of 3 approach".

#### 4.1.7.1	Attempted RAB releases for PS domain

- This measurement provides the number of requested RABs in release attempts for PS domain. The measurement is split into subcounters per release cause.

- CC

- On receipt by the RNC of a RANAP RAB ASSIGNMENT REQUEST message for PS domain, each requested RAB in release attempts is added to the relevant measurement according to the release cause. Possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of RAB Release Attempts. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first. See TS 25.413 [5] and TS 23.107 [2].

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RAB.AttRelPS.Cause
where Cause identifies the release cause.

- RncFunction

- Valid for packet switched traffic.

- UMTS

#### 4.1.7.2	Successful RAB releases for PS domain

- This measurement provides the number of successfully released RABs for PS domain. The measurement is split into subcounters per release cause.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for PS domain, each successfully released RAB is added to the relevant measurement according to the release cause. Possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of RAB Release Successes. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first. See TS 25.413 [5] and TS 23.107 [2].

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

e)	The measurement name has the form RAB.SuccRelPS.Cause
where Cause identifies the release cause.

- RncFunction

- Valid for packet switched traffic.

- UMTS

#### 4.1.7.3	Failed RAB releases for PS domain

- This measurement provides the number of RABs failed to release for PS. The measurement is split into subcounters per failure cause.

- CC

- On transmission by the RNC of a RANAP RAB ASSIGNMENT RESPONSE message for PS domain, each RAB failed to release is added to the relevant measurement according to the failure cause. Possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of RAB Release Failures. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RAB.FailRelPS.Cause
where Cause identifies the failure cause.

- RncFunction

- Valid for packet switched traffic.

- UMTS

#### 4.1.7.4	Void

#### 4.1.7.5	Void

### 4.1.8	RAB setup time

#### 4.1.8.1	RAB CS connection set-up time (Mean)

- This measurement provides the mean time during each granularity period for a RNC to establish a RAB CS connection.

- DER (n=1).

- This measurement is obtained by accumulating the time intervals for each successful RAB establishment between the receipt by the RNC of a RANAP "RAB ASSIGNMENT REQUEST" message to establish a RAB for CS domain, and the first corresponding (based on RAB ID) transmission by the RNC of a RANAP "RAB ASSIGNMENT RESPONSE" message for successfully established RABs over a granularity period using DER, see TS 25.413 [5]. This end value of the time will then be divided by the number of successfully established RABs observed in the granularity period to give the arithmetic mean, the accumulator shall be reinitialised at the beginning of each granularity period.

- Each measurement is an integer value.(in milliseconds).

- RAB.SuccEstabCSSetupTimeMean

- RncFunction

- Valid for circuit switched traffic

- UMTS

#### 4.1.8.2	RAB CS connection set-up time (Maximum)

- This measurement provides the maximum time during each granularity period for a RNC to establish a RAB CS connection.

- GAUGE.

- This measurement is obtained by monitoring the time intervals for each successful RAB establishment between the receipt by the RNC of a RANAP "RAB ASSIGNMENT REQUEST" message to establish a RAB for CS domain, and the first corresponding (based on RAB ID) transmission by the RNC of a RANAP "RAB ASSIGNMENT RESPONSE" message for successfully established RABs see TS 25.413 [5]. The high tide mark of this time will be stored in a gauge, the gauge shall be reinitialised at the beginning of each granularity period.

- Each measurement is an integer value.(in milliseconds).

- RAB.SuccEstabCSSetupTimeMax

- RncFunction

- Valid for circuit switched traffic

- UMTS

#### 4.1.8.3	RAB PS connection set-up time (Mean)

- This measurement provides the mean time during each granularity period for a RNC to establish a RAB PS connection.

- DER (n=1).

- This measurement is obtained by accumulating the time intervals for each successful RAB establishment between the receipt by the RNC of a RANAP "RAB ASSIGNMENT REQUEST" message to establish a RAB for PS domain, and the first corresponding (based on RAB ID) transmission by the RNC of a RANAP "RAB ASSIGNMENT RESPONSE" message for successfully established RABs over a granularity period using DER, see TS 25.413 [5]. This end value of the time will then be divided by the number of successfully established RABs observed in the granularity period to give the arithmetic mean, the accumulator shall be reinitialised at the beginning of each granularity period.

- Each measurement is an integer value.(in milliseconds).

- RAB.SuccEstabPSSetupTimeMean

- RncFunction

- Valid for packet switched traffic.

- UMTS

#### 4.1.8.4	RAB PS connection set-up time (Maximum)

- This measurement provides the maximum time during each granularity period for a RNC to establish a RAB PS connection.

- GAUGE.

- This measurement is obtained by monitoring the time intervals for each successful RAB establishment between the receipt by the RNC of a RANAP "RAB ASSIGNMENT REQUEST" message to establish a RAB for PS domain, and the first corresponding (based on RAB ID) transmission by the RNC of a RANAP "RAB ASSIGNMENT RESPONSE" message for successfully established RABs see TS 25.413 [5]. The high tide mark of this time will be stored in a gauge, the gauge shall be reinitialised at the beginning of each granularity period.

- Each measurement is an integer value.(in milliseconds).

- RAB.SuccEstabPSSetupTimeMax

- RncFunction

- Valid for packet switched traffic.

- UMTS

### 4.1.9	RAB release request by UTRAN

#### 4.1.9.1	RAB release requests for CS domain

- This measurement provides the number of RABs requested to release by UTRAN for CS domain split into subcounters per cause.

- CC

- On transmission by the RNC of a RANAP RAB RELEASE REQUEST message for CS domain, each RAB requested to be released is added to the relevant per cause measurement. Possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of RAB Release Requests for the CS domain. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RAB.RelReqCS.Cause
where Cause identifies the release cause.

- RncFunction

- Valid for circuit switched traffic

- UMTS

#### 4.1.9.2	RAB release requests for PS domain

- This measurement provides the number of RABs requested to release by UTRAN for PS domain split into subcounters per cause.

- CC

- On transmission by the RNC of a RANAP RAB RELEASE REQUEST message for PS domain, each RAB requested to be released is added to the relevant per cause measurement. Possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of RAB Release Requests for the PS domain. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RAB.RelReqPS.Cause 
where Cause identifies the release cause.

- RncFunction

- Valid for packet switched traffic.

- UMTS

#### 4.1.9.3	The number of RAB related to the Iu release request for CS domain

- This measurement provides the number of RAB related to the Iu release request for CS domain. The measurement is split into subcounters per traffic class.

- CC

- On transmission by the RNC of a RANAP IU RELEASE REQUEST message for CS domain, each RAB related to the RANAP IU RELEASE REQUEST message is added to the relevant measurement according to the traffic class requested when the RANAP message IU RELEASE REQUEST is sent to the CS CN.

- Each measurement is an integer value

- RAB.NbrIuRelReqCS.Conv
RAB.NbrIuRelReqCS.Strm
RAB.NbrIuRelReqCS.Intact
RAB.NbrIuRelReqCS.Bgrd

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched traffic

- UMTS

#### 4.1.9.4	The number of RAB related to the Iu release request for PS domain

- This measurement provides the number of RAB related to the Iu release request for PS domain. The measurement is split into subcounters per traffic class.

- CC

- On transmission by the RNC of a RANAP IU RELEASE REQUEST message for PS domain, each RAB related to the RANAP IU RELEASE REQUEST message is added to the relevant measurement according to the traffic class requested when the RANAP message IU RELEASE REQUEST is sent to the PS CN.

- Each measurement is an integer value.

- RAB.NbrIuRelReqPS.Conv
RAB.NbrIuRelReqPS.Strm
RAB.NbrIuRelReqPS.Intact
RAB.NbrIuRelReqPS.Bgrd

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS

## 4.2	Void

## 4.3	Signalling connection establishment

### 4.3.1	Attempted signalling connection establishments for CS domain

- This measurement provides the number of attempts by RNC to establish an Iu control plane connection between the RNC and a CS CN.

NOTE:	There is no confirmation in response to this message to indicate that the CN-RNC connection was successfully setup.

- CC

- Transmission of a RANAP Initial UE message by the RNC to the CN. This is sent by the RNC on receipt of an RRC Initial Direct Transfer message from the UE.

- A single integer value

- SIG.AttConnEstabCS.

- RncFunction

- Valid for circuit switching.

- UMTS

### 4.3.2	Attempted signalling connection establishments for PS domain

- This measurement provides the number of requests by RNC to establish an Iu control plane connection between the RNC and a PS CN.

NOTE:	There is no confirmation in response to this message to indicate that the CN-RNC connection was successfully setup.

- CC

- Transmission of a RANAP Initial UE message by the RNC to the CN. This is sent by the RNC on receipt of an RRC Initial Direct Transfer message from the UE.

- A single integer value

- SIG.AttConnEstabPS.

- RncFunction

- Valid for packet switching.

- UMTS

## 4.4	RRC connection establishment

### 4.4.1	RRC connection establishments

The three measurement types defined in the clause 4.4.1.n are subject to the "2 out of 3 approach".

#### 4.4.1.1	Attempted RRC connection establishments

- This measurement provides the number of RRC connection establishment attempts for each establishment cause.

- CC

- Receipt of an RRC Connection Request message by the RNC from the UE. Each RRC Connection Request message received is added to the relevant per cause measurement. The possible causes are included in TS 25.331 [4]. The sum of all supported per cause measurements shall equal the total number of RRC Connection Establishment attempts. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RRC.AttConnEstab.Cause
where Cause identifies the Establishment Cause.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

#### 4.4.1.2	Failed RRC connection establishments

- This measurement provides the number of RRC establishment failures for each rejection cause.

- CC

- Transmission of an RRC Connection Reject message by the RNC to the UE or an expected RRC CONNECTION SETUP COMPLETE message not received by the RNC. Each RRC Connection Reject message received is added to the relevant per cause measurement. The possible causes are included in TS 25.331 [4].
Each expected RRC CONNECTION SETUP COMPLETE not received by the RNC is added to the measurement cause 'No Reply' (not specified in TS 25.331 [4]).
The sum of all supported per cause measurements shall equal the total number of RRC Connection Establishment Failures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RRC.FailConnEstab.Cause
where Cause identifies the Rejection Cause.
The cause 'No Reply' is identified by the .NoReply suffix.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

#### 4.4.1.3	Successful RRC connection establishments

- This measurement provides the number of successful RRC establishments for each establishment cause.

- CC

- Receipt by the RNC of a RRC CONNECTION SETUP COMPLETE message following a RRC establishment attempt. Each RRC Connection Setup Complete message received is added to the relevant per cause measurement. The possible causes are included in TS 25.331 [4]. The sum of all supported per cause measurements shall equal the total number of RRC Connection Establishments. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RRC.SuccConnEstab.Cause
where Cause identifies the Establishment Cause.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.4.2	RRC connection establishment setup time

#### 4.4.2.1	RRC connection set-up time (Mean)

- This measurement provides the mean time per establishment cause it takes for the RNC to establish a RRC connection during each granularity period. The measurement is split into subcounters per establishment cause.

- DER (n=1)

- This measurement is obtained by accumulating the time intervals for every successful RRC connection establishment per establishment cause between the receipt by the RNC from the UE of a "RRC CONNECTION REQUEST" and the corresponding "RRC CONNECTION SETUP COMPLETE" message over a granularity period using DER.  The end value of this time will then be divided by the number of successful RRC connections observed in the granularity period to give the arithmetic mean, the accumulator shall be reinitialised at the beginning of each granularity period.  The measurement is split into subcounters per establishment cause, see TS 25.331 [4].

- Figure

- Each measurement is an integer value.(in milliseconds)

- RRC.AttConnEstabTimeMean.Cause
where Cause identifies the Establishment Cause.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

#### 4.4.2.2	RRC connection set-up time (Max)

- This measurement provides the maximum time per establishment cause it takes for the RNC in order to establish a RRC connection during each granularity period. The measurement is split into subcounters per establishment cause.

- GAUGE

- This measurement is obtained by monitoring the time intervals for each successful RRC connection establishment per establishment cause between the receipt by the RNC from the UE of a "RRC CONNECTION REQUEST" and the corresponding "RRC CONNECTION SETUP COMPLETE" message, see TS 25.331 [4]. The high tide mark of this time will be stored in a gauge, the gauge shall be reinitialised at the beginning of each granularity period.  The measurement is split into subcounters per establishment cause.

- Figure

- Each measurement is an integer value.(in milliseconds)

- RRC.AttConnEstabTimeMax.Cause
where Cause identifies the Establishment Cause.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.4.3	RRC connection usage

#### 4.4.3.1		General

The amount of RRC connections can be used to indicate user loading levels.

#### 4.4.3.2	RRC connection usage (Mean)

- This measurement provides the average number of simultaneous RRC connections.

- SI

- This measurement is obtained by sampling at a pre-defined interval, the amount of successful RRC connections for each UtranCell and then taking the arithmetic mean.

- A single integer value

- RRC.MeanConn

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

#### 4.4.3.3	RRC connection usage (Maximum)

- This measurement provides the maximum number of simultaneous RRC connections.

- SI

- This measurement is obtained by sampling at a pre-defined interval, the amount of successful RRC connections for each UtranCell and then taking the maximum.

- A single integer value

- RRC.MaxConn.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

## 4.5	RRC connection re-establishment

The three measurement types defined in the subclause 4.5.n are subject to the "2 out of 3 approach".

### 4.5.1	Attempted RRC re-establishments

- This measurement provides the number of RRC re-establishments attempts.

- CC

- Receipt by the RNC of a CELL UPDATE message using the Cell Update cause "Radio link failure". See TS 25.331 [4].

- A single integer value

- RRC.AttConnReEstab.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.5.2	Failed RRC re-establishments

- This measurement provides the number of RRC re-establishment failures.

- CC

- Transmission of an RRC Connection Release message by RNC to the UE or an expected UTRAN Mobility Information Confirm message not received by RNC from the UE. See TS 25.331 [4].
Each RRC Connection Release message received is added to the relevant per cause measurement. The possible causes are included in TS 25.331 [4].
Each expected UTRAN Mobility Information Confirm message not received by the RNC is added to the measurement cause 'No Reply' (not specified in TS 25.331 [4]).
The sum of all supported per cause measurements shall equal the total number of RRC re-establishment failures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RRC.FailConnReEstab.Cause
where Cause identifies the Failure Cause.
The cause 'No Reply' is identified by the .NoReply suffix.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.5.3	Successful RRC re-establishments

- This measurement provides the number of successful RRC re-establishments.

- CC

- Receipt by the RNC of a UTRAN MOBILITY INFORMATION CONFIRM in a CELL UPDATE procedure using the value cause "Radio link failure". See TS 25.331 [4].

- A single integer value

- RRC.SuccConnReEstab.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

## 4.6	RRC connection release

### 4.6.1	Attempted RRC connection releases on DCCH

- This measurement provides the number of RRC connection release attempts per release cause sent from UTRAN to the UE on the DCCH.

- CC

- Transmission of an RRC CONNECTION RELEASE message by the RNC to the UE on DCCH. Each RRC Connection Release message sent on DCCH is added to the relevant per cause measurement. The possible causes are included in TS 25.331 [4]. The sum of all supported per cause measurements shall equal the total number of RRC Connection Release attempts on DCCH. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RRC.AttConnRelDCCH.Cause
where Cause identifies the Release Cause.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.6.2	Attempted RRC connection releases on CCCH

- This measurement provides the number of RRC connection release attempts per release cause sent from UTRAN to the UE on the CCCH.

- CC

- Transmission by the RNC of an RRC CONNECTION RELEASE message to the UE on CCCH. Each RRC Connection Release message sent on CCCH is added to the relevant per cause measurement. The possible causes are included in TS 25.331 [4]. The sum of all supported per cause measurements shall equal the total number of RRC Connection Release attempts on CCCH. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RRC.AttConnRelCCCH.Cause
where Cause identifies the Release Cause.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

## 4.7	RLC connection

### 4.7.1	Number of RLC blocks sent (per Mode)

- This measurement provides the number of RLC blocks sent by the RNC including retransmitted blocks.

- CC

- Transmission of RLC block, see TS 25.322 [10].

- RLC.NbrBlocksSent.TM
RLC.NbrBlocksSent.UM
RLC.NbrBlocksSent.AM

- A single integer value

- RncFunction, per Mode (Transparent, Unacknowledged and Acknowledged).

- Valid for packet switching and circuit switching.

- UMTS

### 4.7.2	Number of RLC blocks Received (per Mode)

- This measurement provides the number of received  RLC blocks by the RNC.

- CC

- Receipt of a RLC blocks from a peer entity and before any error checking, see TS 25.322 [10].

- RLC.NbrBlocksReceived.TM
RLC.NbrBlocksReceived.UM
RLC.NbrBlocksReceived.AM

- A single integer value

- RncFunction per Mode (Transparent, Unacknowledged and Acknowledged).

- Valid for packet switching and circuit switching.

- UMTS

### 4.7.3	Discarded RLC blocks by RNC

- This measurement provides the number of discarded RLC blocks in case of error detection in the RNC (uplink transmission, RNC).

- CC

- Discard of a received block in the RNC, see TS 25.322 [10].

- RLC.DiscardedBlocksByRNC.

- A single integer value

- RncFunction

- Valid for packet switching.

- UMTS

### 4.7.4	Number of Retransmitted RLC blocks in Acknowledge Mode

- This measurement provides the number of retransmitted RLC blocks in  RLC acknowledge mode, detected in the UE and signalled to the RNC (downlink transmission, UE).

- CC

- Receipt of a NACK or SACK block from the peer entity (UE) , see TS 25.322 [10].

- RLC.RetransmittedBlocksToUE.

- A single integer value

- RncFunction

- Valid for packet switching.

- UMTS

## 4.8	Soft handover

### 4.8.1	Radio link additions to active link set (UE side)

The three measurement types defined in the subclause 4.8.1.n for the radio link additions to active link set (UE side) are subject to the "2 out of 3 approach".

#### 4.8.1.1	Attempted radio link additions to active link set (UE side)

- This measurement provides the number of attempted radio link additions during active link set update procedure (UE side) for each cell. This measurement shall be increased for each attempted radio link addition (UE side). This measurement is only valid for FDD mode.

- CC

- Transmission of an ACTIVE SET UPDATE message (RRC) by the serving RNC to the UE. Within an ACTIVE SET UPDATE message more than one radio link can be added. Each existing radio link addition information element shall be considered separately (see TS 25.331 [4]).

- A single integer value

- SHO.AttRLAddUESide.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

#### 4.8.1.2	Successful radio link additions to active link set (UE side)

- This measurement provides the number of successful radio link additions during active link set update procedure (UE side) for each cell. This measurement shall be increased for each successful radio link addition (UE side). This measurement is only valid for FDD mode.

- CC

- Receipt of an ACTIVE SET UPDATE COMPLETE message (RRC), sent by the UE to the SERVING RNC, in response to an ACTIVE SET UPDATE message with one or more existing radio link addition information element. One ACTIVE SET UPDATE COMPLETE message can be related to more than one added radio link. Each successful added radio link shall be considered separately (see TS 25.331 [4]).

- A single integer value

- SHO.SuccRLAddUESide.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

#### 4.8.1.3	Failed radio link additions to active link set (UE side)

- This measurement provides the number of failed radio link additions during active link set Update procedure (UE side) for each cell per cause. For each failure cause a separate subcounter is defined. Every failed radio link addition (UE side) shall be considered separately. This measurement is only valid for FDD mode.

- CC

- Receipt of an ACTIVE SET UPDATE FAILURE message (RRC) sent by UE to the UTRAN in response to an ACTIVE SET UPDATE message with non-empty radio link addition information element or an expected ACTIVE SET UPDATE COMPLETE message not received by the RNC. Each message can be related to more than one radio link.

-	Each failed attempt to add a radio link shall be considered separately and added to the relevant per cause measurement. Failure causes are defined within TS 25.331 [4].

-	Each expected ACTIVE SET UPDATE COMPLETE message not received by the RNC is added to the measurement cause 'No Reply' (not specified in TS 25.331 [4]).

-	The sum of all supported per cause measurements shall equal the total number of failures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form SHO.FailRLAddUESide.Cause
where Cause identifies the failure cause.
The cause 'No Reply' is identified by the .NoReply suffix.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.8.2	Radio link deletions from active link set (UE side)

#### 4.8.2.1	Attempted radio link deletions from active link set (UE side)

- This measurement provides the number of attempted radio link deletions during active link set update procedure (UE side) for each cell. This measurement shall be increased for each attempted radio link deletion (UE side). This measurement is only valid for FDD mode.

- CC

- Transmission of an ACTIVE SET UPDATE message (RRC) by the SERVING RNC to the UE. Within an ACTIVE SET UPDATE message more than one radio link can be removed. Each existing radio link removal information element shall be considered separately (see TS 25.331 [4]).

- A single integer value

- SHO.AttRLDelUESide.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

#### 4.8.2.2	Successful radio link deletions from active link set (UE side)

- This measurement provides the number of successful radio link deletions during active link set update procedure (UE side) for each cell. This measurement shall be increased for each successful radio link deletion (UE side). This measurement is only valid for FDD mode.

- CC

- Receipt of an ACTIVE SET UPDATE COMPLETE message (RRC) sent by UE to the Serving RNC in response to an ACTIVE SET UPDATE message with one or more existing radio link removal information element. One ACTIVE SET UPDATE COMPLETE message can be related to more than one deleted radio link. Each successful deleted radio link shall be considered separately (see TS 25.331 [4]).

- A single integer value

- SHO.SuccRLDelUESide.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.8.3	Measurements related to the soft handover radio link

The following measurements are provided at the best UTRAN cell of the active set (UE side).

The "best" FDD cell has the largest value when the measurement quantity is "Ec/No" or "RSCP". On the other hand, the "best" cell has the smallest value when the measurement quantity is "Pathloss". See TS 25.331 [4].

The other UTRAN cell is other than the best UTRAN cell of active set (UE side).

#### 4.8.3.1	Mean number of the radio link established by the best UTRAN cell

- This measurement provides the mean number of the radio link established by the measured UTRAN cell that is the best UTRAN cell of active set (UE side).

- SI.

- This measurement is obtained by RNC sampling at a pre-defined interval the number of the radio link established by the best UTRAN cell of active set (UE side), and then taking the arithmetic mean.

- A single integer value

- SHO.MeanNbrRLEstab.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

#### 4.8.3.2	Mean number of the radio link established by other UTRAN cell

- This measurement provides the mean number of the radio link established by other UTRAN cell that is other than the best UTRAN cell of active set (UE side), also the best UTRAN cell of active set (UE side) is measured UTRAN cell.

- SI.

- This measurement is obtained by RNC sampling at a pre-defined interval the number of the radio link established by the other UTRAN cell that is other than the best UTRAN cell of active set (UE side), and then taking the arithmetic mean.

- A single integer value

- SHO.MeanNbrRLEstabByOthCell.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

## 4.9	Radio link management procedures

### 4.9.1	Overview

#### 4.9.1.1	Considered radio link management procedures

Performance Measurement definitions in subclause 4.9 are based on the TS 25.423 [6] and TS 25.433 [7].

The following paragraphs are of interest for this purpose:

- Radio Link Setup (NBAP);

- Radio Link Addition (NBAP);

- Radio Link Deletion (NBAP);

- Radio Link Setup (RNSAP);

- Radio Link Addition (RNSAP);

- Radio Link Deletion (RNSAP);

- RADIO LINK SETUP REQUEST (NBAP);

- RADIO LINK SETUP RESPONSE (NBAP);

- RADIO LINK SETUP FAILURE (NBAP);

- RADIO LINK ADDITION REQUEST (NBAP);

- RADIO LINK ADDITION RESPONSE (NBAP);

- RADIO LINK ADDITION FAILURE (NBAP);

- RADIO LINK DELETION REQUEST (NBAP);

- RADIO LINK DELETION RESPONSE (NBAP);

- RADIO LINK SETUP REQUEST (RNSAP);

- RADIO LINK SETUP RESPONSE (RNSAP);

- RADIO LINK SETUP FAILURE (RNSAP);

- RADIO LINK ADDITION REQUEST (RNSAP);

- RADIO LINK ADDITION RESPONSE (RNSAP);

- RADIO LINK ADDITION FAILURE (RNSAP);

- RADIO LINK DELETION REQUEST (RNSAP);

- RADIO LINK DELETION RESPONSE (RNSAP).

These paragraphs show in particular the following diagrams:

- Figure: Radio Link Setup procedure on Iub, Successful Operation

- Figure: Radio Link Setup procedure on Iub, Unsuccessful Operation

- Figure: Radio Link Addition procedure on Iub, Successful Operation

- Figure: Radio Link Addition procedure on Iub, Unsuccessful Operation

- Figure: Radio Link Deletion procedure on Iub, Successful Operation

- Figure: Radio Link Setup procedure on Iur, Successful Operation

- Figure: Radio Link Setup procedure on Iur, Unsuccessful Operation

- Figure: Radio Link Addition procedure on Iur, Successful Operation

- Figure: Radio Link Addition procedure on Iur, Unsuccessful Operation

- Figure: Radio Link Deletion procedure on Iur, Successful Operation

#### 4.9.1.2	Relation between Iub measurements and Iur measurements

The following figure shows the relation between Iub interface and Iur interface. There are two cases for SRNC (CRNC1) to request radio link management activities in this figure.

In case (1), SRNC (CRNC1) sets up/adds/deletes radio links in NodeB1 and NodeB1 is directly controlled by SRNC (CRNC1).

In case (2/2bis), NodeB2 is directly controlled by DRNC (CRNC2). If SRNC (CRNC1) wants to set up/add/delete radio links in NodeB2, SRNC (CRNC1) will send request to DRNC (CRNC2), and DRNC (CRNC2) set up/add/delete radio links in NodeB2. In such case, if DRNC (CRNC2) fails to set up/add radio links in NodeB2 and receives failure message from NodeB2, DRNC (CRNC2) will send failure message back to SRNC (CRNC1). Furthermore if DRNC (CRNC2) has problem inside and fails to send request to NodeB2, it will send failure message back to the SRNC (CRNC1) directly.

- Figure: Relation between Iub interface and Iur interface

Subclause 4.9 has separated measurements for both Iur interface and Iub interface. From the above figure and description, we can see that the Iub interface measurements and Iur interface measurements overlap in some degree. Based on maintenance requirements, these two kinds measurements are needed and they are useful for operators to evaluate both the Iub interface and Iur interface, and to analyze all the failure cases they concern.

### 4.9.2	Radio link setups on Iub

The three measurement types defined in the subclauses 4.9.2.n for radio link setups on Iub are subject to the "2 out of 3 approach".

#### 4.9.2.1	Attempted radio link setups on Iub

- This measurement provides the number of attempted radio link setups on Iub for each cell. This measurement shall be increased for each attempted radio link setup on Iub. This measurement is valid for FDD and TDD mode.

- CC

- Transmission of a RADIO LINK SETUP REQUEST message (NBAP) by the controlling RNC to the NodeB. Within a RADIO LINK SETUP REQUEST message more than one radio link can be set up. Each existing radio link information element shall be considered separately (see TS 25.433 [7]).

- A single integer value

- RLM.AttRLSetupIub.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

#### 4.9.2.2	Successful radio link setups on Iub

- This measurement provides the number of successful radio link setups on Iub for each cell. This measurement shall be increased for each successful radio link setup on Iub. This measurement is valid for FDD and TDD mode.

- CC

- This measurement is based on two different events:

-	Receipt of a RADIO LINK SETUP RESPONSE message (NBAP) sent by NodeB to the controlling RNC in response to a RADIO LINK SETUP REQUEST message with one or more existing radio link information elements. One RADIO LINK SETUP RESPONSE message can be related to more than one radio link. Each radio link that is set up successfully shall be considered separately (see TS 25.433 [7]).

-	Receipt of a RADIO LINK SETUP FAILURE message (NBAP) sent by NodeB to the controlling RNC in response to a RADIO LINK SETUP REQUEST message with at least one Successful RL Information Response information element. One RADIO LINK SETUP FAILURE message can be related to more than one radio link. Each radio link that is set up successfully shall be considered separately (see TS 25.433 [7]).

- A single integer value

- RLM.SuccRLSetupIub.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

#### 4.9.2.3	Failed radio link setups on Iub

- This measurement provides the number of failed radio link setups on Iub for each cell. This measurement shall be increased for each failed radio link setup on Iub. For each failure cause a separate measurement is defined. Every failed radio link setup on Iub shall be considered separately. This measurement is valid for FDD and TDD mode.

- CC

- Receipt of a RADIO LINK SETUP FAILURE message (NBAP) sent by NodeB to the controlling RNC in response to a RADIO LINK SETUP REQUEST message with one or more existing radio link information elements. One RADIO LINK SETUP FAILURE message can be related to more than one radio link. Each failed attempt to set up a radio link shall be considered separately. Failure causes are defined within TS 25.433 [7].

Each expected RADIO LINK SETUP RESPONSE or RADIO LINK SETUP FAILURE not received by the controlling RNC is added to the measurement cause 'No Reply' (not specified in TS 25.433 [7]).

The sum of all supported per cause measurements shall equal the total number of failures. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RLM.FailRLSetupIub.Cause
where Cause identifies the failure cause.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.9.3	Radio link setups on Iur

The three measurement types defined in the subclauses 4.9.3.n for radio link setups on Iur are subject to the "2 out of 3 approach".

#### 4.9.3.1	Attempted radio link setups on Iur

- This measurement provides the number of attempted radio link setups on Iur for each cell. This measurement shall be increased for each attempted radio link setup on Iur. This measurement is valid only for FDD mode.

- CC

- Receipt of a RADIO LINK SETUP REQUEST message (RNSAP) sent by the serving RNC to the drift RNC. Within a RADIO LINK SETUP REQUEST message more than one radio link can be set up. Each existing radio link information element shall be considered separately (see TS 25.423 [6]).

- A single integer value

- RLM.AttRLSetupIur.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

#### 4.9.3.2	Successful radio link setups on Iur

- This measurement provides the number of successful radio link setups on Iur for each cell. This measurement shall be increased for each successful radio link setup on Iur. This measurement is valid only for FDD mode.

- CC

- This measurement is based on two different events:

-	Transmission of a RADIO LINK SETUP RESPONSE message (RNSAP) by the drift RNC to the serving RNC in response to a RADIO LINK SETUP REQUEST message with one or more existing radio link information elements. One RADIO LINK SETUP RESPONSE message can be related to more than one radio link. Each radio link that is set up successfully shall be considered separately (see TS 25.423 [6]).

-	Transmission of a RADIO LINK SETUP FAILURE message (RNSAP) by the drift RNC to the serving RNC in response to a RADIO LINK SETUP REQUEST message with at least one Successful RL Information Response information element. One RADIO LINK SETUP FAILURE message can be related to more than one radio link. Each radio link that is set up successfully shall be considered separately (see TS 25.423 [6]).

- A single integer value

- RLM.SuccRLSetupIur.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

#### 4.9.3.3	Failed radio link setups on Iur

- This measurement provides the number of failed radio link setups on Iur for each cell. This measurement shall be increased for each failed radio link setup on Iur. For each failure cause a separate measurement is defined. Every failed radio link setup on Iur shall be considered separately. This measurement is valid only for FDD mode.

- CC

- Transmission of a RADIO LINK SETUP FAILURE message (RNSAP) by the drift RNC to the serving RNC in response to a RADIO LINK SETUP REQUEST message with one or more existing radio link information elements. One RADIO LINK SETUP FAILURE message can be related to more than one radio link. Each failed attempt to set up a radio link shall be considered separately. Failure causes are defined within TS 25.423 [6].

The sum of all supported per cause measurements shall equal the total number of failures. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RLM.FailRLSetupIur.Cause
where Cause identifies the failure cause.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.9.4	Radio link additions on Iub

The three measurement types defined in the subclauses 4.9.4.n for radio link additions on Iub are subject to the "2 out of 3 approach".

#### 4.9.4.1	Attempted radio link additions on Iub

- This measurement provides the number of attempted radio link additions on Iub for each cell. This measurement shall be increased for each attempted radio link addition on Iub. This measurement is valid for FDD and TDD mode.

- CC

- Transmission of a RADIO LINK ADDITION REQUEST message (NBAP) by the controlling RNC to the NodeB. Within a RADIO LINK ADDITION REQUEST message more than one radio link can be added. Each existing radio link information element shall be considered separately (see TS 25.433 [7]).

- A single integer value

- RLM.AttRLAddIub.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

#### 4.9.4.2	Successful radio link additions on Iub

- This measurement provides the number of successful radio link additions on Iub for each cell. This measurement shall be increased for each successful radio link addition on Iub. This measurement is valid for FDD and TDD mode.

- CC

- This measurement is based on two different events:

-	Receipt of a RADIO LINK ADDITION RESPONSE message (NBAP) sent by NodeB to the controlling RNC in response to a RADIO LINK ADDITION REQUEST message with one or more existing radio link information elements. One RADIO LINK ADDITION RESPONSE message can be related to more than one added radio link. Each successful added radio link shall be considered separately (see TS 25.433 [7]).

-	Receipt of a RADIO LINK ADDITION FAILURE message (NBAP) sent by NodeB to the controlling RNC in response to a RADIO LINK ADDITION REQUEST message with at least one Successful RL Information Response information element. One RADIO LINK ADDITION FAILURE message can be related to more than one radio link. Each successful added radio link shall be considered separately (see TS 25.433 [7]).

- A single integer value

- RLM.SuccRLAddIub.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

#### 4.9.4.3	Failed radio link additions on Iub

- This measurement provides the number of failed radio link additions on Iub for each cell. This measurement shall be increased for each failed radio link addition on Iub. For each failure cause a separate measurement is defined. Every failed radio link addition on Iub shall be considered separately. This measurement is valid for FDD and TDD mode.

- CC

- Receipt of a RADIO LINK ADDITION FAILURE message (NBAP) sent by NodeB to the controlling RNC in response to a RADIO LINK ADDITION REQUEST message with one or more existing radio link information elements. One RADIO LINK ADDITION FAILURE message can be related to more than one radio link. Each failed attempt to add a radio link shall be considered separately. Failure causes are defined within TS 25.433 [7].

Each expected RADIO LINK ADDITION RESPONSE or RADIO LINK ADDITION FAILURE not received by the controlling RNC is added to the measurement cause 'No Reply' (not specified in TS 25.433 [7]).

The sum of all supported per cause measurements shall equal the total number of failures. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RLM.FailRLAddIub.Cause
where Cause identifies the failure cause.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.9.5	Radio link additions on Iur

The three measurement types defined in the subclauses 4.9.5.n for radio link additions on Iur are subject to the "2 out of 3 approach".

#### 4.9.5.1	Attempted radio link additions on Iur

- This measurement provides the number of attempted radio link additions on Iur for each cell. This measurement shall be increased for each attempted radio link addition on Iur. This measurement is valid only for FDD mode.

- CC

- Receipt of a RADIO LINK ADDITION REQUEST message (RNSAP) sent by the serving RNC to the drift RNC. Within a RADIO LINK ADDITION REQUEST message more than one radio link can be added. Each existing radio link information element shall be considered separately (see TS 25.423 [6]).

- A single integer value

- RLM.AttRLAddIur.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

#### 4.9.5.2	Successful radio link additions on Iur

- This measurement provides the number of successful radio link additions on Iur for each cell. This measurement shall be increased for each successful radio link addition on Iur. This measurement is valid only for FDD mode.

- CC

- This measurement is based on two different events:

-	Transmission of a RADIO LINK ADDITION RESPONSE message (RNSAP) by the drift RNC to the serving RNC in response to a RADIO LINK ADDITION REQUEST message with one or more existing radio link information elements. One RADIO LINK ADDITION RESPONSE message can be related to more than one added radio link. Each successful added radio link shall be considered separately (see TS 25.423 [6]).

-	Transmission of a RADIO LINK ADDITION FAILURE message (RNSAP) by the drift RNC to the serving RNC in response to a RADIO LINK ADDITION REQUEST message with at least one Successful RL Information Response information element. One RADIO LINK ADDITION FAILURE message can be related to more than one radio link. Each successful added radio link shall be considered separately (see TS 25.423 [6]).

- A single integer value

- RLM.SuccRLAddIur.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

#### 4.9.5.3	Failed radio link additions on Iur

- This measurement provides the number of failed radio link additions on Iur for each cell. This measurement shall be increased for each failed radio link addition on Iur. For each failure cause a separate measurement is defined. Every failed radio link addition shall be considered separately. This measurement is valid only for FDD mode.

- CC

- Transmission of a RADIO LINK ADDITION FAILURE message (RNSAP) by the drift RNC to the serving RNC in response to a RADIO LINK ADDITION REQUEST message with one or more existing radio link information elements. One RADIO LINK ADDITION FAILURE message can be related to more than one radio link. Each failed attempt to add a radio link shall be considered separately. Failure causes are defined within TS 25.423 [6].

The sum of all supported per cause measurements shall equal the total number of Failures. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RLM.FailRLAddIur.Cause
where Cause identifies the failure cause.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.9.6	Radio link deletions on Iub

#### 4.9.6.1	Attempted radio link deletions on Iub

- This measurement provides the number of attempted radio link deletions on Iub for each cell. This measurement shall be increased for each attempted radio link deletion on Iub. This measurement is valid for FDD and TDD mode.

- CC

- Transmission of a RADIO LINK DELETION REQUEST message (NBAP) by the controlling RNC to the NodeB. Within a RADIO LINK DELETION REQUEST message more than one radio link can be removed. Each existing radio link information element shall be considered separately (see TS 25.433 [7]).

- A single integer value

- RLM.AttRLDelIub.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

#### 4.9.6.2	Successful radio link deletions on Iub

- This measurement provides the number of successful radio link deletions on Iub for each cell. This measurement shall be increased for each successful radio link deletion on Iub. This measurement is valid for FDD and TDD mode.

- CC

- Receipt of a RADIO LINK DELETION RESPONSE message (NBAP) sent by NodeB to the controlling RNC in response to a RADIO LINK DELETION REQUEST message with one or more existing radio link removal information element. One RADIO LINK DELETION RESPONSE message can be related to more than one deleted radio link. Each successful deleted radio link shall be considered separately (see TS 25.433 [7]).

- A single integer value

- RLM.SuccRLDelIub.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.9.7	Radio link deletions on Iur

#### 4.9.7.1	Attempted radio link deletions on Iur

- This measurement provides the number of attempted radio link deletions on Iur for each cell. This measurement shall be increased for each attempted radio link deletion on Iur. This measurement is valid only for FDD mode.

- CC

- Receipt of a RADIO LINK DELETION REQUEST message (RNSAP) sent by the serving RNC to the drift RNC. Within a RADIO LINK DELETION REQUEST message more than one radio link can be removed. Each existing radio link information element shall be considered separately (see TS 25.423 [6]).

- A single integer value

- RLM.AttRLDelIur.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

#### 4.9.7.2	Successful radio link deletions on Iur

- This measurement provides the number of successful radio link deletions on Iur for each cell. This measurement shall be increased for each successful radio link deletion on Iur. This measurement is valid only for FDD mode.

- CC

- Transmission of a RADIO LINK DELETION RESPONSE message (RNSAP) by the drift RNC to the serving RNC in response to a RADIO LINK DELETION REQUEST message with one or more existing radio link removal information element. One RADIO LINK DELETION RESPONSE message can be related to more than one deleted radio link. Each successful deleted radio link shall be considered separately (see TS 25.423 [6]).

- A single integer value

- RLM.SuccRLDelIur.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

## 4.10	Hard handover

### 4.10.1	Void

### 4.10.2	Outgoing intra-NodeB hard handovers

The three measurement types defined in the subclause 4.10.2 for outgoing intra-NodeB hard handovers are subject to the "2 out of 3 approach".

#### 4.10.2.1	Attempted outgoing intra-NodeB hard handovers

- This measurement provides the number of attempted outgoing intra-NodeB hard handovers.

- CC.

- Transmission of a RRC message PHYSICAL CHANNEL RECONFIGURATION, RADIO BEARER SETUP, RADIO BEARER RECONFIGURATION, RADIO BEARER RELEASE, or TRANSPORT CHANNEL RECONFIGURATION from the source RNC to the UE, indicating the attempt of an outgoing intra-NodeB hard handover (see TS 25.331 [4]).

- A single integer value.

- HHO.AttOutIntraNodeB.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic.

- UMTS.

#### 4.10.2.2	Successful outgoing intra-NodeB hard handovers

- This measurement provides the number of successful outgoing intra-NodeB hard handovers.

- CC.

- Receipt of a RRC message PHYSICAL CHANNEL RECONFIGURATION COMPLETE, RADIO BEARER SETUP COMPLETE, RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER RELEASE COMPLETE, or TRANSPORT CHANNEL RECONFIGURATION COMPLETE sent from the UE to the source RNC, indicating a successful outgoing intra-NodeB hard handover (see TS 25.331 [4]).

- A single integer value.

- HHO.SuccOutIntraNodeB.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic.

- UMTS.

#### 4.10.2.3	Failed outgoing intra-NodeB hard handovers

- This measurement provides the number of failed outgoing intra-NodeB hard handovers per cause.

- CC.

- Receipt of a RRC message PHYSICAL CHANNEL RECONFIGURATION FAILURE, RADIO BEARER SETUP FAILURE, RADIO BEARER RECONFIGURATION FAILURE, RADIO BEARER RELEASE FAILURE, or TRANSPORT CHANNEL RECONFIGURATION FAILURE sent from the UE to the source RNC, indicating a failed outgoing intra-NodeB hard handover. Failure causes are defined within TS 25.331 [4].

Each expected RRC message PHYSICAL CHANNEL RECONFIGURATION COMPLETE, RADIO BEARER SETUP COMPLETE, RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER RELEASE COMPLETE, TRANSPORT CHANNEL RECONFIGURATION COMPLETE, PHYSICAL CHANNEL RECONFIGURATION FAILURE, RADIO BEARER SETUP FAILURE, RADIO BEARER RECONFIGURATION FAILURE, RADIO BEARER RELEASE FAILURE, or TRANSPORT CHANNEL RECONFIGURATION FAILURE not received by the source RNC is added to the measurement cause 'No Reply' (not specified in TS 25.331 [4]).

The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form HHO.FailOutIntraNodeB.Cause 
where Cause identifies the failure cause.
The cause 'No Reply' is identified by the .NoReply suffix.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic.

- UMTS.

### 4.10.3	Outgoing inter-NodeB, intra-RNC hard handovers

The three measurement types defined in the subclause 4.10.3 for outgoing inter-NodeB, intra-RNC hard handovers are subject to the "2 out of 3 approach".

#### 4.10.3.1	Attempted outgoing inter-NodeB, intra-RNC hard handovers

- This measurement provides the number of attempted outgoing inter-NodeB, intra-RNC hard handovers.

- CC.

- Transmission of a RRC message PHYSICAL CHANNEL RECONFIGURATION, RADIO BEARER SETUP, RADIO BEARER RECONFIGURATION, RADIO BEARER RELEASE, or TRANSPORT CHANNEL RECONFIGURATION from the source RNC to the UE, indicating the attempt of an outgoing inter-NodeB, intra-RNC hard handover (see TS 25.331 [4]).

- A single integer value.

- HHO.AttOutInterNodeBIntraRNC.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic.

- UMTS.

#### 4.10.3.2	Successful outgoing inter-NodeB, intra-RNC hard handovers

- This measurement provides the number of successful outgoing inter-NodeB, intra-RNC hard handovers.

- CC.

- Receipt of a RRC message PHYSICAL CHANNEL RECONFIGURATION COMPLETE, RADIO BEARER SETUP COMPLETE, RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER RELEASE COMPLETE, or TRANSPORT CHANNEL RECONFIGURATION COMPLETE sent from the UE to the source RNC, indicating a successful outgoing inter-NodeB, intra-RNC hard handover (see TS 25.331 [4]).

- A single integer value.

- HHO.SuccOutInterNodeBIntraRNC.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic.

- UMTS.

#### 4.10.3.3	Failed outgoing inter-NodeB, intra-RNC hard handovers

- This measurement provides the number of failed outgoing inter-NodeB, intra-RNC hard handovers per cause.

- CC.

- Receipt of a RRC message PHYSICAL CHANNEL RECONFIGURATION FAILURE, RADIO BEARER SETUP FAILURE, RADIO BEARER RECONFIGURATION FAILURE, RADIO BEARER RELEASE FAILURE, or TRANSPORT CHANNEL RECONFIGURATION FAILURE sent from the UE to the source RNC, indicating a failed outgoing inter-NodeB, intra-RNC hard handover. Failure causes are defined within TS 25.331 [4].

Each expected RRC message PHYSICAL CHANNEL RECONFIGURATION COMPLETE, RADIO BEARER SETUP COMPLETE, RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER RELEASE COMPLETE, TRANSPORT CHANNEL RECONFIGURATION COMPLETE, PHYSICAL CHANNEL RECONFIGURATION FAILURE, RADIO BEARER SETUP FAILURE, RADIO BEARER RECONFIGURATION FAILURE, RADIO BEARER RELEASE FAILURE, or TRANSPORT CHANNEL RECONFIGURATION FAILURE not received by the source RNC is added to the measurement cause 'No Reply' (not specified in TS 25.331 [4]).

The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form HHO.FailOutInterNodeBIntraRNC.Cause 
where Cause identifies the failure cause.
The cause 'No Reply' is identified by the .NoReply suffix.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic.

- UMTS.

### 4.10.4	Outgoing inter-RNC hard handovers via Iur

The three measurement types defined in the subclause 4.10.4 for outgoing inter-RNC hard handovers are subject to the "2 out of 3 approach".

#### 4.10.4.1	Attempted outgoing inter-RNC hard handovers via Iur

- This measurement provides the number of attempted outgoing inter-RNC hard handovers via Iur. 
This measurement is only valid for FDD mode.

- CC.

- Transmission of a RRC message PHYSICAL CHANNEL RECONFIGURATION, RADIO BEARER SETUP, RADIO BEARER RECONFIGURATION, RADIO BEARER RELEASE, or TRANSPORT CHANNEL RECONFIGURATION from the source RNC to the UE, indicating the attempt of an outgoing inter-RNC hard handover via Iur (see TS 25.331 [4]).

- A single integer value.

- HHO.AttOutInterRNCIur.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic.

- UMTS.

#### 4.10.4.2	Successful outgoing inter-RNC hard handovers via Iur

- This measurement provides the number of successful outgoing inter-RNC hard handovers via Iur. 
This measurement is only valid for FDD mode.

- CC.

- Receipt of a RRC message PHYSICAL CHANNEL RECONFIGURATION COMPLETE, RADIO BEARER SETUP COMPLETE, RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER RELEASE COMPLETE, or TRANSPORT CHANNEL RECONFIGURATION COMPLETE sent from the UE to the source RNC, indicating a successful outgoing inter-RNC hard handover via Iur (see TS 25.331 [4]).

- A single integer value.

- HHO.SuccOutInterRNCIur.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic.

- UMTS.

#### 4.10.4.3	Failed outgoing inter-RNC hard handovers via Iur

- This measurement provides the number of failed outgoing inter-RNC hard handovers via Iur per cause. 
This measurement is only valid for FDD mode.

- CC.

- Receipt of a RRC message PHYSICAL CHANNEL RECONFIGURATION FAILURE, RADIO BEARER SETUP FAILURE, RADIO BEARER RECONFIGURATION FAILURE, RADIO BEARER RELEASE FAILURE, or TRANSPORT CHANNEL RECONFIGURATION FAILURE sent from the UE to the source RNC, indicating a failed outgoing inter-RNC hard handover via Iur. Failure causes are defined within TS 25.331 [4].

Each expected RRC message PHYSICAL CHANNEL RECONFIGURATION COMPLETE, RADIO BEARER SETUP COMPLETE, RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER RELEASE COMPLETE, TRANSPORT CHANNEL RECONFIGURATION COMPLETE, PHYSICAL CHANNEL RECONFIGURATION FAILURE, RADIO BEARER SETUP FAILURE, RADIO BEARER RECONFIGURATION FAILURE, RADIO BEARER RELEASE FAILURE, or TRANSPORT CHANNEL RECONFIGURATION FAILURE not received by the source RNC is added to the measurement cause 'No Reply' (not specified in TS 25.331 [4]).

The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form HHO.FailOutInterRNCIur.Cause 
where Cause identifies the failure cause.
The cause 'No Reply' is identified by the .NoReply suffix.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic.

### 4.10.5	Relocation preparation for outgoing inter-RNC hard handovers switching in the CN

The three measurement types defined in the subclause 4.10.5 for relocation preparation for outgoing inter-RNC hard handovers switching in the CN are subject to the "2 out of 3 approach".

#### 4.10.5.1	Attempted relocation preparation for outgoing inter-RNC hard handovers switching in the CN

- This measurement provides the number of attempted relocation preparation for outgoing inter-RNC hard handovers switching in the CN.

- CC.

- Transmission of a RANAP message RELOCATION REQUIRED from the source RNC to the CN (Source side), indicating an attempted relocation preparation of a outgoing inter-RNC hard handover switching in the CN (see TS 25.413 [5]).

- A single integer value.

- HHO.AttRelocPrepOutInterRNCCN.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic.

- UMTS.

#### 4.10.5.2	Successful relocation preparation for outgoing inter-RNC hard handovers switching in the CN

- This measurement provides the number of successful relocation for outgoing inter-RNC hard handovers switching in the CN.

- CC.

- Receipt of a RANAP message RELOCATION COMMAND sent from the CN (Source side) to the source RNC, indicating a successful relocation preparation of a outgoing inter-RNC hard handover switching in the CN (see TS 25.413 [5]).

- A single integer value.

- HHO.SuccAttRelocPrepOutInterRNCCN.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic.

- UMTS.

#### 4.10.5.3	Failed relocation preparation for outgoing inter-RNC hard handovers switching in the CN

- This measurement provides number of failed relocation for outgoing inter-RNC hard handovers switching in the CN per cause.

- CC.

- Receipt of a RANAP message RELOCATION PREPARATION FAILURE sent from the CN (Source side) to the source RNC, indicating a failed relocation preparation for outgoing inter-RNC hard handover switching in the CN. Failure causes are defined within TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form HHO.FailRelocPrepOutInterRNCCN.Cause 
where Cause identifies the name of the failure cause.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic.

- UMTS.

### 4.10.6	Outgoing inter-RNC hard handovers switching in the CN

The three measurement types defined in the subclause 4.10.6 for outgoing inter-RNC hard handovers switching in the CN are subject to the "2 out of 3 approach".

#### 4.10.6.1	Attempted outgoing inter-RNC hard handovers switching in the CN

- This measurement provides the number of attempted outgoing inter-RNC hard handovers switching in the CN related to UEs.

- CC.

- Transmission of a RRC message PHYSICAL CHANNEL RECONFIGURATION, RADIO BEARER SETUP, RADIO BEARER RECONFIGURATION, RADIO BEARER RELEASE, or TRANSPORT CHANNEL RECONFIGURATION from the source RNC to the UE, indicating the attempt of an inter-RNC hard handover switching in the CN (see TS 25.331 [4]).

- A single integer value.

- HHO.AttOutInterRNCCN.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic.

- UMTS.

#### 4.10.6.2	Successful outgoing inter-RNC hard handovers switching in the CN

- This measurement provides the number of successful outgoing inter-RNC hard handovers switching in the CN related to UEs.

- CC.

- Receipt of a RANAP message Iu RELEASE COMMAND sent from the CN (Source side) to the source RNC, indicating a successful inter-RNC hard handover switching in the CN (see TS 25.413 [5]).

- A single integer value.

- HHO.SuccOutInterRNCCN.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic.

- UMTS.

#### 4.10.6.3	Failed outgoing inter-RNC hard handovers switching in the CN

- This measurement provides the number of failed outgoing inter-RNC hard handovers switching in the CN related to UEs.

- CC.

- Receipt of a RRC message PHYSICAL CHANNEL RECONFIGURATION FAILURE, RADIO BEARER SETUP FAILURE, RADIO BEARER RECONFIGURATION FAILURE, RADIO BEARER RELEASE FAILURE, or TRANSPORT CHANNEL RECONFIGURATION FAILURE sent from the UE to the source RNC, indicating a failed inter-RNC hard handover switching in the CN. Failure causes are defined within TS 25.331 [4].

Each expected RRC message PHYSICAL CHANNEL RECONFIGURATION COMPLETE, RADIO BEARER SETUP COMPLETE, RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER RELEASE COMPLETE, TRANSPORT CHANNEL RECONFIGURATION COMPLETE, PHYSICAL CHANNEL RECONFIGURATION FAILURE, RADIO BEARER SETUP FAILURE, RADIO BEARER RECONFIGURATION FAILURE, RADIO BEARER RELEASE FAILURE, or TRANSPORT CHANNEL RECONFIGURATION FAILURE not received by the source RNC is added to the measurement cause 'No Reply' (not specified in TS 25.331 [4]).

The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form HHO.FailOutInterRNCCN.Cause
where Cause identifies the failure cause.
The cause 'No Reply' is identified by the .NoReply suffix.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic.

- UMTS.

### 4.10.7	Outgoing hard handovers per neighbour cell relation

The three measurement types defined in the subclause 4.10.7 for outgoing hard handovers per neighbour cell relation are subject to the "2 out of 3 approach".

#### 4.10.7.1	Attempted outgoing hard handovers per neighbour cell relation

- This measurement provides the number of attempted outgoing hard handovers per neighbour cell relation.

- CC.

- Transmission of a RRC message PHYSICAL CHANNEL RECONFIGURATION, RADIO BEARER SETUP, RADIO BEARER RECONFIGURATION, RADIO BEARER RELEASE, or TRANSPORT CHANNEL RECONFIGURATION from the source RNC to the UE, indicating the attempt of an outgoing hard handover (see TS 25.331 [4]).

- A single integer value.

- HHO.AttOut.

- UtranRelation.

- Valid for circuit switched and packet switched traffic.

- UMTS.

- This measurement is mainly dedicated to Network Operator Maintenance and Vendor Performance Modelling Communities.

#### 4.10.7.2	Successful outgoing hard handovers per neighbour cell relation

- This measurement provides the number of successful outgoing hard handovers per neighbour cell relation.

- CC.

- Receipt of a RRC message PHYSICAL CHANNEL RECONFIGURATION COMPLETE, RADIO BEARER SETUP COMPLETE, RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER RELEASE COMPLETE, or TRANSPORT CHANNEL RECONFIGURATION COMPLETE sent from the UE to the source RNC, indicating a successful outgoing hard handover (see TS 25.331 [4]).

- A single integer value.

- HHO.SuccOut.

- UtranRelation.

- Valid for circuit switched and packet switched traffic.

- UMTS.

- This measurement is mainly dedicated to Network Operator Maintenance and Vendor Performance Modelling Communities.

#### 4.10.7.3	Failed outgoing hard handovers per neighbour cell relation

- This measurement provides the number of failed outgoing hard handovers per neighbour cell relation per cause.

- CC.

- Receipt of a RRC message PHYSICAL CHANNEL RECONFIGURATION FAILURE, RADIO BEARER SETUP FAILURE, RADIO BEARER RECONFIGURATION FAILURE, RADIO BEARER RELEASE FAILURE, or TRANSPORT CHANNEL RECONFIGURATION FAILURE sent from the UE to the source RNC, indicating a failed outgoing hard handover. Failure causes are defined within TS 25.331 [4].

Each expected RRC message PHYSICAL CHANNEL RECONFIGURATION COMPLETE, RADIO BEARER SETUP COMPLETE, RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER RELEASE COMPLETE, TRANSPORT CHANNEL RECONFIGURATION COMPLETE, PHYSICAL CHANNEL RECONFIGURATION FAILURE, RADIO BEARER SETUP FAILURE, RADIO BEARER RECONFIGURATION FAILURE, RADIO BEARER RELEASE FAILURE, or TRANSPORT CHANNEL RECONFIGURATION FAILURE not received by the source RNC is added to the measurement cause 'No Reply' (not specified in TS 25.331 [4]).

The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form HHO.FailOut.Cause 
where Cause identifies the failure cause.
The cause 'No Reply' is identified by the .NoReply suffix.

- UtranRelation.

- Valid for circuit switched and packet switched traffic.

- UMTS.

- This measurement is mainly dedicated to Network Operator Maintenance and Vendor Performance Modelling Communities.

## 4.11	Relocation

### 4.11.1	Relocations for CS domain

#### 4.11.1.1	Relocation preparations with UE involved for CS domain

The three measurement types defined in the subclause 4.11.1.1.n for relocation preparations with UE involved for CS domain are subject to the "2 out of 3 approach".

##### 4.11.1.1.1	Attempted relocation preparations with UE involved for CS domain

- This measurement provides the number of attempted relocation preparations with UE involved for CS domain.

- CC

- Transmission of a RANAP message RELOCATION REQUIRED from the source RNC to the CS CN (Source side) with Relocation Type set to "UE involved in relocation of SRNS", indicating an attempted relocation preparation with UE involved for CS domain (see TS 25.413 [5]).

- A single integer value

- RELOC.AttPrepUEInvolCS.

- RncFunction

- Valid for circuit switched traffic

- UMTS

##### 4.11.1.1.2	Successful relocation preparations with UE involved for CS domain

- This measurement provides the number of successful relocation preparations with UE involved for CS domain.

- CC

- Receipt of a RANAP message RELOCATION COMMAND sent from the CS CN (Source side) to the source RNC, in response to a RELOCATION REQUIRED message with Relocation Type set to "UE involved in relocation of SRNS", indicating a successful relocation preparation with UE involved for CS domain (see TS 25.413 [5]).

- A single integer value

- RELOC.SuccPrepUEInvolCS.

- RncFunction

- Valid for circuit switched traffic

- UMTS

##### 4.11.1.1.3	Failed relocation preparations with UE involved for CS domain

- This measurement provides the number of failed relocation preparations with UE involved for CS domain per cause.

- CC

- Receipt of a RANAP message RELOCATION PREPARATION FAILURE sent from the CS CN (Source side) to the source RNC, in response to a RELOCATION REQUIRED message with Relocation Type set to "UE involved in relocation of SRNS", indicating a failed relocation preparation with UE involved for CS domain. Failure causes are defined within TS 25.413 [5].

Each expected RANAP message RELOCATION COMMAND or RELOCATION PREPARATION FAILURE not received by the source RNC is added to the measurement cause 'No Reply' (not specified in TS 25.413 [5]).

The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RELOC.FailPrepUEInvolCS.Cause
where Cause identifies the failure cause.

- RncFunction

- Valid for circuit switched traffic

- UMTS

#### 4.11.1.2	Relocation preparations with UE not involved for CS domain

The three measurement types defined in the subclause 4.11.1.2.n for relocation preparations with UE not involved for CS domain are subject to the "2 out of 3 approach".

##### 4.11.1.2.1	Attempted relocation preparations with UE not involved for CS domain

- This measurement provides the number of attempted relocation preparations with UE not involved for CS domain.

- CC

- Transmission of a RANAP message RELOCATION REQUIRED from the source RNC to the CS CN (Source side) with Relocation Type set to "UE not involved in relocation of SRNS", indicating an attempted relocation preparation with UE not involved for CS domain (see TS 25.413 [5]).

- A single integer value

- RELOC.AttPrepUENotInvolCS.

- RncFunction

- Valid for circuit switched traffic

- UMTS

##### 4.11.1.2.2	Successful relocation preparations with UE not involved for CS domain

- This measurement provides the number of successful relocation preparations with UE not involved for CS domain.

- CC

- Receipt of a RANAP message RELOCATION COMMAND sent from the CS CN (Source side) to the source RNC, in response to a RELOCATION REQUIRED message with Relocation Type set to "UE not involved in relocation of SRNS", indicating a successful relocation preparation with UE not involved for CS domain (see TS 25.413 [5]).

- A single integer value

- RELOC.SuccPrepUENotInvolCS.

- RncFunction

- Valid for circuit switched traffic

- UMTS

##### 4.11.1.2.3	Failed relocation preparations with UE not involved for CS domain

- This measurement provides the number of failed relocation preparations with UE not involved for CS domain per cause.

- CC

- Receipt of a RANAP message RELOCATION PREPARATION FAILURE sent from the CS CN (Source side) to the source RNC, in response to a RELOCATION REQUIRED message with Relocation Type set to "UE not involved in relocation of SRNS", indicating a failed relocation preparation with UE not involved for CS domain. Failure causes are defined within TS 25.413 [5].

Each expected RANAP message RELOCATION COMMAND or RELOCATION PREPARATION FAILURE not received by the source RNC is added to the measurement cause 'No Reply' (not specified in TS 25.413 [5]).

The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RELOC.FailPrepUENotInvolCS.Cause
where Cause identifies the failure cause.

- RncFunction

- Valid for circuit switched traffic

- UMTS

#### 4.11.1.3	Relocation resource allocations with UE involved for CS domain

The three measurement types defined in the subclause 4.11.1.3.n for relocation resource allocations with UE involved for CS domain are subject to the "2 out of 3 approach".

##### 4.11.1.3.1	Attempted relocations resource allocations with UE involved for CS domain

- This measurement provides the number of attempted relocation resource allocations with UE involved for CS domain.

- CC

- Receipt of a RANAP message RELOCATION REQUEST sent from the CS CN (Target side) to the target RNC with Relocation Type set to "UE involved in relocation of SRNS", indicating an attempted relocation resource allocation with UE involved for CS domain (see TS 25.413 [5]).

- A single integer value

- RELOC.AttResAllocUEInvolCS.

- RncFunction

- Valid for circuit switched traffic

- UMTS

##### 4.11.1.3.2	Successful relocation resource allocations with UE involved for CS domain

- This measurement provides the number of successful relocation resource allocations with UE involved for CS domain.

- CC

- Transmission of a RANAP message RELOCATION REQUEST ACKNOWLEDGE from the target RNC to the CS CN (Target side), in response to a RELOCATION REQUEST message with Relocation Type set to "UE involved in relocation of SRNS", indicating a successful relocation resource allocation with UE involved for CS domain (see TS 25.413 [5]).

- A single integer value

- RELOC.SuccResAllocUEInvolCS.

- RncFunction

- Valid for circuit switched traffic

- UMTS

##### 4.11.1.3.3	Failed relocation resource allocations with UE involved for CS domain

- This measurement provides the number of failed relocation resource allocations with UE involved for CS domain per cause.

- CC

- Transmission of a RANAP message RELOCATION FAILURE from the target RNC to the CS CN (Target side), in response to a RELOCATION REQUEST message with Relocation Type set to "UE involved in relocation of SRNS", indicating a failed relocation resource allocation with UE involved for CS domain. Failure causes are defined within TS 25.413 [5].

The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RELOC.FailResAllocUEInvolCS.Cause
where Cause identifies the failure cause.

- RncFunction

- Valid for circuit switched traffic

- UMTS

#### 4.11.1.4	Relocation resource allocations with UE not involved for CS domain

The three measurement types defined in the subclause 4.11.1.4.n for relocation resource allocations with UE not involved for CS domain are subject to the "2 out of 3 approach".

##### 4.11.1.4.1	Attempted relocations resource allocations with UE not involved for CS domain

- This measurement provides the number of attempted relocation resource allocations with UE not involved for CS domain.

- CC

- Receipt of a RANAP message RELOCATION REQUEST sent from the CS CN (Target side) to the target RNC with Relocation Type set to "UE not involved in relocation of SRNS", indicating an attempted relocation resource allocation with UE not involved for CS domain (see TS 25.413 [5]).

- A single integer value

- RELOC.AttResAllocUENotInvolCS

- RncFunction

- Valid for circuit switched traffic

- UMTS

##### 4.11.1.4.2	Successful relocation resource allocations with UE not involved for CS domain

- This measurement provides the number of successful relocation resource allocations with UE not involved for CS domain.

- CC

- Transmission of a RANAP message RELOCATION REQUEST ACKNOWLEDGE from the target RNC to the CS CN (Target side), in response to a RELOCATION REQUEST message with Relocation Type set to "UE not involved in relocation of SRNS", indicating a successful relocation resource allocation with UE not involved for CS domain (see TS 25.413 [5]).

- A single integer value

- RELOC.SuccResAllocUENotInvolCS

- RncFunction

- Valid for circuit switched traffic

- UMTS

##### 4.11.1.4.3	Failed relocation resource allocations with UE not involved for CS domain

- This measurement provides the number of failed relocation resource allocations with UE not involved for CS domain per cause.

- CC

- Transmission of a RANAP message RELOCATION FAILURE from the target RNC to the CS CN (Target side), in response to a RELOCATION REQUEST message with Relocation Type set to "UE not involved in relocation of SRNS", indicating a failed relocation resource allocation with UE not involved for CS domain. Failure causes are defined within TS 25.413 [5].

The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RELOC.FailResAllocUENotInvolCS.Cause
where Cause identifies the failure cause.

- RncFunction

- Valid for circuit switched traffic

- UMTS

#### 4.11.1.5	Relocations for CS domain

##### 4.11.1.5.1	Successful relocations for CS domain

- This measurement provides the number of successful relocations for CS domain ('UE involved' and 'UE not involved' Relocations).

- CC

- Receipt of a RANAP message Iu RELEASE COMMAND sent from the CS CN (Source side) to the source RNC in response to a RELOCATION REQUIRED message, indicating a successful relocation for CS domain (see TS 25.413 [5]).

- A single integer value

- RELOC.SuccCS.

- RncFunction

- Valid for circuit switched traffic

- UMTS

### 4.11.2	Relocations for PS domain

#### 4.11.2.1	Relocation preparations with UE involved for PS domain

The three measurement types defined in the subclause 4.11.2.1.n for relocation preparations with UE involved for PS domain are subject to the "2 out of 3 approach".

##### 4.11.2.1.1	Attempted relocation preparations with UE involved for PS domain

- This measurement provides the number of attempted relocation preparations with UE involved for PS domain.

- CC

- Transmission of a RANAP message RELOCATION REQUIRED from the source RNC to the PS CN (Source side) with Relocation Type set to "UE involved in relocation of SRNS", indicating an attempted relocation preparation with UE involved for PS domain (see TS 25.413 [5]).

- A single integer value

- RELOC.AttPrepUEInvolPS.

- RncFunction

- Valid for packet switched traffic.

- UMTS

##### 4.11.2.1.2	Successful relocation preparations with UE involved for PS domain

- This measurement provides the number of successful relocation preparations with UE involved for PS domain.

- CC

- Receipt of a RANAP message RELOCATION COMMAND sent from the PS CN (Source side) to the source RNC, in response to a RELOCATION REQUIRED message with Relocation Type set to "UE involved in relocation of SRNS", indicating a successful relocation preparation with UE involved for PS domain (see TS 25.413 [5]).

- A single integer value

- RELOC.SuccPrepUEInvolPS.

- RncFunction

- Valid for packet switched traffic.

- UMTS

##### 4.11.2.1.3	Failed relocation preparations with UE involved for PS domain

- This measurement provides the number of failed relocation preparations with UE involved for PS domain per cause.

- CC

- Receipt of a RANAP message RELOCATION PREPARATION FAILURE sent from the PS CN (Source side) to the source RNC, in response to a RELOCATION REQUIRED message with Relocation Type set to "UE involved in relocation of SRNS", indicating a failed relocation preparation with UE involved for PS domain. Failure causes are defined within TS 25.413 [5].

Each expected RANAP message RELOCATION COMMAND or RELOCATION PREPARATION FAILURE not received by the source RNC is added to the measurement cause 'No Reply' (not specified in TS 25.413 [5]).

The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RELOC.FailPrepUEInvolPS.Cause
where Cause identifies the failure cause.

- RncFunction

- Valid for packet switched traffic.

- UMTS

#### 4.11.2.2	Relocation preparations with UE not involved for PS domain

The three measurement types defined in the subclause 4.11.2.2.n for relocation preparations with UE not involved for PS domain are subject to the "2 out of 3 approach".

##### 4.11.2.2.1	Attempted relocation preparations with UE not involved for PS domain

- This measurement provides the number of attempted relocation preparations with UE not involved for PS domain.

- CC

- Transmission of a RANAP message RELOCATION REQUIRED from the source RNC to the PS CN (Source side) with Relocation Type set to "UE not involved in relocation of SRNS", indicating an attempted relocation preparation with UE not involved for PS domain (see TS 25.413 [5]).

- A single integer value

- RELOC.AttPrepUENotInvolPS.

- RncFunction

- Valid for packet switched traffic.

- UMTS

##### 4.11.2.2.2	Successful relocation preparations with UE not involved for PS domain

- This measurement provides the number of successful relocation preparations with UE not involved for PS domain.

- CC

- Receipt of a RANAP message RELOCATION COMMAND sent from the PS CN (Source side) to the source RNC, in response to a RELOCATION REQUIRED message with Relocation Type set to "UE not involved in relocation of SRNS", indicating a successful relocation preparation with UE not involved for PS domain (see TS 25.413 [5]).

- A single integer value

- RELOC.SuccPrepUENotInvolPS.

- RncFunction

- Valid for packet switched traffic.

- UMTS

##### 4.11.2.2.3	Failed relocation preparations with UE not involved for PS domain

- This measurement provides the number of failed relocation preparations with UE not involved for PS domain per cause.

- CC

- Receipt of a RANAP message RELOCATION PREPARATION FAILURE sent from the PS CN (Source side) to the source RNC, in response to a RELOCATION REQUIRED message with Relocation Type set to "UE not involved in relocation of SRNS", indicating a failed relocation preparation with UE not involved for PS domain. Failure causes are defined within TS 25.413 [5].

Each expected RANAP message RELOCATION COMMAND or RELOCATION PREPARATION FAILURE not received by the source RNC is added to the measurement cause 'No Reply' (not specified in TS 25.413 [5]).

The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RELOC.FailPrepUENotInvolPS.Cause
where Cause identifies the failure cause.

- RncFunction

- Valid for packet switched traffic.

- UMTS

#### 4.11.2.3	Relocation resource allocations with UE involved for PS domain

The three measurement types defined in the subclause 4.11.2.3.n for relocation resource allocations with UE involved for PS domain are subject to the "2 out of 3 approach".

##### 4.11.2.3.1	Attempted relocations resource allocations with UE involved for PS domain

- This measurement provides the number of attempted relocation resource allocations with UE involved for PS domain.

- CC

- Receipt of a RANAP message RELOCATION REQUEST sent from the PS CN (Target side) to the target RNC with Relocation Type set to "UE involved in relocation of SRNS", indicating an attempted relocation resource allocation with UE involved for PS domain (see TS 25.413 [5]).

- A single integer value

- RELOC.AttResAllocUEInvolPS.

- RncFunction

- Valid for packet switched traffic.

- UMTS

##### 4.11.2.3.2	Successful relocation resource allocations with UE involved for PS domain

- This measurement provides the number of successful relocation resource allocations with UE involved for PS domain.

- CC

- Transmission of a RANAP message RELOCATION REQUEST ACKNOWLEDGE from the target RNC to the PS CN (Target side), in response to a RELOCATION REQUEST message with Relocation Type set to "UE involved in relocation of SRNS", indicating a successful relocation resource allocation with UE involved for PS domain (see TS 25.413 [5]).

- A single integer value

- RELOC.SuccResAllocUEInvolPS.

- RncFunction

- Valid for packet switched traffic.

- UMTS

##### 4.11.2.3.3	Failed relocation resource allocations with UE involved for PS domain

- This measurement provides the number of failed relocation resource allocations with UE involved for PS domain per cause.

- CC

- Transmission of a RANAP message RELOCATION FAILURE from the target RNC to the PS CN (Target side), in response to a RELOCATION REQUEST message with Relocation Type set to "UE involved in relocation of SRNS", indicating a failed relocation resource allocation with UE involved for PS domain. Failure causes are defined within TS 25.413 [5].

The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RELOC.FailResAllocUEInvolPS.Cause
where Cause identifies the failure cause.

- RncFunction

- Valid for packet switched traffic.

- UMTS

#### 4.11.2.4	Relocation resource allocations with UE not involved for PS domain

The three measurement types defined in the subclause 4.11.2.4.n for relocation resource allocations with UE not involved for PS domain are subject to the "2 out of 3 approach".

##### 4.11.2.4.1	Attempted relocations resource allocations with UE not involved for PS domain

- This measurement provides the number of attempted relocation resource allocations with UE not involved for PS domain.

- CC

- Receipt of a RANAP message RELOCATION REQUEST sent from the PS CN (Target side) to the target RNC with Relocation Type set to "UE not involved in relocation of SRNS", indicating an attempted relocation resource allocation with UE not involved for PS domain (see TS 25.413 [5]).

- A single integer value

- RELOC.AttResAllocUENotInvolPS.

- RncFunction

- Valid for packet switched traffic.

- UMTS

##### 4.11.2.4.2	Successful relocation resource allocations with UE not involved for PS domain

- This measurement provides the number of successful relocation resource allocations with UE not involved for PS domain.

- CC

- Transmission of a RANAP message RELOCATION REQUEST ACKNOWLEDGE from the target RNC to the PS CN (Target side), in response to a RELOCATION REQUEST message with Relocation Type set to "UE not involved in relocation of SRNS", indicating a successful relocation resource allocation with UE not involved for PS domain (see TS 25.413 [5]).

- A single integer value

- RELOC.SuccResAllocUENotInvolPS.

- RncFunction

- Valid for packet switched traffic.

- UMTS

##### 4.11.2.4.3	Failed relocation resource allocations with UE not involved for PS domain

- This measurement provides the number of failed relocation resource allocations with UE not involved for PS domain per cause.

- CC

- Transmission of a RANAP message RELOCATION FAILURE from the target RNC to the PS CN (Target side), in response to a RELOCATION REQUEST message with Relocation Type set to "UE not involved in relocation of SRNS", indicating a failed relocation resource allocation with UE not involved for PS domain. Failure causes are defined within TS 25.413 [5].

The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RELOC.FailResAllocUENotInvolPS.Cause
where Cause identifies the failure cause.

- RncFunction

- Valid for packet switched traffic.

- UMTS

#### 4.11.2.5	Relocations for PS domain

##### 4.11.2.5.1	Successful relocations for PS domain

- This measurement provides the number of successful relocations for PS domain ('UE involved' and 'UE not involved' Relocations).

- CC

- Receipt of a RANAP message Iu RELEASE COMMAND sent from the PS CN (Source side) to the source RNC in response to a RELOCATION REQUIRED message, indicating a successful relocation for PS domain (see TS 25.413 [5]).

- A single integer value

- RELOC.SuccPS.

- RncFunction

- Valid for packet switched traffic.

- UMTS

## 4.12	Circuit switched inter-RAT handover

### 4.12.1	Relocation preparation for outgoing circuit switched inter-RAT handovers

The three measurement types defined in the subclause 4.12.1.n for relocation preparation for outgoing circuit switched inter-RAT handovers are subject to the "2 out of 3 approach".

#### 4.12.1.1	Attempted relocation preparation for outgoing circuit switched inter-RAT handovers

- This measurement provides the number of attempted relocation preparations for outgoing circuit switched inter-RAT handovers per neighbour cell.

- CC

- Transmission of a RANAP message RELOCATION REQUIRED from the serving RNC to the CN, indicating an attempted relocation preparation of an outgoing inter-RAT handover (see TS 25.413 [5]).

- A single integer value

- IRATHO.AttRelocPrepOutCS.

- GsmRelation.

- Valid for circuit switched traffic

- UMTS

#### 4.12.1.2	Successful relocation preparation for outgoing circuit switched inter-RAT handovers

- This measurement provides the number of successful relocation preparations for outgoing circuit switched inter-RAT handovers per neighbour cell.

- CC

- Receipt of a RANAP message RELOCATION COMMAND sent from the CN to the serving RNC, indicating a successful relocation preparation of an inter-RAT handover (see TS 25.413 [5]).

- A single integer value

- IRATHO.SuccRelocPrepOutCS.

- GsmRelation.

- Valid for circuit switched traffic

- UMTS

#### 4.12.1.3	Failed relocation preparation for outgoing circuit switched inter-RAT handovers

- This measurement provides number of failed relocation preparations for outgoing circuit switched inter-RAT handovers per neighbour cell per cause.

- CC

- Receipt of a RANAP message RELOCATION PREPARATION FAILURE sent from the CN to the serving RNC, indicating a failed relocation preparation for outgoing inter-RAT handovers. Failure causes are defined within TS 25.413 [5].
The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IRATHO.FailRelocPrepOutCS.Cause
where Cause identifies the failure cause.

- GsmRelation.

- Valid for circuit switched traffic

- UMTS

### 4.12.2	Outgoing circuit switched inter-RAT handovers

The three measurement types defined in the subclause 4.12.2.n for outgoing circuit switched inter-RAT handovers are subject to the "2 out of 3 approach".

#### 4.12.2.1	Attempted outgoing circuit switched inter-RAT handovers

- This measurement provides the number of attempted outgoing circuit switched inter-RAT handovers per neighbour cell from UEs point of view.

- CC

- Transmission of a RRC-message HANDOVER FROM UTRAN COMMAND from serving RNC to the UE, indicating an attempted outgoing inter-RAT handover (see TS 25.331 [4]).

- A single integer value

- IRATHO.AttOutCS.

- GsmRelation.

- Valid for circuit switched traffic

- UMTS

#### 4.12.2.2	Successful outgoing circuit switched inter-RAT handovers

- This measurement provides the number of successful outgoing circuit switched inter-RAT handovers per neighbour cell from UEs point of view.

- CC

- Receipt of a RANAP message IU RELEASE COMMAND sent from the CN to the serving RNC, indicating a successful inter-RAT handover (see TS 25.413 [5]).

- A single integer value

- IRATHO.SuccOutCS.

- GsmRelation.

- Valid for circuit switched traffic

- UMTS

#### 4.12.2.3	Failed outgoing circuit switched inter-RAT handovers

- This measurement provides the number of failed outgoing circuit switched inter-RAT handovers per neighbour cell per cause from UEs point of view, where the UE returned to the original physical channel configuration.

- CC

- Receipt of a RRC message HANDOVER FROM UTRAN FAILURE sent from the UE to the serving RNC, indicating a failed inter-RAT handover. Failure causes are defined within TS 25.331 [4].
The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IRATHO.FailOutCS.Cause
where Cause identifies the failure cause.

- GsmRelation.

- Valid for circuit switched traffic

- UMTS

### 4.12.3	Incoming circuit switched inter-RAT handovers

The three measurement types defined in the subclause 4.12.3.n for incoming circuit switched inter-RAT handovers are subject to the "2 out of 3 approach".

#### 4.12.3.1	Attempted incoming circuit switched inter-RAT handovers

- This measurement provides the number of attempted incoming circuit switched inter-RAT handovers for each cell.

- CC

- Receipt of a RANAP RELOCATION REQUEST message sent from the CN to the target RNC, indicating the attempt of an inter-RAT handover (see TS 25.413 [5]).

- A single integer value

- IRATHO.AttIncCS.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcrValid for circuit switched traffic

- UMTS

#### 4.12.3.2	Successful incoming circuit switched inter-RAT handovers

- This measurement provides the number of successful incoming circuit switched interRAT handovers for each cell.

- CC

- Receipt of a RRC HANDOVER TO UTRAN COMPLETE message sent from the UE to the target RNC, indicating a successful interRAT handover (see TS 25.331 [4]).

- A single integer value

- IRATHO.SuccIncCS.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcrValid for circuit switched traffic

- UMTS

#### 4.12.3.3	Failed incoming circuit switched inter-RAT handovers

- This measurement provides the number of failed incoming circuit switched interRAT handovers per cell per cause.

- CC

- Transmission of a RANAP message RELOCATION FAILURE from the target RNC to the CN, indicating a failed inter-RAT handovers. Failure causes are defined within TS 25.413 [5].
The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IRATHO.FailIncCS.Cause 
where Cause identifies the failure cause.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcrValid for circuit switched traffic

- UMTS

### 4.13	Packet switched inter-RAT handover

### 4.13.1	Outgoing packet switched inter-RAT handovers, UTRAN controlled

The three measurement types defined in the subclause 4.13.1.n for outgoing packet switched inter-RAT handovers, UTRAN controlled are subject to the "2 out of 3 approach".

#### 4.13.1.1	Attempted outgoing packet switched inter-RAT handovers, UTRAN controlled

- This measurement provides the number of attempted outgoing, UTRAN controlled, Packet Switched interRAT handovers per cell.

- CC

- Transmission of a RRC-message, CELL CHANGE ORDER FROM UTRAN, from source RNC to the UE, indicating an attempted outgoing Packet Switched inter-RAT handover (see TS 25.331 [4]).

- A single integer value

- IRATHO.AttOutPSUTRAN.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS

#### 4.13.1.2	Successful outgoing packet switched inter-RAT handovers, UTRAN controlled

- This measurement provides the number of successful outgoing, UTRAN controlled, Packet Switched interRAT handovers per cell.

- CC

- Receipt of a RANAP message, IU RELEASE COMMAND, sent from the PS CN to the source RNC, indicating a successful outgoing Packet Switched inter-RAT handover (see TS 25.413 [5]).

- A single integer value

- IRATHO.SuccOutPSUTRAN.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS

#### 4.13.1.3	Failed outgoing packet switched inter-RAT handovers UTRAN controlled

- This measurement provides the number of failed outgoing, UTRAN controlled, Packet Switched interRAT handovers per cause, where the UE resumes the connection to UTRAN using the same resources used before receiving the cell change order. This is measured per cell.

- CC

- Receipt of an RRC message, CELL CHANGE ORDER FROM UTRAN FAILURE, sent from the UE to the source RNC, indicating a failed inter-RAT handover. Failure causes are defined within TS 25.331 [4].
The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IRATHO.FailOutPSUTRAN.Cause 
where Cause identifies the failure cause.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS

### 4.13.2	Outgoing packet switched inter-RAT handovers, UE controlled

#### 4.13.2.1	Successful outgoing packet switched inter-RAT handovers, UE controlled

- This measurement provides the number of successful outgoing, UE controlled, Packet Switched inter-RAT handovers per cell.

- CC

- Receipt of an RANAP message, SRNS CONTEXT REQUEST, sent from the PS CN to the serving RNC, indicating a successful outgoing UE controlled Packet Switched inter-RAT handover (see TS 25.413 [5]).

- A single integer value

- IRATHO.SuccOutPSUE.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS

## 4.14	Iu connection release

### 4.14.1	Overview

#### 4.14.1.1	Considered Iu connection release procedures

Performance Measurement definitions in this subclause are based on TS 25.413 [5].

The following paragraphs are of interest for this purpose:

- Iu Release Request;

- Iu Release;

- IU RELEASE REQUEST;

- IU RELEASE COMMAND;

- IU RELEASE COMPLETE.

These paragraphs show in particular the following diagrams:

- Figure: Iu Release Request procedure. Successful operation

- Figure: Iu Release procedure. Successful operation

### 4.14.2	Iu connection release request by UTRAN

#### 4.14.2.1	Attempted Iu connection release request by UTRAN for CS domain

- This measurement provides the number of attempted requests by UTRAN to release an Iu connection between the RNC and a CS CN. The measurement is split into subcounters per release cause.

- CC

- Transmission of a RANAP message IU RELEASE REQUEST by the RNC to the CS CN. Each RANAP message IU RELEASE REQUEST sent to the CS CN is added to the relevant per cause measurement. The possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of IU RELEASE REQUEST attempts. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IU.AttConnRelReqUTRANCS.Cause 
where Cause identifies the release cause.

- RncFunction

- Valid for circuit switched traffic

- UMTS

#### 4.14.2.2	Attempted Iu connection release request by UTRAN for PS domain

- This measurement provides the number of attempted requests by UTRAN to release an Iu connection between the RNC and a PS CN. The measurement is split into subcounters per release cause.

- CC

- Transmission of a RANAP message IU RELEASE REQUEST by the RNC to the PS CN. Each RANAP message IU RELEASE REQUEST sent to the PS CN is added to the relevant per cause measurement. The possible release causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of IU RELEASE REQUEST attempts. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IU.AttConnRelReqUTRANPS.Cause 
where Cause identifies the release cause.

- RncFunction

- Valid for packet switched traffic.

- UMTS

### 4.14.3	Iu connection release by CN

#### 4.14.3.1	Attempted Iu connection release by CN for CS domain

- This measurement provides the number of attempted release by a CS CN to an Iu connection between the RNC and a CS CN. The measurement is split into subcounters per release cause.

- CC

- Receipt of a RANAP message IU RELEASE COMMAND sent by the CS CN to the RNC. Each RANAP message IU RELEASE COMMAND received from the CS CN is added to the relevant per cause measurement. The possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of IU RELEASE COMMAND attempts. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IU.AttConnRelCNCS.Cause 
where Cause identifies the release cause.

- RncFunction

- Valid for circuit switching.

- UMTS

#### 4.14.3.2	Attempted Iu connection release by CN for PS domain

- This measurement provides the number of attempted release by a PS CN to an Iu connection between the RNC and a PS CN. The measurement is split into subcounters per release cause.

- CC

- Receipt of a RANAP message IU RELEASE COMMAND sent by the PS CN to the RNC. Each RANAP message IU RELEASE COMMAND received from the PS CN is added to the relevant per cause measurement. The possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of IU RELEASE COMMAND attempts. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IU.AttConnRelCNPS.Cause 
where Cause identifies the release cause.

- RncFunction

- Valid for packet switching.

- UMTS

#### 4.14.3.3	Successful Iu connection release by CN for CS domain

- This measurement provides the number of successful release by a CS CN to an Iu connection between the RNC and a CS CN. The measurement is split into subcounters per release cause.

- CC

- Transmission of a RANAP message IU RELEASE COMPLETE by the RNC to the CS CN. Each RANAP message IU RELEASE COMPLETE sent to the CS CN is added to the relevant per cause measurement. The possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of IU RELEASE COMPLETE. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IU.SuccConnRelCNCS.Cause 
where Cause identifies the release cause.

- RncFunction

- Valid for circuit switching.

- UMTS

#### 4.14.3.4	Successful Iu connection release by CN for PS domain

- This measurement provides the number of successful release by a PS CN to an Iu connection between the RNC and a PS CN. The measurement is split into subcounters per release cause.

- CC

- Transmission of a RANAP message IU RELEASE COMPLETE by the RNC to the PS CN. Each RANAP message IU RELEASE COMPLETE sent to the PS CN is added to the relevant per cause measurement. The possible causes are included in TS 25.413 [5]. The sum of all supported per cause measurements shall equal the total number of IU RELEASE COMPLETE. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IU.SuccConnRelCNPS.Cause 
where Cause identifies the release cause.

- RncFunction

- Valid for packet switching.

- UMTS

## 4.15	Intra-cell DCA

The three measurement types defined in subclause 4.15...n for Intra-cell DCA are subject to the "2 out of 3 approach".

### 4.15.1	Attempted intra-cell DCA

- This measurement provides the number of attempted intra-cell DCA per cell. This  measurement  is only valid for TDD mode.

- CC

- Transmission of a RRC message PHYSICAL CHANNEL RECONFIGURATION, RADIO BEARER SETUP, RADIO BEARER RECONFIGURATION, RADIO BEARER RELEASE, or TRANSPORT CHANNEL RECONFIGURATION from the source RNC to the UE, indicating the attempt of an intra-cell DCA (see TS 25.331 [4]).

- A single integer value

- DCA.AttIntraCell.

- UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.15.2	Successful intra-cell DCA

- This measurement provides the number of successful intra-cell DCA per cell. This  measurement  is only valid for TDD mode.

- CC

- Receipt of a RRC message PHYSICAL CHANNEL RECONFIGURATION COMPLETE, RADIO BEARER SETUP COMPLETE, RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER RELEASE COMPLETE, or TRANSPORT CHANNEL RECONFIGURATION COMPLETE sent from the UE to the source RNC, indicating a successful intra-cell DCA (see TS 25.331 [4]).

- A single integer value

- DCA.SuccIntraCell.

- UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.15.3	Failed intra-cell DCA

- This measurement provides the number of failed intra-cell DCA per cell per cause. This  measurement  is only valid for TDD mode.

- CC

- Receipt of a RRC message PHYSICAL CHANNEL RECONFIGURATION FAILURE, RADIO BEARER SETUP FAILURE, RADIO BEARER RECONFIGURATION FAILURE, RADIO BEARER RELEASE FAILURE, or TRANSPORT CHANNEL RECONFIGURATION FAILURE sent from the UE to the source RNC, indicating a failed intra-cell DCA. Failure causes are defined within TS 25.331 [4].

Each expected RRC message PHYSICAL CHANNEL RECONFIGURATION COMPLETE, RADIO BEARER SETUP COMPLETE, RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER RELEASE COMPLETE, TRANSPORT CHANNEL RECONFIGURATION COMPLETE, PHYSICAL CHANNEL RECONFIGURATION FAILURE, RADIO BEARER SETUP FAILURE, RADIO BEARER RECONFIGURATION FAILURE, RADIO BEARER RELEASE FAILURE, or TRANSPORT CHANNEL RECONFIGURATION FAILURE not received by the source RNC is added to the measurement cause 'No Reply' (not specified in TS 25.331 [4]).

The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form DCA.FailIntraCell.Cause 
where Cause identifies the failure cause.
The cause 'No Reply' is identified by the .NoReply suffix.

- UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

## 4.16	TDD Code Resources

### 4.16.1	UTRAN Cell Max Downlink Code Resources Used

- This measurement provides the number of OVSF codes used in the downlink of  the UTRAN cell. This measurement is split into subcounters according to the Orthogonal Variable Spreading Factor (OVSF) length. This  measurement  is only valid for TDD mode.

- SI.

- This measurement is obtained by sampling at a pre-defined interval, the number of OVSF codes used per Spread Factor (SF), and then taking the arithmetic maximum. The SFs used are: SF= 1, SF= 16.

- Two integer values.

- TCR.DLCodeResUsed.SF1
TCR.DLCodeResUsed.SF16

- UtranCellTDDLcr 
UtranCellTDDHcr

- UtranCell

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.16.2	UTRAN Cell Max Uplink Code Resources Used

- This measurement provides the number of OVSF codes used in the uplink of the UTRAN cell. This measurement is split into subcounters according to the Orthogonal Variable Spreading Factor (OVSF) length. This measurement is only valid for TDD mode.

- SI.

- This measurement is obtained by sampling at a pre-defined interval, the number of OVSF codes used per Spread Factor (SF), and then taking the arithmetic maximum. The SFs used are: SF= 1, SF= 2, SF= 4, SF= 8, SF= 16.

- Five integer values.

- TCR.ULCodeResUsed.SF1 
TCR.ULCodeResUsed.SF2
TCR.ULCodeResUsed.SF4
TCR.ULCodeResUsed.SF8
TCR.ULCodeResUsed.SF16.

- UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.16.3	Mean Downlink Code Resources Used of an UTRAN Cell

- This measurement provides the mean number of OVSF codes used in the downlink of  the UTRAN cell. This measurement is split into subcounters according to the Orthogonal Variable Spreading Factor (OVSF) length. This  measurement  is only valid for TDD mode.

- SI.

- This measurement is obtained by sampling at a pre-defined interval, the number of OVSF codes used per spread factor (SF), and then taking the arithmetic mean. The SFs used are: SF= 1, SF= 16.

- Two integer values.

- TCR.DLMeanCodeResUsed.SF1, 
TCR.DLMeanCodeResUsed.SF16.

- UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.16.4	Mean Uplink Code Resources Used of an UTRAN Cell

- This measurement provides the mean number of OVSF codes used in the uplink of  the UTRAN cell. This measurement is split into subcounters according to the Orthogonal Variable Spreading Factor (OVSF) length. This  measurement  is only valid for TDD mode.

- SI.

- This measurement is obtained by sampling at a pre-defined interval, the number of OVSF codes used per Spread Factor (SF), and then taking the arithmetic mean. The SFs used are: SF= 1, SF= 2, SF= 4, SF= 8, SF= 16.

- Five integer values.

- TCR.ULMeanCodeResUsed.SF1, 
TCR.ULMeanCodeResUsed.SF2,
TCR.ULMeanCodeResUsed.SF4,
TCR.ULMeanCodeResUsed.SF8,
TCR.ULMeanCodeResUsed.SF16.

- UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

## 4.17	Measurements related to TDD UTRAN cell Radio Frequency Carrier

### 4.17.1	Mean Transmitted Carrier Power of an UTRAN Cell

- This measurement provides the mean transmitted carrier power of an UTRAN cell. This measurement is only valid for TDD mode.

- SI.

- This measurement is obtained by sampling at a pre-defined interval, the transmitted carrier power of the same Node B reported, and then taking the arithmetic mean. For an Utran Cell in TDD mode, the minimum granularity for this measurement is a Time Slot (TS).

- In case a TS is configured to work for downlink, the measurement value for Transmitted Carrier Power on the basis of this TS is an integer, with a range from 0 to 100 that maps the value of the measured transmitted carrier power percentage as defined in table 9.46 of TS 25.123 [9]. When a TS is configured to work for uplink, the measurement for Transmitted Carrier Power on the basis of this TS is not valid and should have a NULL value specified in 3GPP TS 32.432 [12]. TS0 to TS6 are valid for both UtranCellTDDLcr and UtranCellTDDHcr, and TS7 to TS14 are valid only for UtranCellTDDHcr.

- CARR.MeanTSTCP.TSx, where x presents 0, 1, 2 … 14.

- UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.17.2	Maximum Transmitted Carrier Power of an UTRAN Cell

- This measurement provides the maximum transmitted carrier power of an UTRAN cell. This measurement is only valid for TDD mode.

- SI.

- This measurement is obtained by sampling at a pre-defined interval, the transmitted carrier power of the same Node B reported, and then taking the maximum. For an Utran Cell in TDD mode, the minimum granularity for this measurement is a Time slot (TS).

- In case a TS is configured to work for downlink, the measurement value for Transmitted Carrier Power on the basis of this TS is an integer, with a range from 0 to 100, that maps the value of the measured transmitted carrier power percentage as defined in table 9.46 of TS 25.123 [9]. When a TS is configured to work for uplink, the measurement for Transmitted Carrier Power on the basis of this TS is not valid and should have a NULL value specified in 3GPP TS 32.432 [12]. TS0 to TS6 are valid for both UtranCellTDDLcr and UtranCellTDDHcr, and TS7 to TS14 are valid only for UtranCellTDDHcr.

- CARR.MaxTSTCP.TSx, where x presents 0, 1, 2 … 14.

- UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.17.3	Mean Received Total Wideband Power of an UTRAN Cell

- This measurement provides the mean received total wide band power of an UTRAN cell. This measurement is only valid for TDD mode.

- SI.

- This measurement is obtained by sampling at a pre-defined interval, the received total wide band power of the same Node B reported, and then taking the arithmetic mean. For an Utran Cell in TDD mode, the minimum granularity for this measurement is a Time slot (TS).

- In case a TS is configured to work for downlink, the measurement value for Transmitted Carrier Power on the basis of this TS is an integer, with a range from 0 to 621 that maps the value of the received total wide band power percentage as defined in table 9.36 of TS 25.123 [9]. When a TS is configured to work for uplink, the measurement for Transmitted Carrier Power on the basis of this TS is not valid and should have a NULL value specified in 3GPP TS 32.432 [12]. TS0 to TS6 are valid for both UtranCellTDDLcr and UtranCellTDDHcr, and TS7 to TS14 are valid only for UtranCellTDDHcr.

- CARR.MeanTSRTWP.TSx, where x presents 0, 1, 2 … 14.

- UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.17.4	The Maximum Received Total Wideband Power of an UTRAN Cell

- This measurement provides the maximum received total wide band power of an UTRAN cell. This measurement is only valid for TDD mode.

- SI.

- This measurement is obtained by sampling at a pre-defined interval, the received total wide band power of the same Node B reported, and then taking the maximum. For an Utran Cell in TDD mode, the minimum granularity for this measurement is a Time Slot (TS).

- In case a TS is configured to work for downlink, the measurement value for Transmitted Carrier Power on the basis of this TS is an integer, with a range from 0 to 621, that maps the value of the received total wide band power percentage as defined in table 9.36 of TS 25.123 [9]. When a TS is configured to work for uplink, the measurement for Transmitted Carrier Power on the basis of this TS is not valid and should have a NULL value specified in 3GPP TS 32.432 [12]. TS0 to TS6 are valid for both UtranCellTDDLcr and UtranCellTDDHcr, and TS7 to TS14 are valid only for UtranCellTDDHcr.

- CARR.MaxTSRTWP.TSx, where x presents 0, 1, 2 … 14.

- UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.17.5	Mean DPCH Transmitted Code Power of an UTRAN Cell

a)	This measurement provides the mean of the DPCH Transmitted Code Power. This measurement is valid only for TDD mode.

b)	SI

c)	This measurement is obtained by sampling at a pre-defined interval, the DPCH  Transmitted Code Power of a given UtranCell, and then taking the arithmetic mean.

d)	A single integer value from 10 to 122, that maps the value of the measured Transmitted Code Power as defined in table 9.49 of TS 25.123 [9].

e)	CARR. MeanDPCHTx

f)	UtranCellTDDLcr 
UtranCellTDDHcr

g)	Valid for circuit switched and packet switched traffic

h)	UMTS

### 4.17.6	Maximum DPCH Transmitted Code Power of an UTRAN Cell

a)	This measurement provides the maximum of the DPCH Transmitted Code Power. This measurement is valid only for TDD mode.

b)	SI

c)	This measurement is obtained by sampling at a pre-defined interval, the DPCH Transmitted Code Power of a given UtranCell, and then taking the arithmetic maximum value.

d)	A single integer value from 10 to 122, that maps the value of the measured Transmitted Code Power as defined in table 9.49 of TS 25.123 [9].

e)	CARR. MaxDPCHTx

f)	UtranCellTDDLcr 
UtranCellTDDHcr

g)	Valid for circuit switched and packet switched traffic

h)	UMTS

### 4.17.7	Mean DPCH Received Signal Code Power of an UTRAN Cell

a)	This measurement provides the mean of the DPCH Received Signal Code Power. This measurement is valid only for TDD mode.

b)	SI

c)	This measurement is obtained by sampling at a pre-defined interval, the DPCH  Received Signal Code Power of a given UtranCell, and then taking the arithmetic mean.

d)	A single integer value from 10 to 122, that maps the value of the measured Received Signal Code Power as defined in table 9.49 of TS 25.123 [9].

e)	CARR. MeanDPCHRx

f)	UtranCellTDDLcr 
UtranCellTDDHcr

g)	Valid for circuit switched and packet switched traffic

h)	UMTS

### 4.17.8	Maximum DPCH Received Signal Code Power of an UTRAN Cell

a)	This measurement provides the maximum of the DPCH Received Signal Code Power. This measurement is valid only for TDD mode.

b)	SI

c)	This measurement is obtained by sampling at a pre-defined interval, the DPCH Received Signal Code Power of a given UtranCell, and then taking the arithmetic maximum value.

d)	A single integer value from 10 to 122, that maps the value of the measured Received Signal Code Power as defined in table 9.49 of TS 25.123 [9].

e)	CARR. MaxDPCHRx

f)	UtranCellTDDLcr 
UtranCellTDDHcr

g)	Valid for circuit switched and packet switched traffic

h)	UMTS

### 4.17.9	Mean Interference Signal Code Power of an UTRAN Cell

- This measurement provides the mean interference signal code power of an UTRAN cell in uplink. This measurement is only valid for TDD mode.

- SI

- This measurement is obtained by sampling at a pre-defined interval, the interference signal code power of the same Node B reported, and then taking the arithmetic mean. For an Utran Cell in TDD mode, the minimum granularity for this measurement is a Time Slot (TS).

- In case a TS is configured to work for uplink, the measurement value for interference signal code power on the basis of this TS is an integer value, with a range from 0 to 127 that maps the value of the interference signal code power as defined in table 9.34 of TS 25.123 [9]. When a TS is configured to work for downlink, the measurement for interference signal code power on the basis of this TS is not valid and should have a NULL value specified in 3GPP TS 32.432 [12]. TS0 to TS6 are valid for both UtranCellTDDLcr and UtranCellTDDHcr, and TS7 to TS14 are valid only for UtranCellTDDHcr.

- CARR.MeanTSISCP.TSx, where x presents 0, 1, 2 … 14.

- UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.17.10	Maximum Interference Signal Code Power of an UTRAN Cell

- This measurement provides the maximum interference signal code power of an UTRAN cell in uplink. This measurement is only valid for TDD mode.

- SI

- This measurement is obtained by sampling at a pre-defined interval, the interference signal code power of the same Node B reported, and then taking the maximum. For an Utran Cell in TDD mode, the minimum granularity for this measurement is a Time Slot (TS).

- In case a TS is configured to work for uplink, the measurement value for interference signal code power on the basis of this TS is an integer value, with a range from 0 to 127 that maps the value of the interference signal code power as defined in table 9.34 of TS 25.123 [9]. When a TS is configured to work for downlink, the measurement for interference signal code power on the basis of this TS is not valid and should have a NULL value specified in 3GPP TS 32.432 [12]. TS0 to TS6 are valid for both UtranCellTDDLcr and UtranCellTDDHcr, and TS7 to TS14 are valid only for UtranCellTDDHcr.

- CARR.MaxTSISCP.TSx, where x presents 0, 1, 2 … 14.

- UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.17.11	Mean transmitted carrier power of all codes not used for HS-PDSCH , HS-SCCH, E-AGCH or E-HICH transmission

- This measurement provides the mean transmitted non-HS carrier power of an UTRAN  cell. 
This measurement is valid only for TDD mode.

- SI

- This measurement is obtained by sampling at a pre-defined interval, the transmitted carrier power 
(extracted from NBAP common measurement report (see TS 25.225 [16], TS 25.433 [7])) 
of all codes not used for HS-PDSCH , HS-SCCH, E-AGCH or E-HICH transmission of a given UtranCell, and then taking the arithmetic mean of the measurement values obtained during the granularity period. For an UTRAN cell in TDD mode, the minimum granularity of this measurement is a Time Slot (TS).

- In case a TS is configured to work for downlink, the measurement value for transmitted carrier power on the basis of this TS is an integer value, with a range from 0 to 100 that maps the value of the transmitted carrier power as defined in table 9.51 of TS 25.123 [9]. When a TS is configured to work for uplink, the measurement for transmitted carrier power on the basis of this TS is not valid and should have a NULL value specified in 3GPP TS 32.432 [15]. TS0 to TS6 are valid for both UtranCellTDDLcr and UtranCellTDDHcr, and TS7 to TS14 are valid only for UtranCellTDDHcr.

- CARR.MeanTDDNonHSTCP.TSx, where x presents 0, 1, 2 … 14.

- UtranCellTDDLcr 
UtranCellTDDHcr

- Valid only for packet switched traffic

- UMTS

### 4.17.12	Maximum transmitted carrier power of all codes not used for HS-PDSCH , HS-SCCH, E-AGCH or E-HICH transmission

- This measurement provides the maximum transmitted non-HS carrier power of an UTRAN  cell. This measurement is valid only for TDD mode.

- SI

- This measurement is obtained by sampling at a pre-defined interval, the transmitted carrier power 
(extracted from NBAP common measurement report (see TS 25.225 [16], TS 25.433 [7])) 
of all codes not used for HS-PDSCH , HS-SCCH, E-AGCH or E-HICH transmission of a given UtranCell, and then taking the arithmetic maximum of the measurement values obtained during the granularity period. For an UTRAN cell in TDD mode, the minimum granularity of this measurement is a Time Slot (TS).

- In case a TS is configured to work for downlink, the measurement value for transmitted carrier power on the basis of this TS is an integer value, with a range from 0 to 100 that maps the value of the transmitted carrier power as defined in table 9.51 of TS 25.123 [9]. When a TS is configured to work for uplink, the measurement for transmitted carrier power on the basis of this TS is not valid and should have a NULL value specified in 3GPP TS 32.432 [15]. TS0 to TS6 are valid for both UtranCellTDDLcr and UtranCellTDDHcr, and TS7 to TS14 are valid only for UtranCellTDDHcr.

- CARR.MaxTDDNonHSTCP.TSx, where x presents 0, 1, 2 … 14.

- UtranCellTDDLcr 
UtranCellTDDHcr

- Valid only for packet switched traffic

- UMTS

## 4.18	Measurements related to the UTRAN cell FDD carrier (RF Performance metrics)

RF metrics can be used to indicate loading levels and abnormal conditions.

### 4.18.1	Mean Transmitted Carrier Power of an UTRAN Cell

- This measurement provides the average of the transmitted carrier power.  This measurement is valid only for FDD mode.

- SI.

- This measurement is obtained by sampling at a pre-defined interval, the transmitted carrier power (see TS 25.133 [8]) for the RF carrier of a given UtranCell, and then taking the arithmetic mean.

- A single integer value from 0 to 100, that maps the value of the measured transmitted carrier power percentage as defined in table 9.43 [8].

- CARR.FDDMeanTCP.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

- Network Operator's Traffic Engineering Community, Equipment Vendor's Performance Modelling Community.

### 4.18.2	Maximum Transmitted Carrier Power of an UTRAN Cell

- This measurement provides the maximum of the transmitted carrier power. This measurement is valid only for FDD mode.

- SI.

- This measurement is obtained by sampling at a pre-defined interval, the transmitted carrier power (see TS 25.133 [8]) for the RF carrier of a given UtranCell, and then taking the maximum value.

- A single integer value from 0 to 100, that maps the value of the measured transmitted carrier power percentage as defined in table 9.43 [8].

- CARR.FDDMaxTCP.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

- Network Operator's Traffic Engineering Community, Equipment Vendor's Performance Modelling Community.

### 4.18.3	Mean Received Total Wideband Power of an UTRAN Cell

- This measurement provides the average of the received total wideband power. This measurement is valid only for FDD mode.

- SI.

- This measurement is obtained by sampling at a pre-defined interval, the received total wideband power (see TS 25.133 [8]) for the RF carrier of a given UtranCell, and then taking the arithmetic mean.

- A single integer value from 0 to 621 that maps the value of the measured received total wideband power as defined In table 9.37[8].

- CARR.FDDMeanRTWP.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

- Network Operator's Traffic Engineering Community, Equipment Vendor's Performance Modelling Community.

### 4.18.4	Maximum Received Total Wideband Power of an UTRAN Cell

- This measurement provides the maximum of the received total wideband power. This measurement is valid only for FDD mode.

- SI.

- This measurement is obtained by sampling at a pre-defined interval, the received wideband power (see TS 25.133 [8]) for the RF carrier of a given UtranCell, and then taking the maximum value.

- A single integer value from 0 to 621 that maps the value of the measured received total wideband power as defined In table 9.37[8]

- CARR.FDDMaxRTWP.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

- Network Operator's Traffic Engineering Community, Equipment Vendor's Performance Modelling Community.

### 4.18.5	Mean Radio Link Transmitted Code Power of an UTRAN Cell

a)	This measurement provides the average of the Radio Link Transmitted Code Power. This measurement is valid for only FDD mode.

b)	SI

c)	This measurement is obtained by sampling at a pre-defined interval, the Radio Link Transmitted Code Power of a given UtranCell, and then taking the arithmetic mean.

d)	A single integer value from 10 to 122, that maps the value of the measured Transmitted Code Power as defined in table 9.46 of TS 25.133 [8].

e)	CARR. MeanRadioLink

f)	UtranCellFDD

g)	Valid for circuit switched and packet switched traffic

h)	UMTS

### 4.18.6	Maximum Radio Link Transmitted Code Power of an UTRAN Cell

a)	This measurement provides the maximum of the Radio Link  Transmitted Code Power. This measurement is valid only for FDD mode.

b)	SI

c)	This measurement is obtained by sampling at a pre-defined interval, the Radio Link  Transmitted Code Power of a given UtranCell, and then taking the arithmetic maximum value.

d)	A single integer value from 10 to 122, that maps the value of the measured Transmitted Code Power as defined in table 9.46 of TS 25.133 [8].

e)	CARR. MaxRadioLink

f)	UtranCellFDD

g)	Valid for circuit switched and packet switched traffic

h)	UMTS

### 4.18.7	Mean transmitted carrier power of all codes not used for HS-PDSCH,  HS-SCCH, E-AGCH, E-RGCH or E-HICH transmission of an UTRAN Cell

- This measurement provides the mean transmitted non-HS carrier power of a serving HS-DSCH cell. 
This measurement is valid only for FDD mode.

- SI.

- This measurement is obtained by sampling at a pre-defined interval, the transmitted carrier power (extracted from NBAP common measurement report (see TS 25.215 [14], TS 25.433 [7])) of all codes not used for 
HS-PDSCH, HS-SCCH, E-AGCH, E-RGCH or E-HICH transmission of a given UtranCell, and then taking the arithmetic mean of the measurement values obtained during the granularity period.

- A single integer value from 0 to 100, that maps the value of the measured transmitted carrier power to a percentage as defined in table 9.64 (see TS 25.133 [8]).

- CARR.MeanFDDNonHSTCP.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

### 4.18.8	Maximum transmitted carrier power of all codes not used for HS-PDSCH,  HS-SCCH, E-AGCH, E-RGCH or E-HICH transmission of an UTRAN Cell

- This measurement provides the maximum transmitted non-HS carrier power of a serving HS-DSCH cell. This measurement is valid only for FDD mode.

- SI.

- This measurement is obtained by sampling at a pre-defined interval, the transmitted carrier power (extracted from NBAP common measurement report (see TS 25.215 [14], TS 25.433 [7])) of all codes not used for HS-PDSCH, HS-SCCH, E-AGCH, E-RGCH or E-HICH transmission of a given UtranCell, and then taking the maximum of the measurement values obtained during the granularity period.

- A single integer value from 0 to 100, that maps the value of the measured transmitted carrier power to a  percentage as defined in table 9.64 (see TS 25.133 [8]).

- CARR.MaxFDDNonHSTCP.

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

## 4.19	FDD Code Resources

### 4.19.1	Code Resources Used of an FDD mode UTRAN Cell

- This measurement provides the number of OVSF codes used of an UTRAN cell. This measurement is split into subcounters according to the Orthogonal Variable Spreading Factor (OVSF) length. This measurement is only valid for FDD mode.

- SI.

- This measurement is obtained by sampling at a pre-defined interval, the number of OVSF codes used per spread factor (SF), and then taking the arithmetic mean. The SF used as following:
SF=4, SF= 8, SF= 16, SF= 32, SF= 64, SF= 128, SF= 256, SF=512.

- Eight integer values.

- CR.CodesUsed.SF4
CR.CodesUsed.SF8
CR.CodesUsed.SF16
CR.CodesUsed.SF32
CR.CodesUsed.SF64
CR.CodesUsed.SF128
CR.CodesUsed.SF256
CR.CodesUsed.SF512

- UtranCellFDD

- Valid for circuit switched and packet switched traffic

- UMTS

## 4.20	Paging

### 4.20.1	Overview

Paging procedures include RANAP Paging procedure, RRC Paging and RRC UE Dedicated Paging procedure.

#### 4.20.1.1	Considered Paging procedures

Performance Measurement definitions in this subclause are based on TS 25.331 [4] and TS 25.413 [5].

The following procedures are of interest for this purpose:

RRC: UE DEDICATED PAGING

RRC: PAGING

RANAP: PAGING

- Figure: RRC Paging procedure

- Figure: RRC UE Dedicated Paging procedure

- Figure: RANAP Paging procedure (Successful operation)

### 4.20.2	Paging request from CN

a)	This measurement provides the number of paging requests received from CN. The measurement is split into subcounters per paging cause.

b)	CC

c)	On receipt by the RNC of a RANAP PAGING message from CN. Each RANAP message PAGING receipt by the RNC is added to the relevant per cause measurement. The possible causes are included in TS 25.413 [5]. 
The sum of all supported per cause measurements shall equal the total number of PAGING requests. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first.

d)	Each measurement is an integer value.

e)	The measurement name has the form Pag.AttCn.Cause 
where Cause identifies the paging cause.

f)	RncFunction

g)	Valid for circuit switched and packet switched traffic

h)	UMTS

### 4.20.3	Paging Type 1

#### 4.20.3.1	Attempted Paging Type 1

a)	This measurement provides the number of attempted Paging Type 1 procedures. The measurement is split into subcounters per paging cause.

b)	CC

c)	On transmission by the RNC of a RRC PAGING TYPE 1 message to the UE. Each RRC PAGING TYPE 1 message sent by the RNC is added to the relevant per cause measurement. The possible causes are included in TS 25.331 [4]. The sum of all supported per cause measurements shall equal the total number of Paging Type 1 attempts. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first.

d)	Each measurement is an integer value.

e)	The measurement name has the form Pag.AttPagType1.Cause 
where Cause identifies the paging cause.

f)	UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

g)	Valid for circuit switched and packet switched traffic

h)	UMTS

### 4.20.4	Paging Type 2

#### 4.20.4.1	Attempted Paging Type 2

a)	This measurement provides the number of attempted Paging Type 2 procedures. The measurement is split into subcounters per paging cause.

b)	CC

c)	On transmission by the RNC of a RRC PAGING TYPE 2 message to the UE. Each RRC PAGING TYPE 2 message sent by the RNC is added to the relevant per cause measurement. The possible causes are included in TS 25.331 [4]. The sum of all supported per cause measurements shall equal the total number of Paging Type 2 attempts. In case only a subset of per cause measurements is supported, a sum measurement subtype will be provided first.

d)	Each measurement is an integer value.

e)	The measurement name has the form Pag.AttPagType2.Cause 
where Cause identifies the paging cause.

f)	UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

g)	Valid for circuit switched and packet switched traffic

h)	UMTS

## 4.21	ATM layer measurement

### 4.21.1	Cell collection

#### 4.21.1.1	Ingress cells (whole stream)

- This measurement provides the total number of ingress cells on ATM node.

- CC

- See clause 5.6.7 in [11].

- A single integer value, see [11].

- ATM.ATML.IngressCells.

- IubLink.

- Valid for circuit switched and packet switched traffic

- UMTS

#### 4.21.1.2	Egress cells (whole stream)

- This measurement provides the total number of egress cells on ATM node.

- CC

- See NumberCellsTrnsd in [11].

- A single integer value, see [11].

- ATM.ATML.EgressCells.

- IubLink.

- Valid for circuit switched and packet switched traffic

- UMTS

## 4.22	HSDPA setup

### 4.22.1	Mac-d setup for HSDPA

The three measurement types defined in this subclause are subject to the "2 out of 3 approach".

#### 4.22.1.1	Attempted Mac-d setups for HSDPA

- This measurement provides the number of attempted Mac-d setups for HSDPA.

- CC

- On transmission by the RNC of a NBAP message RADIO LINK SETUP REQUEST with the “HS-DSCH Information” IE; Or on transmission by the RNC of a NBAP message RADIO LINK RECONFIGURATION PREPARE with the “HS-DSCH MAC-d Flows To Add” or “HS-DSCH Information” IE; Or on transmission by the RNC of a NBAP message RADIO LINK RECONFIGURATION REQUEST with the “HS-DSCH MAC-d Flows To Add” or “HS-DSCH Information” IE. See TS 25.433 [7].

- A single integer value

- HSDPA.AttMacdSetup

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

#### 4.22.1.2	Successful Mac-d setups for HSDPA

- This measurement provides the number of successful Mac-d setups for HSDPA.

- CC

- On receipt by the RNC of a NBAP message RADIO LINK SETUP RESPONSE, corresponding to transmission of the NBAP message RADIO LINK SETUP REQUEST with the “HS-DSCH Information” IE; Or on receipt by the RNC of a NBAP message RADIO LINK RECONFIGURATION READY, corresponding to the transmission of the NBAP message RADIO LINK RECONFIGURATION PREPARE with the “HS-DSCH MAC-d Flows To Add” or “HS-DSCH Information” IE; Or on receipt by the RNC of NBAP message RADIO LINK RECONFIGURATION RESPONSE, corresponding to the transmission of the NBAP message RADIO LINK RECONFIGURATION REQUEST with the “HS-DSCH MAC-d Flows To Add” or “HS-DSCH Information” IE. See TS 25.433 [7].

- A single integer value

- HSDPA.SuccMacdSetup

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

#### 4.22.1.3	Failed Mac-d setups for HSDPA

- This measurement provides the number of failed Mac-d setups for HSDPA. The measurement is split into subcounters per cause.

- CC

- On receipt by the RNC of a NBAP message RADIO LINK SETUP FAILURE, corresponding to transmission of the NBAP message RADIO LINK SETUP REQUEST with the “HS-DSCH Information” IE; Or on receipt by the RNC of a NBAP message RADIO LINK RECONFIGURATION FAILURE, corresponding to the transmission of the NBAP message RADIO LINK RECONFIGURATION PREPARE with the “HS-DSCH MAC-d Flows To Add” or  “HS-DSCH Information” IE; Or on receipt by the RNC of NBAP message RADIO LINK RECONFIGURATION FAILURE, corresponding to the transmission of the NBAP message RADIO LINK RECONFIGURATION REQUEST with the “HS-DSCH MAC-d Flows To Add” or “HS-DSCH Information” IE. Each failed Mac-d setup for HSDPA is added to the relevant measurement according to the failure cause. Possible failure causes are included in TS 25.433 [7]. 

Each expected NBAP message RADIO LINK SETUP FAILURE, corresponding to transmission of the NBAP message RADIO LINK SETUP REQUEST with the “HS-DSCH Information” IE; Or  NBAP message RADIO LINK RECONFIGURATION FAILURE, corresponding to the transmission of the NBAP message RADIO LINK RECONFIGURATION PREPARE with the “HS-DSCH MAC-d Flows To Add” or  “HS-DSCH Information” IE; Or NBAP message RADIO LINK RECONFIGURATION FAILURE, corresponding to the transmission of the NBAP message RADIO LINK RECONFIGURATION REQUEST with the “HS-DSCH MAC-d Flows To Add” or “HS-DSCH Information” IE not received by the RNC is added to the measurement cause 'No Reply' (not specified in TS 25.433 [7]).

The sum of all supported per cause measurements shall equal the total number of failed Mac-d setups for HSDPA. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form HSDPA.FailMacdSetup.Cause
where Cause identifies the failure cause. 
The cause 'No Reply' is identified by the .NoReply suffix.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

### 4.22.2	RB setup for HSDPA

The three measurement types defined in this subclause are subject to the "2 out of 3 approach".

#### 4.22.2.1	Attempted RB setups for HSDPA

- This measurement provides the number of attempted radio bearer setups for HSDPA.

- CC

- On transmission by the RNC of a RRC message RADIO BEARER SETUP with the “Downlink transport channel type”  set to “HS-DSCH” or “DCH + HS-DSCH” in the “RB mapping info” IE of the “RB information to setup”; Or on transmission by the RNC of a RRC message RADIO BEARER RECONFIGURATION with “Downlink transport channel type” set to “HS-DSCH” or “DCH + HS-DSCH” in the “RB mapping info” IE of the “RB information to reconfigure” if the previous “Downlink transport channel type” of this RB is not the “HS-DSCH” or “DCH + HS-DSCH”. See TS 25.331 [4].

- A single integer value

- HSDPA.AttRBSetup

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

#### 4.22.2.2	Successful RB setups for HSDPA

- This measurement provides the number of successful radio bearer setups for HSDPA.

- CC

- On receipt by the RNC of a RRC message RADIO BEARER SETUP COMPLETE, corresponding to the  transmission of the RRC message RADIO BEARER SETUP with the “Downlink transport channel type”  set to “HS-DSCH” or “DCH + HS-DSCH” in the “RB mapping info” IE of the “RB information to setup”; Or on receipt by the RNC of a RRC message RADIO BEARER RECONFIGURATION COMPLETE, corresponding to the transmission of the RRC message RADIO BEARER RECONFIGURATION with “Downlink transport channel type” set to “HS-DSCH” or “DCH + HS-DSCH” in the “RB mapping info” IE of the “RB information to reconfigure” if the previous “Downlink transport channel type” of this RB is not the “HS-DSCH” or “DCH + HS-DSCH”. See TS 25.331 [4].

- A single integer value

- HSDPA.SuccRBSetup

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

#### 4.22.2.3	Failed RB setups for HSDPA

- This measurement provides the number of failed radio bearer setups for HSDPA. The measurement is split into subcounters per cause.

- CC

- On receipt by the RNC of a RRC message RADIO BEARER SETUP FAILURE, corresponding to the  transmission of the RRC message RADIO BEARER SETUP with the “Downlink transport channel type”  set to “HS-DSCH” or “DCH + HS-DSCH” in the “RB mapping info” IE of the “RB information to setup”; Or on receipt by the RNC of a RRC message RADIO BEARER RECONFIGURATION FAILURE, corresponding to the transmission of the RRC message RADIO BEARER RECONFIGURATION with “Downlink transport channel type” set to “HS-DSCH” or “DCH + HS-DSCH” in the “RB mapping info” IE of the “RB information to reconfigure” if the previous “Downlink transport channel type” of this RB is not the “HS-DSCH” or “DCH + HS-DSCH”. Each failed RB setup for HSDPA is added to the relevant measurement according to the failure cause. Possible failure causes are included in TS 25.331 [4]. 

Each expected RRC message RADIO BEARER SETUP FAILURE, corresponding to the  transmission of the RRC message RADIO BEARER SETUP with the “Downlink transport channel type”  set to “HS-DSCH” or “DCH + HS-DSCH” in the “RB mapping info” IE of the “RB information to setup”; Or RRC message RADIO BEARER RECONFIGURATION FAILURE, corresponding to the transmission of the RRC message RADIO BEARER RECONFIGURATION with “Downlink transport channel type” set to “HS-DSCH” or “DCH + HS-DSCH” in the “RB mapping info” IE of the “RB information to reconfigure” if the previous “Downlink transport channel type” of this RB is not the “HS-DSCH” or “DCH + HS-DSCH” not received by the RNC is added to the measurement cause 'No Reply' (not specified in TS 25.331 [4]).

The sum of all supported per cause measurements shall equal the total number of failed RB setups for HSDPA. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form HSDPA.FailRBSetup.Cause
where Cause identifies the failure cause. 
The cause 'No Reply' is identified by the .NoReply suffix.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

## 4.23	Call duration in UTRAN cell

### 4.23.1	Call duration in UTRAN cell for CS domain

- This measurement provides the call duration in utran cell for CS domain. The measurement is split into subcounters per traffic class (See the 3GPP TS23.107 [2]).

- DER (n=1)

- This measurement is obtained by accumulating the time intervals for each traffic class between the call start RRC RADIO BEARER SETUP COMPLETE and the call completed RRC RADIO BEARER RELEASE over a granularity period using DER, For conversational service, the relevant measurement for each specified data rate is required, the detailed data rates are listed below (see TS 25.993 [12]).The accumulator shall be reinitialised at the beginning of each granularity period. The call duration for each traffic class (for each specified data rate of conversational service) for CS domain is added to the relevant measurement. See TS 25.413 [5] and TS 23.107 [2].
uplink <U>:
		1: AMR rate (12.2 10.2 7.95 7.4 6.7 5.9 5.15 4.75) kbps
 		2: 12.2 kbps
 		3: 28.8 kbps
 		4: 32 kbps
 		5: 64 kbps
downlink <D>: 
		1: AMR rate (12.2 10.2 7.95 7.4 6.7 5.9 5.15 4.75) kbps
 		2: 12.2 kbps
 		3: 28.8 kbps
 		4: 32 kbps
 		5: 64 kbps
As indicated above, <U> and <D> are integer values that map to the conversational service specified uplink and downlink data rates, respectively.

- Each measurement is an integer value. (in seconds)

- The measurement name has the form 
RRC.CallDurationCS.Conv.<U><D>
RRC.CallDurationCS.Strm
RRC.CallDurationCS.Intact
RRC.CallDurationCS.Bgrd.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched

- UMTS

## 4.24	Measurements related to channel switches between FACH/DCH and HS-DSCH

### 4.24.1	Measurements related to channel switches from FACH/DCH to HS-DSCH intra UTRAN cell

#### 4.24.1.1	Attempted channel switches from FACH to HS-DSCH

- This measurement provides the number of attempted channel switches from FACH to HS-DSCH in the serving HS-DSCH cell.

- CC

- On transmission by the RNC of a RRC message RADIO BEARER RECONFIGURATION to UE, with the condition that channel switches from FACH to HS-DSCH (see TS 25.331 [4]).

NOTE:	This measurement is dedicated to channel switches from FACH to HS-DSCH.

- A single integer value

- HSDPA.AttFachToHs

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

#### 4.24.1.2	Successful channel switches from FACH to HS-DSCH

- This measurement provides the number of successful channel switches from FACH to HS-DSCH in the serving HS-DSCH cell.

- CC

- On receipt by the RNC of a RRC message RADIO BEARER RECONFIGURATION COMPLETE from UE, with the condition that channel switches from FACH to HS-DSCH (see TS 25.331 [4]).

NOTE:	This measurement is dedicated to channel switches from FACH to HS-DSCH.

- A single integer value

- HSDPA.SuccFachToHs

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

#### 4.24.1.3	Attempted channel switches from DCH to HS-DSCH

- This measurement provides the number of attempted channel switches from DCH to HS-DSCH in the serving HS-DSCH cell.

- CC

- On transmission by the RNC of a RRC message RADIO BEARER RECONFIGURATION to UE, with the condition that channel switches from DCH to HS-DSCH (see TS 25.331 [4]).

- A single integer value

- HSDPA.AttDchToHs

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

#### 4.24.1.4	Successful channel switches from DCH to HS-DSCH

- This measurement provides the number of successful channel switches from DCH to HS-DSCH in the serving HS-DSCH cell.

- CC

- On receipt by the RNC of a RRC message RADIO BEARER RECONFIGURATION COMPLETE from UE, with the condition that channel switches from DCH to HS-DSCH (see TS 25.331 [4]).

- A single integer value

- HSDPA.SuccDchToHs

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

### 4.24.2	Measurements related to channel switches from HS-DSCH to FACH/DCH intra UTRAN cell

#### 4.24.2.1	Attempted channel switches from HS-DSCH to FACH

- This measurement provides the number of attempted channel switches from HS-DSCH to  FACH in the serving HS-DSCH cell.

- CC

- On transmission by the RNC of a RRC message RADIO BEARER RECONFIGURATION to UE, with the condition that channel switches from HS-DSCH to FACH (see TS 25.331 [4]). 

Note: this measurement is dedicated to channel switches from HS-DSCH to FACH.

- A single integer value

- HSDPA.AttHsToFach

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

#### 4.24.2.2	Successful channel switches from HS-DSCH to FACH

- This measurement provides the number of successful channel switches from HS-DSCH to  FACH in the serving HS-DSCH cell.

- CC

- On receipt by the RNC of a RRC message RADIO BEARER RECONFIGURATION COMPLETE from UE, with the condition that channel switches from HS-DSCH to FACH (see TS 25.331 [4]). 

Note: this measurement is dedicated to channel switches from HS-DSCH to FACH.

- A single integer value

- HSDPA.SuccHsToFach

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

#### 4.24.2.3	Attempted channel switches from HS-DSCH to DCH

- This measurement provides the number of attempted channel switches from HS-DSCH to  DCH in the serving HS-DSCH cell.

- CC

- On transmission by the RNC of a RRC message RADIO BEARER RECONFIGURATION to UE, with the condition that channel switches from HS-DSCH to DCH (see TS 25.331 [4]).

- A single integer value

- HSDPA.AttHsToDch

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

#### 4.24.2.4	Successful channel switches from HS-DSCH to DCH

- This measurement provides the number of successful channel switches from HS-DSCH to  DCH in the serving HS-DSCH cell.

- CC

- On receipt by the RNC of a RRC message RADIO BEARER RECONFIGURATION COMPLETE from UE, with the condition that channel switches from HS-DSCH to DCH (see TS 25.331 [4]).

- A single integer value

- HSDPA.SuccHsToDch

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

## 4.25	HSDPA mobility related measurements

### 4.25.1	Serving HS-DSCH cell change

#### 4.25.1.1	Attempted serving HS-DSCH cell changes

- This measurement provides the number of attempted serving HS-DSCH cell changes.

- CC

- On transmission by the RNC of a RRC message PHYSICAL CHANNEL RECONFIGURATION, RADIO BEARER SETUP, RADIO BEARER RECONFIGURATION, RADIO BEARER RELEASE, or TRANSPORT CHANNEL RECONFIGURATION from the source RNC to the UE, also the target cell is different with source cell, indicating the attempt of serving HS-DSCH cell change (see TS 25.331 [4]).

- A single integer value

- HSDPA.AttCellChange

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

#### 4.25.1.2	Successful serving HS-DSCH cell changes

- This measurement provides the number of successfully performed serving HS-DSCH cell changes.

- CC

- On receipt by the RNC of a RRC message PHYSICAL CHANNEL RECONFIGURATION COMPLETE, RADIO BEARER SETUP COMPLETE, RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER RELEASE COMPLETE, or TRANSPORT CHANNEL RECONFIGURATION COMPLETE sent from the UE to the source RNC, also the target cell is different with source cell indicating a successful serving HS-DSCH cell change (see TS 25.331 [4]).

- A single integer value

- HSDPA.SuccCellChange

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

## 4.26	Measurements related to MAC-hs

### 4.26.1	Measurements related to MAC-hs feedback decoding

#### 4.26.1.1	Number of acknowleged transmitted MAC-hs PDUs

- This measurement provides the number of acknowleged transmitted MAC-hs PDUs during the period of measurement, detected in MAC-hs layer in the serving HS-DSCH cell.

- CC.

- On receipt by the NodeB of a PDU acknowledged by ACK message of MAC-hs from UE.

- A single integer value.

- HSDPA.NbrAckdMachsPdu

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS.

#### 4.26.1.2	Number of transmitted MAC-hs PDUs

- This measurement provides the number of transmitted MAC-hs PDUs including acknowledged and unacknowledged PDUs during the period of measurement, detected in MAC-hs layer in the serving HS-DSCH cell.

- CC.

- On transmission by the NodeB of a PDU of MAC-hs to UE.

- A single integer value.

- HSDPA.NbrMachsPdu

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS.

## 4.27	Mean number of HSDPA users in a serving HS-DSCH cell

- This measurement provides the mean number of simultaneous HSDPA users in a serving HS-DSCH cell.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of simultaneous users in a serving HS-DSCH cell and then taking the arithmetic mean.

- A single integer value.

- HSDPA.MeanNbrUser

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS.

## 4.28	Number of octets of acknowledged MAC-hs PDUs

- This measurement provides the number of octets of downlink acknowledged MAC-hs PDUs in the serving HS-DSCH cell.

- CC.

- On transmission by the NodeB of an acknowledged PDU on the MAC-hs layer. The number of octets of MAC-hs layer in one serving HS-DSCH cell is calculated by sum of size of each MAC-hs PDU header including MAC-hs PDU header.

- A single integer value.

- HSDPA.NbrAckdMacHsOcts

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS.

## 4.29	Number of TTIs with user data packets in buffer

- This measurement provides the number of TTIs with user data packets in MAC-hs buffer.

- CC.

- On receipt by the NodeB of a HSDPA frame with user data packets in MAC-hs buffer.

- A single integer value.

- HSDPA.NbrTTINonEmptyBuffer

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS.

## 4.30	HSDPA Release measurements

### 4.30.1	Overview of HSDPA Release

#### 4.30.1.1	General

The HSDPA release is executed via releasing the HS-DSCH. It can happen with RRC Connection Release procedure and various Radio Bearer Control procedures. The RANAP procedures RAB Assignment Request or Iu Release Command can also be involved.

There is always a RRC procedure used but not necessarily a RANAP procedure.

As there are HSDPA release cases that doesn’t involve CN (RANAP) the triggering of required measurements is always based primarily on RRC triggering.

In cases that the CN has initiated the release of a RAB and/or Iu connection that currently has a Radio Bearer including HS-DSCH the RANAP interface is included in the triggering part of the related requirement.

#### 4.30.1.2	Considered RRC procedures

RRC Protocol procedures related HSDPA (HS-DSCH Release)

RRC connection release (TS 25.331: RRC Connection Management procedures chapter 8.1.4):

- Figure: RRC Connection Release procedure on the DCCH

Radio Bearer control /Reconfiguration procedures (TS25.331 Radio Bearer Control Procedures)

- Figure: Radio Bearer Establishment, normal case

- Figure: Radio Bearer Establishment, failure case

- Figure: Radio bearer reconfiguration, normal flow

- Figure: Radio bearer reconfiguration, failure case

- Figure: Radio Bearer Release, normal case

- Figure: Radio Bearer Release, failure case

- Figure: Transport channel reconfiguration, normal flow

- Figure: Transport channel reconfiguration, failure case

- Figure: Physical channel reconfiguration, normal flow

- Figure : Physical channel reconfiguration, failure case

#### 4.30.1.3	Considered RANAP protocol procedures

RAB Assignment (TS 25.413 Chapter 8.2)

- Figure: RAB Assignment procedure. Successful operation.

Iu Release (TS 25.413 Chapter 8.5)

- Figure: Iu Release procedure. Successful operation.

### 4.30.2	Normal HSDPA Release

#### 4.30.2.1	RNC Initiated Release due to user inactivity

- This measurement provides the number of times when the UE is removed from HS-DSCH transport channel due to user inactivity

- CC.

- On transmission of the RRC: RADIO BEARER RECONFIGURATION or RRC: RADIO BEARER RELEASE or RRC CONNECTION RELEASE message by the RNC when there is no more data left in the buffer to send (equals to the RANAP cause 16 user inactivity).

- A single integer value.

- The measurement name has the form HSDPA.SuccHSDSCHreleaseUserInact

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr.

- Valid for packet switched traffic.

- UMTS.

#### 4.30.2.2	Switch to DCH/FACH

The measurement definition is in section 4.24

#### 4.30.2.3	HS-DSCH serving cell change

The measurement definition is in section 4.25

#### 4.30.2.4	CN Initiated release

- This measurement provides the number of times when the UE is removed from HS-DSCH transport channel due to a release initiated by the Core Network.

- CC.

- On reception of RANAP RAB ASSIGNMENT REQUEST or RANAP IU RELEASE COMMAND that results an HS-DSCH release. The release of HS-DSCH is executed by the transmission of the RRC RADIO BEARER RECONFIGURATION or RRC RADIO BEARER RELEASE or RRC CONNECTION RELEASE message.

- A single integer value.

- The measurement name has the form HSDPA.SuccCnInitHSDSCHrelease

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr.

- Valid for packet switched traffic.

- UMTS.

### 4.30.3	Abnormal RB releases for HS-DSCH

- This measurement provides the number of times when the UE is removed from HS-DSCH transport channel, and trigger reason is a failure. The reason can be e.g. RL failure.

- CC.

- On receipt by the RRC: RADIO BEARER RECONFIGURATION COMPLETE, or RRC: RADIO BEARER RELEASE COMPLETE message sent by the UE. Or when the UE does not respond to RRC: RADIO BEARER RECONFIGURATION or RRC: RADIO BEARER RELEASE message. All other release scenarios are considered as abnormal releases, which are not counted in normal releases, i.e. the release is not because of user inactivity or switch to DCH or HS-DSCH serving cell changes or CN initiated release.

d)	A single integer value.

e)	The measurement name has the form HSDPA.failHSDSCHrelease

f)	UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr.

g)	Valid for packet switched traffic.

h)	UMTS.

## 4.31	RRC Connection mobility

### 4.31.1	Cell update

#### 4.31.1.1	Attempted cell update

- This measurement provides the number of cell update attempts per update cause sent from the UE to UTRAN.

- CC

- Receipt of a CELL UPDATE message sent from UE to the RNC. Each cell update message sent is added to the relevant per cause measurement. The possible causes are included in TS 25.331 [4]. The sum of all supported per cause measurements shall equal the total number of cell update attempts. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RRC.AttCellUpdate.Cause
where Cause identifies the Cell Update Cause.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched and packet switched traffic

- UMTS

## 4.32	Measurements related to received uplink transport block

### 4.32.1	Measurements related to received uplink transport block for CS domain

#### 4.32.1.1	Number of received uplink transport blocks of DCH for CS domain

- This measurement provides the number of received uplink transport blocks of DCH during the period of measurement for the CS domain. In case of soft handover, this measurement is valid after combination, based on best UTRAN cell for FDD mode only.

- CC.

- On receipt by the RNC of an uplink transport block of DCH from UE for CS domain. For conversational service, the relevant measurement for each specified data rate is required. The detailed data rates are listed below (see TS 25.993 [12]). 
uplink <U>:
		1: AMR rate (12.2 10.2 7.95 7.4 6.7 5.9 5.15 4.75) kbps
 		2: 12.2 kbps
 		3: 28.8 kbps
 		4: 32 kbps
 		5: 64 kbps
As indicated above <U> maps to the conversational service specified uplink data rates.

- A single integer value.

- FP.NbrBlocksReceivedCS.Conv.<U>
FP.NbrBlocksReceivedCS.Strm
FP.NbrBlocksReceivedCS.Intact
FP.NbrBlocksReceivedCS.Bgrd

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched traffic.

- UMTS.

#### 4.32.1.2	Number of received uplink faulty transport blocks of DCH for CS domain

- This measurement provides the number of received uplink faulty transport blocks of DCH during the period of measurement for CS domain. In case of soft handover, this measurement is valid after combination, based on best UTRAN cell for FDD mode only.

- CC.

- On receipt by the RNC of an uplink faulty transport block using CRC indicator (see TS 25.427[17]) of DCH from UE for CS domain. For conversational service, the relevant measurement for each specified data rate is required. The detailed data rates are listed below (see TS 25.993 [12]).
uplink <U>:
		1: AMR rate (12.2 10.2 7.95 7.4 6.7 5.9 5.15 4.75) kbps
 		2: 12.2 kbps
 		3: 28.8 kbps
 		4: 32 kbps
 		5: 64 kbps
As indicated above <U> maps to the conversational service specified uplink data rates.

- A single integer value.

- FP.NbrErrBlocksReceivedCS.Conv.<U>
FP.NbrErrBlocksReceivedCS.Strm
FP.NbrErrBlocksReceivedCS.Intact
FP.NbrErrBlocksReceivedCS.Bgrd

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for circuit switched traffic.

- UMTS.

### 4.32.2	Measurements related to received uplink transport block for PS domain

#### 4.32.2.1	Number of received uplink transport blocks of DCH for PS domain

- This measurement provides the number of received uplink transport blocks of DCH during the period of measurement for PS domain. In case of soft handover, this measurement is valid after combination, based on best UTRAN cell for FDD mode only.

- CC.

- On receipt by the RNC of an uplink transport block of DCH from UE for PS domain. For conversational service, the relevant measurement for each specified data rate is required. The detailed data rates are listed below (see TS 25.993 [12]).
uplink <U>:
		1: AMR rate (12.2 10.2 7.95 7.4 6.7 5.9 5.15 4.75) kbps
 		2: 12.2 kbps
 		3: 28.8 kbps
 		4: 32 kbps
 		5: 64 kbps
As indicated above <U> maps to the conversational service specified uplink data rates.

- A single integer value.

- FP.NbrBlocksReceivedPS.Conv.<U>
FP.NbrBlocksReceivedPS.Strm
FP.NbrBlocksReceivedPS.Intact
FP.NbrBlocksReceivedPS.Bgrd

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS.

#### 4.32.2.2	Number of received uplink faulty transport blocks of DCH for PS domain

- This measurement provides the number of received uplink faulty transport blocks of DCH during the period of measurement for PS domain. In case of soft handover, this measurement is valid after combination, based on best UTRAN cell for FDD mode only.

- CC.

- On receipt by the RNC of an uplink faulty transport block using CRC indicator (see TS 25.427[17]) of DCH from UE for PS domain. For conversational service, the relevant measurement for each specified data rate is required. The detailed data rates are listed below (see TS 25.993 [12]).
uplink <U>:
		1: AMR rate (12.2 10.2 7.95 7.4 6.7 5.9 5.15 4.75) kbps
 		2: 12.2 kbps
 		3: 28.8 kbps
 		4: 32 kbps
 		5: 64 kbps
As indicated above <U> maps to the conversational service specified uplink data rates.

- A single integer value.

- FP.NbrErrBlocksReceivedPS.Conv.<U>
FP.NbrErrBlocksReceivedPS.Strm
FP.NbrErrBlocksReceivedPS.Intact
FP.NbrErrBlocksReceivedPS.Bgrd

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS.

## 4.33	Measurements related to equipment resources

### 4.33.1	RNC Processor usage

#### 4.33.1.1	Mean processor usage

- This measurement provides the mean usage of each key processor during the granularity period. Each equipment may have more than one key processors, the measurement is split into subcounters per key processor.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the usage of the processor and then taking the arithmetic mean for each key processor.

- Each measurement is an integer value (Unit: %).

- The measurement name has the form EQPT.MeanProcessorUsage.ProcessorID
where ProcessorID identifies the key processor of this equipment, the format of ProcessorID is vendor specific.

- ManagedElement.

- Valid for circuit switched and packet switched traffic.

h)	UMTS.

#### 4.33.1.2	Peak processor usage

- This measurement provides the peak usage of each key processor during the granularity period. Each equipment may have more than one key processors, the measurement is split into subcounters per key processor.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the usage of the processor and then taking the maximum for each key processor.

- Each measurement is an integer value (Unit: %).

- The measurement name has the form EQPT.PeakProcessorUsage.ProcessorID
where ProcessorID identifies the key processor of this equipment, the format of ProcessorID is vendor specific.

- ManagedElement.

- Valid for circuit switched and packet switched traffic.

h)	UMTS.

## 4.34	HSUPA related measurements

Performance Measurement definitions in this subclause are based on TS 25.331 [4] and TS 25.433 [7].

The following procedures are of interest for this purpose:

NBAP: RADIO LINK SETUP REQUEST

NBAP: RADIO LINK RECONFIGURATION PREPARE

NBAP: RADIO LINK RECONFIGURATION REQUEST

NBAP: RADIO LINK ADDITION REQUEST

NBAP: RADIO LINK SETUP RESPONSE

NBAP: RADIO LINK RECONFIGURATION READY

NBAP: RADIO LINK RECONFIGURATION RESPONSE

NBAP: RADIO LINK ADDITION RESPONSE

NBAP: RADIO LINK SETUP FAILURE

NBAP: RADIO LINK RECONFIGURATION FAILURE

NBAP: RADIO LINK RECONFIGURATION FAILURE

NBAP: RADIO LINK ADDITION FAILURE

RRC: RADIO BEARER SETUP

RRC: RADIO BEARER RECONFIGURATION

RRC: RADIO BEARER SETUP COMPLETE

RRC: RADIO BEARER RECONFIGURATION COMPLETE

RRC: RADIO BEARER SETUP FAILURE

RRC: RADIO BEARER RECONFIGURATION FAILURE

Figure : Radio Link Setup procedure, Successful Operation

Figure: Radio Link Setup procedure, Unsuccessful Operation

Figure: Radio Link Reconfiguration Preparation procedure, Successful Operation

Figure: Radio Link Reconfiguration Preparation procedure, Unsuccessful Operation

Figure: Radio Link Addition procedure, Successful Operation

Figure: Radio Link Addition procedure: Unsuccessful Operation

- Figure: Radio Bearer Establishment, normal case

- Figure: Radio Bearer Establishment, failure case

- Figure: Radio bearer reconfiguration, normal flow

- Figure: Radio bearer reconfiguration, failure case

### 4.34.1	HSUPA setup

#### 4.34.1.1	Mac-d setup for HSUPA

The three measurement types defined in this subclause are subject to the "2 out of 3 approach".

##### 4.34.1.1.1	Attempted Mac-d setups for HSUPA

- This measurement provides the number of attempted Mac-d setups for HSUPA.

- CC

- For FDD:

- On transmission by the RNC of a NBAP message RADIO LINK SETUP REQUEST with the “E-DCH FDD Information” IE; Or on transmission by the RNC of a NBAP message RADIO LINK RECONFIGURATION PREPARE with the “E-DCH MAC-d Flows To Add” or “E-DCH FDD Information” IE; Or on transmission by the RNC of a NBAP message RADIO LINK RECONFIGURATION REQUEST with the “E-DCH MAC-d Flows To Add” or “E-DCH FDD Information” IE; Or on transmission by the RNC of a NBAP message RADIO LINK ADDITION REQUEST with the “E-DCH FDD Information” IE. See TS 25.433 [7].

- For TDD:

- On transmission by the RNC of a NBAP message RADIO LINK SETUP REQUEST with the “E-DCH MAC-d Flows Information TDD” IE; Or on transmission by the RNC of a NBAP message RADIO LINK RECONFIGURATION PREPARE with the “E-DCH MAC-d Flows To Add” IE; Or on transmission by the RNC of a NBAP message RADIO LINK RECONFIGURATION REQUEST with the “E-DCH MAC-d Flows To Add” IE; Or on transmission by the RNC of a NBAP message RADIO LINK ADDITION REQUEST with the “E-DCH MAC-d Flows Information TDD” IE. See TS 25.433 [7].

- A single integer value

- HSUPA.AttMacdSetup

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

##### 4.34.1.1.2	Successful Mac-d setups for HSUPA

- This measurement provides the number of successful Mac-d setups for HSUPA.

- CC

- For FDD:

- On receipt by the RNC of a NBAP message RADIO LINK SETUP RESPONSE, corresponding to transmission of the NBAP message RADIO LINK SETUP REQUEST with the “E-DCH FDD Information” IE; Or on receipt by the RNC of a NBAP message RADIO LINK RECONFIGURATION READY, corresponding to the transmission of the NBAP message RADIO LINK RECONFIGURATION PREPARE with the “E-DCH MAC-d Flows To Add” or “E-DCH FDD Information” IE; Or on receipt by the RNC of NBAP message RADIO LINK RECONFIGURATION RESPONSE, corresponding to the transmission of the NBAP message RADIO LINK RECONFIGURATION REQUEST with the “E-DCH MAC-d Flows To Add” or “E-DCH FDD Information” IE; Or on receipt by the RNC of NBAP message RADIO LINK ADDITION RESPONSE, corresponding to the transmission of the NBAP message RADIO LINK ADDITION REQUEST with the “E-DCH FDD Information” IE. See TS 25.433 [7].

- For TDD:

- On receipt by the RNC of a NBAP message RADIO LINK SETUP RESPONSE, corresponding to transmission of the NBAP message RADIO LINK SETUP REQUEST with the “E-DCH MAC-d Flows Information TDD” IE; Or on receipt by the RNC of a NBAP message RADIO LINK RECONFIGURATION READY, corresponding to the transmission of the NBAP message RADIO LINK RECONFIGURATION PREPARE with the “E-DCH MAC-d Flows To Add” IE; Or on receipt by the RNC of NBAP message RADIO LINK RECONFIGURATION RESPONSE, corresponding to the transmission of the NBAP message RADIO LINK RECONFIGURATION REQUEST with the “E-DCH MAC-d Flows To Add” IE; Or on receipt by the RNC of NBAP message RADIO LINK ADDITION RESPONSE, corresponding to the transmission of the NBAP message RADIO LINK ADDITION REQUEST with the “E-DCH MAC-d Flows Information TDD” IE. See TS 25.433 [7].

- A single integer value

- HSUPA.SuccMacdSetup

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

##### 4.34.1.1.3	Failed Mac-d setups for HSUPA

- This measurement provides the number of failed Mac-d setups for HSUPA. The measurement is split into subcounters per cause.

- CC

- For FDD:

- On receipt by the RNC of a NBAP message RADIO LINK SETUP FAILURE, corresponding to transmission of the NBAP message RADIO LINK SETUP REQUEST with the “E-DCH FDD Information” IE; Or on receipt by the RNC of a NBAP message RADIO LINK RECONFIGURATION FAILURE, corresponding to the transmission of the NBAP message RADIO LINK RECONFIGURATION PREPARE with the “E-DCH MAC-d Flows To Add” or “E-DCH FDD Information” IE; Or on receipt by the RNC of NBAP message RADIO LINK RECONFIGURATION FAILURE, corresponding to the transmission of the NBAP message RADIO LINK RECONFIGURATION REQUEST with the “E-DCH MAC-d Flows To Add” or “E-DCH FDD Information” IE; Or on receipt by the RNC of NBAP message RADIO LINK ADDITION FAILURE, corresponding to the transmission of the NBAP message RADIO LINK ADDITION REQUEST with the “E-DCH FDD Information” IE. 

Each expected NBAP message RADIO LINK SETUP RESPONSE, corresponding to transmission of the NBAP message RADIO LINK SETUP REQUEST with the “E-DCH FDD Information” IE; Or RADIO LINK RECONFIGURATION READY, corresponding to the transmission of the NBAP message RADIO LINK RECONFIGURATION PREPARE with the “E-DCH MAC-d Flows To Add” or “E-DCH FDD Information” IE; Or RADIO LINK RECONFIGURATION RESPONSE, corresponding to the transmission of the NBAP message RADIO LINK RECONFIGURATION REQUEST with the “E-DCH MAC-d Flows To Add” or “E-DCH FDD Information” IE; Or RADIO LINK ADDITION RESPONSE, corresponding to the transmission of the NBAP message RADIO LINK ADDITION REQUEST with the “E-DCH FDD Information” IE not received by the RNC is added to the measurement cause 'No Reply' (not specified in TS 25.331 [4]).

- For TDD:

- On receipt by the RNC of a NBAP message RADIO LINK SETUP FAILURE, corresponding to transmission of the NBAP message RADIO LINK SETUP REQUEST with the “E-DCH MAC-d Flows Information TDD” IE; Or on receipt by the RNC of a NBAP message RADIO LINK RECONFIGURATION FAILURE, corresponding to the transmission of the NBAP message RADIO LINK RECONFIGURATION PREPARE with the “E-DCH MAC-d Flows To Add” IE; Or on receipt by the RNC of NBAP message RADIO LINK RECONFIGURATION FAILURE, corresponding to the transmission of the NBAP message RADIO LINK RECONFIGURATION REQUEST with the “E-DCH MAC-d Flows To Add” IE; Or on receipt by the RNC of NBAP message RADIO LINK ADDITION FAILURE, corresponding to the transmission of the NBAP message RADIO LINK ADDITION REQUEST with the “E-DCH MAC-d Flows Information TDD” IE. 

Each expected NBAP message RADIO LINK SETUP RESPONSE, corresponding to transmission of the NBAP message RADIO LINK SETUP REQUEST with the “E-DCH MAC-d Flows Information TDD” IE; Or RADIO LINK RECONFIGURATION READY, corresponding to the transmission of the NBAP message RADIO LINK RECONFIGURATION PREPARE with the “E-DCH MAC-d Flows To Add” IE; Or RADIO LINK RECONFIGURATION RESPONSE, corresponding to the transmission of the NBAP message RADIO LINK RECONFIGURATION REQUEST with the “E-DCH MAC-d Flows To Add” IE; Or RADIO LINK ADDITION RESPONSE, corresponding to the transmission of the NBAP message RADIO LINK ADDITION REQUEST with the “E-DCH MAC-d Flows Information TDD” IE not received by the RNC is added to the measurement cause 'No Reply' (not specified in TS 25.331 [4]).


Each failed Mac-d setup for HSUPA is added to the relevant measurement according to the failure cause. Possible failure causes are included in TS 25.433 [7]. The sum of all supported per cause measurements shall equal the total number of failed Mac-d setups for HSUPA. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- HSUPA.FailMacdSetup.Cause
where Cause identifies the failure cause. 
The cause 'No Reply' is identified by the .NoReply suffix.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

#### 4.34.1.2	RB setup for HSUPA

The three measurement types defined in this subclause are subject to the "2 out of 3 approach".

##### 4.34.1.2.1	Attempted RB setups for HSUPA

- This measurement provides the number of attempted radio bearer setups for HSUPA.

- CC

- On transmission by the RNC of a RRC message RADIO BEARER SETUP with the “Uplink transport channel type”  set to “E-DCH” in the “RB mapping info” IE of the “RB information to setup”; Or on transmission by the RNC of a RRC message RADIO BEARER RECONFIGURATION with “Uplink transport channel type” set to “E-DCH” in the “RB mapping info” IE of the “RB information to reconfigure” if the previous “Uplink transport channel type” of this RB is not the “E-DCH”. See TS 25.331 [4].

- A single integer value

- HSUPA.AttRBSetup

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

##### 4.34.1.2.2	Successful RB setups for HSUPA

- This measurement provides the number of successful radio bearer setups for HSUPA.

- CC

- On receipt by the RNC of a RRC message RADIO BEARER SETUP COMPLETE, corresponding to the  transmission of the RRC message RADIO BEARER SETUP with the “Uplink transport channel type”  set to “E-DCH” in the “RB mapping info” IE of the “RB information to setup”; Or on receipt by the RNC of a RRC message RADIO BEARER RECONFIGURATION COMPLETE, corresponding to the transmission of the RRC message RADIO BEARER RECONFIGURATION with “Uplink transport channel type” set to “E-DCH” in the “RB mapping info” IE of the “RB information to reconfigure” if the previous “Uplink transport channel type” of this RB is not the “E-DCH”. See TS 25.331 [4].

- A single integer value

- HSUPA.SuccRBSetup

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

##### 4.34.1.2.3	Failed RB setups for HSUPA

- This measurement provides the number of failed radio bearer setups for HSUPA. The measurement is split into subcounters per cause.

- CC

- On receipt by the RNC of a RRC message RADIO BEARER SETUP FAILURE, corresponding to the  transmission of the RRC message RADIO BEARER SETUP with the “Uplink transport channel type”  set to “E-DCH” in the “RB mapping info” IE of the “RB information to setup”; Or on receipt by the RNC of a RRC message RADIO BEARER RECONFIGURATION FAILURE, corresponding to the transmission of the RRC message RADIO BEARER RECONFIGURATION with “Uplink transport channel type” set to “E-DCH” or in the “RB mapping info” IE of the “RB information to reconfigure” if the previous “Uplink transport channel type” of this RB is not the “E-DCH”. 

Each expected RRC message RADIO BEARER SETUP COMPLETE, corresponding to the  transmission of the RRC message RADIO BEARER SETUP with the “Uplink transport channel type”  set to “E-DCH” in the “RB mapping info” IE of the “RB information to setup”; Or RADIO BEARER RECONFIGURATION COMPLETE, corresponding to the transmission of the RRC message RADIO BEARER RECONFIGURATION with “Uplink transport channel type” set to “E-DCH” in the “RB mapping info” IE of the “RB information to reconfigure” if the previous “Uplink transport channel type” of this RB is not the “E-DCH” not received by the RNC is added to the measurement cause 'No Reply' (not specified in TS 25.331 [4]).

Each failed RB setup for HSUPA is added to the relevant measurement according to the failure cause. Possible failure causes are included in TS 25.331 [4]. The sum of all supported per cause measurements shall equal the total number of failed RB setups for HSUPA. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- HSUPA.FailRBSetup.Cause
where Cause identifies the failure cause. 
The cause 'No Reply' is identified by the .NoReply suffix.

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

### 4.34.2	Active set related to HSUPA

#### 4.34.2.1	Radio link additions to the E-DCH active set (UE side)

The three measurement types defined for the radio link additions to the E-DCH active set (UE side) are subject to the "2 out of 3 approach".

##### 4.34.2.1.1	Attempted radio link additions to the E-DCH active set (UE side)

- This measurement provides the number of attempted radio link additions to the E-DCH active set (UE side). This measurement shall be increased for each attempted radio link addition (UE side). This measurement is only valid for FDD mode.

- CC

- On transmission by the serving RNC of a RRC message ACTIVE SET UPDATE from the RNC to the UE, IE "E-DCH reconfiguration information": IE "Primary CPICH info" used for the reference ID along with the IE "E-HICH information" to indicate which radio link to add to the E-DCH active set. Within an ACTIVE SET UPDATE message more than one radio link can be added. Each existing radio link addition information element shall be considered separately (see TS 25.331 [4]).

- A single integer value

- HSUPA.AttEdchAddUESide

- UtranCellFDD

- Valid for packet switched traffic

- UMTS

##### 4.34.2.1.2	Successful radio link additions to the E-DCH active set (UE side)

- This measurement provides the number of successful radio link additions to the E-DCH active set (UE side). This measurement shall be increased for each successful radio link addition (UE side). This measurement is only valid for FDD mode.

- CC

- On receipt by the serving RNC of a RRC message ACTIVE SET UPDATE COMPLETE from the UE to the RNC, in response to an ACTIVE SET UPDATE message with one or more existing IE "E-DCH reconfiguration information": IE "Primary CPICH info" used for the reference ID along with the IE "E-HICH information" to indicate which radio link to add to the E-DCH active set. One ACTIVE SET UPDATE COMPLETE message can be related to more than one added radio link. Each successful added radio link shall be considered separately (see TS 25.331 [4]).

- A single integer value

- HSUPA.SuccEdchAddUESide

- UtranCellFDD

- Valid for packet switched traffic

- UMTS

##### 4.34.2.1.3	Failed radio link additions to the E-DCH active set (UE side)

- This measurement provides the number of failed radio link additions to the E-DCH active set (UE side) during E-DCH active set update procedure (UE side) for each cell per cause. For each failure cause a separate subcounter is defined. Every failed radio link addition (UE side) shall be considered separately. This measurement is only valid for FDD mode.

- CC

- Receipt of an ACTIVE SET UPDATE FAILURE message (RRC) sent by UE to the RNC in response to an ACTIVE SET UPDATE message with one or more existing IE "E-DCH reconfiguration information": IE "Primary CPICH info" used for the reference ID along with the IE "E-HICH information" to indicate which radio link to add to the E-DCH active set or an expected ACTIVE SET UPDATE COMPLETE message not received by the RNC. Each message can be related to more than one radio link.

-	Each failed attempt to add a radio link to the E-DCH active set shall be considered separately and added to the relevant per cause measurement. Failure causes are defined within TS 25.331 [4].

-	Each expected ACTIVE SET UPDATE COMPLETE message not received by the RNC is added to the measurement cause 'No Reply' (not specified in TS 25.331 [4]).

-	The sum of all supported per cause measurements shall equal the total number of failures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- HSUPA.FailEdchAddUESide.Cause

- UtranCellFDD

- Valid for packet switched traffic

- UMTS

#### 4.34.2.2	Radio link deletions to the E-DCH active set (UE side)

##### 4.34.2.2.1	Attempted radio link deletions to the E-DCH active set (UE side)

- This measurement provides the number of attempted radio link deletions to the E-DCH active set (UE side). This measurement shall be increased for each attempted radio link deletion (UE side). This measurement is only valid for FDD mode.

- CC

- On transmission by the serving RNC of a RRC message ACTIVE SET UPDATE from the RNC to the UE, IE "E-DCH reconfiguration information": IE "Primary CPICH info" used for the reference ID along with the IE "E-HICH release indicator" to indicate which radio link to remove from the E-DCH active set. Within an ACTIVE SET UPDATE message more than one radio link can be deleted. Each existing radio link deletion information element shall be considered separately (see TS 25.331 [4]).

- A single integer value

- HSUPA.AttEdchDelUESide

- UtranCellFDD

- Valid for packet switched traffic

- UMTS

##### 4.34.2.2.2	Successful radio link deletions to the E-DCH active set (UE side)

- This measurement provides the number of successful radio link deletions to the E-DCH active set (UE side). This measurement shall be increased for each successful radio link deletion (UE side). This measurement is only valid for FDD mode.

- CC

- On receipt by the serving RNC of a RRC message ACTIVE SET UPDATE COMPLETE from the UE to the RNC, in response to an ACTIVE SET UPDATE message with one or more existing IE "E-DCH reconfiguration information": IE "Primary CPICH info" used for the reference ID along with the IE "E-HICH release indicator" to indicate which radio link to remove from the E-DCH active set. One ACTIVE SET UPDATE COMPLETE message can be related to more than one deleted radio link. Each successful deleted radio link shall be considered separately (see TS 25.331 [4]).

- A single integer value

- HSUPA.SuccEdchDelUESide

- UtranCellFDD

- Valid for packet switched traffic

- UMTS

### 4.34.3	Serving E-DCH cell change

#### 4.34.3.1	Outgoing serving E-DCH cell change

##### 4.34.3.1.1	Attempted outgoing serving E-DCH cell change

- This measurement provides the number of attempted outgoing serving E-DCH cell change.

- CC

- On transmission by the RNC of a RRC message RADIO BEARER SETUP, RADIO BEARER RECONFIGURATION, RADIO BEARER RELEASE, TRANSPORT CHANNEL RECONFIGURATION, or PHYSICAL CHANNEL RECONFIGURATION from the source RNC to the UE, indicating the attempt of outgoing serving E-DCH cell change (see TS 25.331 [4]).

- A single integer value

- HSUPA.AttOutCellChange

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

##### 4.34.3.1.2	Successful outgoing serving E-DCH cell change

- This measurement provides the number of successful outgoing serving E-DCH cell change.

- CC

- On receipt by the RNC of a RRC message RADIO BEARER SETUP COMPLETE, RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER RELEASE COMPLETE, TRANSPORT CHANNEL RECONFIGURATION COMPLETE, or PHYSICAL CHANNEL RECONFIGURATION COMPLETE sent from the UE to the source RNC, indicating a successful outgoing serving E-DCH cell change (see TS 25.331 [4]).

- A single integer value

- HSUPA.SuccOutCellChange

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

#### 4.34.3.2	Outgoing serving E-DCH cell change per neighbour cell relation

##### 4.34.3.2.1	Attempted outgoing serving E-DCH cell change per neighbour cell relation

- This measurement provides the number of attempted outgoing serving E-DCH cell change per neighbour cell relation.

- CC

- On transmission by the RNC of a RRC message RADIO BEARER SETUP, RADIO BEARER RECONFIGURATION, RADIO BEARER RELEASE, TRANSPORT CHANNEL RECONFIGURATION, or PHYSICAL CHANNEL RECONFIGURATION from the source RNC to the UE, indicating the attempt of   outgoing serving E-DCH cell change (see TS 25.331 [4]).

- A single integer value

- HSUPA.AttOut

- UtranRelation

- Valid for packet switched traffic

- UMTS

##### 4.34.3.2.2	Successful outgoing serving E-DCH cell change per neighbour cell relation

- This measurement provides the number of successful outgoing serving E-DCH cell change per neighbour cell relation.

- CC

- On receipt by the RNC of a RRC message RADIO BEARER SETUP COMPLETE, RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER RELEASE COMPLETE, TRANSPORT CHANNEL RECONFIGURATION COMPLETE, or PHYSICAL CHANNEL RECONFIGURATION COMPLETE sent from the UE to the source RNC, indicating a successful outgoing serving E-DCH cell change (see TS 25.331 [4]).

- A single integer value

- HSUPA.SuccOut

- UtranRelation

- Valid for packet switched traffic

- UMTS

### 4.34.4	Mean number of HSUPA users of a serving E-DCH cell

- This measurement provides the mean number of simultaneous HSUPA users in a serving E-DCH cell.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of simultaneous users in a serving E-DCH cell and then taking the arithmetic mean.

- A single real value.

- HSUPA.MeanNbrUser

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS.

### 4.34.5	Measurements related to channel switches between RACH/DCH and E-DCH

#### 4.34.5.1	Measurements related to channel switches from RACH/DCH to E-DCH

##### 4.34.5.1.1	Attempted channel switches from RACH to E-DCH

- This measurement provides the number of attempted channel switches from RACH to E-DCH in the serving E-DCH cell.

- CC

- On transmission by the RNC of a RRC message RADIO BEARER RECONFIGURATION, RADIO BEARER SETUP, RADIO BEARER RELEASE, CELL UPDATE CONFIRM, or TRANSPORT CHANNEL RECONFIGURATION to UE, with the condition that channel switches only from RACH to E-DCH (see TS 25.331 [4]).

- A single integer value

- HSUPA.AttRachToEdch

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

##### 4.34.5.1.2	Successful channel switches from RACH to E-DCH

- This measurement provides the number of successful channel switches from RACH to E-DCH in the serving E-DCH cell.

- CC

- On receipt by the RNC of a RRC message RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER SETUP COMPLETEP, RADIO BEARER RELEASE COMPLETE, or TRANSPORT CHANNEL RECONFIGURATION COMPLETE from UE, with the condition that channel switches only from RACH to E-DCH (see TS 25.331 [4]).

- A single integer value

- HSUPA.SuccRachToEdch

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

##### 4.34.5.1.3	Attempted channel switches from DCH to E-DCH

- This measurement provides the number of attempted channel switches from DCH to E-DCH in the serving E-DCH cell.

- CC

- On transmission by the RNC of a RRC message RADIO BEARER RECONFIGURATION, RADIO BEARER SETUPP, RADIO BEARER RELEASE, CELL UPDATE CONFIRM, or TRANSPORT CHANNEL RECONFIGURATION to UE, with the condition that channel switches from DCH to E-DCH  (see TS 25.331 [4]).

- A single integer value

- HSUPA.AttDchToEdch

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

##### 4.34.5.1.4	Successful channel switches from DCH to E-DCH

- This measurement provides the number of successful channel switches from DCH to E-DCH in the serving E-DCH cell.

- CC

- On receipt by the RNC of a RRC message RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER SETUP COMPLETE, RADIO BEARER RELEASE COMPLETE, or TRANSPORT CHANNEL RECONFIGURATION COMPLETE from UE, with the condition that channel switches from DCH to E-DCH  (see TS 25.331 [4]).

- A single integer value

- HSUPA.SuccDchToEdch

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

#### 4.34.5.2	Measurements related to channel switches from E-DCH to RACH/DCH

##### 4.34.5.2.1	Attempted channel switches from E-DCH to RACH

- This measurement provides the number of attempted channel switches from E-DCH to RACH in the serving E-DCH cell.

- CC

- On transmission by the RNC of a RRC message RADIO BEARER RECONFIGURATION, RADIO BEARER SETUP, RADIO BEARER RELEASE, CELL UPDATE CONFIRM, or TRANSPORT CHANNEL RECONFIGURATION to UE, with the condition that channel switches only from E-DCH to RACH  (see TS 25.331 [4]).

- A single integer value

- HSUPA.AttEdchToRach

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

##### 4.34.5.2.2	Successful channel switches from E-DCH to RACH

- This measurement provides the number of successful channel switches from E-DCH to RACH in the serving E-DCH cell.

- CC

- On receipt by the RNC of a RRC message RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER SETUP COMPLETE, RADIO BEARER RELEASE COMPLETE, or TRANSPORT CHANNEL RECONFIGURATION COMPLETE from UE, with the condition that channel switches only from E-DCH to RACH  (see TS 25.331 [4]).

- A single integer value

- HSUPA.SuccEdchToRach

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

##### 4.34.5.2.3	Attempted channel switches from E-DCH to DCH

- This measurement provides the number of attempted channel switches from E-DCH to DCH in the serving E-DCH cell.

- CC

- On transmission by the RNC of a RRC message RADIO BEARER RECONFIGURATION, RADIO BEARER SETUPP, RADIO BEARER RELEASE, CELL UPDATE CONFIRM, or TRANSPORT CHANNEL RECONFIGURATION to UE, with the condition that channel switches from E-DCH to DCH  (see TS 25.331 [4]).

- A single integer value

- HSUPA.AttEdchToDch

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

##### 4.34.5.2.4	Successful channel switches from E-DCH to DCH

- This measurement provides the number of successful channel switches from E-DCH to DCH in the serving E-DCH cell.

- CC

- On receipt by the RNC of a RRC message RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER SETUP COMPLETE, RADIO BEARER RELEASE COMPLETE, or TRANSPORT CHANNEL RECONFIGURATION COMPLETE from UE, with the condition that channel switches from E-DCH to DCH  (see TS 25.331 [4]).

- A single integer value

- HSUPA.SuccEdchToDch

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic

- UMTS

### 4.34.6	Measurements related to MAC-e feedback decoding

#### 4.34.6.1	Number of received MAC-e PDUs

- This measurement provides the number of received MAC-e PDUs including acknowledged and unacknowledged PDUs during the period of measurement, detected in MAC-e layer in the serving E-DCH cell.

- CC.

- On receipt by the NodeB of a PDU of MAC-e from UE.

- A single integer value.

- HSUPA.NbrMacePdu

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS.

#### 4.34.6.2	Number of received acknowledged MAC-e PDUs

- This measurement provides the number of successfully received MAC-e PDUs that has been acknowledged to the UE during the period of measurement, detected in MAC-e layer in the serving E-DCH cell.

- CC.

- On transmission by the NodeB of a ACK on E-HICH from NodeB to UE indicating a correct receiption a MAC-e PDU.

- A single integer value.

- HSUPA.NbrAckdMacePdu

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS.

### 4.34.7	Number of octets of acknowledged MAC-e PDUs

- This measurement provides the number of octets of uplink acknowledged MAC-e PDUs in the E-DCH cell.

- CC.

- On receipt by the NodeB of an acknowledged PDU on the MAC-e layer. The number of octets of MAC-e layer in one E-DCH cell is calculated by sum of size of each MAC-e PDU header including MAC-e PDU header.

- A single integer value.

- HSUPA.NbrAckdMaceOcts

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS.

### 4.34.8	HSUPA Release measurements

#### 4.34.8.1	Normal HSUPA Release

##### 4.34.8.1.1	RNC initiated release due to user inactivity

- This measurement provides the number of times when the UE is removed from E-DCH transport channel due to user inactivity in the serving E-DCH cell.

- CC.

- On transmission by the RNC of the RRC: RADIO BEARER RECONFIGURATION or RRC: RADIO BEARER RELEASE or RRC CONNECTION RELEASE message that indicates an E-DCH release when RNC detected UE has no data to send in a predefined interval (equals to the RANAP cause 16 user inactivity).

- A single integer value.

- HSUPA.SuccEdchReleaseUserInact

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS.

##### 4.34.8.1.2	RNC initiated release due to switch to DCH/RACH

The measurement definition is in section 4.34.5.2.

##### 4.34.8.1.3	RNC initiated release due to serving E-DCH cell change

The measurement definition is in section 4.34.3.

##### 4.34.8.1.4	CN initiated release

- This measurement provides the number of times when the UE is removed from E-DCH transport channel due to a release initiated by the Core Network in the serving E-DCH cell.

- CC.

- On reception by the RNC of RANAP RAB ASSIGNMENT REQUEST or RANAP IU RELEASE COMMAND that results an E-DCH release. The release of E-DCH is executed by the transmission of the RRC RADIO BEARER RECONFIGURATION or RRC RADIO BEARER RELEASE or RRC CONNECTION RELEASE message.

- A single integer value.

- HSUPA.SuccCnInitEdchRelease

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS.

#### 4.34.8.2	Abnormal RB releases for E-DCH

- This measurement provides the number of times when the UE is removed from E-DCH transport channel, and trigger reason is not a normal release, e.g. user inactivity, switch to DCH/RACH, serving E-DCH cell changes or CN initiated release.

- CC.

- On receipt by the RNC of the RRC: RADIO BEARER RECONFIGURATION COMPLETE corresponding to the transmission of the RRC message RADIO BEARER RECONFIGURATION, or RRC: RADIO BEARER RELEASE COMPLETE corresponding to the transmission of the RRC message RADIO BEARER RELEASE. Or when the UE does not respond to RRC: RADIO BEARER RECONFIGURATION or RRC: RADIO BEARER RELEASE message, and the E-DCH release does not categorize as a normal release, e.g. user inactivity, switch to DCH/RACH, serving E-DCH cell changes or CN initiated release.

- A single integer value.

- HSUPA.AbnormalEdchRelease

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS.

## 4.35	 Radio resource utilization related measurements

### 4.35.1 UTRAN Cell Unavailable Time

- This measurement provides the length of time the cell has been unavailable for each cause.

- DER (n=1)

- This measurement is obtained by accumulating the time periods when the cell is unavailable per cause. The possible cause could be “manual intervention”, “fault” or “transport problem”. The sum of all supported per cause measurements shall equal the total time periods of cell unavailability. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value (in seconds). The number of measurements is equal to the number of supported causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RRU.UTRANCellUnavailableTime.cause.

- Where cause identifies the cause resuling in cell unavailable. 
The cause ”manual intervention” is identified by the .ManualIntervention suffix.

- The cause ”fault” is identified by the .Fault suffix.

- The cause ”transport problem” is identified by the .TransportProblem suffix.

- UtranGenericCell

- Valid for packet switched traffic and circuit switched traffic

- This measurement is to support KPI “UTRAN Cell Availability” defined in [18].

## 4.36	MBMS related measurements

### 4.36.1	Number of octets of MBMS data on MAC layer

  - This measurement provides the number of octets of MBMS data on MAC-m layer.

  - CC

  - On transmission by the RNC of a PDU on the MAC layer for MBMS. The number of octets of MAC layer for MBMS is calculated by sum of PDU size including PDU header.

  - A single integer value.

  - MBMS.NbrOctMac

  - UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

  - Valid for packet switched traffic

  - UMTS

### 4.36.2	Session related measurements

#### 4.36.2.1	Attempted MBMS session start

- This measurement provides the number of attempted MBMS session start.

- CC

- On receipt by the RNC of a MBMS SESSION START message from CN (see TS 25.413 [5]), each MBMS SESSION START message is added to the measurement when MBMS resources attempt to be established in relevant cell.

- A single integer value.

- MBMS.AttSessionStart

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS

#### 4.36.2.2	Successful MBMS session start

- This measurement provides the number of successful MBMS session start.

- CC

- Transmission by the RNC of a MBMS SESSION START RESPONSE message to CN (see TS 25.413 [5]), each MBMS SESSION START RESPONSE message is added to the measurement when MBMS resources have been established in relevant cell.

- A single integer value.

- MBMS.SuccSessionStart

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS

#### 4.36.2.3	Attempted MBMS session stop

- This measurement provides the number of attempted MBMS session stop.

- CC

- On receipt by the RNC of a MBMS SESSION STOP message from CN (see TS 25.413 [5]), each MBMS SESSION STOP message is added to the measurement when MBMS resources attempt to be released in relevant cell.

- A single integer value.

- MBMS.AttSessionStop

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS

#### 4.36.2.4	Successful MBMS session stop

- This measurement provides the number of successful MBMS session stop.

- CC

- Transmission by the RNC of a MBMS SESSION STOP RESPONSE message to CN (see TS 25.413 [5]) , each MBMS SESSION STOP RESPONSE message is added to the measurement when MBMS resources have been released in relevant cell,

- A single integer value.

- MBMS.SuccSessionStop

- UtranCellFDD 
UtranCellTDDLcr 
UtranCellTDDHcr

- Valid for packet switched traffic.

- UMTS

## 4.37	Dual-Cell HSDPA related measurements

### 4.37.1	Dual-Cell HSDPA Scheduling duration in UTRAN cell

- This measurement provides the Dual-Cell HSDPA Scheduling duration in UTRAN cell.

- DER (n=1)

- This measurement is obtained by accumulating the time intervals for each Mac-ehs Scheduling between the start of Dual-Cell Scheduling simultaneously and the end of Dual-Cell Scheduling simultaneously over a granularity period using DER, see TS 25.308 [19].The accumulator shall be reinitialised at the beginning of each granularity period. This measurement shall be accumulated only if the UTRAN cell is the primary cell of a Dual-Cell.

- A single integer value. (in milliseconds)

- HSDPA.DualCellSchedulingDuration

- UtranCellFDD

- Valid for packet switched traffic

- UMTS

### 4.37.2	Individual cell HSDPA Scheduling duration in UTRAN cell

- This measurement provides the individual cell HSDPA Scheduling duration in UTRAN cell.

- DER (n=1)

- This measurement is obtained by accumulating the time intervals for each Mac-ehs Scheduling between the start of individual cell Scheduling and the end of individual cell Scheduling over a granularity period using DER, see TS 25.308 [19].The accumulator shall be reinitialised at the beginning of each granularity period.

- A single integer value. (in milliseconds)

- HSDPA.IndividualCellSchedulingDuration

- UtranCellFDD

- Valid for packet switched traffic

- UMTS

### 4.37.3	Attempted Dual-Cell HSDPA establishments

- This measurement provides the number of Dual-Cell HSDPA establishment attempts.

- CC

- On transmission by the RNC of a NBAP message RADIO LINK SETUP REQUEST with the “HS-DSCH Information” and “Additional HS Cell Information RL Setup” IE; Or on transmission by the RNC of a NBAP message RADIO LINK RECONFIGURATION PREPARE with the “HS-DSCH MAC-d Flows To Add” or “HS-DSCH Information” and “Additional HS Cell Information RL Reconf Prep” IE; Or on transmission by the RNC of a NBAP message RADIO LINK RECONFIGURATION REQUEST with the “HS-DSCH MAC-d Flows To Add” or “HS-DSCH Information” and “Additional HS Cell Information RL Reconf Req” IE. See TS 25.433 [7]. This measurement shall be counted only if the UTRAN cell is the primary cell of a Dual-Cell.

- A single integer values.

- HSDPA.EstabAttDCHSDSCH

- UtranCellFDD

- Valid for packet switched traffic

- UMTS

### 4.37.4	Successful Dual-Cell HSDPA establishments

- This measurement provides the number of successfully established Dual-Cell HSDPA.

- CC

- Receipt of a RRC message PHYSICAL CHANNEL RECONFIGURATION COMPLETE corresponding to PHYSICAL CHANNEL RECONFIGURATION with “Downlink HS-PDSCH Information” IE and “Downlink secondary cell info FDD” IE, RADIO BEARER SETUP COMPLETE corresponding to RADIO BEARER SETUP with “Downlink HS-PDSCH Information” IE and “Downlink secondary cell info FDD” IE, RADIO BEARER RECONFIGURATION COMPLETE corresponding to RADIO BEARER RECONFIGURATION with “Downlink HS-PDSCH Information” IE and “Downlink secondary cell info FDD” IE, or TRANSPORT CHANNEL RECONFIGURATION COMPLETE corresponding to TRANSPORT CHANNEL RECONFIGURATION with “Downlink HS-PDSCH Information” IE and “Downlink secondary cell info FDD” IE,sent from the UE to the source RNC, indicating a successful handover/reconfiguration to Dual-cell HS-DSCH or new Dual-cell HS-DSCH setup. See TS 25.331 [4]. This measurement shall be counted only if the UTRAN cell is the primary cell of a Dual-Cell.

- A single integer values.

- HSDPA.EstabSuccDCHSDSCH

- UtranCellFDD

- Valid for packet switched traffic

- UMTS

### 4.37.5	Failed Dual-Cell HSDPA establishments

- This measurement provides the number of failed Dual-Cell HSDPA establishments. This measurement shall be increased for each failed Dual-Cell HSDPA establishments on Iub/Iur or Uu. For each failure cause a separate measurement is defined. Every failed Dual-Cell HSDPA establishments on Iub/Iur or Uu shall be considered separately. This measurement is valid for FDD mode.

- CC

- Receipt of a RADIO LINK SETUP FAILURE message (NBAP) sent by NodeB to the controlling RNC in response to a RADIO LINK SETUP REQUEST message with the “HS-DSCH Information” and “Additional HS Cell Information RL Setup” IE. One RADIO LINK SETUP FAILURE message can be related to more than one radio link. Each failed attempt to set up a Dual-Cell HSDPA radio link shall be considered separately. Failure causes are defined within TS 25.433 [7].

Each expected RADIO LINK SETUP RESPONSE or RADIO LINK SETUP FAILURE corresponding to RADIO LINK SETUP REQUEST with the “HS-DSCH Information” and “Additional HS Cell Information RL Setup” IE not received by the controlling RNC is added to the measurement cause 'No Reply' (not specified in TS 25.433 [7]).

Receipt of a RRC message PHYSICAL CHANNEL RECONFIGURATION FAILURE corresponding to PHYSICAL CHANNEL RECONFIGURATION with “Downlink HS-PDSCH Information” IE and “Downlink secondary cell info FDD” IE, RADIO BEARER SETUP FAILURE corresponding to RADIO BEARER SETUP with “Downlink HS-PDSCH Information” IE and “Downlink secondary cell info FDD” IE, RADIO BEARER RECONFIGURATION FAILURE corresponding to RADIO BEARER RECONFIGURATION with “Downlink HS-PDSCH Information” IE and “Downlink secondary cell info FDD” IE, or TRANSPORT CHANNEL RECONFIGURATION FAILURE corresponding to TRANSPORT CHANNEL RECONFIGURATION with “Downlink HS-PDSCH Information” IE and “Downlink secondary cell info FDD” IE,sent from the UE to the source RNC, indicating a failed handover/reconfiguration to Dual-cell HS-DSCH or failed new Dual-cell HS-DSCH setup. Each failed attempt to set up a Dual-Cell HS-DSCH shall be considered separately. Failure causes are defined within TS 25.331 [4].

Each expected PHYSICAL CHANNEL RECONFIGURATION COMPLETE or PHYSICAL CHANNEL RECONFIGURATION FAILURE corresponding to PHYSICAL CHANNEL RECONFIGURATION with “Downlink HS-PDSCH Information” IE and “Downlink secondary cell info FDD” IE, RADIO BEARER SETUP COMPLETE  or RADIO BEARER SETUP FAILURE corresponding to RADIO BEARER SETUP with “Downlink HS-PDSCH Information” IE and “Downlink secondary cell info FDD” IE, RADIO BEARER RECONFIGURATION COMPLETE or RADIO BEARER RECONFIGURATION FAILURE corresponding to RADIO BEARER RECONFIGURATION with “Downlink HS-PDSCH Information” IE and “Downlink secondary cell info FDD” IE, TRANSPORT CHANNEL RECONFIGURATION COMPLETE or TRANSPORT CHANNEL RECONFIGURATION FAILURE corresponding to TRANSPORT CHANNEL RECONFIGURATION with “Downlink HS-PDSCH Information” IE and “Downlink secondary cell info FDD” IE  not received by the controlling RNC is added to the measurement cause 'No Reply' (not specified in TS 25.331 [4]).

This measurement shall be counted only if the UTRAN cell is the primary cell of a Dual-Cell.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form HSDPA.EstabFailDCHSDSCH.Cause
where Cause identifies the failure cause.

- UtranCellFDD

- Valid for packet switched traffic

- UMTS

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Sep 2007 | SA_37 | SP-070613 | 0031 | -- | Correct ISCP (Interference Signal Code Power) measurements for TDD - Align with 25.123 Requirements for support of radio resource management (TDD) | F | 7.5.0 | 7.6.0 |
| Sep 2007 | SA_37 | SP-070614 | 0032 | -- | Add RNC processor usage related measurements | B | 7.6.0 | 8.0.0 |
| Dec 2007 | SA_38 | SP-070734 | 0035 | -- | Correct measurements about the transmitted carrier power not used for HSPA transmission for TDD - Align with 25.225 | A | 8.0.0 | 8.1.0 |
| Dec 2007 | SA_38 | SP-070744 | 0036 | -- | Add measurements related to E-DCH active set | B | 8.0.0 | 8.1.0 |
| Dec 2007 | SA_38 | SP-070744 | 0037 | -- | Add measurements related to serving E-DCH cell change | B | 8.0.0 | 8.1.0 |
| Dec 2007 | SA_38 | SP-070744 | 0038 | -- | Add measurements related to HSUPA users | B | 8.0.0 | 8.1.0 |
| Dec 2007 | SA_38 | SP-070744 | 0039 | -- | Add measurements related to HSUPA setup | B | 8.0.0 | 8.1.0 |
| Mar 2008 | SP-39 | SP-080069 | 0040 | -- | Correct measurement name for abnormal RB releases for HS-DSCH - Replace HS by HSDPA | F | 8.1.0 | 8.2.0 |
| Mar 2008 | SP-39 | SP-080069 | 0041 | -- | Add measurements related to channel switches between E-DCH and RACH/DCH - Channel switch success rate for HSUPA | B | 8.1.0 | 8.2.0 |
| Mar 2008 | SP-39 | SP-080069 | 0042 | -- | Add measurements related to MAC-e feedback decoding - HSUPA retransmission efficiency | B | 8.1.0 | 8.2.0 |
| Mar 2008 | SP-39 | SP-080069 | 0043 | -- | Add measurement related to octets of acknowledged MAC-e PDUs - HSUPA data volume | B | 8.1.0 | 8.2.0 |
| Mar 2008 | SP-39 | SP-080069 | 0044 | -- | Add measurements related to HSUPA release | B | 8.1.0 | 8.2.0 |
| Jun 2008 | SP-40 | SP-080328 | 0046 | -- | Correct measurements related to TDD UTRAN cell Radio Frequency Carrier | A | 8.2.0 | 8.3.0 |
| Sep 2008 | SP-41 | SP-080465 | 0047 | -- | Correct measurements related to Mac-d setups for HSUPA | F | 8.3.0 | 8.4.0 |
| Sep 2008 | SP-41 | SP-080465 | 0048 | -- | Correct measurements related to channel switch between RACH/DCH and E-DCH | F | 8.3.0 | 8.4.0 |
| Sep 2008 | SP-41 | SP-080465 | 0049 | 1 | Change name and definition of measurements related to serving E-DCH cell change | F | 8.3.0 | 8.4.0 |
| Sep 2008 | SP-41 | SP-080465 | 0050 | 1 | Add measurements related to serving E-DCH cell change | B | 8.3.0 | 8.4.0 |
| Mar 2009 | SP-43 | SP-090207 | 0051 | -- | Add Cell Unavailable Time Measurements | C | 8.4.0 | 8.5.0 |
| Mar 2009 | SP-43 | SP-090207 | 0052 | -- | Addition of MBMS data volume related measurements | B | 8.4.0 | 8.5.0 |
| Mar 2009 | SP-43 | SP-090207 | 0053 | -- | Inclusion of WB-AMR Rates | C | 8.4.0 | 8.5.0 |
| Jun 2009 | SP-44 | SP-090290 | 0054 | -- | Addition of MBMS session related measurements | F | 8.5.0 | 9.0.0 |
| Sep 2009 | SP-45 | SP-090627 | 0055 | -- | Add performance measurements for Dual-Cell HSDPA | B | 9.0.0 | 9.1.0 |