3GPP TS 32.762 V9.1.0 (2009-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects; Telecommunications management;

Evolved Universal Terrestrial Radio Access Network

(E-UTRAN) Network Resource Model (NRM)

Integration Reference Point (IRP):

Information Service (IS)

(Release 9)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

E-UTRAN, management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2009, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	8

4	System overview	8

4.1	Compliance rules	8

5	Modelling approach	8

6	Information Object Classes (IOCs)	9

6.1	Information entities imported and local labels	9

6.2	Class diagram	9

6.2.1	Attributes and relationships	9

6.2.2	Inheritance	11

6.3	Information Object Class (IOC) definitions	12

6.3.1	ENBFunction	12

6.3.1.1	Definition	12

6.3.1.2	Attributes	12

6.3.1.3	Attribute constraints	12

6.3.1.4	Notifications	12

6.3.2	ExternalENBFunction	12

6.3.2.1	Definition	12

6.3.2.2	Attributes	12

6.3.2.3	Attribute constraints	12

6.3.2.4	Notifications	12

6.3.3 	EUtranGenericCell	13

6.3.3.1	Definition	13

6.3.3.2	Attributes	13

6.3.3.3	Attribute constraints	13

6.3.3.4	Notifications	13

6.3.4	ExternalEUtranGenericCell	13

6.3.4.1	Definition	13

6.3.4.2	Attributes	14

6.3.4.3	Attribute constraints	14

6.3.4.4	Notifications	14

6.3.5	EUtranCellFDD	14

6.3.5.1	Definition	14

6.3.5.2	Attributes	14

6.3.5.3	Attribute constraints	14

6.3.5.4	Notifications	14

6.3.6	ExternalEUtranCellFDD	14

6.3.6.1	Definition	14

6.3.6.2	Attributes	15

6.3.6.3	Attribute constraints	15

6.3.6.4	Notifications	15

6.3.7	EUtranCellTDD	15

6.3.7.1	Definition	15

6.3.7.2	Attributes	15

6.3.7.3	Attribute constraints	15

6.3.7.4	Notifications	15

6.3.8	ExternalEUtranCellTDD	15

6.3.8.1	Definition	15

6.3.8.2	Attributes	15

6.3.8.3	Attribute constraints	15

6.3.8.4	Notifications	15

6.3.9	EUtranRelation	16

6.3.9.1	Definition	16

6.3.9.2	Attributes	16

6.3.9.3	Attribute constraints	16

6.3.9.4	Notifications	16

6.3.10	Link_ENB_ENB	16

6.3.10.1	Definition	16

6.3.10.2	Attributes	16

6.3.10.3	Attribute constraints	16

6.3.10.4	Notifications	16

6.3.11	EP_RP_EPS	16

6.3.11.1	Definition	16

6.3.11.2	Attributes	17

6.3.11.3	Attribute constraints	17

6.3.11.4	Notifications	17

6.3.12	SectorEquipmentFunction	17

6.3.12.1	Definition	17

6.3.12.2	Attributes	17

6.3.12.3	Attribute constraints	17

6.3.12.4	Notifications	17

6.3.13	Cdma2000Relation	17

6.3.13.1	Definition	17

6.3.13.2	Attributes	18

6.3.13.3	Attribute constraints	18

6.3.13.4	Notifications	18

6.3.14	ProxyGsmCell	18

6.3.14.1	Definition	18

6.3.15	ProxyUtranCell	18

6.3.15.1	Definition	18

6.4	Information relationship definitions	18

6.4.1	EUtranNeighbourCellRelation (M)	18

6.4.1.1	Definition	18

6.4.1.2	Roles	18

6.4.1.3	Constraints	18

6.4.2	ExternalEUtranNeighbourCellRelation (M)	19

6.4.2.1	Definition	19

6.4.2.2	Roles	19

6.4.2.3	Constraints	19

6.4.3	ExternalCdma2000NeighbourCellRelation (M)	19

6.4.3.1	Definition	19

6.4.3.2	Roles	19

6.4.3.3	Constraints	19

6.5	Information attribute definitions	20

6.5.1	Definition and legal values	20

6.5.2	Constraints	23

6.6	Common Notifications	23

6.6.1	Alarm and configuration notifications	23

6.6.2	Configuration notifications	23

6.7	System State Model	23

Annex A (informative):	Change history	24

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; as identified below:

32.761	Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Requirements

32.762	Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)

32.763	Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Common Object Request Broker Architecture (CORBA) Solution Set (SS)

32.765	Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): eXtensible Markup Language (XML) file format definition

# 1	Scope

The present document is part of an Integration Reference Point (IRP) named E-UTRAN Network Resource Model (NRM) IRP, through which an IRPAgent can communicate configuration management information to one or several IRPManagers concerning E-UTRAN resources. The E-UTRAN NRM IRP comprises a set of specifications defining Requirements, a protocol neutral Information Service and one or more Solution Set(s).

The present document specifies the protocol neutral E-UTRAN NRM IRP: Information Service (IS). It reuses relevant parts of the Generic NRM IRP: IS in 3GPP TS 32.622 [6], either by direct reuse or sub-classing, and in addition to that defines E-UTRAN specific Information Object Classes.

In order to access the information defined by this NRM, an Interface IRP such as the "Basic CM IRP" is needed (3GPP TS 32.602 [7]). However, which Interface IRP is applicable is outside the scope of the present document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 23.003: "Numbering, addressing and identification".

[4]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[5]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[6]	3GPP TS 32.622: "Telecommunication management; Configuration Management (CM); Generic network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[7]	3GPP TS 32.602: "Telecommunication management; Configuration Management (CM); Basic CM Integration Reference Point (IRP) Information Service (IS)".

[8]	3GPP TS 32.612: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP): Information Service (IS)".

[9]	3GPP TS 23.401: "Technical Specification Group Services and System Aspects; General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[10]	3GPP TS 36.331: "Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol specification".

[11]	3GPP TS 36.300: " Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 ".

[12]	3GPP TS 36.211: “Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation”

[13]	3GPP TS 36.101: “Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”

[14]	3GPP TS 36.104: “Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E_UTRA); Base Station (BS) radio transmission and reception”

[15]	3GPP TS 32.500: " Technical Specification Group Services and System Aspects; Telecommunication Management; Self-Organizing Networks (SON); Concepts and requirements"

[16]	3GPP TS 32.150: " Technical Specification Group Services and System Aspects; Telecommunication management; Integration Reference Point (IRP) Concept and definitions"

[17]	3GPP TS 21.905: " Technical Specification Group Services and System Aspects; Vocabulary for 3GPP Specifications"

[18]	3GPP TS 32.111-2: “Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)”

[19]	3GPP TS 23.002: “Network Architecture”

[20]	3GPP TS 32.652: “Telecommunication management; Configuration Management (CM); GERAN network resources Integration Reference Point (IRP); Network Resource Model (NRM)”

[21]	3GPP TS 32.642: “Telecommunication management; Configuration Management (CM); UTRAN network resources Integration Reference Point (IRP); Network Resource Model (NRM)”

[22]	3GPP2 S.S0028-D “OAM&P for cdma2000 (Overview, 3GPP R7 Delta Specification,  3GPP2 Network Resource Model IRP)”

[23]	3GPP TS 32.752: “Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)”

[24]	3GPP TS 36.423: “Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol (X2AP)”

[25]	3GPP TS 36.213: “Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures”

[26]	3GPP TS 32.672: “Technical Specification Group Services and System Aspects; Telecommunication management; Configuration Management (CM); State Management Integration Reference Point (IRP); Information Service (IS)”

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TS 32.150 [16], TS 32.101 [1], TS 32.102 [2] and TS 21.905 [17] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TS 32.150 [16], TS 32.101 [1], TS 32.102 [2] and TS 21.905 [17], in that order.

Association: In general it is used to model relationships between Managed Objects. Associations can be implemented in several ways, such as:

- name bindings,

- reference attributes, and

- association objects.

This IRP stipulates that containment associations shall be expressed through name bindings, but it does not stipulate the implementation for other types of associations as a general rule. These are specified as separate entities in the object models (UML diagrams).

Managed Element (ME): An instance of the Information Object Class ManagedElement defined in TS 32.622 [6].

eNodeB: A logical node responsible for radio transmission/reception in one or more cells to/from the User Equipment. 
It terminates the S1 interface towards the EPC.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TS 32.150 [16], TS 32.101 [1], TS 32.102 [2] and TS 21.905 [17] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TS 32.150 [16], TS 32.101 [1], TS 32.102 [2] and TS 21.905 [17], in that order.

DN	Distinguished Name (see 3GPP TS 32.300 [4])

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

ME	Managed Element

MO	Managed Object

NR	Neighbour cell Relation

PM	Performance Management

RDN	Relative Distinguished Name (see 3GPP TS 32.300 [4])

# 4	System overview

## 4.1	Compliance rules

The following defines the meaning of Mandatory and Optional IOC attributes and associations between IOCs, in Solution Sets to the IRP defined by the present document:

- The IRPManager shall support all mandatory attributes/associations. The IRPManager shall be prepared to receive information related to mandatory as well as optional attributes/associations without failure; however the IRPManager does not have to support handling of the optional attributes/associations.

- The IRPAgent shall support all mandatory attributes/associations. It may support optional attributes/associations.

An IRPAgent that incorporates vendor-specific extensions shall support normal communication with a 3GPP SA5-compliant IRPManager with respect to all Mandatory and Optional information object classes, attributes and associations without requiring the IRPManager to have any knowledge of the extensions.

Given that

- rules for vendor-specific extensions remain to be fully specified, and

- many scenarios under which IRPManager and IRPAgent interwork may exist,

it is recognised that the IRPManager, even though it is not required to have knowledge of vendor-specific extensions, may be required to be implemented with an awareness that extensions can exist and behave accordingly.

# 5	Modelling approach

The modelling approach adopted and used in this IRP is described in TS 32.622 [6].

# 6	Information Object Classes (IOCs)

## 6.1	Information entities imported and local labels

| Label reference | Local label |
| --- | --- |
| 3GPP TS 32.672 [26], attribute, administrativeState | administrativeState |
| 3GPP TS 32.672 [26], attribute, availabilityStatus | availabilityStatus |
| 3GPP TS 32.672 [26], attribute, operationalState | operationalState |
| 3GPP TS 32.622 [6], IOC, Top | Top |
| 3GPP TS 32.622 [6], IOC, ManagedElement | ManagedElement |
| 3GPP TS 32.622 [6], IOC, SubNetwork | SubNetwork |
| 3GPP TS 32.622 [6], IOC, ManagedFunction | ManagedFunction |
| 3GPP TS 32.622 [6], IOC, Link | Link |
| 3GPP TS 32.622 [6], IOC, EP_RP | EP_RP |
| 3GPP TS 32.752 [23], IOC, MMEFunction | MMEFunction |
| 3GPP TS 32.752 [23], IOC, ExternalMMEFunction | ExternalMMEFunction |
| 3GPP TS 32.642 [21], IOC, UtranRelation | UtranRelation |
| 3GPP TS 32.642 [21], IOC, UtranGenericCell | UtranGenericCell |
| 3GPP TS 32.642 [21], IOC, ExternalUtranGenericCell | ExternalUtranGenericCell |
| 3GPP TS 32.642 [21], IOC, AntennaFunction | AntennaFunction |
| 3GPP TS 32.642 [21], IOC, TmaFunction | TmaFunction |
| 3GPP TS 32.652 [20], IOC, GsmRelation | GsmRelation |
| 3GPP TS 32.652 [20], IOC, GsmCell | GsmCell |
| 3GPP TS 32.652 [20], IOC, ExternalGsmCell | ExternalGsmCell |
| 3GPP2 TS S.S0028 [22], IOC, ExternalSector | ExternalSector |

## 6.2	Class diagram

### 6.2.1	Attributes and relationships

Figure 6.2.1.1: Cell view of EUTRAN NRM

Figure 6.2.1.2: Transport view of EUTRAN NRM

Figure 6.2.1.3: Radio equipment view of EUTRAN NRM

Note 1: Either the EUtranGenericCell has a relation to SectorEquipmentFunction, or the EUtranGenericCell has relations to AntennaFunction and TMAFunction.

### 6.2.2	Inheritance

Figure 6.2.2.1: E-UTRAN NRM Inheritance Hierarchy

## 6.3	Information Object Class (IOC) definitions

### 6.3.1	ENBFunction

#### 6.3.1.1	Definition

This IOC represents eNB functionality. For more information about the eNB, see 3GPP TS 23.002 [19].

#### 6.3.1.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| x2BlackList | CM | M | M |
| x2WhiteList | CM | M | M |
| x2HOBlackList | CM | M | M |
| x2IpAddressList | O | M | - |

#### 6.3.1.3	Attribute constraints

| Name | Definition |
| --- | --- |
| x2BlackList Support Qualifier | The condition is “ANR function is supported”. |
| x2WhiteList Support Qualifier | The condition is “ANR function is supported”. |
| x2HOBlackList Support Qualifier | The condition is “ANR function is supported”. |

#### 6.3.1.4	Notifications

The common notifications defined in subclause 6.6.1 are valid for this IOC, without exceptions or additions.

### 6.3.2	ExternalENBFunction

#### 6.3.2.1	Definition

This IOC represents an external eNB functionality. For more information about the eNB, see 3GPP TS 23.002 [19].

#### 6.3.2.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |

#### 6.3.2.3	Attribute constraints

None.

#### 6.3.2.4	Notifications

The common notifications defined in subclause 6.6.2 are valid for this IOC, without exceptions or additions.

### 6.3.3 	EUtranGenericCell

#### 6.3.3.1	Definition

This abstract IOC represents the common properties of an E-UTRAN generic cell. For more information about cells, see 3GPP TS 23.401 [9].

#### 6.3.3.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| cellIdentity | M | M | M |
| cellType | M | M | - |
| cellSize | M | M | M |
| plmnIdList | M | M | M |
| tac | M | M | M |
| pci | M | M | CM |
| pciList | CM | M | M |
| maximumTransmissionPower | M | M | CM |
| referenceSignalPower | M | M | M |
| pb | M | M | M |
| partOfSectorPower | CM | M | M |

| Attribute Name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| operationalState | O | M |  |
| administrativeState | O | M | M |
| availabilityStatus | O | M | - |
| NOTE: No state or status propagation shall be implied. |  |  |  |

#### 6.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| pci CM Write Qualifier | Centralized PCI assignment (see TS 32.500, ref [15] subclause 6.1.6) is supported. |
| pciList CM Support Qualifier | Distributed PCI assignment (see TS 32.500, ref [15] subclause 6.1.6) is supported. |
| partOfSectorPower CM support qualifier | The IOC SectorEquipmentFunction is used. |
| maximumTransmissionPower CM Write Qualifier | The IOC SectorEquipmentFunction is not used. |

#### Notifications

The common notifications defined in subclause 6.6.1 are valid for this IOC, without exceptions or additions.

### 6.3.4	ExternalEUtranGenericCell

#### 6.3.4.1	Definition

This abstract IOC represents the properties of an EUTRAN generic cell controlled by another IRPAgent. This IOC contains necessary attributes for inter-system and intra-system handover. It also contains a subset of the attributes of related IOCs controlled by another IRPAgent. The way to maintain consistency between the attribute values of these IOCs is outside the scope of the present document.

#### 6.3.4.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| pci | M | M | M |
| plmnIdList | M | M | M |
| cellIdentity | M | M | M |

#### 6.3.4.3	Attribute constraints

None.

#### Notifications

The common notifications defined in subclause 6.6.2 are valid for this IOC, without exceptions or additions.

### 6.3.5	EUtranCellFDD

#### 6.3.5.1	Definition

This IOC represents the properties of EUTRAN FDD cell.

#### 6.3.5.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| earfcnDl | M | M | M |
| earfcnUl | M | M | M |

#### 6.3.5.3	Attribute constraints

None.

#### 6.3.5.4	Notifications

The common notifications defined in subclause 6.6.1 are valid for this IOC, without exceptions or additions.

### 6.3.6	ExternalEUtranCellFDD

#### 6.3.6.1	Definition

This IOC represents the common properties of external EUTRAN FDD cell.

#### 6.3.6.2	Attributes

Table 6.3.5.2.1: Attributes of ExternalEUtranCellFDD

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| earfcnDl | M | M | M |
| earfcnUl | M | M | M |

#### 6.3.6.3	Attribute constraints

None.

#### 6.3.6.4	Notifications

The common notifications defined in subclause 6.6.2 are valid for this IOC, without exceptions or additions.

### 6.3.7	EUtranCellTDD

#### 6.3.7.1	Definition

This IOC represents the properties of EUTRAN cell TDD.

#### 6.3.7.2	Attributes

None.

#### 6.3.7.3	Attribute constraints

None.

#### 6.3.7.4	Notifications

The common notifications defined in subclause 6.6.1 are valid for this IOC, without exceptions or additions.

### 6.3.8	ExternalEUtranCellTDD

#### 6.3.8.1	Definition

This IOC represents the common properties of external EUTRAN cell TDD.

#### Attributes

None.

#### 6.3.8.3	Attribute constraints

None.

#### 6.3.8.4	Notifications

The common notifications defined in subclause 6.6.2 are valid for this IOC, without exceptions or additions.

### 6.3.9	EUtranRelation

#### 6.3.9.1	Definition

This IOC represents a NR from one EUtranGenericCell instance to another EUtranGenericCell or ExternalEUtranGenericCell instance. NRs are directional.

#### 6.3.9.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| tCI | M | M | M |
| isRemoveAllowed | CM | M | M |
| isHOAllowed | CM | M | M |
| adjacentCell | M | M | - |
| isICICInformationSendAllowed | CM | M | M |

#### 6.3.9.3	Attribute constraints

| Name | Definition |
| --- | --- |
| isRemoveAllowed Support Qualifier | The condition is “ANR function is supported”. |
| isHOAllowed Support Qualifier | The condition is “ANR function is supported”. |
| isICICInformationSendAllowed Support Qualifier | The condition is “ICIC function is supported”. |

#### 6.3.9.4	Notifications

The common notifications defined in subclause 6.6.1 are valid for this IOC, without exceptions or additions.

### 6.3.10	Link_ENB_ENB

#### 6.3.10.1	Definition

This IOC represents the link between two ENBFunction.

#### Attributes

None.

#### 6.3.10.3	Attribute constraints

None.

#### 6.3.10.4	Notifications

The common notifications defined in subclause 6.6.1 are valid for this IOC, without exceptions or additions.

### 6.3.11	EP_RP_EPS

#### 6.3.11.1	Definition

This IOC represents a generic end point for the EUTRAN system.

#### 6.3.11.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| farEndNeIpAddr | O | M | CM |

#### 6.3.11.3	Attribute constraints

| Name | Definition |
| --- | --- |
| farEndNeIpAddr’s write qualifier | When the EP_RP_EPS object belongs to the same Domain Manager as the eNB pointed by the farEndNeIpAddr attribute, the Write Qualifier of farEndNeIpAddr attribute is not needed. |

#### 6.3.11.4	Notifications

The common notifications defined in subclause 6.6.1 are valid for this IOC, without exceptions or additions.

### 6.3.12	SectorEquipmentFunction

#### 6.3.12.1	Definition

This IOC represents a set of cells within a geographical area that has common functions relating to AntennaFunction, TMAFunction and supporting equipment, such as power amplifier.

#### 6.3.12.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| fqBand | M | M | -- |
| confOutputPower | M | M | M |

#### 6.3.12.3	Attribute constraints

None.

#### 6.3.12.4	Notifications

The common notifications defined in subclause 6.6.1 are valid for this IOC, without exceptions or additions.

### 6.3.13	Cdma2000Relation

#### 6.3.13.1	Definition

This IOC represents a NR from one EUtranGenericCell to a CDMA2000 sector. NRs are directional.

See 3GPP2 TS S.S0028 [22]

#### 6.3.13.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| adjacentSector | M | M | - |

#### 6.3.13.3	Attribute constraints

None.

#### 6.3.13.4	Notifications

The common notifications defined in subclause 6.6.1 are valid for this IOC, without exceptions or additions.

### 6.3.14	ProxyGsmCell

#### 6.3.14.1	Definition

This definition is a Proxy Class stereotype, proxying either a GsmCell or an ExternalGsmCell. See TS 32.652 [20].

### 6.3.15	ProxyUtranCell

#### 6.3.15.1	Definition

This definition is a Proxy Class stereotype, proxying either an UtranCell or an ExternalUtranCell. See TS 32.642 [21].

## 6.4	Information relationship definitions

### 6.4.1	EUtranNeighbourCellRelation (M)

#### 6.4.1.1	Definition

This association represents the unidirectional Neighbour cell Relation (NR) from the EUtranGenericCell containing this EUtranRelation to another EUtranGenericCell.

#### 6.4.1.2	Roles

| Name | Definition |
| --- | --- |
| adjacentCell | This role represents the associated EUtranGenericCell of an EUtranNeighbourCellRelation . |

#### 6.4.1.3	Constraints

Associations EUtranNeighbourCellRelation and  ExternalEUtranNeighbourCellRelation are mutually exclusive.

### 6.4.2	ExternalEUtranNeighbourCellRelation (M)

#### 6.4.2.1	Definition

This association represents the unidirectional Neighbour cell Relation (NR) from the EUtranGenericCell containing this EUtranRelation to an ExternalEUtranGenericCell.

#### 6.4.2.2	Roles

| Name | Definition |
| --- | --- |
| adjacentCell | This role represents the associated ExternalEUtranGenericCell of an ExternalEUtranNeighbourCellRelation. |

#### 6.4.2.3	Constraints

Associations EUtranNeighbourCellRelation and  ExternalEUtranNeighbourCellRelation are mutually exclusive.

### 6.4.3	ExternalCdma2000NeighbourCellRelation (M)

#### 6.4.3.1	Definition

This association represents the unidirectional Neighbour cell Relation (NR) from the EUtranGenericCell containing this Cdma2000Relation to an ExternalSector.

#### 6.4.3.2	Roles

| Name | Definition |
| --- | --- |
| adjacentSector | This role represents the associated ExternalSector of an ExternalCdma2000NeighbourCellRelation. |

#### 6.4.3.3	Constraints

## 6.5	Information attribute definitions

### 6.5.1	Definition and legal values

Table 6.5.1.1 defines the attributes that are present in several Information Object Classes (IOCs) of the present document.

Table 6.5.1.1: Attributes definitions and legal values

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| adjacentCell | This attribute contains the DN of a EUtranGenericCell or ExternalEUtranGenericCell. |  |
| adjacentSector | This attribute contains the DN of an ExternalSector. |  |
| cellIdentity | Unambiguously identify a cell within a PLMN. | Refer to TS 36.331[10] |
| cellSize | See cell-Size in TS 36.423 [24]. | See cell-Size in TS 36.423 [24]. |
| cellType | Cell type for management | Enumerated {femto, pico, macro}. |
| confOutputPower | It defines the allowed total power to use for all cells together in this sector. It may be set by the operator and/or limited by HW limitation or licensed power, e.g.: 20, 40, 60, 80,120 watts |  |
| referenceSignalPower | This defines the cell specific downlink reference signal transmit power, which is described in 3GPP TS 36.213[25] | See 3GPP TS 36.331[10] |
| earfcnUl | Specifies the channel number for the central UL frequency. The mapping from channel number to physical frequency is described in 3GPP specification TS 36.101 [13] subclause 5.7.3. | See EARFCN in TS 36.101 [13] subclause 5.7.3. |
| earfcnDl | Specifies the channel number for the central DL frequency. The mapping from channel number to physical frequency is described in 3GPP specification TS 36.101 [13] subclause 5.7.3. | See EARFCN in TS 36.101 [13] subclause 5.7.3 |
| farEndNeIpAddr | The IP address(s) of the far end network entity to which the reference point is related. This is an IPv4 or an IPv6 address. |  |
| fqBand | This is the frequency band supported by the hardware associated with the SectorEquipmentFunction. The earfcnDl and earfcnUl of cells associated with the SectorEquipmentFunction must be assigned with value within this fqBand value. | See section 5 Table 5.2-1 “E-UTRA frequency band” of TS 36.104 [14]. Other legal values would be applicable for other technologies such as for UTRA. |
| id | An attribute whose "name+value" can be used as an RDN when naming an instance of the object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| isRemoveAllowed (see note 1) | This indicates if the subject EUtranRelation can be removed (deleted) or not. If ‘yes’, the subject EUtranRelation instance can be removed (deleted). If ‘no’, the subject EUtranRelation instance shall not be removed (deleted) by any entity but an IRPManager. | yes, no |
| isHOAllowed (see note 1) | This indicates if HO is allowed or prohibited. If ‘yes’, handover is allowed from source cell to target cell. The source cell is identified by the name-containing EUtranGenericCell of the EUtranRelation that has the isHOAllowed. The target cell is referenced by the EUtranRelation that has this isHOAllowed. If ‘no’, handover shall not be allowed. | yes, no |
| isICICInformationSendAllowed | This indicates if ICIC (Inter Cell Interference Coordination) load information message (see TS 36.423 [24] Section 9.1.2.1 LOAD INFORMATION) sending is allowed or prohibited. If ‘yes’, ICIC load information message sending is allowed from source cell to target cell. The source cell is identified by the name-containing EUtranGenericCell of the EUtranRelation that has the isICICInformationSendAllowed. The target cell is referenced by the EUtranRelation that has this isICICInformationSendAllowed. If ‘no’, ICIC load information message sending shall not be allowed. | yes, no |
| maximumTransmissionPower | This is the maximum possible for all downlink channels, used simultaneously in a cell, added together. |  |
| partOfSectorPower | This is the requested part (i.e. %) of the total radio power available to the SectorEquipmentFunction. The requested % power should be allocated to the cell. | 0 : 100 |
| pb | PB, which is described in Section 5.2 of TS 36.213 [25] | See 3GPP TS 36.213[25] |
| pci | This holds the Physical Cell Identity (PCI) of the cell (for both Centralized and Distributed PCI assignment cases). In the case of Centralized PCI assignment, see TS 36.300, [11] subclause 22.3.5, IRPManager signals a specific value by writing this attribute. | See TS 36.211 [12] subclause 6.11 for legal values of pci. |
| pciList | This holds a list of physical cell identities that can be assigned to the pci attribute by eNB. The assignment algorithm is not specified. This attribute shall be supported if and only if the Distributed PCI Assignment is supported. See TS 32.500, ref [15] subclause 6.1.6. | See TS 36.211 [12] subclause 6.11 for legal values of pci. The number of pci in the list is 1 to 504. |
| plmnIdList | List of unique identities for PLMN. Note: A cell can broadcast up to 6 PLMN-id's. This is to support the case that one cell can be used by up to 6 operators’ core networks. See TS 36.331 [10] section 6.2.2: SystemInformationBlockType1/cellAccessRelatedInformation/plmn-IdentityList is a SEQUENCE (SIZE (1..6)) | A list of at most six entries of PLMN Identifiers. The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC). See TS 23.003 [3] subclause 2.2 and 12.1. |
| tac | Common Tracking Area Code for the PLMNs. The identity used to identify tracking areas. | - It is the Tracking Area Code (TAC). - A cell can only broadcast one TAC. See TS 36.300 [11], section 10.1.7 (PLMNID and TAC relation). - TAC is defined in TS 23.003 [3], section 19.4.2.3. |
| tCI | This is the Target Cell Identifier. It consists of E-UTRAN Cell Global Identifier (ECGI) and Physical Cell Identifier (PCI) of the target cell. The EUtranRelation.tCI identifies the target cell from the perspective of the EUtranGenericCell, the name-containing instance of the subject EUtranRelation instance. | The Target Cell Identifier is defined in TS 36.300 [11]. See TS 36.211 [12] subclause 6.11 for legal values of the PCI. |
| x2BlackList | This is a list of DNs of ENBFunction and ExternalENBFunction. If the target node DN is a member of the source node’s ENBFunction.x2BlackList, the source node is: - Prohibited from sending X2 connection request to target node; - Forced to tear down established X2 connection to target node - Not allowed to accept incoming X2 connection request from target node. The same DN may appear here and in ENBFunction.x2WhiteList. In such case, the DN in x2WhiteList shall be treated as if it is absent. |  |
| x2IpAddressList | Represents one or more IP addresses used by ENBFunction for this ENBFunction’s X2 Interface | One or more IPv4 or IPv6 addresses |
| x2WhiteList | This is a list of DNs of ENBFunction and ExternalENBFunction. If the target node DN is a member of the source node’s ENBFunction.x2WhiteList, the source node : - Is allowed to request the establishment of X2 connection with the target node; - Is not allowed to initiate the tear down of established X2 connection to target node The same DN may appear here and in ENBFunction.x2BlackList. In such case, the DN here shall be treated as if it is absent. |  |
| x2HOBlackList | This is a list of DNs of ENBFunction. The ENBFunction.x2HOBlackList identifies a list of neighbour ENBFunction with whom the subject ENBFunction is prohibited to use X2 interface for HOs even if the X2 interface exists between them. |  |

NOTE: Attributes isRemoveAllowed and isHOAllowed each has 2 legal values, allow (A) and prohibited (P).  The two attributes are semantically equivalent to one attribute with 4 legal values such as:

hOAllow; hOProhibited; hOWhiteListed; hOBlackListed;

where

-	hOAllow == isRemoveAllowed is A and isHOAllowed is A;

-	hOProhibited == isRemoveAllowed is A and isHOAllowed is P;

-	hOWhiteListed == isRemoveAllowed is P and isHOAllowed is A;

-	hOBlackListed == isRemoveAllowed is P and isHOAllowed is P.

Therefore, the choice of an option is FFS.

### 6.5.2	Constraints

None.

## 6.6	Common Notifications

### 6.6.1	Alarm and configuration notifications

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyAttributeValueChange | O |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |
| notifyComments | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyAlarmListRebuilt | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyPotentialFaultyAlarmList | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |

Note that these notifications are issued based on occurrences on the IRPAgent IOC and not on occurrences on other IOCs.

### 6.6.2	Configuration notifications

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAttributeValueChange | O |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |

Note that these notifications are issued based on occurrences on the IRPAgent IOC and not on occurrences on other IOCs.

## 6.7	System State Model

None.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Dec 2008 |  |  |  |  | Presentation to SA for information |  | --- | 1.0.0 |
| Mar 2009 | SP-43 | SP-090074 | -- | -- | Presentation to SA for approval |  | 2.0.0 | 8.0.0 |
| Jun 2009 | SP-44 | SP-090408 | 001 | - | Cleanup, updated figures and improved definitions | F | 8.0.0 | 8.1.0 |
| Jun 2009 | SP-44 | SP-090289 | 002 | - | Clarify x2Whitelist definition | F | 8.0.0 | 8.0.0 |
| Jun 2009 | SP-44 | SP-090408 | 004 | - | Add the missing cellSize attribute in EUtranGenericCell IOC - align with 36.423 | F | 8.0.0 | 8.1.0 |
| Jun 2009 | SP-44 | SP-090408 | 006 | - | IOC Relations and UML updates | F | 8.0.0 | 8.1.0 |
| Jun 2009 | SP-44 | SP-090408 | 007 | - | Add missing IOCs in the Class Diagram | F | 8.0.0 | 8.1.0 |
| Jun 2009 | SP-44 | SP-090408 | 008 | - | Add the missing downlink power related attributes for EUTRAN Cell - align with 36.213 and 36.331 | F | 8.0.0 | 8.1.0 |
| Jun 2009 | SP-44 | SP-090289 | 003 | - | Add downlink power related attributes for EUTRAN Cell | F | 8.0.0 | 8.1.0 |
| Jun 2009 | SP-44 | SP-090290 | 005 | - | Add ICIC management attribute in EUtranRelation | B | 8.1.0 | 9.0.0 |
| Jun 2009 | SP-44 | SP-090408 | 009 | - | Add the missing downlink power related attributes for EUTRAN Cell - align with 36.213 and 36.331 | B | 8.1.0 | 9.0.0 |
| Sep 2009 | SP-45 | SP-090542 | 011 | - | Add missing attribute ‘id’ | A | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090534 | 012 | - | Removing changes introduced by S5-092094 | A | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090542 | 014 | - | Correct Information relationship definitions | A | 9.0.0 | 9.1.0 |
| Sep 2009 | SP-45 | SP-090542 | 017 | - | Cleanup and improvements | F | 9.0.0 | 9.1.0 |