3GPP TSG-SA5 (Telecom Management)	S5-101451

Meeting SA5#71, 10 May - 14 May 2010, Montreal, Canada	revision of S5-101376

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.172 | CR | 0011 |  rev | - |  Current version: | 9.1.1 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Correcting and extending Packet Access service parameters and modeling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Alcatel-Lucent, Nokia Siemens Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM9-SuM |  |  |  |  |  |  | Date:  |  | 03/05/2010 |
| Category:  | C |  |  |  |  |  |  | Release:  |  | Rel-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | - To specify EPSService, EPSSubscribedService, and EPSServiceProfile - To add EPS services support in 32.172 in such a way that it’s semantical relationship to the current PS service support is clear, including name change to be consistent with TS 23.008 terminology. - To bring all packet switched parameters in 32.172 in line with those defined in 3GPP TS 23.008. - To enhance the component data sets of packet access services. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | - Definitions for EPSService, EPSSubscribedService and EPSService Profile are introduced. - PSService, PSSubscribedService, and PSServiceProfile are renamed to GPRSService, GPRSSubscribedService, GPRSServiceProfile. - PSCommonServiceProfile is defined and its relationship to EPSServiceProfile and GPRSServiceProfile is modelled through aggregation. - I-WAN attributes are grouped into an IOC and associated with PSCommonServiceProfile through aggregation. - Non3GPPAccessNetworkParameters and OptHandover3GPP2Parameters IOCs are created and changes made to EPSServiceProfile, EPSSubscribedService, and EPSService. - Numerous name changes and small parameter usage changes based on synchronization with 3GPP TS 23.008 latest version. - Removal of redundant extra listing of optional parameters for all service profiles. - Section 6.3.12.3 simplified to align with common handling of optional attributes. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Not aligned with 3GPP TS 23.008. No support for EPS SuM. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 6.2.1, 6.2.2, 6.3.3, 6.3.7, 6.3.12.3, 6.3.13.3, 6.3.18, 6.5.1, 6.3.12.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  | x |  | O&M Specifications |  |  | 32.175 |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

### 6.2.1	Attributes and relationships

This clause depicts the set of IOCs that encapsulate information relevant for this service. This sub-clause provides the overview of all information object classes in UML. Subsequent clauses provide more detailed specification of various aspects of these Information Object Classes (IOCs).

Figure 1a: SuM NRM naming diagram 1

Figure 1b: SuM NRM Containment and Association diagram 1

Figure 1c: SuM NRM Containment and Association diagram 2

Figure 1d: SuM Supplementary Service NRM Aggregation diagram 1

Figure 1e: SuM Supplementary Service NRM Aggregation diagram 2

Figure 1f: SuM Supplementary Service NRM Aggregation diagram 3

Figure 1g: SuM IMS Service Containment and Association diagram 1

Figure 1.x: Service Profile NRM Association diagram

### 6.2.2	Inheritance

- Figure 2a: SuM NRM Inheritance Hierarchy 1

- Figure 2b: SuM NRM Inheritance Hierarchy 2

- Figure 2c: SuM NRM Inheritance Hierarchy 3

- Figure 2d: SuM NRM Inheritance Hierarchy 4

- Figure 2e: SuM NRM Inheritance Hierarchy 5

- Figure 2f: SuM NRM Inheritance Hierarchy 6

Figure 2g: SuM NRM Inheritance Hierarchy 8

Figure 2h: SuM NRM Inheritance Hierarchy 9

Next Modified Section

### 6.3.x	EPSService

#### 6.3.x.1	Definition

This class represents an EPS service (see 3GPP TS 23.008 [7] provided by a service provider.  This class contains the valid parameters that define the EPS service from a subscription point of view.

#### 6.3.x.2	Attributes

Attributes of EPSService

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| accessPointName | O | M | M |
| access Restriction Data | O | M | M |
| aPNAMBR | M | M | M |
| barringofPacketOrientedServices | O | M | M |
| contextIdentifier | M | M | M |
| operatorDeterminedBarringPLMNspecificData | O | M | M |
| pDNAddress | O | M | M |
| rSZILists | O | M | M |
| sTNSR | O | M | M |
| subscribedChargingCharacteristics | O | M | M |
| subscriberStatus | O | M | M |
| networkAccessMode | M | M | M |
| barringOfOutgoingCalls | O | M | M |
| barringOfRoaming | O | M | M |
| privacyExceptionList | O | M | M |
| gMLCNumbers | O | M | M |
| mOLRList | O | M | M |
| serviceTypes | O | M | M |
| iCSIndicator | O | M | M |
| aPNOIReplacement | O | M | M |
| subscribedUEAMBR | M | M | M |
| aPNConfigurationProfile | M | M | M |
| rFSP | O | M | M |
| vPLMNAddressAllowed-EPS | M | M | M |
| pDNGWIdentity | O | M | M |
| ePSPDNConnectionChargingCharacteristics | O | M | M |
| pDNGWAllocationType | M | M | M |
| aPNLevelAPNOIReplacement | O | M | M |
| closedSubscriberGroupInformation | O | M | M |
| listofauthorizedvisitednetworkidentifiers | O | M | M |
| staticWLANUERemoteIPAddressList | O | M | M |
| w-APNBarringTypeList | O | M | M |
| w-APNChargingDataList | O | M | M |
| wLANUERemoteIPAddress | O | M | M |
| accessDependenceFlag | O | M | M |
| chargingCharacteristics | O | M | M |
| primaryOCSChargingFunctionName | O | M | M |
| secondaryOCSChargingFunctionName | O | M | M |
| primaryChargingCollectionFunctionName | O | M | M |
| secondaryChargingCollectionFunctionName | O | M | M |
| w-APNIdentifierList | O | M | M |
| maxNumberAccessesList | O | M | M |
| maxSubscribedBandwidth | O | M | M |
| subscribed3GPPWLANQoSProfile | O | M | M |
| permanentUserIdentity | O | M | M |
| mIPSubscriberProfile | O | M | M |
| accessRestrictionData | O | M | M |

#### 6.3.x.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (See TS 23.008 [7]).

Next Modified Section

### 6.3.3	GPRSService

#### 6.3.3.1	Definition

This class represents a GPRS service (see 3GPP TS 23.008 [7]) provided by a service provider

This class contains the valid parameters that define the GPRS service from subscription point of view.

#### 6.3.3.2	Attributes

- Attributes of GPRSService

| Attribute name |  | Support Qualifier |  |  | Read Qualifier |  | Write Qualifier |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| networkAccessMode |  | M |  |  | M |  | M |  |
| basicMSISDNIndicator |  | O |  |  | M |  | M |  |
| mSISDNAlertIndicator |  | O |  |  | M |  | M |  |
| rSZIlists |  | O |  |  | M |  | M |  |
| lSAIdentity |  | O |  |  | M |  | M |  |
| lSAPriority |  | O |  |  | M |  | M |  |
| lSAPreferentialAccessIndicator |  | O |  |  | M |  | M |  |
| lSAActiveModeSupportIndicator |  | O |  |  | M |  | M |  |
| lSAOnlyAccessIndicator |  | O |  |  | M |  | M |  |
| lSAActiveModeIndicator |  | O |  |  | M |  | M |  |
| vPLMNIdentifier |  | O |  |  | M |  | M |  |
| accessRestrictionData |  | O |  |  | M |  | M |  |
| privacyExceptionList |  | O |  |  | M |  | M |  |
| gMLCNumbers |  | O |  |  | M |  | M |  |
| mOLRList |  | O |  |  | M |  | M |  |
| serviceTypes |  | O |  |  | M |  | M |  |
| gGSNNumber |  | M |  |  | M |  | M |  |
| provisionofTeleservice |  | O |  |  | M |  | M |  |
| transferofSMoption |  | M |  |  | M |  | M |  |
| subscriberStatus |  | M |  |  | M |  | M |  |
| barringofPacketOrientedServices |  | O |  |  | M |  | M |  |
| pDPType |  | O |  |  | M |  | M |  |
| pDPAddress |  | O |  |  | M |  | M |  |
| accessPointName |  | O |  |  | M |  | M |  |
| vPLMNAddressAllowed |  | O |  |  | M |  | M |  |
| qualityofServiceSubscribed |  | O |  |  | M |  | M |  |
| pDPContextChargingCharacteristics |  | O |  |  | M |  | M |  |
| gPRSCAMELSubscriptionInformationGPRSCSI |  | O |  |  | M |  | M |  |
| mOShortMessageServiceCAMELSubscriptionInformationMOSMSCSI |  | O |  |  | M |  | M |  |
| mTShortMessageServiceCAMELSubscriptionInformationMTSMSCSI |  | O |  |  | M |  | M |  |
| mOSMSCSISGSNNegotiatedCAMELCapabilityHandling |  | O |  |  | M |  | M |  |
| mTSMSCSISGSNNegotiatedCAMELCapabilityHandling |  | O |  |  | M |  | M |  |
| mobilityManagementforGPRSeventnotificationMGCSI |  | O |  |  | M |  | M |  |
| mGCSINegotiatedCAMELCapabilityHandling |  | O |  |  | M |  | M |  |
| gsmSCFaddressforCSI |  | O |  |  | M |  | M |  |
| subscribedChargingCharacteristics |  | O |  |  | M |  | M |  |
| closedSubscriberGroupInformation |  | O |  |  | M |  | M |  |
| gSMSCFaddresslistForODB |  | O |  |  | M |  | M |  |
| barringOfOutgoingCalls |  | O |  |  | M |  | M |  |
| barringOfRoaming |  | O |  |  | M |  | M |  |
| operatorDeterminedBarringPLMNspecificData |  | O |  |  | M |  | M |  |
| sTNSR |  | O |  |  | M |  | M |  |
| iCSIndicator |  | O |  |  | M |  | M |  |
| listofauthorizedvisitednetworkidentifiers |  | O |  |  | M |  | M |  |
| wLANUERemoteIPaddress |  | O |  |  | M |  | M |  |
| staticWLANUERemoteIPAddressList |  | O |  |  | M |  | M |  |
| w-APNBarringTypeList |  | O |  |  | M |  | M |  |
| w-APNChargingDataList |  | O |  |  | M |  | M |  |
| accessDependenceFlag |  | O |  |  | M |  | M |  |
| chargingCharacteristics |  | O |  |  | M |  | M |  |
| primaryOCSChargingFunctionName |  | O |  |  | M |  | M |  |
| secondaryOCSChargingFunctionName |  | O |  |  | M |  | M |  |
| primaryChargingCollectionFunctionName |  | O |  |  | M |  | M |  |
| secondaryChargingCollectionFunctionName |  | O |  |  | M |  | M |  |
| w-APNIdentifierList |  | O |  |  | M |  | M |  |
| maxNumberAccessesList |  | O |  |  | M |  | M |  |
| maxSubscribedBandwidth |  | O |  |  | M |  | M |  |
| subscribed3GPPWLANQoSProfile |  | O |  |  | M |  | M |  |

#### 6.3.3.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (See TS 23.008 [7]).

Next Modified Section

### 6.3.y	EPSSubscribedService

#### 6.3.y.1	Definition

This class represents subscription to an EPS service (see 3GPP TS 23.008 [7]) for a subscriber.  The entire set of EPS subscribed services owned by a subscriber constitutes the subscriber’s Subscribed Services Profile for EPS services.

This class contains the agreed (between the Subscriber and the Service Provider) parameter settings for an EPS service that a Subscriber subscribes to.

#### 6.3.y.2	Attributes

Attributes of EPSSubscribedService

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| accessPointName | O | M | M |
| access Restriction Data | O | M | M |
| aPNAMBR | M | M | M |
| barringofPacketOrientedServices | O | M | M |
| contextIdentifier | M | M | M |
| operatorDeterminedBarringPLMNspecificData | O | M | M |
| pDNAddress | O | M | M |
| rSZILists | O | M | M |
| sTNSR | O | M | M |
| subscribedChargingCharacteristics | O | M | M |
| subscriberStatus | O | M | M |
| networkAccessMode | M | M | M |
| barringOfOutgoingCalls | O | M | M |
| barringOfRoaming | O | M | M |
| privacyExceptionList | O | M | M |
| gMLCNumbers | O | M | M |
| mOLRList | O | M | M |
| serviceTypes | O | M | M |
| iCSIndicator | O | M | M |
| aPNOIReplacement | O | M | M |
| subscribedUEAMBR | M | M | M |
| aPNConfigurationProfile | M | M | M |
| rFSP | O | M | M |
| vPLMNAddressAllowed-EPS | M | M | M |
| pDNGWIdentity | O | M | M |
| ePSPDNConnectionChargingCharacteristics | O | M | M |
| pDNGWAllocationType | M | M | M |
| aPNLevelAPNOIReplacement | O | M | M |
| closedSubscriberGroupInformation | O | M | M |
| Listofauthorizedvisitednetworkidentifiers | O | M | M |
| wLANUERemoteIPaddress | O | M | M |
| staticWLANUERemoteIPAddressList | O | M | M |
| w-APNBarringTypeList | O | M | M |
| w-APNChargingDataList | O | M | M |
| accessDependenceFlag | O | M | M |
| chargingCharacteristics | O | M | M |
| primaryOCSChargingFunctionName | O | M | M |
| secondaryOCSChargingFunctionName | O | M | M |
| primaryChargingCollectionFunctionName | O | M | M |
| secondaryChargingCollectionFunctionName | O | M | M |
| w-APNIdentifierList | O | M | M |
| maxNumberAccessesList | O | M | M |
| maxSubscribedBandwidth | O | M | M |
| subscribed3GPPWLANQoSProfile | O | M | M |
| permanentUserIdentity | O | M | M |
| mIPSubscriberProfile | O | M | M |
| accessRestrictionData | O | M | M |

#### 6.3.y.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (See TS 23.008 [7]).

Next Modified Section

### 6.3.7	GPRSSubscribedService

#### 6.3.7.1	Definition

This class represents subscription to a GPRS service (see 3GPP TS 23.008 [7]) for a subscriber.  The entire set of GPRS subscribed services owned by a subscriber constitutes the subscriber’s Subscribed Services Profile for GPRS services.

#### 6.3.7.2	Attributes

- Attributes of GPRSSubscribedService

| Attribute name |  | Support Qualifier |  | Read Qualifier |  | Write Qualifier |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| networkAccessMode |  | M |  | M |  | M |  |
| basicMSISDNIndicator |  | O |  | M |  | M |  |
| mSISDNAlertIndicator |  | O |  | M |  | M |  |
| rSZIlists |  | O |  | M |  | M |  |
| lSAIdentity |  | O |  | M |  | M |  |
| lSAPriority |  | O |  | M |  | M |  |
| lSAPreferentialAccessIndicator |  | O |  | M |  | M |  |
| lSAActiveModeSupportIndicator |  | O |  | M |  | M |  |
| lSAOnlyAccessIndicator |  | O |  | M |  | M |  |
| lSAActiveModeIndicator |  | O |  | M |  | M |  |
| vPLMNIdentifier |  | O |  | M |  | M |  |
| accessRestrictionData |  | O |  | M |  | M |  |
| privacyExceptionList |  | O |  | M |  | M |  |
| gMLCNumbers |  | O |  | M |  | M |  |
| mOLRList |  | O |  | M |  | M |  |
| serviceTypes |  | O |  | M |  | M |  |
| gGSNNumber |  | M |  | M |  | M |  |
| provisionofTeleservice |  | O |  | M |  | M |  |
| transferofSMoption |  | M |  | M |  | M |  |
| subscriberStatus |  | M |  | M |  | M |  |
| barringofPacketOrientedServices |  | O |  | M |  | M |  |
| pDPType |  | O |  | M |  | M |  |
| pDPAddress |  | O |  | M |  | M |  |
| accessPointName |  | O |  | M |  | M |  |
| vPLMNAddressAllowed |  | O |  | M |  | M |  |
| qualityofServiceSubscribed |  | O |  | M |  | M |  |
| pDPContextChargingCharacteristics |  | O |  | M |  | M |  |
| gPRSCAMELSubscriptionInformationGPRSCSI |  | O |  | M |  | M |  |
| mOShortMessageServiceCAMELSubscriptionInformationMOSMSCSI |  | O |  | M |  | M |  |
| mTShortMessageServiceCAMELSubscriptionInformationMTSMSCSI |  | O |  | M |  | M |  |
| mOSMSCSISGSNNegotiatedCAMELCapabilityHandling |  | O |  | M |  | M |  |
| mTSMSCSISGSNNegotiatedCAMELCapabilityHandling |  | O |  | M |  | M |  |
| mobilityManagementforGPRSeventnotificationMGCSI |  | O |  | M |  | M |  |
| mGCSINegotiatedCAMELCapabilityHandling |  | O |  | M |  | M |  |
| gsmSCFaddressforCSI |  | O |  | M |  | M |  |
| subscribedChargingCharacteristics |  | O |  | M |  | M |  |
| closedSubscriberGroupInformation |  | O |  | M |  | M |  |
| gSMSCFaddresslistForODB |  | O |  | M |  | M |  |
| barringOfOutgoingCalls |  | O |  | M |  | M |  |
| barringOfRoaming |  | O |  | M |  | M |  |
| operatorDeterminedBarringPLMNspecificData |  | O |  | M |  | M |  |
| sTNSR |  | O |  | M |  | M |  |
| iCSIndicator |  | O |  | M |  | M |  |
| listofauthorizedvisitednetworkidentifiers |  | O |  | M |  | M |  |
| wLANUERemoteIPaddress |  | O |  | M |  | M |  |
| staticWLANUERemoteIPAddressList |  | O |  | M |  | M |  |
| w-APNBarringTypeList |  | O |  | M |  | M |  |
| w-APNChargingDataList |  | O |  | M |  | M |  |
| accessDependenceFlag |  | O |  | M |  | M |  |
|  |  |  |  | - |  | - |  |
| chargingCharacteristics |  | O |  | M |  | M |  |
| primaryOCSChargingFunctionName |  | O |  | M |  | M |  |
| secondaryOCSChargingFunctionName |  | O |  | M |  | M |  |
| primaryChargingCollectionFunctionName |  | O |  | M |  | M |  |
| secondaryChargingCollectionFunctionName |  | O |  | M |  | M |  |
| w-APNIdentifierList |  | O |  | M |  | M |  |
| maxNumberAccessesList |  | O |  | M |  | M |  |
| maxSubscribedBandwidth |  | O |  | M |  | M |  |
| subscribed3GPPWLANQoSProfile |  | O |  | M |  | M |  |

#### 6.3.7.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

Next Modified Section

#### 6.3.12.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

Next Modified Section

### 6.3.z	EPSServiceProfile

#### 6.3.z.1	Definition

This class represents for a subscribed EPS service (see 3GPP TS 23.008 [7]) the service preferences and settings chosen for a user.

#### 6.3.z.2	Attributes

Attributes of EPSServiceProfile

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| aPNAMBR | M | M | M |
| contextIdentifier | M | M | M |
| pDNAddress | O | M | M |
| aPNOIReplacement | O | M | M |
| subscribedUEAMBR | M | M | M |
| aPNConfigurationProfile | M | M | M |
| rFSP | O | M | M |
| pDNGWIdentity | O | M | M |
| ePSPDNConnectionChargingCharacteristics | O | M | M |
| vPLMNAddressAllowed-EPS | M | M | M |
| pDNGWAllocationType | M | M | M |
| aPNLevelAPNOIReplacement | O | M | M |
| roamingRestrictionDueToUnsupportedFeatureMME | M | M | - |
| roamingRestrictionDueToUnsupportedFeatureSGSN | M | M | - |
| mSPurgedForEPSFlag | M | M | - |
| uRRPMME | O | M | - |
| mMEName | M | M | - |
| sGSNName | M | M | - |
| iWFNumber | O | M | - |
| sGSNAreaRestrictedFlag | M | M | - |
| theOptHandover3GPP2Par | M | M | O |
| theNon3GPPAccessPar | M | M | O |
| thePSCommonPar | M | M | M |

#### 6.3.z.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

Next Modified Section

#### 6.3.12.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

Next Modified Section

### 6.3.13	GPRSSServiceProfile

#### 6.3.13.1	Definition

This class represents for a subscribed GPRS service (see 3GPP TS 23.008 [7]) the service preferences and settings chosen for a user.

#### 6.3.13.2	Attributes

- Attributes of GPRSServiceProfile

| Attribute name |  | Support Qualifier |  | Read Qualifier |  | Write Qualifier |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| basicMSISDNIndicator |  | O |  | M |  | M |  |
| mSISDNAlertIndicator |  | O |  | M |  | M |  |
| vLRNumber |  | M |  | M |  | - |  |
|  |  | - |  | - |  | - |  |
|  |  |  |  |  |  | - |  |
|  |  |  |  |  |  | - |  |
| lSAIdentity |  | O |  | M |  | M |  |
| lSAPriority |  | O |  | M |  | M |  |
| lSAPreferentialAccessIndicator |  | O |  | M |  | M |  |
| lSAActiveModeSupportIndicator |  | O |  | M |  | M |  |
| lSAOnlyAccessIndicator |  | O |  | M |  | M |  |
| lSAActiveModeIndicator |  | O |  | M |  | M |  |
| vPLMNIdentifier |  | O |  | M |  | M |  |
| ageIndicator |  | O |  | M |  | - |  |
| sGSNNumber |  | M |  | M |  | - |  |
| gGSNNumber |  | M |  | M |  | M |  |
| mNRG |  | M |  | M |  | - |  |
| provisionofTeleservice |  | O |  | M |  | M |  |
| transferofSMoption |  | M |  | M |  | M |  |
| mSpurgedforGPRSflag |  | M |  | M |  | - |  |
| pDPType |  | O |  | M |  | M |  |
| pDPAddress |  | O |  | M |  | M |  |
| gGSNlist |  | M |  | M |  | - |  |
| qualityofServiceSubscribed |  | O |  | M |  | M |  |
| pDPContextIdentifier |  | O |  | M |  | - |  |
| pDPContextChargingCharacteristics |  | O |  | M |  | M |  |
| gPRSCAMELSubscriptionInformationGPRSCSI |  | O |  | M |  | M |  |
| mOShortMessageServiceCAMELSubscriptionInformationMOSMSCSI |  | O |  | M |  | M |  |
| mTShortMessageServiceCAMELSubscriptionInformationMTSMSCSI |  | O |  | M |  | M |  |
| mOSMSCSISGSNNegotiatedCAMELCapabilityHandling |  | O |  | M |  | M |  |
| mTSMSCSISGSNNegotiatedCAMELCapabilityHandling |  | O |  | M |  | M |  |
| mobilityManagementforGPRSeventnotificationMGCSI |  | O |  | M |  | M |  |
| mGCSINegotiatedCAMELCapabilityHandling |  | O |  | M |  | M |  |
| gPRSCSINegotiatedCAMELCapabilityHandling |  | O |  | M |  | - |  |
| sGSNSupportedCAMELPhases |  | O |  | M |  | - |  |
| sGSNOfferedCAMEL4CSIs |  | O |  | M |  | - |  |
| gsmSCFaddressforCSI |  | O |  | M |  | M |  |
| gSMSCFaddresslistForODB |  | O |  | M |  | M |  |
| iPSMGWNumber |  | O |  | M |  | - |  |
| mNRR-SGSN |  | O |  | M |  | - |  |
| notificationToCSEFlagForODB |  | O |  | M |  | - |  |
| thePSCommonPar |  | M |  | M |  | M |  |

#### 6.3.13.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

Next Modified Section

#### 6.3.18	SuMGPRSSupplService

#### 6.3.18.1	Definition

This class represents a Supplementary Service for a GPRS service (see 3GPP TS 23.008 [7]) provided by a service provider

This class contains the valid parameters that define the GPRS supplementary service from subscription point of view.

#### 6.3.18.2	Attributes

- Attributes of SuMGPRSSupplService

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| sS-Code | M | M | M |
| provisioningState | M | M | M |
| activationState | M | M | M |

#### 6.3.18.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (See TS 23.008 [7]).

Next Modified Section

### 6.3.xx	PSCommonParameters

#### 6.3.xx.1	Definition

This class represents for a subscribed GPRS or EPS service (see 3GPP TS 23.008 [7]) the service preferences and settings (attributes) chosen for a user that are common to both services.  Either a single instance of this IOC or two separate instances may be associated to a user who possesses both services.

#### 6.3.xx.2	Attributes

- Attributes of PSCommonParameters

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| rSZIlists | O | M | M |
| roamingRestrictedintheSGSNduetounsupportedfeature | M | M | - |
| accessRestrictionData | O | M | M |
| subscriberStatus | O | M | M |
| barringOfOutgoingCalls | O | M | M |
| barringOfRoaming | O | M | M |
| operatorDeterminedBarringPLMNspecificData | O | M | M |
| barringofPacketOrientedServices | O | M | M |
| traceReference2 | O | M | - |
| traceDepth | O | M | - |
| listOfNETypestoTrace | O | M | - |
| triggeringEvents | O | M | - |
| listOfInterfacesToTrace | O | M | - |
| traceActivatedinSGSN | O | M | - |
| iPAddressOfTraceCollectionEntity | O | M | - |
| accessPointName | O | M | M |
| privacyExceptionList | O | M | M |
| gMLCNumbers | O | M | M |
| iCSIndicator | O | M | M |
| mOLRList | O | M | M |
| serviceTypes | O | M | M |
| subscribedChargingCharacteristics | O | M | M |
| sTNSR | O | M | M |
| networkAccessMode | M | M | M |
| closedSubscriberGroupInformation | O | M | M |
| theIWANPar | M | M | O |

#### 6.3.xx.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

Next Modified Section

6.3.yy	IWANParameters

#### 6.3.yy.1	Definition

This class represents for a subscribed GPRS or EPS service (see 3GPP TS 23.008 [7]) the service preferences and settings chosen for a user that are relevant to IWAN services.

#### 6.3.yy.2	Attributes

- Attributes of IWANParameters

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| w-APN | O | M | - |
| listofauthorizedvisitednetworkidentifiers | O | M | M |
| 3GPPAAAServerName | O | M | - |
| 3GPPAAAProxyName | O | M | - |
| wLANUERemoteIPaddress | O | M | M |
| staticWLANUERemoteIPAddressList | O | M | M |
| w-APNBarringTypeList | O | M | M |
| w-APNChargingDataList | O | M | M |
| accessDependenceFlag | O | M | M |
| i-WLANAccessType | O | M | - |
| wLANIdentifierList | O | M | M |
| maxNumberAccessesList | O | M | M |
| maxSubscribedBandwidth | O | M | M |
| subscribed3GPPWLANQoSProfile | O | M | M |
| chargingCharacteristics | M | M | M |
| primaryOCSChargingFunctionName | O | M | M |
| secondaryOCSChargingFunctionName | O | M | M |
| primaryChargingCollectionFunctionName | O | M | M |
| secondaryChargingCollectionFunctionName | O | M | M |
| userStatus | M | M | - |

#### 6.3.yy.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

Next Modified Section

### 6.3.zz	OptHandover3GPP2Parameters

#### 6.3.zz.1	Definition

This class represents for a subscribed EPS service (see 3GPP TS 23.008 [7]) the service preferences and settings chosen for a user that are relevant to the optimization of handover to a 3GPP2 network.

#### 6.3.zz.2	Attributes

- Attributes of OptHandover3GPP2Parameters

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| rATType | O | M | - |

#### 6.3.zz.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

Next Modified Section

### 6.3.xxx	Non3GPPAccessParameters

#### 6.3.xxx.1	Definition

This class represents for a subscribed EPS service (see 3GPP TS 23.008 [7]) the service preferences and settings chosen for a user that are relevant to use of an non-3GPP access network.

#### 6.3.xxx.2	Attributes

- Attributes of NonGPPAccessParameters

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| rATType | O | M | - |
| visitedNetworkIdentifier | O | M | - |
| mIPSubscriberProfile | M | M | M |
| permanentUserIdentity | M | M | M |

#### 6.3.xxx.3	Attribute constraints

The optional attributes shall be supported for the following criteria: The NE supports the parameter (see TS 23.008 [7]).

## 6.5	Information attribute definitions

### 6.5.1	Definitions and legal values

The following table defines the attributes that are present in several IOCs of the present document.

- Attributes

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| iMSServiceId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IMSService object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| cSServiceId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the CSService object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| pSServiceId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the PSService object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| cSServiceProfileId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the CSServiceProfile object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| pSServiceProfileId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the PSServiceProfile object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| iMSServiceProfileId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IMSServiceProfile object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| cSSubscribedServiceId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the CSSubscribedService object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| pSSubscribedServiceId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the PSSubscribedService object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| iMSSubscribedServiceId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IMSSubscribedService object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| iMSPublicIdentificationId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IMSPublicIdentification object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| mobileUserDataId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the MobileUserData object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| suMCredentialsId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the SuMCredentials object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| suMSubscriberProfileId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the SuMSubscriberProfile object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| suMSubscriberProfileInfo | A description of (information for) the Subscriber Profile. It is an “Any” type with free structure and format. |  |
| suMUserId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the SuMUser object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| userLabel | A user-friendly (and user assigned) name of the associated object. IOCs inheriting from ManagedFunction will have this attribute. Other IOCs may have it as well, if explicitly defined for them. |  |
| agreement-contract | Models the agreement relationship to a subscribed service. This attribute contains a list of the DN(s) of the related SuMSubscribedService instance(s). |  |
| agreement-offer | Models the agreement relationship to a service. An attribute whose ‘name+value’ can be used as DN when referencing an instance of the SuMService object class. |  |
| description-service | Models the description relationship to a service profile. This attribute contains a list of the DN(s) of the related SuMServiceProfile instance(s). |  |
| description-specification | Models the description relationship to a subscribed service. An attribute whose ‘name+value’ can be used as DN when referencing an instance of the SuMSubscribedService object class. |  |
| relatedIMSPublicId-thePublicId | Models the relatedIMSPublicId relationship from a user’s credentials to the IMS Public Identification. This attribute contains a list of the DN(s) of the related IMSPublicIdentification instance(s). |  |
| relatedIMSServiceProfile-theServiceProfile | Models the relatedIMSServiceProfile relationship from a user’s credentials to an IMS service profile. This attribute contains a list of the DN(s) of the related IMSServiceProfile instance(s). |  |
| relatedMobileUserData-theMobileUserData | Models the relatedMobileUserData relationship from a user to the user data of a mobile User Equipment. This attribute contains a list of the DN(s) of the related MobileUserData instance(s). |  |
| relatedServiceProfile-theServiceProfile | Models the relatedServiceProfile relationship from the user data of a mobile User Equipment to a service profile. This attribute contains a list of the DN(s) of the related SuMServiceProfile derived instance(s). |  |
| userSubscriberRelation-theSubscriber | Models the userSubscriberRelation relationship from a user to a subscriber. This attribute contains a list of the DN(s) of the related SuMSubscriberProfile instance(s). |  |
| userSubscriberRelation-theUser | Models the userSubscriberRelation relationship from a subscriber to a user. This attribute contains a list of the DN(s) of the related SuMUser instance(s). |  |
| thePSCommonParameters | Models the common packet access service profile relationship from either a GPRS service profile or an EPS service profile. This attribute contains one of DN of the related PSCommonParameters instance. |  |
| theIWANParameters | Models the relationship between the common packet access service parameters and the I-WAN service parameters. This attribute contains the DN of the related IWANParameters instance. |  |
| theOptHandover3GPP2Parameters | Models the relationship between the EPS service profile when designed for optimal handover to a 3GPP2 access network. This attribute contains the DN of the related OptHandover3GPP2Parameters instance. |  |
| theNon3GPPAccessParameters | Models the relationship between the EPS service profile and another set of service parameters needed to when access is via a non 3GPP network. This attribute contains the DN of the related Non3GPPAccessParameters instance. |  |

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| iMSI | 23.008 clause 2.1.1.1 |  |
| networkAccessMode | 23.008 clause 2.1.1.2 |  |
| mSISDN | 23.008 clause 2.1.2 |  |
| multinumberingMSISDNs | 23.008 clause 2.1.3 |  |
| basicMSISDNIndicator | 23.008 clause 2.1.3.1 |  |
| mSISDNAlertIndicator | 23.008 clause 2.1.3.2 |  |
| lMSI | 23.008 clause 2.1.8 |  |
| mobileStationCategory | 23.008 clause 2.2.1 |  |
| lMUIdentifier | 23.008 clause 2.2.2 |  |
| iMEISV | 23.008 clause 2.2.3 |  |
| subscriptionRestriction | 23.008 clause 2.4.10 |  |
| rSZIlists | 23.008 clause 2.4.11.1 |  |
| mSCAreaRestrictedFlag | 23.008 clause 2.4.12 |  |
| sGSNarearestrictedflag | 23.008 clause 2.4.14 |  |
| oDBinducedBarringData | 23.008 clause 2.4.15.1 |  |
| roamingRestrictionDueToUnsupportedFeature | 23.008 clause 2.4.15.2 |  |
| roamingRestrictedintheSGSNduetounsupportedfeature | 23.008 clause 2.4.15.3 |  |
| lSAIdentity | 23.008 clause 2.4.17.1 |  |
| lSAPriority | 23.008 clause 2.4.17.2 |  |
| lSAPreferentialAccessIndicator | 23.008 clause 2.4.17.2A |  |
| lSAActiveModeSupportIndicator | 23.008 clause 2.4.17.2B |  |
| lSAOnlyAccessIndicator | 23.008 clause 2.4.17.3 |  |
| lSAActiveModeIndicator | 23.008 clause 2.4.17.4 |  |
| vPLMNIdentifier | 23.008 clause 2.4.17.5 |  |
| accessRestrictionData | 23.008 clause 2.4.18 |  |
| vLRNumber | 23.008 clause 2.4.5 |  |
| mSCNumber | 23.008 clause 2.4.6 |  |
| sGSNNumber | 23.008 clause 2.4.8.1 |  |
| gGSNNumber | 23.008 clause 2.4.8.2 |  |
| provisionOfBearerService | 23.008 clause 2.5.1 |  |
| provisionOfTeleservice | 23.008 clause 2.5.2 |  |
| bCAllocation | 23.008 clause 2.5.3 |  |
| transferofSMoption | 23.008 clause 2.5.4 |  |
| mNRG | 23.008 clause 2.7.2 |  |
| checkSSIndicator | 23.008 clause 2.7.4.4 |  |
| mSPurgedForNonGPRSFlag | 23.008 clause 2.7.5 |  |
| mSpurgedforGPRSflag | 23.008 clause 2.7.6 |  |
| mNRRMSC | 23.008 clause 2.7.7 |  |
| subscriberStatus | 23.008 clause 2.8.1 |  |
| barringOfOutgoingCalls | 23.008 clause 2.8.2.1 |  |
| barringOfIncomingCalls | 23.008 clause 2.8.2.2 |  |
| barringOfRoaming | 23.008 clause 2.8.2.3 |  |
| barringOfPremiumRateCalls | 23.008 clause 2.8.2.4 |  |
| barringOfSupplementaryServiceManagement | 23.008 clause 2.8.2.5 |  |
| barringOfRegistrationOfCallForwarding | 23.008 clause 2.8.2.6 |  |
| barringOfInvocationOfCallTransfer | 23.008 clause 2.8.2.7 |  |
| barringofPacketOrientedServices | 23.008 clause 2.8.2.8 |  |
| operatorDeterminedBarringPLMNspecificData | 23.008 clause 2.8.3 |  |
| notificationToCSEFlagForODB | 23.008 clause 2.8.4 |  |
| gsmSCFaddresslistForODB | 23.008 clause 2.8.5 |  |
| messagesWaitingData | 23.008 clause 2.10.1 |  |
| mobileStationNotReachableFlag | 23.008 clause 2.10.2 |  |
| memoryCapacityExceededFlag | 23.008 clause 2.10.3 |  |
| traceReference | 23.008 clause 2.11.1 |  |
| traceType | 23.008 clause 2.11.2 |  |
| operationsSystemsIdentity | 23.008 clause 2.11.3 |  |
| hLRTraceType | 23.008 clause 2.11.4 |  |
| mAPErrorOnTrace | 23.008 clause 2.11.5 |  |
| traceActivatedInVLR | 23.008 clause 2.11.6 |  |
| traceActivatedinSGSN | 23.008 clause 2.11.7 |  |
| vGCSGroupMembershipList | 23.008 clause 2.12.1 |  |
| vBSGroupMembershipList | 23.008 clause 2.12.2 |  |
| broadcastCallInitiationAllowedList | 23.008 clause 2.12.2.1 |  |
| pDPType | 23.008 clause 2.13.1 |  |
| gGSNlist | 23.008 clause 2.13.11 |  |
| qualityofServiceSubscribed | 23.008 clause 2.13.12 |  |
| pDPAddress | 23.008 clause 2.13.2 |  |
| pDPContextIdentifier | 23.008 clause 2.13.24 |  |
| pDPContextChargingCharacteristics | 23.008 clause 2.13.25 |  |
| accessPointName | 23.008 clause 2.13.6 |  |
| vPLMNAddressAllowed | 23.008 clause 2.13.8 |  |
| vPLMNAddressAllowed-EPS | 23.008 clause 2.13.40 |  |
| originatingCAMELSubscriptionInformationOCSI | 23.008 clause 2.14.1.1/3.1 |  |
| gPRSCAMELSubscriptionInformationGPRSCSI | 23.008 clause 2.14.1.10/2.14.4.4 |  |
| dialledServiceCAMELSubscriptionInformationDCSI | 23.008 clause 2.14.1.11/3.7 |  |
| mobilityManagementforGPRSeventnotificationMGCSI | 23.008 clause 2.14.1.12/2.14.4.4 |  |
| terminatingCAMELSubscriptionInformationTCSI | 23.008 clause 2.14.1.2 |  |
| vMSCTerminatingCAMELSubscriptionInformationVTCSI | 23.008 clause 2.14.1.2/3.2 |  |
| locationInformationSubscriberStateInformation | 23.008 clause 2.14.1.3 |  |
| uSSDCAMELSubscriptionInformationUCSI | 23.008 clause 2.14.1.4 |  |
| sSInvocationNotificationSSCSI | 23.008 clause 2.14.1.5/3.2 |  |
| translationInformationFlagTIFCSI | 23.008 clause 2.14.1.6/3.6 |  |
| shortMessageServiceCAMELSubscriptionInformationMOSMSCSI | 23.008 clause 2.14.1.8/2.14.3.5 |  |
| mOShortMessageServiceCAMELSubscriptionInformationMOSMSCSI | 23.008 clause 2.14.1.8/2.14.4.1 |  |
| shortMessageServiceCAMELSubscriptionInformationMTSMSCSI | 23.008 clause 2.14.1.9/2.14.3.6 |  |
| mTShortMessageServiceCAMELSubscriptionInformationMTSMSCSI | 23.008 clause 2.14.1.9/2.14.4.2. |  |
| gPRSCSINegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| mCSINegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| mGCSINegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| mOSMSCSISGSNNegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| mOSMSCSIVLRNegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| mTSMSCSISGSNNegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| mTSMSCSIVLRNegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| oCSINegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| sSCSINegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| vTCSINegotiatedCAMELCapabilityHandling | 23.008 clause 2.14.2.1 |  |
| SGSNOfferedCAMEL4CSIs | 23.008 clause 2.14.2.2A |  |
| vLROfferedCAMEL4CSIs | 23.008 clause 2.14.2.2A |  |
| sGSNSupportedCAMELPhases | 23.008 clause 2.14.2.2 |  |
| vLRSupportedCAMELPhases | 23.008 clause 2.14.2.2 |  |
| gsmSCFaddressforCSI | 23.008 clause 2.14.2.4 |  |
| uSSDGeneralCAMELServiceInformationUGCSI | 23.008 clause 2.14.2.3 |  |
| iSTAlertTimer | 23.008 clause 2.15.1 |  |
| privacyExceptionList | 23.008 clause 2.16.1.1 |  |
| gMLCNumbers | 23.008 clause 2.16.1.2 |  |
| mOLRList | 23.008 clause 2.16.1.3 |  |
| serviceTypes | 23.008 clause 2.16.1.4 |  |
| ageIndicator | 23.008 clause 2.17.1 |  |
| cSAllocationRetentionpriority | 23.008 clause 2.18.1 |  |
| subscribedChargingCharacteristics | 23.008 clause 2.19.1 |  |
| iMSPrivateUserIdentity | 23.008 clause 3.1.1 |  |
| publicUserIdentity | 23.008 clause 3.1.2 |  |
| barringIndication | 23.008 clause 3.1.3 |  |
| listofauthorizedvisitednetworkidentifiers | 23.008 clause 3.1.4 23.008 clause 3B.1.4 |  |
| registrationStatus | 23.008 clause 3.2.1 |  |
| sCSCFName | 23.008 clause 3.2.2 |  |
| diameterClientAddressofSCSCF | 23.008 clause 3.2.3 |  |
| serverCapabilities | 23.008 clause 3.4.1 |  |
| subscribedMediaProfileIdentifier | 23.008 clause 3.6.1 |  |
| initialFilterCriteria | 23.008 clause 3.5.2 |  |
| applicationServerInformation | 23.008 clause 3.5.3 |  |
| serviceIndication | 23.008 clause 3.5.4 |  |
| primaryEventChargingFunctionName | 23.008 clause 3.7.1 |  |
| secondaryEventChargingFunctionName | 23.008 clause 3.7.2 |  |
| primaryChargingCollectionFunctionName | 23.008 clause 3.7.3 23.008 clause 3B.7.4 |  |
| secondaryChargingCollectionFunctionName | 23.008 clause 3.7.4 23.008 clause 3B.7.5 |  |
| gsmSCFaddressforIMCSI | 23.008 clause 3.8.4 |  |
| iMSSFaddressforIMCSI | 23.008 clause 3.8.5 |  |
| oIMCSI | 23.008 clause 3.8.1 |  |
| vTIMCSI | 23.008 clause 3.8.2 |  |
| dIMCS | 23.008 clause 3.8.3 |  |
| gAAServiceType | 23.008 clause 3A.1 |  |
| gAAServiceIdentifier | 23.008 clause 3A.2 |  |
| gBAUserSecuritySettings | 23.008 clause 3A.3 |  |
| userSecuritySetting | 23.008 clause 3A.4 |  |
| userPublicIdentity | 23.008 clause 3A.5 |  |
| gAAAuthorizationflag | 23.008 clause 3A.6 |  |
| keyLifetime | 23.008 clause 3A.8 |  |
| uICCSecuritySetting | 23.008 clause 3A.9 |  |
| nAFGroupIdentity | 23.008 clause 3A.11 |  |
| w-APN | 23.008 clause 3B.1.3 |  |
| 3GPPAAAServerName | 23.008 clause 3B.1.6 |  |
| wLANUERemoteIPaddress | 23.008 clause 3B.1.10 |  |
| staticWLANUERemoteIPAddressList | 23.008 clause 3B.5.1.4 |  |
| w-APNBarringTypeList | 23.008 clause 3B.5.1.2 |  |
| w-APNChargingDataList | 23.008 clause 3B.5.1.3 |  |
| accessDependenceFlag | 23.008 clause 3B.5.2 |  |
| i-WLANAccessType | 23.008 clause 3B.5.3 |  |
| chargingCharacteristics | 23.008 clause 3B.7.1.1 |  |
| primaryOCSChargingFunctionName | 23.008 clause 3B.7.2 |  |
| secondaryOCSChargingFunctionName | 23.008 clause 3B.7.3 |  |
| sS-Code | 23.008 clause 2.6 29.002 clause 7.6.4.1 |  |
| provisioningState | 23.008 clause 2.6 29.002 clause 7.6.4.2 |  |
| activationState | 23.008 clause 2.6 29.002 clause 7.6.4.2 |  |
| registrationState | 23.008 clause 2.6 29.002 clause 7.6.4.2 |  |
| overrideCategory | 23.008 clause 2.6 29.002 clause 7.6.4.4 |  |
| cLIRestrictionOption | 23.008 clause 2.6 29.002 clause 7.6.4.5 |  |
| forwarded-toNumber | 23.008 clause 2.6 29.002 clause 7.6.2.22 |  |
| forwarded-toSubaddress | 23.008 clause 2.6 29.002 clause 7.6.2.23 |  |
| noReplyConditionTimer | 23.008 clause 2.6 29.002 clause 7.6.4.7 |  |
| maximumEntitledPriority | 23.008 clause 2.6 29.002 clause 7.6.4.41 |  |
| defaultPriority | 23.008 clause 2.6 29.002 clause 7.6.4.41 |  |
| nbrUser | 23.008 clause 2.6 29.002 clause 7.6.4.47 |  |
| nbrSB | 23.008 clause 2.6 29.002 clause 7.6.4.47 |  |
| TraceReference2 | 23.008 Clause 2.11.9 |  |
| TraceDepth | 23.008 Clause 2.11.10 |  |
| ListofNeTypestoTrace | 23.008 Clause 2.11.11 |  |
| TriggeringEvents | 23.008 Clause 2.11.12 |  |
| ListofInterfacestoTrace | 23.008 Clause 2.11.13 |  |
| iCSIndicator | 23.008 Clause 2.20.1 |  |
| sTNSR | 23.008 Clause 2.21.1 |  |
| 3GPPAAAProxyName | 23.008 Clause 3B.1.5 |  |
| w-APNIdentifierList | 23.008 Clause 3B.5.1.1 |  |
| maxNumberAccessesList | 23.008 Clause 3B.5.1.5 |  |
| maxSubscribedBandwidth | 23.008 Clause 3B.6.1 |  |
| subscribed3GPPWLANQoSProfile | 23.008 Clause 3B.6.4 |  |
| userStatus | 23.008 Clause 3B.2.1 |  |
| rATType | 23.008 Clause 2.13.75 |  |
| iPAddressOfTraceCollectionEntity | 23.008 Clause 2.11.14 |  |
| aPNConfigurationProfile | 23.008 Clause 2.13.31 |  |
| pDNAddress | 23.008 Clause 2.13.39 |  |
| permanentUserIdentity | 23.008 Clause 2.13.79 |  |
| visitedNetworkIdentifier | 23.008 Clause 2.13.82 |  |
| mIPSubscriberProfile | 23.008 Clause 2.13.86 |  |
| aPNAMBR | 23.008 Clause 2.13.32 |  |
| contextIdentifier | 23.008 Clause 2.13.38 |  |
| aPNOIReplacement | 23.008 Clause 2.13.29 |  |
| subscribedUEAMBR | 23.008 Clause 2.13.30 |  |
| rFSP | 23.008 Clause 2.13.33 |  |
| pDNGWIdentity | 23.008 Clause 2.13.41 |  |
| ePSPDNConnectionChargingCharacteristics | 23.008 Clause 2.13.71 |  |
| pDNGWAllocationType | 23.008 Clause 2.13.105 |  |
| aPNLevelAPNOIReplacement | 23.008 Clause 2.13.109 |  |
| closedSubscriberGroupInformation | 23.008 Clause 2.4.22 |  |
| roamingRestrictionDueToUnsupportedFeatureMME | 23.008 Clause 2.4.15.3a |  |
| mSPurgedForEPSFlag | 23.008 Clause 2.7.6A |  |
| uRRPMME | 23.008 Clause 2.7.9.1 |  |
| mMEName | 23.008 Clause 2.13.26 |  |
| sGSNName | 23.008 Clause 2.13.100 |  |
| iWFNumber | 23.008 Clause 2.4.8.3 |  |

End of Modifications