3GPP TSG-SA5 (Telecom Management)	S5-101570

Meeting SA5#71, 10-14 May 2010, Montreal, Canada	revision of S5-101531

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.409 | CR | 0017 |  rev | - |  Current version: | 9.1.1 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add emergency session related measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM10 |  |  |  |  |  |  | Date:  |  | 14/05/2010 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | Emergency session function is an important function of IMS. The emergency session related measurements can help operator to evaluate the emergency session processing capacity of IMS network entity, analyze the establishement failure reasons of emergency session and solve the problems. The emergency session establishment procedure may not comply with the emergency configuration policy, and then these measurements can be used to check the effectiveness of the policy. So it is proposed to add the emergency session related measurements. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | - Add the reference of emergency session - Add definitions of emergency session related measurements. - Add use case of emergency session related measurements. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 2, 4.1.11, Annex A, Annex B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication and/or edition number or version number) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]		3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]		3GPP TS 32.102: "Telecommunication management; Architecture".

[3]		3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[4]		3GPP TS 32.632: "Telecommunication management; Configuration Management (CM); Core Network Resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[5]		3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[6]		3GPP TS 23.003: "Numbering, addressing and identification".

[7]		3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[8] 		3GPP TS 24.229: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[9]		3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx Interfaces; Signalling flows and message".

[10] 	3GPP TS 29.229: "Cx and Dx interfaces based on the Diameter protocol; Protocol details".

[11] 	3GPP TS 29.163: "Interworking between the IP Multimedia (IM) Core Network (CN) subsystem and Circuit Switched (CS) networks".

[12]	3GPP TS 32.404: "Performance Management (PM); Performance measurements - Definitions and template".

[13]	3GPP TS 29.207: "Policy control over Go interface".

[14]	3GPP TS 29.208: "End-to-end Quality of Service (QoS) signalling flows".

[15]	IETF RFC 3588: "Diameter Base Protocol".

[16]	3GPP TS 23.008: "Organization of subscriber data".

[17]	3GPP TS 29.328: "IP Multimedia Subsystem (IMS) Sh interface; Signalling flows and message contents".

[18]	3GPP TS 29.329: "Sh interface based on the Diameter protocol; Protocol details".

[19]	ITU-T Recommendation Q.850: "Usage of cause and location in the Digital Subscriber Signalling System No. 1 and the Signalling System No. 7 ISDN User Part".

[20]	ITU-T Recommendation H.248.1: "Gateway control protocol: Version 3".

[21]	ITU-T Recommendation H.248.8: "Gateway control protocol: Error code and service change reason description".

[22]	IETF RFC 3265: "Session Initiation Protocol (SIP)-Specific Event Notification".

[23]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

[24]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[25]	3GPP TS 23.141: "Presence service; Architecture and functional description; Stage 2".

[26]	3GPP TR 23.979: "3GPP enablers for Open Mobile Alliance (OMA); Push-to-talk over Cellular (PoC) services; Stage 2".

[27]	OMA-TS-XDM_Core: "XML Document Management Specification".

[28]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[29]	3GPP TS 32.407: "Performance Management (PM); Performance measurements Core Network (CN) Circuit Switched (CS) domain".

[30]	3GPP TS 32.732: "Telecommunication management; IP Multimedia Subsystem (IMS) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[31]	3GPP TS 23.203: "Policy and charging control architecture".

[32]	3GPP TS 29.212: "Policy and charging control over Gx reference point".

[33]	3GPP TS 29.213: "Policy and charging control signalling flows and Quality of Service (QoS) parameter mapping".

[34]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[35]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

Next Modified Section

### 4.1.11	 Emergency session procedure related measurements

The three measurements defined in this subclause are subject to the “2 out of 3 approach”.

#### 4.1.11.1	Attempted emergency session procedures

- This measurement provides the number of attempted emergency session procedures at CSCF. This measurement is valid for P-CSCF.

- CC.

- On receipt by the CSCF of a SIP initial request message except SIP_REGISTER for the emergency session (see 3GPP TS 23.167 [35], TS 24.229 [8]).

- A single integer value.

- SC.AttEmgSess.

- CscfFunction.

- Valid for packet switching.

- IMS.

#### 4.1.11.2	Successful emergency session procedures

- This measurement provides the number of successful emergency session procedures at CSCF. This measurement is valid for P-CSCF.

- CC.

- On transmission by the CSCF of a SIP_200_OK response message corresponding to the SIP initial request message except SIP_REGISTER for the emergency session (see 3GPP TS 23.167 [35], TS 24.229 [8]).

- A single integer value.

- SC.SuccEmgSess.

- CscfFunction.

- Valid for packet switching.

- IMS.

#### 4.1.11.3	Failed emergency session procedures

- This measurement provides the number of failed emergency session procedures at CSCF. The measurement is split into subcounters per response status codes. This measurement is valid for P-CSCF.

- CC

- On transmission by the CSCF of a response message with response code (e.g. SIP_380) indicating a failed response corresponding to the SIP initial request message except SIP_REGISTE for the emergency session (see 3GPP TS 23.167 [35], TS 24.229 [8]). Each failed session establishment is added to the relevant measurement according to the status code. The sum of all supported per status code measurements shall equal the total number of failed session establishments. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form SC. FailEmgSess.Cause
where Cause identifies the status code.

- CscfFunction

- Valid for packet switching.

- IMS.

Next Modified Section

# Annex A (normative): 
Use cases for performance measurements defintion

This annex provides the concrete use cases for the IMS performance measurements defined in this spec.

# A.1 Use case of Immediate Messaging procedure related measurements at CSCF

IMS users shall be able to exchange immediate messages with each other by using the Immediate Messaging procedure. CSCF may delivery successfully the immediate messages, also may refuse the immediate messages or respond to the sender with fail indications.If the Immediate Messaging procedure at CSCF is fail, the users’ subjective feel to the network will be bad.So it is necessary to define Immediate Messaging procedure related measurements for operator to evaluate the rate of success and failure at CSCF in IMS network.

# A.2 Use case of IP-CAN Session Establishment related measurements

IP-CAN session is a main procedure in policy and charging control architecture (see 3GPP TS 29.213 [33]). If IP-CAN session establishment is failed, it means that PCEF cannot get PCC rules from PCRF and PCEF shall reject the attempt for UE-initiated resource request. So it is necessary to define IP-CAN session establishment related measurements to monitor or evaluate session performance of PCC procedures.

# A.3 Use case of emergency session related measurements

Emergency session function is an important function at P-CSCF. The emergency session related measurements can help operator to evaluate the emergency session processing capacity of IMS network entity, analyze the establishement failure reasons of emergency session and solve the problems. The emergency session establishment procedure may not comply with the emergency configuration policy, and these measurements can be used to check the effectiveness of the policy.

# Annex B (informative):
Use case of IP-CAN Session Establishment related measurements

- IP-CAN session is a main procedure in policy and charging control architecture (see 3GPP TS 29.213 [33]). If IP-CAN session establishment is failed, it means that PCEF cannot get PCC rules from PCRF and PCEF shall reject the attempt for UE-initiated resource request. So it is necessary to define IP-CAN session establishment related measurements to monitor or evaluate session performance of PCC procedures. Void.

End of Modifications