3GPP TSG-SA5 (Telecom Management)	S5-101528

Meeting SA5#71, 10-14 May 2010, Montreal, Canada	revision of S5-101457

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0152 |  rev | - |  Current version: | 10.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Charging information for Emergency IMS Sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Nokia Siemens Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH9 |  |  |  |  |  |  | Date:  |  | 14/05/2010 |
| Category:  | A |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | Add the possibility to collect Charging information from the serving network node based on the requirements in TS 22.115 |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Add the ASN.1 for E-CSCF-CDR definition. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | No possibility to collect Charging information from the serving network node |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.2.1, 5.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  | X |  | O&M Specifications |  |  | 32.260 |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

### 5.2.1	Generic ASN.1 definitions

This subclause contains generic CDR syntax definitions, where the term "generic" implies that these constructs are applicable for more than one domain/service/subsystem. Examples of this are syntax definitions that are imported from non-charging 3GPP TSs, e.g. TS 29.002 [214].

GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

CallReferenceNumber, NumberOfForwarding

FROM MAP-CH-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CH-DataTypes (13) version6 (6) }

-- from TS 29.002 [214]

AddressString, BasicServiceCode, IMSI, IMEI, ISDN-AddressString, LCSClientExternalID, LCSClientInternalID

FROM MAP-CommonDataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18) version6 (6) }

-- from TS 29.002 [214]

DestinationRoutingAddress

FROM CAP-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) cap-datatypes (52) version1 (0) }

-- from TS 29.078 [217]

DefaultCallHandling, DefaultSMS-Handling, NotificationToMSUser, ServiceKey

FROM MAP-MS-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version6 (6) }

-- from TS 29.002 [214]

Ext-GeographicalInformation, LCSClientType, LCS-Priority, LocationType

FROM MAP-LCS-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-LCS-DataTypes (25) version7 (7)}

-- from TS 29.002 [214]

PositionMethodFailure-Diagnostic, UnauthorizedLCSClient-Diagnostic

FROM MAP-ER-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-ER-DataTypes (17) version7 (7)}

--

-- from TS 29.002 [214]

--

BasicService

FROM Basic-Service-Elements	{ itu-t identified-organization (4) etsi (0)

196 basic-service-elements (8) }

--

-- from "Digital Subscriber Signalling System No. one (DSS1) protocol" ETS 300 196 [310]

--

ObjectInstance

FROM CMIP-1 {joint-iso-itu-t ms (9) cmip (1) version1 (1) protocol (3)}

--

-- from Rec. X.2ab[304]  Editor’s note:  clarify if this definition is still needed.  It appears that it ends in Nirvana.

ManagementExtension

FROM Attribute-ASN1Module {joint-iso-itu-t ms (9) smi (3) part2 (2) asn1Module (2) 1}

--

-- from Rec. X.721 [305]  Editor’s note:  clarify if this definition is still needed.

--

AE-title

FROM ACSE-1 {joint-iso-itu-t association-control (2) abstract-syntax (1) apdus (0) version (1) };

--

-- From Rec. X.2cd[306].  Note that the syntax of AE-title to be used is from

-- ITU-T Rec. X.227 / ISO 8650 corrigendum and not "ANY"

--  Editor’s note:  clarify if this definition is still needed.  It appears that it ends in Nirvana.

;

------------------------------------------------------------------------------

--

--  Generic Data Types

--

------------------------------------------------------------------------------

BCDDirectoryNumber		::= OCTET STRING

--

-- This type contains the binary coded decimal representation of

-- a directory number e.g. calling/called/connected/translated number.

-- The encoding of the octet string is in accordance with the

-- the elements "Calling party BCD number", "Called party BCD number"

-- and "Connected number" defined in TS 24.008 [208].

-- This encoding includes type of number and number plan information

-- together with a BCD encoded digit string.

-- It may also contain both a presentation and screening indicator

-- (octet 3a).

- For the avoidance of doubt, this field does not include

-- octets 1 and 2, the element name and length, as this would be

-- redundant.

--

CallDuration 			::= INTEGER

--

-- The call duration is counted in seconds.

-- For successful calls /sessions / PDP contexts, this is the chargeable duration.

-- For call attempts this is the call holding time.

--

Editor’s note:  the explanation above should be removed as proper definitions are required in the individual CDR parameter descriptions in TS 32.250[10] – TS 32.275 [35]

CalledNumber			::= BCDDirectoryNumber

RecordType 	::= INTEGER

{--

--	Record values 0..17 are CS specific.

--	The contents are defined in TS 32.250 [10]

moCallRecord		(0),

mtCallRecord		(1),

roamingRecord		(2),

incGatewayRecord	(3),

outGatewayRecord	(4),

transitCallRecord	(5),

moSMSRecord			(6),

mtSMSRecord			(7),

moSMSIWRecord		(8),

mtSMSGWRecord		(9),

ssActionRecord		(10),

hlrIntRecord		(11),

locUpdateHLRRecord	(12),

locUpdateVLRRecord	(13),

commonEquipRecord	(14),

moTraceRecord		(15),	--- used in earlier releases

mtTraceRecord		(16),	--- used in earlier releases

termCAMELRecord	    (17),

--

--	Record values 18..22 are GPRS specific.

--	The contents are defined in TS 32.251 [11]

--

sgsnPDPRecord		(18),

sgsnMMRecord		(20),

sgsnSMORecord		(21),

sgsnSMTRecord		(22),

--

--  Record values 23..25 are CS-LCS specific.

--	The contents are defined in TS 32.250 [10]

--

mtLCSRecord			(23),

moLCSRecord			(24),

niLCSRecord			(25),

--

--  Record values 26..28 are GPRS-LCS specific.

--	The contents are defined in TS 32.251 [11]

--

sgsnMtLCSRecord		(26),

sgsnMoLCSRecord		(27),

sgsnNiLCSRecord		(28),

--

--  Record values 30..62 are MMS specific.

--  The contents are defined in TS 32.270 [30]

--

mMO1SRecord			(30),

mMO4FRqRecord		(31),

mMO4FRsRecord		(32),

mMO4DRecord			(33),

mMO1DRecord			(34),

mMO4RRecord			(35),

mMO1RRecord			(36),

mMOMDRecord			(37),

mMR4FRecord			(38),

mMR1NRqRecord		(39),

mMR1NRsRecord		(40),

mMR1RtRecord		(41),

mMR1AFRecord		(42),

mMR4DRqRecord		(43),

mMR4DRsRecord		(44),

mMR1RRRecord		(45),

mMR4RRqRecord		(46),

mMR4RRsRecord		(47),

mMRMDRecord			(48),

mMFRecord			(49),

mMBx1SRecord		(50),

mMBx1VRecord		(51),

mMBx1URecord		(52),

mMBx1DRecord		(53),

mM7SRecord			(54),

mM7DRqRecord		(55),

mM7DRsRecord		(56),

mM7CRecord			(57),

mM7RRecord			(58),

mM7DRRqRecord		(59),

mM7DRRsRecord		(60),

mM7RRqRecord		(61),

mM7RRsRecord		(62),

--

--  Record values 63..69, 70, 82 are IMS specific.

--  The contents are defined in TS 32.260 [20]

--

sCSCFRecord			(63),

pCSCFRecord			(64),

iCSCFRecord			(65),

mRFCRecord			(66),

mGCFRecord			(67),

bGCFRecord			(68),

aSRecord			(69),

eCSCFRecord			[70],

iBCFRecord			(82),

--

--  Record values 70 is for Flow based Charging

--  The contents are defined in TS 32.251 [11]

--

--

--  Record values 71..75 are LCS specific.

--  The contents are defined in TS 32.271 [31]

--

lCSGMORecord		(71),

lCSRGMTRecord		(72),

lCSHGMTRecord		(73),

lCSVGMTRecord		(74),

lCSGNIRecord		(75),

--

--  Record values 76..79 are MBMS specific.

--  The contents are defined in TS 32.251 [11]

--  Record values 76 and 77 are MBMS bearer context specific

--

sgsnMBMSRecord		(76),

ggsnMBMSRecord		(77),

--

--  And TS 32.273 [33]

--  Record values 78 and 79 are MBMS service specific

--  and defined in TS 32.273 [33]

--

sUBBMSCRecord		(78),

cONTENTBMSCRecord	(79),

--

--  Record Values 80..81 are PoC specific.

--  The contents are defined in TS 32.272 [32]

--

pPFRecord			(80),

cPFRecord			(81)

--  Record values 84..85 are EPC specific.

--  The contents are defined in TS 32.251 [11]

--

sGWRecord			(84),

pGWRecord			(85)

--

--  Record Value 83 is MMTel specific.

--  The contents are defined in TS 32.275 [35]

--

mMTelRecord			(83)

}

CallingNumber	::= BCDDirectoryNumber

#### …

Next Modified Section

#### 5.2.3.1	IMS CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.260 [20].

IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

-- EXPORTS everything

IMPORTS

RecordType, IPAddress, ManagementExtensions, NodeAddress, LocalSequenceNumber, SubscriptionID, TimeStamp, ServiceContextID, ServiceSpecificInfo

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

;

------------------------------------------------------------------------------

--

--  IMS RECORDS

--

------------------------------------------------------------------------------

IMSRecord ::= CHOICE

--

-- Record values 63-69, 82 are IMS specific

--

{

sCSCFRecord			[63] SCSCFRecord,

pCSCFRecord			[64] PCSCFRecord,

iCSCFRecord			[65] ICSCFRecord,

mRFCRecord			[66] MRFCRecord,

mGCFRecord			[67] MGCFRecord,

bGCFRecord			[68] BGCFRecord,

aSRecord			[69] ASRecord,

eCSCFRecord			[70] ECSCFRecord,

iBCFRecord			[82] IBCFRecord

}

…

ECSCFRecord 	::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node						[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address				[7] InvolvedParty OPTIONAL,

privateUserID						[8] GraphicString OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers			[14] InterOperatorIdentifierlist OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber				[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components		[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress							[22] NodeAddress OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions					[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

list-Of-Associated-URI				[27] ListOfInvolvedParties OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation			[29] OCTET STRING OPTIONAL,

serviceContextID					[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components 	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

numberPortabilityRouting			[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting				[35] CarrierSelectRouting OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

applicationServersInformation		[40] SEQUENCE OF ApplicationServersInformation OPTIONAL,

requested-Party-Address				[41] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity 	[42] ListOfInvolvedParties OPTIONAL

}

--

-- Editor’s note: The completeness of all parameters for the E-CSCF CDR is ffs.

--

------------------------------------------------------------------------------

--

--  IMS DATA TYPES

--

------------------------------------------------------------------------------

…

END

End of modifications